3GPP TSG-SA5 Meeting #159	S5-250533

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	New solution for topic 11

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

[2]	3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture; Phase 3

# 3	Rationale

New solution for

# 4	Detailed proposal

First change

#### 6.1.4.x	Solution #1.x: MME CDF/CGF based solution for S&F operation charging with User Plane CIoT

This solution which relying on EPC CDF/CGF for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the TS 23.401 [5], only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data.

UE using satellite access with S&F operation indication in attach or TAU request procedure is known by MME, CDR generation in MME can be used to charge the S&F operation of the User Plane CIoT service. For the whole EPC on-board architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC on-board architecture is the same as the Figure 6.1.4.4-2 of the solution#1.4.

The MME onboard reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger event:

-	Attach/TAU to the MME onboard for the CIoT UP Optimizations;

Table 6.1.4.x-1 shows the needed extensions for the M-CDR compared with the Table 6.1.4.1 in TS 32.251 [7].

Table 6.1.4.4-1 Extension to M-CDR for S&F
 (3GPP TS 32.251 [7] – Table 6.1.4.1)

| Field | Category | Description |
| --- | --- | --- |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the UE, extended with RAT types for satellite. |
| Satellite information | OC | This field holds all the satellite information that support to finish the store and forward satellite operation. |
| Satellite Access Indicator | OC | This field indicates the use of satellite access. |
| S&F Indicator | OC | This field indicates the use of store and forward satellite operation. |
| S&F Monitoring List | OC | This field holds the satellite IDs that used for the store and forward satellite operation. |

Second change

#### 6.1.5.1	Solutions evaluation for Key issue #1.1

Solutions #1.1 addresses Key issue #1.1, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The new trigger events are Short Message sent by a UE via the MME on-board (MO direction) to the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.2 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT CP Optimizations and Service Request via the MME onboard for the CIoT CP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.3 addresses Key issue #1.1, for the User Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT UP Optimizations and Service Request via the MME onboard for the CIoT UP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator and S&F Monitoring lists.

Solutions #1.4 addresses Key issue #1.1, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The CDF/CGF may be deployed on the satellite. The new trigger events are Short Message sent by a UE via the MME on-board (MO direction) to the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring List, S&F Duration and S&F Data volume.

Solutions #1.5 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new charging information. The CDF/CGF may be deployed on the satellite. The extension to CPDT-SNN-CDR for S&F charging include RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.6 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new charging information. The CDF/CGF may be deployed on the satellite. There is no enhancement required for the SCEF CDRs needed.

Solutions #1.x addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-03, for the User Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach/TAU to the MME onboard for the CIoT UP Optimizations and Service Request. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator and S&F Monitoring lists.

All solutions propose to extend the triggers for MME CDRs and the information in the MME CDRs, except solution #1.6 which proposes to reuse the SCEF CDR. The solutions #1.1, #1.2, and #1.3 also have some indication of extending Diameter, this however not described.

End of changes