3GPP TSG-SA5 Meeting #159	S5-250534

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	Solution update for topic 3

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

# 3	Rationale

Update of solution architecture update as well as correcting missing references and references to wrong potential requirements.

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

[4]	3GPP TR 28.844: "Study on charging aspects of satellite in the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TR 22.822: "Study on using Satellite Access in 5G ".

[7]	3GPP TS 32.251: " Packet Switched (PS) domain charging".

[8]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

[12]	3GPP TS 32.253: "Control Plane (CP) data transfer domain charging".

[13]	3GPP TS 32.299: "Diameter charging applications".

[14]	3GPP TS 32.298: "Charging Data Record (CDR) parameter description".

[x]	3GPP TS 32.257: "Telecommunication management; Charging management; Edge computing domain charging".

Second change

#### 6.4.1.1	Use Case #4.1: MNO charges SCC

This use case focuses on SCC and MNO business scenario.

An SCC has a subscription with an MNO which rents the satellite from an SSP, to allow MNO to provide UE-sat-UE to its subscribers.

The charging party and charged party can be:

-	Charged party: the SCC identified by the UE.

-	Charging party: MNO.

The MNO charges the subscribers based on the usage of satellites to provide the UE-sat-UE communication service.

Potential charging requirements: REQ- CH_ SAT_PH3-01.

Third change

#### 6.4.1.2	Use Case #4.2: MNO charged by SSP

This use case focuses on MNO and SSP business scenario.

An MNO has a wholesale agreement to use the satellite from an SSP.

The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP

The SSP charges MNO based on the usage of satellites.

Potential charging requirements: REQ- CH_ SAT_PH3-01.

Fourth change

##### 6.3.4.1.2	Architecture description

Figure 6.3.4.1.2-1: Satellite resource usage charging architecture

The SMF and CEF interact with CHF using Nchf interface. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP. The wholesale part is only included for completeness and is currently outside the scope of 3GPP SA5.

SMF collect charging data related to satellite backhaul (satellite backhaul category, data volumes) for each subscriber according to TS 32.255 [8].

CEF collect charging data related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes) according to TS 32.257 [x].

Fifth change

##### 6.3.4.1.3	Procedures description

The message flows for satellite backhaul for each subscriber would be the same as in clause 5.2.2 of TS 32.255 [8]. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP.

The message flows for infrastructure resource usage would be the same as in clause 5.2.2 of TS 32.257 [x]. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP.

The message flows for EAS deployment would be the same as in clause 5.2.3 of TS 32.257 [x]. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP.

End of changes