3GPP TSG-SA5 Meeting #159 	S5-250551

# Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Discussion Paper on modelling energy related information

# Document for:	Discussion

Agenda Item:	6.19.20.1

# 1	Decision/action requested

Discuss the proposal

# 2	References

[1]	TR 28.880 v2.0.0: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Discussion

SA1 requirements on ‘energy related information’ are captured in TS 22.261.

The Energy Information Function (EIF) newly introduced into TS 23.501 may consume gNB and UPF Energy Consumption and Data Volume measurements from OAM, but also, when SA5 has specified them, other energy related information such as gNB / UPF carbon emission and renewable energy factor.

In Rel-19, it is proposed that the following gNB and UPF energy related information is introduced in SA5 specification(s):

- Carbon Emission (CE) – see definition in TR 28.880

- Carbon Emission Efficiency (CEE) – see definition in TR 28.880. This term could be named differently in the future

- Renewable Energy Factor (REF) - see definition in TR 28.880.

The following key points need to be considered to specify this energy related information:

- Key Point #1: According to ITU-T L.1333, ICT (Information and Communication Technology) sites (corresponding to ‘operator sites’ in TR 28.880) can be powered via 1-to-3 types of different energy supplies, namely:

- GRID_ELECTRICITY: the energy consumed comes from grid

- BACKUP_ENERGY: the energy comes from backup diesel or other types, or from generators

- LOCALLY_GENERATED_ENERGY: energy provided by local generators not used for backup functionality, e.g., solar, wind generator. LOCALLY_GENERATED_ENERGY is also known as OFF_GRID ENERGY

In our proposed model, the IOC EnergySupplyType represents these 3 types of energy supply. Not more than 3 MOIs of this IOC should exist. Its attribute energySupplyTypeUserLabel may have one of the 3 aforementioned values.

- Key Point #2: The information capturing that operator sites are powered using GRID_ELECTRICITY and/or BACKUP_ENERGY and/or LOCALLY_GENERATED_ENERGY is stored in MNO systems / databases external to the 3GPP management system. Let’s call these systems / databases the ‘MNO energy management system’.

It is assumed here that the 3GPP management system has no machine-to-machine interface to the MNO energy management system (whether such an interface is to be specified in the future is FFS). As a consequence, if required by the 3GPP management system, this information will have to be gathered by the MNO from the MNO energy management system and provisioned into the 3GPP management system via the Provisioning MnS.

- Key Point #3: The granularity of this information is usually per-operator site (ICT site). However, in some scenarios, the granularity may be coarser, i.e. per sub-network (corresponding e.g. to a region), or finer, i.e. per network element / network function (e.g. a gNB). To allow this, it is proposed that a proxy class ‘ManagedEntity’ is used to represent the different possibilities. The proposed <<ProxyClass>> ManagedEntity can then represent SubNetwork, or ColocatedNFsGroup or ManagedElement or ManagedFunction.

In our proposed model, ColocatedNFsGroup is a new IOC to represent a group of network elements or network functions co-located on the same operator site. Co-location refers to the sharing of the same type(s) of energy supply by the group of network elements or network functions. Its attribute memberMEList is a list of DNs of the members of the group.

- Key Point #4: in our proposed model, the IOC EnergySupplyInfo captures, for any ManagedEntity (cf. Key Point #3), which type(s) of energy supply (cf. Key Point #1) is used.

  - EnergySupplyInfo is name-contained by ManagedEntity. If a managed entity is powered via Grid and Backup electricity (cf. Key Point #1), there will be 2 EnergySupplyInfo MOIs name-contained by this ManagedEntity

  - Its attribute energySupplyTypeRef is used to refer to the type of energy supply (cf. Key Point #1).

  - Its attribute energySupplyTypeRatio indicates the percentage of energy supplied to the ManagedEntity from the type of energy supply.

- Key Point #5: for each type of energy supply (cf. key point #1), the energy may come from 1-to-many type(s) of energy source, e.g. nuclear and/or coal, and/or wind and/or … ETSI OEU 020 1.1.1 (2020-03) informative annex A.3 provides a list of types of energy source. Other references may be used by the MNO, if available.

In our proposed model, the IOC EnergySourceType represents these types of energy source.

  - Its attribute energySourceTypeUserLabel captures the type of energy source (possible value: ‘nuclear’ or ‘coal’ or ‘solar’ or …).

  - Its attribute energySourceCef captures the corresponding Carbon Emission Factor (CEF), also called Standard Emission Factor in ETSI OEU 020 1.1.1 (2020-03) informative annex A. Its unit is kg CO2eq / kWh.

  - Its attribute renewableEnergyFactor is a percentage (0..100) indicating to which extent the type of energy source is renewable or not. To which extent a type of energy source is to be considered as renewable or not is under the responsibility of the MNO. For example, solar and wind could be considered as 100% renewable

- Key Point #6: for the energy provided from the Grid (cf. GRID_ELECTRICITY in Key Point #1) by MNO energy supplier, it is assumed that the energy related information mentioned in Key Point #5 is periodically provided by the energy supplier to the MNO, e.g. as part of the information of the invoice or by other means.

- Key Point #7: for the energy provided from the Off-Grid (cf. LOCALLY_GENERATED_ENERGY in Key Point #1), it is assumed that the energy related information mentioned in Key Point #5 is:

  - Either periodically provided by the off-grid energy supplier (if any) to the MNO, e.g. as part of the information of the invoice or by other means,

  - Or directly known to the MNO energy management system in case the local off-grid energy supply solution is an MNO asset.

- Key Point #8: for the energy provided from the local back-up battery (cf. BACKUP_ENERGY in Key Point #1), 3 possibilities (not mutually exclusive):

  - The battery is (partly or wholly) filled in from the Grid; in this case, the energy related information mentioned in Key Point #5 is obtained as per Key Point #6,

  - The battery is (partly or wholly) filled in from the local Off-Grid; in this case, the energy related information mentioned in Key Point #5 is obtained as per Key Point #7,

  - The battery is (partly or wholly) filled in using local means, e.g. gasoline, generator; in this case, the energy related information mentioned in key Point #45 is captured by the MNO in the MNO energy management system.

- Key Point #9: in our proposed model, the IOC EnergySourceInfo captures, for any EnergySupplyInfo (cf. Key Point #4), which type(s) of energy source (cf. Key Point #5) is used.

  - EnergySourceInfo is name-contained by EnergySupplyInfo. If an EnergySupplyInfo MOI uses 2 types of energy source (cf. Key Point #5), there will be 2 EnergySourceInfo MOIs name-contained by this EnergySupplyInfo,

  - Its attribute energySourceRatio indicates the percentage of energy supplied by a given type of energy source.

- Key Point #10: In case the MNO is not in possession of this energy related information on a per gNB and/or UPF granularity, the MNO should take instead the per operator site (where the gNB / UPF is located) energy related information.

- Key Point #11: gNB / UPF Carbon Emission (CE) should be modelled as a new KPI in TS 28.554, calculated based on their:

  - Energy Consumption (EC) (already defined in TS 28.552 / TS 28.554)

  - EnergySupplyInfo

  - EnergySourceInfo

- Key Point #12: gNB / UPF Carbon Emission Efficiency (CEF) should be modelled as a new KPI in TS 28.554, calculated based on their:

  - CarbonEmission (cf. Key Point #11)

  - DataVolume (already defined in TS 28.552)

Proposal: it is proposed to introduce the corresponding NRM into TS 28.310, as a new clause. The diagram here below shows the proposed relationship diagram:

An illustration of how this model would be instantiated on a simplified example would give the following instance diagram: