3GPP TSG-SA5 Meeting #159	S5-250552

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Orange, AT&T

# Title:	R19 pCR TR 28.869 add Observability for CNFs

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.869 Study on cloud aspects of management and orchestration

# 3	Rationale

- The objective of this pCR is to adapt the management functions to the cloud native evolution.

- This pCR proposes a new use case “Observability for Cloud Native Network functions” in TR 28.869 (Study on cloud aspects of management and orchestration).

- If not adopted, it would slow down the adoption of Cloud native concepts in 5GA/6G networks.

# 4	Detailed proposal

1st Modified Section

5.2.7	Use case #7: Observability for Cloud Native Network functions

5.2.7.1	Description

As we move to a native cloud architecture, it is essential that we adapt our network management standards to ensure optimal efficiency and increased service resiliency. As a key player in this area, we propose to integrate the following elements into the standard:

1. **Cloud Native network management solution ** Adopt an approach that supports the integration of containerized microservice-based network and infrastructure management solutions, allowing for increased scalability and flexibility.

2. **Observability** Implement robust observability mechanisms for proactive telemetry data analysis, failure detection and rapid response, building on gitops and other industry standard methods.

3. **Interoperability** Ensure interoperability between different vendors, providers and operators for harmonized network management in a multi-cloud environment.

4. **Energy Efficiency** - Integrate  EE by design in the network management solution.

observability solution shall be capable of measuring the EE of cloud native network functions using the management processes, in line with 3GPP industry initiatives.

5. **Security** - security by design for open observability protecting confidentiality and integrity of data transfer. Adoption of secure communication layers such as TLS.

6. **AI driven** - We need to consider adoption of AI-driven approaches to make the most of observability data in order to understand the root-cause analysis, data transaction monitoring, automated discovery, geo-hosted network functions topology mapping, predictive maintenance, capacity planning, security threats identificationand utilization for further optimization (as examples and not necessarily limited to).

This proposal is in line with the discussions held at the 3GPP SA5 meetings and meets the needs expressed by operators and vendors.

5.2.7.2	Potential requirements

PREQ-OBS_CNF-Cloud-OAM-01: Generate PM Counters and Metrics

PREQ-OBS_CNF-Cloud-OAM-02: Generate Logs

PREQ-OBS_CNF-Cloud-OAM-03: Generate Traces

PREQ-OBS_CNF-Cloud-OAM-04: Generate Events

PREQ-OBS_CNF-Cloud-OAM-05: Industry standard open format

PREQ-OBS_CNF-Cloud-OAM-06: Provide an industry standard data collection and reporting mechanism

PREQ-OBS_CNF-Cloud-OAM-07: Provide data correlation capabilities.

5.2.7.3	Potential solutions

Solution 1: Use of Open Telemetry as an observability framework

CNF /

Figure 1: Open Telemetry diagram

The figure above is showing the Open telemetry architecture diagram as detailed on www.opentelemetry.io

The use of this framework in the 3GPP context can be achived by the integration of this into the network management system as a MnS.

The main benefits of using such an approach would be: cloud-native, vendor-agnostic instrumentation, scalability, better AI use, higher performance, reduced TTM, interoperability, reliability.

Solution 2: Use of NETCONF events as a complementary data source

SNMP is the legacy network management protocol, standardized by IETF and used to send alarms (SNMP traps) from network components or upper layer systems (at the EMS NBI for example). SNMP is widely used, and SNMP v3 is recommended with the associated security features.

Other protocols are emerging to replace SNMP which are more scalable, secure and support flexible integration in a cloud native environment:

NETCONF (IETF standard) intends to replace SNMP. NETCONF is using YANG (IETF standard: YANG v1.1 specifications in RFC7950) as a data modelling language which models configuration and state data of network elements.

RFC 8632 defines a YANG data model for alarm management. It includes functions for alarm-list management, alarm shelving, and notifications to inform management systems. There are also operations to manage the operator state of an alarm and administrative alarm procedures.

The evolution of the definition of alarm moves from focused on events reporting a deviation from normal operation towards a definition to an undesired « state » that « requires an operator action ».

Solution 3: Rely on OpenTelemetry Semantic Conventions in this context

The Semantic Conventions define a common set of (semantic) attributes which provide meaning to data when collecting, producing and consuming it. The Semantic Conventions specify among other things span names and kind, metric instruments and units as well as attribute names, types, meaning and valid values. The benefit to using Semantic Conventions is in following a common naming scheme that can be standardized across a codebase, libraries, and platforms. This allows easier correlation and consumption of data.*

(*) from https://opentelemetry.io/docs/specs/semconv/

End of modification