3GPP TSG-SA5 Meeting #159	S5-250570

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0252 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel 19 CR TS 28.105 Various corrections for Clause 6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-01-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are a number of spelling mistakes which are corrected. There are also incomplete clause names which are corrected. There is an incomplete quote from 28.104 that changes the meaning of the quoted text, the quote is completed from 28.104, now accurately reflecting that it refers to an MDA capability where previously it could be perceived as MDAS/MDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Various spelling errors corrected, incomplete headings are fixed, incomplete quote from 28.104 is replaced with the full quote. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Poor qulaity spec may lead to implementation errors. Misleading quotation from another published spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.2.9.3, 6.2c.2.4.3, 6.2c.2.4.4, 6.5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 6.2b.2.9.3	ML model performance indicators query and selection for ML Training

ML model performance evaluation and management are needed during training. The related performance indicators need to be collected and analyzed. The MnS producer of ML model training should determine which indicators are needed, i.e., select some indicators based on the use case and use these indicators for performance evaluation.

The ML MnS consumer may have different requests on AI/ML performance, depending on its use case and requirements, which may imply that different performance indicators may be relevant for performance evaluation. The MnS producer for ML model training can be queried to provide the information on supported performance indicators referring to ML model training. Such performance indicators for training may be for example accuracy/precision/recall/F1-score/MSE/MAE.  Based on supported performance indicators as well as based on consumer’s requirements, the MnS consumer for ML model training may request a sub-set of supported performance indicators to be monitored and used for performance evaluation. Management capabilities are needed to enable the MnS consumer for ML model training or to query the supported performance indicators and select a sub-set of performance indicators to be used for performance evaluation.

Next Change

##### 6.2c.2.4.3	ML model performance indicators query and selection for ML model testing

In a similar way as for training, the MnS producer of ML model training or testing should determine which indicators are needed, i.e., select some indicators based on the use case and use these indicators for performance evaluation.The ML MnS consumer for ML model testing may have different requests on AI/ML performance, depending on its use case and requirements, which may imply that different performance indicators may be relevant for performance evaluation. The procedure is the same as described in 6.2b.2.9.3 for training.

##### 6.2c.2.4.4	MnS consumer policy-based selection of ML model performance indicators for ML model testing

In a similar way as for training, the MnS consumer for ML model testing may have differentiated levels of interest in the different performance dimensions or metrics. Thus, depending on its use case, the AI/ML MnS consumer may indicate the preferred behaviour and performance requirement that needs to be considered during testing. The same description in 6.2b.2.9.4 applies for policy based selection of performance indiactors for testing.

Next Change

#### 6.5.3.2	Use cases

##### 6.5.3.2.1	Identifying capabilities of ML models

Network functions, especially network automation functions, may need to rely on capabilities of ML models that are not internal to those network functions to accomplish the desired automation (inference). For example, as stated in TS 28.104 [2], “An MDA Function may optionally be deployed as one or more AI/ML inference function(s) in which the relevant ML models are used for inference per the corresponding MDA capability.” Similarly, owing to the differences in the kinds and complexity of intents that need to be fulfilled, an intent fulfillment solution may need to employ the capabilities of existing AI/ML inference functions to fulfill the intents. In any such case, management services are required to identify the capabilities of those existing ML models that are employed by AI/ML inference functions.

Figure 6.5.3.2.1-1: Request and reporting on AI/ML inference capabilities

Figure 6.5.3.2.1-1 shows that the consumer may wish to obtain information about the available AI/ML inference capabilities to determine how to use them for the consumer's needs, e.g., for fulfillment of intent targets or other automation targets.

End Changes