3GPP TSG SA WG5 Meeting #159 	S5-250704

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	CATT

# Title:	Add TDF based solution for S&F operation charging with User Plane CIoT

# Document for:	Approval

# Agenda Item:	7.5.1

1	Decision

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.846: " Study on charging aspects of satellite access Phase 3 ".

3	Rationale

According to the conclusion of the SA2 SAT_Ph3_ARCH, the feature of S&F operation only supports the EPC in the Rel-19. Only delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) have been supported in the Rel-19.

This pCR proposes to add S&F operation charging solutions for User Plane CIoT.

As specified in the TS 23.401, only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data. Moreover, the UE using satellite access with S&F operation indication in the initial attach or the service request procedure can only be known by MME. The P-GW can not get the information about the S&F. In addition, the EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE.

The TS23.401, TS 23.203 and TS32.251 need to be enhanced to enable the S&F information to be transferred from the AS to TDF via PCRF. Therefore, this solution proposes to use TDF to trigger to charge the S&F operation of the User Plane CIoT service.

4	Detailed proposal

The following changes are proposed to be incorporated into the new TR.

1st Modified Section

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

[4]	3GPP TR 28.844: "Study on charging aspects of satellite in the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TR 22.822: "Study on using Satellite Access in 5G ".

[7]	3GPP TS 32.251: " Packet Switched (PS) domain charging".

[8]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

[12]	3GPP TS 32.253: "Control Plane (CP) data transfer domain charging".

[13]	3GPP TS 32.299: "Diameter charging applications".

[14]	3GPP TS 32.298: "Charging Data Record (CDR) parameter description".

[x]	3GPP TS 23.203: "Policy and charging control architecture".

2nd Modified Section

6.1.4.x	Solution #1.x: TDF Charging Trigger Function (CTF) based solution for S&F operation charging with User Plane CIoT

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the TS 23.401[5], only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data. The EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. The TS23.401[5], TS 23.203[x] and TS32.251[7] would need to be enhanced to enable the S&F information to be transferred from the AS to TDF via PCRF.

For the whole EPC onboard architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC on-board architecture is shown below:

Figure 6.1.4.x-1: Whole EPC on-board charging architecture

The TDF onboard reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following chargeable events:

-	PDN connection creation/release via the AS onboard for the User Plane data transfer.

Table 6.1.4.x-1 shows the enhancements needed for TDF-CDR compared with the Table 6.1.14.1 in TS 32.251 [7].

Table 6.1.4.X-1 Extension to TDF-CDR for S&F (3GPP TS 32.251 [7] – Table 6.1.14.1)

| Field | Category | Description |
| --- | --- | --- |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the UE, extended with RAT types for satellite. |
| Satellite information | OC | This field holds all the satellite information that support to finish the store and forward satellite operation. |
| Satellite Access Indicator | OC | This field indicates the use of satellite access. |
| S&F Indicator | OC | This field indicates the use of store and forward satellite operation. |
| S&F Monitoring List | OC | This field holds the satellite IDs that used for the store and forward satellite operation. |
| S&F Duration | OC | This field holds the storage duration of data on the satellite. |
| S&F Data volume | OC | This field holds the data volume stored on the satellite. |

End of Modified Sections