3GPP TSG-SA5 Meeting #159	S5-250896

Sophia-Antipolis, FR, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0060 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Add informative annex on how EIF can collect energy information from OAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, the Energy Information Function (EIF) has been introduced in TS 23.501 as a new 5G core network function. The EIF collects energy related information about gNBs and UPFs from the OAM. It is required to explain how EIF and OAM can interact in this context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce an informative annex to explain how EIF and OAM can interact to enable the EIF to collect energy related information about gNBs and UPFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How EIF can collect energy related information about gNBs and UPFs from OAM would not be described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[3]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface".

[4]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[5]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[6]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[7]	Void.

[8]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[9]	3GPP TS 32.435: "Telecommunication management; Performance measurement; eXtensible Markup Language (XML) file format definition".

[10]	3GPP TS 32.436: "Telecommunication management; Performance measurement: Abstract Syntax Notation 1 (ASN.1) file format definition".

[11]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[12]	3GPP TS 38.401: "NG-RAN; Architecture description".

[13]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[14]	Void.

[15]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[16]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[17]	3GPP TS 32.551: "Energy Saving Management (ESM); Concepts and requirements".

[18]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[19]	ETSI GR NFV-IFA 015 V3.4.1 (2020-06): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Report on NFV Information Model".

[20]	ETSI GR NFV 003 V1.6.1 (2021-03): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[21]	3GPP TS 28.530: " Management and orchestration; Concepts, use cases and requirements".

[22]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[23]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[24]	Void.

[25]	ETSI GS NFV-IFA 027 (V4.3.1) (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[26]	ETSI ES 202 336-12 (V1.2.1) (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[27]	ETSI GS NFV-IFA 008 V3.5.1 (2021-11): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[28]	ETSI GS NFV-TST 008 V3.3.1 (2020-12): "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification".

[29]	ETSI GS NFV-IFA 006 (V3.7.1) (2022-11): " Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Vi-Vnfm reference point – Interface and Information Model Specification".

[30]	3GPP TS 28.111: "Fault management".

[x]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

Next change

Annex X (Informative): Example of how EIF (as MnS consumer) may collect energy related information from MnS producer

As shown by Figure 4.2.18-2 (Non-roaming architecture for Energy Efficiency and Energy Saving), and as described in clause 5.51.2.2 (Energy Consumption information collection) of TS 23.501 [x], the Energy Information Function (EIF) may collect energy related information from OAM, mainly:

- gNB energy consumption (see TS 23.501 [x] clause 5.51.2.2)

- gNB data volume (see TS 23.501 [x] clause 5.51.2.2)

- UPF energy consumption (see TS 23.501 [x] clause 5.51.2.2)

- UPF data volume (see TS 23.501 [x] clause 5.51.2.2)

- other information if available (see TS 23.501 [x] clause 5.51.2.4)

.

To achieve this, EIF acts as a Management Service (MnS) Consumer, and consume Management Services (MnS) Management service APIs.

For sake of simplicity, the rest of this annex focuses on collecting UPF energy related information from OAM only.

In 5GC:

- 5GC NF (incl. UPF) instances are uniquely identified by nfInstanceId

- NRF maintains the list of nfInstanceId of all 5GC NF instances, including UPFs (nfType = UPF)

In 3GPP Management System:

- 5GC NF instances are represented by Managed Object Instances (MOIs)

- MOIs are uniquely identified by Distinguished Names (DNs)

, and nfInstanceId is different from the DN of MOIs. However, it is worth noticing that nfInstanceId (used within 5GC to uniquely identify 5G core network functions) is captured, for each Information Object Class (IOC) representing these 5G core network functions in the 3GPP management system), via the attribute managedNFProfile.nfInstanceId – see TS 28.541 [11].

To be able to collect UPF energy related information, EIF has at least the following information:

- the address of a Provisioning MnS producer (say Prov_MnS_A),

- the DN of the SubNetwork MOI (say SN_X) which name-contains the UPFFunction MOIs representing the UPFs for which EIF wants to collect energy related information.

NOTE: in case the UPF instances do not all belong (directly or indirectly) to the same sub-network, the information above is needed for each sub-network.

The above information can be either configured by OAM or pre-provisioned in the EIF.

To collect UPF energy related information modelled as KPI(s) (cf. TS 28.554), the procedure is as follows:

Figure X-1: Sequence diagram – EIF (as Provisioning MnS consumer) collects DNs of UPF instances from Provisioning MnS producer

1/ EIF needs to get, from OAM, the DN of the UPFFunction MOIs which represent the 5GC UPF instances about which EIF wants to collect KPIs

1-1/ EIF sends a getMOIAttributes operation request to the Provisioning MnS producer with as the following list of input parameters:

- baseObjectInstance = SN_X, i.e. the DN of the SubNetwork MOI which name-contains the UPFFunction MOIs

-  scopeType = BASE_ALL, to indicate that indicates that the base object (i.e. SN_X) and all of its subordinate objects (incl. the leaf objects) are selected

- filter, i.e. criteria to be applied to the objects selected. The criteria is that, to be select, MOIs shall have their objectClass attribute equal to ‘UPFFunction’, and their nfInstanceId attribute equal to the nfInstanceId of one of the UPF instances about which EIF wants to collect KPIs

- attributeListIn, i.e. the list of attributes whose values are to be returned by the MnS producer

1-2/ The Provisioning MnS producer processes EIF’s request

1-3/ The Provisioning MnS producer sens a getMOIAttributes operation response to EIF with, for each selected MOI:

- its managedObjectClass attribute value (‘UPFFunction’ in this case)

- its managedObjectInstance attribute value, i.e. its DN

- its nfInstanceId attribute value

2/ EIF requests to create a PerfMetricJob MOI to collect required KPIs:

Figure X-2: Sequence diagram – EIF (as Provisioning MnS consumer) initiates a PerfMetricJob to collect KPIs

2-1: EIF sends a createMOI operation (see TS 28.532 clause 11.1.1.1) request to the Provisioning MnS producer Prov_MnS_A to request the creation of a PerfMetricJob MOI (cf. TS 28.622 [y] clause 4.3.31), with the following (non-exhaustive) list of input parameters:

•	performanceMetrics = <name of the KPI(s) to be collected>, i.e. ECNF (i.e. UPF Energy Consumption - see TS 28.554 [18]), GTP.InDataOctN3UPF and GPT.OutDataOctN3UPF (i.e. UPF Data Volume provided by the number of octets of incoming (respectively outgoing) GTP data packets on the N3 interface which have been generated by the GTP-U protocol entity – see TS 28.552 [15])

•	granularityPeriod, to indicate the granularity period to be applied to produce expected energy related information KPI value(s), in seconds;

•	objectInstances, , i.e. the list of DNs of the UPFFunction MOIs representing the UPFs for which EIF wants to collect the energy related information KPI value(s) – see step 1;

•	reportingCtrl, to indicate the method and associated control parameters for reporting the produced KPI value(s) to the MnS consumer. Three methods are available:

-	file-based reporting with selection of the file location by the MnS producer (i.e. OAM). In this case, EIF indicates the fileReportingPeriod which specifies the time window during which collected measurements are stored into the same file before the file is closed and a new file is opened. Again, two options:

- the MnS producer supports file-based reporting and storing files on the MnS producer, or

- the MnS producer supports file-based reporting, storing files on the MnS producer and implicit notification subscription (i.e. EIF will automatically receive a notification when a new file is ready, with no need to subscribe). The EIF indicates the notificationRecipientAddress, i.e. the address of the notification recipient

-	file-based reporting with selection of the file location by the EIF. In this case, EIF indicates the fileLocation

-	stream-based reporting. In this case, the EIF indicates streamTarget, i.e. the location at which the MnS producer will send the streamed data.

2-2/ Provisioning MnS producer Prov_MnS_A receives the createMOI operation request from EIF and starts the KPI(s) calculation

2-3/ Provisioning MnS producer Prov_MnS_A sends responses to EIF as follows:

- In case of stream-based reporting, OAM sends out the energy related information KPI value(s) to EIF as soon as they are available, via WebSocket. The serialization format can be either GPB or ASN.1

- In case of file-based reporting, OAM fills in a file with energy related information KPI value(s) (one per granularity period) until the reporting period is reached and, depending on EIF preference (indicated via aforementioned input parameters)

- stores the file locally, or

- stores the file locally and sends a notification to EIF (see notificationRecipientAddress input parameter) indicating that the file is available, or

- stores the file locally and sends a notification to EIF indicating that the file is available at the specified location (see notificationRecipientAddress input parameter)

The file format can be either ASN.1 or XML.

2-4/ EIF, as Provisioning MnS consumer) gets requested energy related information KPI value(s).

End of change