3GPP TSG-SA5 Meeting #159	S5-250943

Sophia-Antipolis, France, 17 - 21 February 2025                           (revision of S5-250431)

# Source:	NTT DOCOMO, Nokia

# Title:	Rel-19 pCR TS 28.567 Add use-case and Requirements for CCL for Fault Management

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

The group is asked to discuss and approval

# 2	References

[1]	3GPP TR 28.867: "Study on closed control loop management"

# 3	Rationale

This contribution proposes to add in TS 28.567 the following:

- Introduction for use-case on CCL for fault management, as the use-case was studied in Clause 5.5.1 of TR 28.867 and recommended for normative phase in Clause 6.6 of TR 28.867.

- Requirements for use-case on CCL for fault management as it was studied in Clause 5.5.2 and recommended for normative phase in Clause 6.6 of TR 28.867

# 4	Detailed proposal

1st Change

## 5.a.2.b	Closed Control Loops for fault management - CCLFM

This use case describes a scenario in which an MnS consumer may request a CCL for fault management. The consumer may request to identify the root cause of the fault and take actions to mitigate and/or resolve the root cause for a given list of alarms. Furthermore, the request may include policies and actions specified by an MnS consumer in order to mitigate and/or resolve the root causes for the given alarms.

Based on the request, a CCL may take action to further enhance the correlation of alarms, for example, correlation of alarms with change in PM/KPIs and/or fault supervision events to find solutions to mitigate and/or resolve the identified root causes.  In addition, a fault management CCL may clear the alarms that otherwise have to be manually cleared by the MnS consumer, which are defined as ADMC Alarms  in TS 28.111.

The MnS producer reports the result of fault management. The report may include information regarding the status of each alarm, including any identified root cause and correlation information, which may indicate the successful mitigation and/or resolving of root causes for any given alarm.

2nd Change

## 5.a.3	Requirements

| REQ-CCLFM-01 | The 3GPP management system shall have the capability to allow MnS consumer to request a closed control loop for fault management | CCLFM |
| --- | --- | --- |
| REQ-CCLFM-02 | The 3GPP management system shall have the capability to allow MnS consumer to get a report from the closed control loop regarding the status of fault management | CCLFM |

End of changes