3GPP TSG-SA5 Meeting #159	S5-251026

Sophia-Antipolis, France, 17 - 21 February 2025				revision of S5-250635

# Source:	Ericsson, Deutsche Telekom

# Title:	Discussion paper on 5GS predefined PCC rules

# Document for:	Approval

Agenda Item:	6.3.11

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 23.203:  "Policy and charging control system architecture"

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2"

[3]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

[4]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[5]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3"

# 3	Rationale

### 3.1	Background

Comparison of PC Rule in TS 29.512 (Clause 4.1.4.2) and PccRule in TS 28.541 (Clause 5),

PC Rule in TS 29.512 (Clause 4.1.4.2)

Definition and Types:

- A PCC rule is a set of information elements enabling the detection of a service data flow and providing parameters for policy control and/or charging control. There are two types of PCC rules:

1) Dynamic PCC Rules: Provisioned by the PCF to the SMF and can be predefined or dynamically generated in the PCF.

2) Predefined PCC Rules: Preconfigured in the SMF and can be activated or deactivated by the PCF.

Components:

- Rule Identifier: Uniquely identifies the PCC rule within a PDU session and is used for referencing between PCF and SMF.

- Precedence: Determines the order of service data flow templates for enforcement and charging.

- Service Data Flow Template: Specifies traffic patterns for IP or Ethernet PDU traffic.

- QoS Parameters: Defines the QoS requirements for the service data flow.

- Charging Parameters: Provides details for charging control.

Functionality:

- Dynamic Rules: Can be installed, modified, or removed at any time.

- Predefined Rules: Can be grouped into rule bases for dynamic activation by the PCF.

Key Notes:

- Predefined rules within a rule base are not individually known by the PCF, only the base identifier is known.

- The PCF handles the interaction logic based on events related to PCC rules​ (29512-i60)​.

PccRule in TS 28.541 (Clause 5)

Structure and Attributes:

- A PccRule data structure includes various attributes such as pccRuleId, precedence, and flowDescription.

- pccRules map attribute: Used within the SmPolicyDecision data structure to include impacted PCC rules, identified by pccRuleId.

- Attributes for Activation: To activate a predefined PCC rule, the pccRuleId and, if applicable, refCondData must be provided. For dynamic rules, additional attributes like precedence and packet filter usage are set.

Operations:

- Activation/Deactivation: The PCF includes the PccRule data structure to activate predefined rules or set it to "NULL" to deactivate.

- Modification: For modifying dynamic rules, additional attributes are set within the PccRule data structure.

- Rule Versioning: Optionally supported; each rule can have a content version assigned and updated upon modification.

Key Notes:

- The pccRules map includes all relevant data structures and their identifiers to ensure the rule is installed correctly regardless of the QoS flow procedure outcome.

- The PCF can manage QoS and charging parameters dynamically, depending on service requirements ​(29512-i60)​.

Summary

Both TS 29.512 and TS 28.541 define and manage PCC rules, but with slight differences in their focus and details:

- TS 29.512 provides a detailed definition and components of PCC rules, emphasizing their types (dynamic and predefined) and their operational mechanisms.

- TS 28.541 focuses on the structure of the PccRule data, including the map attributes for PCC rules within the policy decision process, and emphasizes operations such as activation, modification, and rule versioning.

Both specifications ensure that the PCC rules are correctly installed, modified, and enforced, with mechanisms to handle dynamic service requirements and charging controls.

There are two types of PCC rules, dynamic PCC rules and predefined PCC rules. The predefined PCC rules are provisioned into the SMF by the operator through OAM. An MnS consumer should be able provisioning PCC rules into an SMF. To support the provisioning of predefined PCC rules, a dataType has been defined that captures the information of a PCC rule. The dataType definition can be found in clause 5.3.81 in TS 28.541 [4]. For the purpose of this contribution the dataType for PredefinedPCCRuleSet is shown in Table 3.1.

Table 3.1 Attributes in a PredefinedPCCRuleSet specifying a PCC rule, see TS 28.541 [4].

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pccRuleId | M | T | T | F | T |
| flowInfoList | CM | T | T | F | T |
| applicationId | CM | T | T | F | T |
| appDescriptor | O | T | T | F | T |
| contentVersion | O | T | T | F | T |
| precedence | CM | T | T | F | T |
| afSigProtocol | O | T | T | F | T |
| isAppRelocatable | O | T | T | F | T |
| isUeAddrPreserved | O | T | T | F | T |
| qosData | M | T | T | F | T |
| altQosParams | O | T | T | F | T |
| trafficControlData | M | T | T | F | T |
| conditionData | O | T | T | F | T |
| tscaiInputUl | O | T | T | F | T |
| tscaiInputDl | O | T | T | F | T |

The definition of a PCC rule as specified in TS 29.512 [5] can be found in Table 5.6.2.6-1.

### 3.2	Comparison

The comparison of the attributes in predefinedPCCRuleSet [4] and a PCC rule [5] is shown in the following table:

Table 3.2: PCC Comparison of information elements for in predefinedPCCRuleSet and PCC rule

|  | Attribute name TS 29.512 [5] Table 5.6.2.6-1 | Attribute name TS 28.541 [4] Table in clause 5.3.81.2 | Attribute can be predefined |
| --- | --- | --- | --- |
| 1 | flowInfos | flowInfoList | Yes |
| 2 | appId | applicationId | Yes |
| 3 | appDescriptor | appDescriptor | Yes |
| 4 | contVer | contentVersion | Yes |
| 5 | pccRuleId | pccRuleId | Yes |
| 6 | precedence | precedence | Yes |
| 7 | afSigProtocol | afSigProtocol | Yes |
| 8 | appReloc | isAppRelocatable | Yes |
| 9 | easRedisInd | - | Yes |
| 10 | addrPreserInd | isUeAddrPreserved | Yes |
| 11 | refQosData | qosData (NOTE) | Yes |
| 12 | refAltQosParams | altQosParams (NOTE) | Yes |
| 13 | refTcData | trafficControlData (NOTE) | Yes |
| 14 | refChgData | - | Yes |
| 15 | refChgN3gData | - | Yes |
| 16 | refUmData | - | No |
| 17 | refUmN3gData | - | No |
| 18 | refCondData | conditionData (NOTE) | Yes |
| 19 | refQosMon | - | No |
| 20 | protoDescDl | - | Yes |
| 21 | protoDescUl | - | Yes |
| 22 | tscaiInputUl | tscaiInputUl | Yes |
| 23 | tscaiInputDl | tscaiInputDl | Yes |
| 24 | tscaiTimeDom | - | Yes |
| 25 | capBatAdaptation | - | Yes |
| 26 | ddNotifCtrl | - | No |
| 27 | ddNotifCtrl2 | - | No |
| 28 | disUeNotif | - | Yes |
| 29 | packFiltAllPrec | - | Yes |
| 30 | nscSuppFeats | - | Yes |
| 31 | callInfo | - | No |
| 32 | traffParaData | - | No |
| NOTE: The meaning of an attribute starting with ref is that is contains only the reference to where the information can be found. For example the refQoSData contains a reference to the qosData i.e. it does not contain the qosData itself. |  |  |  |

All attributes which name begins with ref needs further investigation since these attributes references a new data structure. This data structure may contain different attributes which can be managed separately.

### 3.3	Conclusion

The tables from TS 29.512 [5] and TS 28.541 [4] share some common elements such as rule identification, precedence, and QoS parameters (5QI, ARP). However, TS 29.512 includes more detailed elements related to charging and service data flow handling, while TS 28.541 has additional attributes focusing on application descriptors, traffic control data, and other optional parameters for enhanced flexibility in rule definition and management .

- To reflect the PCC rules from TS 29.512 that are currently not defined in the predefinedPccRuleSet in TS 28.541 it is proposed to add the missing rule attributes to the TS 28.541. The missing attributes are shown in Table 3.4:

Table 3.3: Attributes to be added to predefinedPCCRuleSet

|  | Missing attributes from predefinedPCCRuleSet |
| --- | --- |
| 9 | easRedisInd |
| 20 | protoDescDl |
| 21 | protoDescUl |
| 24 | tscaiTimeDom |
| 25 | capBatAdaptation |
| 28 | disUeNotif |
| 29 | packFiltAllPrec |
| 30 | nscSuppFeats |

-	The handling of attributes starting with ref needs a separate discussion. Which is independent of adding the other missing attributes.

Table 3.4: Refenced attributes

|  | Attribute containing references |
| --- | --- |
| 11 | refQosData |
| 12 | refAltQosParams |
| 13 | refTcData |
| 14 | refChgData |
| 15 | refChgN3gData |
| 16 | refUmData |
| 17 | refUmN3gData |
| 18 | refCondData |

### 3.4	Recommendation

1) The recommended solution is to submit the CR’s to SA5 meeting for the required updates to TS 28.541.

Rel-19 CR 28.541 Align predefinedPccRuleSet attributes with the definitions in TS 29.512 (category F)

2) Continue the discussion on attributes starting with ref

# 4	Detailed proposal

The group is asked to endorse the recommendation in 3.4.