3GPP TSG-SA5 Meeting #159	S5-251058

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.111 | CR | 0024 | rev | 2 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.111 Add a restriction in notifyChangedAlarmGeneral |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When perceivedSeverity is set to 'CLEARED', the notifyClearedAlarm notification should be sent based on the statement listed below, rather than notifyChangedAlarmGeneral: - clause 6.12: when perceivedSeverity changes to 'CLEARED', only the notify cleared alarm notification is sent. - clause 8.3: notifyClearedAlarm is generated when the perceivedSeverity of an existing AlarmRecord changes to 'CLEARED'. - clause 8.6 (deprecated): the description of notifyChangedAlarm specifies an exceptation is when AlarmRecord changes to 'CLEARED'. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an exception to the notifyChangedAlarmGeneral, that is when the perceivedSeverity is changed to 'CLEARED'. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Describes a wrong situation triggering the notifychangedAlarmGeneral notification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11, 8.4.1, 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.11	Reliability of alarm lists

Alarm lists may become unreliable for numerous reasons. Due to the organisation of managed objects (that can be alarmed and have related alarm records in the alarm list) in hierarchical object trees, alarm records relating to a complete subtree are typically becoming unreliable. For example, consider a subnetwork manager that loses the connection to one of the managed elements it manages. In this case the alarm records relating to the complete object subtree starting at the object representing the managed element are not updated any more and hence unreliable.

Alarm lists advertise unreliable parts by indicating the base objects of unreliable subtrees in the (multi-valued) unreliable alarm scope attribute. When the complete alarm list is unreliable, the unreliable alarm scope attribute shall specify the object instance of the MnS agent. When the bad part of the alarm list has been rebuilt and is up to date again the corresponding base object of the previously unreliable subtree is removed from the unreliable alarm scope attribute. An empty attribute indicates that the complete alarm list is reliable.

Next Change

8.4	notifyChangedAlarmGeneral

8.4.1	Definition

This notification is generated by the MnS producer when one or more of the following attributes of an AlarmRecord instance in the AlarmList changes its value: perceivedSeverity (except to the value "CLEARED"), backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector. From the attributes listed above, only those that changed value shall be included in the notification. In case a change happens immediately before percievedSeverity is set to cleared, this notification shall notify other changes before the notifyClearedAlarm notification is sent.

The notification parameters depend on the alarmType and are different for non-security and security alarms. If the alarmType is "Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm" the alarm is considered to be non-security related. If the alarmType is "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation" or "Time Domain Violation" the alarm is considered to be security related.

8.4.2	Input parameters

Table 8.4.2-1: Input parameters for notifyChangedAlarmGeneral

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 8.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 8.2.1 |  |
| notificationType | M | "notifyChangedAlarmGeneral" |  |
| eventTime | M | alarmRecord.alarmChangedTime |  |
| systemDN | M | See clause 8.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| specificProblem | O | alarmRecord.specificProblem |  |
| perceivedSeverity | O | alarmRecord.perceivedSeverity | Value shall not be "CLEARED" |
| backedUpStatus | CO | alarmRecord.backedUpStatus | Used only in non-security notifications. |
| backUpObject | CO | alarmRecord.backUpObject | Used only in non-security notifications. |
| trendIndication | CO | alarmRecord.trendIndication | Used only in non-security notifications. |
| thresholdInfo | CO | alarmRecord.thresholdInfo | Used only in non-security notifications. |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| stateChangeDefinition | CO | alarmRecord.stateChange | Used only in non-security notifications. |
| monitoredAttributes | CO | alarmRecord.monitoredAttributes | Used only in non-security notifications. |
| proposedRepairActions | CO | alarmRecord.proposedRepairActions | Used only in non-security notifications. |
| additionalText | O | alarmRecord.additionalText |  |
| additionalInformation | O | alarmRecord.additionalInformation |  |
| rootCauseIndicator | O | alarmRecord.rootCauseIndicator |  |
| serviceUser | CM | alarmRecord.securityServiceUser | Available if security alarms are supported. Used only in security notifications. This may contain no information if the identification of the service-user (requesting the service) is not known. |
| serviceProvider | CM | alarmRecord.securityServiceProvider | Available if security alarms are supported. Used only in security notifications. This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | CM | alarmRecord.securityAlarmDetector | Available if security alarms are supported. Used only in security notifications. This may contain no information if the detector of the security alarm is the serviceProvider. |
| changedAlarmAttributes | O | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

End of Change