3GPP TSG-SA5 Meeting #159	S5-251059

Sophia Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0075 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.313 Use case and requirements for Inter-CU LTM and conditional LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of requirements and use case for the development of support for inter-CU LTM and conditional LTM, in order to align with TS 38.300, TS 38.331 and TS 38.423. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added requirements for PM conditional LTM. Added use case for conditional LTM. Added requirement for enabling and disabling conditional LTM from one cell to another cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Stage 1 for conditional LTM and for enabling and disabling LTM would hinder the development of an interoperable solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.8, 6.4.1.x (new), 7.1.8.2.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.1.8	LTM management

REQ-DLTM-FUN-1	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable LTM cell switch from one cell to another cell.

REQ-DLTM-FUN-2	The producer of NF performance assurance MnS should have the capability to produce measurements related to LTM.

REQ-DLTM-FUN-3	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable conditional LTM cell switch from one cell to another cell.

REQ-DLTM-FUN-4	The producer of NF performance assurance MnS should have the capability to produce measurements related to conditional LTM.

Next change

#### 6.4.1.x	Conditional LTM (Conditional Lower-layer Triggered Mobility)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure parameters related to LTM in cells in order to improve conditional LTM performance. |  |
| Actors and Roles | D-SON management function to support the conditional LTM function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS. - The producer of performance assurance MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - Conditional LTM is not in operation from the source cell to the target cell. |  |
| Begins when | The D-SON management function intends to enable conditional LTM from the source cell to the target cell. |  |
| Step 1 (M) | The D-SON management function requests the producer of NF provisioning MnS to configure parameters for the management of conditional LTM on the source cell. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable conditional LTM from a source cell to a target cell. |  |
| Step 3 (M) | The LTM function detects handover issues (e.g. too late conditional LTM cell switch, too early conditional LTM cell switch and conditional LTM cell switch to a wrong cell) by analysing reports from UEs and network side information, and acts to mitigate the conditional LTM issues by adjusting conditional LTM related RRC parameters sent from the gNB to the UE. |  |
| Step 4 (M) | The D-SON management function collects conditional LTM related measurements and analyses them to evaluate the conditional LTM performance. |  |
| Step 5 (M) | If the D-SON management function does not find the conditional LTM performance satisfactory, it updates the configuration parameters for conditional LTM. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | Conditional LTM is in operation from the source cell to the target cell. |  |
| Traceability | REQ-DLTM-FUN-3, REQ-DLTM-FUN-4. |  |

Next change

### 7.1.8	MRO for Lower-layer Triggered Mobility (LTM)

#### 7.1.8.1	MnS component type A

MRO for LTM re-uses the component A for MRO, see clause 7.1.2.1.

#### 7.1.8.2	MnS Component Type B definition

##### 7.1.8.2.1	Control information

These parameters are used to control the LTM function.

Table 7.1.8.2.1-1: MRO for LTM control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| LTM function control | This attribute allows the operator to enable/disable the LTM functionality. | Boolean |
| Control function for conditional LTM | This attribute allows the operator to enable/disable the conditional LTM functionality. | Boolean |

End of changes