3GPP TSG-SA5 Meeting #159	S5-251078

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Discussion on the transfer of data over UP for option 3

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	RP-243316 LS on AI/ML UE sided data collection

[2]	RP-242389 LS on AIML data collection

3	Rationale

3.1 Background

According to the “actions” in reply LS RP-243316 [1], we propose to analyze the transfer of data over UP for option 3 to show the impact of SA5.

According to the “Table 7.2.1.3.2-1” in LS RP-242389 [2], the option 3 of the AI/ML-specific Data Transfer Path:

- Option 3: UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server

3.2 Analysis of the potential impact for SA5

In currently SA5 definition, the collection mechanisms related to UE data collection consists of two parts: UE->gNB and gNB->OAM. For UE and gNB data transmission, transmission between the UP tunnel and CP tunnel may be distinguished. However, for gNB and OAM data transmission, SA5 does not distinguish between the UP tunnel and CP tunnel.

From SA5 point of view, the following assumptions about the termination points of the UP tunnel for this solution:

- Opt1: If the UP tunnel is between UE and Server, SA5 does not see any difference between solution 3 and solution 1a)

- Opt2: If the UP tunnel is between UE and 5GC, SA5 does see any difference between solution 3 and solution 2

- Opt3: If the UP tunnel is between UE and gNB, SA5 needs enhancement with UP tunnel transferring between OAM and gNB

Assume that the UP tunnel is between UE and gNB, the potential solution can be as following:

- Potential solution: The UE uses the UP tunnel to transmit data to the gNB, and the gNB uses the SA5 standard operation to transmit data to the OAM (e.g. transmit data to TCE), then the OAM transmits data to the server.

- SA5 impact: The procedure needs enhancement with UP tunnel transferring between gNB and UE. And UP tunnel transferring between gNB and UE is within the scope of the RAN discussion.

4	Detailed proposal

Based on the analysis above, it is proposed to take the analysis above into consideration when drafting the LS reply.