3GPP TSG-SA5 Meeting #159	S5-251081

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei, China Mobile, China Unicom, CATT

# Title:	DP on OAM supporting Ambient IoT feature

# Document for:	Endorsement

Agenda Item:	6.2.2

1	Decision/action requested

The group is asked to discuss and endorsement.

2	References

[1]	3GPP SP-241980: "Way forward proposal on Architecture support of Ambient IoT"

[2]	3GPP SP-241979: "New WID on Architecture support of Ambient power-enabled Internet of Things"

[3]	3GPP RP-243326: "New WID on Rel-19 Ambient IoT"

[4]	3GPP TR 38.848: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)"

[5]	3GPP TR 38.769: "Study on solutions for ambient IoT (Internet of Things) (Release 19) "

[6]	3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things (Release 19) "

[7]	3GPP TS 23.abc: " Architecture support for Ambient power-enabled Internet of Things; Stage 2"

3	Rationale

3.1	Progress in SA and RAN

Progress in SA

SA#106 endorsed SP-241980 [1] which gives guidance to SA5 saying

"

SA5 is expected to bring the related work item (including charging) for approval in TSG#107

"

SA#106 endorsed the SA2 new WID on Architecture support of Ambient power-enabled Internet of Things which limits the Rel-19 scope to Topology 1 with both direct interface option and indirect interface option.

Progress in RAN

RAN#106 approved the RAN new WID on Rel-19 Ambient IoT [2] which limits the Rel-19 scope to Deployment Scenario 1 and Topology 1.

3.2	OAM supporting A-IoT

3.2.1	NR NRM enhancement

Topology 1 represents the way where the Ambient IoT device directly and bidirectionally communicates with a base station as introduced in TR 38.848 [4]. The following is a logical system architecture defined in TR 38.769 [5].

"

6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface is the NG-C interface.

"

Observation#1: The logical system architecture is composed of the RAN supporting A-IoT feature which performs both common reader capability and RAN node capability.

Potential management aspects#1: According to Observation#1, it is proposed to enhance NR NRM to configure the RAN with reader capability and RAN node capability.

Observation#2: The interface between A-IoT RAN and A-IoT CN is NG-C interface.

Potential management aspects#2: According to Observation #2, it is proposed to enhance NG interface NRM to configure the interface between RAN and A-IoT CN (i.e., AIOTF in case of direct interface option).

3.2.2	5GC NRM enhancement

SA2 TR 23.700-13 [6] concludes that a new core network function (AIOTF) is introduced to support Ambient IoT functionality. SA2 TS 23.abc [7] clause 4.2.2 describes architecture for both direct interface and indirect interface.

"

4.2.2.1	General

An A-IoT RAN may support one or more AIoT Readers which provide communication services towards Ambient IoT Devices. 5GS system architecture for Ambient IoT supports the following paths to access an AIoT RAN:

-	Direct Path: AIOTF communicates with a AIOT RAN via a direct interface, as shown in clause 4.2.2.2.

-	Indirect Path via AMF: AIOT RAN and the AIOTF communicate indirectly via an AMF, as shown in clause 4.2.2.3.

4.2.2.2	Direct interface between AIOTF and A-IOT RAN

Figure 4.2.2.2-1 depicts the Ambient IoT architecture showing how the AIOTF access A-IoT RAN by a direct interface.

Figure 4.2.2.2-1: A-IoT RAN – AIOT Direct Interface Architecture

…

"

Observation#3: A new core network function (i.e., AIOTF) will be introduced in R19.

Potential management aspects#3: According to Observation#3 it is proposed to enhance 5GC NRM to configure AIOTF and its interfaces with RAN and other CN NFs (e.g., AMF, NEF).

4	Detailed proposal

It proposes to endorse following statements:

Proposal#1. Revise AdNRM_Ph3 WID to capture NRM enhancement supporting Rel-19 A-IoT feature.

Proposal#2. Endorse the minimal set of potential management aspects#1, #2 and #3 as guidance for NRM enhancement supporting Rel-19 A-IoT feature based on the normative progress of RAN WGs and SA2.