3GPP TSG-SA5 Meeting #160	S5-251362

Goteborg, Sweden, 07 - 11 April 2025

# Source:	ZTE Corporation

# Title:	Input to DraftCR Rel-19 TS 28.105 Add New AI/ML functionalities management scenario for LMF

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

As specified in clause 4.3.8 in TS 23.273, it supports to request ML model provision from NWDAF containing MTLF for LMF-based AI/ML Positioning, i.e., the inference for Positioning can be performed in LMF. However, the ML training function for training a positioning ML model can be located in OAM or MTLF.

AI/ML functionalities management scenarios should be updated to reflect the management support for LMF based AI/ML Positioning.

4	Detailed proposal

Start of the First Change

4a.2	AI/ML functionalities management scenarios (relation with managed AI/ML features)

The ML training function and/or AI/ML inference function can be located in the RAN domain MnS consumer (e.g. cross-domain management system) or the domain-specific management system (i.e. a management function for RAN or CN), or Network Function.

For MDA, the ML training function can be located inside or outside the MDAF. The AI/ML inference function is in the MDAF.

For NWDAF, the ML training function can be located in the MTLF of the NWDAF or the management system, and the AI/ML inference function is in the AnLF.

For RAN, the ML training function and AI/ML inference function can both be located in the gNB, or the ML training function can be located in the management system and AI/ML inference function is located in the gNB.

For LMF-based AI/ML Positioning, the ML training function can be located in the MTLF of the NWDAF or CN-domain management function, and the AI/ML inference function can be located in the LMF.

Therefore, there might exist several location scenarios for ML training function and AI/ML inference function.

Scenario 1:

The ML training function and AI/ML inference function are both located in the 3GPP management system (e.g. RAN domain management function). For instance, for RAN domain-specific MDA, the ML training function and AI/ML inference functions for MDA can be located in the RAN domain-specific MDAF. As depicted in figure 4a.2-1.

Figure 4a.2-1: Management for RAN domain specific MDAF

Similarly, for CN domain-specific MDA the ML training function and AI/ML inference function can be located in CN domain-specific MDAF.

Scenario 2:

For RAN AI/ML capabilities the ML training function is located in the 3GPP RAN domain-specific management function while the AI/ML inference function is located in gNB. See figure 4a.2-2.

Figure 4a.2-2: Management where the ML model training is located in RAN domain management function and AI/ML inference is located in gNB

Scenario 3:

The ML training function and AI/ML inference function are both located in the gNB. See figure 4a.2-3.

Figure 4a.2-3: Management where the ML model training and AI/ML inference are both located in gNB

Scenario 4:

For NWDAF, the ML training function and AI/ML inference function are both located in the NWDAF. See figure 4a.2-4.

Figure 4a.2-4: Management where the ML model training and AI/ML inference are both located in CN

Scenario 5:

For LMF-based AI/ML Positioning, the ML training function can be located in the CN domain management function, and the AI/ML inference function can be located in the LMF. See figure 4a.2-5.

Figure 4a.2-5: Management for LMF-based AI/ML Positioning where the ML model training is located in CN domain management function and AI/ML inference is located in LMF

Scenario 6:

For LMF-based AI/ML Positioning, the ML training function can be located in the MTLF of the NWDAF, and the AI/ML inference function can be located in the LMF. See figure 4a.2-6.

Figure 4a.2-6: Management for LMF-based AI/ML Positioning where the ML model training is located in the MTLF of NWDAF and AI/ML inference is located in the LMF

End of the First Change