3GPP TSG-SA5 Meeting #160	S5-251446

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0321 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add description of excavating of implicit intent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 28.312 R19 introduces the requirement for implicit intents but the relevant description is inadequate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the use case of excavating of implicit intent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of implicit intent is inadequate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.9, 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.3.9	Negotiation on fulfilment of intents

5.3.9.1	Introduction

For a given intent, an MnS Consumer may express a feasible intent, but the producer may have multiple ways to fulfil it. One of these solutions might be better in one aspect and another solution might be better in another aspect. Although this intent is feasible, the producer may not have the knowledge and capability to determine which solution is appropriate.

Example, the MnS Consumer may express intent for energy saving but only specify the target for energy consumption reduction without more constraints, e.g. reduce 20 % energy consumption. There will be numerous solutions to fulfil the energy consumption target with various impacts, e.g. one solution may reduce the coverage and one may reduce the number of users.

The MnS consumer may have concerns on the possible impact of the selected solution by the Producer, which means that negotiation with the MnS Consumer is necessary. In such cases, the MnS consumer and Mns producer may negotiate on the best way to fulfil the intent.

Intent negotiation is a set of procedures which are applicable throughout the life cycle of an intent.  Several negotiations are possible for a feasible intent, many employing interactions that are similar.

NOTE 1:	Some of these negotiations can also be applicable during the feasibility check process.

5.3.9.2	Checking for fulfillable outcomes

The MnS consumer wants to know possible fulfillable outcomes for a given intent. The MnS consumer creates an intent that should be evaluated by the MnS producer to see options which the MnS producer can deliver.

Figure 5.3.9.2-1: MnS consumer requests and receives a list of possible fulfillable outcomes on an intent.

Many dynamic factors impact possible outcomes.  As a result, it is unrealistic to expect all potential outcomes to be reported.   The list of possible outcomes an MnS Producer may provide is left to implementation.

Subsequently, the MnS producer provides a report indicating what is fulfillable for each intent aspect (intentExpectation and expectationTarget) within that intent. Since different fulfillable outcomes may have different impacts, the report should include the relative impacts of each outcome. The relative impact is the MnS producer’s computed effect of a specific fulfilment of an intent, intent expectation or expectation target on the network and on other intents, intent expectations or expectation targets.

NOTE 1:  the mechanism by which MnS Consumer can indicate a preference for a specific possible outcome is out of scope of this TS.

NOTE 2: How the Producer would report “the relative impacts of each outcome” is FFS

5.3.9.3	Checking for best possible outcome on an intent, intent expectation, or expectation target

The MnS consumer wants to know the best possible outcome for a given intent or intent expectation or expectation target. This could be prior to or during fulfilment.

Figure 5.3.9.3-1: Checking for best possible outcome on intent or
intent expectation or expectation target

The MnS consumer creates an intent that should be evaluated by the MnS producer with a request to provide the best possible outcome. Subsequently, the MnS producer provides a report indicating that best possible outcome. The best possible outcome is defined in 2 possible scenarios as follows:

-	The request is to evaluate specific expectation target(s) in an intent with multiple expectation targets (e.g. multiple expectations or one expectation with multiple expectation targets): The best possible outcome is the best value on specific expectation target(s) that maintains the other expectation targets to within the ranges specified in the intent.

-	The request is to evaluate all expectation targets in an intent with multiple expectation targets (e.g. multiple expectations or one expectation with multiple expectation targets): The best possible outcome is the best value on each expectation target that maintains the other expectation targets to within the ranges specified in the intent.

The MnS producer should support a possible outcomes report that lists the possible outcomes for any of the two scenarios, the report including the related impact on other targets in the intent or on other metrics and contexts.

5.3.9.4	MnS producer to provide information about possible fulfilment of the intent

The MnS consumer wants and requests to know for a specific property what the MnS producer recommends what could be applied for particular intent characteristics, i.e. the MnS consumer wants to query MnS producer for recommendations for particular intent characteristics.. The MnS producer has attempted to fulfil the intent and indicated that it cannot be fulfilled, so the MnS consumer asks the MnS producer to recommend what changes could be made to their intent or another of their intents to make the intent fulfillable. The MnS consumer can receive recommendations from MnS producer and use the response to modify their existing intents in an attempt to improve the ability to fulfil the new intents.

Figure 5.3.9.4-1: Enabling the MnS consumer to request and receive a recommendation on
the possible fulfillable intent properties prior to or in case of failure of fulfilment.

Subsequently, the MnS producer provides a recommended-changes report indicating the information about possible changes that would enable fulfilment of the intent. The MnS producer should support a recommended-changes report that lists information about possible fulfilment of intent, e.g., candidate changes to unfulfillable targets within an unfulfillable expectation.

5.3.9.5	MnS consumer advises on preferred alternatives

The MnS consumer wants an intent fulfilled. The intent is feasible, but the MnS producer has multiple alternatives related to fulfilling the intent. The MnS producer wants the MnS consumer to advise on their (the MnS consumer's) preference among these alternatives.

NOTE 1:	An alternative is the combination a set of expectation target values that the MnS producer can achieve together with their (expected) impacts on the network (objects). E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

Figure 5.3.9.5-1: MnS consumer advises the MnS producer
on the preferences among alternatives at the MnS producer

After the MnS consumer creates an intent to be fulfilled, the MnS producer determines that there are multiple alternatives, so the MnS producer and provides a report to the MnS consumer so that the MnS consumer may help chose the best alternative.

The report to the MnS consumer may include:

-	The list of available/fulfillable expectation target values that the MnS producer is able to apply/achieve.

-	The expected relative impacts of the different alternatives - on aspects of the submitted intent or their  intents and intent expectations.

-	A request to the MnS consumer to select one among the alternatives.

Given the alternatives, the MnS consumer takes any combination of these actions:

-	Chooses and indicates the preferred alternative.

-	Defines the relative importance of their expectation Targets (in the form of a utility function) so that the MnS producer may consider these in deciding upon the solution/ solution approach/ closed loops/ action/ outcome to be applied/deployed/achieved.

-	Provides the relative satisfaction level that each of the alternatives achieves. The satisfaction level is the computation of the utility achieved by each alternative according to the MnS consumer’s utility function. It is an integer in the range  [0,100] . The highest possible value indicates that the solution provided by the MnS producer achieves the best possible outcomes that the MnS consumer expected, e.g., that it achieves the highest range of a target whose desired values were defining as falling in a range. Correspondingly, the values lower than 100 indicate how far from the maximum satisfaction the outcome is.

5.3.9.x	MnS consumer demands the excavating of implicit intent

The MnS producer receives the intent sent by the MnS consumer. The intent contains implicitIntentIndex that indicates whether the MnS producer is required to excavate implicit information about the MnS consumer's intent, that is, the MnS consumer's unstated but latent concern, or the change to other expectation targets brought about by the realization of the intent. The implicit information refers to implicit intent, which contains at least one implicit intent expectation.

MnS producers obtain implicitIntentIndex; If implicitIntentIndex is True, the implicit information associated with the intent is determined according to the intent-related data; If implicitIntentIndex is False, the step of implicit intent information discovery is not performed.

Then the MnS producer determines the implicit information associated with the intent when a feasible condition with the intent is detected. The MnS producer sends an intent report with implicit information to MnS consumer, so that the MnS consumer can determine corresponding intent expectations according to the intent report and generate new intents. The MnS producer receives the new intent sends by the MnS consumer, and generates corresponding intent instance according to the new intent.

5.3.9.7	Requirements

REQ-Intent_NEGOT_01: The intent-driven MnS producer should have the capability to provide possible outcomes to the MnS Consumer.

REQ-Intent_NEGOT_02: The intent-driven MnS producer should have the capability enabling MnS Consumer to  decide appropriate outcome for fulfilment based on possible outcomes.

NOTE 1:	The definition for outcome is For Further Study.

REQ_INT_NEGOT-03: The intent driven MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide information on the alternative fulfillable outcomes for an intent.

NOTE 2:	An alternative is the combination of a set of expectation target values that the MnS producer achieves together with their (expected) impacts.

NOTE 3:	The impacts refer to information about the changes and outcomes on the expectation objects of the intent and on other related intents from the same intent MnS consumer. The impact refers to outcomes of the contexts and targets that MnS consumers have not explicitly pointed out. The exact characterization of what is reported is For Further Study. E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

NOTE 4:	Example changes could include: omitting certain intentExpectations and/or expectationTarget(s) or changing the properties of intentExpectations and/or expectationTarget(s).

REQ_INT_NEGOT-04: The intent driven MnS producer should support a capability to provide to an MnS consumer an intent report indicating the alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets and the expected relative impacts of the different alternatives.

REQ_INT_NEGOT-05: The intent driven MnS producer should support a capability to request an MnS consumer to indicate its preference among a set of alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets.

REQ_INT_NEGOT-06: The intent driven MnS producer should support a capability enabling an authorized MnS consumer to provide information on a description of its preferences (e.g. in form of a utility function or satisfaction index) that should be used by the MnS producer to select among the alternatives available at the MnS producer.

REQ_INT_NEGOT-07: The intent driven MnS producer should support a capability to request the MS consumer to provide an evaluation of the MnS producer's alternatives based on the expected relative impacts of the different alternatives.

REQ_INT_NEGOT-08: The intent driven MnS producer should support a capability enabling an MnS consumer to provide an evaluation of the MnS producer's alternatives to then be used to select among the alternatives.

REQ_INT_NEGOT-09: The intent driven MnS producer should support a capability to inform an authorized MnS consumer about an alternative among multiple alternatives that has been selected and (will be/has been) applied for the submitted intent.

REQ_INT_NEGOT-10: The intent driven MnS producer should support a capability to inform an authorized MnS consumer that since no more improvement to intent fulfillment is possible with the applied alternative and request the MnS consumer to provide extra information which helps improve satisfaction.

REQ_INT_NEGOT-11: The intent driven MnS producer should support a capability enabling an authorized consumer to inform the producer that the alternative selected by the MnS producer was not satisfactory and another alternative should be applied.

REQ_INT_NEGOT-12: The intent driven MnS producer should support a capability enabling an authorized MnS consumer to provide information on the degree of satisfaction which the MnS producer uses to differently attempt the fulfillment.

INT_NEGOT_REQ 13: The MnS producer should support a capability to provide an intent report including information on what is achievable for each intentExpectation or expectationTarget within that intent and the relative cost/impact of achieving that outcome.

INT_NEGOT_REQ 14: The MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide the best possible outcome on an intent or intent expectation or expectation target.

INT_NEGOT_REQ 15: The MnS producer should support a capability to provide an intent report including information on the best possible outcome on intent or intent expectation or expectation target.

NOTE 5: This requirement could be combined with REQ-13.

INT_NEGOT_REQ-16: The MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide information on what changes could be made to the intent properties or to properties of other of their intents to make the intent fulfillable.

NOTE 6: Further discussion is needed for this requirement.

INT_NEGOT_REQ-17: The MnS producer may support a capability to provide a report indicating the changes, which if applied to the intent, would make the intent fulfillable

NOTE 7: The definition of outcomes is to be clarified

INT_NEGOT_REQ-18: The MnS producer should support a capability to provide implicit intent, i.e., the additional intent expectation target and contexts, which includes the information that MnS consumers have not explicitly pointed out in its intent.

2nd Change

6.2.1.2	Class definition

6.2.1.2.1	Intent <<InformationObjectClass>>

6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

The Intent IOC contains one or multiple IntentExpectation(s) which includes MnS consumer's requirements, goals and contexts given to a 3GPP system.

The Intent IOC also contains intentAdminState to support intent suspension mechanism. In case MnS consumer wants to suspend an intent, MnS consumer can request MnS producer to configure attribute intentAdminState with the value "DEACTIVATED". A suspended intent means this intent is not considered for fulfillment. In case MnS consumer wants to resume an intent on the MnS producer side when the intent is suspended, MnS consumer can request MnS producer to configure attribute intentAdminState with the value "ACTIVATED".

The attribute "intentReportControl" indicates the intent report control and subscription information. MnS consumer needs to specify the values for the attribute "intentReportControl" when the MnS consumer wants to obtain the intent report with customized requirements by default, instead of triggering a separate subscription action.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is "Intent" and the value of attribute objectInstance is the DN of the instance of Intent IOC.

The Intent IOC includes contextSelectivity respectively used to define how to select among the stated intentContexts.

The Intent IOC includes implicitIntentIndex used to indicate whether the excavate of implicit information is enabled. In case the MnS consumer wants the MnS producer to discover the implicit information like additional intent expectation which is not explicitly pointed out, the implicitIntentIndex will be “True”.

The Intent IOC includes expectationSelectivity used to define how to select among the stated intentExpectations. It enables the MnS consumer to trigger evaluation of different alternative intentExpectations. The MnS consumer can provide alternatives descriptions of service-offers as intent expectations that describes the different candidate characteristics of the desired service from the MnS consumer's point of view that the MnS consumer wants to be validated. By providing the intent containing an expectationSelectivity for feasibility checking, the MnS consumer indicates how the set of candidate services expressed in the intentExpectations are to be validated, i.e., "ALL_OF", "ONE_OF", "ANY_OF" the intentExpectations.

End of changes