3GPP TSG-SA5 Meeting #159	S5-251514

Göteborg, Sweden, 7 - 11 April 2025

# Source:	Ericsson

# Title:	Format of user identity

# Document for:	Discussion

Agenda Item:	7.4.9

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[2]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems ".

[3]	3GPP TS 32.240: "Charging Architecture and Principles"

[4]	3GPP TS 32.255: "5G data connectivity domain charging; Stage 2"

[5]	3GPP TS 32.291: "5G system, charging service; Stage 3"

# 3	Rationale

## 3.1	Background

The current study TR 32.853 [2] has concluded that the 3GPP UAV ID (3GPP UAV ID is used to identify the UAV, as GPSI in the format of External Identifier).

In the TS 23.502 [1] clause 5.52.3 the following is stated about device information:

"When a non-3GPP device is connected to the UE, the UE may include the Non-3GPP Device Connection Information in PDU Session Modification Request to SMF"

The formats are described as:

"For an Ethernet PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier,

-	MAC address used in PDU session,

-	Optionally, VLAN tag ID that is associated with the non-3GPP device used in PDU session.

For an IPv4 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier,

-	IPv4 Address associated with the non-3GPP device used in PDU session,

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

For an IPv6 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

- Non-3GPP Device Identifier,

- IPv6 Address/prefix(sub) associated with the non-3GPP device used in PDU session,

- Optionally, port ranges associated with the non-3GPP device used in PDU session."

In TS 32.291 [] the following identities related to the user and/or subscriber are defined:

-	subscriberIdentifier: Identifier of the subscriber that uses the requested service.

-	tenantIdentifier: Identifier of the tenant.

-	userInformation: including information of user and user equipment: servedGPSI and servedPEI.

## 3.2	Solutions

### 3.2.1	General

Since 3GPP UAV ID is seen as a GPSI and the Device Identifier in the case of it being a MAC it could be seen as a PEI since the format of Pei in TS 29.517 [] can be MAC address, the most obvious solution would be to extend the User Information.

### 3.2.2	UserInformation as list and device type part of device id

In this solution a new list of user information is introduced containing and the current UserInformation is extended with both IP addresses and other non-3GPP device types.

Table 3.2.2-1: Definition of type PDUSessionChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| userInformation | UserInformation | OM | 0..1 | contains information of user equipment |  |
| userInformationList | UserInformation | OM | 0..N | contains information of devices connected to the user equipment | DeviceId, UAS |
| … |  |  |  |  |  |

Table 3.2.2-2: Definition of type UserInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servedGPSI | Gpsi | OC | 0..1 | the Generic Public Subscription Identifier (GPSI) of the served party |  |
| servedPEI | Pei | OC | 0..1 | the identification of Permanent Equipment Identifier, it may contain an IMEI, IMEISV, MAC address, or IEEE Extended Unique Identifier |  |
| servedIP | IPAddr | OC | 0..1 | the IP Address of the user equipment or devices connected to the user equipment | DeviceId |
| portNumber | Uinteger | OC | 0..N | the port number of IP Address used by the user equipment or devices connected to the user equipment | DeviceId |
| n3gppDeviceId | string | OC | 0..N | non-3GPP Device Identifier used for device identifiers not conforming to any of the defined formats in UserInformation. The part before the first “-“ describes the type of device id. Pattern: "^(\w+-\S+)$". | DeviceId, UAS |
| unauthenticatedFlag | boolean | OC | 0..1 | indicates the served SUPI is not authenticated |  |
| roamerInOut | RoamerInOut | OC | 0..1 | In-bound or Out-bound roamer |  |

### 3.2.3	UserInformation as list and separate device type

In this solution a new list of user information is introduced containing and the current UserInformation is extended with non-3GPP device types that can contain IP addresses etc.

Table 3.2.3-1: Definition of type PDUSessionChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| userInformation | UserInformation | OM | 0..1 | contains information of user equipment |  |
| userInformationList | UserInformation | OM | 0..N | contains information of devices connected to the user equipment | DeviceId, UAS |
| … |  |  |  |  |  |

Table 3.2.3-2: Definition of type UserInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servedGPSI | Gpsi | OC | 0..1 | the Generic Public Subscription Identifier (GPSI) of the served party |  |
| servedPEI | Pei | OC | 0..1 | the identification of Permanent Equipment Identifier, it may contain an IMEI, IMEISV, MAC address, or IEEE Extended Unique Identifier |  |
| n3gppDeviceId | N3gppDeviceId | OC | 0..N | non-3GPP Device Identifier used for device identifiers not conforming to any of the defined formats in UserInformation | DeviceId, UAS |
| unauthenticatedFlag | boolean | OC | 0..1 | indicates the served SUPI is not authenticated |  |
| roamerInOut | RoamerInOut | OC | 0..1 | In-bound or Out-bound roamer |  |

Table 3.2.3-3: Definition of type N3gppDeviceId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deviceId | string | OC | 0..1 | non-3GPP Device Identifier |  |
| deviceIdExt | string | OC | 0..N | any extensions to the non-3GPP Device Identifier, e.g. port number |  |
| deviceType | string | OC | 0..1 | type of device identifier, e.g. UAV, IP address |  |

### 3.2.4	UserInformation with separate device type

In this solution a new list of user information is introduced containing and the current UserInformation is extended with non-3GPP device types that can contain IP addresses etc.

Table 3.2.4-1: Definition of type PDUSessionChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| userInformation | UserInformation | OM | 0..1 | contains information of user equipment |  |
| … |  |  |  |  |  |

Table 3.2.4-2: Definition of type UserInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servedGPSI | Gpsi | OC | 0..1 | the Generic Public Subscription Identifier (GPSI) of the served party. |  |
| servedPEI | Pei | OC | 0..1 | the identification of Permanent Equipment Identifier, it may contain an IMEI, IMEISV, MAC address, or IEEE Extended Unique Identifier. |  |
| n3gppDeviceId | N3gppDeviceId | OC | 0..N | non-3GPP Device Identifier used for device identifiers not conforming to any of the defined formats in UserInformation | DeviceId, UAS |
| unauthenticatedFlag | boolean | OC | 0..1 | indicates the served SUPI is not authenticated |  |
| roamerInOut | RoamerInOut | OC | 0..1 | In-bound or Out-bound roamer |  |

Table 3.2.4-3: Definition of type N3gppDeviceId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deviceId | string | OC | 0..1 | non-3GPP Device Identifier |  |
| deviceIdExt | String | OC | 0..N | any extensions to the non-3GPP Device Identifier, e.g. port number |  |
| deviceType | string | OC | 0..1 | type of device identifier, e.g. UAV, IP address |  |

## 3.10	Evaluation

The main difference between solution in clause 3.2.2 and 3.2.3 is if known identifiers should be added or not, the advantage with adding known is that it is easier to pars the information while it increases the information to parse.

The main difference between solution in clause 3.2.4 and any of the other is that this only allows one each of the types GPSI, and PEI meaning for GPSI there can only be either MSISDN or External Identifier, and for PEI it would be either IMEI or MAC.

## 3.11	Observations

Observation 1: Having known identifiers explicitly defined increases the possible use of these.

Observation 2: Allowing both MSISDN and External Identifier as well as both MAC and IMSI could be advantageous.

# 4	Detailed proposal

## 4.1	Combination of clause 3.2.2 and 3.2.3

A combination of clause 3.2.2 and 3.2.3 would be the optimal solution.

Table 4.1-1: Definition of type PDUSessionChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| userInformation | UserInformation | OM | 0..1 | contains information of user equipment |  |
| userInformationList | UserInformation | OM | 0..N | contains information of devices connected to the user equipment | DeviceId, UAS |
| … |  |  |  |  |  |

Table 4.1-2: Definition of type UserInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servedGPSI | Gpsi | OC | 0..1 | the Generic Public Subscription Identifier (GPSI) of the served party |  |
| servedPEI | Pei | OC | 0..1 | the identification of Permanent Equipment Identifier, it may contain an IMEI, IMEISV, MAC address, or IEEE Extended Unique Identifier |  |
| servedIP | IPAddr | OC | 0..1 | the IP Address of the user equipment or devices connected to the user equipment | DeviceId |
| portNumber | Uinteger | OC | 0..N | the port number of IP Address used by the user equipment or devices connected to the user equipment | DeviceId |
| n3gppDeviceId | N3gppDeviceId | OC | 0..N | non-3GPP Device Identifier used for device identifiers not conforming to any of the defined formats in UserInformation | DeviceId, UAS |
| unauthenticatedFlag | boolean | OC | 0..1 | indicates the served SUPI is not authenticated |  |
| roamerInOut | RoamerInOut | OC | 0..1 | In-bound or Out-bound roamer |  |

Table 4.1-3: Definition of type N3gppDeviceId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deviceId | string | OC | 0..1 | non-3GPP Device Identifier |  |
| deviceIdExt | string | OC | 0..N | any extensions to the non-3GPP Device Identifier |  |
| deviceType | string | OC | 0..1 | type of device identifier |  |