3GPP TSG-SA5 Meeting #160	S5-251551

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | InputToDraftCR | rev | <Rev#> | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 InputToDraftCR TS 28.105 add use case of Management of Local explanation in AI/ML inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HUAWEI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Use Case and Requirement of Management of Local explanation in AI/ML inference has been discussed and agreed (in TR 28.858). This CR propose to add the use case and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new use case Management of Local explanation in AI/ML inference . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case of managing Local explanation in AI/ML inference will not be supported in SA5 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.X (new), 6.5.X.1(New), 6.5.X.2, 6.5.X.2.1(new), 6.5.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.5	AI/ML inference

6.5.X	AI/ML inference explainability

6.5.X.1	Description

Explainable ML refers to a process that enables the consumers (e.g. human operator) to understand and trust the outputs provided by ML models. In essence, explainable ML is about making the decision-making of ML comprehensible to its consumers. On a broad level, explainable ML can be categorized into two types:

-	Local explanation: The aim is to explain individual outputs provided by an ML model, i.e., it focuses on explaining why a specific output was generated by the ML model for a particular input data sample.

-	Global explanation: The aim is to explain the whole ML model behaviour, i.e., it focuses on explaining how the ML model works in general across several or all possible input samples.

6.5.X.2	Use cases

6.5.X.2.1	Management of Local explanation in AI/ML inference

Local explanation is an additional information that the MnS consumer can use to analyse the ML model's potential impact in terms of network performance gain and make informed decisions regarding deactivation of AI/ML inference or fallback to previous version of the ML model. Local explanation in AI/ML inference can be used for supporting various AI/ML functions (e.g. MDA functions, etc.) when the ML training function and AI/ML inference is located in OAM.

Once the ML model is trained and deployed for inference. The generated local explanations by the AI/ML inference MnS producer may need to be reported to the AI/ML inference MnS consumer. These local explanations provide additional information that the MnS consumer can use to analyse the ML model's potential impact on network performance. This analysis helps in making informed decisions regarding the deactivation of AI/ML inference or fallback to a previous version of the ML model. By considering network performance PM/KPIs, model performance, and local explanations, the MnS consumer can effectively identify the ML model contributing to network performance degradation when several ML models are active in the network. E.g. for coverage anlaysis scenarios, the local explanations explain why the recommendation was made and provide counterfactual explanations, such as what would happen if the recommendation was not used. In addition, the local explanation may also indicate the affected network performance metrics.

6.5.X.3	Requirements for management of AIML inference explainability

Table 6.5.2.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML-INF-EXP-01 | The AI/ML Inference MnS producer should have a capability to inform an authorized MnS consumer of the capabilites of Local explanation in AI/ML inference | Management of Local explanation in AI/ML inference (clause 6.5.X.2.1) |
| REQ-ML-INF-EXP-02 | The AI/ML Inference MnS producer should have a capability to provide the generated local explanations for inference for an authorized consumer. | Management of Local explanation in AI/ML inference (clause 6.5.X.2.1) |

End of changes