3GPP TSG-SA5 Meeting #160	S5-251567

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei

Title:	Pseudo-CR on TS 28.561 NDT NRMs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.1.0

Work Item:	NDT

Comments

This contribution proposes to define operation and information model for NDT.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]					3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[4]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[5]	3GPP TS 28.111: "Management and orchestration; Fault Management (FM)".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[a]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[f]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

* * * Next Change * * * *

6	Management service for NDT (stage 2)

Editor's note: this clause will specifiy the stage2 definition for NDT capabilities and services which may include the procedure, management operations and management information (e.g., NDT modelling).

X.1	Management operation for NDT (MnS component type A)

The operations (e.g. createMOI operations) and notifications (e.g. notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [a] can be used for NDT management, including NDT lifecycle management, NDT service request management and NDT service report management. The NDT, NDTServiceRequest, NDTServiceReport can be treated as Managed Object instances.

Following is the IS to support NDT lifecycle management:

Table X.1-1: IS to support NDT lifecycle management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT function | getMOIAttributes operation |

Following is the IS to support NDT service request management:

Table X.1-2: IS to support NDT service request management

| NDT lifecycle management | IS operation |
| --- | --- |
| Create an NDT service request | createMOI operation |
| Delete an NDT service request | deleteMOI operation |
| Modify an NDT service request | modifyMOIAttributes operation |
| Query an NDT service request | getMOIAttributes operation |

Following is the IS to support NDT report management:

Table X.1-2: IS to support NDT report management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT report | getMOIAttributes operation |
| Subscribe an NDT report | createMOI operation |
| Notify an NDT report | notifyMOIAttributeValueChanges notification |
| Unsubscribe an NDT report | deleteMOI operation |
| Query an NDT report subscription | getMOIAttributes operation |

X.2	Information model for NDT (MnS component typeB)

X.2.1	Information model definition

X.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [b], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [b], DataType, TimeWindow | TimeWindow |

X.2.1.2	Class diagram

X.2.1.2.1	Relationship

Figure X.2.1.2.1-1: Relationship UML diagram for NDT management

X.2.1.2.2	Inheritance

Figure X.2.1.2.1-1: Inheritance UML diagram for NDT management

X.2.1.3	Class definition

X.2.1.3.1	NDT <<InformationObjectClass>>

X.2.1.3.1.1	Definition

This IOC represents the properties of an NDT.

The attribute "supportedNdtCapabilities" indicates the capabilities that are support by the NDT instance. The ENUM value is Verification, DataGeneration, SupportingAutomationFunction.

The attribute "ndtStatus" indicates lifecycle status of an NDT. The ENUM value is activation, operation, deactivation.

X.2.1.3.1.2	Attributes

The NDT IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNdtCapabilities | M | T | F | F | T |
| ndtStatus | M | T | T | F | T |

X.2.1.3.2	NDTService <<InformationObjectClass>>

X.2.1.3.2.1	Definition

This IOC represents the properties of an NDT service demand created by an MnS consumer.

The attribute "ndtServiceType" indicates the service type requested by MnS consumer. The ENUM value is Verification, DataGeneration, SupportingAutomationFunction.

The attribute "ndtServiceStatus" indicates status of an NDT service, i.e., a simulation/emulation job. The ENUM value is activation, operation, deactivation.

The attribute "networkScope" indicates the scope of mobile network that needs to be simulated/emulated.

The attribute "configurations" indicates configuration that defined by MnS consumer and will be configured to NDT.

The attribute "networkScenario" indicates network scenario that defined by MnS consumer and will be configured to NDT.

The attribute "ndtServiceTime" is a time window which indicates if the simulation/emulation is for the past, present, or future.

The attribute "performanceRequirements" represents the performance requirements for NDT modelling, e.g., the synchronization frequency, maximum run time for each simulation task, precision, etc.

X.2.1.3.2.2	Attributes

The NDTService IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ndtServiceID | M | T | T | F | T |
| ndtServiceType | M | T | T | F | T |
| ndtServiceStatus | M | T | T | F | T |
| networkScope | M | T | T | F | T |
| configurations | M | T | T | F | T |
| networkScenario | M | T | T | F | T |
| ndtServiceTime | O | T | T | F | T |
| performanceRequirements | O | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtReference | M | T | T | F | T |

NOTE: The detail of attributes (e.g., datatype) needs further definition. For instance, the networkScope can be structured by networkType (e.g., RAN SubNetwork, CN SubNetwork), networkInstance (e.g., DN of RAN SubNetwork, DN of CN SubNetwork) networkScopeFilter (e.g., Filters on geographic area, gNB list, frequency range, CN NF list)

X.2.1.3.3	NDTReport <<InformationObjectClass>>

X.2.1.3.3.1	Definition

This IOC represents the properties of an NDT service report corresponding to an NDT service request.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT.

X.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ndtReportID | M | T | T | F | T |
| performanceData | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtServiceReference | M | T | T | F | T |
| ndtReference | M | T | T | F | T |

* * * Next Change * * * *

A.1.1	Relationship UML diagram for NDT management

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class ManagedEntity <<ProxyClass>>

class NDT <<InformationObjectClass>>

class NDTService <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

note right of ManagedEntity

Represents SubNetwork

end note

ManagedEntity "1" *-- "*" NDT: <<names>>

NDT "1" *-- "*" NDTService: <<names>>

NDT "1" *-- "*" NDTReport: <<names>>

NDTService "1" <--> "*" NDTReport

@enduml

A.1.2	Inheritance UML diagram for NDT management

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<InformationObjectClass>> \n  Top " as TOP{}

class "<<InformationObjectClass>>\n NDT " as NDT{}

class "<<InformationObjectClass>>\n NDTService " as NDTService{}

class "<<InformationObjectClass>>\n NDTReport " as NDTReport{}

TOP  <|--  NDT

TOP  <|--  NDTService

TOP  <|--  NDTReport

@enduml

* * * End of Changes * * * *