3GPP TSG-SA5 Meeting #160	S5-251571

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei

Title:	Pseudo-CR on TR 28.869 Add an example in Annex B

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.3.0

Work Item:	FS_Cloud_OAM

Comments

TS 28.533 clause 4.5 introduces two deployment scenarios of Management Function. One is that Management Function can be deployed as a separate entity to provide MnS, the other Management Function is embedded in Network Function to provide MnS. See the description in TS 28.533 below:

“

Management Function can be deployed as a separate entity or embedded in Network Function to provide MnS(s). Following figure 4.5.2 shows an example (on the left) which MnF deployed as a separate entity to provide MnS(s) and another example (on the right) which MnF is embedded in Network Function to provide MnS(s):

”

Annex B in TR 28.869 only captures one MnF deployment scenario, i.e., MnF is embedded in Network Function to provide MnS. Therefore, it is proposed to add a new example for management data streaming solution based on message bus which adapts to the deployment scenario where MnF is deployed as a separate entity.

Proposed Changes

* * * First Change * * * *

Annex B (informative):
Example implementations using management data streaming solution based on message bus

Figures B-1 below illustrates an example implementation using the potential solution for data streaming based on message bus. An NF directly send the produced data to the message broker and a MnF in the management system consumes data from the message broker. The MnF can request e.g. creation of the PM or trace jobs in the NF, subscribe to PM metrics as well as convey message broker end-point information using the management data job control interface.

In service view, as illustrated in figure B-2, the MnS producer implemented in the NF produces management data and sends to the MnS consumer implemented in the MnF via message bus communication. The MnS consumer uses the services exposed by the MnS producer to configure the PM and trace jobs, subscribe to data, and convey the end-point information to the MnS producer.

Figure B-1: Implementation example 1, NF produce data to MnF directly via message broker

Figure B-2: Service involved in implementation example 1

As illustrated in Figure B-x, the MnS producer implemented in the MnF A produces management data and sends to the MnS consumer implemented in the MnF B. The MnS consumer uses the services provided by the MnS producer to configure the PM and trace jobs, subscribe to data, and convey the end-point information to the MnS producer. The MnS producer reports the management data streaming based on message bus.

Figure B-x: Example of streaming based on message bus when MnF is deployed as a separate entity

* * * End of Changes * * * *