3GPP TSG-SA5 Meeting #160	S5-251584

Goteborg, Sweden, 07 - 11 April 2025

# Source:                         NTT DOCOMO, China Mobile

# Title:	pCR TR 28.869 3GPP management architecture evolution based on PaaS

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval

# 2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add a use case and solution for 3GPP management architecture evolution based on PaaS.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

### 5.1.X	Use case #X: 3GPP management architecture evolution based on PaaS

#### 5.1.X.1	Description

A Telco Platform as a Service (PaaS) is a set of cloud services (related to compute, network, storage, and management) that facilitate the deployment and operation of Network Functions, as software applications, on an underlying generic and decoupled cloud infrastructure.

3GPP standardization work is currently missing modeling and interactions that can enable the use of PaaS services; PaaS capabilities can be used to support advanced OAM capabilities and improve management efficiency in a standardized way. For example, a Telco PaaS can offer support for OAM operations on top of different cloud IaaS platforms. This can also enhance portability of cloud native VNFs from one cloud environment to another one.

This use case considers the interaction of the 3GPP management system with a Telco PaaS management entity through a PaaS reference point.

#### 5.1.X.2	Potential requirements

REQ _PaaS-1 The PaaS reference point should support the capability allowing the 3GPP management system to interact with external (non-3GPP) management entities to support the management of cloud native VNFs.

#### 5.X.1.3	Potential solutions

##### 5.X.1.3.1	High level architecture evolution

As shown in figure 5.X.1.3.1-1, this solution introduces a management entity providing Telco PaaS services that interacts with 3GPP management system to support the management of cloud-native VNFs via a generic PaaS reference point.

Figure 5.X.1.3-1: High level architecture evolution solution for 3GPP management architecture evolution based on PaaS

NOTE: the specification of the PaaS reference point is outside of the 3GPP management system. Existing interfaces specified by other SDOs and/or other de-facto solutions can be used to support communication and interactions over the PaaS reference point between the 3GPP management system and PaaS services.

End of Changes