3GPP TSG-SA5 Meeting #160	S5-251636

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Improve clauses on Managing planned configurations

# Document for:	Approval, Information, Discussion

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.11	Managing planned configurations

Planned configuration descriptors are created, updated and deleted by MnS consumers on MnS producers.

Multiple planned configuration descriptors can be maintained on a MnS producer. Each planned configuration is identified by an identifier. The identifier is assigned by the MnS producer.

## 6.12	Managing planned configuration groups

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. The planned configurations referred to in a planned configuration group shall exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

## 6.13	Managing fallback configurations

Fallback configuration descriptors are not directly managed by MnS consumers. They are created automatically by MnS producers when the "enableFallback" attribute in an activation job is set to true. Only the "name", "version", and "description" can be set by MnS consumers once the object is created.

When many fallbacks are enabled, the situation can become soon complex and the risk for misconfigurations increases. MnS producers may therefore apply implementation specific strategies to reduce the risk for misconfigurations. For example, they may allow only for one fallback configuration to exist; if another fallback is enabled, the existing fallback configuration is overwritten. Or they may allow for multiple fallback configurations only if they affect different parts of the network.

Fallback configurations can be deleted by MnS consumers, or by the MnS producer based on an implementation specific policies.

No fallback on the activation of a fallback configuration is possible.

## 6.14	Managing trigger conditions

Conditions are created, updated and deleted by MnS consumers on MnS producers. For each condition shall be specified which activation jobs are triggered by this condition.

## 6.15	Managing validations

Validation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request validation the MnS consumer needs to create a validation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be validated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be validated, a descriptor may be specified directly in the validation job creation request.

When only one planned configuration is requested to be validated no control parameter needs to be specified in the request. For validating planned configuration groups, the "isFailOnConflict" parameter needs to be specified.

The MnS producer adds the following information elements to the job: "state", "startedAt", "stoppedAt", "result", "memberConflicts", and "errors".

The information element "errors" is a structured data type with the following properties:

-	The mandatory "type" property that provides high level error information.

-	The optional "title" that provides a short, human-readable summary of the problem type. It shall not change from occurrence to occurrence of the problem.

-	The optional "reason" property" that provides more details on the error conditions than the "type" property.

-	The mandatory "badOp" property that specifies the operation, that cannot be satisfied.

Further details can be found in TS 32.158 [2].

Multiple validation jobs can run in parallel, even if the planned configurations target the same portions of the current configuration.

## 6.16	Managing activations

Activation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request activation the MnS consumer needs to create an activation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be activated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be activated, a descriptor may be specified also directly in the activation request.

For falling back to an old configuration, the identifier of a fallback configuration descriptor needs to be specified.

Activation shall start immediately upon creation of the activation job, unless the "isImmediateActivation" parameter is set to false. In this case the activation is triggered by a trigger condition that points to the activation job. Note that the activation job does not know the related conditions.

For immediate activation an activation job can activate a planned configuration only once. If the planned configuration is to be activated a second time a new job needs to be created. For conditional activation, in contrast, an activation job can activate the same planned configuration multiple times. For example, if a planned configuration is for weekdays and another planned configuration is for weekends, two activation jobs can be used for activating both planned configurations. One job is triggered by a trigger condition triggering the activation when the weekend starts, the other job is triggered by a trigger condition when the weekdays start.

Multiple activation jobs running can run in parallel if the planned configurations do not update the same portions of the current configuration.

Furthermore, the following control parameters need to be specified in the job creation request: "isEnableFallback", and "serviceImpact".

The MnS producer adds the following information elements to the job: "state", "startedAt", "stoppedAt", "result", and "errors".

Only planned configurations, that are successfully validated, should be activated. Therefor a MnS consumer should always create a validation job first. Only after successful validation of a planned configuration, a job for activating the planned configuration should be created. In case the planned configuration was not validated the MnS producer shall create a validation job for the planned configuration. The activation is started only if the planned configuration is valid. If the planned configuration is invalid the activation is not started and the result attribute of the activation indicates that the activation failed.

A MnS consumer can request a MnS producer to cancel a running activation process. The behavior depends on the application mode:

- For "ATOMIC" the configuration is rolled back.

- For "BEST_EFFORT" the activation is stopped.

- For "STOP_ON_ERROR" the activation is stopped

End of modifications