3GPP TSG-SA5 Meeting #160	S5-251749

Goteborg, Sweden, 07 - 11 April 2025

Source:	Samsung

Title:	Rel-19 pCR TS 28.567 Procedure and metrics for CCL performance monitoring

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add solution for CCL performance monitoring as was agreed in the CCL study in TR 28.867

Proposed Changes

* * * First Change * * * *

# 7	Procedures

## 7.x CCL Performance Monitoring

When the PA MnS consumer notices that a slice or network performance is degrading, it may require to know information about available CCLs that have the goals related to this performance degradation.This may imply that the performance of the related CCL is not as expected. This requires performance management to be done on the available CCL including further actions such as evaluating and updating closed control loops. The metrics for assessing performance  of CCLs, for example, total number of occurrences of a goal breach, time taken by CCL to meet a breached goal, total number of conflicts occurred by a CCL are defined in clause 8. A procedure for performance management of CCLs involving these performance metrics is described below

Figure 7.x-1: Performance monitoring procedure for a closed control loop

Step 1. PA/CCL MnS consumer notices that a certain performance metric of a SLS or a network starts degrading.

Step 2. PA/CCL MnS consumer sends getMOIAttributeRequest message to PA/CCL MnS producer for getting information about all CCLs attributes.

Step 3. PA/CCL MnS producer provides this information of all CCLs to the consumer in getMOIAttributeResponse message.

Step 4. PA/CCL MnS consumer identifies the CCL (n) which is responsible for maintaining the performance of slice or network.

Step 5. PA/CCL MnS consumer sends createMOI(PerfMetricJob) request to PA/CCL MnS producer for obtaining status of following performance metrics for that particular CCL(n) as defined in clause 8 - TotalAssuranceGoalBreach, TimeCorrectiveGoalMeet, TotalCclConflicts_Filter.

Step 6. PA/CCL MnS producer provides requested performance metric values via createMOI() Response message to PA/CCL MnS consumer.

Step 7. PA/CCL MnS consumer has two choices – either to update the existing CCL n (of step 4) to achieve the desired sls/assurance goal or to create a new CCL for the same. If PA/CCL MnS consumer chooses to modify an existing CCL, it sends a modifyMOIAttributes request message for that CCL or it can also update by sending changeMOIs request message to PA/CCL MnS producer.

Step8. Accordingly, PA/CCL MnS producer sends modifyMOIAttributes Response or changeMOIs response message to PA/CCL MnS consumer for the updated attributes of CCL n.

Step9. If PA/CCL MnS consumer chooses to create a new CCL for the desired SLS/assurance goal, it does so by sending createMOI Request message to PA/CCL MnS producer.

Step10. PA/CCL MnS producer provides createMOI() Response message for the newly created CCL MOI to PA/CCL MnS consumer.

# 8	CCL Performance Metrics

The performance metrics to evaluate performance of a CCL for its optimal execution are defined in order to enable operators to track the effectiveness of closed loop automation, identify areas for improvement, and make informed adjustments to CCL functionalities.

## 8.1	Total number of occurrences of an assurance goal breach:

a)	This measurement provides the total number of occurrences when an assurance goal, as defined in CCL is breached during an observation time period.

b)	CC.

c)	This is measured by counting each incidence when an assurance goal is breached and incrementing the corresponding counter by one for each such occurrence within an observation time period.

d)	An integer value.

e)	The measurement name has the form TotalAssuranceGoalBreach.

f)	CCL Provider.

g)	Valid for packet switched traffic.

h)	5GS.

## 8.2	Time taken by CCL to meet a breached goal:

a)	This measurement provides the time taken by a CCL to meet a breached goal after activating it again.

b)	DER.

c)	This is measured by considering the time stamp when an assurance goal is breached and subtracting it from the time stamp when that goal is met after activating the CCL with required changes.

d)	Each measurement is an integer representing the mean delay in milliseconds.

e)	The measurement name has the form TimeBreachedGoalRecovery.

f)	CCL Provider.

g)	Valid for packet switched traffic.

h)	5GS.

## 8.3	Total number of conflicts occurred by a CCL:

a)	This measurement provides the total number of conflicts that occur between a CCL under consideration and any other CCL during an observation time period.

b)	CC.

c)	This is measured by counting each incidence when conflict occurs between a CCL under consideration and the other CCL and incrementing the corresponding counter by one for each such occurrence within an observation time period.

d)	An integer value.

e)	The measurement name has the form TotalCclConflicts_Filter, where filter is either Implicit or Explicit. Implicit represents the action conflict i.e. conflict between two existing CCL and target represents the explicit conflict i.e. conflict between an existing CCL and a requested CCL.

f)	CCL Provider.

g)	Valid for packet switched traffic.

h)	5GS.

* * * End of Changes * * * *