# 3GPP TSG SA WG5 Meeting #160                 	           S5-251809

# Goteborg, SWEDEN 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0521 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add charging information for store and forward satellite operation of SMS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-CH |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusion of the TR 28.846, in order to support the store and forward satellite operation of SMS service, the charging information for S&F in the CDR need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add charging information for S&F in the CDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The store and forward of SMS service is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5, 6.1.6, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251642. |  |  |  |  |  |  |  |  |

First change

6.1.5	SMS-MO data in SGSN/MME (S-SMO-CDR)

If enabled, an S-SMO-CDR SGSN/MME Mobile originated SMS record shall be produced for each short message sent by a mobile subscriber via the SGSN/via the MME to the SMSC. 
The fields in the record are specified in table 6.1.5.1.

Table 6.1.5.1: SGSN/MME Mobile originated SMS record (S-SMO-CDR)

| Field | Category |  | Description |
| --- | --- | --- | --- |
|  | SGSN | MME |  |
| Record Type | M | M | SGSN/MME Mobile Originated SMS. |
| Retransmission | - | OC | This parameter, when present, indicates that information from retransmitted Charging Data Requesthas been used in this CDR. |
| Served IMSI | M | M | The IMSI of the subscriber. |
| Served IMEI | OC | OC | The IMEI or IMEISV of the ME, if available. |
| Served MSISDN | OC | OC | The primary MSISDN of the subscriber, if available. For MSISDN-less subscription, this field holds the Dummy MSISDN as defined in TS 23.003 [103]. |
| MS Network Capability | OM | - | The mobile station network capability. |
| Service Centre | OM | OM | The address (E.164) of the SMS-service centre. |
| Serving Node Type | M | M | Type of node handling the SMS (SGSN or MME). |
| Recording Entity | OM | OM | The E.164 number of the SGSN/MME. |
| Serving Node Address | - | OM | The address of the MME used. |
| Serving Node IPv6 Address Address | - | OC | The IPv6 address, in case of IPv4v6 dual stack, of the MME. |
| MME Name | - | OM | The operation Identity of the MME. |
| MME Realm | - | OM | The operation Realm Identity of the MME. |
| Location Area Code | OM | - | The Location Area Code from which the message originated. |
| Routing Area Code | OM | - | The Routing Area Code from which the message originated. |
| Cell Identifier | OM | - | The Cell Identity for GSM or Service Area Code (SAC) for UMTS from which the message originated. |
| User Location Information | - | OC | This field contains the User Location (User location Information as defined in TS 29.274 [210]) from which the message originated. |
| User Location Information Time | - | OC | This field contains the time at which the user location information was acquired. |
| Message Reference | M | M | A reference provided by the MS uniquely identifying this message. |
| Event Time Stamp | M | M | The time at which the message was received by the SGSN/MME from the subscriber. |
| SMS Result | C | C | The result of the attempted delivery if unsuccessful. |
| Node ID | OM | OM | Name of the recording entity. |
| Local Record Sequence Number | OM | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| Charging Characteristics | M | M | The Charging Characteristics flag set used by the SGSN/MME. |
| Serving Node PLMN Identifier | Oc | Oc | This field holds the PLMN Identifier (MCC and MNC) serving the UE during this record. Only one value is included in case multiple PLMN Ids are supported by the SGSN/MME. |
| CN Operator Selection Entity | OC | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |
| RAT Type | OC | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. |
| Destination Number | OM | OM | The destination short message subscriber number. |
| CAMEL Information | OC | - | Set of CAMEL information related to SMS session. For more information see Description of Record Fields. This field is present if CAMEL service is activated. |
| Charging Characteristics Selection Mode | OM | OM | Holds information about how Charging Characteristics were selected. |
| Record Extensions | OC | OC | A set of network operator/ manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Satellite Store and Forward information | OC | OC | This field holds all the satellite information that support to finish the Store and Forward Satellite operation. |
| Store and Forward indicator | OC | OC | This field holds the use of Store and Forward Satellite operation. |
| S&F Monitoring list | OC | OC | This field holds the satellite IDs that used for the Store and Forward Satellite operation. |
| Store duration | OC | OC | This field holds the storage duration of data on the satellite. |
| Store data volume | OC | OC | This field stores the amount of data stored on the satellite. |

Second change

6.1.6	SMS-MT data in SGSN/MME (S-SMT-CDR)

If enabled, an SGSN/MME Mobile terminated SMS record shall be produced for each short message received by a mobile subscriber via the SGSN/ from the SMSC via the MME. The fields in the record are specified in table 6.1.6.1.

Table 6.1.6.1: SGSN/MME Mobile terminated SMS record (S-SMT-CDR)

| Field | Category |  | Description |
| --- | --- | --- | --- |
|  | SGSN | MME |  |
| Record Type | M | M | SGSN/MME Mobile Terminated SMS. |
| Retransmission | - | OC | This parameter, when present, indicates that information from retransmitted Charging Data Request has been used in this CDR. |
| Served IMSI | M | M | The IMSI of the subscriber. |
| Served IMEI | OC | OC | The IMEI or IMEISV of the ME, if available. |
| Served MSISDN | OC | OC | The primary MSISDN of the subscriber. |
| MS Network Capability | OM | - | The mobile station network capability. |
| Service Centre | OM | OM | The address (E.164) of the SMS-service centre. |
| Serving Node Type | M | M | Type of node handling the SMS (SGSN or MME) |
| Recording Entity | OM | OM | The E.164 number of the SGSN/MME. |
| Serving Node Address | - | OM | The address of the MME used |
| Serving Node IPv6 Address Address | - | OC | The IPv6 address, in case of IPv4v6 dual stack, of the MME. |
| MME Name | - | OM | The Diameter Identity of the MME |
| MME Realm | - | OM | The Diameter Realm Identity of the MME |
| Location Area Code | OM | - | The Location Area Code to which the message was delivered. |
| Routing Area Code | OM | - | The Routing Area Code to which the message was delivered. |
| Cell Identifier | OM | - | The Cell Identity for GSM or Service Area Code (SAC) for UMTS to which the message was delivered. |
| User Location Information | - | OC | This field contains the User Location (User location Information as defined in TS 29.274 [210]) to which the message was delivered, if available. |
| User Location Information Time | - | OC | This field contains the time at which the user location information was acquired. |
| Event Time Stamp | M | M | Delivery time stamp, time at which message was sent to the MS by the SGSN/MME. |
| SMS Result | C | C | The result of the attempted delivery if unsuccessful. |
| Node ID | OM | OM | Name of the recording entity. |
| Local Record Sequence Number | OM | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| Charging Characteristics | M | M | The Charging Characteristics flag set used by the SGSN/MME. |
| Serving Node PLMN Identifier | Oc | Oc | This field holds the PLMN Identifier (MCC and MNC) serving the UE during this record. Only one value is included in case multiple PLMN Ids are supported by the SGSN/MME |
| CN Operator Selection Entity | OC | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |
| RAT Type | OC | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. |
| Charging Characteristics Selection Mode | OM | OM | Holds information about how Charging Characteristics were selected. |
| CAMEL Information | OC | - | Set of CAMEL information related to SMS session. For more information see Description of Record Fields. This field is present if CAMEL service is activated. |
| Originating Address | OC | OC | The address of the originating SME. |
| Record Extensions | OC | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Satellite Store and Forward information | OC | OC | This field holds all the satellite information that support to finish the Store and Forward Satellite operation. |
| Store and Forward indicator | OC | OC | This field holds the use of Store and Forward Satellite operation. |
| S&F Monitoring list | OC | OC | This field holds the satellite IDs that used for the Store and Forward Satellite operation. |
| Store duration | OC | OC | This field holds the storage duration of data on the satellite. |
| Store data volume | OC | OC | This field stores the amount of data stored on the satellite. |

Third change

6.3.2	Detailed message format for offline charging

The following clause specifies per Operation Type the charging data that are sent by MME, S-GW, ePDG, TWAG, P-GW and TDF.

The Operation Types are listed in the following order: S (Start)/I (Interim)/S (Stop)/E (Event). Therefore, when all Operation Types are possible it is marked as SISE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. SIS or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. SI-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.1 illustrates the basic structure of the supported fields in the Charging Data Request message for PS offline charging.

Table 6.3.2.1: Supported fields in Charging Data Request message

| Information Element | Node Type | S-GW | ePDG | P-GW | TWAG | TDF |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Realm |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Destination Domain |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Number |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| User Name |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Destination Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Interval |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination State |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination Timestamp |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Proxy Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Route Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Token |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Service Information with PS and IMS Information |  |  |  |  |  |  |
| Supported Features |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Subscription Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| IMSI Unauthenticated Flag |  | SIS- | SIS- | SIS- | SIS- | - |
| Node Functionality |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Id |  | SIS- | SIS- | SIS- | SIS- | - |
| Charging per IP-CAN Session Indicator |  | - | - | S--- | - | - |
| Node Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDN Connection Charging Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDP/PDN Type |  | SIS- | SIS- | SIS- | SIS- | - |
| SGi PtP Tunnelling Method |  | - | - | SIS- | - | - |
| SCS/AS Address |  | - | - | SIS- | - | - |
| PDP/PDN Address |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDP/PDN Address prefix length |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Dynamic Address Flag |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Dynamic Address Flag Extension |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Serving Node Address |  | SIS- | - | SIS- | - | SIS- |
| Serving Node Type |  | SIS- | - | SIS- |  | SIS- |
| SGW Change |  | S--- | S--- | - | S--- | - |
| P-GW Address |  | SIS- | - | SIS- | SIS- | SIS- |
| TDF Address |  | - | - | - | - | SIS- |
| SGW Address |  | SIS- | - | - | - | - |
| ePDG Address |  | - | SIS- | - | - | - |
| TWAG Address |  | - | - | SIS- | - | - |
| CG Address |  | - | - | - | - | - |
| IMSI MCC MNC |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PGW MCC MNC |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| TDF MCC MNC |  | - | - | - | - | SIS- |
| NSAPI |  | - | - | SIS- | - | - |
| Called Station Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Session Stop Indicator |  | - | - | --S- | - | --S- |
| Selection Mode |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Characteristics |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Characteristics Selection Mode |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Serving Node MCC MNC |  | SIS- | - | SIS- | - | SIS- |
| MS Time Zone |  | SIS- | - | SIS- | - | SIS- |
| Charging Rule Base Name |  | - | - | SIS- | - | - |
| ADC Rule Base Name |  | - | - | - | - | SIS- |
| User Location Info |  | SIS- | - | SIS- | - | SIS |
| User Location Info Time |  | SIS- | - | SIS- | - | - |
| PSCell Information |  | SIS- | - | - | - | - |
| User CSG Information |  | SIS- | - | SIS- | - | SIS- |
| 3GPP2 User Location Info |  | SIS- | - | SIS- | - | SIS- |
| TWAN User Location Information |  | - | - | SIS- | SIS- | - |
| UWAN User Location Information |  | - | SIS- | SIS- | - | - |
| Presence Reporting Area Information |  | SIS- | - | SIS- | - | - |
| RAT Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Furnish Charging Information |  | - | - | SIS- | - | - |
| Offline Charging |  | - | - | - | - | - |
| QoS Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Traffic data volumes |  | -IS- | -IS- | - | -IS- | - |
| Service data container |  | - | - | -IS- | - | -IS- |
| User Equipment Info |  | - | - | - | - | S--- |
| Terminal-Information |  | S--- | S--- | S--- | S--- | S--- |
| Start time |  | S--- | S--- | S--- | S--- | S--- |
| Stop time |  | --S- | --S- | --S- | --S- | --S- |
| Change Condition |  | -IS- | -IS- | -IS- | -IS- | -IS- |
| Diagnostics |  | --S- | --S- | --S- | --S- | --S- |
| Enhanced Diagnostics |  | --S- | --S- | --S- | --S- | - |
| Low Priority Indicator |  | S--- | - | S--- | - | - |
| CN Operator Selection Entity |  | SIS- | - | SIS- | - | - |
| NBIFOM Support |  | - | - | S--- | - | - |
| NBIFOM Mode |  | - | - | S--- | - | - |
| CP CIoT EPS Optimisation Indicator |  | SIS- | - | - | - | - |
| UNI PDU CP Only Flag |  | SIS- | - | SIS- | - | - |
| Serving PLMN Rate Control |  | SI-- | - | SI-- | - | - |
| APN Rate Control |  | - | - | SI-- | - | - |
| MO exception data counter |  | SIS- | - | SIS- | - | - |
| 3GPP PS Data Off Status |  | - | - | SIS- | - | - |
| RAN Secondary RAT Usage Reports |  | -IS- | - | -IS- | - | - |

Table 6.3.2.1a illustrates the basic structure of the supported fields in the Charging Data Request message for "SMS over MME Charging".

Table 6.3.2.1a: Supported fields in Charging Data Request message for "SMS over MME Charging"

| Information Element | Node Type | MME |
| --- | --- | --- |
|  | Supported Operation Types | S/I/S/E |
| Session Identifier |  | ---E |
| Originator Host |  | ---E |
| Originator Realm |  | ---E |
| Destination Domain |  | ---E |
| Operation Type |  | ---E |
| Operation Number |  | ---E |
| Operation Identifier |  | ---E |
| User Name |  | ---E |
| Destination Host |  | ---E |
| Operation Interval |  | ---E |
| Origination State |  | ---E |
| Origination Timestamp |  | ---E |
| Proxy Information |  | ---E |
| Route Information |  | ---E |
| Operation Token |  | ---E |
| Service Information with PS, IMS, MMS and SMS Information |  |  |
| Subscription Id |  | ---E |
| Node Functionality |  | ---E |
| Node Id |  | ---E |
| MME Number for MT SMS |  | ---E |
| MME Name |  | ---E |
| MME Realm |  | ---E |
| Serving Node Type |  | ---E |
| SGSN Address |  | ---E |
| Charging Characteristics |  | ---E |
| Charging Characteristics Selection Mode |  | ---E |
| 3GPP User Location Info |  | ---E |
| User Location Info Time |  | ---E |
| RAT Type |  | ---E |
| Terminal Information |  | ---E |
| Recipient Address |  | ---E |
| SMSC Address |  | ---E |
| Originator Address |  | ---E |
| Submission Time |  | ---E |
| Message Id |  | ---E |
| CN Operator Selection Entity |  | ---E |
| Satellite Store and Forward information |  | ---E |

Table 6.3.2.2 illustrates the basic structure of the supported fields in the Charging Data Response message for PS offline charging, and "SMS over MME Charging".

Table 6.3.2.2: Supported fields in Charging Data Response message

| Information Element | Node Type | S-GW | ePDG | TWAG | P-GW | TDF |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Result |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Domain |  |  |  |  |  |  |
| Operation Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Number |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Interval |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Error Reporting Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination State |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination Timestamp |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Proxy Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Route Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Service Information with PS Information |  |  |  |  |  |  |
| Supported Features |  | SIS- | SIS- | SIS- | SIS- | SIS- |

End of change