3GPP TSG-SA5 Meeting #160	S5-251817

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0063 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 Addition of charging principles for CAPIF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of charging principles for CAPIF using CCF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addtion of CCF in charging priciples |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CCF will not be able to charge for exposure services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.0, and 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251513. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] – [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS): Applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] – [56]	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] – [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] – [199]	Void

[200]	3GPP TS 23.501:"System Architecture for the 5G System".

[201]	3GPP TS 23.502:"Procedures for the 5G System".

[202]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[203]– [229]	Void

[230]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[231]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

[232] - [235]	Void

[236]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[237] – [242]	Void

[243]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[244] – [299]	Void

[300] – [399]	Void.

* * * Next Change * * * *

### 5.1.0	General

The following are high level charging requirements for northbound API for Exposure Function which are specified in TS 23.682 [243], TS 23.501 [200], and TS23 23.222 [202]:

For northbound API invocation/notification, the SCEF, NEF, or CCF shall collect the following charging information:

-	invocations/notifications count of the northbound APIs.

-	identification of the SCS/AS or AF and the associated northbound API invocation/notification.

-	timestamp of the northbound API invocation/notification.

-	northbound API related information, e.g. location.

* * * Next Change * * * *

### 5.1.1	Northbound API procedures

All procedures that operate across the T8 reference point, as specified in 3GPP TS 23.682 [243] and TS 29.122 [230], are covered, which are the following:

-	Monitoring

-	Resource management of Background Data Transfer

-	Changing the chargeable party at session set up or during the session

-	Non-IP Data Delivery

-	Device Triggering

-	Group Message Delivery

-	Reporting of Network Status

-	Communication Pattern Parameters Provisioning

-	PFD Management

-	Enhanced Coverage Restriction Control

-	Network Parameter Configuration

-	Setting up an AS session with required QoS

-	MSISDN-less Mobile Originated SMS

-	Request for time synchronization and time sensitive communication service

-	Request for Ranging and Sidelink Positioning service

The following clauses 5.2 and 5.3 describe the trigger conditions and simplified message flows for Event Based Charging (IEC/ECUR), with interfaces specified in 3GPP TS 32.299 [50].

The Northbound APIs supported by the NEF via the set of exposed services defined in 3GPP TS 23.502 [201] are covered for converged charging, with the trigger conditions and message flows defined in clause 5.4.

The Northbound APIs supported by the CAPIF via the set of exposed service APIs defined in 3GPP TS 23.222 [202] are covered, which are the following:

-	Service activation: Onboarding the API invoker to the CAPIF, as specified in TS 23.222 [202], clause 8.1;

-	Service in-activation: Offboarding the API invoker from the CAPIF, as specified in TS 23.222 [202], clause 8.2.

-	Service API publish, as specified in the TS 23.222 [202] clause 8.3

-	Service API unpublish, as specified in the TS 23.222 [202] clause 8.4

-	Service API retrieve, as specified in the TS 23.222 [202] clause 8.5

-	Service API update, as specified in the TS 23.222 [202] clause 8.6

-	Discover service APIs, as specified in TS 23.222 [202] Clause 8.7.

-	Subscription, un-subscription and notifications for the CAPIF events (e.g. API notification for the Monitoring service API invocation) as specified in the TS 23.222 [202] clause 8.8.

-	Revoking subscription of the CAPIF events, as specified in the TS 23.222 [202] clause 8.9.

* * * End of Changes * * * *