r3GPP TSG-SA5 Meeting #160	S5-251824

Goteborg, SWEDEN 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0440 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.260 Charging principle for IMS DC application download charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2-CH |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The event-based charging for the IMS DC application download is a new charging scenario to support the monetization of IMS DC service on per application download event basis. The solution based on DCSF is recommended into normative work, per TR 28.851 solution #2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduce the trigger condition for DCSF 2. Introduce the message flow for IMS DC charging from DCSF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to charge the IMS DC service based on the application download event. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3, 5.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of S5-251471. |  |  |  |  |  |  |  |  |

First Change

5.4.3	Applicable triggers for IMS charging

The IMS nodes for which SIP method a charging Data Request is sent shall be operator configurable. The tables below describe all possible Charging Data Requests that might be sent from IMS nodes.

The generation of a Charging Data Request message by the IMS nodes in response to a particular "triggering SIP method" shall be operator configurable (enable or disable).

Table 5.4.3.1: Default Trigger conditions in IMS Node

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SIP INVITE | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Initial] IEC: Charging Data Request [Event] |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable |  |
| Change of charging conditions |  |  |  |  | SCUR: Charging Data Request [Update] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change) | - | Immediate | No | No |  |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or UPDATE (e.g. change in media components) | - | Immediate | No | No |  |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body | - | Immediate | No | No |  |
| SIP 4xx, 5xx or 6xx response, indicating an unsuccessful SIP RE-INVITE or UPDATE | - | Immediate | No | No |  |
| Any other SIP message during a SIP session, that allows the SIP session to continue | - | Immediate | Yes | No |  |
| CHF limit |  |  |  |  |  |
| Expiry of time limit | - | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | - | Immediate | No | Yes |  |
| Quota management |  |  |  |  |  |
| Time threshold reached | - | Immediate | No | Yes |  |
| Time quota exhausted | - | Immediate | No | Yes |  |
| Unit quota exhausted | - | Immediate | No | Yes |  |
| Expiry of quota validity time | - | Immediate | No | Yes |  |
| Expiry of quota holding time | - | Immediate | No | Yes |  |
| Re-authorization request by CHF | - | Immediate | No | Yes |  |
| Other |  |  |  |  |  |
| Management intervention | - | Immediate | No | No |  |
| SIP BYE message | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP 2xx acknowledging a SIP BYE | - | Immediate | Not Applicable | Not Applicable |  |
| Aborting a SIP session set-up procedure, using an internal trigger or a SIP CANCEL | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 3xx final or redirection response | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging non-session related SIP messages | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 4xx, 5xx or 6xx final response, indicating an unsuccessful procedure | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] PEC: Charging Data Request [Event] |
| Sent HTTP Response with application download URL from DCSF to UE | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

For converged charging, the following details of chargeable events and corresponding actions in the IMS nodes are defined in Table 5.4.3.2:

Table 5.4.3.2: Chargeable events and their related actions in IMS nodes

| Chargeable event | Conditions | IMS node action |
| --- | --- | --- |
| SIP INVITE |  | SCUR: Charging Data Request [Initial] with a possible request quota for later use |
| SIP NOTIFY |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP MESSAGE |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP REGISTER |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP SUBSCRIBE |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP REFER |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP PUBLISH |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change) |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or UPDATE (e.g. change in media components) |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 4xx, 5xx or 6xx response, indicating an unsuccessful SIP RE-INVITE or UPDATE |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| Any other SIP message during a SIP session, that allows the SIP session to continue |  | Start new counts with time stamps for all active service data flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | SCUR: Charging Data Request [Update] with a possible request quota. |
| Expiry of time limit | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of limit of number of charging condition changes | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time threshold reached | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time quota exhausted | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Unit quota exhausted | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of quota validity time | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of quota holding time | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Re-authorization request by CHF | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Management intervention |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| SIP BYE message, both normal and abnormal session termination cases |  | SCUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 2xx acknowledging a SIP BYE | If last user location information of originating/ terminating party is required | SCUR:Charging Data Request [Termination] Close the counts with time stamps |
| SIP 2xx acknowledging non-session related SIP messages |  | ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| Aborting a SIP session set-up procedure, using an internal trigger or a SIP CANCEL |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| Deregistration |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 3xx final or redirection response |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 4xx, 5xx or 6xx final response, indicating an unsuccessful procedure |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps IEC: Charging Data Request [Event] PEC: Charging Data Request [Event] |
| Sent HTTP Response with application download URL from DCSF to UE |  | PEC: Charging Data Request [Event] from DCSF to CHF for reporting the IMS DC application download event |

2nd Change

5.4.x	Message flows

5.4.x.1	General

The flows described in the present document specify the charging communications between IMS Nodes and CHF for different converged charging scenarios.

5.4.x.2	IMS data channel charging from DCSF

The converged charging to support event-based charging for IMS DC application download may be performed by DCSF. The DCSF shall be able to perform converged charging by interacting with CHF, for charging data related to IMS DC application download event, based on PEC scenario specified in TS 32.290 [4].

Figure 5.4.x.2-1 depicts the charging procedure for IMS DC application download based on DCSF.

Figure 5.4.x.2-1: PEC Charging Procedure for IMS DC application download

1. Originating UE initiate the bootstrap DC set up procedure, as specified in step 1-20 of Figure AC.7.1-1 TS 23.228 [201]. Boostrap DC has been established between originating/terminating MF and originating/terminating UE, as specified in step 21-24 of Figure AC.7.1-1 TS 23.228 [201].

2. In an established bootstrap DC, a UE may send application request message to MF using HTTP Get Request to download dedicated DC application(s).

The MF, acting as HTTP proxy, prepare the subscriber specific HTTP URL corresponding to the stream ID of the bootstrap DC and forward the request to the DCSF.

The DCSF, acting as HTTP web server, provide the subscriber specific HTTP URL with the requested DC application(s) in HTTP response to the UE through MF.

NOTE 1:	The detailed procedures to download DC application to the UE are out of scope as specified in TS 23.228 [201].

2ch-a. After DCSF sent HTTP response to the UE, the DCSF sends Charging Data Request [Event] to CHF for this DC application download event.

2ch-b. The CHF creates the CDR for this DC application download event.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the DCSF.

End of Change