3GPP TSG-SA5 Meeting #160	S5-251828

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0061 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 Add charging principle for IMS network capabilities exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2-CH |  |  |  |  |  | Date: |  |  | 2024-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 28.851 topic 3 Support IMS network capabilities exposure, it concludes that Northbound API converged charging for IMS network capabilities exposure need to be supported (i.e. solution #3.1). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add IMS network capabilities exposure to the list of exposed service API. Add IMS Node as an example of API Target Network Function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging for IMS network capabilities exposure cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 6.3.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] – [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS): Applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] – [56]	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] – [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] – [199]	Void

[200]	3GPP TS 23.501:"System Architecture for the 5G System".

[201]	3GPP TS 23.502:"Procedures for the 5G System".

[202] – [229]	Void

[230]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[231]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[232] – [2235]	Void

[236]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[237] – [242]	Void

[243]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[244] – [299]	Void

[300] – [399]	Void.

2nd change

5.1.1	Northbound API procedures

All procedures that operate across the T8 reference point, as specified in 3GPP TS 23.682 [243] and TS 29.122 [230], are covered, which are the following:

-	Monitoring

-	Resource management of Background Data Transfer

-	Changing the chargeable party at session set up or during the session

-	Non-IP Data Delivery

-	Device Triggering

-	Group Message Delivery

-	Reporting of Network Status

-	Communication Pattern Parameters Provisioning

-	PFD Management

-	Enhanced Coverage Restriction Control

-	Network Parameter Configuration

-	Setting up an AS session with required QoS

-	MSISDN-less Mobile Originated SMS

-	Request for Ranging and Sidelink Positioning service

The following clauses 5.2 and 5.3 describe the trigger conditions and simplified message flows for Event Based Charging (IEC/ECUR), with interfaces specified in 3GPP TS 32.299 [50].

The Northbound APIs supported by the NEF via the set of exposed services defined in 3GPP TS 23.502 [201] are covered for converged charging, with the trigger conditions and message flows defined in clause 5.4. All procedures are specified in TS 29.522 [231], which includes the following examples:

-	Time synchronization exposure service

-	IMS event exposure service

2nd change

6.3.1.4.1	Definition of the NEF API Charging Information

Network Exposure Function API specific charging information is provided within the NEF API Charging Information. The detailed structure of the NEF API Charging Information can be found in table 6.3.1.4.1.

Table 6.3.1.4.1: Structure of the NEF API Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| External Individual Identifier | OC | This parameter holds the external Identifier of the individual UE, e.g., the GPSI. |
| External Individual Id List | OC | This parameter holds a list of external identifiers, used instead of External Individual Identifier if there is more than one. |
| Internal Individual Identifier | OC | This parameter holds the internal Identifier of the individual UE e.g., the SUPI. |
| Internal Individual Id List | OC | This parameter holds a list of internal identifiers, used instead of Internal Individual Identifier if there is more than one. |
| External Group Identifier | OC | This parameter holds the external identifier for a group of individual UE(s). |
| Internal Group Identifier | OC | The internal Identifier identifying a group of individual UE(s). |
| API Direction | OC | This field holds the direction to indicate if it is an API invocation from an AF or notification to an AF. |
| API Target Network Function | OC | This field holds the identifier of the network function that either is the destination of the API invocation or triggers the notification: e.g. PCF, UDM, AMF, SMF, UPF, IMS Node. |
| API Result Code | OC | This parameter holds the result of API Invocation. |
| API Name | M | This field holds the name of the API invoked. |
| API Operation | OC | This field holds the operation of the API invoked, together with detailed description. |
| API Reference | OC | This field holds the reference to the definition of the format of the API invocation, this can be a URI or refence to the standard where it's specified |
| API Content | OC | This field holds the actual content of the API invocation, in the format described by the API Reference |

End of changes