3GPP TSG-SA5 Meeting #160	S5-252087

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.560 | CR | 0004 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing ENs in TS 28.560 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, Ericsson, Nokia, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Monstra-OAM |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The contribution aims to address the following editors notes: - Editor's Note: To check whether we need an identifier for an object instance beyond DN. - Editor's note: To further define how specific interface instances is to be identified. The EN in bullet 1 above is mainly an editorial change, since according to TS 28.560, the Network Function is already identified by the Distinguished Name making the Editor’s Note redundant. The EN in bullet 2 above address the need to identify the signalling over a specific interface instance, since the current specification includes a list of interface names. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor’s Note #1 Remove the Editor’s Note #2. Add new attribute “networkInterfaceInstanceList” which is DN of the network interface instances. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing and incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.1.1, 6.2.1.2, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Cleaned cover sheet |  |  |  |  |  |  |  |  |

First Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Generic management services".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 33.501: " Security architecture and procedures for 5G System ".

[5]	3GPP TS 28.622: "Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	IETF RFC8086: GRE-in-UDP Encapsulation.

[9]	IETF draft-ietf-opsawg-pcapng-04: PCAP Next Generation (pcapng) Capture File Format.

[10]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[11]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[x]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”

[y]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”

Next Modified Section

## 6.2	Class definitions

### 6.2.1	StmCtrl <<IOC>>

#### 6.2.1.1	Definition

This IOC represents STM jobs. It can be name-contained by SubNetwork, ManagedElement, or ManagedFunction.

reportingNFList specifies the network function whose signalling traffic is to be monitored. If this parameter is not present or it is empty, then all Network Functions within the SubNetwork or ManagedElement shall be monitored. This parameter shall be omitted if the STM control object is specified under a ManagedFunction.

networkInterfaceTypeList specifies the target network interface type to be monitored. networkInterfaceInstanceList specifies the target network interface instance to be monitored. If networkInterfaceInstanceList is not present or it is empty, then the network interface type specified in networkInterfaceTypeList are applied. If both parameters are not present or are empty, then all applicable interface types from the target NF shall be monitored.

Editor's Note: To further define the type of signalling traffic/messages that need to be monitored.

stmTargetUri is the URI of the STM consumer that shall receive the monitored signalling message copies.

administrativeState is used by the STM consumer to lock or unlock the StmCtrl instance in order to stop or start the signalling traffic monitoring.

operationalState is used by STM consumer to report its working state.

#### 6.2.1.2	Attributes

The StmCtrl IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [5] subclause 4.3.29) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportingNFList | CM | T | T | F | T |
| networkInterfaceTypeList | M | T | T | F | T |
| networkInterfaceInstanceList | O | T | T | F | T |
| stmTargetUri | M | T | T | T | T |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | F | T |

#### 6.2.1.3	Attribute constraints

None.

#### 6.2.1.4	Notifications

The common notifications defined in 3GPP TS 28.622 [5] subclause 4.5 are valid for this IOC, without exceptions or additions.

Next Modified Section

### 6.3.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| reportingNFList | List of Network Function Distinguished Name. allowedValues: N/A | Type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkInterfaceTypeList | List of network interface type to be monitored. The applicable network interface type name(s) are specified based on subclause 4.2.3 of 3GPP TS 23.501 [7] and clause 4.2.1 of TS 23.273 [Y].. The value "ALL" is specified for the case if all the applicable interface type of the network function shall be monitored. allowedValues: ALL, N2, N4, N5, N7, N8, N10, N11, N12, N13, N14, N15, N16, N17, N18, N20, N21, N22, N23, N26, N28, N29, N30, N33, N34, N35, N36, N37, N40, N41, N42, N51, N52, N58, N59, N60, N61, N62, N63, N80, N81, N82, N83, N84, N85, N86, N87, N88, N89, N96, NL1, NL2, NL5, NL6, NL8, NL9.. | Type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: ALL isNullable: False |
| NetworkInterfaceInstanceList | List of interface type instances to be monitored. See clause 5.2.1 of 28.541 [X] allowedValues: DN of the following MOIs: EP_N2, EP_N4, EP_N5, EP_N7, EP_N8, EP_N10, EP_N11, EP_N12, EP_N13, EP_N14, EP_N15, EP_N16, EP_N17, EP_N18, EP_N20, EP_N21, EP_N22, EP_N23, EP_N26, EP_N28, EP_N29, EP_N30, EP_N33, EP_N34, EP_N35, EP_N36, EP_N37, EP_N40, EP_N41, EP_N42, EP_N51, EP_N52, EP_N58, EP_N59, EP_N60, EP_N61, EP_N62, EP_N63, EP_N80, EP_N81, EP_N82, EP_N83, EP_N84, EP_N85, EP_N86, EP_N87, EP_N88, EP_N89, EP_N96, EP_NL1, EP_NL2, EP_NL5, EP_NL6, EP_NL8, EP_NL9. | Type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| stmTargetUri | It specifies the Uniform Resource Identifier (URI) of the streaming target where the signalling traffic shall be sent. The detailed URI structure is defined in clause 4.4 of 3GPP TS 32.158 [6]. | type: Uri multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the STM consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the STM producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |

End of Modified Sections