| 3GPP TS 28.541 V18.12.0 (2025-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	37

Introduction	38

1	Scope	39

2	References	39

3	Definitions of terms, symbols and abbreviations	43

3.1	Terms	43

3.2	Symbols	43

3.3	Abbreviations	43

4	Information model definitions for NR NRM	44

4.1	Imported and associated information	44

4.1.1	Imported information entities and local labels	44

4.1.2	Associated information entities and local labels	44

4.2	Class diagram	44

4.2.1	Class diagram for gNB and en-gNB	44

4.2.1.1	Relationships	44

4.2.1.2	Inheritance	53

4.3	Class definitions	58

4.3.1	GNBDUFunction	58

4.3.1.1	Definition	58

4.3.1.2	Attributes	58

4.3.1.3	Attribute constraints	58

4.3.1.4	Notifications	59

4.3.2	GNBCUCPFunction	59

4.3.2.1	Definition	59

4.3.2.2	Attributes	59

4.3.2.3	Attribute constraints	60

4.3.2.4	Notifications	60

4.3.3	GNBCUUPFunction	61

4.3.3.1	Definition	61

4.3.3.2	Attributes	61

4.3.3.3	Attribute constraints	61

4.3.3.4	Notifications	62

4.3.4	NRCellCU	62

4.3.4.1	Definition	62

4.3.4.2	Attributes	62

4.3.4.3	Void	62

4.3.4.4	Notifications	62

4.3.5	NRCellDU	62

4.3.5.1	Definition	62

4.3.5.2	Attributes	63

4.3.5.3	Attribute constraints	64

4.3.5.4	Notifications	64

4.3.6	NRSectorCarrier	64

4.3.6.1	Definition	64

4.3.6.2	Attributes	64

4.3.6.3	Attribute constraints	65

4.3.6.4	Notifications	65

4.3.7	BWP	65

4.3.7.1	Definition	65

4.3.7.2	Attributes	65

4.3.7.3	Attribute constraints	66

4.3.7.4	Notifications	66

4.3.8	EP_E1	66

4.3.8.1	Definition	66

4.3.8.2	Attributes	66

4.3.8.3	Attribute constraints	66

4.3.8.4	Notifications	66

4.3.9	EP_XnU	66

4.3.9.1	Definition	66

4.3.9.2	Attributes	66

4.3.9.3	Attribute constraints	67

4.3.9.4	Notifications	67

4.3.10	EP_NgC	67

4.3.10.1	Definition	67

4.3.10.2	Attributes	67

4.3.10.3	Attribute constraints	67

4.3.10.4	Notifications	67

4.3.11	EP_NgU	67

4.3.11.1	Definition	67

4.3.11.2	Attributes	68

4.3.11.3	Attribute constraints	68

4.3.11.4	Notifications	68

4.3.12	EP_F1C	68

4.3.12.1	Definition	68

4.3.12.2	Attributes	68

4.3.12.3	Attribute constraints	68

4.3.12.4	Notifications	68

4.3.13	EP_F1U	69

4.3.13.1	Definition	69

4.3.13.2	Attributes	69

4.3.13.3	Attribute constraints	69

4.3.13.4	Notifications	69

4.3.14	EP_S1U	69

4.3.14.1	Definition	69

4.3.14.2	Attributes	69

4.3.14.3	Attribute constraints	69

4.3.14.4	Notifications	69

4.3.15	EP_X2C	70

4.3.15.1	Definition	70

4.3.15.2	Attributes	70

4.3.15.3	Attribute constraints	70

4.3.15.4	Notifications	70

4.3.16	EP_X2U	70

4.3.16.1	Definition	70

4.3.16.2	Attributes	70

4.3.16.3	Attribute constraints	70

4.3.16.4	Notifications	70

4.3.17	EP_XnC	71

4.3.17.1	Definition	71

4.3.17.2	Attributes	71

4.3.17.3	Attribute constraints	71

4.3.17.4	Notifications	71

4.3.18	ExternalGNBCUCPFunction	71

4.3.18.1	Definition	71

4.3.18.2	Attributes	71

4.3.18.3	Attribute constraints	71

4.3.18.4	Notifications	71

4.3.19	ExternalGNBCUUPFunction	72

4.3.19.1	Definition	72

4.3.19.2	Attributes	72

4.3.19.3	Attribute constraints	72

4.3.19.4	Notifications	72

4.3.20	ExternalGNBDUFunction	72

4.3.20.1	Definition	72

4.3.20.2	Attributes	72

4.3.20.3	Attribute constraints	72

4.3.20.4	Notifications	72

4.3.21	ExternalUPFFunction	73

4.3.21.1	Definition	73

4.3.21.2	Attributes	73

4.3.21.3	Attribute constraints	73

4.3.21.4	Notifications	73

4.3.22	ExternalAMFFunction	73

4.3.22.1	Definition	73

4.3.22.2	Attributes	73

4.3.22.3	Attribute constraints	73

4.3.22.4	Notifications	73

4.3.23	Void	74

4.3.24	ENBFunction <<ProxyClass>>	74

4.3.24.1	Definition	74

4.3.24.2	Attributes	74

4.3.24.3	Attribute constraints	74

4.3.24.4	Notifications	74

4.3.25	GNBCUCPFunction <<ProxyClass>>	74

4.3.25.1	Definition	74

4.3.25.2	Attributes	74

4.3.25.3	Attribute constraints	74

4.3.25.4	Notifications	74

4.3.26	GNBCUUPFunction <<ProxyClass>>	74

4.3.26.1	Definition	74

4.3.26.2	Attributes	74

4.3.26.3	Attribute constraints	74

4.3.26.4	Notifications	74

4.3.27	GNBDUFunction <<ProxyClass>>	75

4.3.27.1	Definition	75

4.3.27.2	Attributes	75

4.3.27.3	Attribute constraints	75

4.3.27.4	Notifications	75

4.3.28	ServingGWFFunction <<ProxyClass>>	75

4.3.28.1	Definition	75

4.3.28.2	Attributes	75

4.3.28.3	Attribute constraints	75

4.3.28.4	Notifications	75

4.3.29	UPFFunction <<ProxyClass>>	75

4.3.29.1	Definition	75

4.3.29.2	Attributes	75

4.3.29.3	Attribute constraints	75

4.3.29.4	Notifications	75

4.3.30	AMFFunction <<ProxyClass>>	76

4.3.30.1	Definition	76

4.3.30.2	Attributes	76

4.3.30.3	Attribute constraints	76

4.3.30.4	Notifications	76

4.3.31	Void	76

4.3.32	NRCellRelation	76

4.3.32.1	Definition	76

4.3.32.2	Attributes	76

4.3.32.3	Attribute constraints	77

4.3.32.4	Notifications	77

4.3.33	NRFreqRelation	77

4.3.33.1	Definition	77

4.3.33.2	Attributes	77

4.3.33.3	Attribute constraints	77

4.3.33.4	Notifications	78

4.3.34	Void	78

4.3.35	ExternalNRCellCU	78

4.3.35.1	Definition	78

4.3.35.2	Attributes	78

4.3.35.3	Attribute constraints	78

4.3.35.4	Notifications	78

4.3.36	RRMPolicyRatio	78

4.3.36.1	Definition	78

4.3.36.2	Attributes	79

4.3.36.3	Attribute constraints	80

4.3.36.4	Notifications	80

4.3.37	S-NSSAI <<dataType>>	80

4.3.37.1	Definition	80

4.3.37.2	Attributes	80

4.3.37.3	Attribute constraints	80

4.3.37.4	Notifications	80

4.3.38	NRFrequency	80

4.3.38.1	Definition	80

4.3.38.2	Attributes	80

4.3.38.3	Attribute constraints	81

4.3.38.4	Notifications	81

4.3.39	CommonBeamformingFunction	81

4.3.39.1	Definition	81

4.3.39.2	Attributes	81

4.3.39.3	Attribute constraints	81

4.3.39.4	Notifications	81

4.3.40	Beam	81

4.3.40.1	Definition	81

4.3.40.2	Attributes	82

4.3.40.3	Attribute constraints	82

4.3.40.4	Notifications	82

4.3.41	PLMNInfo <<dataType>>	82

4.3.41.1	Definition	82

4.3.41.2	Attributes	82

4.3.41.3	Attribute constraints	83

4.3.41.4	Notifications	83

4.3.42	RRMPolicyMember <<dataType>>	83

4.3.42.1	Definition	83

4.3.42.2	Attributes	83

4.3.42.3	Attribute constraints	83

4.3.42.4	Notifications	83

4.3.43	RRMPolicy_	83

4.3.43.1	Definition	83

4.3.43.2	Attributes	83

4.3.43.3	Attribute constraints	84

4.3.43.4	Notifications	84

4.3.44	RRMPolicyManagedEntity <<ProxyClass>>	84

4.3.44.1	Definition	84

4.3.44.2	Attributes	84

4.3.44.3	Attribute constraints	84

4.3.44.4	Notifications	84

4.3.45	GNBCUCPNeighbour <<ProxyClass>>	85

4.3.45.1	Definition	85

4.3.45.2	Attributes	85

4.3.45.3	Attribute constraints	85

4.3.45.4	Notifications	85

4.3.46	GNBCUUPNeighbour <<ProxyClass>>	85

4.3.46.1	Definition	85

4.3.46.2	Attributes	85

4.3.46.3	Attribute constraints	85

4.3.46.4	Notifications	85

4.3.47	MappingSetIDBackhaulAddress  <<dataType>>	86

4.3.47.1	Definition	86

4.3.47.2	Attributes	86

4.3.47.3	Attribute constraints	86

4.3.47.4	Notifications	86

4.3.48	BackhaulAddress  <<dataType>>	86

4.3.48.1	Definition	86

4.3.48.2	Attributes	86

4.3.48.3	Attribute constraints	86

4.3.48.4	Notifications	86

4.3.49	Void	86

4.3.50	RimRSGlobal	86

4.3.50.1	Definition	86

4.3.50.2	Attributes	87

4.3.50.3	Attribute constraints	87

4.3.50.4	Notifications	87

4.3.51	FrequencyDomainPara <<dataType>>	87

4.3.51.1	Definition	87

4.3.51.2	Attributes	87

4.3.51.3	Attribute constraints	88

4.3.51.4	Notifications	88

4.3.52	SequenceDomainPara  <<dataType>>	88

4.3.52.1	Definition	88

4.3.52.2	Attributes	88

4.3.52.3	Attribute constraints	89

4.3.52.4	Notifications	89

4.3.53	TimeDomainPara  <<dataType>>	89

4.3.53.1	Definition	89

4.3.53.2	Attributes	89

4.3.53.3	Attribute constraints	89

4.3.53.4	Notifications	89

4.3.54	RimRSReportConf <<dataType>>	89

4.3.54.1	Definition	89

4.3.54.2	Attributes	89

4.3.54.3	Attribute constraints	90

4.3.54.4	Notifications	90

4.3.55	RimRSReportInfo <<dataType>>	90

4.3.55.1	Definition	90

4.3.55.2	Attributes	90

4.3.55.3	Attribute constraints	90

4.3.55.4	Notifications	90

4.3.56	RimRSSet	90

4.3.56.1	Definition	90

4.3.56.2	Attributes	91

4.3.56.3	Attribute constraints	91

4.3.56.4	Notifications	91

4.3.57	DANRManagementFunction	91

4.3.57.1	Definition	91

4.3.57.2	Attributes	91

4.3.57.3	Attribute constraints	91

4.3.57.4	Notifications	91

4.3.58	DESManagementFunction	91

4.3.58.1	Definition	91

4.3.58.2	Attributes	92

4.3.58.3	Attribute constraints	92

4.3.58.4	Notification	93

4.3.59	DRACHOptimizationFunction	93

4.3.59.1	Definition	93

4.3.59.2	Attributes	93

4.3.59.3	Attribute constraints	93

4.3.59.4	Notifications	93

4.3.60	DMROFunction	93

4.3.60.1	Definition	93

4.3.60.2	Attributes	93

4.3.60.3	Attribute constraints	94

4.3.60.4	Notifications	94

4.3.61	DPCIConfigurationFunction	94

4.3.61.1	Definition	94

4.3.61.2	Attributes	94

4.3.61.3	Attribute constraints	94

4.3.61.4	Notifications	94

4.3.62	CPCIConfigurationFunction	95

4.3.62.1	Definition	95

4.3.62.2	Attributes	95

4.3.62.3	Attribute constraints	95

4.3.62.4	Notifications	95

4.3.63	CESManagementFunction	95

4.3.63.1	Definition	95

4.3.63.2	Attributes	95

4.3.63.3	Attribute constraints	96

4.3.63.4	Notification	96

4.3.64	AddressWithVlan <<dataType>>	96

4.3.64.1	Definition	96

4.3.64.2	Attributes	97

4.3.64.3	Attribute constraints	97

4.3.64.4	Notifications	97

4.3.65	TceIDMappingInfo  <<dataType>>	97

4.3.65.1	Definition	97

4.3.65.2	Attributes	97

4.3.65.3	Attribute constraints	97

4.3.65.4	Notifications	97

4.3.66	Void	97

4.3.67	OperatorDU	97

4.3.67.1	Definition	97

4.3.67.2	Attributes	98

4.3.67.3	Attribute Constraints	98

4.3.67.4	Notifications	98

4.3.68	NROperatorCellDU	98

4.3.68.1	Definition	98

4.3.68.2	Attributes	99

4.3.68.3	Attribute Constraints	99

4.3.68.4	Notifications	99

4.3.69	DLBOFunction	99

4.3.69.1	Definition	99

4.3.69.2	Attributes	99

4.3.69.3	Attribute constraints	99

4.3.69.4	Notifications	100

4.3.70	CCOFunction	100

4.3.70.1	Definition	100

4.3.70.2	Attributes	100

4.3.70.3	Attribute constraints	100

4.3.70.4	Notifications	100

4.3.71	CCOWeakCoverageParameters	100

4.3.71.1	Definition	100

4.3.71.2	Attributes	100

4.3.71.3	Attribute constraints	100

4.3.71.4	Notifications	100

4.3.72	CCOPilotPollutionParameters	101

4.3.72.1	Definition	101

4.3.72.2	Attributes	101

4.3.72.3	Attribute constraints	101

4.3.72.4	Notifications	101

4.3.73	CCOOvershootCoverageParameters	101

4.3.73.1	Definition	101

4.3.73.2	Attributes	101

4.3.73.3	Attribute constraints	101

4.3.73.4	Notifications	101

4.3.74	CCOParameters	101

4.3.74.1	Definition	101

4.3.74.2	Attributes	101

4.3.74.3	Attribute constraints	102

4.3.74.4	Notifications	102

4.3.75	ParameterRange <<dataType>>	102

4.3.75.1	Definition	102

4.3.75.2	Attributes	102

4.3.75.3	Attribute constraints	102

4.3.75.4	Notifications	102

4.3.76	BWPSet	102

4.3.76.1	Definition	102

4.3.76.2	Attributes	102

4.3.76.3	Attribute constraints	103

4.3.76.4	Notifications	103

4.3.77	NTNFunction	103

4.3.77.1	Definition	103

4.3.77.2	Attributes	103

4.3.77.3	Attribute constraints	103

4.3.77.4	Notifications	103

4.3.78	EphemerisInfoSet	103

4.3.78.1	Definition	103

4.3.78.2	Attributes	103

4.3.78.3	Attribute constraints	103

4.3.78.4	Notifications	103

4.3.79	Ephemeris<<dataType>>	104

4.3.79.1	Definition	104

4.3.79.2	Attributes	104

4.3.79.3	Attribute constraints	104

4.3.79.4	Notifications	104

4.3.80	PositionVelocity <<dataType>>	104

4.3.80.1	Definition	104

4.3.80.2	Attributes	104

4.3.80.3	Attribute constraints	105

4.3.80.4	Notifications	105

4.3.81	Orbital <<dataType>>	105

4.3.81.1	Definition	105

4.3.81.2	Attributes	105

4.3.81.3	Attribute constraints	105

4.3.81.4	Notifications	105

4.3.82	QceIdMappingInfo <<dataType>>	105

4.3.82.1	Definition	105

4.3.82.2	Attributes	105

4.3.82.3	Attribute constraints	105

4.3.82.4	Notifications	105

4.3.83	MappedCellIdInfo  <<dataType>>	106

4.3.83.1	Definition	106

4.3.83.2	Attributes	106

4.3.83.3	Attribute constraints	106

4.3.83.4	Notifications	106

4.3.84	LoadTimeThreshold <<dataType>>	106

4.3.84.1	Definition	106

4.3.84.2	Attributes	106

4.3.84.3	Attribute constraints	106

4.3.84.4	Notifications	106

4.3.85	EsNotAllowedTimePeriod <<dataType>>	106

4.3.85.1	Definition	106

4.3.85.2	Attributes	106

4.3.85.3	Attribute constraints	107

4.3.85.4	Notifications	107

4.3.86	UeAccDelayProbability <<dataType>>	107

4.3.86.1	Definition	107

4.3.86.2	Attributes	107

4.3.86.3	Attribute constraints	107

4.3.86.4	Notifications	107

4.3.87	UeAccProbability <<dataType>>	107

4.3.87.1	Definition	107

4.3.87.2	Attributes	107

4.3.87.3	Attribute constraints	107

4.3.87.4	Notifications	107

4.3.88	NRNetwork	108

4.3.88.1	Definition	108

4.3.88.2	Attributes	108

4.3.88.3	Attribute constraints	108

4.3.88.4	Notifications	108

4.3.89	EUtraNetwork	108

4.3.89.1	Definition	108

4.3.89.2	Attributes	108

4.3.89.3	Attribute constraints	108

4.3.89.4	Notifications	108

4.3.90	NRECMappingRule	108

4.3.90.1	Definition	108

4.3.90.2	Attributes	109

4.3.90.3	Attribute Constraints	109

4.3.90.4	Notifications	109

4.4	Attribute definitions	110

4.4.1	Attribute properties	110

4.5	Common notifications	145

4.5.1	Alarm notifications	145

4.5.2	Configuration notifications	145

4.5.3	Threshold Crossing notifications	145

5	Information Model definitions for 5GC NRM	146

5.1	Imported information entities and local labels	146

5.2	Class diagram	146

5.2.1	Class diagram of 5GC NFs	146

5.2.1.1	Relationships	146

5.2.1.2	Inheritance	160

5.2.2	Class diagram of AMF Region/AMF Set	167

5.2.2.1	Relationships	167

5.2.2.2	Inheritance	168

5.3	Class definitions	168

5.3.1	AMFFunction	168

5.3.1.1	Definition	168

5.3.1.2	Attributes	168

5.3.1.3	Attribute constraints	169

5.3.1.4	Notifications	169

5.3.2	SMFFunction	169

5.3.2.1	Definition	169

5.3.2.2	Attributes	169

5.3.2.3	Attribute constraints	170

5.3.2.4	Notifications	170

5.3.3	UPFFunction	170

5.3.3.1	Definition	170

5.3.3.2	Attributes	170

5.3.3.3	Attribute constraints	171

5.3.3.4	Notifications	171

5.3.4	N3IWFFunction	171

5.3.4.1	Definition	171

5.3.4.2	Attributes	171

5.3.4.3	Attribute constraints	171

5.3.4.4	Notifications	171

5.3.5	PCFFunction	171

5.3.5.1	Definition	171

5.3.5.2	Attributes	171

5.3.5.3	Attribute constraints	172

5.3.5.4	Notifications	172

5.3.6	AUSFFunction	172

5.3.6.1	Definition	172

5.3.6.2	Attributes	172

5.3.6.3	Attribute constraints	172

5.3.6.4	Notifications	172

5.3.7	UDMFunction	172

5.3.7.1	Definition	172

5.3.7.2	Attributes	173

5.3.7.3	Attribute constraints	173

5.3.7.4	Notifications	173

5.3.8	UDRFunction	173

5.3.8.1	Definition	173

5.3.8.2	Attributes	173

5.3.8.3	Attribute constraints	173

5.3.8.4	Notifications	173

5.3.9	UDSFFunction	174

5.3.9.1	Definition	174

5.3.9.2	Attributes	174

5.3.9.3	Attribute constraints	174

5.3.9.4	Notifications	174

5.3.10	NRFFunction	174

5.3.10.1	Definition	174

5.3.10.2	Attributes	174

5.3.10.3	Attribute constraints	174

5.3.10.4	Notifications	175

5.3.11	NSSFFunction	175

5.3.11.1	Definition	175

5.3.11.2	Attributes	175

5.3.11.3	Attribute constraints	175

5.3.11.4	Notifications	175

5.3.12	AFFunction	175

5.3.12.1	Definition	175

5.3.12.2	Attributes	175

5.3.12.3	Attribute constraints	175

5.3.12.4	Notifications	175

5.3.13	DNFunction	176

5.3.13.1	Definition	176

5.3.14	SMSFFunction	176

5.3.14.1	Definition	176

5.3.14.2	Attributes	176

5.3.14.3	Attribute constraints	176

5.3.14.4	Notifications	176

5.3.15	LMFFunction	176

5.3.15.1	Definition	176

5.3.15.2	Attributes	176

5.3.15.3	Attribute constraints	177

5.3.15.4	Notifications	177

5.3.16	NGEIRFunction	177

5.3.16.1	Definition	177

5.3.16.2	Attributes	177

5.3.16.3	Attribute constraints	177

5.3.16.4	Notifications	177

5.3.17	SEPPFunction	178

5.3.17.1	Definition	178

5.3.17.2	Attributes	178

5.3.17.3	Attribute constraints	178

5.3.17.4	Notifications	178

5.3.18	NWDAFFunction	178

5.3.18.1	Definition	178

5.3.18.2	Attributes	178

5.3.18.3	Attribute constraints	179

5.3.18.4	Notifications	179

5.3.19	EP_N2	179

5.3.19.1	Definition	179

5.3.19.2	Attributes	179

5.3.19.3	Attribute constraints	179

5.3.19.4	Notifications	179

5.3.20	EP_N3	179

5.3.20.1	Definition	179

5.3.20.2	Attributes	179

5.3.20.3	Attribute constraints	180

5.3.20.4	Notifications	180

5.3.21	EP_N4	180

5.3.21.1	Definition	180

5.3.21.2	Attributes	180

5.3.21.3	Attribute constraints	180

5.3.21.4	Notifications	180

5.3.22	EP_N5	180

5.3.22.1	Definition	180

5.3.22.2	Attributes	180

5.3.22.3	Attribute constraints	181

5.3.22.4	Notifications	181

5.3.23	EP_N6	181

5.3.23.1	Definition	181

5.3.23.2	Attributes	181

5.3.23.3	Attribute constraints	181

5.3.23.4	Notifications	181

5.3.24	EP_N7	181

5.3.24.1	Definition	181

5.3.24.2	Attributes	181

5.3.24.3	Attribute constraints	181

5.3.24.4	Notifications	181

5.3.25	EP_N8	182

5.3.25.1	Definition	182

5.3.25.2	Attributes	182

5.3.25.3	Attribute constraints	182

5.3.25.4	Notifications	182

5.3.26	EP_N9	182

5.3.26.1	Definition	182

5.3.26.2	Attributes	182

5.3.26.3	Attribute constraints	182

5.3.26.4	Notifications	182

5.3.27	EP_N10	182

5.3.27.1	Definition	182

5.3.27.2	Attributes	183

5.3.27.3	Attribute constraints	183

5.3.27.4	Notifications	183

5.3.28	EP_N11	183

5.3.28.1	Definition	183

5.3.28.2	Attributes	183

5.3.28.3	Attribute constraints	183

5.3.28.4	Notifications	183

5.3.29	EP_N12	183

5.3.29.1	Definition	183

5.3.29.2	Attributes	183

5.3.29.3	Attribute constraints	184

5.3.29.4	Notifications	184

5.3.30	EP_N13	184

5.3.30.1	Definition	184

5.3.30.2	Attributes	184

5.3.30.3	Attribute constraints	184

5.3.30.4	Notifications	184

5.3.31	EP_N14	184

5.3.31.1	Definition	184

5.3.31.2	Attributes	184

5.3.31.3	Attribute constraints	185

5.3.31.4	Notifications	185

5.3.32	EP_N15	185

5.3.32.1	Definition	185

5.3.32.2	Attributes	185

5.3.32.3	Attribute constraints	185

5.3.32.4	Notifications	185

5.3.33	EP_N16	185

5.3.33.1	Definition	185

5.3.33.2	Attributes	185

5.3.33.3	Attribute constraints	185

5.3.33.4	Notifications	186

5.3.34	EP_N17	186

5.3.34.1	Definition	186

5.3.34.2	Attributes	186

5.3.34.3	Attribute constraints	186

5.3.34.4	Notifications	186

5.3.35	EP_N20	186

5.3.35.1	Definition	186

5.3.35.2	Attributes	186

5.3.35.3	Attribute constraints	186

5.3.35.4	Notifications	186

5.3.36	EP_N21	187

5.3.36.1	Definition	187

5.3.36.2	Attributes	187

5.3.36.3	Attribute constraints	187

5.3.36.4	Notifications	187

5.3.37	EP_N22	187

5.3.37.1	Definition	187

5.3.37.2	Attributes	187

5.3.37.3	Attribute constraints	187

5.3.37.4	Notifications	187

5.3.38	EP_N26	187

5.3.38.1	Definition	187

5.3.38.2	Attributes	188

5.3.38.3	Attribute constraints	188

5.3.38.4	Notifications	188

5.3.39	Void	188

5.3.40	Void	188

5.3.41	EP_S5C	188

5.3.41.1	Definition	188

5.3.41.2	Attributes	188

5.3.41.3	Attribute constraints	188

5.3.41.4	Notifications	188

5.3.42	EP_S5U	188

5.3.42.1	Definition	188

5.3.42.2	Attributes	189

5.3.42.3	Attribute constraints	189

5.3.42.4	Notifications	189

5.3.43	EP_Rx	189

5.3.43.1	Definition	189

5.3.43.2	Attributes	189

5.3.43.3	Attribute constraints	189

5.3.43.4	Notifications	189

5.3.44	EP_MAP_SMSC	189

5.3.44.1	Definition	189

5.3.44.2	Attributes	189

5.3.44.3	Attribute constraints	190

5.3.44.4	Notifications	190

5.3.45	EP_NL1	190

5.3.45.1	Definition	190

5.3.45.2	Attributes	190

5.3.45.3	Attribute constraints	190

5.3.45.4	Notifications	190

5.3.46	EP_NL2	190

5.3.46.1	Definition	190

5.3.46.2	Attributes	190

5.3.46.3	Attribute constraints	191

5.3.46.4	Notifications	191

5.3.47	EP_N27	191

5.3.47.1	Definition	191

5.3.47.2	Attributes	191

5.3.47.3	Attribute constraints	191

5.3.47.4	Notifications	191

5.3.48	EP_N31	191

5.3.48.1	Definition	191

5.3.48.2	Attributes	191

5.3.48.3	Attribute constraints	191

5.3.48.4	Notifications	192

5.3.49	ExternalNRFFunction	192

5.3.49.1	Definition	192

5.3.49.2	Attributes	192

5.3.49.3	Attribute constraints	192

5.3.49.4	Notifications	192

5.3.50	ExternalNSSFFunction	192

5.3.50.1	Definition	192

5.3.50.2	Attributes	192

5.3.50.3	Attribute constraints	192

5.3.50.4	Notifications	192

5.3.51	AMFSet	193

5.3.51.1	Definition	193

5.3.51.2	Attributes	193

5.3.51.3	Attribute constraints	193

5.3.51.4	Notifications	193

5.3.52	AMFRegion	193

5.3.52.1	Definition	193

5.3.52.2	Attributes	193

5.3.52.3	Attribute constraints	194

5.3.52.4	Notifications	194

5.3.53	ExternalAMFFunction	194

5.3.53.1	Definition	194

5.3.53.2	Attributes	194

5.3.53.3	Attribute constraints	194

5.3.53.4	Notifications	194

5.3.54	ManagedNFProfile <<dataType>>	194

5.3.54.1	Definition	194

5.3.54.2	Attributes	195

5.3.54.3	Attribute constraints	195

5.3.54.4	Notifications	195

5.3.55	HostAddr <<choice>>	195

5.3.55.1	Definition	195

5.3.56	Void	196

5.3.57	Void	196

5.3.58	AusfInfo <<dataType>>	196

5.3.58.1	Definition	196

5.3.58.2	Attributes	196

5.3.58.3	Attribute constraints	196

5.3.58.4	Notifications	196

5.3.59	Void	196

5.3.60	Void	196

5.3.61	Void	197

5.3.62	EP_N32	197

5.3.62.1	Definition	197

5.3.62.2	Attributes	197

5.3.62.3	Attribute constraints	197

5.3.62.4	Notifications	197

5.3.63	ExternalSEPPFunction	197

5.3.63.1	Definition	197

5.3.63.2	Attributes	197

5.3.63.3	Attribute constraints	197

5.3.63.4	Notifications	198

5.3.64	SEPPFunction <<ProxyClass>>	198

5.3.64.1	Definition	198

5.3.64.2	Attributes	198

5.3.64.3	Attribute constraints	198

5.3.64.4	Notifications	198

5.3.65	NEFFunction	198

5.3.65.1	Definition	198

5.3.65.2	Attributes	198

5.3.65.3	Attribute constraints	198

5.3.65.4	Notifications	198

5.3.66	SCPFunction	199

5.3.67.1	Definition	199

5.3.67.2	Attributes	199

5.3.67.3	Attribute constraints	199

5.3.67.4	Notifications	199

5.3.68	SupportedFunction <<dataType>>	199

5.3.68.1	Definition	199

5.3.68.2	Attributes	199

5.3.68.3	Attribute constraints	199

5.3.68.4	Notifications	199

5.3.69	CommModel <<dataType>>	199

5.3.69.1	Definition	199

5.3.69.2	Attributes	200

5.3.69.3	Attribute constraints	200

5.3.69.4	Notifications	200

5.3.70	QFQoSMonitoringControl	200

5.3.70.1	Definition	200

5.3.70.2	Attributes	200

5.3.70.3	Attribute constraints	201

5.3.70.4	Notifications	201

5.3.71	QFPacketDelayThresholdsType <<dataType>>	201

5.3.71.1	Definition	201

5.3.71.2	Attributes	201

5.3.71.3	Attribute constraints	201

5.3.71.4	Notifications	201

5.3.72	GtpUPathQoSMonitoringControl	201

5.3.72.1	Definition	201

5.3.72.2	Attributes	201

5.3.72.3	Attribute constraints	202

5.3.72.4	Notifications	202

5.3.73	GtpUPathDelayThresholdsType <<dataType>>	202

5.3.73.1	Definition	202

5.3.73.2	Attributes	202

5.3.73.3	Attribute constraints	202

5.3.73.4	Notifications	202

5.3.74	Void	203

5.3.75	Configurable5QISet	203

5.3.75.1	Definition	203

5.3.75.2	Attributes	203

5.3.75.3	Attribute constraints	203

5.3.75.4	Notifications	203

5.3.76	FiveQICharacteristics	203

5.3.76.1	Definition	203

5.3.76.2	Attributes	203

5.3.76.3	Attribute constraints	203

5.3.76.4	Notifications	203

5.3.77	PacketErrorRate <<dataType>>	204

5.3.77.1	Definition	204

5.3.77.2	Attributes	204

5.3.77.3	Attribute constraints	204

5.3.77.4	Notifications	204

5.3.78	FiveQiDscpMappingSet	204

5.3.78.1	Definition	204

5.3.78.2	Attributes	204

5.3.78.3	Attribute constraints	204

5.3.78.4	Notifications	204

5.3.79	FiveQiDscpMapping <<dataType>>	204

5.3.79.1	Definition	204

5.3.79.2	Attributes	205

5.3.79.3	Attribute constraints	205

5.3.79.4	Notifications	205

5.3.80	PredefinedPccRuleSet	205

5.3.80.1	Definition	205

5.3.80.2	Attributes	205

5.3.80.3	Attribute constraints	205

5.3.80.4	Notifications	205

5.3.81	PccRule <<dataType>>	205

5.3.81.1	Definition	205

5.3.81.2	Attributes	206

5.3.81.3	Attribute constraints	206

5.3.81.4	Notifications	206

5.3.82	FlowInformation <<dataType>>	206

5.3.82.1	Definition	206

5.3.82.2	Attributes	206

5.3.82.3	Attribute constraints	207

5.3.82.4	Notifications	207

5.3.83	EthFlowDescription <<dataType>>	207

5.3.83.1	Definition	207

5.3.83.2	Attributes	207

5.3.83.3	Attribute constraints	207

5.3.83.4	Notifications	207

5.3.84	QoSData <<dataType>>	207

5.3.84.1	Definition	207

5.3.84.2	Attributes	208

5.3.84.3	Attribute constraints	208

5.3.84.4	Notifications	208

5.3.85	ARP <<dataType>>	208

5.3.85.1	Definition	208

5.3.85.2	Attributes	208

5.3.85.3	Attribute constraints	208

5.3.85.4	Notifications	208

5.3.86	TrafficControlData <<dataType>>	209

5.3.86.1	Definition	209

5.3.86.2	Attributes	209

5.3.86.3	Attribute constraints	209

5.3.86.4	Notifications	209

5.3.87	RedirectInformation <<dataType>>	209

5.3.87.1	Definition	209

5.3.87.2	Attributes	209

5.3.87.3	Attribute constraints	209

5.3.87.4	Notifications	210

5.3.88	RouteToLocation <<dataType>>	210

5.3.88.1	Definition	210

5.3.88.2	Attributes	210

5.3.88.3	Attribute constraints	210

5.3.88.4	Notifications	210

5.3.89	RouteInformation <<dataType>>	210

5.3.89.1	Definition	210

5.3.89.2	Attributes	210

5.3.89.3	Attribute constraints	210

5.3.89.4	Notifications	211

5.3.90	UpPathChgEvent <<dataType>>	211

5.3.90.1	Definition	211

5.3.90.2	Attributes	211

5.3.90.3	Attribute constraints	211

5.3.90.4	Notifications	211

5.3.91	SteeringMode <<dataType>>	211

5.3.91.1	Definition	211

5.3.91.2	Attributes	211

5.3.91.3	Attribute constraints	212

5.3.91.4	Notifications	212

5.3.92	ConditionData  <<dataType>>	212

5.3.92.1	Definition	212

5.3.92.2	Attributes	212

5.3.92.3	Attribute constraints	212

5.3.92.4	Notifications	212

5.3.93	TscaiInputContainer  <<dataType>>	212

5.3.93.1	Definition	212

5.3.93.2	Attributes	212

5.3.93.3	Attribute constraints	213

5.3.93.4	Notifications	213

5.3.94	Dynamic5QISet	213

5.3.94.1	Definition	213

5.3.94.2	Attributes	213

5.3.94.3	Attribute constraints	213

5.3.94.4	Notifications	213

5.3.95	NetworkSliceInfo <<dataType>>	213

5.3.95.1	Definition	213

5.3.95.2	Attributes	213

5.3.95.3	Attribute constraints	213

5.3.95.4	Notifications	213

5.3.96	NSACFFunction	214

5.3.96.1	Definition	214

5.3.96.2	Attributes	214

5.3.96.3	Attribute constraints	214

5.3.96.4	Notifications	214

5.3.97	SnssaiInfo <<dataType>>	214

5.3.97.1	Definition	214

5.3.97.2	Attributes	214

5.3.97.3	Attribute constraints	214

5.3.97.4	Notifications	214

5.3.98	NsacfInfoSnssai <<dataType>>	214

5.3.98.1	Definition	214

5.3.98.2	Attributes	215

5.3.98.3	Attribute constraints	215

5.3.98.4	Notifications	215

5.3.99	EP_N60	215

5.3.99.1	Definition	215

5.3.99.2	Attributes	215

5.3.99.3	Attribute constraints	215

5.3.99.4	Notifications	215

5.3.100	EP_N33	216

5.3.100.1	Definition	216

5.3.100.2	Attributes	216

5.3.100.3	Attribute constraints	216

5.3.100.4	Notifications	216

5.3.101	DDNMFFunction	216

5.3.101.1	Definition	216

5.3.101.2	Attributes	216

5.3.101.3	Attribute constraints	216

5.3.101.4	Notifications	216

5.3.102	EP_Npc4	217

5.3.102.1	Definition	217

5.3.102.2	Attributes	217

5.3.102.3	Attribute constraints	217

5.3.102.4	Notifications	217

5.3.103	EP_Npc6	217

5.3.103.1	Definition	217

5.3.103.2	Attributes	217

5.3.103.3	Attribute constraints	217

5.3.103.4	Notifications	217

5.3.104	EP_Npc7	218

5.3.104.1	Definition	218

5.3.104.2	Attributes	218

5.3.104.3	Attribute constraints	218

5.3.104.4	Notifications	218

5.3.105	GUAMInfo <<dataType>>	218

5.3.105.1	Definition	218

5.3.105.2	Attributes	218

5.3.105.3	Attribute constraints	218

5.3.105.4	Notifications	218

5.3.106	TaiRange <<dataType>>	218

5.3. 106.1	Definition	218

5.3.106.2	Attributes	219

5.3.106.3	Attribute constraints	219

5.3.106.4	Notifications	219

5.3.107	NRTACRange <<dataType>>	219

5.3.107.1	Definition	219

5.3.107.2	Attributes	219

5.3.107.3	Attribute constraints	219

5.3.107.4	Notifications	219

5.3.108	SCPInfo <<dataType>>	219

5.3.108.1	Definition	219

5.3.108.2	Attributes	220

5.3.108.3	Attribute constraints	220

5.3.108.4	Notifications	220

5.3.109	SCPDomainInfo <<dataType>>	220

5.3.109.1	Definition	220

5.3.109.2	Attributes	220

5.3.109.3	Attribute constraints	220

5.3.109.4	Notifications	220

5.3.110	IpEndPoint <<dataType>>	221

5.3.110.1	Definition	221

5.3.110.2	Attributes	221

5.3.110.3	Attribute constraints	221

5.3.110.4	Notifications	221

5.3.111	IPv4AddressRange <<dataType>>	221

5.3.111.1	Definition	221

5.3.111.2	Attributes	221

5.3.111.3	Attribute constraints	221

5.3.111.4	Notifications	221

5.3.112	IPv6PrefixRange <<dataType>>	221

5.3.112.1	Definition	221

5.3.112.2	Attributes	222

5.3.112.3	Attribute constraints	222

5.3.112.4	Notifications	222

5.3.113	EASDFFunction	222

5.3.113.1	Definition	222

5.3.113.2	Attributes	222

5.3.113.3	Attribute constraints	222

5.3.113.4	Notifications	222

5.3.114	EP_N88x	222

5.3.114.1	Definition	222

5.3.114.2	Attributes	222

5.3.114.3	Attribute constraints	223

5.3.114.4	Notifications	223

5.3.115	SNPNInfo <<dataType>>	223

5.3.115.1	Definition	223

5.3.115.2	Attributes	223

5.3.115.3	Attribute constraints	223

5.3.115.4	Notifications	223

5.3.116	SNPNId <<dataType>>	223

5.3.116.1	Definition	223

5.3.116.2	Attributes	223

5.3.116.3	Attribute constraints	223

5.3.116.4	Notifications	223

5.3.117	EP_Npc8	224

5.3.117.1	Definition	224

5.3.117.2	Attributes	224

5.3.117.3	Attribute constraints	224

5.3.117.4	Notifications	224

5.3.118	DefaultNotificationSubscription <<dataType>>	224

5.3.118.1	Definition	224

5.3.118.2	Attributes	224

5.3.118.3	Attribute constraints	224

5.3.118.4	Notifications	224

5.3.119	EcmConnectionInfo	225

5.3.119.1	Definition	225

5.3.119.2	Attributes	225

5.3.119.3	Attribute constraints	225

5.3.119.4	Notifications	225

5.3.120	5GCNfConnEcmInfo <<dataType>>	225

5.3.120.1	Definition	225

5.3.120.2	Attributes	226

5.3.120.3	Attribute constraints	226

5.3.120.4	Notifications	226

5.3.121	UPFConnInfo <<dataType>>	226

5.3.121.1	Definition	226

5.3.121.2	Attributes	226

5.3.121.3	Attribute constraints	226

5.3.121.4	Notifications	226

5.3.122	SnssaiSmfInfoItem <<dataType>>	226

5.3.122.1	Definition	226

5.3.122.2	Attributes	226

5.3.122.3	Attribute constraints	226

5.3.122.4	Notifications	227

5.3.123	DnnSmfInfoItem <<dataType>>	227

5.3.123.1	Definition	227

5.3.123.2	Attributes	227

5.3.123.3	Attribute constraints	227

5.3.123.4	Notifications	227

5.3.124	Void	227

5.3.125	InterfaceUpfInfoItem <<dataType>>	227

5.3.125.1	Definition	227

5.3.125.2	Attributes	227

5.3.125.3	Attribute constraints	227

5.3.125.4	Notifications	228

5.3.126	AtsssCapability <<dataType>>	228

5.3.126.1	Definition	228

5.3.126.2	Attributes	228

5.3.126.3	Attribute constraints	228

5.3.126.4	Notifications	228

5.3.127	IpInterface <<dataType>>	228

5.3.127.1	Definition	228

5.3.127.2	Attributes	228

5.3.127.3	Attribute constraints	229

5.3.127.4	Notifications	229

5.3.128	SupiRange <<dataType>>	229

5.3.128.1	Definition	229

5.3.128.2	Attributes	229

5.3.128.3	Attribute constraints	229

5.3.128.4	Notifications	229

5.3.129	IdentityRange <<dataType>>	229

5.3.129.1	Definition	229

5.3.129.2	Attributes	229

5.3.129.3	Attribute constraints	229

5.3.129.4	Notifications	230

5.3.130	ProSeCapability <<dataType>>	230

5.3.130.1	Definition	230

5.3.130.2	Attributes	230

5.3.130.3	Attribute constraints	230

5.3.130.4	Notifications	230

5.3.131	V2xCapability <<dataType>>	230

5.3.131.1	Definition	230

5.3.131.2	Attributes	230

5.3.131.3	Attribute constraints	230

5.3.131.4	Notifications	230

5.3.132	InternalGroupIdRange <<dataType>>	231

5.3.132.1	Definition	231

5.3.132.2	Attributes	231

5.3.132.3	Attribute constraints	231

5.3.132.4	Notifications	231

5.3.133	SuciInfo <<dataType>>	231

5.3.133.1	Definition	231

5.3.133.2	Attributes	231

5.3.133.3	Attribute constraints	231

5.3.133.4	Notifications	231

5.3.134	SharedDataIdRange <<dataType>>	231

5.3.134.1	Definition	231

5.3.134.2	Attributes	231

5.3.134.3	Attribute constraints	232

5.3.134.4	Notifications	232

5.3.135	PfdData <<dataType>>	232

5.3.135.1	Definition	232

5.3.135.2	Attributes	232

5.3.135.3	Attribute constraints	232

5.3.135.4	Notifications	232

5.3.136	AfEventExposureData <<dataType>>	232

5.3.136.1	Definition	232

5.3.136.2	Attributes	232

5.3.136.3	Attribute constraints	232

5.3.136.4	Notifications	232

5.3.137	UnTrustAfInfo <<dataType>>	233

5.3.137.1	Definition	233

5.3.137.2	Attributes	233

5.3.137.3	Attribute constraints	233

5.3.137.4	Notifications	233

5.3.138	SnssaiInfoItem <<dataType>>	233

5.3.138.1	Definition	233

5.3.138.2	Attributes	233

5.3.138.3	Attribute constraints	233

5.3.138.4	Notifications	233

5.3.139	ExtSnssai <<dataType>>	233

5.3.139.1	Definition	233

5.3.139.2	Attributes	233

5.3.139.3	Attribute constraints	234

5.3.139.4	Notifications	234

5.3.140	SnssaiExtension <<dataType>>	234

5.3.140.1	Definition	234

5.3.140.2	Attributes	234

5.3.140.3	Attribute constraints	234

5.3.140.4	Notifications	234

5.3.141	DnnInfoItem <<dataType>>	234

5.3.141.1	Definition	234

5.3.141.2	Attributes	234

5.3.141.3	Attribute constraints	234

5.3.141.4	Notifications	234

5.3.142	SdRange <<dataType>>	235

5.3.142.1	Definition	235

5.3.142.2	Attributes	235

5.3.142.3	Attribute constraints	235

5.3.142.4	Notifications	235

5.3.142a	NsacfInfo <<dataType>>	235

5.3.142a.1	Definition	235

5.3.142a.2	Attributes	235

5.3.142a.3	Attribute constraints	235

5.3.142a.4	Notifications	235

5.3.142b	NsacfCapability <<dataType>>	235

5.3.142b.1	Definition	235

5.3.142b.2	Attributes	235

5.3.142b.3	Attribute constraints	236

5.3.142b.4	Notifications	236

5.3.142c	NwdafInfo <<dataType>>	236

5.3.142c.1	Definition	236

5.3.142c.2	Attributes	236

5.3.142c.3	Attribute constraints	236

5.3.142c.4	Notifications	236

5.3.142d	NwdafCapability <<dataType>>	236

5.3.142d.1	Definition	236

5.3.142d.2	Attributes	236

5.3.142d.3	Attribute constraints	236

5.3.142d.4	Notifications	237

5.3.142e	MlAnalyticsInfo <<dataType>>	237

5.3.142e.1	Definition	237

5.3.142e.2	Attributes	237

5.3.142e.3	Attribute constraints	237

5.3.142e.4	Notifications	237

5.3.143	PlmnIdNid <<dataType>>	237

5.3.143.1	Definition	237

5.3.143.2	Attributes	237

5.3.143.3	Attribute constraints	237

5.3.143.4	Notifications	237

5.3.143a	SeppInfo <<dataType>>	238

5.3.143a.1	Definition	238

5.3.143a.2	Attributes	238

5.3.143a.3	Attribute constraints	238

5.3.143a.4	Notifications	238

5.3.143b	UdsfInfo <<dataType>>	238

5.3.143b.1	Definition	238

5.3.143b.2	Attributes	238

5.3.143b.3	Attribute constraints	238

5.3.143b.4	Notifications	238

5.3.143c	SmsfInfo <<dataType>>	238

5.3.143c.1	Definition	238

5.3.143c.2	Attributes	239

5.3.143c.3	Attribute constraints	239

5.3.143c.4	Notifications	239

5.3.143d	PlmnRange <<dataType>>	239

5.3.143d.1	Definition	239

5.3.143d.2	Attributes	239

5.3.143d.3	Attribute constraints	239

5.3.143d.4	Notifications	239

5.3.143e	UdmInfo <<dataType>>	239

5.3.143e.1	Definition	239

5.3.143e.2	Attributes	239

5.3.143e.3	Attribute constraints	239

5.3.143e.4	Notifications	240

5.3.144	UdrInfo <<dataType>>	240

5.3.144.1	Definition	240

5.3.144.2	Attributes	240

5.3.144.3	Attribute constraints	240

5.3.144.4	Notifications	240

5.3.144a	LmfInfo <<dataType>>	240

5.3.144a.1	Definition	240

5.3.144a.2	Attributes	240

5.3.144a.3	Attribute constraints	241

5.3.144a.4	Notifications	241

5.3.144b	TrustAfInfo <<dataType>>	241

5.3.144b.1	Definition	241

5.3.144b.2	Attributes	241

5.3.144b.3	Attribute constraints	241

5.3.144b.4	Notifications	241

5.3.144c	EasdfInfo <<dataType>>	241

5.3.144c.1	Definition	241

5.3.144c.2	Attributes	241

5.3.144c.3	Attribute constraints	241

5.3.144c.4	Notifications	241

5.3.144d	SnssaiEasdfInfoItem  <<dataType>>	242

5.3.144d.1	Definition	242

5.3.144d.2	Attributes	242

5.3.144d.3	Attribute constraints	242

5.3.144d.4	Notifications	242

5.3.144e	DnnEasdfInfoItem <<dataType>>	242

5.3.144e.1	Definition	242

5.3.144e.2	Attributes	242

5.3.144e.3	Attribute constraints	242

5.3.144e.4	Notifications	242

5.3.145	NSSAAFFunction	242

5.3.145.1	Definition	242

5.3.145.2	Attributes	242

5.3.145.3	Attribute constraints	243

5.3.145.4	Notifications	243

5.3.146	NssaafInfo <<dataType>>	243

5.3.146.1	Definition	243

5.3.146.2	Attributes	243

5.3.146.3	Attribute constraints	243

5.3.146.4	Notifications	243

5.3.147	EP_N58	243

5.3.147.1	Definition	243

5.3.147.2	Attributes	243

5.3.147.3	Attribute constraints	244

5.3.147.4	Notifications	244

5.3.148	EP_N59	244

5.3.148.1	Definition	244

5.3.148.2	Attributes	244

5.3.148.3	Attribute constraints	244

5.3.148.4	Notifications	244

5.3.149	NrfInfo <<dataType>>	244

5.3.149.1	Definition	244

5.3.149.2	Attributes	245

5.3.149.3	Attribute constraints	245

5.3.149.4	Notifications	245

5.3.150	CHFFunction	245

5.3.150.1	Definition	245

5.3.150.2	Attributes	246

5.3.150.3	Attribute constraints	246

5.3.150.4	Notifications	246

5.3.151	ChfInfo <<dataType>>	246

5.3.151.1	Definition	246

5.3.151.2	Attributes	246

5.3.151.3	Attribute constraints	246

5.3.151.4	Notifications	246

5.3.152	EP_N40	246

5.3.152.1	Definition	246

5.3.152.2	Attributes	247

5.3.152.3	Attribute constraints	247

5.3.152.4	Notifications	247

5.3.153	EP_N41	247

5.3.153.1	Definition	247

5.3.153.2	Attributes	247

5.3.153.3	Attribute constraints	247

5.3.153.4	Notifications	247

5.3.154	EP_N42	247

5.3.154.1	Definition	247

5.3.154.2	Attributes	247

5.3.154.3	Attribute constraints	248

5.3.154.4	Notifications	248

5.3.155	EP_N28	248

5.3.155.1	Definition	248

5.3.155.2	Attributes	248

5.3.155.3	Attribute constraints	248

5.3.155.4	Notifications	248

5.3.156	MFAFFunction	248

5.3.156.1	Definition	248

5.3.156.2	Attributes	248

5.3.156.3	Attribute constraints	249

5.3.156.4	Notifications	249

5.3.157	MfafInfo <<dataType>>	249

5.3.157.1	Definition	249

5.3.157.2	Attributes	249

5.3.157.3	Attribute constraints	249

5.3.157.4	Notifications	249

5.3.158	DCCFFunction	249

5.3.158.1	Definition	249

5.3.158.2	Attributes	249

5.3.158.3	Attribute constraints	250

5.3.158.4	Notifications	250

5.3.159	DccfInfo <<dataType>>	250

5.3.159.1	Definition	250

5.3.159.2	Attributes	250

5.3.159.3	Attribute constraints	250

5.3.159.4	Notifications	250

5.3.160	AmfInfo <<dataType>>	250

5.3.160.1	Definition	250

5.3.160.2	Attributes	250

5.3.160.3	Attribute constraints	250

5.3.160.4	Notifications	251

5.3.161	SmfInfo <<dataType>>	251

5.3.161.1	Definition	251

5.3.161.2	Attributes	251

5.3.161.3	Attribute constraints	251

5.3.161.4	Notifications	251

5.3.162	UpfInfo <<dataType>>	251

5.3.162.1	Definition	251

5.3.162.2	Attributes	252

5.3.162.3	Attribute constraints	252

5.3.162.4	Notifications	252

5.3.163	PcfInfo <<dataType>>	252

5.3.163.1	Definition	252

5.3.163.2	Attributes	252

5.3.163.3	Attribute constraints	253

5.3.163.4	Notifications	253

5.3.164	NefInfo <<dataType>>	253

5.3.164.1	Definition	253

5.3.164.2	Attributes	253

5.3.164.3	Attribute constraints	253

5.3.164.4	Notifications	253

5.3.165	BSFFunction	253

5.3.165.1	Definition	253

5.3.165.2	Attributes	253

5.3.165.3	Attribute constraints	254

5.3.165.4	Notifications	254

5.3.166	BsfInfo <<dataType>>	254

5.3.166.1	Definition	254

5.3.166.2	Attributes	254

5.3.166.3	Attribute constraints	254

5.3.166.4	Notifications	254

5.3.167	AANFFunction	254

5.3.167.1	Definition	254

5.3.167.2	Attributes	254

5.3.167.3	Attribute constraints	255

5.3.167.4	Notifications	255

5.3.168	AanfInfo <<dataType>>	255

5.3.168.1	Definition	255

5.3.168.2	Attributes	255

5.3.168.3	Attribute constraints	255

5.3.168.4	Notifications	255

5.3.169	EP_N61	255

5.3.169.1	Definition	255

5.3.169.2	Attributes	255

5.3.169.3	Attribute constraints	255

5.3.169.4	Notifications	256

5.3.170	EP_N62	256

5.3.170.1	Definition	256

5.3.170.2	Attributes	256

5.3.170.3	Attribute constraints	256

5.3.170.4	Notifications	256

5.3.171	EP_N63	256

5.3.171.1	Definition	256

5.3.171.2	Attributes	256

5.3.171.3	Attribute constraints	256

5.3.171.4	Notifications	256

5.3.172	TSCTSFFunction	257

5.3.172.1	Definition	257

5.3.172.2	Attributes	257

5.3.172.3	Attribute constraints	257

5.3.172.4	Notifications	257

5.3.173	TsctsfInfo <<dataType>>	257

5.3.173.1	Definition	257

5.3.173.2	Attributes	257

5.3.173.3	Attribute constraints	257

5.3.173.4	Notifications	257

5.3.174	EP_N84	257

5.3.174.1	Definition	257

5.3.174.2	Attributes	258

5.3.174.3	Attribute constraints	258

5.3.174.4	Notifications	258

5.3.175	EP_N85	258

5.3.175.1	Definition	258

5.3.175.2	Attributes	258

5.3.175.3	Attribute constraints	258

5.3.175.4	Notifications	258

5.3.176	EP_N86	258

5.3.176.1	Definition	258

5.3.176.2	Attributes	258

5.3.176.3	Attribute constraints	259

5.3.176.4	Notifications	259

5.3.177	EP_N87	259

5.3.177.1	Definition	259

5.3.177.2	Attributes	259

5.3.177.3	Attribute constraints	259

5.3.177.4	Notifications	259

5.3.178	EP_N89	259

5.3.178.1	Definition	259

5.3.178.2	Attributes	259

5.3.178.3	Attribute constraints	260

5.3.178.4	Notifications	260

5.3.179	EP_N96	260

5.3.179.1	Definition	260

5.3.179.2	Attributes	260

5.3.179.3	Attribute constraints	260

5.3.179.4	Notifications	260

5.3.180	GMLCFunction	260

5.3.180.1	Definition	260

5.3.180.2	Attributes	260

5.3.180.3	Attribute constraints	260

5.3.180.4	Notifications	261

5.3.181	GmlcfInfo <<dataType>>	261

5.3.181.1	Definition	261

5.3.181.2	Attributes	261

5.3.181.3	Attribute constraints	261

5.3.181.4	Notifications	261

5.3.182	EP_NL3	261

5.3.182.1	Definition	261

5.3.182.2	Attributes	261

5.3.182.3	Attribute constraints	261

5.3.182.4	Notifications	261

5.3.183	EP_NL5	261

5.3.183.1	Definition	261

5.3.183.2	Attributes	262

5.3.183.3	Attribute constraints	262

5.3.183.4	Notifications	262

5.3.184	EP_NL6	262

5.3.184.1	Definition	262

5.3.184.2	Attributes	262

5.3.184.3	Attribute constraints	262

5.3.184.4	Notifications	262

5.3.185	EP_NL9	262

5.3.185.1	Definition	262

5.3.185.2	Attributes	262

5.3.185.3	Attribute constraints	263

5.3.185.4	Notifications	263

5.3.186	NTNPLMNRestrictionsInfo <<dataType>>	263

5.3.186.1	Definition	263

5.3.186.2	Attributes	263

5.3.186.3	Attribute constraints	263

5.3.186.4	Notifications	263

5.3.187	BlockedLocationInfo <<dataType>>	263

5.3.187.1	Definition	263

5.3.187.2	Attributes	263

5.3.187.3	Attribute constraints	263

5.3.187.4	Notifications	263

5.3.188	SatelliteCoverageInfo <<dataType>>	264

5.3.188.1	Definition	264

5.3.188.2	Attributes	264

5.3.188.3	Attribute constraints	264

5.3.188.4	Notifications	264

5.3.189	NtnLocationInfo <<dataType>>	264

5.3.189.1	Definition	264

5.3.189.2	Attributes	264

5.3.189.3	Attribute constraints	264

5.3.189.4	Notifications	264

5.3.190	N2InterfaceAmfInfo <<dataType>>	265

5.3.190.1	Definition	265

5.3.190.2	Attributes	265

5.3.190.3	Attribute constraints	265

5.3.190.4	Notifications	265

5.3.191	A2xCapability <<dataType>>	265

5.3.191.1	Definition	265

5.3.191.2	Attributes	265

5.3.191.3	Attribute constraints	265

5.3.191.4	Notifications	265

5.3.192	MBUPFFunction	265

5.3.192.1	Definition	265

5.3.192.2	Attributes	266

5.3.192.3	Attribute constraints	266

5.3.192.4	Notifications	266

5.3.193	MbUpfInfo <<dataType>>	266

5.3.193.1	Definition	266

5.3.193.2	Attributes	266

5.3.193.3	Attribute constraints	266

5.3.193.4	Notifications	266

5.3.194	SnssaiUpfInfoItem <<dataType>>	266

5.3.194.1	Definition	266

5.3.194.2	Attributes	267

5.3.194.3	Attribute constraints	267

5.3.194.4	Notifications	267

5.3.195	DnnUpfInfoItem <<dataType>>	267

5.3.195.1	Definition	267

5.3.195.2	Attributes	267

5.3.195.3	Attribute constraints	267

5.3.195.4	Notifications	267

5.3.196	EP_N3mb	267

5.3.196.1	Definition	267

5.3.196.2	Attributes	267

5.3.196.3	Attribute constraints	268

5.3.196.4	Notifications	268

5.3.197	EP_N19mb	268

5.3.197.1	Definition	268

5.3.197.2	Attributes	268

5.3.197.3	Attribute constraints	268

5.3.197.4	Notifications	268

5.3.198	EP_N4mb	268

5.3.198.1	Definition	268

5.3.198.2	Attributes	268

5.3.198.3	Attribute constraints	269

5.3.198.4	Notifications	269

5.3.199	EP_Nmb9	269

5.3.199.1	Definition	269

5.3.199.2	Attributes	269

5.3.199.3	Attribute constraints	269

5.3.199.4	Notifications	269

5.3.200	MBSMFFunction	269

5.3.200.1	Definition	269

5.3.200.2	Attributes	269

5.3.200.3	Attribute constraints	269

5.3.200.4	Notifications	270

5.3.201	Void	270

5.3.202	MbSmfInfo <<dataType>>	270

5.3.202.1	Definition	270

5.3.202.2	Attributes	270

5.3.202.3	Attribute constraints	270

5.3.202.4	Notifications	270

5.3.203	TmgiRange <<dataType>>	270

5.3.203.1	Definition	270

5.3.203.2	Attributes	270

5.3.203.3	Attribute constraints	270

5.3.203.4	Notifications	270

5.3.204	MbsSession <<dataType>>	271

5.3.204.1	Definition	271

5.3.204.2	Attributes	271

5.3.204.3	Attribute constraints	271

5.3.204.4	Notifications	271

5.3.205	SnssaiMbSmfInfoItem <<dataType>>	271

5.3.205.1	Definition	271

5.3.205.2	Attributes	271

5.3.205.3	Attribute constraints	271

5.3.205.4	Notifications	271

5.3.206	DnnMbSmfInfoItem <<dataType>>	271

5.3.206.1	Definition	271

5.3.206.2	Attributes	271

5.3.206.3	Attribute constraints	272

5.3.206.4	Notifications	272

5.3.207	MbsServiceAreaInfo <<dataType>>	272

5.3.207.1	Definition	272

5.3.207.2	Attributes	272

5.3.207.3	Attribute constraints	272

5.3.207.4	Notifications	272

5.3.208	Ssm <<dataType>>	272

5.3.208.1	Definition	272

5.3.208.2	Attributes	272

5.3.208.3	Attribute constraints	272

5.3.208.4	Notifications	272

5.3.209	Tmgi <<dataType>>	272

5.3.209.1	Definition	272

5.3.209.2	Attributes	273

5.3.209.3	Attribute constraints	273

5.3.209.4	Notifications	273

5.3.210	MbsServiceId <<dataType>>	273

5.3.210.1	Definition	273

5.3.210.2	Attributes	273

5.3.210.3	Attribute constraints	273

5.3.210.4	Notifications	273

5.3.211	MbsServiceArea <<dataType>>	273

5.3.211.1	Definition	273

5.3.211.2	Attributes	273

5.3.211.3	Attribute constraints	273

5.3.211.4	Notifications	274

5.3.212	Ncgi <<dataType>>	274

5.3.212.1	Definition	274

5.3.212.2	Attributes	274

5.3.212.3	Attribute constraints	274

5.3.212.4	Notifications	274

5.3.213	EP_N16mb	274

5.3.213.1	Definition	274

5.3.213.2	Attributes	274

5.3.213.3	Attribute constraints	274

5.3.213.4	Notifications	274

5.3.214	EP_N11mb	275

5.3.214.1	Definition	275

5.3.214.2	Attributes	275

5.3.214.3	Attribute constraints	275

5.3.214.4	Notifications	275

5.3.215	EP_Nmb1	275

5.3.215.1	Definition	275

5.3.215.2	Attributes	275

5.3.215.3	Attribute constraints	275

5.3.215.4	Notifications	275

5.3.216	5GDdnmfInfo <<dataType>>	276

5.3.216.1	Definition	276

5.3.216.2	Attributes	276

5.3.216.3	Attribute constraints	276

5.3.216.4	Notifications	276

5.3.217	HssInfo <<dataType>>	276

5.3.217.1	Definition	276

5.3.217.2	Attributes	276

5.3.217.3	Attribute constraints	276

5.3.217.4	Notifications	276

5.3.218	NetworkNodeDiameterAddress <<dataType>>	276

5.3.218.1	Definition	276

5.3.218.2	Attributes	277

5.3.218.3	Attribute constraints	277

5.3.218.4	Notifications	277

5.3.219	ImsiRange <<dataType>>	277

5.3.219.1	Definition	277

5.3.219.2	Attributes	277

5.3.219.3	Attribute constraints	277

5.3.219.4	Notifications	277

5.3.220	MNPFFunction	277

5.3.220.1	Definition	277

5.3.220.2	Attributes	277

5.3.220.3	Attribute constraints	278

5.3.220.4	Notifications	278

5.3.221	MnpfInfo <<dataType>>	278

5.3.221.1	Definition	278

5.3.221.2	Attributes	278

5.3.221.3	Attribute constraints	278

5.3.221.4	Notifications	278

5.3.222	EP_SM12	278

5.3.222.1	Definition	278

5.3.222.2	Attributes	278

5.3.222.3	Attribute constraints	278

5.3.222.4	Notifications	278

5.3.223	EP_SM13	279

5.3.223.1	Definition	279

5.3.223.2	Attributes	279

5.3.223.3	Attribute constraints	279

5.3.223.4	Notifications	279

5.3.224	EP_SM14	279

5.3.224.1	Definition	279

5.3.224.2	Attributes	279

5.3.224.3	Attribute constraints	279

5.3.224.4	Notifications	279

5.3.225	EP_N34	279

5.3.225.1	Definition	279

5.3.225.2	Attributes	280

5.3.225.3	Attribute constraints	280

5.3.225.4	Notifications	280

5.3.226	AnLFFunction	280

5.3.226.1	Definition	280

5.3.226.2	Attributes	280

5.3.226.3	Attribute constraints	280

5.3.226.4	Notifications	280

5.3.227	DnnTsctsfInfoItem <<dataType>>	280

5.3.227.1	Definition	280

5.3.227.2	Attributes	281

5.3.227.3	Attribute constraints	281

5.3.227.4	Notifications	281

5.3.228	SliceExpiryInfo <<dataType>>	281

5.3.228.1	Definition	281

5.3.228.2	Attributes	281

5.3.228.3	Attribute constraints	281

5.3.228.4	Notifications	281

5.3.229	NfInfo <<dataType>>	281

5.3.229.1	Definition	281

5.3.229.2	Attributes	281

5.3.229.3	Attribute constraints	281

5.3.229.4	Notifications	281

5.3.230	PcscfInfo <<dataType>>	282

5.3.230.1	Definition	282

5.3.230.2	Attributes	282

5.3.230.3	Attribute constraints	282

5.3.230.4	Notifications	282

5.3.231	SatelliteBackhaulInfo <<dataType>>	282

5.3.231.1	Definition	282

5.3.231.2	Attributes	282

5.3.231.3	Attribute constraints	282

5.3.231.4	Notifications	282

5.3.232	NTNGlobalRanNodeID <<dataType>>	283

5.3.232.1	Definition	283

5.3.232.2	Attributes	283

5.3.232.3	Attribute constraints	283

5.3.232.4	Notifications	283

5.3.233	DnaiSatelliteMapping <<dataType>>	283

5.3.233.1	Definition	283

5.3.233.2	Attributes	283

5.3.233.3	Attribute constraints	283

5.3.233.4	Notifications	283

5.3.234	TrpInfo <<dataType>>	284

5.3.234.1	Definition	284

5.3.234.2	Attributes	284

5.3.234.3	Attribute constraints	284

5.3.234.4	Notifications	284

5.3.235	TrpMappingInfo <<dataType>>	284

5.3.235.1	Definition	284

5.3.235.2	Attributes	284

5.3.235.3	Attribute constraints	284

5.3.235.4	Notifications	284

5.3.236	NFProfile <<dataType>>	284

5.3.236.1	Definition	284

5.3.236.2	Attributes	285

5.3.236.3	Attribute constraints	285

5.3.236.4	Notifications	285

5.3.237	NFService <<dataType>>	285

5.3.237.1	Definition	285

5.3.237.2	Attributes	285

5.3.237.3	Attribute constraints	285

5.3.237.4	Notifications	285

5.4	Attribute definitions	286

5.4.1	Attribute properties	286

5.5	Common notifications	361

5.5.1	Alarm notifications	361

5.5.2	Configuration notifications	361

5.5.3	Threshold Crossing notifications	361

5A	Information model definitions for SBA support of IMS	362

5A.1	Imported information entities and local labels	362

5A.2	Class diagram	362

5A.2.1	Class diagram for SBA support of IMS	362

5A.2.1.1	Relationships	362

5A.2.1.2	Inheritance	363

5A.3	Class definitions	363

5A.3.1	EP_N5	363

5A.3.1.1	Definition	363

5A.3.1.2	Attributes	363

5A.3.2	EP_N70	364

5A.3.2.1	Definition	364

5A.3.2.2	Attributes	364

5A.3.3	EP_N71	364

5A.3.3.1	Definition	364

5A.3.3.2	Attributes	364

6	Information model definitions for network slice NRM	364

6.1	Imported information entities and local labels	364

6.2	Class diagram	365

6.2.1	Relationships	365

6.2.2	Inheritance	367

6.3	Class definitions	368

6.3.1	NetworkSlice	368

6.3.1.1	Definition	368

6.3.1.2	Attributes	368

6.3.1.3	Attribute constraints	368

6.3.1.4	Notifications	368

6.3.2	NetworkSliceSubnet	369

6.3.2.1	Definition	369

6.3.2.2	Attributes	369

6.3.2.3	Attribute constraints	370

6.3.2.4	Notifications	370

6.3.3	ServiceProfile <<dataType>>	370

6.3.3.1	Definition	370

6.3.3.2	Attributes	371

6.3.3.3	Attribute constraints	371

6.3.3.4	Notifications	371

6.3.4	SliceProfile <<dataType>>	372

6.3.4.1	Definition	372

6.3.4.2	Attributes	372

6.3.4.3	Attribute constraints	372

6.3.4.4	Notifications	372

6.3.5	NsInfo <<dataType>>	372

6.3.5.1	Definition	372

6.3.5.2	Attributes	372

6.3.5.3	Attribute constraints	372

6.3.5.4	Notifications	372

6.3.6	ServAttrCom <<dataType>>	373

6.3.6.1	Definition	373

6.3.6.2	Attributes	373

6.3.6.3	Attribute constraints	373

6.3.6.4	Notifications	373

6.3.7	DelayTolerance<<dataType>>	373

6.3.7.1	Definition	373

6.3.7.2	Attributes	373

6.3.7.3	Attribute constraints	373

6.3.7.4	Notifications	373

6.3.8	DeterministicComm <<dataType>>	374

6.3.8.1	Definition	374

6.3.8.2	Attributes	374

6.3.8.3	Attribute constraints	374

6.3.8.4	Notifications	374

6.3.9	XLThpt<<dataType>>	374

6.3.9.1	Definition	374

6.3.9.2	Attributes	374

6.3.9.3	Attribute constraints	374

6.3.9.4	Notifications	374

6.3.10	Void	375

6.3.11	MaxPktSize <<dataType>>	375

6.3.11.1	Definition	375

6.3.11.2	Attributes	375

6.3.11.3	Attribute constraints	375

6.3.11.4	Notifications	375

6.3.12	MaxNumberofPDUSessions <<dataType>>	375

6.3.12.1	Definition	375

6.3.12.2	Attributes	375

6.3.12.3	Attribute constraints	376

6.3.12.4	Notifications	376

6.3.13	Void	376

6.3.14	KPIMonitoring <<dataType>>	376

6.3.14.1	Definition	376

6.3.14.2	Attributes	376

6.3.14.3	Attribute constraints	376

6.3.14.4	Notifications	376

6.3.15	UserMgmtOpen<<dataType>>	376

6.3.15.1	Definition	376

6.3.15.2	Attributes	377

6.3.15.3	Attribute constraints	377

6.3.15.4	Notifications	377

6.3.16	V2XCommMode<<dataType>>	377

6.3.16.1	Definition	377

6.3.16.2	Attributes	377

6.3.16.3	Attribute constraints	377

6.3.16.4	Notifications	377

6.3.17	TermDensity<<dataType>>	377

6.3.17.1	Definition	377

6.3.17.2	Attributes	378

6.3.17.3	Attribute constraints	378

6.3.17.4	Notifications	378

6.3.18	EP_Transport	378

6.3.18.1	Definition	378

6.3.18.2	Attributes	378

6.3.18.3	Attribute constraints	378

6.3.18.4	Notifications	379

6.3.19	EP_Application <<ProxyClass>>	379

6.3.19.1	Definition	379

6.3.19.2	Attributes	379

6.3.19.3	Attribute constraints	379

6.3.19.4	Notifications	379

6.3.20	NBIoT <<dataType>>	379

6.3.20.1	Definition	379

6.3.20.2	Attributes	379

6.3.20.3	Attribute constraints	379

6.3.20.4	Notifications	379

6.3.21	Void	380

6.3.22	Void	380

6.3.23	CNSliceSubnetProfile<<dataType>>	380

6.3.23.1	Definition	380

6.3.23.2	Attributes	380

6.3.23.3	Attribute constraints	380

6.3.23.4	Notifications	380

6.3.24	RANSliceSubnetProfile<<dataType>>	380

6.3.24.1	Definition	380

6.3.24.2	Attributes	381

6.3.24.3	Attribute constraints	381

6.3.24.4	Notifications	381

6.3.25	TopSliceSubnetProfile<<dataType>>	381

6.3.25.1	Definition	381

6.3.25.2	Attributes	382

6.3.25.3	Attribute constraints	382

6.3.25.4	Notifications	382

6.3.26	Positioning <<dataType>>	382

6.3.26.1	Definition	382

6.3.26.2	Attributes	383

6.3.26.3	Attribute constraints	383

6.3.26.4	Notifications	383

6.3.27	Synchronicity <<dataType>>	383

6.3.27.1	Definition	383

6.3.27.2	Attributes	383

6.3.27.3	Attribute constraints	383

6.3.27.4	Notifications	383

6.3.28	Void	384

6.3.29	Void	384

6.3.30	EnergyEfficiency <<dataType>>	384

6.3.30.1	Definition	384

6.3.30.2	Attributes	384

6.3.30.3	Attribute constraints	384

6.3.30.4	Notifications	384

6.3.31	RadioSpectrum <<dataType>>	384

6.3.31.1	Definition	384

6.3.31.2	Attributes	384

6.3.31.3	Attribute constraints	384

6.3.31.4	Notifications	385

6.3.32	N6Protection <<dataType>>	385

6.3.32.1	Definition	385

6.3.32.2	Attributes	385

6.3.32.3	Attribute constraints	385

6.3.32.4	Notifications	385

6.3.33	SecFunc <<dataType>>	385

6.3.33.1	Definition	385

6.3.33.2	Attributes	385

6.3.33.3	Attribute constraints	385

6.3.33.4	Notifications	385

6.3.34	NSSAASupport <<dataType>>	386

6.3.34.1	Definition	386

6.3.34.2	Attributes	386

6.3.34.3	Attribute constraints	386

6.3.34.4	Notifications	386

6.3.35	LogicalInterfaceInfo <<dataType>>	386

6.3.35.1	Definition	386

6.3.35.2	Attributes	386

6.3.35.3	Attribute constraints	386

6.3.35.4	Notifications	386

6.3.36	NetworkSliceSubnetProviderCapabilities	387

6.3.36.1	Definition	387

6.3.36.2	Attributes	387

6.3.36.3	Attribute constraints	387

6.3.36.4	Notifications	387

6.3.37	FeasibilityCheckAndReservationJob	387

6.3.37.1	Definition	387

6.3.37.2	Attributes	388

6.3.37.3	Attribute constraints	389

6.3.37.4	Notifications	389

6.3.38	MaxNumberofUEs <<dataType>>	389

6.3.38.1	Definition	389

6.3.38.2	Attributes	389

6.3.38.3	Attribute constraints	389

6.3.38.4	Notifications	389

6.3.39	NetworkSliceController	390

6.3.39.1	Definition	390

6.3.39.2	Attributes	390

6.3.39.3	Attribute constraints	391

6.3.39.4	Notifications	391

6.3.40	NetworkSliceSubnetController	391

6.3.40.1	Definition	391

6.3.40.2	Attributes	391

6.3.41	ConnectionPointInfo <<datatype>>	392

6.3.41.1	Definition	392

6.3.41.2	Attributes	392

6.3.41.3	Attribute constraints	392

6.3.41.4	Notifications	392

6.3.42	NonIPSupport<<dataType>>	392

6.3.42.1	Definition	392

6.3.42.2	Attributes	392

6.3.42.3	Attribute constraints	393

6.3.42.4	Notifications	393

6.3.43	IsolationProfile	393

6.3.43.1	Definition	393

6.3.43.2	Attributes	393

6.3.43.3	Attribute constraints	393

6.3.43.4	Notifications	393

6.3.44	ResourceIsolationRule <<dataType>>	393

6.3.44.1	Definition	393

6.3.44.2	Attributes	394

6.3.44.3	Attribute constraints	394

6.3.44.4	Notifications	394

6.3.45	DataNetwork <<dataType>>	394

6.3.45.1	Definition	394

6.3.45.2	Attributes	394

6.3.45.3	Attribute constraints	394

6.3.45.4	Notifications	394

6.3.46	DataNetworkAccess <<dataType>>	395

6.3.46.1	Definition	395

6.3.46.2	Attributes	395

6.3.46.3	Attribute constraints	395

6.3.46.4	Notifications	395

6.3.47	DataAccess <<dataType>>	395

6.3.47.1	Definition	395

6.3.47.2	Attributes	395

6.3.47.3	Attribute constraints	395

6.3.47.4	Notifications	395

6.3.48	TunnellingMechanism <<dataType>>	396

6.3.48.1	Definition	396

6.3.48.2	Attributes	396

6.3.48.3	Attribute constraints	396

6.3.48.4	Notifications	396

6.3.49	LboAllowed <<dataType>>	396

6.3.49.1	Definition	396

6.3.49.2	Attributes	396

6.3.49.3	Attribute constraints	396

6.3.49.4	Notifications	396

6.4	Attribute definition	397

6.4.1	Attribute properties	397

6.5	Common notifications	417

6.5.1	Alarm notifications	417

6.5.2	Configuration notifications	418

6.5.3	Threshold Crossing notifications	418

7	Solution Set (SS)	418

7.1	OpenAPI Definitions of NR and NG-RAN NRM	418

7.2	OpenAPI Definitions of 5GC NRM	419

7.3	OpenAPI Definitions of slice and network slice subnet NRM	419

7.4	YANG Definitions for NR and NG-RAN	419

7.5	YANG Definitions for 5GC	420

7.6	YANG Definitions for slice and network slice subnet	422

Annex A (normative):	Cell state handling	423

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	423

A.2	Combined state diagram for gNB cell	423

Annex B (normative):	NetworkSlice and NetworkSliceSubnet state handling	426

B.1	NetworkSlice instance state handling	426

B.2	State handling of NetworkSliceSubnet instance	427

Annex C (normative):	Void	429

Annex D (normative):	Void	430

Annex E (normative):	Void	431

Annex F (normative):	Void	432

Annex G (normative):	Void	433

Annex H (normative):	Void	433

Annex I (normative):	Void	434

Annex J (normative):	Void	435

Annex K (normative):	Void	436

Annex L (normative):	Relation of GSMA GST, ServiceProfile and SliceProfile	437

L.1	General	437

L.2	GSMA GST, ServiceProfile and sliceProfile	437

Annex M (normative):	Managed NF Service state handling	439

M.1	Combined state diagram for a Managed NF Service	439

Annex N (normative):	Void	441

Annex O (informative):	 QoS model usage for NG-RAN	442

O.1	Overview	442

O.2	General usage	442

O.3	NG-RAN MOCN network sharing usage recommendation	443

Annex P (informative):	 Mapping between GSMA GST and ServiceProfile	444

Annex Q (informative):	PlantUML source code	447

Q.1	5GC NRM relationship diagram	447

Q.1.1	The 5GC NF NRM containment/naming relationship	447

Q.1.2	The Figure 5.2.1.1-2 for the transport view of AMF NRM	448

Q.1.3	The Figure 5.2.1.1-3 for the transport view of SMF NRM	449

Q.1.4	The Figure 4.2.1.1-23: NRM fragment to support energy saving based on Energy Cost Reporting	450

Q.1.5	(Void)	451

Q.1.6	(Void)	451

Q.1.7	Figure 5.2.1.1-X Transport view of MME NRM	451

Q.2	5GC NRM inheritance diagram	451

Q.2.1	The inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM	451

Q.2.2	Figure 5.2.1.2-2 the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM	452

Q.3 NetworkSlice NRM relationship diagram	454

Q.3.1	The NetworkSlice NRM containment/naming relationship	454

Q.3.2	The Transport EP NRM containment/naming relationship	455

Q.3.3	The NetworkSlice NRM containment/naming relationship	456

Q.3.4	The feasibility check and resource reservation containment/naming relationship	456

Q.3.5	The IsolationProfile NRM containment/naming relationship	457

Q.3.6	The IsolationPofile to NetworkSlice relationship	457

Q.3.7	The IsolationPofile to NetworkSliceSubnet relationship	458

Q.4	NetworkSlice inheritance diagram	458

Q.4.1	Network slice inheritance diagram	458

Q.4.2	Feasibility check inheritance diagram	459

Q.4.3	NetworkSliceController inheritance diagram	459

Annex R (informative):	Change history	461

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	Void

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	Void

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

[49]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[50]	GSMA NG.116 - Generic Network Slice Template Version 9.0 (2023-04-27).

[51]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[52]	3GPP TS 33.501: "Security architecture and procedures for the 5G System".

[53]	3GPP TS 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz ".

[54]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[55]	3GPP TS 38.215: "NR; Physical layer measurements".

[56]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[57]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[58]	3GPP TS 38.423: "NR; Xn application protocol (XnAP)".

[59]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[60]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[63]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[64]	IEEE 802.3-2015: "IEEE Standard for Ethernet".

[65]	IEEE 802.1Q-2014: "Bridges and Bridged Networks".

[66]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[67]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[68]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[69]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[70]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[71]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[72]	3GPP TS 28.705: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[73]	3GPP TS 23.304: " Proximity based Services (ProSe) in the 5G System".

[74]	IETF RFC 8436: " Update to IANA Registration Procedures for Pool 3 Values in the Differentiated Services Field Codepoints (DSCP) Registry".

[75]	ECMA-262: "ECMAScript® Language Specification", https://www.ecma-international.org/ecma-262/5.1/.

[76]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[77]	IANA: "SMI Network Management Private Enterprise Codes", http://www.iana.org/assignments/enterprise-numbers.

[78]	3GPP TS 23.548:" 5G System Enhancements for Edge Computing; Stage 2".

[79]	3GPP TS 28.538: "Edge Computing Management".

[80]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[81]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[82]	IETF RFC 5952: "A recommendation for IPv6 address text representation".

[83]	Void

[84]	Void

[85]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[86]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[87]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[88]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[89]	IETF RFC 8345: "A YANG Data Model for Network Topologies".

[90]	YANG Data Models for 'Attachment Circuits'-as-a-Service (ACaaS) https://datatracker.ietf.org/doc/draft-boro-opsawg-teas-attachment-circuit/.

[91]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[92]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[93]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS);Stage 2".

[94]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[95]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[96]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[97]	3GPP TS 29.503: "Unified Data Management Services".

[98]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[99]	Void.

[100]	3GPP TS 23.540: "5G System: Technical realization of Service Based Short Message Service; Stage 2".

[101]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[102] 	IETF RFC 3393: "IP Packet Delay Variation Metric for IP Performance Metrics (IPPM)".

[103] 	IETF RFC 5481: "Packet Delay Variation Applicability Statement".

[104]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration"

[105]	3GPP TS 28.105: " Artificial Intelligence / Machine Learning (AI/ML) management ".

[106]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[107]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[108]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[109]	3GPP TS 28.111: " Management and orchestration; Fault Management (FM)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 28.540 [10].

## 3.2	Symbols

void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 38.401 [4], TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] , TS 23.501 [2], TS 38.401 [4] and TS 28.540 [10].

BWP	Bandwidth part

CHO	Conditional Handover

CM	Configuration Management

DAPS	Dual Active Protocol Stack

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

POP	Participating Operator

RIM	Remote interference management

RIM-RS	Remote interference management reference signal

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function

# 4	Information model definitions for NR NRM

## 4.1	Imported and associated information

### 4.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.662 [11], IOC, SectorEquipmentFunction | SectorEquipmentFunction |
| TS 28.658 [19], IOC, ExternalENBFunction | ExternalENBFunction |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.658 [19], IOC, EUtranCellFDD | EUtranCellFDD |
| TS 28.658 [19], IOC, EUtranCellTDD | EUtranCellTDD |
| TS 28.658 [19], dataType, PLMNId | PLMNId |
| TS 28.658 [19], IOC, ENBFunction | ENBFunction |
| TS 28.708 [21], IOC, ExternalServingGWFunction | ExternalServingGWFunction |
| TS 28.658 [19], IOC, ExternalEUtranCellFDD | ExternalEUtranCellFDD |
| TS 28.658 [19], IOC, ExternalEUtranCellTDD | ExternalEUtranCellTDD |
| TS 28.658 [19], IOC, AdjacentCell | AdjacentEUtranCell |
| TS 28.658 [19], IOC, EUtranFrequency | EUtranFrequency |
| TS 28.658 [19], IOC, EUtranFreqRelation | EUtranFreqRelation |
| TS 28.658 [19], IOC, EUtranRelation | EUtranCellRelation |
| TS 28.622 [30], dataType, Tai | Tai |
| TS 28.622 [30], choice, NpnId | NpnId |

### 4.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.105 [105], IOC, MLModel | MLModel |
| TS 28.105 [105], IOC, AIMLInferenceFunction | AIMLInferenceFunction |

## 4.2	Class diagram

### 4.2.1	Class diagram for gNB and en-gNB

#### 4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment scenario, represents the gNB defined in TS 38.401[4]. In this scenario, a gNB is represented by a combination of a GNBCUCPFunction, one or more GNBCUUPFunctions and one or more GNBDUFunctions.

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1. In this scenario, a gNB-CU is represented by a combination of a GNBCUCPFunction and one or more GNBCUUPFunctions, whereas a gNB-DU is represented by a GNBDUFunction.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2. In this scenario, a gNB-CU-CP is represented by a GNBCUCPFunction, a gNB-CU-UP is represented by a GNBCUUPFunction, and a gNB-DU is represented by a GNBDUFunction.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier,<<IOC>>BWP, <<IOC>>SectorEquipmentFunction and <<IOC>> BWPSet for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

NOTE 1:	The above NRM fragment uses SubNetwork to hold both NR and LTE external entities and frequencies.

Figure 4.2.1.1-5: Cell Relation view for all deployment scenarios

NOTE 2:	The above NRM fragment uses ExternalNRNetwork to hold NR external entities and frequency and using ExternalEUtraNetwork to hold LTE external entities and frequency.

Figure 4.2.1.1-6: NRM fragment for abstract RRM Policies

Figure 4.2.1.1-6a: NRM fragment for RRMPolicyRatio

Figure 4.2.1.1-7: NRM fragment to support RIM

Figure 4.2.1.1-8: NRM fragment for pre-configured 5QIs in NG-RAN

NOTE 3: Void

Figure 4.2.1.1-9: NRM fragment for DANR Management

Figure 4.2.1.1-10: NRM fragment for DES Management

Figure 4.2.1.1-11: NRM fragment for DRACH Management

Figure 4.2.1.1-12: NRM fragment for DMRO Management

Figure 4.2.1.1-13: NRM fragment for DPCI Management

Figure 4.2.1.1-14: NRM fragment for CES Management

Figure 4.2.1.1-15: NRM fragment for CPCI Management

Figure 4.2.1.1-16: NRM fragment for dynamically assigned 5QIs in NG-RAN

Figure 4.2.1.1-17: NRM fragment for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

Figure 4.2.1.1-18: NRM fragment for F1 related EPs to support individual F1 interface for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

Figure 4.2.1.1-19: NRM fragment for DLBO Management

Figure 4.2.1.1-20: NRM fragment for CCO Management

Figure 4.2.1.1-21: NRM fragment to support NTN management

Figure 4.2.1.1-22: NRM fragment for ExternalGNBCUCPFunction, ExternalGNBCUUPFunction and ExternalGNBDUFunction

Figure 4.2.1.1-23: NRM fragment for energy cost mapping rule

#### 4.2.1.2	Inheritance

This clause depicts the inheritance relationships.

Figure 4.2.1.2-1: NR NRM fragment in all deployment scenarios

Figure 4.2.1.2-2: NRM fragment for EPs in all deployment scenarios

Figure 4.2.1.2-3: NRM fragment for NRNetwork and EUtraNetwork

Figure 4.2.1.2-4: NRM fragment for Beam, CommonBeamformingFunction

Figure 4.2.1.2-5: NRM fragment for RRM Policies

Figure 4.2.1.2-6: NRM fragment for NRFreqRelation, NRFrequency and NRCellRelation

Figure 4.2.1.2-7: NRM fragment for C-SON, D-SON

Figure 4.2.1.2-8: NRM fragment to support RIM

Figure 4.2.1.2-9:  NRM fragment for OperatorDU and NROperatorCellDU

Figure 4.2.1.2-10: NRM fragment for CCO Management

Figure 4.2.1.2-11: NRM fragment for NTN management

Figure 4.2.1.2-12: NRM fragment for energy cost mapping rule

## 4.3	Class definitions

### 4.3.1	GNBDUFunction

#### 4.3.1.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBCUUPFunction IOC provide the management of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split and 3-split NG-RAN architecture, this IOC provides the management representation of gNB-DU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

#### 4.3.1.2	Attributes

The GNBDUFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBDUId | M | T | T | F | T |
| gNBDUName | O | T | T | F | T |
| gNBId | CM | T | T | F | T |
| gNBIdLength | CM | T | T | F | T |
| rimRSReportConf | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | CO | T | T | F | T |
| dynamic5QISetRef | CO | T | F | F | T |

#### 4.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| gNBId S | Condition: NG-RAN MOCN network sharing with multiple Cell Identity broadcast feature is not supported |
| gNBIdLength S | Condition: NG-RAN MOCN network sharing with multiple Cell Identity broadcast feature is not supported |
| configurable5QISetRef S | Condition: Configurable5QISet is name contained by SubNetwork or ManagedElement. |
| dynamic5QISetRef S | Condition: Dynamic5QISet is name contained by SubNetwork or ManagedElement. |

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.2	GNBCUCPFunction

#### 4.3.2.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC provide management representation of the gNB-CU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN deployment scenario, this IOC provides management representation of gNB-CU-CP defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C. | <<IOC>>EP_XnC, <<IOC>>EP_NgC. |
| en-gNB | <<IOC>>EP_X2C, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_X2C, <<IOC>>EP_F1C. | <<IOC>>EP_X2C. |

#### 4.3.2.2	Attributes

The GNBCUCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | O | T | T | F | T |
| pLMNId | M | T | T | T | T |
| x2BlockList | CM | T | T | F | T |
| x2AllowList | CM | T | T | F | T |
| xnBlockList | M | T | T | F | T |
| xnAllowList | M | T | T | F | T |
| x2HOBlockList | CM | T | T | F | T |
| xnHOBlockList | M | T | T | F | T |
| mappingSetIDBackhaulAddressList | CM | T | T | F | T |
| tceIDMappingInfoList | CM | T | T | F | T |
| dDAPSHOControl | CM | T | T | F | T |
| dCHOControl | CM | T | T | F | T |
| qceIdMappingInfoList | CM | T | T | F | T |
| mdtUserConsentReqList | CM | T | T | F | T |
| mappedCellIdInfoList | CO | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | CO | T | T | F | T |
| dynamic5QISetRef | CO | T | F | F | T |
| ephemerisInfoSetRef | CO | T | F | F | T |
| nRECMappingRuleRef | CM | T | T | F | T |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| x2BlockList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| x2AllowList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| x2HOBlockList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| mappingSetIDBackhaulAddressList | Condition: Remote Interference Management function is supported. |
| tceIDMappingInfolist | Condition: MDT Function is supported. |
| dDAPSHOControl | Condition: DAPS is supported. |
| dCHOControl | Condition: CHO is supported. |
| mappedCellIdInfoList | Condition: NTN is supported. |
| configurable5QISetRef | Condition: Configurable5QISet is name contained by SubNetwork or ManagedElement |
| dynamic5QISetRef | Condition: Dynamic5QISet is name contained by SubNetwork or ManagedElement |
| ephemerisInfoSetRef | Condition: NTN is supported. |
| qceIdMappingInfoList | Condition: QMC Function is supported. |
| mdtUserConsentReqList | Condition: MDT Function is supported. |
| nRECMappingRuleRef | Condition: Energy cost reporting is supported |

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.3	GNBCUUPFunction

#### 4.3.3.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC provide management representation of gNB-CU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN deployment scenario, this IOC provides management representation of gNB-CU-UP defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U. | <<IOC>>EP_XnU, <<IOC>>EP_NgU. |
| en-gNB | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U. | <<IOC>>EP_X2U, <<IOC>>EP_S1U. |

#### 4.3.3.2	Attributes

The GNBCUUPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBCUUPId | M | T | F | T | T |
| pLMNInfoList | M | T | T | F | T |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | CO | T | T | F | T |
| dynamic5QISetRef | CO | T | F | F | T |

#### 4.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configurable5QISetRef S | Condition: Configurable5QISet is name contained by SubNetwork or ManagedElement |
| dynamic5QISetRef S | Condition: Dynamic5QISet is name contained by SubNetwork or ManagedElement |

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	NRCellCU

#### 4.3.4.1	Definition

This IOC represents the part of NR cell information that is responsible for the management of inter-cell mobility and neighbour relations via ANR.

#### 4.3.4.2	Attributes

The NRCellCU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T (Note) | F | T |
| Attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | F | F | T |
| NOTE: Whether the attribute "pLMNId" in the PLMNInfo can be writable depends on the implementation. |  |  |  |  |  |

Note 1:	 Void.

Note 2:	Void.

#### 4.3.4.3	Void

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.5	NRCellDU

#### 4.3.5.1	Definition

This IOC represents the part of NR cell information that describes the specific resources instances.

An NR cell transmits SS/PBCH block and always requires downlink transmission at a certain carrier frequency with a certain channel bandwidth. Transmission may be performed from multiple sector-carriers using different transmission points, and these may be configured with different carrier frequencies and channel bandwidths, as long as they are aligned to the cell's downlink resource grids as defined in subclause 4.4 in TS 38.211 [32]. The values of arfcnDL and bSChannelBwDL attributes define the resource grids which each sector-carrier needs to be aligned to. See subclauses 5.3 and 5.4.2 of TS 38.104 [12] for definitions of BS channel bandwidth and NR-ARFCN, respectively.

An NR cell requires an uplink in order to provide initial access. In case of TDD, the values of arfcnUL and bSChannelBwUL have to always be set to the same values as for the corresponding DL attributes. For both FDD and TDD, the arfcnUL and bSChannelBwUL define uplink resource grids to which each sector-carrier needs to align to.

An NR cell can in addition be configured with a supplementary uplink, which has its own arfcnSUL and bSChannelBwSUL, which define resource grids for supplementary uplink sector-carriers.

Each of downlink, uplink and supplementary uplink (if configured) need an initial bandwidth part (BWP), which defines resources to be used by UEs during and immediately after initial access. Additional BWPs can be either configured or calculated by gNB internally and be applied to UEs dynamically by gNB based on e.g. UE capability and bandwidth need of each UE.

BWPs are configured individually (bWPRef) or via sets (bWPSetRef).

NOTE: Void

#### 4.3.5.2	Attributes

The NRCellDU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | CM | T | T | F | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| cellState | M | T | F | F | T |
| pLMNInfoList | CM | T | T | F | T |
| nPNIdentityList | CM | T | T | F | T |
| nRPCI | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| rimRSMonitoringStartTime | O | T | T | F | T |
| rimRSMonitoringStopTime | O | T | T | F | T |
| rimRSMonitoringWindowDuration | O | T | T | F | T |
| rimRSMonitoringWindowStartingOffset | O | T | T | F | T |
| rimRSMonitoringWindowPeriodicity | O | T | T | F | T |
| rimRSMonitoringOccasionInterval | O | T | T | F | T |
| rimRSMonitoringOccasionStartingOffset | O | T | T | F | T |
| ssbFrequency | CM | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | CM | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrierRef | M | T | T | F | T |
| nRFrequencyRef | CO | T | T | F | T |
| choice |  |  |  |  |  |
| > bWPRef | CM | T | T | F | T |
| > bWPSetRef | CM | T | T | F | T |
| victimSetRef | CM | T | T | F | T |
| aggressorSetRef | O | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: Void |  |  |  |  |  |

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cellLocalId S | Condition: NG-RAN MOCN network sharing with multiple Cell Identity broadcast feature is not supported |
| pLMNInfoList S | Condition: NG-RAN MOCN network sharing with multiple Cell Identity broadcast feature is not supported |
| nRTAC S | Condition: NG-RAN MOCN network sharing with multiple Cell Identity broadcast feature is not supported |
| bSChannelBwUL S | Condition: The cell has an uplink (FDD or TDD) |
| bSChannelBwSUL S | Condition: The cell has a supplementary uplink |
| nRFrequencyRef S | Condition: Non-split deployment scenario is supported |
| ssbFrequency S | Condition: nRFrequencyRef is not used. |
| ssbSubCarrierSpacing S | Condition: nRFrequencyRef is not used. |
| victimSetRef S | Condition: RIM feature is supported |
| bWPRef S | Condition: BWP sets are not supported. |
| bWPSetRef S | Condition: BWP sets are supported. |

#### 4.3.5.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.6	NRSectorCarrier

#### 4.3.6.1	Definition

This <<IOC>>NRSectorCarrier represents the resources of each transmission point associated to corresponding cell(s). These in general have different physical locations (of the antennae), and possibly different frequencies or bandwidths. The UE is not directly aware of which NRSectorCarrier resources the network uses for its connection.

An NR sector-carrier can have downlink, uplink or both as specified by txDirection. Attributes related to unavailable direction (DL or UL) shall not be set.

Additional NRSectorCarriers not directly associated to one cell only can also be configured.

If a value of arfcnDL, arfcnUL, bSChannelBwDL or bSChannelBwUL can be derived unambiguously from the referring cell, then that attribute needs not be present. That will not be possible if the NRSectorCarrier is used for supplementary uplink, if it is not directly associated to a cell, or if the sector-carrier uses only a part of the cell's channel bandwidth. Thus, at least in those cases the applicable attributes have to be present and their values need to be set.

#### 4.3.6.2	Attributes

The NRSectorCarrier IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| txDirection | M | T | T | F | T |
| configuredMaxTxPower | CM | T | T | F | T |
| configuredMaxTxEIRP | CM | T | T | F | T |
| arfcnDL | CM | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| bSChannelBwDL | CM | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| sectorEquipmentFunctionRef | M | T | T | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configuredMaxTxPower | Condition: The sector-carrier has a downlink. Configuration of Tx power at antenna port reference point is supported. |
| configuredMaxTxEIRP | Condition: The sector-carrier has a downlink. Configuration of emitted isotropic radiated power is supported. |
| arfcnDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of arfcnDL. |
| arfcnUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of arfcnUL. |
| bSChannelBwDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of bSChannelBwDL. |
| bSChannelBwUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of bSChannelBwUL. |

#### 4.3.6.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.7	BWP

#### 4.3.7.1	Definition

This IOC represents a bandwidth part (BWP) defined in 3GPP TS 38.211 [32], subclause 4.4.5. A bandwidth part is related to downlink, uplink or supplementary uplink resource grids, and is defined by its subcarrier spacing (SCS), cyclic prefix and location and size related to the common resource grid for the applicable SCS.

A BWP can be either an initial BWP used for initial access, or other ("regular") BWP configured for relevant UEs that support the BWP's characteristics.

#### 4.3.7.2	Attributes

The BWP IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bwpContext | M | T | T | F | T |
| isInitialBwp | M | T | T | F | T |
| subCarrierSpacing | M | T | T | F | T |
| cyclicPrefix | M | T | T | F | T |
| startRB | M | T | T | F | T |
| numberOfRBs | M | T | T | F | T |

#### 4.3.7.3	Attribute constraints

None.

#### 4.3.7.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.8	EP_E1

#### 4.3.8.1	Definition

This IOC represents the local end point of the logical link, supporting E1 interface between gNB-CU-CP and gNB-CU-UP. The E1 interface is defined in 3GPP TS 38.401 [4].

#### 4.3.8.2	Attributes

The EP_E1 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.8.3	Attribute constraints

None.

#### 4.3.8.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.9	EP_XnU

#### 4.3.9.1	Definition

This IOC represents the one end-point of a logical link supporting the Xn user plane (Xn-U) interface. The Xn-U interface provides non-guaranteed delivery of user plane PDUs between two NG-RAN nodes. The user plane PDUs are carried on GTP-U/UDP/IP/Data link layer/Physical layer stack. See subclause 7.2 of 3GPP TS 38.420 [6].

#### 4.3.9.2	Attributes

The EP_XnU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.9.3	Attribute constraints

None.

#### 4.3.9.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.10	EP_NgC

#### 4.3.10.1	Definition

This IOC represents the local end point of the control plane interface (NG-C) between the gNB and AMF. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol).

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.10.2	Attributes

The EP_NgC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.10.3	Attribute constraints

None.

#### 4.3.10.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.11	EP_NgU

#### 4.3.11.1	Definition

This IOC represents the local end point of the NG user plane (NG-U) interface between the gNB and UPF. The interface provides non-guaranteed delivery of user plane PDUs between the gNB and UPF. GTP-U is baseline for this interface.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.11.2	Attributes

The EP_NgU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 4.3.11.3	Attribute constraints

None.

#### 4.3.11.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.12	EP_F1C

#### 4.3.12.1	Definition

This IOC represents the local end point of the control plane interface (F1-C) between the gNB-DU and gNB-CU or gNB-CU-CP. The transport network layer is based on IP transport with the SCTP on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol). See subclause 7.1 of 3GPP TS 38.470 [7].

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.12.2	Attributes

The EP_F1C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.12.3	Attribute constraints

None.

#### 4.3.12.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.13	EP_F1U

#### 4.3.13.1	Definition

This IOC represents the local end point of the user plane interface (F1-U) between the gNB-DU and gNB-CU or gNB-CU-UP. The transport network layer is based on IP transport, with the UDP and GTP-U on top of IP.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.13.2	Attributes

The EP_F1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 4.3.13.3	Attribute constraints

None.

#### 4.3.13.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.14	EP_S1U

#### 4.3.14.1	Definition

This IOC represents the local end point of the logical link, supporting S1-U interface towards a S-GW node. The S1-U interface is defined in 3GPP TS 36.410 [14].

#### 4.3.14.2	Attributes

The EP_S1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.14.3	Attribute constraints

None.

#### 4.3.14.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.15	EP_X2C

#### 4.3.15.1	Definition

This IOC represents the local end point of the logical link, supporting X2-C application protocols used in EN-DC, to a neighbour eNB or en-gNB node, which is defined in 3GPP TS 36.423 [15]. EN-DC is defined in 3GPP TS 37.340 [9].

#### 4.3.15.2	Attributes

The EP_X2C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.15.3	Attribute constraints

None.

#### 4.3.15.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.16	EP_X2U

#### 4.3.16.1	Definition

This IOC represents the local end-point of a logical link supporting the X2 user plane (X2-U) interface used in EN-DC, which is defined in 3GPP TS 36.425 [16].

#### 4.3.16.2	Attributes

The EP_X2U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.16.3	Attribute constraints

None.

#### 4.3.16.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.17	EP_XnC

#### 4.3.17.1	Definition

This IOC represents the local gNB node end point of the logical link, supporting Xn Application protocols, to a neighbour NG-RAN node (including gNB and ng-eNB). The Xn Application PDUs are carried over SCTP/IP/Data link layer/Physical layer stack. See subclause 7 of 3GPP TS 38.420 [6].

#### 4.3.17.2	Attributes

The EP_XnC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.17.3	Attribute constraints

None

#### 4.3.17.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.18	ExternalGNBCUCPFunction

#### 4.3.18.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUCPFunction managed by another management function. For more information about GNBCUCPFunction, see subclause 4.3.2.

#### 4.3.18.2	Attributes

The ExternalGNBCUCPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | F | T |

#### 4.3.18.3	Attribute constraints

None.

#### 4.3.18.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.19	ExternalGNBCUUPFunction

#### 4.3.19.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUUPFunction managed by another management function. For more information about GNBCUUPFunction, see subclause 4.3.3.

#### 4.3.19.2	Attributes

The ExternalGNBCUUPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.20	ExternalGNBDUFunction

#### 4.3.20.1	Definition

This IOC represents the properties, known by the management function, of a GNBDUFunction managed by another management function. For more information about GNBDUFunction, see subclause 4.3.1.

#### 4.3.20.2	Attributes

The ExternalGNBDUFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | T | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.20.3	Attribute constraints

None.

#### 4.3.20.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.21	ExternalUPFFunction

#### 4.3.21.1	Definition

This IOC represents the properties, known by the management function, of a UPFFunction managed by another management function. For more information about UPFFunction, see subclause 5.3.3.

#### 4.3.21.2	Attributes

The ExternalUPFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.22	ExternalAMFFunction

#### 4.3.22.1	Definition

This IOC represents the properties, known by the management function, of an AMFFunction managed by another management function. For more information about AMFFunction, see subclause 5.3.

#### 4.3.22.2	Attributes

The ExternalAMFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.22.3	Attribute constraints

None

#### 4.3.22.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.23	Void

### 4.3.24	ENBFunction <<ProxyClass>>

#### 4.3.24.1	Definition

This IOC represents an <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.2	Attributes

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.3	Attribute constraints

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.4	Notifications

See respective IOCs.

### 4.3.25	GNBCUCPFunction <<ProxyClass>>

#### 4.3.25.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.3	Attribute constraints

See respective IOCs.

#### 4.3.25.4	Notifications

See respective IOCs.

### 4.3.26	GNBCUUPFunction <<ProxyClass>>

#### 4.3.26.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.4	Notifications

See respective IOCs.

### 4.3.27	GNBDUFunction <<ProxyClass>>

#### 4.3.27.1	Definition

This IOC represents an <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.2	Attributes

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.3	Attribute constraints

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.4	Notifications

See respective IOCs.

### 4.3.28	ServingGWFFunction <<ProxyClass>>

#### 4.3.28.1	Definition

This IOC represents an <<IOC>>ServingGWFFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.2	Attributes

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.3	Attribute constraints

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.4	Notifications

See respective IOCs.

### 4.3.29	UPFFunction <<ProxyClass>>

#### 4.3.29.1	Definition

This IOC represents an <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.2	Attributes

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.3	Attribute constraints

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.4	Notifications

See respective IOCs.

### 4.3.30	AMFFunction <<ProxyClass>>

#### 4.3.30.1	Definition

This IOC represents an <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.2	Attributes

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.3	Attribute constraints

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.4	Notifications

See respective IOCs.

### 4.3.31	Void

### 4.3.32	NRCellRelation

#### 4.3.32.1	Definition

This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance.

The source cell can be a NRCellCU instance. This is the case for an Intra-NR neighbour cell relation.

The source cell can be a EUtranGenericCell instance. This is the case for Inter-LTE-NR neighbour cell relation, from E-UTRAN to NR. See 3GPP TS 28.658 [19].

Neighbour cell relations are unidirectional.

#### 4.3.32.2	Attributes

The NRCellRelation IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nRTCI | O | T | T | F | T |
| cellIndividualOffset | M | T | T | F | T |
| isRemoveAllowed | CM | T | T | F | T |
| isHOAllowed | CM | T | T | F | T |
| isESCoveredBy | CM | T | T | F | T |
| isENDCAllowed | CM | T | T | F | T |
| isMLBAllowed | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| nRFreqRelationRef | M | T | T | F | T |
| adjacentNRCellRef | M | T | T | F | T |

#### 4.3.32.3	Attribute constraints

| Name | Definition |
| --- | --- |
| isRemoveAllowed | Condition: ANR function is supported in the source cell. |
| isHOAllowed | Condition: ANR function is supported in the source cell. |
| isESCoveredBy | Condition: Energy Saving function is supported. |
| isENDCAllowed | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| isMLBAllowed | Condition: MLB function is supported in the source cell. |

#### 4.3.32.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.33	NRFreqRelation

#### 4.3.33.1	Definition

This IOC, together with the target NRFrequency, represents the frequency properties applicable to the referencing NRCellRelation.

#### 4.3.33.2	Attributes

The NRFreqRelation IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| offsetMO | O | T | T | F | F |
| blockListEntry | O | T | T | F | F |
| blockListEntryIdleMode | O | T | T | F | F |
| cellReselectionPriority | O | T | T | F | F |
| cellReselectionSubPriority | O | T | T | F | F |
| pMax | O | T | T | F | F |
| qOffsetFreq | O | T | T | F | F |
| qQualMin | O | T | T | F | F |
| qRxLevMin | M | T | T | F | F |
| threshXHighP | M | T | T | F | F |
| threshXHighQ | CM | T | T | F | F |
| threshXLowP | M | T | T | F | F |
| threshXLowQ | CM | T | T | F | F |
| tReselectionNr | M | T | T | F | F |
| tReselectionNRSfHigh | O | T | T | F | F |
| tReselectionNRSfMedium | O | T | T | F | F |
| attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | T | F | F |

#### 4.3.33.3	Attribute constraints

| Name | Definition |
| --- | --- |
| threshXHighQ | Condition: RSRQ used in SIB4. |
| threshXLowQ | Condition: RSRQ used in SIB4. |

#### 4.3.33.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.34	Void

### 4.3.35	ExternalNRCellCU

#### 4.3.35.1	Definition

This abstract IOC represents the properties of an NRCellCU controlled by another Management Service Provider. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by Management Service Provider. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 4.3.35.2	Attributes

The ExternalNRCellCU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| plmnIdList | M | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | T | F | T |

#### 4.3.35.3	Attribute constraints

None.

#### 4.3.35.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.36	RRMPolicyRatio

#### 4.3.36.1	Definition

This IOC represents the properties of RRMPolicyRatio. RRMPolicyRatio is one realization of abstract RRMPolicy_ IOC. RRMPolicyRatio has three attributes, apart from those inherited (DN, resourceType, rRMPolicyMemberList).

Figure 4.3.36-1 Structure of RRMPolicyRatio

-	The attribute rRMPolicyMaxRatio defines the maximum resource usage quota for the associated rRMPolicyMemberList, including at least one of shared resources, prioritized resources and dedicated resources. For the same resource type, the sum of the ‘rRMPolicyMaxRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same ManagedEntity can be greater than 100.

-	The attribute rRMPolicyMinRatio defines the minimum resource usage quota for the associated RRMPolicyMemberList, including at least one of prioritized resources and dedicated resources, which means the resources quota that need to be guaranteed for use by the associated rRMPolicyMemberList. For the same resource type, the sum of the ‘rRMPolicyMinRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same ManagedEntity shall be less than or equal to 100.

-	The attribute rRMPolicyDedicatedRatio defines the dedicated resource usage quota for the RRMPolicyMemberList, including dedicated resources. For the same resource type, the sum of the ‘rRMPolicyDedicatedRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same ManagedEntity shall be less than or equal to 100.

The following are the definition for above mentioned three resource categories:

-	Shared resources: means the resources that are shared with other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The shared resources are not guaranteed for use by the associated rRMPolicyMemberList. The shared resources quota is represented by [rRMPolicyMaxRatio-rRMPolicyMinRatio].

-	Prioritized resources: means the resources are preferentially used by the associated RRMPolicyMemberList. These resources are guaranteed for use by the associated RRMPolicyMemberList when it needs to use them. When not used, these resources may be used by other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The prioritized resources quota is represented by [rRMPolicyMinRatio-rRMPolicyDedicatedRatio]

-	Dedicated resources: means the resources are dedicated for use by the associated RRMPolicyMemberList. These resources can not be shared even if the associated RRMPolicyMember does not use them. The Dedicated resources quota is represented by [rRMPolicyDedicatedRatio].

NOTE: The resources shown in Figure 4.3.36-1 could be reserved or allocated for user and bearer related requests, e.g. a user connection, a PDU session, etc., if the user is entitled to use the allocated resources according the ratios defined above.

#### 4.3.36.2	Attributes

The RRMPolicyRatio IOC includes attributes inherited from RRMPolicy_ IOC (defined in  clause 4.3.43) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rRMPolicyMaxRatio | M | T | T | F | T |
| rRMPolicyMinRatio | M | T | T | F | T |
| rRMPolicyDedicatedRatio | O | T | T | F | T |

#### 4.3.36.3	Attribute constraints

None

#### 4.3.36.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.37	S-NSSAI <<dataType>>

#### 4.3.37.1	Definition

This data type represents an S-NSSAI. An NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of an SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (See TS 23.003 [13]).

#### 4.3.37.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sST | M | T | T | F | T |
| sD | O | T | T | F | T |

#### 4.3.37.3	Attribute constraints

None

#### 4.3.37.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.38	NRFrequency

#### 4.3.38.1	Definition

This IOC represents certain NR frequency properties.

#### 4.3.38.2	Attributes

The NRFrequency IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| absoluteFrequencySSB | M | T | T | F | T |
| ssbSubCarrierSpacing | M | T | T | F | T |
| multiFrequencyBandListNR | O | T | F | F | T |

#### 4.3.38.3	Attribute constraints

None.

#### 4.3.38.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.39	CommonBeamformingFunction

#### 4.3.39.1	Definition

This <<IOC>>CommonBeamformingFunction represents common beamforming functionality (eg: SSB beams) for the NRSectorCarrier.

The CommonBeamformingFunction provides capability to configure the advanced antenna for a sector carrier. The configuration capability is provided by selection of coverageShape, digitalTilt and digitalAzimuth. These attributes represent the wanted coverage area and radiation pattern on a sector carrier related to an antenna transmission point.

This configuration capability assumes the system shall handle configuration of SSB beams within the sector carrier. Individual SSB beams within a sector carrier cannot be independently configured as this depends on many conditions and constraints, for instance TDD patterns, allocations of PRACH occasions, SIB1 and mobility considerations.

The associated <<IOC>> Beam provides information beam direction and beam width for the associated SSB beams as a result of the configuration. The beams addressed in this definition are the common beams.  There may be more than one beam per CommonBeamformingFunction for the NRSectorCarrier.

#### 4.3.39.2	Attributes

The CommonBeamformingFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageShape | M | T | T | F | T |
| digitalTilt | M | T | T | F | T |
| digitalAzimuth | M | T | T | F | T |

#### 4.3.39.3	Attribute constraints

None.

#### 4.3.39.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.40	Beam

#### 4.3.40.1	Definition

This <<IOC>>Beam represents the per-Beam information required for, e.g. beam performance management utilizing measurements generated in the RAN. TS 38.104 [12] relates to beam transmission, TS 38.215 [55] to beam measurements, and TS 38.331 [54] to reporting of those measurements and associated beam failure Information Elements, clauses 5.5.3, 5.5.5.2, 6.3.2. 6.2.2.

Measurements on common beams may be correlated with associated spatial beam information to assist use cases like troubleshooting performance problems, or SON functions like Coverage & Capacity Optimization.

<<IOC>>Beam can have spatial attributes of horizontal/azimuth (ie: Phi φ-axis) and vertical/tilt (ie: Theta θ-axis) beam pointing direction and beam width attributes.  There may be more than one beam per CommonBeamformingFunction for an NRSectorCarrier.  Informational note, beam direction and width are characteristics—a representation—of directional energy vectors.

#### 4.3.40.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| beamIndex | M | T | F | F | T |
| beamType | O | T | F | F | T |
| beamAzimuth | CM | T | F | F | T |
| beamTilt | CM | T | F | F | T |
| beamHorizWidth | CM | T | F | F | T |
| beamVertWidth | CM | T | F | F | T |

#### 4.3.40.3	Attribute constraints

The Beam IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Name | Definition |
| --- | --- |
| beamAzimuth S | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamTilt S | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamHorizWidth S | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamVertWidth S | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |

#### 4.3.40.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.41	PLMNInfo <<dataType>>

#### 4.3.41.1	Definition

This <<dataType>> represents the PLMN supported by the <<IOC>> using this <<dataType>> as one of its attributes. In case of network slicing feature is supported, this <<dataType>> also represents the S-NSSAI in the PLMN supported by the <<IOC>> using this <<dataType>> as one of its attributes.

#### 4.3.41.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 4.3.41.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI S | Condition: Network slicing feature is supported. |

#### 4.3.41.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.42	RRMPolicyMember <<dataType>>

#### 4.3.42.1	Definition

This <<dataType>> represents an RRM Policy member that will be part of a rRMPolicyMemberList. A RRMPolicyMember is defined by its pLMNId and sNSSAI (S-NSSAI). The members in a rRMPolicyMemberList is assigned a specific amount of RRM resources based on settings in RRMPolicy_.

#### 4.3.42.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 4.3.42.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI S | Condition: Network slicing is supported |

#### 4.3.42.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.43	RRMPolicy_

#### 4.3.43.1	Definition

This IOC represents the properties of an abstract RRMPolicy. The RRMPolicy_ IOC needs to be subclassed to be instantiated. It defines two attributes apart from those inherited from TOP IOC, the resourceType attribute defines type of resource (PRB, PRB in uplink, PRB in downlink,RRC connected users, DRB usage etc.) and the rRMPolicyMemberList attribute defines the RRMPolicyMember(s)that is subject to this policy. An RRM resource (defined in resourceType attribute) is located in NRCellDU, NRCellCU, GNBDUFunction, GNBCUCPFunction or in GNBCUUPFunction. The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC, see the inheritance in Figure 4.2.1.2-1. This RRM framework allows adding new policies, both standardized or as vendor specific, by inheriting from the abstract RRMPolicy_ IOC.

#### 4.3.43.2	Attributes

The RRMPolicy_ IOC have the following attributes, apart from those inherited from TOP IOC (defined in TS 28.622 [30]):

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| resourceType | M | T | T | F | T |
| rRMPolicyMemberList | M | T | T | F | T |

#### 4.3.43.3	Attribute constraints

None.

#### 4.3.43.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.44	RRMPolicyManagedEntity <<ProxyClass>>

#### 4.3.44.1	Definition

This represents an <<IOC>>NRCellCU, or an <<IOC>>NRCellDU or an <<IOC>>GNBCUUPFunction, or an <<IOC>>GNBCUCPFunction, or an <<IOC>>GNBDUFunction.

If <<IOC>>NRCellCU is used, which means that a RRMPolicy shall be applied to an RRM resource in the NRCellCU. The possible RRM resource(s) owned by NRCellCU is defined in the resourceType attribute.

If <<IOC>>NRCellDU is used, which means that a RRMPolicy shall be applied to an RRM resource in the NRCellDU. The possible RRM resource(s) owned by NRCellDU is defined in the resourceType attribute.

If <<IOC>>GNBCUUPFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBCUUPFunction. The possible RRM resource(s) owned by GNBCUUPFunction is defined in the resourceType attribute.

If <<IOC>>GNBCUCPFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBCUCPFunction. The possible RRM resource(s) owned by GNBCUCPFunction is defined in the resourceType attribute.

If <<IOC>>GNBDUFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBDUFunction. The possible RRM resource(s) owned by GNBDUFunction is defined in the resourceType attribute.

#### 4.3.44.2	Attributes

See that defined in <<IOC>>NRCellCU, <<IOC>>NRCellDU, <<IOC>>GNBCUUPFunction, <<IOC>>GNBCUCPFunction or <<IOC>>GNBDUFunction.

#### 4.3.44.3	Attribute constraints

See that defined in <<IOC>>NRCellCU, <<IOC>>NRCellDU, <<IOC>>GNBCUUPFunction, <<IOC>>GNBCUCPFunction, or <<IOC>>GNBDUFunction.

#### 4.3.44.4	Notifications

See respective IOCs.

### 4.3.45	GNBCUCPNeighbour <<ProxyClass>>

#### 4.3.45.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.3	Attribute constraints

See that defined in <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.4	Notifications

See respective IOCs.

### 4.3.46	GNBCUUPNeighbour <<ProxyClass>>

#### 4.3.46.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.4	Notifications

See respective IOCs.

### 4.3.47	MappingSetIDBackhaulAddress  <<dataType>>

#### 4.3.47.1	Definition

This data type represents the properties describing the mapping relationship between set ID and backhaul address of gNB.

#### 4.3.47.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setId | M | T | T | F | T |
| backhaulAddress | M | T | T | F | T |

#### 4.3.47.3	Attribute constraints

None.

#### 4.3.47.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.48	BackhaulAddress  <<dataType>>

#### 4.3.48.1	Definition

This data type represents the properties describing the backhaul address of gNB.

#### 4.3.48.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| tAI | M | T | T | F | T |

#### 4.3.48.3	Attribute constraints

None.

#### 4.3.48.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.49	Void

### 4.3.50	RimRSGlobal

#### 4.3.50.1	Definition

This IOC is used to represent global/common Remote Interference Management (RIM) Reference Signal (RS) resource allocated for the whole network. Resource for RIM-RS transmission is defined by Sequence domain resource, Time domain resource and Frequency resource. The configure parameters of the RIM RS resource are applied to all Sets of RIM RS Resource across gNBs/cells in the network.

#### 4.3.50.2	Attributes

The RimRSGlobal IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| frequencyDomainPara | M | T | T | F | T |
| sequenceDomainPara | M | T | T | F | T |
| timeDomainPara | M | T | T | F | T |

#### 4.3.50.3	Attribute constraints

None.

#### 4.3.50.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.51	FrequencyDomainPara <<dataType>>

#### 4.3.51.1	Definition

This data type defines configuration parameters of frequency domain resource to support RIM RS.

#### 4.3.51.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rimRSSubcarrierSpacing | M | T | T | F | T |
| rIMRSBandwidth | M | T | T | F | T |
| nrofGlobalRIMRSFrequencyCandidates | M | T | T | F | T |
| rimRSCommonCarrierReferencePoint | M | T | T | F | T |
| rimRSStartingFrequencyOffsetIdList | M | T | T | F | T |

#### 4.3.51.3	Attribute constraints

None.

#### 4.3.51.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.52	SequenceDomainPara  <<dataType>>

#### 4.3.52.1	Definition

This data type defines configuration parameters of sequence domain resource to support RIM RS.

#### 4.3.52.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nrofRIMRSSequenceCandidatesofRS1 | M | T | T | F | T |
| rimRSScrambleIdListofRS1 | M | T | T | F | T |
| nrofRIMRSSequenceCandidatesofRS2 | O | T | T | F | T |
| rimRSScrambleIdListofRS2 | O | T | T | F | T |
| enableEnoughNotEnoughIndication | M | T | T | F | T |
| rIMRSScrambleTimerMultiplier | M | T | T | F | T |
| rIMRSScrambleTimerOffset | M | T | T | F | T |

#### 4.3.52.3	Attribute constraints

None.

#### 4.3.52.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.53	TimeDomainPara  <<dataType>>

#### 4.3.53.1	Definition

This  data type defines configuration parameters of time domain resource to support RIM RS.

#### 4.3.53.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlULSwitchingPeriod1 | M | T | T | F | T |
| symbolOffsetOfReferencePoint1 | M | T | T | F | T |
| dlULSwitchingPeriod2 | O | T | T | F | T |
| symbolOffsetOfReferencePoint2 | O | T | T | F | T |
| totalnrofSetIdofRS1 | M | T | T | F | T |
| totalnrofSetIdofRS2 | O | T | T | F | T |
| nrofConsecutiveRIMRS1 | M | T | T | F | T |
| nrofConsecutiveRIMRS2 | O | T | T | F | T |
| consecutiveRIMRS1List | M | T | T | F | T |
| consecutiveRIMRS2List | M | T | T | F | T |
| enablenearfarIndicationRS1 | O | T | T | F | T |
| enablenearfarIndicationRS2 | O | T | T | F | T |

#### 4.3.53.3	Attribute constraints

None.

#### 4.3.53.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.54	RimRSReportConf <<dataType>>

#### 4.3.54.1	Definition

This data type defines RIM-RS reporting configuration.

#### 4.3.54.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportIndicator | M | T | T | F | T |
| reportInterval | M | T | T | F | T |
| nrofRIMRSReportInfo | M | T | T | F | T |
| maxPropagationDelay | O | T | T | F | T |
| rimRSReportInfoList | M | T | T | F | T |

#### 4.3.54.3	Attribute constraints

None.

#### 4.3.54.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.55	RimRSReportInfo <<dataType>>

#### 4.3.55.1	Definition

This  data type defines necessary reporting information derived from the detected RIM-RS, including

1)	The detected set ID;

2)	Propagation delay in number of OFDM symbols

3)	Functionality of the RS (RS-1 or RS-2, Enough or Not enough mitigation for RS-1).

NOTE:

RS-1 is equivalent to RIM-RS type 1 (see 38.211 [32], subclause 7.4.1.6).

RS-2 is equivalent to RIM-RS type 2 (see 38.211 [32], subclause 7.4.1.6).

Enough mitigation for RS-1 means "Enough" / "Not enough" indication functionality is enabled for RIM RS-1 and RIM-RS type 1 is used to indicate 'enough mitigation' functionality.

Not enough mitigation for RS-1 means "Enough" / "Not enough" indication functionality is enabled for RIM RS-1 and RIM-RS type 1 is used to indicate 'Not enough mitigation' functionality.

#### 4.3.55.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| detectedSetID | M | T | T | F | T |
| propagationDelay | O | T | T | F | T |
| functionalityOfRIMRS | M | T | T | F | T |

#### 4.3.55.3	Attribute constraints

None.

#### 4.3.55.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.56	RimRSSet

#### 4.3.56.1	Definition

This IOC is used to represent aggressor or victim Set organized by OAM. The RIM RS Resource is assigned to each Set, which is identified by triple indices set of <Time domain index, Frequency domain index, and  Sequence index >. The triple indices set can be derived by setId attribute (See subclause 7.4.1.6 in TS 38.211 [32]).

#### 4.3.56.2	Attributes

The RimRSSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following

attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setId | M | T | T | F | T |
| setType | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellDURef | M | T | F | F | T |

#### 4.3.56.3	Attribute constraints

None.

#### 4.3.56.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.57	DANRManagementFunction

#### 4.3.57.1	Definition

This IOC contains attributes to support the D-SON function of ANR Management (See clause 6.4.1.3 in TS 28.313 [57]).

#### 4.3.57.2	Attributes

The DANRManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intrasystemANRManagementSwitch | M | T | T | F | T |
| intersystemANRManagementSwitch | M | T | T | F | T |

#### 4.3.57.3	Attribute constraints

None.

#### 4.3.57.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.58	DESManagementFunction

#### 4.3.58.1	Definition

This IOC represents the management capabilities of Distributed Energy Saving (ES) functions. This is provided for Energy Saving purposes.

This Function can be implemented as SON (See clause 6.2.3.0 in TS 28.310 [71]) and/or AI/ML feature (See TS 28.105 [105]) .Attribute mLModelRefList indicates that AI/ML is supported for this function. Attribute aIMLInferenceFunctionRefList indicates that AI/ML Inference Function is supported for this function.

NOTE: in the case where multiple DESManagementFunction MOIs exist at different levels of the containment tree, the DESManagementFunction MOI at the lower level overrides the DESManagementFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.58.2	Attributes

The DESManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| desSwitch | M | T | T | F | T |
| intraRatEsActivationOriginalCellLoadParameters | CM | T | T | F | T |
| intraRatEsActivationCandidateCellsLoadParameters | CM | T | T | F | T |
| intraRatEsDeactivationCandidateCellsLoadParameters | CM | T | T | F | T |
| esNotAllowedTimePeriod | O | T | T | F | T |
| interRatEsActivationOriginalCellParameters | CM | T | T | F | T |
| interRatEsActivationCandidateCellParameters | CM | T | T | F | T |
| interRatEsDeactivationCandidateCellParameters | CM | T | T | F | T |
| energySavingState | M | T | F | F | T |
| isProbingCapable | O | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRefList | CM | T | F | F | T |
| aIMLInferenceFunctionRefList | CM | T | F | F | T |

#### 4.3.58.3	Attribute constraints

| Name | Definition |
| --- | --- |
| intraRatEsActivationOriginalCellLoadParameters S | The condition is " the cell acts as an original cell". |
| intraRatEsActivationCandidateCellsLoadParameters S | The condition is " the cell acts as a candidate cell". |
| intraRatEsDeactivationCandidateCellsLoadParameters S | The condition is " the cell acts as a candidate cell". |
| interRatEsActivationOriginalCellParameters CM S | The condition is "The cell acts as an original cell". |
| interRatEsActivationCandidateCellParameters CM S | The condition is "The cell acts as a candidate cell". |
| interRatEsDeactivationCandidateCellParameters CM S | The condition is "The cell acts as a candidate cell". |
| mLModelRefList | The condition is "MLModel is supported for this function". |
| aIMLInferenceFunctionRefList | The condition is "AIMLInferenceFunction is supported for this function ". |

#### 4.3.58.4	Notification

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.59	DRACHOptimizationFunction

#### 4.3.59.1	Definition

This IOC contains attributes to support the D-SON function of RACH optimization (See clause 7.1.1 in TS 28.313 [57]).

NOTE: in the case where multiple DRACHOptimizationFunction MOIs exist at different levels of the containment tree, the DRACHOptimizationFunction MOI at the lower level overrides the DRACHOptimizationFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.59.2	Attributes

The DRACHOptimizationFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ueAccProbabilityDist | M | T | T | F | T |
| ueAccDelayProbabilityDist | M | T | T | F | T |
| drachOptimizationControl | M | T | T | F | T |

#### 4.3.59.3	Attribute constraints

None.

#### 4.3.59.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.60	DMROFunction

#### 4.3.60.1	Definition

This IOC contains attributes to support Distributed MRO function.

This Function can be implemented as SON (See clause 7.1.2 in TS 28.313 [57]) and/or AI/ML feature (See TS 28.105 [105]) .Attribute mLModelRefList indicates that AI/ML is supported for this function. Attribute aIMLInferenceFunctionRefList indicates that AI/ML Inference Function is supported for this function.

NOTE	In the case where multiple DMROFunction MOIs exist at different levels of the containment tree, the DMROFunction MOI at the lower level overrides the DMROFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.60.2	Attributes

The DMROFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dmroControl | M | T | T | F | T |
| maximumDeviationHoTriggerLow | M | T | T | F | T |
| maximumDeviationHoTriggerHigh | M | T | T | F | T |
| minimumTimeBetweenHoTriggerChange | M | T | T | F | T |
| tstoreUEcntxt | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRefList | CM | T | F | F | T |
| aIMLInferenceFunctionRef | CM | T | F | F | T |

#### 4.3.60.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mLModelRefList | The condition is "MLModel is supported for this function". |
| aIMLInferenceFunctionRef | The condition is "AIMLInferenceFunction is supported for this function ". |

#### 4.3.60.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.61	DPCIConfigurationFunction

#### 4.3.61.1	Definition

This IOC contains attributes to support the Distributed SON function of PCI configuration (See clause 7.1.3 in TS 28.313 [57]).

NOTE: in the case where multiple DPCIConfigurationFunction MOIs exist at different levels of the containment tree, the DPCIConfigurationFunction MOI at the lower level overrides the DPCIConfigurationFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.61.2	Attributes

The DPCIConfigControlFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dPciConfigurationControl | M | T | T | F | T |
| nRPciList | M | T | T | F | T |

#### 4.3.61.3	Attribute constraints

None.

#### 4.3.61.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.62	CPCIConfigurationFunction

#### 4.3.62.1	Definition

This IOC contains attributes to support the Centralized SON function of PCI configuration (see clause 7.2.1 in TS 28.313 [57]).

NOTE:	In the case where multiple CPCIConfigurationFunction MOIs exist at different levels of the containment tree, the CPCIConfigurationFunction MOI at the lower level overrides the CPCIConfigurationFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.62.2	Attributes

The CPCIConfigurationFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cPciConfigurationControl | M | T | T | F | T |
| cSonPciList | M | T | T | F | T |

#### 4.3.62.3	Attribute constraints

None.

#### 4.3.62.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.63	CESManagementFunction

#### 4.3.63.1	Definition

This IOC represents the management capabilities of Centralized SON Energy Saving (ES) functions. (see clause 6.2.2 of TS 28.310 [71]) This is provided for Energy Saving purposes.

NOTE: in the case where multiple CESManagementFunction MOIs exist at different levels of the containment tree, the CESManagementFunction MOI at the lower level overrides the CESManagementFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.63.2	Attributes

The CESManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cesSwitch | M | T | T | F | T |
| energySavingControl | M | T | T | F | T |
| energySavingState | M | T | T | F | T |
| intraRatEsActivationOriginalCellLoadParameters | CM | T | T | F | T |
| intraRatEsActivationCandidateCellsLoadParameters | CM | T | T | F | T |
| intraRatEsDeactivationCandidateCellsLoadParameters | CM | T | T | F | T |
| esNotAllowedTimePeriod | O | T | T | F | T |
| interRatEsActivationOriginalCellParameters | CM | T | T | F | T |
| interRatEsActivationCandidateCellParameters | CM | T | T | F | T |
| interRatEsDeactivationCandidateCellParameters | CM | T | T | F | T |

#### 4.3.63.3	Attribute constraints

| Name | Definition |
| --- | --- |
| intraRatEsActivationOriginalCellLoadParameters S | The condition is "Intra-RAT domain centralized SON energy saving is supported AND the cell acts as an original cell". |
| intraRatEsActivationCandidateCellsLoadParameters S | The condition is "Intra-RAT domain centralized SON energy saving is supported AND the cell acts as a candidate cell". |
| intraRatEsDeactivationCandidateCellsLoadParameters S | The condition is "Intra-RAT domain centralized SON energy saving is supported AND the cell acts as a candidate cell". |
| interRatEsActivationOriginalCellParameters CM S | The condition is "The cell acts as an original cell" AND inter-RAT domain centralized SON energy saving is supported. |
| interRatEsActivationCandidateCellParameters CM S | The condition is "The cell acts as a candidate cell" AND inter-RAT domain centralized SON energy saving is supported. |
| interRatEsDeactivationCandidateCellParameters CM S | The condition is "The cell acts as a candidate cell" AND inter-RAT domain centralized SON energy saving is supported. |

#### 4.3.63.4	Notification

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.64	AddressWithVlan <<dataType>>

#### 4.3.64.1	Definition

This data type represents the address including IP address and VLAN Id (e.g. localAddress of EP_NgC) used for initialization of the underlying transport.

#### 4.3.64.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| iPAddress | O | T | T | F | T |
| vLANId | O | T | T | F | T |

#### 4.3.64.3	Attribute constraints

None

#### 4.3.64.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.65	TceIDMappingInfo  <<dataType>>

#### 4.3.65.1	Definition

This data type represents the properties describing the mapping relationship between TCE ID, PLMN where TCE resides and IP address of TCE.

#### 4.3.65.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tceIPAddress | M | T | T | F | T |
| tceID | M | T | T | F | T |
| pLMNTarget | M | T | T | F | T |

#### 4.3.65.3	Attribute constraints

None.

#### 4.3.65.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.66	Void

### 4.3.67	OperatorDU

#### 4.3.67.1	Definition

This IOC contains attributes to support operator specific gNB-DU level information to support the NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature. An instance of OperatorDU <<IOC>> should be created and configured for each POP. When configured the attributes override those in parent GNBDUFunction instance.

The OperatorDU <<IOC>> is only used to support NG-RAN MOCN with multiple cell identity broadcast feature. If NG-RAN MOCN with multiple cell identity broadcast feature is not supported, is not used.

The attributes configurable5QISetRef and dynamic5QISetRef are used to refer to 5QI set for each POP.

The following table identifies the necessary end points required for the representation of shared gNB and shared en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| Shared gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| Shared en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

For scenarios with an F1 interface supporting multiple PLMN broadcast, the values of the EP_F1C and EP_F1U attributes contained by different OperatorDU of the same GNBDUFunction should be same.

#### 4.3.67.2	Attributes

The OperatorDU IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | CO | T | T | F | T |
| dynamic5QISetRef | CO | T | F | F | T |

#### 4.3.67.3	Attribute Constraints

| Name | Definition |
| --- | --- |
| configurable5QISetRef | Condition: NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported. |
| dynamic5QISetRef | Condition: NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported. |

#### 4.3.67.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 4.3.68	NROperatorCellDU

#### 4.3.68.1	Definition

The NROperatorCellDU <<IOC>> contains attributes to support operator specific cell level information (including cellLocalId, pLMNInfoList, nRTAC) to support NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature. An instance of NROperatorCellDU <<IOC>> should be created and configured for each POP. When configured the attributes override those in the associated NRCellDU instance.

The NROperatorCellDU <<IOC>> is only used to support NG-RAN MOCN with multiple cell identity broadcast feature. If the NG-RAN MOCN with multiple cell identity broadcast feature is not supported, this IOC is not used.

#### 4.3.68.2	Attributes

The NROperatorCellDU IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| administrativeState | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellDURef | M | T | T | F | T |

#### 4.3.68.3	Attribute Constraints

| Name | Definition |
| --- | --- |
| nRTAC S | Condition: 5G SA (Stand Alone) is supported. |

#### 4.3.68.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 4.3.69	DLBOFunction

#### 4.3.69.1	Definition

This IOC contains attributes to support the Distributed function of LBO.

This Function can be implemented as SON (See TS 28.313 [57]) and/or AI/ML feature (See TS 28.105 [105]) .Attribute mLModelRefList indicates that AI/ML is supported for this function. Attribute aIMLInferenceFunctionRefList indicates that AI/ML Inference Function is supported for this function.

NOTE	In the case where multiple DLBOFunction MOIs exist at different levels of the containment tree, the DLBOFunction MOI at the lower level overrides the DLBOFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.69.2	Attributes

The DLBOFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlboControl | M | T | T | F | T |
| maximumDeviationHoTriggerLow | M | T | T | F | T |
| maximumDeviationHoTriggerHigh | M | T | T | F | T |
| minimumTimeBetweenHoTriggerChange | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRefList | CM | T | F | F | T |
| aIMLInferenceFunctionRefList | CM | T | F | F | T |

#### 4.3.69.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mLModelRefList | The condition is "MLModel is supported for this function". |
| aIMLInferenceFunctionRefList | The condition is "AIMLInferenceFunction is supported for this function ". |

#### 4.3.69.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.70	CCOFunction

#### 4.3.70.1	Definition

This IOC contains attributes to support the C-SON function of Capacity and Coverage optimization (See clause 7.2.3 in TS 28.313 [57]).

NOTE: in the case where multiple CCOFunction MOIs exist at different levels of the containment tree, the CCOFunction MOI at the lower level overrides the CCOFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.70.2	Attributes

The CCOFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCOControl | M | T | T | F | T |

#### 4.3.70.3	Attribute constraints

None.

#### 4.3.70.4	Notifications

The common notifications defined in sub clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.71	CCOWeakCoverageParameters

#### 4.3.71.1	Definition

This IOC represents the properties of CCOWeakCoverageParameters. CCOWeakCoverageParameters is one realization of abstract CCOParameters IOC.

#### 4.3.71.2	Attributes

The IOC includes attributes inherited from CCOParameters IOC

#### 4.3.71.3	Attribute constraints

None.

#### 4.3.71.4	Notifications

The common notifications defined in sub clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.72	CCOPilotPollutionParameters

#### 4.3.72.1	Definition

This IOC represents the properties of CCOPilotPollutionParameters. CCOPilotPollutionParameters is one realization of abstract CCOParameters IOC.

#### 4.3.72.2	Attributes

The IOC includes attributes inherited from CCOParameters IOC.

#### 4.3.72.3	Attribute constraints

None.

#### 4.3.72.4	Notifications

The common notifications defined in sub clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.73	CCOOvershootCoverageParameters

#### 4.3.73.1	Definition

This IOC represents the properties of CCOOvershootCoverageParameters. CCOOvershootCoverageParameters is one realization of abstract CCOParameters IOC.

#### 4.3.73.2	Attributes

The CCOFunction IOC includes attributes inherited from CCOParameters IOC .

#### 4.3.73.3	Attribute constraints

None.

#### 4.3.73.4	Notifications

The common notifications defined in sub clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.74	CCOParameters

#### 4.3.74.1	Definition

This IOC represents the properties of an abstract CCOParameters. The CCOParameters IOC needs to be subclassed to be instantiated.

The CCOWeakCoverageParameters IOC, CCOPilotPollutionParameters IOC, CCOOvershootCoverageParameters IOC is the realization of a CCOParameters IOC, see the inheritance in Figure 4.2.1.2-10.

#### 4.3.74.2	Attributes

The CCOParameters IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| downlinkTransmitPowerRange | O | T | T | F | T |
| antennaTiltRange | O | T | T | F | T |
| antennaAzimuthRange | O | T | T | F | T |
| digitalTiltRange | O | T | T | F | T |
| digitalAzimuthRange | O | T | T | F | T |
| coverageShapeList | O | T | T | F | T |

#### 4.3.74.3	Attribute constraints

None.

#### 4.3.74.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.75	ParameterRange <<dataType>>

#### 4.3.75.1	Definition

This data type represents the adjustment range for parameters.

#### 4.3.75.2	Attributes

The data type includes the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxValue | M | T | T | F | T |
| minValue | M | T | T | F | T |

#### 4.3.75.3	Attribute constraints

None.

#### 4.3.75.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.76	BWPSet

#### 4.3.76.1	Definition

This IOC represents a set of bandwidth part (BWP) set.

The set contains references to all BWPs that can be used by one UE.

There are assumed to be up to 4 DL, 4 UL and 4 SUL BWP for a total of 12 BWP per set.

#### 4.3.76.2	Attributes

The BWP IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bWPList | M | T | T | F | T |

#### 4.3.76.3	Attribute constraints

None.

#### 4.3.76.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.77	NTNFunction

#### 4.3.77.1	Definition

This IOC contains attributes to support the non-terrestrial NR access.

#### 4.3.77.2	Attributes

The NTNFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nTNpLMNInfoList | O | T | T | F | T |
| nTNTAClist | O | T | T | F | T |

Editor’s note: The attributes of NTNFunction needs further work.

#### 4.3.77.3	Attribute constraints

None.

#### 4.3.77.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.78	EphemerisInfoSet

#### 4.3.78.1	Definition

This IOC represents the satellite ephemeris information describing the orbital trajectory information or coordinates for the NTN vehicles.

#### 4.3.78.2	Attributes

The EphemerisInfoSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]).

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ephemerisInfos | M | T | T | F | T |

#### 4.3.78.3	Attribute constraints

None.

#### 4.3.78.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.79	Ephemeris<<dataType>>

#### 4.3.79.1	Definition

This data type represents the satellite ephemeris related information. The ephemeris data format may be expressed either in format of position and velocity state vector or in format of orbital parameters.

#### 4.3.79.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| satelliteId | M | T | F | F | T |
| epochTime | M | T | F | F | T |
| CHOICE_1 positionVelocity | CM | T | F | F | T |
| CHOICE_2 orbital | CM | T | F | F | T |

NOTE: It is an ASN.1 CHOICE for an ephemeris be expressed either in format of position and velocity state vector or in format of orbital parameters, see TS 38.331 [54].

#### 4.3.79.3	Attribute constraints

| positionVelocity S | Condition: Ephemeris is in format of position and velocity. |
| --- | --- |
| orbital S | Condition: Ephemeris is in format of orbital. |

#### 4.3.79.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.80	PositionVelocity <<dataType>>

#### 4.3.80.1	Definition

This data type defines configuration parameters to support satellite position and velocity state.

#### 4.3.80.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| positionX | M | T | F | F | T |
| positionY | M | T | F | F | T |
| positionZ | M | T | F | F | T |
| velocityVX | M | T | F | F | T |
| velocityVY | M | T | F | F | T |
| velocityVZ | M | T | F | F | T |

#### 4.3.80.3	Attribute constraints

None.

#### 4.3.80.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.81	Orbital <<dataType>>

#### 4.3.81.1	Definition

This data type defines configuration parameters of orbital trajectory information to support satellite access.

#### 4.3.81.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| semiMajorAxis | M | T | F | F | T |
| eccentricity | M | T | F | F | T |
| periapsis | M | T | F | F | T |
| longitude | M | T | F | F | T |
| inclination | M | T | F | F | T |
| meanAnomaly | M | T | F | F | T |

#### 4.3.81.3	Attribute constraints

None.

#### 4.3.81.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.82	QceIdMappingInfo <<dataType>>

#### 4.3.82.1	Definition

This data type represents the properties describing the mapping relationship between the unique identity of QoE collection entity, PLMN where QoE collection entity resides, and the IP address of the QoE collection entity.

#### 4.3.82.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qoECollectionEntityAddress | M | T | T | F | T |
| qoECollectionEntityIdentity | M | T | T | F | T |
| pLMNTarget | M | T | T | F | T |

#### 4.3.82.3	Attribute constraints

None.

#### 4.3.82.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.83	MappedCellIdInfo  <<dataType>>

#### 4.3.83.1	Definition

This data type represents the mapping relationship between Mapped Cell IDs and geographical areas (see clause 16.14.5 of TS 38.300 [3]).

#### 4.3.83.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ntnGeoArea | M | T | T | F | T |
| mappedCellId | M | T | T | F | T |

#### 4.3.83.3	Attribute constraints

None.

#### 4.3.83.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.84	LoadTimeThreshold <<dataType>>

#### 4.3.84.1	Definition

This data type represents the traffic load threshold and the time duration.

#### 4.3.84.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| threshold | O | T | T | F | T |
| timeDuration | O | T | T | F | T |

#### 4.3.84.3	Attribute constraints

None

#### 4.3.84.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.85	EsNotAllowedTimePeriod <<dataType>>

#### 4.3.85.1	Definition

This data type represents a time period during which inter-RAT energy saving is not allowed. Time period is valid on the specified day and time of every week.

#### 4.3.85.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| startTime | O | T | T | F | T |
| endTime | O | T | T | F | T |
| daysOfWeek | O | T | T | F | T |

#### 4.3.85.3	Attribute constraints

None

#### 4.3.85.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.86	UeAccDelayProbability <<dataType>>

#### 4.3.86.1	Definition

This data type represents the target Access Delay probability (ADP) for the RACH optimization function.

#### 4.3.86.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetProbability | O | T | T | F | T |
| accessDelay | O | T | T | F | T |

#### 4.3.86.3	Attribute constraints

None

#### 4.3.86.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.87	UeAccProbability <<dataType>>

#### 4.3.87.1	Definition

This data type represents the target Access Probability (APn) for the RACH optimization function.

#### 4.3.87.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetProbability | O | T | T | F | T |
| numberOfPreamblesSent | O | T | T | F | T |

#### 4.3.87.3	Attribute constraints

None

#### 4.3.87.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.88	NRNetwork

#### 4.3.88.1	Definition

This IOC represents a set of NR frequencies and external NR entities (e.g. ExternalGNBCUCPFunction). This is used to differentiate these entities and frequencies from those in EUTRAN.

#### 4.3.88.2	Attributes

The NRNetwork IOC includes the attributes inherited from Top IOC (defined in TS 28.622 [30]).  It contains no additional attributes.

#### 4.3.88.3	Attribute constraints

None.

#### 4.3.88.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.89	EUtraNetwork

#### 4.3.89.1	Definition

This IOC represents a set of EUTRAN frequencies and external EUTRAN entities (e.g. ExternalENBFunction). This is used to differentiate these entities and frequencies from those in NR.

#### 4.3.89.2	Attributes

The EUtraNetwork includes the attributes inherited from Top IOC (defined in TS 28.622 [30]).  It contains no additional attributes.

#### 4.3.89.3	Attribute constraints

None.

#### 4.3.89.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.90	NRECMappingRule

#### 4.3.90.1	Definition

This <<IOC>> represents the unified mapping rule, applicable for the group of gNBs (within a certain area), for the calculation of the energy cost (see TS 34.423[58]).

NRECMappingRule can be name-contained by SubNetwork or ManagedElement. It applies to all ManagedEntity contained by the parent.

#### 4.3.90.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ecMRInputMinimumValue | M | T | T | F | T |
| ecMRInputMaximumValue | M | T | T | F | T |
| ecTimeInterval | M | T | T | F | T |

#### 4.3.90.3	Attribute Constraints

None.

#### 4.3.90.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| NRCellDU.administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: LOCKED, SHUTTING DOWN, UNLOCKED. The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellState | It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active). The Inactive and Active definitions are in accordance with TS 38.401 [4]: "Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs; Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs." allowedValues: IDLE, INACTIVE, ACTIVE. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for downlink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnSUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for supplementary uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamAzimuth | The azimuth of a beam transmission, which means the horizontal beamforming pointing angle (beam peak direction) in the (Phi) φ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53] as well as TS 28.662 [11]. The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Zero degree implies explicit antenna bearing (boresight). Positive angle implies clockwise from the antenna bearing. allowedValues: [-1800 ..1800] 0.1 degree | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamHorizWidth | The Horizontal beamWidth of a beam transmission, which means the horizontal beamforming half-power (3dB down) beamwidth in the (Phi) φ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53]. allowedValues: [0..3599] 0.1 degree | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamIndex | Index of the beam. For example, please see subclause 6.3.2 of TS 38.331 [54] where the ssb-Index in the rsIndexResults element of MeasResultNR is defined. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamTilt | The tilt of a beam transmission, which means the vertical beamforming pointing angle (beam peak direction) in the (Theta) θ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53] as well as TS 28.662 [11]. The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Positive value implies downtilt. allowedValues: [-900..900] 0.1 degree | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamType | The type of the beam. allowedValues: "SSB-BEAM" | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamVertWidth | The Vertical beamWidth of a beam transmission, which means the vertical beamforming half-power (3dB down) beamwidth in the (Theta) θ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53]. allowedValues: [0...1800] 0.1 degree | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDL | BS Channel BW in MHz. for downlink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | BS Channel BW in MHz.for uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwSUL | BS Channel BW in MHz.for supplementary uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxPower | This is the maximum transmission power in milliwatts (mW) at the antenna port for all downlink channels, used simultaneously in a cell, added together. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxEIRP | This is the maximum emitted isotropic radiated power (EIRP) in dBm for all downlink channels, used simultaneously in a cell, added together [12]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageShape | Identifies the sector carrier coverage shape described by the envelope of the contained SSB beams. The coverage shape is implementation dependent. allowedValues: 0 : 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalTilt | Digitally-controlled tilt through beamforming. It represents the vertical pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical vertical tilt of the selected coverageShape. Positive value gives downwards tilt and negative value gives upwards tilt. allowedValues: [-900..900] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalAzimuth | Digitally-controlled azimuth through beamforming. It represents the horizontal pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical horizontal pan of the selected coverageShape. Positive value gives azimuth to the right and negative value gives an azimuth to the left. allowedValues: [-1800 ..1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cyclicPrefix | Cyclic prefix as defined in TS 38.211 [32], subclause 4.2. allowedValues: NORMAL, EXTENDED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localAddress | This parameter specifies the localAddress used for initialization of the underlying transport. The AddressWithVlan <dataType> is defined in clause 4.3.64. | type: AddressWithVlan multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AddressWithVlan.iPaddress | This parameter specifies the IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AddressWithVlan. vlanId | This parameter specifies the local VLAN Id (See IEEE 802.1Q [39]) used for initialization of the underlying transport. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of subclause 8.2 of TS 38.300 [3]. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdLength | This indicates the number of bits for encoding the gNB ID. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 22 .. 32. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUId | It uniquely identifies the DU at least within a gNB-CU. See 'gNB-DU ID' in subclause 9.3.1.9 of 3GPP TS 38.473 [8]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUUPId | It uniquely identifies the gNB-CU-UP at least within a gNB-CU-CP. See 'gNB-CU-UP ID' in subclause 9.3.1.15 of 3GPP TS 38.463 [48]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUName | It identifies the Central Entity of a NR node, see subclause 9.2.1.4 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUName | It identifies the Distributed Entity of a NR node, see subclause 9.2.1.5 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellLocalId | It identifies a NR cell of a gNB. It, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or OperatorDU (for MOCN network sharing scenario) or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See subclause 8.2 of TS 38.300 [3]. The NCI can be constructed by encoding the gNB Identifier using gNBId (of the parent GNBCUCPFunction or GNBDUFunction or OperatorDU (for MOCN network sharing scenario) or ExternalCUCPFunction) and cellLocalId where the gNB Identifier field is of length specified by gNBIdLength (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. The NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. See relation between NCI and NCGI subclause 8.2 of TS 38.300 [3]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See 3GPP TS 36.211 subclause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTAC | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 [13] and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473 [8]. d) For a 5G SA (Stand Alone), it has a non-null value. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUCPFunction.pLMNId | It specifies the PLMN identifier to be used as part of the global RAN node identity. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUUPFunction.pLMNIdList | This is a list of PLMN identifiers. It defines from which set of PLMNs an UE must have as its serving PLMN to be allowed to use the GNB-CU-UP. allowedValues: Not applicable. | type: PLMNId multiplicity: 1..12 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NRCellCU.pLMNInfoList | It defines which PLMNs that can be served by the NR cell, and which S-NSSAIs can be supported by the NR cell for corresponding PLMN in case of network slicing feature is supported. The pLMNId of the first entry of the list is the PLMNId used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| NRCellDU.pLMNInfoList | It defines which PLMNs that can be served by the NR cell, and which S-NSSAIs can be supported by the NR cell for corresponding PLMN in case of network slicing feature is supported. The pLMNId of the first entry of the list is the PLMNId used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| nPNIdentityList | It defines which NPNs that can be served by the NR cell, and which CAG IDs or NIDs can be supported by the NR cell for corresponding PNI-NPN or SNPN in case of the cell is NPN-only cell. (NPN-Identity referring to TS 38.331 [54]) allowedValues: Not applicable. | type: NpnId multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ExternalNRCellCU.pLMNIdList | It defines which PLMNs that are assumed to be served by the NR Cell in another gNB-CU-CP. This list is either updated by the managed element itself (e.g. due to ANR, signalling over Xn etc) or by consumer over the standard interface. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1..12 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rRMPolicyMemberList | It represents the list of RRMPolicyMember (s) that the managed object is supporting. A RRMPolicyMember <<dataType>> include the PLMNId <<dataType>> and S-NSSAI <<dataType>>. allowedValues: N/A | type: RRMPolicyMember multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| resourceType | The resource type of interest for an RRM Policy. allowedValues: PRB, PRB_UL, PRB_DL (for NRCellDU, GNBDUFunction) RRC_CONNECTED_USERS (for NRCellCU, GNBCUCPFunction) DRB (for GNBCUUPFunction) See NOTE 2and NOTE 4 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sNSSAIList | It represents the list of S-NSSAI the managed object is supporting. The S-NSSAI is defined in 3GPP TS 23.003 [13]. allowedValues: See 3GPP TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sST | This attribute specifies the Slice/Service type (SST) of the network slice. allowedValues: See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sD | This attribute specifies the Slice Differentiator (SD), which is optional information that complements the slice/service type(s) to differentiate amongst multiple Network Slices. Pattern: '^[A-Fa-f0-9]{6}$' See clause 5.15.2 of 3GPP TS 23.501 [2]. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rRMPolicyMaxRatio | This attribute specifies the maximum percentage of radio resources that can be used by the associated rRMPolicyMemberList. The maximum percentage of radio resources include at least one of the shared resources, prioritized resources and dedicated resources. For the same resource type, the sum of the ‘rRMPolicyMaxRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same ManagedEntity can be greater than 100. allowedValues: 0 : 100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| rRMPolicyMinRatio | This attribute specifies the minimum percentage of radio resources that can be used by the associated rRMPolicyMemberList. The minimum percentage of radio resources including at least one of prioritized resources and dedicated resources. For the same resource type, the sum of the ‘rRMPolicyMinRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same ManagedEntity shall be less than or equal to 100. allowedValues: 0 : 100 NOTE: Void. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| rRMPolicyDedicatedRatio | This attribute specifies the percentage of radio resource that dedicatedly used by the associated rRMPolicyMemberList. For the same resource type, the sum of the ‘rRMPolicyDedicatedRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same ManagedEntity shall be less than or equal to 100. allowedValues:0 : 100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| subCarrierSpacing | Subcarrier spacing configuration for a BWP. See subclause 5 in TS 38.104 [12]. allowedValues: [15, 30, 60, 120] depending on the frequency range FR1 or FR2. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| txDirection | Indicates if the transmission direction is downlink (DL), uplink (UL) or both downlink and uplink (DL and UL). allowedValues: DL, UL, DL_AND_UL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bwpContext | It identifies whether the object is used for downlink, uplink or supplementary uplink. allowedValues: DL, UL, SUL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: INITIAL, OTHER | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startRB | Offset in common resource blocks to common resource block 0 for the applicable subcarrier spacing for a BWP. This corresponds to N_BWP_start, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 0 to N_grid_size – 1, where N_grid_size equals the number of resource blocks for the BS channel bandwidth, given the subcarrier spacing of the BWP. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfRBs | Number of physical resource blocks for a BWP. This corresponds to N_BWP_size, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 1 to N_grid_size – startRB of the BWP. Se startRB for definition of N_grid_size. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTCI | This is the Target NR Cell Identifier. It consists of NR Cell Identifier (NCI) and Physical Cell Identifier of the target NR cell (nRPCI). The NRRelation.nRTCI identifies the target cell from the perspective of the NRCell, the name-containing instance of the subject NRCellCU instance. allowedValues: Not applicable. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| adjacentNRCellRef | This attribute contains the DN of an adjacentNRCell (NRCellCU or ExternalNRCellCU) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbFrequency | Indicates cell defining SSB frequency domain position Frequency of the cell defining SSB transmission. The frequency provided in this attribute identifies the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block. The frequency must be positioned on the NR global frequency raster, as defined in TS 38.101-1 [42] subclause 5.4.2. and within bSChannelBwDL. allowedValues: 0..3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRFrequencyRef | This attribute contains the DN of the referenced NRFrequency. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRrFreqRelationRef | This attribute contains the DN of the referenced NRFreqRelation. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRSectorCarrierRef | This attribute contains the DN of the referenced NRSectorCarrier. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bWPRef | This attribute contains a list of referenced BWPs. allowedValues: DN of a BWP. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sectorEquipmentFunctionRef | This attribute contains the DN of the referenced SectorEquipmentFunction. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| offsetMO | It is a list of offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR. See offsetMO of subclause 5.5.4 of TS 38.331 [54]. The list is ordered as rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS. This is a list of integer values representing, in sequence: rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS, sinrOffsetCSI-RS. See Q-OffsetRangeList in subclause of subclause 6.3.1 of 3GPP TS 38.331 [54]. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 20, 22, 24 } | type: Integer multiplicity: 6 isOrdered: True isUnique: False defaultValue: 0 isNullable: False |
| cellIndividualOffset | It is a list of offset values for the neighbour cell. Used when UE is in connected mode. The unit is 1dB. It is defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS. See TS 38.331 [54]. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 20, 22, 24 } | type: Integer multiplicity: 6 isOrdered: True isUnique: False defaultValue: 0 isNullable: False |
| blockListEntry | It specifies a list of PCI (physical cell identity) that are exclude-listed in EUTRAN measurements as described in 3GPP TS 38.331 [54]. allowedValues: { 0…503 } | type: Integer multiplicity: 0..16 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| blockListEntryIdleMode | It specifies a list of PCI (physical cell identity) that are exclude-listed in SIB4 and SIB5 as described in TS 38.331 [54]. allowedValues: { 0…1007 } | type: Integer multiplicity: 0..16 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cellReselectionPriority | It is the absolute priority of the carrier frequency used by the cell reselection procedure. See CellReselectionPriority IE in TS 38.331 [54]. It corresponds to the parameter priority in 3GPP TS 38.304 [49]. Value 0 means lowest priority. The UE behaviour when no value is entered is specified in subclause 5.2.4.1 of 3GPP TS 38.304 [49]. The value must not already used by other RAT, i.e. equal priorities between RATs are not supported. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| cellReselectionSubPriority | It indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. See CellReselectionSubPriority IE in TS 38.331 [54]. allowedValues: { 0.2, 0.4, 0.6, 0.8 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pMax | It calculates the parameter Pcompensation (defined in 3GPP TS 38.304 [49]), at cell reselection to an Cell. Its unit is 1 dBm. It corresponds to parameter PEMAX in 3GPP TS 38.101-1 [42]. allowedValues: { -30..33 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qOffsetFreq | It is the frequency specific offset applied when evaluating candidates for cell reselection. See TS 38.331 [54]. Its unit is 1 dB. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 20, 22, 24 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| qQualMin | It indicates the minimum required quality level in the cell (dB). See qQualMin in TS 38.304 [49]. Unit is 1 dB. Value 0 means that it is not sent and UE applies in such case the (default) value of negative infinity for Qqualmin. Sent in SIB3 or SIB5. allowedValues: { -34..-3, 0 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qRxLevMin | It indicates the required minimum received Reference Symbol Received Power (RSRP) level in the (E-UTRA) frequency for cell reselection. It corresponds to Qrxlevmin defined in 3GPP TS 38.304 [49]. It is broadcast in SIB3 or SIB5, depending on whether the related frequency is intra- or inter-frequency. Its unit is 1 dBm and resolution is 2. allowedValues: { -140..-44 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXHighP | This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold. It corresponds to the ThreshX, HighP in 3GPP TS 38.304 [49]. Its unit is 1 dB and resolution is 2. allowedValues: { 0..62 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXHighQ | This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold. It corresponds to the ThreshX, HighQ in TS 38.304 [49]. Its unit is 1 dB. allowedValues: { 0..31 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXLowP | This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR might have a specific threshold. It corresponds to ThreshX, LowP in TS 38.304 [49]. Its unit is 1 dB. Its resolution is 2. allowedValues: { 0..62 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXLowQ | This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR might have a specific threshold. It corresponds to ThreshX, LowQ in TS 38.304 [49]. Its unit is 1 dB. allowedValues: {0..31}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNr | It is the cell reselection timer and corresponds to parameter TreselectionRAT for NR defined in 38.331 [54]. Its unit is in seconds. allowedValues: {0..7}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNRSfHigh | The attribute t-ReselectionNr (a parameter TreselectionNR in TS 38.304 [49]) is multiplied with this factor if the UE is in high mobility state. It corresponds to the parameter Speed dependent ScalingFactor for TreselectionNr for medium high state in 3GPP TS 38.304 [49]. The unit is one %. Value mapping: 25 = 0.25 50 = 0.5 75 = 0.75 100 = 1.0 allowedValues: {25, 50, 75, 100}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNRSfMedium | The attribute t-ReselectionNR (a parameter "TreselectionNR in TS 38.304 [49]”) is multiplied with this factor if the UE is in medium mobility state. It corresponds to the parameter Speed dependent ScalingFactor for TreselectionNr for medium mobility state in 3GPP TS 38.304 [49]. Its unit is one %. Value mapping: 25 = 0.25 50 = 0.5 75 = 0.75 100 = 1.0 allowedValues: {25, 50, 75, 100}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| absoluteFrequencySSB | The absolute frequency applicable for a downlink NR carrier frequency associated with the SSB. allowedValues: {0.. 3279165}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbSubCarrierSpacing | This SSB is used for for synchronization. See subclause 5 in TS 38.104 [12]. Its units are in kHz. allowedValues: {15, 30, 120, 240}. Note that the allowed values of SSB used for representing data, by e.g. a BWP, are: 15, 30, 60 and 120 in units of kHz. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| multiFrequencyBandListNR | It is a list of additional frequency bands the frequency belongs to. The list is automatically set by the gNB. allowedValues: {1..256 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbPeriodicity | Indicates cell defined SSB periodicity in number of subframes (ms). The SSB periodicity in msec is used for the rate matching purpose. allowedValues: 5, 10, 20, 40, 80, 160. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbOffset | Indicates cell defining SSB time domain position. Defined as the offset of the measurement window, in number of subframes (ms), in which to receive SS/PBCH blocks, where allowed values depend on the ssbPeriodicity. allowedValues: ssbPeriodicity5 ms 0..4, ssbPeriodicity10 ms 0..9, ssbPeriodicity20 ms 0..19, ssbPeriodicity40 ms 0..39, ssbPeriodicity80 ms 0..79, ssbPeriodicity160 ms 0..159. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbDuration | Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (ms) (see 38.213 [41], clause 4.1. allowedValues: 1, 2, 3, 4, 5. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringStartTime | This field configures the time when the gNB attempts to start RIM-RS monitoring. allowedValues: Not applicable. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringStopTime | This field configures the time when the gNB stops RIM-RS monitoring. allowedValues: Not applicable. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mappingSetIDBackhaulAddressList | The attribute specifies a list of mappingSetIDBackhaulAddress which is defined as a datatype (see clause 4.3.47). Which is used to retrieve the backhaul address of the victim set. allowedValues: Not applicable | type: MappingSetIDBackhaulAddress multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| backhaulAddress | The attribute specifies backhaulAddress which is defined as a datatype (see clause 4.3.48). allowedValues: Not applicable | type: BackhaulAddress multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| setId | This specifies the set ID of a victim Set (RIM-RS1 Set) or aggressor Set (RIM-RS2 set). (See subclause 7.4.1.6 in TS 38.211 [32]). allowedValues: The bit length of the set ID is maximum 22bit. See NOTE 10. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tAI | Indicates the TAI (see subclause 9.3.3.11 in TS 38.413[5]), including pLMNId ID and nRTAC. allowedValues: Not applicable | type: TAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isRemoveAllowed | This indicates if the subject NRCellRelation can be removed (deleted) or not. If TRUE, the subject NRCellRelation instance can be removed (deleted). If FALSE, the subject NRCellRelation instance shall not be removed (deleted) by any entity but an MnS consumer. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isHOAllowed | This indicates if HO is allowed or prohibited. If TRUE, handover is allowed from source cell to target cell. The source cell is identified by the name-containing NRCellCU of the NRCellRelation that contains the isHOAllowed. The target cell is referenced by the NRCellRelation that contains this isHOAllowed. If FALSE, handover shall not be allowed. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intrasystemANRManagementSwitch | This attribute determines whether the intra-system ANR function is activated or deactivated. If “TRUE”, the intra-system ANR function may add or remove intra NG-RAN Neighbour Relations, i.e. add or remove NRCellRelation instances from NRCellCU of this GNBCUCPFunction. If “FALSE”, the intra-system ANR Function must not add or remove Neighbour Relations, i.e. add or remove NRCellRelation instances from NRCellCU of this GNBCUCPFunction. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intersystemANRManagementSwitch | This attribute determines whether the inter-system ANR function is activated or deactivated. If “TRUE”, the inter-system ANR function may add or remove inter-system Neighbour Relations, i.e. add or remove EUtranRelation instances from NRCellCU of this GNBCUCPFunction. If “FALSE”, the inter-system ANR Function must not add or remove inter-system Neighbour Relations, i.e. add or remove EUtranRelation instances from NRCellCU of this GNBCUCPFunction. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| desSwitch | This attribute determines whether the Distributed SON energy saving function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cesSwitch | This attribute determines whether the Centralized SON energy saving function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySavingControl | This attribute allows the Centralized SON energy saving function to initiate energy saving activation or deactivation. allowedValues: TO_BE_ENERGY_SAVING, TO_BE_NOT_ENERGY_SAVING | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySavingState | Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is toBeEnergySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is toBeNotEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. allowedValues: IS_NOT_ENERGY_SAVING, IS_ENERGY_SAVING. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intraRatEsActivationOriginalCellLoadParameters | This attribute is relevant, if the cell acts as an original cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow a cell to enter the energySaving state. The time duration indicates how long the load needs to have been below the threshold. allowedValues: loadThreshold: Integer 0..100 (Percentage of PRB usage, see 3GPP TS 36.314 [13]) timeDuration: Integer (in unit of seconds) | type: LoadTimeThreshold multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intraRatEsActivationCandidateCellsLoadParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms level to allow a n ‘original’ cell to enter the energySaving state. Threshold and duration are applied to the candidate cell(s) which will provides coverage backup of an original cell when it is in the energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it will provide backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energy saving state. allowedValues: loadThreshold: Integer 0..100 (Percentage of PRB usage (see 3GPP TS 36.314 [13]) ) timeDuration: Integer (in unit of seconds) | type: LoadTimeThreshold multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intraRatEsDeactivationCandidateCellsLoadParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed ES algorithms to allow a cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell when it which provides coverage backup for the cell in energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it provides backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. allowedValues: loadThreshold: Integer 0..100 (Percentage of PRB usage (see 3GPP TS 36.314 [13]) ) timeDuration: Integer (in unit of seconds) | type: LoadTimeThreshold multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LoadTimeThreshold.threshold | This attribute indicates a traffic load threshold. allowedValues: Integer | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LoadTimeThreshold.timeDuration | This attribute indicates a duration in unit of seconds. allowedValues: Integer | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| esNotAllowedTimePeriod | This attribute can be used to prevent a cell entering energySaving state. This attribute indicates a list of time periods during which inter-RAT energy saving is not allowed. Time period is valid on the specified day and time of every week. allowedValues: N/A | type: EsNotAllowedTimePeriod multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| EsNotAllowedTimePeriod.startTime | This attribute indicates a time of day as a start time for a period. Time of day is in HH:MM or H:MM 24-hour format per UTC time zone. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EsNotAllowedTimePeriod.endTime | This attribute indicates a valid time of day as an end time for a period. The endTime should be later than startTime. Time of day is in HH:MM or H:MM 24-hour format per UTC time zone. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EsNotAllowedTimePeriod.daysOfWeek | This attribute indicates a day in a week. allowedValues: MONDAY, TUESDAY, WEDNESDAY, THURSDAY, FRIDAY, SATURDAY, SUNDAY | type: <<enumeration>> multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| interRatEsActivationOriginalCellParameters | This attribute is relevant, if the cell acts as an original cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state. The time duration indicates how long the traffic load (both for UL and DL) needs to have been below the threshold. In case the original cell is an EUTRAN cell, the load information refers to Composite Available Capacity Group IE (see 3GPP TS 36.413 [12] Annex B.1.5) and the following applies: Load = (100 - ‘Capacity Value’ ) * ‘Cell Capacity Class Value’, where ‘Capacity Value’ and ‘Cell Capacity Class Value’ are defined in 3GPP TS 36.423 [7]. In case the original cell is a UTRAN cell, the load information refers to Cell Load Information Group IE (see 3GPP TS 36.413 [12] Annex B.1.5) and the following applies: Load= ‘Load Value’ * ‘Cell Capacity Class Value’, where ‘Load Value’ and ‘Cell Capacity Class Value’ are defined in 3GPP TS 25.413 [19]. If the ‘Cell Capacity Class Value’ is not known, then ‘Cell Capacity Class Value’ should be set to 1 when calculating the load, and the load threshold should be set in range of 0..100. allowedValues: loadThreshold: Integer 0..10000 timeDuration: Integer 0..900 (in unit of seconds) | type: LoadTimeThreshold multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| interRatEsActivationCandidateCellParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state. Threshold and time duration are applied to the candidate cell(s) which will provides coverage backup of an original cell when it is in the energySaving state. The time duration indicates how long the traffic load (both for UL and DL) in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energySaving state. In case the candidate cell is a UTRAN or GERAN cell, the load information refers to Cell Load Information Group IE(see 3GPP TS 36.413 [12] Annex B.1.5) and the following applies: Load= ‘Load Value’ * ‘Cell Capacity Class Value’, where ‘Load Value’ and ‘Cell Capacity Class Value’ are defined in 3GPP TS 25.413 [19] (for UTRAN) / TS 48.008 [20] (for GERAN). If the ‘Cell Capacity Class Value’ is not known, then ‘Cell Capacity Class Value’ should be set to 1 when calculating the load, and the load threshold should be set in range of 0..100. allowedValues: loadThreshold: Integer 0..10000 timeDuration: Integer 0..900 (in unit of seconds) | type: LoadTimeThreshold multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| interRatEsDeactivationCandidateCellParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed inter-RAT ES algorithms to allow an original cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell which provides coverage backup for the cell in energySaving state. The time duration indicates how long the traffic load (either for UL or DL) in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. For the load see the definition of interRatEsActivationCandidateCellParameters. allowedValues: loadThreshold: Integer 0..10000 timeDuration: Integer 0..900 (in unit of seconds) | type: LoadTimeThreshold multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isProbingCapable | This attribute indicates whether this cell is capable of performing the ES probing procedure. During this procedure the eNB owning the cell indicates its presence to UEs for measurement purposes, but prevents idle mode UEs from camping on the cell and prevents incoming handovers to the same cell. If this parameter is absent, then probing is not done. allowedValues: YES, NO | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dmroControl | This attribute determines whether the MRO function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dDAPSHOControl | This attribute determines whether the DAPS handover function is enabled or disabled. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dCHOControl | This attribute determines whether the CHO handover function is enabled or disabled. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlboControl | This attribute determines whether the D-LBO function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cSonPciList | This holds a list of physical cell identities that can be assigned to the pci attribute by gNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the C-SON PCI configuration is supported. See TS 28.313, ref [57] subclause 7.1.3. allowedValues: See TS 38.211 [32] subclause 7.4.2.1 for legal values of pci. The number of pci in the list is 0 to 1007. | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ueAccProbabilityDist | This is a list of target Access Probability (APn) for the RACH optimization function. Each instance APn of the list is the probability that the UE gets access on the RACH channel per cell within n number of preambles sent over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, APn, is a pair (a, n) where a is the targetProbability (in %) and n is the number of preambles sent. The legal values for a are 25, 50, 75, 90. The legal values for n are 1 to 200. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and n is vendor-specific. | type: UeAccProbability multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ueAccDelayProbabilityDist | This is a list of target Access Delay probability (ADP) for the RACH optimization function. Each instance ADP of the list is the target time before the UE gets access on the RACH channel per cell, for the P percent of the successful RACH Access attempts with lowest accessDelay, over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, ADp, is a pair (p, d) where p is the targetProbability (in %) and d is the access delay (in milliseconds). The legal values for p are 25, 50, 75, 90. The legal values for d are 10 to 560. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for p and d is vendor-specific. | type: UeAccDelayProbability multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetProbability | This attribute indicates a probability (in %). allowedValues: 0..100 | type: Integer multiplicity:0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfPreamblesSent | This attribute indicates the number of preambles sent used to configure a wanted distribution of RACH preambles in a vendor implemented DRACH optimisation function. allowedValues: 1..200 Note: The DRACH optimization function may configure preambleTransMax as defined in TS 38.331 [54]. The allowed values for preambleTransMax are 3, 4, 5, 6, 7, 8, 10, 20, 50, 100, 200 (see 38.331 [54], subclause 6.3.2). | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| accessDelay | This attribute indicates the access delay in unit of milliseconds. allowedValues: 10..560 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| drachOptimizationControl | This attribute determines whether the RACH Optimization function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRPciList | This holds a list of physical cell identities that can be assigned to the NR cells. This attribute shall be supported if D-SON PCI configuration function is supported. See subclause 8.2.3, 8.3.1 in TS 28.313 [57]. allowedValues: See TS 38.211 [32] subclause 7.4.2 for legal values of pci. The number of pci in the list is 0 to 1007. | type: Integer multiplicity: 0..1007 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dPciConfigurationControl | This attribute determines whether the Distributed SON PCI configuration Function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cPciConfigurationControl | This attribute determines whether the Centralized SON PCI configuration function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maximumDeviationHoTriggerLow | This parameter defines the maximum allowed lower deviation of the Handover Trigger, from the default point of operation (see clause 15.5.2.5 in TS 38.300 [3] and clause 9.2.2.61 in TS 38.423 [58].) allowedValues: -20..20 Unit: 0.5 dB | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maximumDeviationHoTriggerHigh | This parameter defines the maximum allowed upper deviation of the Handover Trigger, from the default point of operation (see clause 15.5.2.5 in TS 38.300 [3]. and clause 9.2.2.61 in TS 38.423 [58].) allowedValues: -20..20 Unit: 0.5 dB | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minimumTimeBetweenHoTriggerChange | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.2.5 in TS 38.300 [3]). allowedValues: 0..604800 Unit: Seconds | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tstoreUEcntxt | The timer used for detection of too early HO, too late HO and HO to wrong cell. Corresponds to Tstore_UE_cntxt timer described in clause 15.5.2.5 in TS 38.300 [3]. This attribute is used for Mobility Robustness Optimization. allowedValues: 0..1023 Unit: 100 milliseconds | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configurable5QISetRef | This is the DN of Configurable5QISet. The detailed definition for Configurable5QISet see clause 5.3.75. allowedValues: DN of the Configurable5QISet MOI. | type: DN multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dynamic5QISetRef | This is the DN of Dynamic5QISet. The detailed definition for Dynamic5QISet see clause 5.3.94. allowedValues: DN of the Dynamic5QISet MOI. | type: DN multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| frequencyDomainPara | This attribute defines configuration parameters of frequency domain resource to support RIM RS. allowedValues: Not applicable. | type: FrequencyDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sequenceDomainPara | This attribute defines configuration parameters of sequence domain resource to support RIM RS. allowedValues: Not applicable. | type: SequenceDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeDomainPara | This attribute defines configuration parameters of time domain resource to support RIM RS. allowedValues: Not applicable. | type: TimeDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSSubcarrierSpacing | It is the subcarrier spacing configuration (μ) for the RIM-RS. Subcarrier spacing Δf=2μ∙15 kHz. (see 38.211 [32], subclause 5.3.3). allowedValues: 0, 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rIMRSBandwidth | It is the bandwidth of the RIM-RS in resource blocks (see 38.211 [32], subclause 5.3.3). For carrier bandwidth larger than 20MHz, this attributer should be 96 if subcarrier spacing is15kHz; 48 or 96 if subcarrier spacing is 30kHz; For carrier bandwidth smaller than or equal to 20MHz, this attribute should be Minimum of {96 , bandwidth of downlink carrier in number of PRBs} if subcarrier spacing is15kHz; Minimum of {48, bandwidth of downlink carrier in number of PRBs } if subcarrier spacing is 30kHz; allowedValues: 1,2..96 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofGlobalRIMRSFrequencyCandidates | It is the number of candidate frequency resources in the whole network (NfRIM) (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2,4 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSCommonCarrierReferencePoint | This attribute is used to configure the common reference point for RIM RS. Where represents the frequency-location of point A expressed as in ARFCN. See 3GPP TS 38.211 [32] subclause 4.4.4.2 allowedValues: 0..3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSStartingFrequencyOffsetIdList | It is a list of configured frequency offsets in units of resource blocks, where each element is the frequency offset relative to a configured reference point for RIM-RS. The size of the list is nrofGlobalRIMRSFrequencyCandidates and the resulting frequency resource blocks of RIM-RS corresponding to different configured frequency offset have no overlapping bandwidth. (see 38.211 [32], subclause 7.4.1.6). . allowedValues: 0..maxNrofPhysicalResourceBlocks-1 where maxNrofPhysicalResourceBlocks = 550 | type: Integer multiplicity: 1, 2, 4 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nrofRIMRSSequenceCandidatesofRS1 | It is the number of candidate sequences assigned for RIM RS-1 (NsRIM,1) (see 38.211 [32], subclause 7.4.1.6). It should be even when enableEnoughNotEnoughIndication for RS-1 is ON allowedValues: 1,2..8 see NOTE 10 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSScrambleIdListofRS1 | It is a list of configured scrambling identities for RIM RS-1 (see 38.211 [32], subclause 7.4.1.6). The size of the list is nrofRIMRSSequenceCandidatesofRS1. allowedValues: 0..2^10-1 | type: Integer multiplicity: 1, 2..8 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nrofRIMRSSequenceCandidatesofRS2 | It is the number of candidate sequences assigned for RIM RS-2 (NsRIM,2) (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2..8 See NOTE 10. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSScrambleIdListofRS2 | It is a list of configured scrambling identities for RIM RS-2 (see 38.211 [32], subclause 7.4.1.6).. The size of the list is nrofRIMRSSequenceCandidatesofRS2. allowedValues: 0..2^10-1 | type: Integer multiplicity: 1, 2..8 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| enableEnoughNotEnoughIndication | It is indication of whether “Enough” / “Not enough” indication functionality is enabled for RIM RS-1 (see 38.211 [32], subclause 7.4.1.6). If the indication is "ENABLE", the first half of nrofRIMRSSequenceCandidatesofRS1 sequences indicates "Not enough mitigation", and the second half indicates "Enough mitigation", where, "Enough mitigation" indicates that IoT going back to certain level at victim side and/or no further interference mitigation actions are needed at aggressor side "Not enough mitigation" indicates that IoT exceeding certain level at victim side and/or further interference mitigation actions are needed at aggressor side enableEnoughNotEnoughIndication is equivalent to EnoughIndication (see 38.211 [32], subclause 7.4.1.6) allowedValues: "ENABLE", "DISABLE" see NOTE 8 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| rIMRSScrambleTimerMultiplier | It is parameter multiplier factor γ for initialization seed of the pseudo-random sequence ci (see 38.211 [32], subclause 7.4.1.6.2). allowedValues: 0,1,….2^31-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rIMRSScrambleTimerOffset | It is parameter offset δ for initialization seed of the pseudo-random sequence ci (see 38.211 [32], subclause 7.4.1.6.2). allowedValues: 0,1,….2^31-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlULSwitchingPeriod1 | This attribute is used to configure the first uplink-downlink switching period (P1) for RIM RS transmission in the network, where one RIM RS is configured in one uplink-downlink switching period. (see 38.211 [32], subclause 7.4.1.6). When only one TDD-UL-DL-Pattern is configured, only dl-UL-SwitchingPeriod1 is configured, where P1 equals to the transmission periodicity of the TDD-UL-DL-Pattern. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, only dl-UL-SwitchingPeriod1 is configured, where P1 equals to the addition of the concatenated transmission periodicity of the two TDD-UL-DL-Patterns. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both dl-UL-SwitchingPeriod1 and dl-UL-SwitchingPeriod2 are configured, where P1 equals to the transmission periodicity of the first TDD-UL-DL-Pattern. P1 is equivalent to Tper,1RIM (see 38.211 [32], subclause 7.4.1.6). See NOTE 6 allowedValues: MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS4, MS5, MS10, MS20, if a single uplink-downlink period is configured for RIM-RS purposes; MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS3, MS4, MS5, MS10, MS20, if two uplink-downlink periods are configured for RIM-RS purposes. see NOTE 9 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| symbolOffsetOfReferencePoint1 | This attribute is used to configure the reference point in the first uplink-downlink switching period, which is the symbols offset of the reference point after the starting boundary of the first uplink-downlink switching period. It’s Configured together with dl-UL-SwitchingPeriod1 (see 38.211 [32], subclause 7.4.1.6). When only one TDD-UL-DL-Pattern is configured, the reference point configured for the first uplink-downlink switching period is the DL transmission boundary of the TDD-UL-DL-Pattern. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, the reference point configured for the first uplink-downlink switching period is the DL transmission boundary of the TDD-UL-DL-Pattern where the RIM-RS resource is configured. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, the reference points configured for first uplink-downlink switching period is the DL transmission boundary of the first TDD-UL-DL-Pattern. allowedValues: 2, 3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlULSwitchingPeriod2 | This attribute is used to configure the second uplink-downlink switching period (P2) for RIM RS transmission in the network, where one RIM RS is configured in one uplink-downlink switching period (see 38.211 [32], subclause 7.4.1.6). When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both dl-UL-SwitchingPeriod1 and dl-UL-SwitchingPeriod2 are configured, where P2 equals to the transmission periodicity of the second TDD-UL-DL-Pattern, and where (P1 + P2) divides 20 ms. allowedValues: MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS3, MS4, MS5, MS10 P2 is equivalent to Tper,2RIM (see 38.211 [32], subclause 7.4.1.6) See NOTE 9 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| symbolOffsetOfReferencePoint2 | This attribute is used to configure the reference point in the second uplink-downlink switching period, which is the symbol offset of the reference point after starting boundary of the second uplink-downlink switching period. Configured together with dl-UL-SwitchingPeriod2 (see 38.211 [32], subclause 7.4.1.6). When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, the reference points configured for second uplink-downlink switching period is the DL transmission boundary of the second TDD-UL-DL-Pattern. allowedValues: 2, 3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalnrofSetIdofRS1 | It is the total number of set IDs for RIM RS-1 (NsetIDRIM,1) (see 38.211 [32], subclause 7.4.1.6). allowedValues: 0,1...2^22-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalnrofSetIdofRS2 | It is the total number of set IDs for RIM RS-2 (NsetIDRIM,2) (see 38.211 [32], subclause 7.4.1.6). allowedValues: 0,1...2^22 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofConsecutiveRIMRS1 | It is the number of consecutive uplink-downlink switching periods for RS-1 (R1) for repetition/near-far indication:. (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2,4,8 see NOTE 7 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofConsecutiveRIMRS2 | It is the number of consecutive uplink-downlink switching periods for RS-2 (R2) for repetition/near-far indication. (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2,4,8 see NOTE 7 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| consecutiveRIMRS1List | It is used to configure the OFDM symbol position(s) of RIM RS-1 within the uplink-downlink switching period. It is a list of symbol offset of RIM RS-1 (Nsymb,refRIM, 1) before the reference point. The size of the list is nrofConsecutiveRIMRS1 (see 38.211 [32], subclause 7.4.1.6). The resulting RIM RS-1 symbols and its reference point shall belong to the same 10ms frame. . allowedValues: 2,3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| consecutiveRIMRS2List | It is used to configure the OFDM symbol position(s) of RIM RS-2 within the uplink-downlink switching period. It is a list of symbol offset of RIM RS-2 (Nsymb,refRIM, 2) before the reference point. The size of the list is nrofConsecutiveRIMRS2 (see 38.211 [32], subclause 7.4.1.6). The resulting RIM RS-2 symbols and its reference point shall belong to the same 10ms frame. . allowedValues: 2,3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| enablenearfarIndicationRS1 | It is indication of whether near-far functionality is enabled for RIM RS1. If the indication is “ENABLE”, the first half of nrofConsecutiveRIMRS1 (R1) consecutive uplink-downlink switching period is for "Near" indication with R1/2 repetitions, the second half of R1 consecutive uplink-downlink switching period is for "Far" indication with R1/2 repetitions. allowedValues: "ENABLE", "DISABLE" see NOTE 10. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| enablenearfarIndicationRS2 | It is indication of whether near-far functionality is enabled for RIM RS2. If the indication is “ENABLE”, the first half of nrofConsecutiveRIMRS2 (R2) consecutive uplink-downlink switching period is for "Near" indication with R2/2 repetitions, the second half of R2 consecutive uplink-downlink switching period is for "Far" indication with R2/2 repetitions. allowedValues: "ENABLE", "DISABLE" see NOTE 10. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| rimRSReportConf | It is used to configure gNBs to report the all necessary information derived from the detected RIM-RS to OAM. allowedValues: Not applicable | type: RimRSReportConf multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportIndicator | It is used to enable or disable the RS report on a gNB. If the indication is “ENABLE”, the gNB starts to periodically report necessary information derived from the detected RIM-RS to OAM. If the indication is “DISABLE”, the gNB stops reporting. allowedValues: ENABLE, DISABLE | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| reportInterval | It is used to define reporting interval of a gNB in ms. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSReportInfo | It is used to define the maximum number of RIMRSReportInfo in a single report. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxPropagationDelay | It is used to define the maximum reported OFDM symbol number for the propagation delay of the detected RIM-RS in each RIMRSReportInfo. allowedValues: 0, 1..20*2*maxNrofSymbols-1, where maxNrofSymbols=14. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSReportInfoList | It represents a list (the length of the list is nrofRIMRSReportInfo) of necessary information derived from the detected RIM-RS. allowedValues: Not applicable | type: RimRSReportInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| detectedSetID | This attribute indicates the Set ID of the detected RIM-RS. allowedValues: 0,1...max{totalnrofSetIdofRS1, totalnrofSetIdofRS2}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| propagationDelay | This attribute indicates the propagation delay of the detected RIM-RS, in number of OFDM symbol. allowedValues: 0, 1.. maxPropagationDelay. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| functionalityOfRIMRS | This attribute indicates the functionality of the detected RIM-RS. If the indication of enableEnoughNotEnoughIndication is “enable”, valid values are {RS2, RS1_FOR_ENOUGH_MITIGATION, RS1_FOR_NOT_ENOUGH_MITIGATION}; If the indication of enableEnoughNotEnoughIndication is “disable”, valid values are {RS1, RS2}. RS1_FOR_ENOUGH_MITIGATION means RIM-RS type 1 is used to indicate 'enough mitigation' functionality. RS1_FOR_NOT_ENOUGH_MITIGATION means RIM-RS type 1 is used to indicate 'Not enough mitigation' functionality. allowedValues: RS1, RS2, RS1_FOR_ENOUGH_MITIGATION, RS1_FOR_NOT_ENOUGH_MITIGATION | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowDuration | This attribute configures a duration of the monitoring window in which gNB monitors the RIM-RS, in unit of Pt, where Pt is the RIM-RS transmission periodicity in units of uplink-downlink switching period (see 38.211 [32], subclause 7.4.1.6). This field is configured together with rimRSMonitoringInterval, rimRSMonitoringWindowStartingOffset, rimRSMonitoringOccasionInterval and rimRSMonitoringOccasionStartingOffset. The duration of the monitoring window is expected to be larger than or equal to M*Pt, where M is the interval between adjacent monitoring occasions within the monitoring window (configured by rimRSMonitoringInterval). The absolute duration of the monitoring window is not expected to be larger than the periodicity of the monitoring window (configured by rimRSMonitoringWindowPeriodicity). Only the earliest NT consecutive detection durations in each RIM-RS transmission periodicity (Pt) in the monitoring window are taken as valid time for monitoring potential interference, and they are consecutively monitored in the monitoring window, while the residual part of each RIM-RS transmission periodicity is not used for discovering potential interference, where, a consecutive detection duration spans P1*R1 (if only P1 is configured) or P1+P22*R1 (if both P1 and P2 are configured), where, R1 is the number of consecutive uplink-downlinkswitching periods for RS-1 (configured by nrofConsecutiveRIMRS1), P1 is the first uplink-downlinkswitching period (configured by dlULSwitchingPeriod1), P2 is the second uplink-downlink switching period (configured by dlULSwitchingPeriod2), and NT=NsetIDRIM,1NfRIMNsRIM,1if enableEnoughNotEnoughIndication is "disable"2NsetIDRIM,1NfRIMNsRIM,1if enableEnoughNotEnoughIndication is "enable" NsetIDRIM,1 is the total number of set IDs for RIM RS-1 (configured by totalnrofSetIdofRS1), NfRIM is the number of candidate frequency resources in the whole network (configured by nrofGlobalRIMRSFrequencyCandidates), and NsRIM,1 is the number of candidate sequences assigned for RIM RS-1 (configured by nrofRIMRSSequenceCandidatesofRS1). allowedValues: 1,2,..2^14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowPeriodicity | This attribute configures the periodicity of the monitoring window, in unit of hours. allowedValues: 1, 2, 3, 4, 6, 8, 12, 24 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowStartingOffset | This attribute configures the start offset of the first monitoring window within one day, in unit of hours. allowedValues: 0,1,2..23 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringOccasionInterval | This attribute configures the interval between adjacent monitoring occasions (M) within the monitoring window, in unit of consecutive detection duration. M is expected to be prime to NT, where NT is given in above attribute rimRSMonitoringWindowDuration. allowedValues: 1,2..NT-1. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringOccasionStartingOffset | This attribute configures the start offset of the first monitoring occasions within the monitoring window (SM), in unit of consecutive detection duration. gNB starts monitoring potential interference from the SM-th consecutive detection duration in the first complete RIM-RS transmission periodicity (Pt) within the monitoring window. allowedValues: 0,1,2..M-1 where M is the the interval between adjacent monitoring occasions within the monitoring window (configured by rimRSMonitoringOccasionInterval) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| victimSetRef | This attribute contains the DN of a victim Set (RimRSSet) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aggressorSetRef | This attribute contains the DN of an aggressor Set (RimRSSet) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| setType | The attribute specifies type of a RIM-RS Set . RIM RS1 is generated and transmitted by victim to indicate its suffering remote interference, and RIM RS2 is generated and transmitted by aggressor to measure if Remote Interference still exist If the attribute value is “RS1”, the RIM-RS Set is victim set. If the attribute value is “RS2”, the RIM-RS Set is aggressor set. allowedValues: RS1, RS2. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRCellDURef | This attribute contains the DN of a NR Cell (NRCellDU) allowedValues: Not applicable. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| isENDCAllowed | This indicates if EN-DC is allowed or prohibited. If TRUE, the target cell is allowed to be used for EN-DC. The target cell is referenced by the NRCellRelation that contains this isENDCAllowed. If FALSE, EN-DC shall not be allowed. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| x2BlockList | This is a list of GeNBIds. If the target node GeNBId is a member of the source node’s NRCellCU.x2BlockList, the source node is: 1) prohibited from sending X2 connection requests to the target node; 2) forced to tear down an established X2 connection to the target node; 3) not allowed to accept incoming X2 connection requests from the target node. The same GeNBId may appear here and in NRCellCU.x2AllowList. In such case, the GeNBId in x2AllowList shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnBlockList | This is a list of GgNBIds. If the target node GgNBId is a member of the source node’s NRCellCU.xnBlockList, the source node is: 1) prohibited from sending Xn connection requests to the target node; 2) forced to tear down an established Xn connection to the target node; 3) not allowed to accept incoming Xn connection requests from the target node. The same GgNBId may appear here and in NRCellCU.xnAllowList. In such case, the GgNBId in xnAllowList shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2AllowList | This is a list of GeNBIds. If the target node GeNBId is a member of the source node’s NRCellCU.x2AllowList, the source node is: 1) allowed to request the establishment of an X2 connection to the target node; 2) not allowed to initiate the tear down of an established X2 connection to the target node The same GeNBId may appear here and in NRCellCU.x2BlockList. In such case, the GeNBId here shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnAllowList | This is a list of GgNBIds. If the target node GgNBId is a member of the source node’s NRCellCU.xnAllowList, the source node is: 1) allowed to request the establishment of Xn connection with the target node; 2) not allowed to initiate the tear down of an established Xn connection to the target node The same GgNBId may appear here and in NRCellCU.xnBlockList. In such case, the GgNBId here shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnHOBlockList | This is a list of GgNBIds. For all the entries in NRCellCU.xnHOBlockList, the subject NRCellCU is prohibited to use the Xn interface for HOs even if an Xn interface exists to the target cell. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2HOBlockList | This is a list of GeNBIds. For all the entries in NRCellCU.x2HOBlockList, the subject NRCellCU is prohibited to use the X2 interface for HOs even if an X2 interface exists to the target cell. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tceIDMappingInfoList | This attribute includes a list of TCE ID, PLMN where TCE resides and the corresponding TCE IP address. It is used in Logged MDT case to provide the information to the gNodeB or GNBCUCPFunction to get the corresponding TCE IP address when there is an MDT log received from the UE. allowedValues: Not applicable | type: TceIDMappingInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tceIPAddress | This attribute indicates IP address of TCE. (See subclause 4.1.1.9.2 in TS 32.422[68]) | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tceID | This attribute indicates TCE Id. (See subclause 4.1.1.9.2 in TS 32.422[68]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNTarget | In TceIDMappingInfo datatype, this attribute indicates the PLMN where TCE resides. (See subclauses 4.1.1.9.2 and 4.9.2 in TS 32.422 [68]) In QceIdMappingInfo datatype, this attribute indicates the PLMN where QoE collection entity resides. allowedValues: N/A | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isMLBAllowed | This indicates if mobility load balancing is allowed or prohibited from source cell to target cell. If TRUE, load balancing is allowed from source cell to target cell. The source cell is identified by the name-containing NRCellCU of the NRCellRelation that contains the isMLBAllowed. The target cell is referenced by the NRCellRelation that contains this isLBAllowed. In case of isHOAllowed is FALSE, mobility load balancing is prohibited by handover from source cell to target cell. If FALSE, load balancing shall be prohibited from source cell to target cell. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NROperatorCellDU.nRCellDURef | This attribute contains the DN of the referenced NRCellDU. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| downlinkTransmitPowerRange | It indicates adjustment range (including maximum value, minimum value) of downlinkTransmitPower to optimize radio coverage. allowedValues: minValue: [0..100] maxValue: [0..100] | type: ParameterRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| antennaTiltRange | It indicates adjustment range (including maximum value, minimum value) of antennaTilt to optimize radio coverage. allowedValues: minValue: [-900..900] in unit 0.1 degree maxValue: [-900..900] in unit 0.1 degree | type: ParameterRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| antennaAzimuthRange | It indicates adjustment range (including maximum value, minimum value) of antennaAzimuth to optimize radio coverage. allowedValues: minValue: [-1800..1800] in unit 0.1 degree maxValue: [-1800..1800] in unit 0.1 degree | type: ParameterRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalTiltRange | It indicates adjustment range (including maximum value, minimum value) of digitalTilt to optimize radio coverage. allowedValues: minValue: [-900..900] in unit 0.1 degree maxValue: [-900..900] in unit 0.1 degree | type: ParameterRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalAzimuthRange | It indicates adjustment range (including maximum value, minimum value) of digitalAzimuth to optimize radio coverage. allowedValues: minValue: [-1800..1800] in unit 0.1 degree maxValue: [-1800..1800] in unit 0.1 degree | type: ParameterRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageShapeList | It indicates the coverage shape of specific sites which can be selected to optimize radio coverage. allowedValues: 0 .. 65535 | type: Integer multiplicity: 0..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| cCOControl | This attribute determines whether the centralized SON CCO Function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxValue | It indicates the maximum value of the parameter. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minValue | It indicates the minimum value of the parameter. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NROperatorCellDU.administrativeState | It indicates the administrative state of the NROperatorCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. The value of this attribute is effective only when the value of the attribute NRCellDU.administrativeState = UNLOCKED, if the value of the attribute NRCellDU.administrativeState is LOCKED or SHUTTING DOWN, the value of this attribute shall be treated same as the value of NRCellDU.administrativeState. allowedValues: LOCKED, SHUTTING DOWN, UNLOCKED. The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| bWPSetRef | Contains the DN of a BWP set (BWPSet). allowedValues: Not applicable | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| bWPList | Defines the list of DN of BWPs associated to the BWPSet. allowedValues: Not applicable | type: DN multiplicity: 0..12 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ephemerisInfoSetRef | This is the DN of EphemerisInfoSet. allowedValues: DN of the EphemerisInfoSet MOI. | type: DN multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ephemerisInfos | This is the list of Ephemeris related information. allowedValues: N/A | type: Ephemeris multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NTNFunction.nTNpLMNInfoList | It defines which PLMNs that can be served by the NR NTN cell, and which S-NSSAIs can be supported by the NR NTN cell for corresponding PLMN in case of network slicing feature is supported. The pLMNId of the first entry of the list is the PLMNId used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| NTNFunction.nTNTACList | It is the list of Tracking Area Codes (either legacy TAC or extended TAC) for NR NTN. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: NrTac multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| satelliteId | This attribute indicates satellite Id.number. It shall be formatted as a fixed 5-digit string, padding with leading digits “0” to complete a 5-digit length. allowedValues: N/A Pattern: '^[0-9]{5}$' | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| epochTime | It defines the ephemeris reference time., allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| positionVelocity | It indicates ephemeris is in format of NTN payload position and velocity state vectors. allowedValues: N/A | type: PositionVelocity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| orbital | It indicates ephemeris is in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [95]. allowedValues: N/A | type: Orbital multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| positionX | X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3. allowedValues: 0..604800 Unit: meter | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| positionY | X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3. allowedValues: 0..604800 Unit: meter | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| positionZ | X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3. allowedValues: 0..604800 Unit: meter | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| velocityVX | X, Y, Z coordinate of satellite velocity state vector in ECEF. Step of 0.06 m/s. Actual value = field value * 0.06. allowedValues: -131072..131071 Unit: meter/second | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| velocityVY | X, Y, Z coordinate of satellite velocity state vector in ECEF. Step of 0.06 m/s. Actual value = field value * 0.06. allowedValues: -131072..131071 Unit: meter/second | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| velocityVZ | X, Y, Z coordinate of satellite velocity state vector in ECEF. Step of 0.06 m/s. Actual value = field value * 0.06. allowedValues: -131072..131071 Unit: meter/second | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| semiMajorAxis | Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [95]. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3). allowedValues: 0..8589934591 Unit: meter | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| eccentricity | Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [95]. Step 1.431 * 10-8. Actual value = field value * (1.431 * 10-8). allowedValues: -524288..524287 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| periapsis | Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [95]. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8). allowedValues: 0..16777215 Unit: radian | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| longitude | Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [95]. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8). allowedValues: 0..2097151 Unit: radian | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| inclination | Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [95]. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8). allowedValues: -524288..524287 Unit: radian | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| meanAnomaly | Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [95]. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8). allowedValues: 0..16777215 Unit: radian | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| qoECollectionEntityAddress | Specifies the IP address to which the QMC reports shall be transferred. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qoECollectionEntityIdentity | Specifies a unique identity of the QoE collection entity to which the QMC reports shall be transferred. (For details, please see subclause 5 of TS 28.405[104]) allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qceIdMappingInfoList | It identifies a list of relationship between the identity of the QoE collection entity, PLMN where QoE collection entity resides, and the IP address of the QoE collection entity. allowedValues: N/A | type: QceIdMappingInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mdtUserConsentReqList | It represents a list of MDT measurement names that are subject to user consent at MDT activation. Any MDT measurement, whose name is not specified in this list, is not subject to user consent at MDT activation. allowedValues: M1, M2, M3, M4, M5, M6, M7, M8, M9, MDT_UE_LOCATION. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mappedCellIdInfoList | This attribute provides the list of mapping between geographical location and Mapped Cell ID. allowedValues: Not applicable | type: MappedCellIdInfo multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ntnGeoArea | This attribute indicates a specific geographical location mapped to Mapped Cell ID(s). allowedValues: N/A | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mappedCellId | This attribute is in format of NCGI to indicate a fixed geographical area (See subclause 16.14.5 in TS 38.300[3]). allowedValues: N/A | type: Ncgi multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelRefList | This attribute holds a DN list of MLModel (See TS 28.105 [105]). | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceFunctionRefList | This attribute holds a DN list of AIMLInferenceFunction (See TS 28.105 [105]) . | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nRECMappingRuleRef | This is the DN of NRECMappingRule. An empty value indicates the NRECMappingRule contained by parent, e.g. SubNetwok or ManagedElement, applies. allowedValues: Not applicable | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ecTimeInterval | This attribute specifies the time interval (in seconds) used by the gNB for averaging the measured energy consumption values for computing the energy cost. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ecMRInputMinimumValue | This attribute specifies the energy consumption value mapping to the minimum energy cost value. It is based on the minimum energy consumption values among all gNBs within the group for the corresponding energy cost mapping rule. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ecMRInputMaximumValue | This attribute specifies the energy consumption value mapping to the maximum energy cost value. It is based on the maximum energy consumption values among all gNBs within the group for the corresponding energy cost mapping rule. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: Void NOTE 2: The radio resource can be signaling resources (e.g. RRC connected users) or user plane resources (e.g. PRB, PRB UL, PRB DL, DRB). Different RRM Policy may be applied for different types of radio resources. E.g. RRMPolicyRatio is used for PRB resource. When the resource type is PRB the policy applies for both uplink and downlink, and ‘PRB UL’ and ‘PRB DL’ are not used. NOTE 3: Void NOTE 4: A RRM Policy can make use of the defined policy (e.g. RRMPolicyRatio) or a vendor specific RRM Policy. NOTE 5: For Global gNB Identifiers, the entries are formatted according to the pattern <mcc><mnc>-<gNBIdLength>-<gNBId>, where <mcc> is three digits, <mnc> two or three digits, <gNBIdLength> is a string containing a number n as digits, in the range 22 to 32, and <gNBId> is a string containing digits for the number 0 to 2n-1. For Global eNB Identifiers, the entries are formatted according to the pattern <mcc><mnc>-<eNBIdLength>-<eNBId>, where <mcc> is three digits, <mnc> two or three digits, <gNBIdLength> is a string containing a number m as digits, m being one of 18, 20, 21 or 22, and <eNBId> is a string containing digits for the number 0 to 2m-1. NOTE 6: The maximum number of total RIM RS sequence within 10ms is 32 regardless single or two uplink-downlink period are configured in the 10ms. NOTE 7: 1. The maximum number of consecutive uplink-downlink switching periods for repetition/near-far-functionality is 8 (the number can be either 2, 4, or 8) with near-far functionality and with repetition. 2. The maximum number of consecutive uplink-downlink switching periods for repetition is 4 (the number can be either 1, 2, or 4) without near-far functionality and with repetition only. 3. The maximum number of consecutive uplink-downlink switching periods is 2 with near-far functionality only and without repetition. NOTE 8: (for information): “Not enough mitigation” means aggressor gNB needs to increase the interference mitigation level (i.e., further interference mitigation actions) (e.g., further reducing the DL transmission power on DL symbols at aggressor side), while “Enough mitigation” means aggressor gNB keeping the current interference mitigation level unchanged (i.e., no further interference mitigation actions) (e.g., remaining the DL transmission power on DL symbols unchanged at aggressor side). NOTE 9: Value MS0P5 corresponds to 0.5 ms, MS0P625 corresponds to 0.625 ms, MS1 corresponds to 1 ms, MS1P25 corresponds to 1.25 ms, and so on. NOTE 10: RIM RS-1, RIM-RS1，RIM RS1 is equivalent to RIM-RS type 1 (see 38.211 [32], clause 7.4.1.6) RIM RS-2, RIM-RS2，RIM RS2 is equivalent to RIM-RS type 2 (see 38.211 [32], clause 7.4.1.6). |  |  |

## 4.5	Common notifications

### 4.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.111 [109], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | O | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

Note:	notifyChangedAlarm is deprecated in TS 28.111 [109].

### 4.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyMOIChanges | O | -- |
| notifyEvent | O | -- |

### 4.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

# 5	Information Model definitions for 5GC NRM

## 5.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |
| TS 25.538 [79], IOC, EASFunction | EASFunction |
| TS 25.538 [79], IOC, EESFunction | EESFunction |
| TS 25.538 [79], IOC, ECSFunction | ECSFunction |
| TS 28.538 [79], datatype, ServingLocation | ServingLocation |
| TS 28.622 [30], datatype, TimeWindow | TimeWindow |
| TS 28.708 [21], IOC, MMEFunction | MMEFunction |

## 5.2	Class diagram

### 5.2.1	Class diagram of 5GC NFs

#### 5.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of 5GC NFs definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.1.1-1 shows the 5GC NF NRM containment/naming relationship.

Figure 5.2.1.1-1: 5GC NRM containment/naming relationship

Figure 5.2.1.1-2: Transport view of AMF NRM

The Figure 5.2.1.1-3 shows the transport view of SMF NRM.

Figure 5.2.1.1-3: Transport view of SMF NRM

The Figure 5.2.1.1-4 shows the transport view of UPF NRM.

Figure 5.2.1.1-4: Transport view of UPF NRM

The Figure 5.2.1.1-5 shows the transport view of N3IWF NRM.

Figure 5.2.1.1-5: Transport view of N3IWF NRM

The Figure 5.2.1.1-6 shows the transport view of PCF NRM.

Figure 5.2.1.1-6: Transport view of PCF NRM

The Figure 5.2.1.1-7 shows the transport view of AUSF NRM.

Figure 5.2.1.1-7: Transport view of AUSF NRM

The Figure 5.2.1.1-8 shows the transport view of UDM NRM.

Figure 5.2.1.1-8: Transport view of UDM NRM

The Figure 5.2.1.1-9 shows the transport view of NRF NRM.

Figure 5.2.1.1-9: Transport view of NRF NRM

The Figure 5.2.1.1-10 shows the transport view of NSSF NRM.

Figure 5.2.1.1-10: Transport view of NSSF NRM

The Figure 5.2.1.1-11 shows the transport view of SMSF NRM.

Figure 5.2.1.1-11: Transport view of SMSF NRM

The Figure 5.2.1.1-12 shows the transport view of 5G location service related NRM.

Figure 5.2.1.1-12: Transport view of LMF NRM

The Figure 5.2.1.1-13 shows the transport view of 5G-EIR NRM.

Figure 5.2.1.1-13: Transport view of 5G-EIR NRM

The Figure 5.2.1.1-14 shows the transport view of SEPP NRM.

Figure 5.2.1.1-14: Transport view of SEPP NRM

The Figure 5.2.1.1-15 shows the NRM fragment for control of QoS monitoring per QoS flow per UE.

Figure 5.2.1.1-15: NRM fragment for control of QoS monitoring per QoS flow per UE

The Figure 5.2.1.1-16 shows the NRM fragment for control of GTP-U path QoS monitoring.

Figure 5.2.1.1-16: NRM fragment for control of GTP-U path QoS monitoring

The Figure 5.2.1.1-17 shows the NRM fragment for pre-configured 5QIs in 5GC.

Figure 5.2.1.1-17: NRM fragment for pre-configured 5QIs in 5GC

The Figure 5.2.1.1-18 shows the NRM fragment for 5QI and DSCP mapping.

Figure 5.2.1.1-18: NRM fragment for 5QI and DSCP mapping.

The Figure 5.2.1.1-19 shows the NRM fragment for predefined PCC rule.

Figure 5.2.1.1-19: NRM fragment for predefined PCC rule

The Figure 5.2.1.1-20 shows the NRM fragment for dynamically assigned 5QIs in 5GC.

Figure 5.2.1.1-20: NRM fragment for dynamically assigned 5QIs in 5GC

The Figure 5.2.1.1-21 shows the transport view of NSACF NRM.

Figure 5.2.1.1-21: Transport view of NSACF NRM

Figure 5.2.1.1-22: Transport view of NEF NRM

The Figure 5.2.1.1-23 shows the transport view of 5G DDNMF NRM.

Figure 5.2.1.1-23: Transport view of 5G DDNMF NRM

The Figure 5.2.1.1-24 shows the transport view of 5G EASDF NRM.

Figure 5.2.1.1-24: Transport view of 5G EASDF NRM

Figure 5.2.1.1-25: EcmConnectionInfo NRM

Figure 5.2.1.1-26: AFFunction NRM

Figure 5.2.1.1-27: NSSAAFFunction NRM

The Figure 5.2.1.1-28 shows the transport view of CHFFunction NRM:

Figure 5.2.1.1-28: Transport view of CHFFunction NRM

The Figure 5.2.1.1-29 shows the transport view of AANFFunction NRM:

Figure 5.2.1.1-29: Transport view of AANFFunction NRM fragment

The Figure 5.2.1.1-30 shows the transport view of GMLCFunction NRM:

Figure 5.2.1.1-30: Transport view of GMLCFunction NRM fragment

The Figure 5.2.1.1-31 shows the transport view of NWDAFFunction NRM:

Figure 5.2.1.1-31: Transport view of NWDAFFunction NRM fragment

The Figure 5.2.1.1-32 shows the transport view of TSCTSFFunction NRM:

Figure 5.2.1.1-32: Transport view of TSCTSFFunction NRM fragment

The Figure 5.2.1.1-33 shows the transport view of MBSMFFunction NRM:

Figure 5.2.1.1-33: Transport view of MB-SMF Function NRM fragment

The Figure 5.2.1.1-34 shows the transport view of MBUPFFunction NRM:

Figure 5.2.1.1-34: Transport view of MB-UPF Function NRM fragment

The Figure 5.2.1.1-35 shows the transport view of SCPFunction NRM:

Figure 5.2.1.1-35: Transport view of SCPFunction NRM fragment

Figure 5.2.1.1-36: Functional view of NWDAF NRM fragment

Figure 5.2.1.1-X: Transport view of MME NRM

#### 5.2.1.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM.

Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

Figure 5.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM.

Figure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

Figure 5.2.1.2-3: Inheritance hierarchy for IOC QFQoSMonitoringControl

Figure 5.2.1.2-4: Inheritance hierarchy for IOC GtpUPathQoSMonitoringControl

Figure 5.2.1.2-5: Inheritance hierarchy for IOC Configurable5QISet

Figure 5.2.1.2-6: Inheritance hierarchy for IOC FiveQiDscpMapping

Figure 5.2.1.2-7: Inheritance hierarchy for predefined PCC rule modeling

Figure 5.2.1.2-8: Inheritance hierarchy for IOC Dynamic5QISet

Figure 5.2.1.2-9: Inheritance hierarchy for EcmConnectionInfo

Figure 5.2.1.2-10: FiveQICharacteristics Inheritance

Figure 5.2.1.2-11: Inheritance hierarchy for NWDAF functional NRM

### 5.2.2	Class diagram of AMF Region/AMF Set

#### 5.2.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of AMF Region/AMF Set definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.2.1-1 shows the AMF Region/AMF Set NRM containment/naming relationship.

Figure 5.2.2.1-1: AMF Region/AMF Set NRM

#### 5.2.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.2.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the AMF Region/AMF Set NRM.

Figure 5.2.2.2-1: AMF Region/AMF Set Inheritance

## 5.3	Class definitions

### 5.3.1	AMFFunction

#### 5.3.1.1	Definition

This IOC represents the AMF functionality in 5GC. For more information about the AMF, see 3GPP TS 23.501 [2].

The attribute sliceExpiryInfo is used when the validity information of a network slice need to be configured. The attribute sliceExpiryInfo.pLMNInfo indicates the network slice to which the validity information applies.

#### 5.3.1.2	Attributes

The AMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| cNSIIdList | CM | T | F | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| amfInfo | M | T | T | F | T |
| nTNPLMNRestrictionsList | M | T | T | F | T |
| satelliteCoverageInfoList | CM | T | T | F | T |
| sliceExpiryInfo | CM | T | T | F | T |
| satelliteBackhaulInfoList | CM | T | T | F | T |
| mdtUserConsentReqList | CM | T | T | F | T |
| mappedCellIdInfoList | CO | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFSetRef | M | T | T | F | T |

#### 5.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cNSIIdList S | Condition: Network slicing feature is supported and the NSI ID is configured for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. |
| satelliteCoverageInfoList S | Condition: Present if 5G NR satellite access is used |
| sliceExpiryInfo S | Condition: Network slicing expiration feature is supported. |
| satelliteBackhaulInfoList S | Condition: Present if AMF supports the reporting of satellite backhaul information and indicating the satellite backhaul category change to SMF. |
| mdtUserConsentReqList | Condition: MDT Function is supported. |
| mappedCellIdInfoList S | Condition: Present if 5G NR satellite access is used |

#### 5.3.1.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.2	SMFFunction

#### 5.3.2.1	Definition

This IOC represents the SMF function in 5GC. For more information about the SMF, see 3GPP TS 23.501 [2].

#### 5.3.2.2	Attributes

The SMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| cNSIIdList | CM | T | F | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| smfInfo | M | T | T | F | T |
| dnaiSatelliteMappingList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 5.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cNSIIdList CM S | Condition: Network slicing feature is supported. |
| dnaiSatelliteMappingList S | Condition: Present if SMF supports selecting UPF on-board satellite |

#### 5.3.2.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.3	UPFFunction

#### 5.3.3.1	Definition

This IOC represents the UPF function in 5GC. For more information about the UPF, see TS 23.501 [2].

#### 5.3.3.2	Attributes

The UPFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| cNSIIdList | CM | T | T | F | T |
| energySavingControl | CM | T | T | F | T |
| energySavingState | CM | T | F | F | T |
| managedNFProfile | M | T | T | F | T |
| supportedBMOList | O | T | T | F | T |
| upfInfo | M | T | T | F | T |

#### 5.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cNSIIdList CM S | The condition is "network slicing feature is supported". |
| energySavingControl CM S | Condition: The energy saving feature is supported for the edge UPF. |
| energySavingState CM S | Condition: The energy saving feature is supported for the edge UPF. |

#### 5.3.3.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.4	N3IWFFunction

#### 5.3.4.1	Definition

This IOC represents the N3IWF function which is used to enable non-3GPP access networks connected to the 5GC. For more information about the N3IWF, see TS 23.501 [2].

#### 5.3.4.2	Attributes

The N3IWFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.4.3	Attribute constraints

None.

#### 5.3.4.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.5	PCFFunction

#### 5.3.5.1	Definition

This IOC represents the PCF function in 5GC. For more information about the PCF, see TS 23.501 [2].

#### 5.3.5.2	Attributes

The PCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| supportedBMOList | O | T | T | F | T |
| pcfInfo | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |
| predefinedPccRuleSetRefs | O | T | T | F | T |

#### 5.3.5.3	Attribute constraints

None.

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.6	AUSFFunction

#### 5.3.6.1	Definition

This IOC represents the AUSF function in 5GC. For more information about the AUSF, see TS 23.501 [2].

#### 5.3.6.2	Attributes

The AUSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| ausfInfo | M | T | T | F |  |

#### 5.3.6.3	Attribute constraints

None.

#### 5.3.6.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.7	UDMFunction

#### 5.3.7.1	Definition

This IOC represents the UDM function in 5GC. For more information about the UDM, see TS 23.501 [2].

#### 5.3.7.2	Attributes

The UDMFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| eCSAddrConfigInfo | O | T | T | F | T |
| udmInfo | O | T | T | F | T |
| mdtUserConsentReqList | CM | T | T | F | T |

#### 5.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mdtUserConsentReqList | Condition: MDT Function is supported. |

#### 5.3.7.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.8	UDRFunction

#### 5.3.8.1	Definition

This IOC represents the UDR function in 5GC. For more information about the UDR, see TS 23.501 [2].

#### 5.3.8.2	Attributes

The UDRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| udrInfo | O | T | T | F | T |

#### 5.3.8.3	Attribute constraints

None.

#### 5.3.8.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.9	UDSFFunction

#### 5.3.9.1	Definition

This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in 3GPP TS 23.501 [2]. For more information about the UDSF, see TS 23.501 [2].

#### 5.3.9.2	Attributes

The UDSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| udsfInfo | M | T | T | F | T |

#### 5.3.9.3	Attribute constraints

None.

#### 5.3.9.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.10	NRFFunction

#### 5.3.10.1	Definition

This IOC represents the NRF function in 5GC. For more information about the NRF, see TS 23.501 [2].

#### 5.3.10.2	Attributes

The NRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| nFProfileList | CM | T | T | F | T |
| cNSIIdList | CO | T | T | F | T |
| nrfInfo | O | T | T | F | T |

#### 5.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nFProfileList S | Condition: NF profile is registered and deregistered by management system. |
| cNSIIdList S | Condition: Network slicing feature is supported and the NSI ID is configured for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. |

#### 5.3.10.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.11	NSSFFunction

#### 5.3.11.1	Definition

This IOC represents the NSSF function in 5GC. For more information about the NSSF, see TS 23.501 [2].

#### 5.3.11.2	Attributes

The NSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| cNSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.11.3	Attribute constraints

None.

#### 5.3.11.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.12	AFFunction

#### 5.3.12.1	Definition

This IOC represents the trust AF function in 5GC. For more information about the AF, see TS 23.501 [2].

#### 5.3.12.2	Attributes

The AFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| trustAfInfo | M | T | T | F | T |

#### 5.3.12.3	Attribute constraints

None.

#### 5.3.12.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.13	DNFunction

#### 5.3.13.1	Definition

This IOC is defined for only purpose to describe the IOCs representing Data Network (DN) interaction interface with 5GC (i.e. EP_N6). It has no any attributes defined.

### 5.3.14	SMSFFunction

#### 5.3.14.1	Definition

This IOC represents the SMSF function defined in TS 23.501 [2].

#### 5.3.14.2	Attributes

The SMSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| smsfInfo | O | T | T | F | T |

#### 5.3.14.3	Attribute constraints

None.

#### 5.3.14.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.15	LMFFunction

#### 5.3.15.1	Definition

This IOC represents the LMF function defined in TS 23.501 [2].

#### 5.3.15.2	Attributes

The LMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| lmfInfo | O | T | T | F | T |
| mappedCellIdInfoList | CO | T | T | F | T |
| ephemerisInfos | CO | T | T | F | T |
| trpInfoList | CO | T | T | F | T |

#### 5.3.15.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mappedCellIdInfoList | Condition: Present if 5G NR satellite access is used |
| ephemerisInfos | Condition: Present if 5G NR satellite access is used |
| trpInfoList | Condition: Present if 5G NR satellite access is used |

#### 5.3.15.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.16	NGEIRFunction

#### 5.3.16.1	Definition

This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see 3GPP TS 23.501 [2].

#### 5.3.16.2	Attributes

The NGEIRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList S | Condition: network slicing feature is supported. |

#### 5.3.16.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.17	SEPPFunction

#### 5.3.17.1	Definition

This IOC represents the SEPP function which support message filtering and policing on inter-PLMN control plane interface. For more information about the SEPP, see TS 23.501 [2].

#### 5.3.17.2	Attributes

The SEPPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | F | T | T |
| sEPPType | M | T | F | T | T |
| sEPPId | M | T | F | T | T |
| fqdn | M | T | T | F | T |
| seppInfo | M | T | T | F | T |

#### 5.3.17.3	Attribute constraints

None.

#### 5.3.17.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.18	NWDAFFunction

#### 5.3.18.1	Definition

This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see TS 23.501 [2]. Several attributes (including "nwdafInfo", " administrativeState " and "ManagedNFProfile.servingScope") are used to control the functionalities (identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF instance.

The attribute "ManagedNFProfile.servingScope" is used to represent specified certain geographical area(s) can be served by the NWDAF instance.

The attribute "NwdafInfo.taiList" and " NwdafInfo.taiRangeList " is used to represent specified certain tracking area(s) can be served by the NWDAF instance.

#### 5.3.18.2	Attributes

The NWDAFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| networkSliceInfoList | CM | T | T | T | T |
| nwdafInfo | M | T | T | F | T |
| administrativeState | O | T | T | F | T |
| nwdafLogicalFuncSupported | O | T | F | F | T |

#### 5.3.18.3	Attribute constraints

| Name | Definition |
| --- | --- |
| networkSliceInfoList | Condition: Network slicing feature is supported. |

#### 5.3.18.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.19	EP_N2

#### 5.3.19.1	Definition

This IOC represents the N2 interface between (R)AN and AMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.19.2	Attributes

The EP_N2 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.19.3	Attribute constraints

None.

#### 5.3.19.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.20	EP_N3

#### 5.3.20.1	Definition

This IOC represents the N3 interface between (R)AN and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.20.2	Attributes

The EP_N3 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 5.3.20.3	Attribute constraints

None.

#### 5.3.20.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.21	EP_N4

#### 5.3.21.1	Definition

This IOC represents the N4 interface between SMF and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.21.2	Attributes

The EP_N4 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.21.3	Attribute constraints

None.

#### 5.3.21.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.22	EP_N5

#### 5.3.22.1	Definition

This IOC represents the N5 interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.22.2	Attributes

The EP_N5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.22.3	Attribute constraints

None.

#### 5.3.22.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.23	EP_N6

#### 5.3.23.1	Definition

This IOC represents the N6 interface between UPF and DN, which is defined in 3GPP TS 23.501 [2].

#### 5.3.23.2	Attributes

The EP_N6 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.23.3	Attribute constraints

None.

#### 5.3.23.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.24	EP_N7

#### 5.3.24.1	Definition

This IOC represents the N7 interface between SMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.24.2	Attributes

The EP_N7 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.24.3	Attribute constraints

None.

#### 5.3.24.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.25	EP_N8

#### 5.3.25.1	Definition

This IOC represents the N8 interface between AMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.25.2	Attributes

The EP_N8 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.25.3	Attribute constraints

None.

#### 5.3.25.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.26	EP_N9

#### 5.3.26.1	Definition

This IOC represents the N7 interface between two UPFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.26.2	Attributes

The EP_N9 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.26.3	Attribute constraints

None.

#### 5.3.26.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.27	EP_N10

#### 5.3.27.1	Definition

This IOC represents the N10 interface between SMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.27.2	Attributes

The EP_N10 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.27.3	Attribute constraints

None.

#### 5.3.27.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.28	EP_N11

#### 5.3.28.1	Definition

This IOC represents the N11 interface between AMF and SMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.28.2	Attributes

The EP_N11 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.28.3	Attribute constraints

None.

#### 5.3.28.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.29	EP_N12

#### 5.3.29.1	Definition

This IOC represents the N12 interface between AMF and AUSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.29.2	Attributes

The EP_N12 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.29.3	Attribute constraints

None.

#### 5.3.29.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.30	EP_N13

#### 5.3.30.1	Definition

This IOC represents the N13 interface between AUSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.30.2	Attributes

The EP_N13 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.30.3	Attribute constraints

None.

#### 5.3.30.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.31	EP_N14

#### 5.3.31.1	Definition

This IOC represents the N14 interface between two AMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.31.2	Attributes

The EP_N14 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.31.3	Attribute constraints

None.

#### 5.3.31.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.32	EP_N15

#### 5.3.32.1	Definition

This IOC represents the N15 interface between AMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.32.2	Attributes

The EP_N15 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.32.3	Attribute constraints

None.

#### 5.3.32.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.33	EP_N16

#### 5.3.33.1	Definition

This IOC represents the N16 interface between two SMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.33.2	Attributes

The EP_N16 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.33.3	Attribute constraints

None.

#### 5.3.33.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.34	EP_N17

#### 5.3.34.1	Definition

This IOC represents the N17 interface between AMF and 5G-EIR, which is defined in 3GPP TS 23.501 [2].

#### 5.3.34.2	Attributes

The EP_N17 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.34.3	Attribute constraints

None.

#### 5.3.34.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.35	EP_N20

#### 5.3.35.1	Definition

This IOC represents the N20 interface between AMF and SMSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.35.2	Attributes

The EP_N20 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.35.3	Attribute constraints

None.

#### 5.3.35.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.36	EP_N21

#### 5.3.36.1	Definition

This IOC represents the N21 interface between SMSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.36.2	Attributes

The EP_N21 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.36.3	Attribute constraints

None.

#### 5.3.36.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.37	EP_N22

#### 5.3.37.1	Definition

This IOC represents the N22 interface between AMF and NSSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.37.2	Attributes

The EP_N22 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.37.3	Attribute constraints

None.

#### 5.3.37.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.38	EP_N26

#### 5.3.38.1	Definition

This IOC represents the N26 interface between AMF and MME, which is defined in 3GPP TS 23.501 [2].

#### 5.3.38.2	Attributes

The EP_N26 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.38.3	Attribute constraints

None.

#### 5.3.38.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.39	Void

### 5.3.40	Void

### 5.3.41	EP_S5C

#### 5.3.41.1	Definition

This IOC represents the S5-C interface between SGW and SMF/PGW-C, which is defined in 3GPP TS 23.501 [2].

#### 5.3.41.2	Attributes

The EP_S5C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.41.3	Attribute constraints

None.

#### 5.3.41.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.42	EP_S5U

#### 5.3.42.1	Definition

This IOC represents the S5-U interface between SGW and UPF/PGW-U, which is defined in 3GPP TS 23.501 [2].

#### 5.3.42.2	Attributes

The EP_S5U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.42.3	Attribute constraints

None.

#### 5.3.42.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.43	EP_Rx

#### 5.3.43.1	Definition

This IOC represents the Rx interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.43.2	Attributes

The EP_Rx IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.43.3	Attribute constraints

None.

#### 5.3.43.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.44	EP_MAP_SMSC

#### 5.3.44.1	Definition

This IOC represents the MAP interface between SMSF and MSC-IWMSC/GMSC, which is defined in 3GPP TS 23.040 [22].

#### 5.3.44.2	Attributes

The EP_MAP_SMSC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.44.3	Attribute constraints

None.

#### 5.3.44.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.45	EP_NL1

#### 5.3.45.1	Definition

This IOC represents the NL1 interface between AMF and LMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.45.2	Attributes

The EP_NL1 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.45.3	Attribute constraints

None.

#### 5.3.45.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.46	EP_NL2

#### 5.3.46.1	Definition

This IOC represents the NL2 interface between AMF and GMLC, which is defined in TS 23.273 [93].

#### 5.3.46.2	Attributes

The EP_NL2 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.46.3	Attribute constraints

None.

#### 5.3.46.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.47	EP_N27

#### 5.3.47.1	Definition

This IOC represents an end point of N27 interface between vNRF and hNRF, which is defined in 3GPP TS 29.510 [23].

#### 5.3.47.2	Attributes

The EP_N27 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.47.3	Attribute constraints

None.

#### 5.3.47.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.48	EP_N31

#### 5.3.48.1	Definition

This IOC represents an end point of N31 interface between vNSSF and hNSSF, which is defined in 3GPP TS 29.531 [24].

#### 5.3.48.2	Attributes

The EP_N31 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.48.3	Attribute constraints

None.

#### 5.3.48.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.49	ExternalNRFFunction

#### 5.3.49.1	Definition

This IOC represents external NRF function controlled by another management domain. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.49.2	Attributes

The ExternalNRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.49.3	Attribute constraints

None.

#### 5.3.49.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.50	ExternalNSSFFunction

#### 5.3.50.1	Definition

This IOC represents external NSSF function controlled by another management domain. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.50.2	Attributes

The ExternalNSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.50.3	Attribute constraints

None.

#### 5.3.50.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.51	AMFSet

#### 5.3.51.1	Definition

This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice. For more information about the AMF Set, see 3GPP TS 23.501 [2].

#### 5.3.51.2	Attributes

The AMFSet IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFSetId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFRegionRef | M | T | T | F | T |
| aMFSetMemberList | M | T | T | F | T |

#### 5.3.51.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList S | Condition: Network slicing feature is supported. |

#### 5.3.51.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.52	AMFRegion

#### 5.3.52.1	Definition

This IOC represents the AMF Region which consists one or multiple AMF Sets. For more information about the AMF Region, see 3GPP TS 23.501 [2].

#### 5.3.52.2	Attributes

The AMFRegion IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFRegionId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFSetListRef | M | T | T | F | T |

#### 5.3.52.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList S | Condition: Network slicing feature is supported. |

#### 5.3.52.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.53	ExternalAMFFunction

#### 5.3.53.1	Definition

This IOC represents an external AMF functionality used in EN-DC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.53.2	Attributes

The ExternalAMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |

#### 5.3.53.3	Attribute constraints

None.

#### 5.3.53.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.54	ManagedNFProfile <<dataType>>

#### 5.3.54.1	Definition

This data type represents a Profile definition of a Managed NF (See TS 23.501 [2]).

#### 5.3.54.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfInstanceId | M | T | F | T | F |
| nfType | M | T | F | F | F |
| heartBeatTimer | O | T | T | F | T |
| hostAddr | M | T | T | F | T |
| authzInfo | O | T | T | F | T |
| allowedPLMNs | O | T | T | F | T |
| sNPNList | O | T | T | F | T |
| allowedSNPNs | O | T | T | F | T |
| allowedNfTypes | O | T | T | F | T |
| allowedNfDomains | O | T | T | F | T |
| allowedNSSAIs | O | T | T | F | T |
| locality | O | T | T | F | T |
| capacity | O | T | T | F | T |
| recoveryTime | O | T | F | F | F |
| nfServicePersistence | O | T | F | F | F |
| nfSetIdList | O | T | T | F | T |
| nfProfileChangesSupportInd | O | T | F | F | F |
| defaultNotificationSubscriptions | O | T | F | F | F |
| servingScope | O | T | T | F | T |
| lcHSupportInd | O | T | F | F | T |
| olcHSupportInd | O | T | F | F | T |
| nfSetRecoveryTimeList | O | T | F | F | T |
| serviceSetRecoveryTimeList | O | T | F | F | F |
| scpDomains | O | T | T | F | T |
| vendorId | O | T | T | F | T |
| interPlmnFqdn | O | T | T | F | T |

#### 5.3.54.3	Attribute constraints

None.

#### 5.3.54.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.55	HostAddr <<choice>>

#### 5.3.55.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.55.1-1: HostAddr <<choice>> for data types.

Figure 5.3.55.1-1: HostAddr <<choice>> for data types

NOTE:	The IpAddress can be IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38]). Refer TS 23.003 [13] for Fqdn.

### 5.3.56	Void

### 5.3.57	Void

### 5.3.58	AusfInfo <<dataType>>

#### 5.3.58.1	Definition

This data type represents the information of an AUSF NF Instance (see TS 29.510 [23]).

#### 5.3.58.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | T | F |
| supiRanges | O | T | T | F | T |
| routingIndicators | O | T | T | F | T |
| suciInfos | O | T | T | F | T |

#### 5.3.58.3	Attribute constraints

None.

#### 5.3.58.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.59	Void

### 5.3.60	Void

### 5.3.61	Void

### 5.3.62	EP_N32

#### 5.3.62.1	Definition

This IOC represents an end point of N32 interface between cSEPP and pSEPP, which is defined in 3GPP TS 23.501 [2] and 33.501 [52].

#### 5.3.62.2	Attributes

The EP_N32 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| remotePlmnId | M | T | T | F | T |
| remoteSeppAddress | M | T | T | F | T |
| remoteSeppId | O | T | T | F | T |
| n32cParas | O | T | T | F | T |
| n32fPolicy | O | T | T | F | T |
| withIPX | M | T | T | F | T |

#### 5.3.62.3	Attribute constraints

None.

#### 5.3.62.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.63	ExternalSEPPFunction

#### 5.3.63.1	Definition

This IOC represents the properties, known by the management function, of a SEPP managed by another management function. For more information about SEPPFunction, see subclause 5.3.17.

#### 5.3.63.2	Attributes

The ExternalSEPPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | F | F | T |
| sEPPId | M | T | F | T | T |
| fqdn | M | T | F | F | T |

#### 5.3.63.3	Attribute constraints

None.

#### 5.3.63.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.64	SEPPFunction <<ProxyClass>>

#### 5.3.64.1	Definition

This IOC represents an <<IOC>>SEPPFunction and <<IOC>>ExternalSEPPFunction.

#### 5.3.64.2	Attributes

See that defined in <<IOC>>SEPPFunction and <<IOC>>ExternalSEPPFunction.

#### 5.3.64.3	Attribute constraints

See respective IOCs.

#### 5.3.64.4	Notifications

See respective IOCs.

### 5.3.65	NEFFunction

#### 5.3.65.1	Definition

This IOC represents the NEF function in 5GC. For more information about the NEF, see TS 23.501 [2].

#### 5.3.65.2	Attributes

The NEFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| capabilityList | M | T | T | F | T |
| isCAPIFSup | M | T | F | T | F |
| nefInfo | O | T | T | F | T |

#### 5.3.65.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList S | Condition: Network slicing feature is supported. |

#### 5.3.65.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.66	SCPFunction

#### 5.3.67.1	Definition

This IOC represents a Service Communication Proxy, which is defined in  TS 23.501 [2].

#### 5.3.67.2	Attributes

The SCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedFuncList | M | T | T | F | T |
| address | M | T | T | F | T |
| scpInfo | M | T | T | F | T |

#### 5.3.67.3	Attribute constraints

None.

#### 5.3.67.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.68	SupportedFunction <<dataType>>

#### 5.3.68.1	Definition

This dataType represents a functionality supported by a SCP, which is defined in 3GPP TS 23.501 [2].

#### 5.3.68.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| function | M | T | T | F | T |
| policy | O | T | T | F | T |

#### 5.3.68.3	Attribute constraints

None.

#### 5.3.68.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.69	CommModel <<dataType>>

#### 5.3.69.1	Definition

This data type represents a communication model definition (See TS 23.501 [22]).

#### 5.3.69.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | M | T | T | F | T |
| commModelType | M | T | T | F | T |
| targetNFServiceList | M | T | T | F | T |
| commModelConfiguration | M | T | T | F | T |

#### 5.3.69.3	Attribute constraints

None

#### 5.3.69.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.70	QFQoSMonitoringControl

#### 5.3.70.1	Definition

This IOC specifies the capabilities and properties for control of QoS monitoring per QoS flow per UE. For more information about QoS monitoring per QoS flow per UE, see 3GPP TS 23.501 [2].

If the QoS monitoring per QoS flow per UE is enabled, the SMF requests the PSA UPF to perform the QoS monitoring per QoS flow per UE based on the attributes of the instance of this IOC.

#### 5.3.70.2	Attributes

The QFQoSMonitoringControl IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qFQoSMonitoringState | M | T | T | F | T |
| qFMonitoredSNSSAIs | M | T | T | F | T |
| qFMonitored5QIs | M | T | T | F | T |
| isEventTriggeredQFMonitoringSupported | M | T | F | F | T |
| isPeriodicQFMonitoringSupported | M | T | F | F | T |
| isSessionReleasedQFMonitoringSupported | M | T | F | F | T |
| qFPacketDelayThresholds | CM | T | T | F | T |
| qFMinimumWaitTime | CM | T | T | F | T |
| qFMeasurementPeriod | CM | T | T | F | T |
| qFMonitoredSatelliteBackhaulCategories | O | T | T | F | T |

#### 5.3.70.3	Attribute constraints

| Name | Definition |
| --- | --- |
| qFPacketDelayThresholds S | Condition: isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “Yes”. |
| qFMinimumWaitTime S | Condition: isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “Yes”. |
| qFMeasurementPeriod S | Condition: isPeriodicQFMonitoringSupported attribute of the same MOI is set to “Yes”. |

#### 5.3.70.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.71	QFPacketDelayThresholdsType <<dataType>>

#### 5.3.71.1	Definition

This data type specifies the thresholds for reporting the packet delay for QoS monitoring per QoS flow per UE, see TS 29.244 [56].

#### 5.3.71.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| thresholdDl | M | T | T | F | T |
| thresholdUl | M | T | T | F | T |
| thresholdRtt | M | T | T | F | T |

#### 5.3.71.3	Attribute constraints

None

#### 5.3.71.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.72	GtpUPathQoSMonitoringControl

#### 5.3.72.1	Definition

This IOC specifies the capabilities and properties for control of GTP-U path QoS monitoring. For more information about the GTP-U path QoS monitoring, see 3GPP TS 23.501 [2].

If the GTP-U path QoS monitoring is enabled, the SMF requests the UPF(s) and NG-RAN to perform the GTP-U path QoS monitoring based on the attributes of the instance of this IOC.

#### 5.3.72.2	Attributes

The GtpUPathQoSMonitoringControl IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gtpUPathQoSMonitoringState | M | T | T | F | T |
| gtpUPathMonitoredSNSSAIs | M | T | T | F | T |
| monitoredDSCPs | M | T | T | F | T |
| isEventTriggeredGtpUPathMonitoringSupported | M | T | F | F | T |
| isPeriodicGtpUMonitoringSupported | M | T | F | F | T |
| isImmediateGtpUMonitoringSupported | M | T | F | F | T |
| gtpUPathDelayThresholds | CM | T | T | F | T |
| gtpUPathMinimumWaitTime | CM | T | T | F | T |
| gtpUPathMeasurementPeriod | CM | T | T | F | T |

#### 5.3.72.3	Attribute constraints

| Name | Definition |
| --- | --- |
| gtpUPathDelayThresholds S | Condition: isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “Yes”. |
| gtpUPathMinimumWaitTime S | Condition: isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “Yes”. |
| gtpUPathMeasurementPeriod S | Condition: isPeriodicGtpUMonitoringSupported attribute of the same MOI is set to “Yes”. |

#### 5.3.72.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.73	GtpUPathDelayThresholdsType <<dataType>>

#### 5.3.73.1	Definition

This data type specifies the thresholds for reporting the packet delay for GTP-U path QoS monitoring, see TS 29.244 [56].

#### 5.3.73.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| n3AveragePacketDelayThreshold | M | T | T | F | T |
| n3MinPacketDelayThreshold | M | T | T | F | T |
| n3MaxPacketDelayThreshold | M | T | T | F | T |
| n9AveragePacketDelayThreshold | M | T | T | F | T |
| n9MinPacketDelayThreshold | M | T | T | F | T |
| n9MaxPacketDelayThreshold | M | T | T | F | T |

#### 5.3.73.3	Attribute constraints

None

#### 5.3.73.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.74	Void

### 5.3.75	Configurable5QISet

#### 5.3.75.1	Definition

This IOC specifies the pre-configured 5QIs, including their QoS characteristics,  see TS 23.501 [2].

Sets are referenced by attributes (configurable5QISetRef) in applicable MOIs. For consistency it is recommended that referenced 5QI sets be defined within the same subtree.

#### 5.3.75.2	Attributes

The Configurable5QISet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) .

#### 5.3.75.3	Attribute constraints

None.

#### 5.3.75.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.76	FiveQICharacteristics

#### 5.3.76.1	Definition

This specifies the 5QI value and the corresponding QoS characteristics for a 5QI.

#### 5.3.76.2	Attributes

The FiveQICharacteristics IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQIValue | M | T | T/F (NOTE) | F | T |
| resourceType | M | T | T/F (NOTE) | F | T |
| priorityLevel | O | T | T/F (NOTE) | F | T |
| packetDelayBudget | O | T | T/F (NOTE) | F | T |
| packetErrorRate | O | T | T/F (NOTE) | F | T |
| averagingWindow | O | T | T/F (NOTE) | F | T |
| maximumDataBurstVolume | O | T | T/F (NOTE) | F | T |
| NOTE: The isWritable qualifier is "T" if the attribute 1) describes a 5QI in Configurable5QISet MOI, or 2) describes a 5QI in Dynamic5QISet MOI which is associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed; The isWritable qualifier is "F" otherwise. |  |  |  |  |  |

#### 5.3.76.3	Attribute constraints

None

#### 5.3.76.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.77	PacketErrorRate <<dataType>>

#### 5.3.77.1	Definition

This data type specifies the Packet Error Rate of a configurable 5QI.

#### 5.3.77.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scalar | M | T | T/F (NOTE) | F | T |
| exponent | M | T | T/F (NOTE) | F | T |
| NOTE: The isWritable qualifier is “T” if the attribute 1) describes a 5QI in Configurable5QISet MOI, or 2) describes a 5QI in Dynamic5QISet MOI which is associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed; The isWritable qualifier is "F" otherwise. |  |  |  |  |  |

#### 5.3.77.3	Attribute constraints

None

#### 5.3.77.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.78	FiveQiDscpMappingSet

#### 5.3.78.1	Definition

This IOC specifies the set of mapping between 5QIs and DSCP.

#### 5.3.78.2	Attributes

The FiveQiDscpMappingSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQiDscpMappingList | M | T | T | F | T |

#### 5.3.78.3	Attribute constraints

None.

#### 5.3.78.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.79	FiveQiDscpMapping <<dataType>>

#### 5.3.79.1	Definition

This data type specifies the mapping between 5QIs to DSCP.

#### 5.3.79.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQIValues | M | T | T | F | T |
| dscp | M | T | T | F | T |

#### 5.3.79.3	Attribute constraints

None

#### 5.3.79.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.80	PredefinedPccRuleSet

#### 5.3.80.1	Definition

This IOC specifies the predefined PCC rules, which are configured to SMF and referenced by PCF, see 3GPP TS 23.503 [59].

#### 5.3.80.2	Attributes

The PredefinedPccRuleSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| predefinedPccRules | M | T | T | F | T |

#### 5.3.80.3	Attribute constraints

None.

#### 5.3.80.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.81	PccRule <<dataType>>

#### 5.3.81.1	Definition

This data type specifies the PCC rule, see TS 29.512 [60].

#### 5.3.81.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pccRuleId | M | T | T | F | T |
| flowInfoList | CM | T | T | F | T |
| applicationId | CM | T | T | F | T |
| appDescriptor | O | T | T | F | T |
| contentVersion | O | T | T | F | T |
| precedence | CM | T | T | F | T |
| afSigProtocol | O | T | T | F | T |
| isAppRelocatable | O | T | T | F | T |
| isUeAddrPreserved | O | T | T | F | T |
| qosData | M | T | T | F | T |
| altQosParams | O | T | T | F | T |
| trafficControlData | M | T | T | F | T |
| conditionData | O | T | T | F | T |
| tscaiInputUl | O | T | T | F | T |
| tscaiInputDl | O | T | T | F | T |

#### 5.3.81.3	Attribute constraints

| Name | Definition |
| --- | --- |
| flowInfoList S | Condition: The applicationId is not supported. |
| applicationId S | Condition: The flowInfoList is not supported. |
| precedence S | Condition: The flowInfoList is provided. |

#### 5.3.81.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.82	FlowInformation <<dataType>>

#### 5.3.82.1	Definition

This data type specifies the flow information of a PCC rule.

#### 5.3.82.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| flowDescription | M | T | T | F | T |
| ethFlowDescription | M | T | T | F | T |
| packFiltId | M | T | T | F | T |
| packetFilterUsage | M | T | T | F | T |
| tosTrafficClass | M | T | T | F | T |
| spi | M | T | T | F | T |
| flowLabel | O | T | T | F | T |
| flowDirection | M | T | T | F | T |

#### 5.3.82.3	Attribute constraints

None

#### 5.3.82.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.83	EthFlowDescription <<dataType>>

#### 5.3.83.1	Definition

This data type describes an Ethernet flow.

#### 5.3.83.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| destMacAddr | M | T | T | F | T |
| ethType | M | T | T | F | T |
| fDesc | CM | T | T | F | T |
| fDir | M | T | T | F | T |
| sourceMacAddr | M | T | T | F | T |
| vlanTags | M | T | T | F | T |
| srcMacAddrEnd | O | T | T | F | T |
| destMacAddrEnd | O | T | T | F | T |

#### 5.3.83.3	Attribute constraints

| Name | Definition |
| --- | --- |
| fDesc S | Condition: The ethType is IP. |

#### 5.3.83.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.84	QoSData <<dataType>>

#### 5.3.84.1	Definition

This data type specifies the QoS control policy data for a service flow of a PCC rule.

#### 5.3.84.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qosId | M | T | T | F | T |
| fiveQIValue | M | T | T | F | T |
| maxbrUl | O | T | T | F | T |
| maxbrDl | O | T | T | F | T |
| gbrUl | O | T | T | F | T |
| gbrDl | O | T | T | F | T |
| arp | M | T | T | F | T |
| qosNotificationControl | O | T | T | F | T |
| reflectiveQos | O | T | T | F | T |
| sharingKeyDl | O | T | T | F | T |
| sharingKeyUl | O | T | T | F | T |
| maxPacketLossRateDl | O | T | T | F | T |
| maxPacketLossRateUl | O | T | T | F | T |
| extMaxDataBurstVol | O | T | T | F | T |

#### 5.3.84.3	Attribute constraints

None.

#### 5.3.84.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.85	ARP <<dataType>>

#### 5.3.85.1	Definition

This data type specifies the allocation and retention priority of a QoS control policy.

#### 5.3.85.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priorityLevel | M | T | T | F | T |
| preemptCap | M | T | T | F | T |
| preemptVuln | M | T | T | F | T |

#### 5.3.85.3	Attribute constraints

None

#### 5.3.85.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.86	TrafficControlData <<dataType>>

#### 5.3.86.1	Definition

This data type specifies the traffic control data for a service flow of a PCC rule.

#### 5.3.86.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tcId | M | T | T | F | T |
| flowStatus | M | T | T | F | T |
| redirectInfo | O | T | T | F | T |
| addRedirectInfo | O | T | T | F | T |
| muteNotif | O | T | T | F | T |
| trafficSteeringPolIdDl | O | T | T | F | T |
| trafficSteeringPolIdUl | O | T | T | F | T |
| routeToLocs | M | T | T | F | T |
| traffCorreInd | O | T | T | F | T |
| upPathChgEvent | O | T | T | F | T |
| steerFun | O | T | T | F | T |
| steerModeDl | O | T | T | F | T |
| steerModeUl | O | T | T | F | T |
| mulAccCtrl | O | T | T | F | T |
| sNSSAIList | O | T | T | F | T |

#### 5.3.86.3	Attribute constraints

None

#### 5.3.86.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.87	RedirectInformation <<dataType>>

#### 5.3.87.1	Definition

This data type specifies the redirect information for traffic control in the PCC rule.

#### 5.3.87.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| redirectEnabled | M | T | T | F | T |
| redirectAddressType | M | T | T | F | T |
| redirectServerAddress | M | T | T | F | T |

#### 5.3.87.3	Attribute constraints

None

#### 5.3.87.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.88	RouteToLocation <<dataType>>

#### 5.3.88.1	Definition

This data type specifies a list of location which the traffic shall be routed to for the AF request.

#### 5.3.88.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnai | M | T | T | F | T |
| routeInfo | CM | T | T | F | T |
| routeProfId | CM | T | T | F | T |

#### 5.3.88.3	Attribute constraints

| Name | Definition |
| --- | --- |
| routeInfo S | Condition: The routeProfId is not supported. |
| routeProfId S | Condition: The routeInfo is not supported. |

#### 5.3.88.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.89	RouteInformation <<dataType>>

#### 5.3.89.1	Definition

This data type specifies the traffic routing information.

#### 5.3.89.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | CM | T | T | F | T |
| ipv6Addr | CM | T | T | F | T |
| portNumber | M | T | T | F | T |

#### 5.3.89.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4Addr S | Condition: The ipv6Addr is not supported. |
| ipv6Addr S | Condition: The ipv4Addr is not supported. |

#### 5.3.89.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.90	UpPathChgEvent <<dataType>>

#### 5.3.90.1	Definition

This data type specifies the information about the AF subscriptions of the UP path change, see TS 29.512 [60].

#### 5.3.90.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationUri | M | T | T | F | T |
| notifCorreId | M | T | T | F | T |
| dnaiChgType | M | T | T | F | T |
| afAckInd | O | T | T | F | T |

#### 5.3.90.3	Attribute constraints

None

#### 5.3.90.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.91	SteeringMode <<dataType>>

#### 5.3.91.1	Definition

This data type specifies the traffic distribution rule, see TS 29.512 [60].

#### 5.3.91.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| steerModeValue | M | T | T | F | T |
| active | CM | T | T | F | T |
| standby | O | T | T | F | T |
| threeGLoad | CM | T | T | F | T |
| prioAcc | CM | T | T | F | T |

#### 5.3.91.3	Attribute constraints

| Name | Definition |
| --- | --- |
| active S | Condition: The steerModeValue supports “ACTIVE_STANDBY”. |
| threeGLoad S | Condition: The steerModeValue supports “LOAD_BALANCING”. |
| prioAcc S | Condition: The steerModeValue supports “PRIORITY_BASED”. |

#### 5.3.91.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.92	ConditionData  <<dataType>>

#### 5.3.92.1	Definition

This data type specifies the condition data for a PCC rule.

#### 5.3.92.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| condId | M | T | T | F | T |
| activationTime | O | T | T | F | T |
| deactivationTime | O | T | T | F | T |
| accessType | O | T | T | F | T |
| ratType | O | T | T | F | T |

#### 5.3.92.3	Attribute constraints

None

#### 5.3.92.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.93	TscaiInputContainer  <<dataType>>

#### 5.3.93.1	Definition

This data type specifies the transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE for a PCC rule, see TS 29.512 [60].

#### 5.3.93.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| periodicity | O | T | T | F | T |
| burstArrivalTime | O | T | T | F | T |

#### 5.3.93.3	Attribute constraints

None

#### 5.3.93.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.94	Dynamic5QISet

#### 5.3.94.1	Definition

This IOC specifies the dynamically assigned 5QIs including their QoS characteristics, see 3GPP TS 23.501 [2]. The instance of this IOC shall not be created or modified by the MnS consumer except for the instance associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed.

#### 5.3.94.2	Attributes

The Dynamic5QISet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]).

#### 5.3.94.3	Attribute constraints

None.

#### 5.3.94.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.95	NetworkSliceInfo <<dataType>>

#### 5.3.95.1	Definition

This data type represents information of network slice when the NWDAF is authorized to collect the management data of the network slice.

#### 5.3.95.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNSSAI | M | T | T | F | T |
| cNSIId | CM | T | T | F | T |
| networkSliceRef | M | T | T | F | T |

#### 5.3.95.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cNSIId | Condition: Network slicing feature is supported and the NSI ID is supported by 5GC. |

#### 5.3.95.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.96	NSACFFunction

#### 5.3.96.1	Definition

This IOC represents the Network Slice Admission Control Function (NSACF) in 5GC. The NSACF monitors and controls the number of registered UEs per network slice for the network slices that are subject to Network Slice Admission Control (NSAC). The NSACF is configured with the maximum number of UEs per network slice and the maximum number of PDU Sessions per network slice which are allowed to be served by each network slice that is subject to NSAC. For more information about the NSACF and admission control procedure, see  TS 23.501 [2] and TS 23.502 [2].

#### 5.3.96.2	Attributes

The NSACFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managedNFProfile | M | T | T | F | T |
| nsacfInfoSnssaiList | M | T | T | F | T |
| nsacfInfo | M | T | T | F | T |

#### 5.3.96.3	Attribute constraints

None

#### 5.3.96.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.97	SnssaiInfo <<dataType>>

#### 5.3.97.1	Definition

This data type defines generic information for a S-NSSAI.

#### 5.3.97.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfo | M | T | T | F | T |
| administrativeState | M | T | T | F | T |

#### 5.3.97.3	Attribute constraints

None

#### 5.3.97.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.98	NsacfInfoSnssai <<dataType>>

#### 5.3.98.1	Definition

This data type defines NSACF specific information per S-NSSAI .

#### 5.3.98.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| snssaiInfo | M | T | T | F | T |
| isSubjectToNsac | M | T | T | F | T |
| maxNumberofUEs | M | T | T | F | T |
| eACMode | CM | T | F | F | T |
| activeEacThreshold | CM | T | T | F | T |
| deactiveEacThreshold | CM | T | T | F | T |
| numberofUEs | O | T | F | F | T |
| uEIdList | O | T | F | F | T |
| taiListtAI | O | T | T | F | T |
| maxNumberofPDUSessions | M | T | T | F | T |

#### 5.3.98.3	Attribute constraints

| Name | Definition |
| --- | --- |
| eACMode S | Condition: early access control feature is supported. |
| activeEacThreshold S | Condition: early access control feature is supported. |
| deactiveEacThreshold S | Condition: early access control feature is supported. |

#### 5.3.98.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.99	EP_N60

#### 5.3.99.1	Definition

This IOC represents the N60 interface between AMF and ASACF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.99.2	Attributes

The EP_N60 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.99.3	Attribute constraints

None.

#### 5.3.99.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.100	EP_N33

#### 5.3.100.1	Definition

This IOC represents the N33 interface between NEF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.100.2	Attributes

The EP_N33 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.100.3	Attribute constraints

None.

#### 5.3.100.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.101	DDNMFFunction

#### 5.3.101.1	Definition

This IOC represents the 5G DDNMF which is the logical function handling network related actions required for dynamic 5G ProSe Direct Discovery. For more information about the 5G DDNMF, see 3GPP TS 23.304 [73].

#### 5.3.101.2	Attributes

The DDNMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.101.3	Attribute constraints

None.

#### 5.3.101.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.102	EP_Npc4

#### 5.3.102.1	Definition

This IOC represents the Npc4 interface between the UDM and 5G DDNMF, which is defined in 3GPP TS 23.304 [73].

#### 5.3.102.2	Attributes

The EP_Npc4 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.102.3	Attribute constraints

None.

#### 5.3.102.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.103	EP_Npc6

#### 5.3.103.1	Definition

This IOC represents the Npc6 interface between the 5G DDNMF in the HPLMN and the 5G DDNMF in a Local PLMN (5G ProSe Direct Discovery), which is defined in TS 23.304 [73].

#### 5.3.103.2	Attributes

The EP_Npc6 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.103.3	Attribute constraints

None.

#### 5.3.103.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.104	EP_Npc7

#### 5.3.104.1	Definition

This IOC represents the Npc7 interface between the 5G DDNMF in the HPLMN and the 5G DDNMF in the VPLMN, which is defined in TS 23.304 [73].

#### 5.3.104.2	Attributes

The EP_Npc7 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.104.3	Attribute constraints

None.

#### 5.3.104.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.105	GUAMInfo <<dataType>>

#### 5.3.105.1	Definition

This <<dataType>> represents the GUAM identifier, a global unique identifier for the AMF.

#### 5.3.105.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |

#### 5.3.105.3	Attribute constraints

None.

#### 5.3.105.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.106	TaiRange <<dataType>>

#### 5.3. 106.1	Definition

This <<dataType>> represents the range of TAIs.

#### 5.3.106.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| nRTACRangeList | M | T | T | F | T |

#### 5.3.106.3	Attribute constraints

None.

#### 5.3.106.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.107	NRTACRange <<dataType>>

#### 5.3.107.1	Definition

This <<dataType>> represents the range of TACs.

#### 5.3.107.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nRTACstart | O | T | T | F | T |
| nRTACend | O | T | T | F | T |
| nRTACpattern | O | T | T | F | T |

Either the start and end attributes, or the pattern attribute, shall be present.

#### 5.3.107.3	Attribute constraints

None.

#### 5.3.107.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.108	SCPInfo <<dataType>>

#### 5.3.108.1	Definition

This <<dataType>> represents the information of a Service Communication Proxy (SCP) instance as defined in 3GPP TS 29.510 [23].

#### 5.3.108.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scpDomainInfoList | O | T | T | F | T |
| scpPrefix | O | T | T | F | T |
| scpPorts | O | T | T | F | T |
| addressDomains | O | T | T | F | T |
| ipv4Addresses | O | T | T | F | T |
| ipv6Prefixes | O | T | T | F | T |
| ipv4AddrRanges | O | T | T | F | T |
| ipv6PrefixRanges | O | T | T | F | T |
| servedNfSetIdList | O | T | T | F | T |
| remotePlmnList | O | T | T | F | T |
| remoteSnpnList | O | T | T | F | T |
| ipReachability | O | T | T | F | T |
| scpCapabilities | O | T | T | F | T |

#### 5.3.108.3	Attribute constraints

None.

#### 5.3.108.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.109	SCPDomainInfo <<dataType>>

#### 5.3.109.1	Definition

This <<dataType>> represents the SCP domain specific information as defined in 3GPP TS 29.510 [23].

#### 5.3.109.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scpFQND | O | T | T | F | T |
| scpEndPoints | O | T | T | F | T |
| scpPorts | O | T | T | F | T |
| scpPrefix | O | T | T | F | T |

If any of these attributes is present for a given SCP domain, it shall apply instead of the attributes fqdn, Ipv4Addresses and Ipv4Addresses within the NFProfile data type for the corresponding SCP Domain.

If none of these attributes is present for a given SCP domain, the attributes fqdn, Ipv4Addresses, and Ipv4Addresses within the NFProfile data type shall apply for the corresponding SCP Domain.

If scpPorts attribute is present, it has precedence over the scpPorts attribute of ScpInfo.

#### 5.3.109.3	Attribute constraints

None.

#### 5.3.109.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.110	IpEndPoint <<dataType>>

#### 5.3.110.1	Definition

This <<dataType>> represents the IP end points considering both IPv4 and IPv6 addresses.

#### 5.3.110.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| hostAddr | M | T | T | F | T |
| transport | O | T | T | F | T |
| port | O | T | T | F | T |

#### 5.3.110.3	Attribute constraints

None.

#### 5.3.110.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.111	IPv4AddressRange <<dataType>>

#### 5.3.111.1	Definition

This <<dataType>> represents the range of IPv4 addresses.

#### 5.3.111.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| IPv4AddrRangeStart | M | T | T | F | T |
| IPv4AddrRangeEnd | M | T | T | F | T |

#### 5.3.111.3	Attribute constraints

None.

#### 5.3.111.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.112	IPv6PrefixRange <<dataType>>

#### 5.3.112.1	Definition

This <<dataType>> represents the range of IPv6 address prefix.

#### 5.3.112.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| IPv6PrefRangeStart | M | T | T | F | T |
| IPv6PrefRangeEnd | M | T | T | F | T |

#### 5.3.112.3	Attribute constraints

None.

#### 5.3.112.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.113	EASDFFunction

#### 5.3.113.1	Definition

This IOC represents the Edge Application Server Discovery Function (EASDF) in 5GC which can handle the DNS messages according to the instruction from the SMF.

For more information about the 5G EASDF, see 3GPP TS 23.548 [78].

#### 5.3.113.2	Attributes

The EASDFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| serverAddr | M | T | T | F | T |
| easdfInfo | O | T | T | F | T |

#### 5.3.113.3	Attribute constraints

None.

#### 5.3.113.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.114	EP_N88x

#### 5.3.114.1	Definition

This IOC represents the N88 interface between the EASDF and SMF, which is defined in  TS 23.501 [2].

#### 5.3.114.2	Attributes

The EP_N88 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.114.3	Attribute constraints

None.

#### 5.3.114.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.115	SNPNInfo <<dataType>>

#### 5.3.115.1	Definition

This <<dataType>> represents the SNPN identifier and associated S-NSSAI.

#### 5.3.115.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNPNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 5.3.115.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI S | Condition: slicing feature is supported. |

#### 5.3.115.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.116	SNPNId <<dataType>>

#### 5.3.116.1	Definition

This <<dataType>> represents the information of a SNPN identification.

#### 5.3.116.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mCC | M | T | T | F | T |
| mNC | M | T | T | F | T |
| nId | M | T | T | F | T |

#### 5.3.116.3	Attribute constraints

None.

#### 5.3.116.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.117	EP_Npc8

#### 5.3.117.1	Definition

This IOC represents the Npc8 interface between the PCF and 5G DDNMF, which is defined in 3GPP TS 23.304 [73].

#### 5.3.117.2	Attributes

The EP_Npc8 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.117.3	Attribute constraints

None.

#### 5.3.117.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.118	DefaultNotificationSubscription <<dataType>>

#### 5.3.118.1	Definition

This <<dataType>> represents the range of default notification subscriptions as defined in 3GPP TS 29.510 [23].

#### 5.3.118.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationType | M | T | F | F | F |
| callbackURI | M | T | F | F | F |
| n1MessageClass | CM | T | F | F | F |
| n2InformationClass | CM | T | F | F | F |
| versions | O | T | F | F | F |
| binding | O | T | F | F | F |

#### 5.3.118.3	Attribute constraints

| Name | Definition |
| --- | --- |
| n1MessageClass S | Condition: This attribute shall be present only if notificationType of N1_MESSAGES is supported. |
| n2InformationClass S | Condition: This attribute shall be present only if notificationType of N2_INFORMATION is supported. |

#### 5.3.118.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.119	EcmConnectionInfo

#### 5.3.119.1	Definition

This IOC contains attributes to enable ECSP management system to connect EDN NFs (i.e., EAS, ECS, and EES) to 5GC NFs (i.e., UPF, PCF, NEF, SCEF) (See clause 7.4.3 and 7.4.4 in TS 28.538 [79]).

#### 5.3.119.2	Attributes

The EcmConnectionInfo IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eASServiceArea | CM | T | T | F | T |
| eESServiceArea | CM | T | T | F | T |
| eDNServiceArea | CM | T | T | F | T |
| eASIpAddress | CM | T | T | F | T |
| eESIpAddress | CM | T | T | F | T |
| eCSIpAddress | CM | T | T | F | T |
| ednIdentifier | M | T | T | F | T |
| ecmConnectionType | CM | T | T | F | T |
| 5GCNfConnEcmInfoList | CM | T | F | F | T |
| uPFConnectionInfo | CM | T | F | F | T |

#### 5.3.119.3	Attribute constraints

| Name | Definition |
| --- | --- |
| eASServiceArea S | Condition: EAS connection to 5GC NF is supported |
| eASIpAddress S | Condition: EAS connection to 5GC NF is supported |
| eESServiceArea S | Condition: EES connection to 5GC NF is supported |
| eESIpAddress S | Condition: EES connection to 5GC NF is supported |
| eDNServiceArea S | Condition: EAS or ECS connection to 5GC NF is supported |
| eCSIpAddress S | Condition: ECS connection to 5GC NF is supported |
| ecmConnectionType S | Condition: EAS connection to 5GC NF is supported |
| 5GCNfConnEcmInfoList S | Condition: EAS, EES, and ECS connections to 5GC NF is supported |
| uPFConnectionInfo S | Condition: EAS connection to UPF is supported |

#### 5.3.119.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.120	5GCNfConnEcmInfo <<dataType>>

#### 5.3.120.1	Definition

This data type specifies the 5GC NF connection information.

#### 5.3.120.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| 5GCNFType | M | T | T | F | T |
| 5GCNFIpAddress | M | T | T | F | T |
| 5GCNFRef | M | T | T | F | T |

Editor’s note: Role based attribute of 5GCNfRef inside the dataType needs further work.

#### 5.3.120.3	Attribute constraints

None

#### 5.3.120.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.121	UPFConnInfo <<dataType>>

#### 5.3.121.1	Definition

This data type specifies the UPF connection information.

#### 5.3.121.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| uPFIpAddress | M | T | T | F | T |
| uPFRef | M | T | T | F | T |

#### 5.3.121.3	Attribute constraints

None

#### 5.3.121.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.122	SnssaiSmfInfoItem <<dataType>>

#### 5.3.122.1	Definition

This <<dataType>> represents the set of parameters supported by SMF for a given S-NSSAI.

#### 5.3.122.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNSSAI | M | T | T | F | T |
| dnnSmfInfoList | M | T | T | F | T |

#### 5.3.122.3	Attribute constraints

None.

#### 5.3.122.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.123	DnnSmfInfoItem <<dataType>>

#### 5.3.123.1	Definition

This <<dataType>> represents the set of parameters supported by SMF for a given DNN.

The absence of ‘dnaiList’ indicates the DNN can be selected for any DNAI.

#### 5.3.123.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnn | M | T | T | F | T |
| dnaiList | O | T | T | F | T |

#### 5.3.123.3	Attribute constraints

None.

#### 5.3.123.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.124	Void

### 5.3.125	InterfaceUpfInfoItem <<dataType>>

#### 5.3.125.1	Definition

This <<dataType>> provides information of a given IP interface of a UPF, (see clause 6.1.6.2.24 TS 29.510 [23]). The information provided shall include at least one of the addressing parameters: ipv4EndpointAddresses, ipv6EndpointAddresses or fqdn.

#### 5.3.125.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| interfaceType | M | T | T | F | T |
| ipv4EndpointAddresses | CM | T | T | F | T |
| ipv6EndpointAddresses | CM | T | T | F | T |
| fqdn | CM | T | T | F | T |
| networkInstance | O | T | T | F | T |

#### 5.3.125.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4EndpointAddresses | Condition: The both ipv6EndpointAddresses and fqdn are not present. |
| ipv6EndpointAddresses | Condition: The both ipv4EndpointAddresses and fqdn are not present. |
| fqdn | Condition: The both ipv4EndpointAddresses and ipv6EndpointAddresses are not present. |

Note that at least one of the attributes shall contain a value.

#### 5.3.125.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.126	AtsssCapability <<dataType>>

#### 5.3.126.1	Definition

This <<dataType>> provides information of a given IP interface of a UPF

#### 5.3.126.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| atsssLL | CM | T | T | F | T |
| mptcp | CM | T | T | F | T |
| rttWithoutPmf | CM | T | T | F | T |

#### 5.3.126.3	Attribute constraints

| Name | Definition |
| --- | --- |
| atsssLL CM S | Access Traffic Steering, Switching and Splitting procedures using the ATSSS-LL steering functionality is supported. |
| Mptcp CM S | Access Traffic Steering, Switching and Splitting procedures using the MPTCP steering functionality is supported. |
| rttWithoutPmf CM S | This attribute is present and set to true, if the mptcp attribute is present and set to true too. |

#### 5.3.126.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.127	IpInterface <<dataType>>

#### 5.3.127.1	Definition

This <<dataType>> provides information of a given IP interface related to one or more of the following functions: (i) Trusted WLAN Interworking Function (TWIF), (ii) Trusted Non-3GPP Gateway Function (TNGF), and (iii) Wireline Access Gateway Function (WAGF). The information provided shall include at least one of the addressing parameters: ipv4EndpointAddresses, ipv6EndpointAddresses or fqdn.

#### 5.3.127.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4EndpointAddresses | CM | T | T | F | T |
| ipv6EndpointAddresses | CM | T | T | F | T |
| fqdn | CM | T | T | F | T |

#### 5.3.127.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4EndpointAddresses | Condition: The other two attributes are not present. |
| ipv6EndpointAddresses | Condition: The other two attributes are not present. |
| fqdn | Condition: The other two attributes are not present. |

Note that at least one of the attributes shall contain a value.

#### 5.3.127.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.128	SupiRange <<dataType>>

#### 5.3.128.1	Definition

This data type represents a ranges of SUPIs that can be served by the AUSF instance. (See TS 29.510 [23]).

#### 5.3.128.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| start | O | T | T | F | T |
| end | O | T | T | F | T |
| pattern | O | T | T | F | T |

#### 5.3.128.3	Attribute constraints

None.

#### 5.3.128.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.129	IdentityRange <<dataType>>

#### 5.3.129.1	Definition

This data type represents a ranges of GPSIs that can be served by the PCF instance. (See TS 29.510 [23]).

#### 5.3.129.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| start | O | T | T | F | T |
| end | O | T | T | F | T |
| pattern | O | T | T | F | T |

#### 5.3.129.3	Attribute constraints

None.

#### 5.3.129.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.130	ProSeCapability <<dataType>>

#### 5.3.130.1	Definition

This data type represents the supported ProSe Capability by the PCF. (See TS 29.510 [23]).

#### 5.3.130.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| proseDirectDiscovery | O | T | T | F | T |
| proseDirectCommunication | O | T | T | F | T |
| proseL2UetoNetworkRelay | O | T | T | F | T |
| proseL3UetoNetworkRelay | O | T | T | F | T |
| proseL2RemoteUe | O | T | T | F | T |
| proseL3RemoteUe | O | T | T | F | T |

#### 5.3.130.3	Attribute constraints

None.

#### 5.3.130.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.131	V2xCapability <<dataType>>

#### 5.3.131.1	Definition

This data type represents the supported V2X Capability by the PCF. (See TS 29.510 [23]).

#### 5.3.131.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| lteV2x | O | T | T | F | T |
| nrV2x | O | T | T | F | T |

#### 5.3.131.3	Attribute constraints

None.

#### 5.3.131.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.132	InternalGroupIdRange <<dataType>>

#### 5.3.132.1	Definition

This data type represents a range of Group IDs (internal group identities), either based on a numeric range, or based on regular-expression matching. (See TS 29.510 [23]).

#### 5.3.132.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| start | O | T | T | F | T |
| end | O | T | T | F | T |
| pattern | O | T | T | F | T |

#### 5.3.132.3	Attribute constraints

None.

#### 5.3.132.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.133	SuciInfo <<dataType>>

#### 5.3.133.1	Definition

This data type represents SUCI information containing Routing Indicator and Home Network Public Key ID. (See TS 29.510 [23]).

#### 5.3.133.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| routingInds | O | T | T | F | T |
| hNwPubKeyIds | O | T | T | F | T |

#### 5.3.133.3	Attribute constraints

None.

#### 5.3.133.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.134	SharedDataIdRange <<dataType>>

#### 5.3.134.1	Definition

This data type represents a range of Shared Data IDs that identify shared data available in the UDR instance. (See TS 29.510 [23]).

#### 5.3.134.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pattern | O | T | T | F | T |

#### 5.3.134.3	Attribute constraints

None.

#### 5.3.134.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.135	PfdData <<dataType>>

#### 5.3.135.1	Definition

This data type represents the list of Application IDs and/or AF IDs managed by a given NEF Instance. (See clause 6.1.6.2.49 TS 29.510 [23]).

#### 5.3.135.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| appIds | O | T | T | F | T |
| afIds | O | T | T | F | T |

#### 5.3.135.3	Attribute constraints

None.

#### 5.3.135.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.136	AfEventExposureData <<dataType>>

#### 5.3.136.1	Definition

This data type represents the AF Event Exposure data managed by a given NEF Instance. (See clause 6.1.6.2.50 TS 29.510 [23]).

#### 5.3.136.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| afEvents | M | T | T | F | T |
| afIds | O | T | T | F | T |
| appIds | O | T | T | F | T |

#### 5.3.136.3	Attribute constraints

None.

#### 5.3.136.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.137	UnTrustAfInfo <<dataType>>

#### 5.3.137.1	Definition

This data type represents information of an untrusted AF Instance. (See clause 6.1.6.2.95 TS 29.510 [23]).

#### 5.3.137.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| afId | M | T | T | F | T |
| sNssaiInfoList | O | T | T | F | T |
| mappingInd | O | T | T | F | T |

#### 5.3.137.3	Attribute constraints

None.

#### 5.3.137.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.138	SnssaiInfoItem <<dataType>>

#### 5.3.138.1	Definition

This data type represents set of parameters supported by NF for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510 [23]).

#### 5.3.138.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssai | M | T | T | F | T |
| dnnInfoList | M | T | T | F | T |

#### 5.3.138.3	Attribute constraints

None.

#### 5.3.138.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.139	ExtSnssai <<dataType>>

#### 5.3.139.1	Definition

This data type represents set of parameters supported by NF for a given S-NSSAI. (See clause 6.1.6.2.97 TS 29.510 [23]).

#### 5.3.139.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| snssai | M | T | T | F | T |
| snssaiExtension | O | T | T | F | T |

#### 5.3.139.3	Attribute constraints

None.

#### 5.3.139.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.140	SnssaiExtension <<dataType>>

#### 5.3.140.1	Definition

This data type represents set of parameters supported by NF for a given DNN. (See clause 5.4.4.66 of TS 29.571 [61]).

#### 5.3.140.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sdRanges | CM | T | T | F | T |
| wildcardSd | CM | T | T | F | T |

#### 5.3.140.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sdRanges S | Condition: sdRanges is supported. |
| wildcardSd S | Condition: wildcardSd is supported. |

#### 5.3.140.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.141	DnnInfoItem <<dataType>>

#### 5.3.141.1	Definition

This data type represents set of parameters supported by NF for a given DNN. (See clause 6.1.6.2.98 TS 29.510 [23]).

The absence of ‘dnaiList’ indicates the DNN can be selected for any DNAI.

#### 5.3.141.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnn | M | T | T | F | T |

#### 5.3.141.3	Attribute constraints

None.

#### 5.3.141.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.142	SdRange <<dataType>>

#### 5.3.142.1	Definition

This data type represents a SD range. (See clause 5.4.4.6.67 TS 29.571 [61]).

#### 5.3.142.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| start | M | T | T | F | T |
| end | M | T | T | F | T |

#### 5.3.142.3	Attribute constraints

None.

#### 5.3.142.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.142a	NsacfInfo <<dataType>>

#### 5.3.142a.1	Definition

This data type represents the information of an NSACF NF Instance (see TS 29.510 [23]).

#### 5.3.142a.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nsacfCapability | M | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |

#### 5.3.142a.3	Attribute constraints

None.

#### 5.3.142a.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.142b	NsacfCapability <<dataType>>

#### 5.3.142b.1	Definition

This data type represents the NSACF service capability. (See TS 29.510 [23]).

#### 5.3.142b.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportUeSAC | CO | T | T | F | T |
| supportPduSAC | CO | T | T | F | T |

#### 5.3.142b.3	Attribute constraints

None.

#### 5.3.142b.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.142c	NwdafInfo <<dataType>>

#### 5.3.142c.1	Definition

This data type represents specific data for the NWDAF. (See TS 29.510 [23]).

#### 5.3.142c.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nwdafEvents | CM | T | T | F | T |
| eventIds | CM | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| nwdafCapability | O | T | T | F | T |
| analyticsDelay | O | T | T | F | T |
| servingNfTypeList | O | T | T | F | T |
| servingNfSetIdList | O | T | T | F | T |
| mlAnalyticsList | CM | T | T | F | T |

#### 5.3.142c.3	Attribute constraints

| Name | Definition |
| --- | --- |
| eventIds S | Condition: Nnwdaf_AnalyticsInfo service supports the eventIds. |
| nwdafEvents S | Condition: Nnwdaf_EventsSubscription service supports the nwdafEvents. |
| mlAnalyticsList S | Condition: Nnwdaf_MLModelProvision service supports the ML Analytics Filter information |

#### 5.3.142c.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.142d	NwdafCapability <<dataType>>

#### 5.3.142d.1	Definition

This data type represents the capability supported by the NWDAF. (See TS 29.510 [23]).

#### 5.3.142d.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| analyticsAggregation | O | T | T | F | T |
| analyticsMetadataProvisioning | O | T | T | F | T |

#### 5.3.142d.3	Attribute constraints

None.

#### 5.3.142d.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.142e	MlAnalyticsInfo <<dataType>>

#### 5.3.142e.1	Definition

This data type represents ML Analytics Filter information supported by the Nnwdaf_MLModelProvision service (See TS 29.510 [23]).

#### 5.3.142e.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mlAnalyticsIds | CM | T | T | F | T |
| sNSSAIList | O | T | T | F | T |
| trackingAreaList | O | T | T | F | T |
| mlModelInterInfo | O | T | T | F | T |
| flCapabilityType | O | T | T | F | T |
| flTimeInterval | O | T | T | F | T |

#### 5.3.142e.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mlAnalyticsIds S | Condition: Network slicing feature is supported and the NWDAF is allowedauthorized to collect the management data of the network slices. |

#### 5.3.142e.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.143	PlmnIdNid <<dataType>>

#### 5.3.143.1	Definition

This <<dataType>> represents the SCP domain specific information as defined in TS 29.510 [23].

#### 5.3.143.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mcc | M | T | T | F | T |
| mnc | M | T | T | F | T |
| nid | CM | T | T | F | T |

#### 5.3.143.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nid S | Condition: The SNPN is supported. |

#### 5.3.143.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.143a	SeppInfo <<dataType>>

#### 5.3.143a.1	Definition

This data type represents information of a SEPP instance, as described in clause 6.1.6.2.72 of TS 29.510 [23].

#### 5.3.143a.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| seppPrefix | O | T | T | F | T |
| seppPorts | CM | T | T | F | T |
| remotePlmnList | O | T | T | F | T |
| remoteSnpnList | O | T | T | F | T |

#### 5.3.143a.3	Attribute constraints

| Name | Definition |
| --- | --- |
| seppPorts S | Condition: If the HTTP client does not use the default HTTP port number, i.e. TCP port 80 for "http" URIs or TCP port 443 for "https" URIs as specified in IETF RFC 7540 [9] when sending a request to the SEPP. |

#### 5.3.143a.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.143b	UdsfInfo <<dataType>>

#### 5.3.143b.1	Definition

This data type represents information related to UDSF, as described in clause 6.1.6.2.63 of TS 29.510 [23].

#### 5.3.143b.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | O | T | T | F | T |
| supiRanges | O | T | T | F | T |
| storageIdRanges | CM | T | T | F | T |

#### 5.3.143b.3	Attribute constraints

| Name | Definition |
| --- | --- |
| storageIdRanges S | Condition: the UDSF's supported realms and storages are not determined by the UDSF's consumer. |

#### 5.3.143b.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.143c	SmsfInfo <<dataType>>

#### 5.3.143c.1	Definition

This data type represents specific data for a SMSF Instance. (See clause 6.1.6.2.113 TS 29.510 [23]).

#### 5.3.143c.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| roamingUeInd | O | T | T | F | T |
| remotePlmnRangeList | O | T | T | F | T |

#### 5.3.143c.3	Attribute constraints

None.

#### 5.3.143c.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.143d	PlmnRange <<dataType>>

#### 5.3.143d.1	Definition

This data type represents range of PLMN IDs. (See clause 6.1.6.2.34 TS 29.510 [23]).

#### 5.3.143d.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| start | O | T | T | F | T |
| end | O | T | T | F | T |
| pattern | O | T | T | F | T |

#### 5.3.143d.3	Attribute constraints

None.

#### 5.3.143d.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.143e	UdmInfo <<dataType>>

#### 5.3.143e.1	Definition

This data type represents information of an UDM NF Instance. (See clause 6.1.6.2.7 TS 29.510 [23]).

#### 5.3.143e.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | O | T | T | F | T |
| supiRanges | O | T | T | F | T |
| gpsiRanges | O | T | T | F | T |
| externalGroupIdentifiersRanges | O | T | T | F | T |
| routingIndicators | O | T | T | F | T |
| internalGroupIdentifiersRanges | O | T | T | F | T |
| suciInfos | O | T | T | F | T |

#### 5.3.143e.3	Attribute constraints

None.

#### 5.3.143e.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.144	UdrInfo <<dataType>>

#### 5.3.144.1	Definition

This data type represents information of an UDR NF Instance. (See clause 6.1.6.2.6 TS 29.510 [23]).

#### 5.3.144.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | O | T | T | F | T |
| supiRanges | O | T | T | F | T |
| gpsiRanges | O | T | T | F | T |
| externalGroupIdentifiersRanges | O | T | T | F | T |
| supportedDataSets | O | T | T | F | T |
| sharedDataIdRanges | O | T | T | F | T |

#### 5.3.144.3	Attribute constraints

None.

#### 5.3.144.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.144a	LmfInfo <<dataType>>

#### 5.3.144a.1	Definition

This data type represents the Specific data for the LMF. (See clause 6.1.6.2.46 TS 29.510 [23]).

#### 5.3.144a.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servingClientTypes | CM | T | T | F | T |
| lmfId | O | T | T | F | T |
| servingAccessTypes | CM | T | T | F | T |
| servingAnNodeTypes | CM | T | T | F | T |
| servingRatTypes | CM | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| supportedGADShapes | O | T | T | F | T |

#### 5.3.144a.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servingClientTypes CM S | Condition: the LMF is dedicated to serve the listed external client type(s), e.g. emergency client. |
| servingAccessTypes CM S | Condition: not all access types are supported. |
| servingAnNodeTypes CM S | Condition: not all AN node types are supported. |
| servingRatTypes CM S | Condition: not all RAT types are supported. |

#### 5.3.144a.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.144b	TrustAfInfo <<dataType>>

#### 5.3.144b.1	Definition

This data type represents the information of a trusted AF Instance. (See clause 6.1.6.2.96 TS 29.510 [23]).

#### 5.3.144b.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiInfoList | O | T | T | F | T |
| afEvents | O | T | T | F | T |
| appIds | O | T | T | F | T |
| internalGroupId | O | T | T | F | T |
| mappingInd | O | T | T | F | T |

#### 5.3.144b.3	Attribute constraints

None.

#### 5.3.144b.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.144c	EasdfInfo <<dataType>>

#### 5.3.144c.1	Definition

This data type represents EASDF specific data. (See clause 5.4.5.1 of  TS 29.571 [61]).

#### 5.3.144c.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiEasdfInfoList | O | T | T | F | T |
| easdfN6IpAddressList | O | T | T | F | T |
| upfN6IpAddressList | O | T | T | F | T |

#### 5.3.144c.3	Attribute constraints

None.

#### 5.3.144c.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.144d	SnssaiEasdfInfoItem  <<dataType>>

#### 5.3.144d.1	Definition

This data type represents parameters supported by the EASDF per S-NSSAI. (See clause 6.1.6.2.78 TS 29.510 [23]).

#### 5.3.144d.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssai | M | T | T | F | T |
| dnnEasdfInfoList | M | T | T | F | T |

#### 5.3.144d.3	Attribute constraints

None.

#### 5.3.144d.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.144e	DnnEasdfInfoItem <<dataType>>

#### 5.3.144e.1	Definition

This data type represents parameters supported by the EASDF per DNN. (See clause 6.1.6.2.79 TS 29.510 [23]).

#### 5.3.144e.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnn | M | T | T | F | T |
| dnaiList | O | T | T | F | T |

#### 5.3.144e.3	Attribute constraints

None.

#### 5.3.144e.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.145	NSSAAFFunction

#### 5.3.145.1	Definition

This IOC represents the NSSAAF function in 5GC. For more information about the NSSAAF, see TS 23.501 [2].

#### 5.3.145.2	Attributes

The NSSAAFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| cNSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| nssaafInfo | O | T | T | F | T |

#### 5.3.145.3	Attribute constraints

None.

#### 5.3.145.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.146	NssaafInfo <<dataType>>

#### 5.3.146.1	Definition

This data type represents the information of a NSSAAF NF Instance. (See clause 6.1.6.2.104 TS 29.510 [23]).

#### 5.3.146.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supiRanges | O | T | T | F | T |
| internalGroupIdentifiersRanges | O | T | T | F | T |

#### 5.3.146.3	Attribute constraints

None.

#### 5.3.146.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.147	EP_N58

#### 5.3.147.1	Definition

This IOC represents an end point of N58 interface between NSSAAF and AMF, which is defined in TS 23.501 [2] and 33.501 [52].

#### 5.3.147.2	Attributes

The EP_N58 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.147.3	Attribute constraints

None.

#### 5.3.147.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.148	EP_N59

#### 5.3.148.1	Definition

This IOC represents an end point of N59 interface between NSSAAF and UDM, which is defined in TS 23.501 [2] and 33.501 [52].

#### 5.3.148.2	Attributes

The EP_N59 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.148.3	Attribute constraints

None.

#### 5.3.148.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.149	NrfInfo <<dataType>>

#### 5.3.149.1	Definition

This data type represents information of an NRF NF Instance, used in hierarchical NRF deployments. (See clause 6.1.6.2.31 TS 29.510 [23]).

#### 5.3.149.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servedUdrInfo | O | T | T | F | T |
| servedUdrInfoList | O | T | T | F | T |
| servedUdmInfo | O | T | T | F | T |
| servedUdmInfoList | O | T | T | F | T |
| servedAusfInfo | O | T | T | F | T |
| servedAusfInfoList | O | T | T | F | T |
| servedAmfInfo | O | T | T | F | T |
| servedAmfInfoList | O | T | T | F | T |
| servedSmfInfo | O | T | T | F | T |
| servedSmfInfoList | O | T | T | F | T |
| servedUpfInfo | O | T | T | F | T |
| servedUpfInfoList | O | T | T | F | T |
| servedPcfInfo | O | T | T | F | T |
| servedPcfInfoList | O | T | T | F | T |
| servedBsfInfo | O | T | T | F | T |
| servedBsfInfoList | O | T | T | F | T |
| servedChfInfo | O | T | T | F | T |
| servedChfInfoList | O | T | T | F | T |
| servedNefInfo | O | T | T | F | T |
| servedNwdafInfo | O | T | T | F | T |
| servedNwdafInfoList | O | T | T | F | T |
| servedPcscfInfoList | O | T | T | F | T |
| servedGmlcInfo | O | T | T | F | T |
| servedLmfInfo | O | T | T | F | T |
| servedNfInfo | O | T | T | F | T |
| servedHssInfoList | O | T | T | F | T |
| servedUdsfInfo | O | T | T | F | T |
| servedUdsfInfoList | O | T | T | F | T |
| servedScpInfoList | O | T | T | F | T |
| servedSeppInfoList | O | T | T | F | T |
| servedAanfInfoList | O | T | T | F | T |
| served5gDdnmfInfo | O | T | T | F | T |
| servedMfafInfoList | O | T | T | F | T |
| servedEasdfInfoList | O | T | T | F | T |
| servedDccfInfoList | O | T | T | F | T |
| servedMbSmfInfoList | O | T | T | F | T |
| servedTsctsfInfoList | O | T | T | F | T |
| servedMbUpfInfoList | O | T | T | F | T |
| servedTrustAfInfo | O | T | T | F | T |
| servedNssaafInfo | O | T | T | F | T |

#### 5.3.149.3	Attribute constraints

None.

#### 5.3.149.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.150	CHFFunction

#### 5.3.150.1	Definition

This IOC represents the CHF function. For more information about the CHF, see TS 23.501 [2] and TS 32.240 [88].

#### 5.3.150.2	Attributes

The CHFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| chfInfo | O | T | T | F | T |

#### 5.3.150.3	Attribute constraints

None.

#### 5.3.150.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.151	ChfInfo <<dataType>>

#### 5.3.151.1	Definition

This data type represents the Specific data for the CHF. (See clause 6.1.6.2.32 TS 29.510 [23]).

#### 5.3.151.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supiRangeList | O | T | T | F | T |
| gpsiRangeList | O | T | T | F | T |
| plmnRangeList | O | T | T | F | T |
| groupId | O | T | T | F | T |
| primaryChfInstance | CM | T | T | F | T |
| secondaryChfInstance | CM | T | T | F | T |

#### 5.3.151.3	Attribute constraints

| Name | Definition |
| --- | --- |
| primaryChfInstance CM S | Condition: the CHF instance serves as a secondary CHF instance of another primary CHF instance. |
| secondaryChfInstance CM S | Condition: the CHF instance serves as a primary CHF instance of another secondary CHF instance. |

#### 5.3.151.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.152	EP_N40

#### 5.3.152.1	Definition

This IOC represents an end point of N40 interface between CHF and SMF, which is defined in TS 23.501 [2] and TS 32.240 [88].

#### 5.3.152.2	Attributes

The EP_N40 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.152.3	Attribute constraints

None.

#### 5.3.152.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.153	EP_N41

#### 5.3.153.1	Definition

This IOC represents an end point of N41 interface between AMF and CHF in HPLMN, which is defined in TS 23.501 [2] and TS 32.240 [88].

#### 5.3.153.2	Attributes

The EP_N41 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.153.3	Attribute constraints

None.

#### 5.3.153.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.154	EP_N42

#### 5.3.154.1	Definition

This IOC represents an end point of N42 interface between AMF and CHF in VPLMN, which is defined in TS 23.501 [2] and TS 32.240 [88].

#### 5.3.154.2	Attributes

The EP_N42 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.154.3	Attribute constraints

None.

#### 5.3.154.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.155	EP_N28

#### 5.3.155.1	Definition

This IOC represents an end point of N28 interface between CHF and PCF, which is defined in TS 23.501 [2] and TS 32.240 [88].

#### 5.3.155.2	Attributes

The EP_N28 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.155.3	Attribute constraints

None.

#### 5.3.155.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.156	MFAFFunction

#### 5.3.156.1	Definition

This IOC represents the MFAF function in 5GC. For more information about the MFAF, see TS 23.501 [2].

#### 5.3.156.2	Attributes

The MFAFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| mfafInfo | O | T | T | F | T |

#### 5.3.156.3	Attribute constraints

None.

#### 5.3.156.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.157	MfafInfo <<dataType>>

#### 5.3.157.1	Definition

This data type represents the Specific data for the MFAF. (See clause 6.1.6.2.75 TS 29.510 [23]).

#### 5.3.157.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servingNfTypeList | O | T | T | F | T |
| servingNfSetIdList | O | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |

#### 5.3.157.3	Attribute constraints

None.

#### 5.3.157.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.158	DCCFFunction

#### 5.3.158.1	Definition

This IOC represents the DCCF function in 5GC. For more information about the DCCF, see TS 23.501 [2].

#### 5.3.158.2	Attributes

The DCCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| cNSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| dccfInfo | O | T | T | F | T |

#### 5.3.158.3	Attribute constraints

None.

#### 5.3.158.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.159	DccfInfo <<dataType>>

#### 5.3.159.1	Definition

This data type represents the Specific data for the DCCF. (See clause 6.1.6.2.46 TS 29.510 [23]).

#### 5.3.159.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servingNfTypeList | O | T | T | F | T |
| servingNfSetIdList | O | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |

#### 5.3.159.3	Attribute constraints

None.

#### 5.3.159.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.160	AmfInfo <<dataType>>

#### 5.3.160.1	Definition

This data type represents information of an AMF Instance. (See clause 6.1.6.2.11 TS 29.510 [23]).

#### 5.3.160.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aMFRegionId | M | T | T | F | T |
| aMFSetId | M | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| gUAMIdList | M | T | F | F | T |
| backupInfoAmfFailure | O | T | T | F | T |
| backupInfoAmfRemoval | O | T | T | F | T |
| n2InterfaceAmfInfo | O | T | T | F | T |
| amfOnboardingCapability | O | T | T | F | T |
| highLatencyCom | O | T | T | F | T |

#### 5.3.160.3	Attribute constraints

None.

#### 5.3.160.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.161	SmfInfo <<dataType>>

#### 5.3.161.1	Definition

This data type represents information of an SMF Instance. (See clause 6.1.6.2.12 TS 29.510 [23]).

#### 5.3.161.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiSmfInfoList | M | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| pgwFqdn | O | T | T | F | T |
| pgwIpAddrList | O | T | T | F | T |
| accessType | O | T | T | F | T |
| priority | O | T | T | F | T |
| vsmfSupportInd | O | T | T | F | T |
| pgwFqdnList | CM | T | T | F | T |
| ismfSupportInd | O | T | T | F | T |
| smfOnboardingCapability | O | T | T | F | T |
| smfUPRPCapability | O | T | T | F | T |

#### 5.3.161.3	Attribute constraints

| Name | Definition |
| --- | --- |
| pgwFqdnList CM S | Condition: Present if pgwFqdn attribute is present. |

#### 5.3.161.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.162	UpfInfo <<dataType>>

#### 5.3.162.1	Definition

This data type represents information of an UPF Instance. (See clause 6.1.6.2.13 TS 29.510 [23]).

#### 5.3.162.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiUpfInfoList | M | T | T | F | T |
| smfServingArea | O | T | T | F | T |
| interfaceUpfInfoList | O | T | T | F | T |
| iwkEpsInd | O | T | F | F | T |
| pduSessionTypes | O | T | F | F | T |
| atsssCapability | CM | T | F | F | T |
| ueIpAddrInd | O | T | F | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| wAgfInfo | CM | T | T | F | T |
| tngfInfo | CM | T | T | F | T |
| twifInfo | CM | T | T | F | T |
| priority | O | T | T | F | T |
| redundantGtpu | O | T | F | F | T |
| ipups | O | T | T | F | T |
| dataForwarding | O | T | T | F | T |
| supportedPfcpFeatures | O | T | F | F | T |
| sxaInd | O | T | F | F | T |

#### 5.3.162.3	Attribute constraints

| Name | Definition |
| --- | --- |
| atsssCapability CM S | If presents, then one of the atsssLL or mptcp shall be supported. Otherwise, if not present, the UPF shall be regarded with no ATSSS capability. |
| wAgfInfo CM S | The condition is “the UPF is collocated with W-AGF”. |
| tngfInfo CM S | The condition is “the UPF is collocated with TNGF”. |
| twifInfo CM S | The condition is “the UPF is collocated with TWIF”. |

#### 5.3.162.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.163	PcfInfo <<dataType>>

#### 5.3.163.1	Definition

This data type represents information of a PCF Instance. (See clause 6.1.6.2.20 TS 29.510 [23]).

#### 5.3.163.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | O | T | T | F | T |
| dnnList | O | T | T | F | T |
| supiRanges | O | T | T | F | T |
| gpsiRanges | O | T | T | F | T |
| rxDiamHost | CM | T | T | F | T |
| rxDiamRealm | CM | T | T | F | T |
| v2xSupportInd | O | T | F | F | T |
| proseSupportInd | O | T | F | F | T |
| proseCapability | CM | T | T | F | T |
| v2xCapability | CM | T | T | F | T |
| a2xSupportInd | O | T | F | F | T |
| a2xCapability | CM | T | T | F | T |
| rangingSlPosSupportInd | O | T | F | F | T |

#### 5.3.163.3	Attribute constraints

| Name | Definition |
| --- | --- |
| rxDiamHost CM S | Condition: Rx interface feature is supported. |
| rxDiamRealm CM S | Condition: Rx interface feature is supported. |
| proseCapability CM S | Condition: ProSe Capability is supported. |
| v2xCapability CM S | Condition: V2X Capability is supported. |
| a2xCapability CM S | Condition: the PCF supports A2X Capability. |

#### 5.3.163.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.164	NefInfo <<dataType>>

#### 5.3.164.1	Definition

This data type represents information of an NEF Instance. (See clause 6.1.6.2.48 TS 29.510 [23]).

The absence of ‘dnaiList’ indicates the NEF can be selected for any DNAI.

#### 5.3.164.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tailList | O | T | F | T | F |
| taiRangeList | O | T | F | T | F |
| nefId | CM | T | T | F | T |
| pfdData | O | T | F | F | T |
| afEeData | O | T | F | F | T |
| gpsiRanges | O | T | T | F | T |
| externalGroupIdentifiersRanges | O | T | T | F | T |
| servedFqdnList | O | T | T | F | T |
| dnaiList | O | T | T | F | T |
| unTrustAfInfoList | O | T | T | F | T |
| uasNfFunctionalityInd | O | T | T | F | T |
| multiMemAfSessQosInd | O | T | T | F | T |
| memberUESelAssistInd | O | T | T | F | T |

#### 5.3.164.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nefId CM S | Condition: NIDD service is supported. |

#### 5.3.164.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.165	BSFFunction

#### 5.3.165.1	Definition

This IOC represents the BSF function in 5GC. For more information about the BSF, see TS 23.503 [59].

#### 5.3.165.2	Attributes

The BSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| cNSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| bsfInfo | O | T | T | F | T |

#### 5.3.165.3	Attribute constraints

None.

#### 5.3.165.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.166	BsfInfo <<dataType>>

#### 5.3.166.1	Definition

This data type represents the Specific data for the BSF. (See clause 6.1.6.2.21 TS 29.510 [23]).

#### 5.3.166.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4AddressRanges | O | T | T | F | T |
| dnnList | O | T | T | F | T |
| ipDomainList | O | T | T | F | T |
| ipv6PrefixRanges | O | T | T | F | T |
| rxDiamHost | CM | T | T | F | T |
| rxDiamRealm | CM | T | T | F | T |
| groupId | O | T | T | F | T |
| supiRanges | O | T | T | F | T |
| gpsiRanges | O | T | T | F | T |

#### 5.3.166.3	Attribute constraints

| Name | Definition |
| --- | --- |
| rxDiamHost CM S | Condition: the BSF supports Rx interface. |
| rxDiamRealm CM S | Condition: the BSF supports Rx interface. |

#### 5.3.166.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.167	AANFFunction

#### 5.3.167.1	Definition

This IOC represents the AAnF function in 5GC. For more information about the AANF, see TS 33.535 [91].

#### 5.3.167.2	Attributes

The AANFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| aanfInfo | O | T | T | F | T |

#### 5.3.167.3	Attribute constraints

None.

#### 5.3.167.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.168	AanfInfo <<dataType>>

#### 5.3.168.1	Definition

This data type represents the Specific data for the AAnF. (See clause 6.1.6.2.73 TS 29.510 [23]).

#### 5.3.168.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| routingIndicators | O | T | T | F | T |

#### 5.3.168.3	Attribute constraints

None.

#### 5.3.168.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.169	EP_N61

#### 5.3.169.1	Definition

This IOC represents an end point of N61 interface between AUSF and AAnF, which is defined in TS 23.501 [2] and TS 33.535 [91].

#### 5.3.169.2	Attributes

The EP_N61 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.169.3	Attribute constraints

None.

#### 5.3.169.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.170	EP_N62

#### 5.3.170.1	Definition

This IOC represents an end point of N62 interface between AF and AAnF, which is defined in TS 23.501 [2] and TS 33.535 [91].

#### 5.3.170.2	Attributes

The EP_N62 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.170.3	Attribute constraints

None.

#### 5.3.170.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.171	EP_N63

#### 5.3.171.1	Definition

This IOC represents an end point of N63 interface between NEF and AAnF, which is defined in TS 23.501 [2] and 33.535 [91].

#### 5.3.171.2	Attributes

The EP_N63 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.171.3	Attribute constraints

None.

#### 5.3.171.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.172	TSCTSFFunction

#### 5.3.172.1	Definition

This IOC represents the TSCTSF function in 5GC. For more information about the TSCTSF, see TS 23.501 [2].

#### 5.3.172.2	Attributes

The TSCTSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| tsctsfInfo | O | T | T | F | T |

#### 5.3.172.3	Attribute constraints

None.

#### 5.3.172.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.173	TsctsfInfo <<dataType>>

#### 5.3.173.1	Definition

This data type represents the Specific data for the TSCTSF. (See clause 6.1.6.2.91 TS 29.510 [23]).

#### 5.3.173.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiInfoList | O | T | T | F | T |
| externalGroupIdentifiersRanges | O | T | T | F | T |
| supiRanges | O | T | T | F | T |
| gpsiRanges | O | T | T | F | T |
| internalGroupIdentifiersRanges | O | T | T | F | T |

#### 5.3.173.3	Attribute constraints

None.

#### 5.3.173.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.174	EP_N84

#### 5.3.174.1	Definition

This IOC represents an end point of N84 interface between TSCTSF and PCF, which is defined in TS 23.501 [2].

#### 5.3.174.2	Attributes

The EP_N84 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.174.3	Attribute constraints

None.

#### 5.3.174.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.175	EP_N85

#### 5.3.175.1	Definition

This IOC represents an end point of N85 interface between TSCTSF and NEF, which is defined in TS 23.501 [2].

#### 5.3.175.2	Attributes

The EP_N85 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.175.3	Attribute constraints

None.

#### 5.3.175.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.176	EP_N86

#### 5.3.176.1	Definition

This IOC represents an end point of N86 interface between TSCTSF and AF, which is defined in TS 23.501 [2].

#### 5.3.176.2	Attributes

The EP_N86 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.176.3	Attribute constraints

None.

#### 5.3.176.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.177	EP_N87

#### 5.3.177.1	Definition

This IOC represents an end point of N87 interface between TSCTSF and UDM, which is defined in TS 23.501 [2].

#### 5.3.177.2	Attributes

The EP_N87 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.177.3	Attribute constraints

None.

#### 5.3.177.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.178	EP_N89

#### 5.3.178.1	Definition

This IOC represents an end point of N89 interface between TSCTSF and AMF, which is defined in TS 23.501 [2].

#### 5.3.178.2	Attributes

The EP_N89 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.178.3	Attribute constraints

None.

#### 5.3.178.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.179	EP_N96

#### 5.3.179.1	Definition

This IOC represents an end point of N96 interface between TSCTSF and NRF, which is defined in TS 23.501 [2].

#### 5.3.179.2	Attributes

The EP_N96 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.179.3	Attribute constraints

None.

#### 5.3.179.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.180	GMLCFunction

#### 5.3.180.1	Definition

This IOC represents the GMLC function in 5GC. For more information about the 5G GMLC, see TS 23.501 [2] and TS 23.273 [93].

#### 5.3.180.2	Attributes

The GMLCFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| gmlcInfo | O | T | T | F | T |

#### 5.3.180.3	Attribute constraints

None.

#### 5.3.180.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.181	GmlcfInfo <<dataType>>

#### 5.3.181.1	Definition

This data type represents the Specific data for the GMLC. (See clause 6.1.6.2.47 TS 29.510 [23]).

#### 5.3.181.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servingClientTypes | O | T | T | F | T |
| gmlcNumbers | O | T | T | F | T |

#### 5.3.181.3	Attribute constraints

None.

#### 5.3.181.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.182	EP_NL3

#### 5.3.182.1	Definition

This IOC represents the NL3 interface between vGMLC and hGMLC, which is defined in TS 23.273 [93].

#### 5.3.182.2	Attributes

The EP_NL3 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.182.3	Attribute constraints

None.

#### 5.3.182.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.183	EP_NL5

#### 5.3.183.1	Definition

This IOC represents the NL5 interface between NEF and GMLC, which is defined in TS 23.273 [93].

#### 5.3.183.2	Attributes

The EP_NL5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.183.3	Attribute constraints

None.

#### 5.3.183.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.184	EP_NL6

#### 5.3.184.1	Definition

This IOC represents the NL6 interface between UDM and GMLC, which is defined in TS 23.273 [93].

#### 5.3.184.2	Attributes

The EP_NL6 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.184.3	Attribute constraints

None.

#### 5.3.184.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.185	EP_NL9

#### 5.3.185.1	Definition

This IOC represents the NL9 interface between NWDAF and GMLC, which is defined in TS 23.273 [93].

#### 5.3.185.2	Attributes

The EP_NL9 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.185.3	Attribute constraints

None.

#### 5.3.185.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.186	NTNPLMNRestrictionsInfo <<dataType>>

#### 5.3.186.1	Definition

This datatype defines the location restrictions per PLMN that relates to non-terrestrial network access.

#### 5.3.186.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| blockedLocationInfoList | M | T | T | F | T |

#### 5.3.186.3	Attribute constraints

None.

#### 5.3.186.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.187	BlockedLocationInfo <<dataType>>

#### 5.3.187.1	Definition

This datatype defines the information related with the location for which the PLMN access restrictions are to be applied in case of NTN

#### 5.3.187.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| blockedLocation | M | T | T | F | T |
| blockedDurWindow | M | T | T | F | T |
| blockedSlice | M | T | T | F | T |

#### 5.3.187.3	Attribute constraints

None.

#### 5.3.187.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.188	SatelliteCoverageInfo <<dataType>>

#### 5.3.188.1	Definition

This is the <<datatype>> for attribute satelliteCoverageInfoList.

This datatype defines information related to NR Satellite RAT type and corresponding information of satellite coverage.

#### 5.3.188.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nRSatelliteRATtype | M | T | T | F | T |
| locationInfo | M | T | T | F | T |

#### 5.3.188.3	Attribute constraints

None.

#### 5.3.188.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.189	NtnLocationInfo <<dataType>>

#### 5.3.189.1	Definition

This datatype defines the information about locations and corresponding time windows for which the satellite coverage will be available or unavailable.

#### 5.3.189.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| location | M | T | T | F | T |
| availabilityWindows | CM | T | T | F | T |
| nonAvailabilityWindows | CM | T | T | F | T |

#### 5.3.189.3	Attribute constraints

| Name | Definition |
| --- | --- |
| availabilityWindows S | Condition: Either availabilityWindows or nonAvailabilityWindows will be present. |
| nonAvailabilityWindows S | Condition: Either availabilityWindows or nonAvailabilityWindows will be present. |

#### 5.3.189.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.190	N2InterfaceAmfInfo <<dataType>>

#### 5.3.190.1	Definition

This data type represents AMF N2 interface information. (See clause 6.1.6.2.26 TS 29.510 [23]).

#### 5.3.190.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4EndpointAddress | CM | T | T | F | T |
| ipv6EndpointAddress | CM | T | T | F | T |
| amfName | O | T | T | F | T |

#### 5.3.190.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4EndpointAddress CM S | Condition: if ipv6EndpointAddress is not included. |
| ipv6EndpointAddress CM S | Condition: if ipv4EndpointAddress is not included.. |

#### 5.3.190.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.191	A2xCapability <<dataType>>

#### 5.3.191.1	Definition

This data type indicates the supported A2X Capability by the PCF. (See clause 6.1.6.2.120 TS 29.510 [23]).

#### 5.3.191.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| lteA2x | O | T | T | F | T |
| nrA2x | O | T | T | F | T |

#### 5.3.191.3	Attribute constraints

None.

#### 5.3.191.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.192	MBUPFFunction

#### 5.3.192.1	Definition

This IOC represents the MB-UPF function defined in TS 23.501 [2] and TS 23.247 [96].

#### 5.3.192.2	Attributes

The MBUPFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| mbUpfInfo | O | T | T | F | T |

#### 5.3.192.3	Attribute constraints

None.

#### 5.3.192.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.193	MbUpfInfo <<dataType>>

#### 5.3.193.1	Definition

This data type represents the specific data for the MB-UPF. (See clause 6.1.6.2.94 TS 29.510 [23]).

#### 5.3.193.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiMbUpfInfoList | M | T | T | F | T |
| mbSmfServingArea | O | T | T | F | T |
| interfaceMbUpfInfoList | O | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| priority | O | T | T | F | T |
| supportedPfcpFeatures | O | T | T | F | T |

#### 5.3.193.3	Attribute constraints

None.

#### 5.3.193.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.194	SnssaiUpfInfoItem <<dataType>>

#### 5.3.194.1	Definition

This data type represents set of parameters supported by UPF for a given S-NSSAI. (See clause 6.1.6.2.14 TS 29.510 [23]).

#### 5.3.194.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssai | M | T | T | F | T |
| dnnUpfInfoList | M | T | T | F | T |
| redundantTransport | O | T | T | F | T |

#### 5.3.194.3	Attribute constraints

None.

#### 5.3.194.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.195	DnnUpfInfoItem <<dataType>>

#### 5.3.195.1	Definition

This data type represents set of parameters supported by UPF for a given DNN. (See clause 6.1.6.2.15 TS 29.510 [23]).

#### 5.3.195.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnn | M | T | T | F | T |
| dnaiList | O | T | T | F | T |
| pduSessionTypes | O | T | T | F | T |
| ipv4AddressRanges | O | T | T | F | T |
| ipv6PrefixRanges | O | T | T | F | T |
| natedIpv4AddressRanges | O | T | T | F | T |
| natedIpv6PrefixRanges | O | T | T | F | T |
| ipv4IndexList | O | T | T | F | T |
| ipv6IndexList | O | T | T | F | T |
| networkInstance | O | T | T | F | T |
| dnaiNwInstanceList | O | T | T | F | T |

#### 5.3.195.3	Attribute constraints

| mbsAreaSessions CM S | Condition: For an MBS session with location dependent content. |
| --- | --- |

#### 5.3.195.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.196	EP_N3mb

#### 5.3.196.1	Definition

This IOC represents the N3mb interface between MB-UPF and NG-RAN, which is defined in TS 23.247 [96].

#### 5.3.196.2	Attributes

The EP_N3mb IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.196.3	Attribute constraints

None.

#### 5.3.196.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.197	EP_N19mb

#### 5.3.197.1	Definition

This IOC represents the N19mb interface between MB-UPF and UPF, which is defined in TS 23.247 [96].

#### 5.3.197.2	Attributes

The EP_N19mb IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.197.3	Attribute constraints

None.

#### 5.3.197.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.198	EP_N4mb

#### 5.3.198.1	Definition

This IOC represents the N4mb interface between MB-UPF and MB-SMF, which is defined in TS 23.247 [96].

#### 5.3.198.2	Attributes

The EP_N4mb IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.198.3	Attribute constraints

None.

#### 5.3.198.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.199	EP_Nmb9

#### 5.3.199.1	Definition

This IOC represents the Nmb9 interface between MB-UPF and MBSTF, which is defined in TS 23.247 [96].

#### 5.3.199.2	Attributes

The EP_Nmb9 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.199.3	Attribute constraints

None.

#### 5.3.199.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.200	MBSMFFunction

#### 5.3.200.1	Definition

This IOC represents the MB-SMF function defined in TS 23.501 [2] and TS 23.247 [98].

#### 5.3.200.2	Attributes

The MBSMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| mbSmfInfo | O | T | T | F | T |

#### 5.3.200.3	Attribute constraints

None.

#### 5.3.200.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.201	Void

### 5.3.202	MbSmfInfo <<dataType>>

#### 5.3.202.1	Definition

This data type represents the specific data for the MB-SMF. (See clause 6.1.6.2.85 TS 29.510 [23]).

#### 5.3.202.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssaiInfoList | O | T | T | F | T |
| tmgiRangeList | O | T | T | F | T |
| taiList | O | T | T | F | T |
| taiRangeList | O | T | T | F | T |
| mbsSessionList | O | T | T | F | T |

#### 5.3.202.3	Attribute constraints

None.

#### 5.3.202.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.203	TmgiRange <<dataType>>

#### 5.3.203.1	Definition

This data type represents range of TMGIs. (See clause 6.1.6.2.86 TS 29.510 [23]).

#### 5.3.203.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mbsServiceIdStart | M | T | T | F | T |
| mbsServiceIdEnd | M | T | T | F | T |
| pLMNId | M | T | T | F | T |
| nId | O | T | T | F | T |

#### 5.3.203.3	Attribute constraints

None.

#### 5.3.203.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.204	MbsSession <<dataType>>

#### 5.3.204.1	Definition

This data type represents MBS Session served by an MB-SMF. (See clause 6.1.6.2.87 TS 29.510 [23]).

#### 5.3.204.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | M | T | T | F | T |
| mbsAreaSessions | CM | T | T | F | T |

#### 5.3.204.3	Attribute constraints

| mbsAreaSessions CM S | Condition: For an MBS session with location dependent content. |
| --- | --- |

#### 5.3.204.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.205	SnssaiMbSmfInfoItem <<dataType>>

#### 5.3.205.1	Definition

This data type represents parameters supported by an MB-SMF for a given S-NSSAI. (See clause 6.1.6.2.88 TS 29.510 [23]).

#### 5.3.205.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sNssai | M | T | T | F | T |
| dnnInfoList | M | T | T | F | T |

#### 5.3.205.3	Attribute constraints

None.

#### 5.3.205.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.206	DnnMbSmfInfoItem <<dataType>>

#### 5.3.206.1	Definition

This data type represents parameters supported by an MB-SMF for a given DNN. (See clause 6.1.6.2.89 TS 29.510 [23]).

#### 5.3.206.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnn | M | T | T | F | T |

#### 5.3.206.3	Attribute constraints

None.

#### 5.3.206.4	Notifications

The subclause 5.5  of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.207	MbsServiceAreaInfo <<dataType>>

#### 5.3.207.1	Definition

This data type represents MBS Service Area Information for Location dependent MBS session. (See clause 5.9.4.15 TS 29.571 [61]).

#### 5.3.207.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| areaSessionId | M | T | T | F | T |
| mbsServiceArea | M | T | T | F | T |

#### 5.3.207.3	Attribute constraints

None.

#### 5.3.207.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.208	Ssm <<dataType>>

#### 5.3.208.1	Definition

This data type represents Source specific IP multicast address identifying the MBS session. (See clause 5.9.4.3 TS 29.571 [61]).

#### 5.3.208.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sourceIpAddr | O | T | T | F | T |
| destIpAddr | O | T | T | F | T |

#### 5.3.208.3	Attribute constraints

None.

#### 5.3.208.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.209	Tmgi <<dataType>>

#### 5.3.209.1	Definition

This data type represents TMGI identifying the MBS session. (See clause 5.9.4.2 TS 29.571 [61]).

#### 5.3.209.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mbsServiceId | O | T | T | F | T |
| pLMNId | O | T | T | F | T |

#### 5.3.209.3	Attribute constraints

None.

#### 5.3.209.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.210	MbsServiceId <<dataType>>

#### 5.3.210.1	Definition

This data type represents MBS session identifier. (See clause 5.9.4.1 TS 29.571 [61]).

#### 5.3.210.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tmgi | O | T | T | F | T |
| Ssm | O | T | T | F | T |
| nId | O | T | T | F | T |

#### 5.3.210.3	Attribute constraints

None.

#### 5.3.210.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.211	MbsServiceArea <<dataType>>

#### 5.3.211.1	Definition

This data type represents MBS Service Area Information for Location dependent MBS session. (See clause 5.9.4.4 TS 29.571 [61]).

#### 5.3.211.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ncgiList | O | T | T | F | T |
| taiList | O | T | T | F | T |

#### 5.3.211.3	Attribute constraints

None.

#### 5.3.211.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.212	Ncgi <<dataType>>

#### 5.3.212.1	Definition

This data type represents the NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. (See clause 8.2 of TS 38.300 [3] and clause 5.4.4.6 TS 29.571 [61]).

#### 5.3.212.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| nrCellId | M | T | T | F | T |
| nId | CM | T | T | F | T |

#### 5.3.212.3	Attribute constraints

| nId CM S | Condition: Network Identifier, shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the NR cell belongs to. |
| --- | --- |

#### 5.3.212.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.213	EP_N16mb

#### 5.3.213.1	Definition

This IOC represents the N16mb interface between MB-SMF and SMF, which is defined in TS 23.247 [98].

#### 5.3.213.2	Attributes

The EP_N16mb IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.213.3	Attribute constraints

None.

#### 5.3.213.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.214	EP_N11mb

#### 5.3.214.1	Definition

This IOC represents the N11mb interface between AMF and MB-SMF, which is defined in TS 23.247 [98].

#### 5.3.214.2	Attributes

The EP_N11mb IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.214.3	Attribute constraints

None.

#### 5.3.214.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.215	EP_Nmb1

#### 5.3.215.1	Definition

This IOC represents the Nmb1 interface between MB-SMF and MBSF, which is defined in TS 23.247 [98].

#### 5.3.215.2	Attributes

The EP_Nmb1 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.215.3	Attribute constraints

None.

#### 5.3.215.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.216	5GDdnmfInfo <<dataType>>

#### 5.3.216.1	Definition

This data type represents the Specific data for the 5G DDMF NF. (See clause 6.1.6.2.74 TS 29.510 [23]).

#### 5.3.216.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |

#### 5.3.216.3	Attribute constraints

None.

#### 5.3.216.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.217	HssInfo <<dataType>>

#### 5.3.217.1	Definition

This data type represents the Specific data for the HSS NF. (See clause 6.1.6.2.57 TS 29.510 [23]).

#### 5.3.217.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | O | T | T | F | T |
| imsiRanges | O | T | T | F | T |
| imsPrivateIdentityRanges | O | T | T | F | T |
| imsPublicIdentityRanges | O | T | T | F | T |
| msisdnRanges | O | T | T | F | T |
| externalGroupIdentifiersRanges | O | T | T | F | T |
| hssDiameterAddress | O | T | T | F | T |
| additionalDiamAddresses | O | T | T | F | T |

#### 5.3.217.3	Attribute constraints

None.

#### 5.3.217.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.218	NetworkNodeDiameterAddress <<dataType>>

#### 5.3.218.1	Definition

This data type represents the Diameter Address of a Network Node. (See clause 6.2.6.2.10 TS 29.503 [97]).

#### 5.3.218.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| name | O | T | T | F | T |
| realm | O | T | T | F | T |

#### 5.3.218.3	Attribute constraints

None.

#### 5.3.218.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.219	ImsiRange <<dataType>>

#### 5.3.219.1	Definition

This data type represents a range of IMSIs (subscriber identities), either based on a numeric range, or based on regular-expression matching. (See clause 6.1.6.2.58 TS 29.510 [23]).

#### 5.3.219.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| start | O | T | T | F | T |
| end | O | T | T | F | T |
| pattern | O | T | T | F | T |

#### 5.3.219.3	Attribute constraints

None.

#### 5.3.219.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.220	MNPFFunction

#### 5.3.220.1	Definition

This IOC represents the MNPF NF defined in TS 23.540 [100].

#### 5.3.220.2	Attributes

The MNPFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| mnpfInfo | O | T | T | F | T |

#### 5.3.220.3	Attribute constraints

None.

#### 5.3.220.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.221	MnpfInfo <<dataType>>

#### 5.3.221.1	Definition

This data type represents the specific data for the MNPF. (See clause 6.1.6.2.109 TS 29.510 [23]).

#### 5.3.221.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| msisdnRanges | M | T | T | F | T |

#### 5.3.221.3	Attribute constraints

None.

#### 5.3.221.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.222	EP_SM12

#### 5.3.222.1	Definition

This IOC represents the SM12 interface between MNPF and SMS-GMSC, and it is defined in TS 23.540 [100].

#### 5.3.222.2	Attributes

The EP_SM12 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.222.3	Attribute constraints

None.

#### 5.3.222.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.223	EP_SM13

#### 5.3.223.1	Definition

This IOC represents the SM13 interface between MNPF and SCP, and it is defined in TS 23.540 [100].

#### 5.3.223.2	Attributes

The EP_SM13 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.223.3	Attribute constraints

None.

#### 5.3.223.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.224	EP_SM14

#### 5.3.224.1	Definition

This IOC represents the SM14 interface between MNPF and NRF, and it is defined in TS 23.540 [100].

#### 5.3.224.2	Attributes

The EP_SM14 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.224.3	Attribute constraints

None.

#### 5.3.224.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.225	EP_N34

#### 5.3.225.1	Definition

This IOC represents the N34 interface between NWDAF and NSSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.225.2	Attributes

The EP_N34 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.225.3	Attribute constraints

None.

#### 5.3.225.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.226	AnLFFunction

#### 5.3.226.1	Definition

This IOC represents the Analytics logical function (AnLF) contained by NWDAF (see TS 23.288 [101]).

The AnLF may be supported by AI/ML feature (See TS 28.105 [105]). Attribute mLModelRef indicates that AI/ML is supported for this function. Attribute AIMLInferenceFunctionRef indicates that AI/ML Inference Function is supported for this function.

#### 5.3.226.2	Attributes

This IOC includes attributes inherited from ManagedFunction (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| activationStatus | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRefList | CM | T | F | F | T |
| aIMLInferenceFunctionRefList | CM | T | F | F | T |

#### 5.3.226.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mLModelRefList | The condition is "AIML feature is supported". |
| aIMLInferenceFunctionRefList | The condition is "AIMLInferenceFunction is supported". |

#### 5.3.226.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.227	DnnTsctsfInfoItem <<dataType>>

#### 5.3.227.1	Definition

This data type represents set of parameters supported by TSCTSF for a given DNN. (See clause 6.1.6.2.93 TS 29.510 [23]).

#### 5.3.227.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnn | M | T | T | F | T |

#### 5.3.227.3	Attribute constraints

None.

#### 5.3.227.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.228	SliceExpiryInfo <<dataType>>

#### 5.3.228.1	Definition

This <<dataType>> represents the information about the time at which the network slice is scheduled to be expired as it is not required anymore.

#### 5.3.228.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfo | M | T | F | F | T |
| expiryTime | M | T | F | F | T |

#### 5.3.228.3	Attribute constraints

None.

#### 5.3.228.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.229	NfInfo <<dataType>>

#### 5.3.229.1	Definition

This data type represents the information of a generic NF Instance. (See clause 6.1.6.2.56 TS 29.510 [23]).

#### 5.3.229.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfType | M | T | T | F | T |

#### 5.3.229.3	Attribute constraints

None.

#### 5.3.229.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.230	PcscfInfo <<dataType>>

#### 5.3.230.1	Definition

This data type represents the information of a P-CSCF NF Instance. (See clause 6.1.6.2.53 TS 29.510 [23]).

#### 5.3.230.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| accessType | O | T | T | F | T |
| dnnList | O | T | T | F | T |
| gmFqdn | O | T | T | F | T |
| gmIpv4Addresses | O | T | T | F | T |
| gmIpv6Addresses | O | T | T | F | T |
| mwFqdn | O | T | T | F | T |
| mwIpv4Addresses | O | T | T | F | T |
| mwIpv6Addresses | O | T | T | F | T |
| servedIpv4AddressRanges | O | T | T | F | T |

#### 5.3.230.3	Attribute constraints

None.

#### 5.3.230.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.231	SatelliteBackhaulInfo <<dataType>>

#### 5.3.231.1	Definition

This datatype defines information related to satellite backhaul category and corresponding information of (R)AN.

#### 5.3.231.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nTNGlobalRanNodeID | M | T | T | F | T |
| satelliteBackhaulCategory | M | T | T | F | T |
| geoSatelliteId | CM | T | T | F | T |

#### 5.3.231.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geoSatelliteId S | Condition: Present if UPF is deployed on a GEO satellite. NOTE 1: In the present document, only the feature of deploying UPF on GEO satellite is supported. |

#### 5.3.231.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.232	NTNGlobalRanNodeID <<dataType>>

#### 5.3.232.1	Definition

This datatype identifies which (R)AN node the satellite backhaul type is applicable to.

#### 5.3.232.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| CHOICE_1 n3IwfId | CM | T | T | F | T |
| CHOICE_2 gNbId | CM | T | T | F | T |
| CHOICE_3 ngeNbId | CM | T | T | F | T |
| CHOICE_4 wagfId | CM | T | T | F | T |
| CHOICE_5 tngfId | CM | T | T | F | T |
| CHOICE_6 twifId | CM | T | T | F | T |

#### 5.3.232.3	Attribute constraints

| Name | Definition |
| --- | --- |
| n3IwfId S | Condition: Present if the (R)AN Node represents a N3IWF. |
| gNbId S | Condition: Present if the (R)AN Node represents a gNB. |
| ngeNbId S | Condition: Present if the (R)AN Node represents a NG-eNB. |
| wagfId S | Condition: Present if the (R)AN Node represents a W-AGF. |
| tngfId S | Condition: Present if the (R)AN Node represents a TNGF. |
| twifId S | Condition: Present if the (R)AN Node represents a TWIF. |

#### 5.3.232.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.233	DnaiSatelliteMapping <<dataType>>

#### 5.3.233.1	Definition

This data type provides the mapping between DNAI and GEO satellite ID, with each GEO Satellite ID assigned one or more DNAI values. SMF uses this attribute to select UPF on-board GEO satellite as the PSA UPF or UL CL/BP and local PSA.

#### 5.3.233.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnaiList | M | T | T | F | T |
| geoSatelliteId | M | T | T | F | T |

#### 5.3.233.3	Attribute constraints

None.

#### 5.3.233.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.234	TrpInfo <<dataType>>

#### 5.3.234.1	Definition

This data type represents the mapping relationship between gNB and TRPs.

#### 5.3.234.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| trpMappingInfoList | M | T | T | F | T |

#### 5.3.234.3	Attribute constraints

None

#### 5.3.234.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.235	TrpMappingInfo <<dataType>>

#### 5.3.235.1	Definition

This data type represents the mapping relationship between satellite and TRPs.

#### 5.3.235.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| satelliteId | CO | T | T | F | T |
| trpIds | M | T | T | F | T |

NOTE: TRP ID should also be configured in NR NRM.

#### 5.3.235.3	Attribute constraints

| Name | Definition |
| --- | --- |
| satelliteId | Condition: Present if NR satellite access is used |

#### 5.3.235.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.236	NFProfile <<dataType>>

#### 5.3.236.1	Definition

This data type represents the NF profile stored in NRF, defined in TS 29.510 [23].

#### 5.3.236.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfInstanceId | M | T | F | T | T |
| nfType | M | T | F | F | T |
| nfStatus | M | T | T | F | T |
| plmnList | CM | T | T | F | T |
| sNssais | O | T | T | F | T |
| fqdn | M | T | T | F | T |
| interPlmnFqdn | CM | T | T | F | T |
| nfServices | O | T | T | F | T |

#### 5.3.236.3	Attribute constraints

| Name | Definition |
| --- | --- |
| plmnList | Condition: if plmnList is available for the NF. |
| interPlmnFqdn | Condition: if the NF needs to be discoverable by other NFs in a different PLMN |

#### 5.3.236.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.237	NFService <<dataType>>

#### 5.3.237.1	Definition

This data type represents the NF Service defined in TS 29.510 [23].

#### 5.3.237.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceInstanceId | M | T | F | T | T |
| serviceName | M | T | F | T | T |
| versions | M | T | F | F | T |
| schema | M | T | T | F | T |
| fqdn | O | T | T | F | T |
| interPlmnFqdn | O | T | T | F | T |
| ipEndPoints | O | T | T | F | T |
| apiPrefix | O | T | T | F | T |
| allowedPLMNs | O | T | T | F | T |
| allowedNfTypes | O | T | T | F | T |
| allowedNSSAIs | O | T | T | F | T |

#### 5.3.237.3	Attribute constraints

None.

#### 5.3.237.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aMFIdentifier | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aMFSetId | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aMFSetMemberList | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aMFRegionId | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gUAMIdList | List of supported Globally Unique AMF Ids (GUAMIs). | type: GUAMInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| backupInfoAmfFailure | List of GUAMIs for which the AMF acts as a backup for AMF failure. | type: GUAMInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| backupInfoAmfRemoval | List of GUAMIs for which the AMF acts as a backup for planned AMF removal. | type: GUAMInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). | type: String multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nFProfileList | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. allowedValues: N/A | type: NFProfile multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cNSIIdList | It is a set of NSI ID. NSI ID is an identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. See NSI ID definition in clause 3.1 of TS 23.501 [2] and subclause 6.1.6.2.7 of TS 29.531 [24]. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySavingControl | This attribute allows management system to initiate energy saving activation or deactivation for the edge UPF. allowedValues: TO_BE_ENERGYSAVING, TO_BE_NOT_ENERGYSAVING. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| energySavingState | This attribute specifies the status regarding the energy saving in the edge UPF. If the value of energySavingControl is TO_BE_ENERGYSAVING, then it shall be tried to achieve the value IS_ENERGYSAVING for the energySavingState. If the value of energySavingControl is TO_BE_NOT_ENERGYSAVING, then it shall be tried to achieve the value IS_NOT_ENERGYSAVING for the energySavingState. allowedValues: IS_NOT_ENERGYSAVING, IS_ENERGYSAVING. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sNSSAIList | See subclause 4.4.1. |  |
| pLMNInfoList | It defines the PLMN(s) of a Network Function. | type: PLMNInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sBIFQDN | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| interPlmnFqdn | If the NF needs to be discoverable by other NFs in a different PLMN, then an FQDN that is used for inter-PLMN routing as specified in 3GPP TS 23.003 [13] shall be registered with the NRF. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sBIServiceList | It is used to indicate the all supported NF services registered on service-based interface. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nRTAClist | It is the list of Tracking Area Codes (either legacy TAC or extended TAC). allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| taiList | The list of TAIs. | type: TAI multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| taiRangeList | The range of TAIs. | type: TAIRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sNssaiSmfInfoList | List of parameters supported by the SMF per S-NSSAI | type: SnssaiSmfInfoItem multiplicity: * isOrdered: False isUnique: Ture defaultValue: None isNullable: False |
| dnnSmfInfoList | List of parameters supported by the SMF per DNN | type: DnnSmfInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dnn | String representing a Data Network as defined in clause 9A of 3GPP TS 23.003 [13]; it shall contain either a DNN Network Identifier, or a full DNN with both the Network Identifier and Operator Identifier, as specified in 3GPP TS 23.003 [13] clause 9.1.1 and 9.1.2. It shall be coded as string in which the labels are separated by dots (e.g. "Label1.Label2.Label3"). Whether the dnn data type contains just the DNN Network Identifier, or the Network Identifier plus the Operator Identifier, shall be documented in each API where this data type is used. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dnaiList | List of Data network access identifiers supported for this DNN. allowedValues: DNAI (Data network access identifier), see clause 5.6.7 of 3GPP TS 23.501 [2]. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pgwFqdn | The FQDN of the PGW if the SMF is a combined SMF/PGW-C. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pgwIpAddrList | The PGW IP addresses of the combined SMF/PGW-C. It allows the NF Service consumer to find the target combined SMF/PGW-C by PGW IP Address, e.g., when only PGW IP Address is available. | type: IpAddr multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| vsmfSupportInd | Used by an SMF to explicitly indicate the support of V-SMF capability and its preference to be selected as V-SMF. When present it indicate whether the V-SMF capability is supported by the SMF: - TRUE: V-SMF capability supported by the SMF - FALSE: V-SMF capability not supported by the SMF. When absent the V-SMF capability support of the SMF is not specified. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pgwFqdnList | When present, this attribute provides additional FQDNs to the FQDN indicated in the pgwFqdn attribute. The pgwFqdnList attribute may be present if the pgwFqdn attribute is present. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nRTACRangeList | The range of TACs. | type: NRTACRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nRTACstart | First value identifying the start of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g., TAC ranges). 3-octet string identifying a tracking area code, each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TAC shall appear first in the string, and the character representing the 4 least significant bit of the TAC shall appear last in the string. Pattern: "^([A-Fa-f0-9]{4}¦[A-Fa-f0-9]{6})$" | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTACend | Last value identifying the end of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g. TAC ranges). 3-octet string identifying a tracking area code, each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TAC shall appear first in the string, and the character representing the 4 least significant bit of the TAC shall appear last in the string. Pattern: "^([A-Fa-f0-9]{4}¦[A-Fa-f0-9]{6})$" | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTACpattern | Pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of TAC's belonging to this range. A TAC value is considered part of the range if and only if the TAC string fully matches the regular expression. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedBMOList | It is used to indicate the list of supported BMOs (Bridge Managed Objects) required for integration with TSN system. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| managedNFProfile | This parameter defines profile for managed NF (See TS 23.501 [2]). allowedValues: N/A | type: ManagedNFProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nfInstanceId | This parameter defines unique identity of the NF Instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [44] allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nfType | This parameter defines type of Network Function allowedValues: See TS 23.501[2] for NF types | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| heartBeatTimer | Time between two consecutive heart-beat messages from an NF Instance to the NRF defined in seconds. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| fqdn | This parameter defines FQDN of the Network Function (See TS 23.003 [13]) allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ipAddress | This parameter defines IP Address of the Network Function. It can be IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| authzInfo | This parameter defines NF Specific Service authorization information. It shall include the NF type (s) and NF realms/origins allowed to consume NF Service(s) of NF Service Producer (See TS 23.501[2]). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| allowedPLMNs | PLMNs allowed to access the NF instance. If not provided, any PLMN is allowed to access the NF. | type: PLMNId multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sNPNList | SNPN(s) of the Network Function. This attributeIE shall be present if the NF pertains to one or more SNPNs. (see clauses 6.1.6 in 3GPP TS 29.510 [23]). | type: SNPNInfoID multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| allowedSNPNs | SNPNs allowed to access the NF instance. The absence of this attribute in the NF profile indicates that no SNPN, other than the SNPN(s) registered in the snpnList attribute of the NF Profile, is allowed to access the service instance. | type: SNPNId multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mCC | This is the Mobile Country Code (MCC) of the PLMN identifier. See TS 23.003 [13] subclause 2.2 and 12.1. allowedValues: a bounded string of 3 characters representing 3 digits. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mNC | This is the Mobile Network Code (MNC) of the PLMN identifier. See TS 23.003 [13] subclause 2.2 and 12.1. allowedValues: A bounded string of 2 or 3 characters representing 2 or 3 digits. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nId | Network Identity; Shall be present if PlmnIdNid identifies an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in 3GPP TS 23.501 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| allowedNfTypes | Type of the NFs allowed to access the NF instance. If not provided, any NF type is allowed to access the NF. allowedValues: See TS 23.501[2] for NF types | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| allowedNfDomains | Pattern (regular expression according to the ECMA-262 dialect [75]) representing the NF domain names within the PLMN of the NRF allowed to access the NF instance. If not provided, any NF domain is allowed to access the NF. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| allowedNSSAIs | S-NSSAI of the allowed slices to access the NF instance. If not provided, any slice is allowed to access the NF. | type: S-NSSAI multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| locality | The parameter defines information about the location of the NF instance (e.g. geographic location, data center) defined by operator (See TS 29.510[23]). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| capacity | This parameter defines static capacity information in the range of 0-65535, expressed as a weight relative to other NF instances of the same type; if capacity is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]) allowedValues: 0-65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| recoveryTime | Timestamp when the NF was (re)started. The NRF shall notify NFs subscribed to receiving notifications of changes of the NF profile, if the NF recoveryTime is changed. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nfServicePersistence | This parameter indicates whether the different service instances of a same NF Service in the NF instance, supporting a same API version, are capable to persist their resource state in shared storage and therefore these resources are available after a new NF service instance supporting the same API version is selected by a NF Service Consumer (see TS 29.510 [23]). | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nfSetIdList | A NF Set Identifier is a globally unique identifier of a set of equivalent and interchangeable CP NFs from a given network that provide distribution, redundancy and scalability (see clause 5.21.3 of 3GPP TS 23.501 [2]). An NF Set Identifier shall be constructed from the MCC, MNC, NID (for SNPN), NF type and a Set ID. A NF Set Identifier shall be formatted as the following string: set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC> for a NF Set in a PLMN, or set<Set ID>.<nftype>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC> for a NF Set in a SNPN. At most one NF Set ID shall be indicated per PLMN-ID or SNPN of the NF. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nfProfileChangesSupportInd | This parameter indicates if the NF Service Consumer supports or does not support receiving NF Profile Changes. It may be present in the NFRegister or NFUpdate (NF Profile Complete Replacement) request and shall be absent in the response (see Annex B 3GPP TS 29.510 [23]). | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| defaultNotificationSubscriptions | Notification endpoints for different notification types. This attribute may contain multiple default subscriptions for a same notification type; in that case, those default subscriptions are used as alternative notification endpoints. | type: DefaultNotificationSubscription multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| notificationType | This parameter indicates the types of notifications used in Default Notification URIs in the NF Profile of an NF Instance. allowedValues: "N1_MESSAGES", "N2_INFORMATION", "LOCATION_NOTIFICATION", ”DATA_REMOVAL_NOTIFICATION”, "DATA_CHANGE_NOTIFICATION", "LOCATION_UPDATE_NOTIFICATION", "NSSAA_REAUTH_NOTIFICATION", "NSSAA_REVOC_NOTIFICATION" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| callbackURI | This attribute contains a default notification endpoint to be used by a NF Service Producer towards an NF Service Consumer that has not registered explicitly a callback URI in the NF Service Producer (e.g. as a result of an implicit subscription). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n1MessageClass | This attribute (if it is present) identifies that class of N1 messages shall be notified as per TS 29.518 [80]. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n2InformationClass | This attribute (if it is present) identifies that class of N2 messages shall be notified as per TS 29.518 [80]. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| versions | This attribute identifies the API versions (e.g. "v1") supported for the default notification type. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| binding | This attribute shall contain the value of the Binding Indication for the default subscription notification (i.e. the value part of "3gpp-Sbi-Binding" header), as specified in clause 6.12.4 of 3GPP TS 29.500 [76]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servingScope | This parameter indicates the served geographical areas of a NF instance. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lcHSupportInd | This parameter indicates whether the NF supports or does not support Load Control based on LCI Header (see clause 6.3 of 3GPP TS 29.500 [76]). | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| olcHSupportInd | This parameter indicates whether the NF supports or does not support Overload Control based on OCI Header (see clause 6.4 of 3GPP TS 29.500 [76]). | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| nfSetRecoveryTimeList | This parameter contains the recovery time of NF Set(s) indicated by the NfSetId, where the NF instance belongs. | type: DateTime multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceSetRecoveryTimeList | This parameter contains the recovery time of NF Service Set(s) configured in the NF instance, which are indicated by the NfServiceSetId. | type: DateTime multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| scpDomains | This parameter shall carry the list of SCP domains the SCP belongs to, or the SCP domain the NF (other than SCP) or the SEPP belongs to. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| vendorId | Vendor ID of the NF instance, according to the IANA-assigned "SMI Network Management Private Enterprise Codes" [77]. allowedValues: 6 decimal digits; if the SMI code has less than 6 digits, it shall be padded with leading digits "0" to complete a 6-digit string value. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| hostAddr | This parameter defines host address of a NF allowedValues: N/A | type: HostAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priority | This parameter defines Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection; lower values indicate a higher priority. If priority is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]). allowedValues: 0-65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedDataSets | This parameter defines list of supported data sets in the UDR instance (See TS 29.510[23]). allowedValues: "SUBSCRIPTION", "POLICY", EXPOSURE", "APPLICATION", "A_PFD", "A_AFTI", "A_IPTV", "A_BDT", "A_SPD", "A_EASD", "A_AMI", "P_UE", "P_SCD", "P_BDT", "P_PLMNUE", "P_NSSCD". | type: ENUM multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| nFSrvGroupId | This parameter defines identity of the group that is served by the NF instance (See TS 29.510[23]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| smfServingArea | This parameter defines the SMF service area(s) the UPF can serve (See TS 29.510[23]). If not provided, the UPF can serve any SMF service area. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| interfaceUpfInfoList | List of User Plane interfaces configured on the UPF. When this parameter is provided in the NF Discovery response, the NF Service Consumer (e.g., SMF) may use this information for UPF selection. | type: InterfaceUpfInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| interfaceType | This parameter defines the type of User Plane (UP) interface. allowedValues: "N3" "N6" "N9" "DATA_FORWARDING" "N6MB" "N19MB" "N3MB" "NMB9" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ipv4EndpointAddresses | Available endpoint IPv4 address(es) of the User Plane interface. | type: Ipv4Addr multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ipv6EndpointAddresses | Available endpoint IPv6 address(es) of the User Plane interface. | type: Ipv6Addr multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkInstance | Network Instance (See TS 29.244 [56]) associated to the User Plane interface | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iwkEpsInd | Indicates whether interworking with EPS is supported by the UPF. allowedValues: True: Supported False: Not Supported | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| pduSessionTypes | Indicates the type(s) of a PDU session. allowedValues: “IPV4” “IPV6” “IPV4V6” as per clause 5.8.2.2.1 TS 23.501 [2] “UNSTRUCTURED” “ETHERNET” | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| atsssCapability | Indicate the ATSSS capability of the UPF. | type: AtsssCapability multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| atsssLL | Indicates the ATSSS-LL capability to support procedures related to Access Traffic Steering, Switching, Splitting (see clauses 4.2.10, 5.32 of TS 23.501 [2]). allowedValues: True: Supported False: Not Supported | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| mptcp | Indicates the MPTCP capability to support procedures related to Access Traffic Steering, Switching, Splitting (see clauses 4.2.10, 5.32 of TS 23.501 [2]). allowedValues: True: Supported False: Not Supported | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| rttWithoutPmf | Indicates whether the UPF supports RTT measurement without PMF (see clauses 5.32.2, 6.3.3.3 of TS 23.501 [2]). allowedValues: True: Supported False: Not Supported. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| ueIpAddrInd | Indicates whether the UPF supports allocating UE IP addresses/prefixes. allowedValues: True: supported False: not supported | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| wAgfInfo | Indicate that the UPF is collocated with W-AGF. If not present, the UPF is not collocated with Wireline Access Gateway Function (W-AGF). | type: IpInterface multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tngfInfo | Indicate that the UPF is collocated with TNGF. If not present, the UPF is not collocated with Trusted Non-3GPP Gateway Function (TNGF). | type: IpInterface multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| twifInfo | Indicate that the UPF is collocated with TWIF. If not present, the UPF is not collocated with Trusted WLAN Interworking Function (TWIF). | type: IpInterface multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| redundantGtpu | Indicates whether the UPF supports redundant GTP-U path. allowedValues: True: supported False: not supported | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| ipups | Indicates whether the UPF is configured for Inter-PLMN User Plane Security (IPUPS). Any UPF can support the IPUPS functionality. In network deployments where specific UPFs are used to provide IPUPS, UPFs configured for providing IPUPS services shall be selected. allowedValues: True: The UPF is configured for IPUPS. False: The UPF is not configured for IPUPS | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| dataForwarding | Indicates whether the UPF is configured for data forwarding. Based on operator policies, if dedicated UPFs are preferred to be used for indirect data forwarding during handover scenarios, when setting up the indirect data forwarding tunnel, the SMF should preferably select a UPF configured for data forwarding and use the network instance indicated in the Network Instance ID associated to the DATA_FORWARDING interface type in the interfaceUpfInfoList attribute. allowedValues: True: the UPF is configured for data forwarding False: the UPF is not configured for data forwarding If the UPF is configured for data forwarding, it shall support UP network interface with type "DATA_FORWARDING". | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| supportedPfcpFeatures | Supported Packet Forwarding Control Protocol (PFCP) Features. A string used to indicate the PFCP features supported by the UPF, which encodes the "UP Function Features" as specified in Table 8.2.25-1 of TS 29.244 [56] (starting from Octet 5), in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and each two characters shall represent one octet of "UP Function Features" (starting from Octet 5, to higher octets). For each two characters representing one octet, the first character representing the 4 most significant bits of the octet and the second character the 4 least significant bits of the octet. The supported PFCP features shall be provisioned in addition and be consistent with the existing UPF features (atsssCapability, ueIpAddrInd, redundantGtpu and ipups), e.g., if the ueIpAddrInd is set to "true", then the UEIP flag shall also be set to "1" in the supported PFCP features. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isESCoveredBy | This indicates whether the adjacentCell provides no, partial or full coverage for the cell which name-contains the NRCellRelation instance. Adjacent cells with this attribute equal to "FULL" are recommended to be considered as candidate cells to take over the coverage when the original cell state is about to be changed to energySaving. All adjacent cells with this attribute value equal to "PARTIAL" are recommended to be considered as entirety of candidate cells to take over the coverage when the original cell state is about to be changed to energySaving. allowedValues: NO, PARTIAL, FULL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commModelList | The attribute specifies a list of commModel which is defined as a datatype (see clause 5.3.69). It can be used by NF and NF services to interact with each other in 5G Core network (see TS 23.501 [2]). allowedValues: Not applicable | type: CommModel multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| groupId | This parameter identiies a list of target NF services on which the same communication model is applied to. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commModelType | This parameter defines communication model used by a NF to interact with NF service(s) (See TS 23.501 [2]). allowedValues:”DIRECT_COMMUNICATION_WO_NRF”, “DIRECT_COMMUNICATION_WITH_NRF”, “INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY”, “INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetNFServiceList | This parameter lists target NF services sharing same communication model and configuration. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| commModelConfiguration | This parameter defines configuration parameters for specific communication model for a group of NF Services. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedFuncList | This parameter lists functionalities supported by a SCP. Refer to TS 23.501 [2]. | type: SupportedFunction multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| address | This parameter defines address of a SCP instance, it can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN (See TS 23.003 [13]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| function | This parameter defines name of a functionality supported by a SCP. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| policy | This parameter defines configuration policies of a functionality supported by a SCP. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| capabilityList | This parameter lists capabilities supported by a NEF. Refer to TS 23.501 [2]. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| isCAPIFSup | This parameter defines if the NEF support Common API Framework. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sEPPType | This parameter defines the type of a SEPP entity. Refer to TS 33.501 [52]. allowedValues: “CSEPP”, “PSEPP” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sEPPId | This parameter is identifier of a SEPP, it is unique inside a PLMN. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remotePlmnId | This parameter defines PLMNId of the remote SEPP. allowedValues: N/A | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteSeppAddress | This parameter defines address of the remote SEPP. It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN(See TS 23.003 [13]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteSeppId | This parameter defines identifier of the remote SEPP. it is unique inside a PLMN. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n32cParas | This attribute is used to configure parameters to establish security link between two SEPPs. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n32fPolicy | This attribute is used to configure policies to protect the messages exchanged between SEPPs. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| withIPX | This attribute defines if there’s an IPX interconnected between two SEPPs. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fiveQiDscpMappingList | It provides the list of mapping between 5QIs and DSCP. allowedValues: N/A | type: FiveQiDscpMapping multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| fiveQIValues | It indicates a list of 5QI value. allowedValues: 0 - 255 | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dscp | It indicates a DSCP. allowedValues: 0 – 255 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configurable5QISetRef | This is the DN of Configurable5QISet. allowedValues: DN of the Configurable5QISet MOI. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dynamic5QISetRef | This is the DN of Dynamic5QISet MOI. allowedValues: DN of the Dynamic5QISet MOI. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fiveQIValue | It identifies the 5QI value. allowedValues: 0 – 255 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| resourceType | It indicates the Resource Type of a 5QI, as specified in TS 23.501 [2]. allowedValues: "GBR", NON_GBR", "DELAY_CRITICAL_GBR" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priorityLevel | It indicates the Priority Level of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 127 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| packetDelayBudget | It indicates the Packet Delay Budget (in unit of 0.5ms) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 1023 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| packetErrorRate | It indicates the Packet Error Rate of a 5QI, as specified in TS 23.501 [2]. allowedValues: N/A | type: PacketErrorRate multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| averagingWindow | It indicates the Averaging Window (in unit of ms) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 4095 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maximumDataBurstVolume | It indicates the Maximum Data Burst Volume (in unit of Byte) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 4095 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scalar | The Packet Error Rate of a 5QI expressed as Scalar x 10-k where k is the Exponent. This attriutes indicates the Scalar of this expression. allowedValues: 0 - 9 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| exponent | The Packet Error Rate of a 5QI expressed as Scalar x 10-k where k is the Exponent. This attriutes indicates the Exponent of this expression. allowedValues: 0 - 9 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gtpUPathQoSMonitoringState | It indicates the state of GTP-U path QoS monitoring for URLLC service. allowedValues: "ENABLED", "DISABLED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: ENABLED isNullable: False |
| gtpUPathMonitoredSNSSAIs | It specifies the S-NSSAIs for which the GTP-U path QoS monitoring is to be performed. allowedValues: See 3GPP TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| monitoredDSCPs | It specifies the DSCPs for which the GTP-U path QoS monitoring is to be performed. allowedValues: See 3GPP TS 29.244 [56] | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| isEventTriggeredGtpUPathMonitoringSupported | It indicates whether the event triggered GTP-U path QoS monitoring reporting based on thresholds is supported, see 3GPP TS 29.244 [56]. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: False |
| isPeriodicGtpUMonitoringSupported | It indicates whether the periodic GTP-U path QoS monitoring reporting is supported, see 3GPP TS 29.244 [56]. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: False |
| isImmediateGtpUMonitoringSupported | It indicates whether the immediate GTP-U path QoS monitoring reporting is supported, see 3GPP TS 29.244 [56]. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: False |
| gtpUPathDelayThresholds | It specifies the thresholds for reporting the packet delay for the GTO-U path QoS monitoring, if the isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “yes”. The packet delay will be reported to SMF when it exceeds the threshold (in milliseconds). allowedValues: N/A. | type: GtpUPathDelayThresholdsType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gtpUPathMinimumWaitTime | It specifies the minimum waiting time (in seconds) between two consecutive reports for event triggered GTP-U path QoS monitoring reporting, if the isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gtpUPathMeasurementPeriod | It specifies the period (in seconds) for reporting the packet delay for GTP-U path QoS monitoring, if the isPeriodicGtpUMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n3AveragePacketDelayThreshold | It specifies the threshold for reporting the average packet delay of a GTP-U path on N3 interface. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n3MinPacketDelayThreshold | It specifies the threshold for reporting the minimum packet delay of a GTP-U path on N3 interface. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n3MaxPacketDelayThreshold | It specifies the threshold for reporting the maxinum packet delay of a GTP-U path on N3 interface. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n9AveragePacketDelayThreshold | It specifies the threshold for reporting the average packet delay of a GTP-U path on N9 interface. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n9MinPacketDelayThreshold | It specifies the threshold for reporting the minimum packet delay of a GTP-U path on N9 interface. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| n9MaxPacketDelayThreshold | It specifies the threshold for reporting the maxinum packet delay of a GTP-U path on N9 interface. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qFQoSMonitoringState | It indicates the state of QoS monitoring per QoS flow per UE for URLLC service. allowedValues: "ENABLED", "DISABLED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Enabled isNullable: False |
| qFMonitoredSNSSAIs | It specifies the S-NSSAIs for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: See 3GPP TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| qFMonitored5QIs | It specifies the 5QIs for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: See 3GPP TS 23.501[2] | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| isEventTriggeredQFMonitoringSupported | It indicates whether the event based QoS monitoring reporting per QoS flow per UE is supported, see 3GPP TS 29.244 [56]. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: False |
| isPeriodicQFMonitoringSupported | It indicates whether the periodic QoS monitoring reporting per QoS flow per UE is supported, see 3GPP TS 29.244 [56]. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: False |
| isSessionReleasedQFMonitoringSupported | It indicates whether the session release based QoS monitoring reporting per QoS flow per UE is supported, see 3GPP TS 29.244 [56]. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: False |
| qFPacketDelayThresholds | It specifies the thresholds for reporting the packet delay between PSA and UE for QoS monitoring per QoS flow per UE, if the isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “yes”.”. The packet delay will be reported by PSA UPF to SMF when it exceeds the threshold (in milliseconds). allowedValues: see 3GPP TS 29.244 [56]. | type: QFPacketDelayThresholdsType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qFMinimumWaitTime | It specifies the minimum waiting time (in seconds) between two consecutive reports for event triggered QoS monitoring reporting per QoS flow per UE, if the isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qFMeasurementPeriod | It specifies the period (in seconds) for reporting the packet delay for QoS monitoring per QoS flow per UE, if the isPeriodicQFMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdDl | It specifies the threshold for reporting the DL packet delay between PSA UPF and UE. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdUl | It specifies the threshold for reporting the UL packet delay between PSA UPF and UE. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdRtt | It specifies the threshold for reporting the round-trip packet delay between PSA UPF and UE. allowedValues: see 3GPP TS 29.244 [56]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| predefinedPccRules | It specifies the predefined PCC Rules, see TS 25.503 [59]. allowedValues: N/A | type: PccRule multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pccRuleId | It identifies the PCC rule. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flowInfoList | It is a list of IP flow packet filter information. allowedValues: N/A | type: FlowInformation multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| applicationId | A reference to the application detection filter configured at the UPF. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| appDescriptor | It is the ATSSS rule application descriptor. allowedValues: see TS 29.571 [61]. | type: BitString multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| contentVersion | Indicates the content version of the PCC rule. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| precedence | It indicates the order in which this PCC rule is applied relative to other PCC rules within the same PDU session. allowedValues: 0..255. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| afSigProtocol | Indicates the protocol used for signalling between the UE and the AF. allowedValues: “NO_INFORMATION”, “SIP”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “NO_INFORMATION” isNullable: False |
| isAppRelocatable | It indicates the application relocation possibility.allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| isUeAddrPreserved | It Indicates whether UE IP address should be preserved. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| qosData | It contains the QoS control policy data for a PCC rule. allowedValues: N/A | type: QoSData multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| altQosParams | It contains the QoS control policy data for the Alternative QoS parameter sets of the service data flow. Only the "qosId" attribute, "5qi" attribute, "maxbrUl" attribute, "maxbrDl" attribute, "gbrUl" attribute and "gbrDl" attribute are applicable within the QosData data type. This data type represents an ordered list, where the lower the index of the array for a given entry, the higher the priority. allowedValues: N/A | type: QoSData multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| trafficControlData | It contains the traffic control policy data for a PCC rule. allowedValues: N/A | type: TrafficControlData multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conditionData | It contains the condition data for a PCC rule. allowedValues: N/A | type: ConditionData multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tscaiInputUl | It contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction). allowedValues: N/A | type: TscaiInputContainer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tscaiInputDl | It contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction). allowedValues: N/A | type: TscaiInputContainer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flowDescription | It defines a packet filter for an IP flow. allowedValues: see TS 29.214 [62]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ethFlowDescription | It defines a packet filter for an Ethernet flow. allowedValues: see TS 29.514 [62]. | type: EthFlowDescription multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| destMacAddr | It specifies the destination MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ethType | A two-octet string that represents the Ethertype, as described in IEEE 802.3 [64] and IETF RFC 7042 [63] in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the ethType shall appear first in the string, and the character representing the 4 least significant bits of the ethType shall appear last in the string. allowedValues: see IEEE 802.3 [64] and IETF RFC 7042 [63]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fDesc | It contains the flow description for the Uplink or Downlink IP flow. It shall be present when the ethtype is IP. allowedValues: see flowDescription in TS 29.214 [62]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fDir | It indicates the packet filter direction. allowedValues: "DOWNLINK", "UPLINK". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sourceMacAddr | It specifies the source MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vlanTags | It specifies the Customer-VLAN and/or Service-VLAN tags containing the VID, PCP/DEI fields as defined in IEEE 802.1Q [65] and IETF RFC 7042 [63]. The first/lower instance in the array stands for the Customer-VLAN tag and the second/higher instance in the array stands for the Service-VLAN tag. Each field is encoded as a two-octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the PCP/DEI field shall appear first in the string, followed by character representing the 4 most significant bits of the VID field, and the character representing the 4 least significant bits of the VID field shall appear last in the string. If only Service-VLAN tag is provided, empty string for Customer-VLAN tag shall be provided. allowedValues: see IEEE 802.1Q [65] and IETF RFC 7042 [63]. | type: String multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| srcMacAddrEnd | It specifies the source MAC address end. If this attribute is present, the sourceMacAddr attribute specifies the source MAC address start. E.g. srcMacAddrEnd with value 00-10-A4-23-3E-FE and sourceMacAddr with value 00-10-A4-23-3E-02 means all MAC addresses from 00-10-A4-23-3E-02 up to and including 00-10-A4-23-3E-FE. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| destMacAddrEnd | It specifies the destination MAC address end. If this attribute is present, the destMacAddr attribute specifies the destination MAC address start. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| packFiltId | It is the identifier of the packet filter. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| packetFilterUsage | It indicates if the packet shall be sent to the UE. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| tosTrafficClass | It contains the Ipv4 Type-of-Service and mask field or the Ipv6 Traffic-Class field and mask field. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| spi | It is the security parameter index of the IPSec packet, see IETF RFC 4301 [66]. allowedValues: see IETF RFC 4301 [66]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flowLabel | It specifies the Ipv6 flow label header field. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flowDirection | It indicates the direction/directions that a filter is applicable. allowedValues: “DOWNLINK”, “UPLINK”, “BIDIRECTIONAL”, “UNSPECIFIED”. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qosId | It identifies the QoS control policy data for a PCC rule. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxbrUl | It represents the maximum uplink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps" allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxbrDl | It represents the maximum downlink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gbrUl | It represents the guaranteed uplink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gbrDl | It represents the guaranteed downlink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| extMaxDataBurstVol | It denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB, see TS 29.512 [60]. allowedValues: 4096..2000000. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arp | It indicates the allocation and retention priority. allowedValues: N/A. | type: ARP multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ARP.priorityLevel | It defines the relative importance of a resource request. allowedValues: 1..15. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| preemptCap | It defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level. allowedValues: "NOT_PREEMPT", "MAY_PREEMPT". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| preemptVuln | It defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level. allowedValues: "NOT_PREEMPTABLE", "PREEMPTABLE". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qosNotificationControl | It indicates whether notifications are requested from 3GPP NG-RAN when the GFBR can no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. allowedValues: "TRUE", "FALSE". | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| reflectiveQos | Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow. allowedValues: "TRUE", "FALSE". | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| sharingKeyDl | It indicates, by containing the same value, what PCC rules may share resource in downlink direction. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sharingKeyUl | It indicates, by containing the same value, what PCC rules may share resource in uplink direction. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxPacketLossRateDl | It indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow. allowedValues: 0..1000. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxPacketLossRateUl | It indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow. allowedValues: 0..1000. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tcId | It univocally identifies the traffic control policy data within a PDU session. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flowStatus | It represents whether the service data flow(s) are enabled or disabled. See TS 29.514 [67]. allowedValues: “ENABLED-UPLINK”, “ENABLED-DOWNLINK”, “ENABLED”, “DISABLED”, “REMOVED”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “ENABLED” isNullable: False |
| redirectInfo | It indicates whether the detected application traffic should be redirected to another controlled address. allowedValues: N/A. | type: RedirectInformation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| addRedirectInfo | It contains the additional redirect information indicating whether the detected application traffic should be redirected to another controlled address. allowedValues: N/A. | type: RedirectInformation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| redirectEnabled | It indicates whether the redirect instruction is enabled. allowedValues: "TRUE", "FALSE". | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| redirectAddressType | It indicates the type of redirect address, see TS 29.512 [60]. allowedValues: " IPV4_ADDR", "IPV6_ADDR", “URL”, “SIP_URI”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| redirectServerAddress | It indicates the address of the redirect server. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| muteNotif | It indicates whether applicat'on's start or stop notification is to be muted. allowedValues: "TRUE", "FALSE". | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| trafficSteeringPolIdDl | It references to a pre-configured traffic steering policy for downlink traffic at the SMF, see TS 29.512 [60]. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trafficSteeringPolIdUl | It references to a pre-configured traffic steering policy for uplink traffic at the SMF, see TS 29.512 [60]. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| routeToLocs | It provides a list of location which the traffic shall be routed to for the AF request. allowedValues: N/A. | type: RouteToLocation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| traffCorreInd | It indicates the traffic correlation. allowedValues: "TRUE", "FALSE". | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| dnai | It represents the DNAI (Data network access identifier), see 3GPP TS 23.501 [2]. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| routeInfo | It provides the traffic routing information. allowedValues: N/A. | type: RouteInformation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ipv4Addr | It defines the Ipv4 address of the tunnel end point in the data network, formatted in the "dotted decimal" notation. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$'. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ipv6Addr | It defines the Ipv6 address of the tunnel end point in the data network. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ipv6AddrPrefix | String identifying an IPv6 address prefix formatted according to clause 4 of IETF RFC 5952 [82]. IPv6Prefix data type may contain an individual /128 IPv6 address. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])¦([0-9]{2})¦(1[0-1][0-9])¦(12[0-8])))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$' | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| portNumber | It defines the UDP port number of the tunnel end point in the data network, see TS 29.571 [61]. allowedValues: N/A. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| routeProfId | It identifies the routing profile. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| upPathChgEvent | It contains the information about the AF subscriptions of the UP path change. allowedValues: N/A. | type: UpPathChgEvent multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationUri | It provides notification address (Uri) of AF receiving the event notification. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notifCorreId | It is used to set the value of Notification Correlation ID in the notification sent by the SMF, see TS 29.512 [60]. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dnaiChgType | It indicates the type of DNAI change, see TS 29.512 [60]. allowedValues: “EARLY”, “EARLY_LATE”, “LATE”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| afAckInd | It identifies whether the AF acknowledgement of UP path event notification is expected. allowedValues: “TRUE”, “FALSE”. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| steerFun | It indicates the applicable traffic steering functionality, see TS 29.512 [60]. allowedValues: “MPTCP”, “ATSSS_LL”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| steerModeDl | It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for downlink traffic. allowedValues: N/A. | type: SteeringMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| steerModeUl | It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for uplink traffic. allowedValues: N/A. | type: SteeringMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mulAccCtrl | It indicates whether the service data flow, corresponding to the service data flow template, is allowed or not allowed. allowedValues: "ALLOWED", "NOT_ALLOWED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NOT_ALLOWED" isNullable: False |
| steerModeValue | It indicates the value of the steering mode, see TS 29.512 [60]. allowedValues: “ACTIVE_STANDBY”, “LOAD_BALANCING”, “SMALLEST_DELAY”, “PRIORITY_BASED”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| active | It indicates the active access, see TS 29.571 [61]. allowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| standby | It indicates the Standby access, see TS 29.571 [61]. allowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threeGLoad | It indicates the traffic load to steer to the 3GPP Access expressed in one percent. allowedValues: 0..100. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| prioAcc | It indicates the high priority access, see TS 29.571 [61]. allowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condId | It uniquely identifies the condition data. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| activationTime | It indicates the time (in date-time format) when the decision data shall be activated, see TS 29.512 [60] and TS 29.571 [61]. allowedValues: N/A. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| deactivationTime | It indicates the time (in date-time format) when the decision data shall be deactivated, see TS 29.512 [60] and TS 29.571 [61]. allowedValues: N/A. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| accessType | It provides the condition of access type of the UE when the session AMBR shall be enforced, see TS 29.512 [60]. If this attribute is included in SmfInfo, it shall contain the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF. If not included, it shall be assumed the both access types are supported. allowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". | type: ENUM multiplicity: 1..2 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ratType | It provides the condition of RAT type of the UE when the session AMBR shall be enforced, see TS 29.512 [60] and TS 29.571 [61]. allowedValues: "NR", "EUTRA", “WLAN”, “VIRTUAL”, “NBIOT”, “WIRELINE”, “WIRELINE_CABLE”, “WIRELINE_BBF”, “LTE-M”, “NR_U”, “EUTRA_U”, “TRUSTED_N3GA”, “TRUSTED_WLAN”, “UTRA”, “GERA”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| periodicity | It identifies the time period between the start of two bursts in reference to the TSN GM. allowedValues: see TS 29.571 [61]. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| burstArrivalTime | Indicates the arrival time (in date-time format) of the data burst in reference to the TSN GM. allowedValues: see TS 29.571 [61]. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nsacfInfoSnssaiList | It represents a list of NSACF information per S-NSSAI. allowedValues: N/A | type: NsacfInfoSnssai multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| snssaiInfo | It defines generic information for a S-NSSAI. The information includes global unique identifier of a Network Slice (see [2] for definition of Network Slice) and adminstrativeState of the Network Slice allowedValues: N/A. | type: SnssaiInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSubjectToNsac | It defines if the Network Slice subjects to network slice admission control. The value is set to False if the maxNumberofUEs attribute in corresponding SliceProfile is absent. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| NsacfInfoSnssai.maxNumberofUEs | It defines the maximum number of UEs which are allowed to be served by the Network Slice that is subject to network slice admission control. This number could be derived from maxNumberofUEs defined in corresponding SliceProfile. allowedValues: 0 - 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| eACMode | It represents if early admission control (EAC) mode is activated. allowedValues: ACTIVE, INACTIVE | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: INACTIVE isNullable: False |
| activeEacThreshold | It defines threshold in percentage value of the number of the UEs registered with the network slice to the maximum number of UEs allowed to register with the network slice. The eACMode is set to active when the number of the UEs registered with the network slice is above this threshold. allowedValues: 0 - 100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| deactiveEacThreshold | It defines threshold in percentage value of the number of the UEs registered with the network slice to the maximum number of UEs allowed to register with the network slice. The eACMode is set to inactive when the number of the UEs registered with the network slice is below this threshold. allowedValues: 0 - 100 Note: If this attribute is absent, activeEacThreshhold is used to trigger deactivation of eACMode. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| numberofUEs | It represents the number of the UEs registered with the network slice. This attribute is updated by NSACF. allowedValues: 0 - 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uEIdList | It represents the UEs registered with the network slice. This attribute is updated by NSACF. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkSliceInfoList | The attribute specifies a list of NetworkSliceInfo which is defined as a datatype (see clause 5.3.95). It is used by an authorized consumer, e.g. NWDAF, to facilitate the data collection from OAM. allowedValues: N/A | type: NetworkSliceInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkSliceRef | This holds a DN of the NetworkSlice managed object relating to the NetworkSlice instance differentiated by sNSSAI and optional cNSIId. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sNSSAI | It represents the S-NSSAI the NetworkSlice managed object is supporting. The S-NSSAI is defined in TS 23.003 [13]. allowedValues: See TS 23.003 [13] | type: S-NSSAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cNSIId | It represents NSI ID which is an identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. See NSI ID definition in clause 3.1 of TS 23.501 [2] and subclause 6.1.6.2.7 of TS 29.531 [24]. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eCSAddrConfigInfo | It represents one or more FQDN(s) and/or IP address(es) of Edge Configuration Server(s), and of an ECS Provider ID. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aMFSet.aMFRegionRef | This is the DN of AMFRegion instance of the AMFSet. This holds a DN of AMFRegion instance for which the AMFSet instance belongs to. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aMFSetRef | This is the DN of AMFSet. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aMFSetListRef | This holds a list of DN of AMFSet instances in the same AMFRegion instance. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serverAddr | This attribute indicates the DNS server address for the PDU Session (see clause 6.2.2.2 in TS 23.548 [78]) allowedValues: Not applicable. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NsacfInfoSnssai.maxNumberofPDUSessions | It defines the maximum number of concurrent PDU sessions supported by the network slic. This number could be derived from maxNumberofPDUSessions defined in corresponding SliceProfile. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASServiceArea | This parameter defines the EAS service area (see clause 7.3.3.6 in TS 23.558 [81]). allowedValues: N/A | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESServiceArea | This parameter defines the EES service area (see clause 7.3.3.5 in TS 23.558 [81]). allowedValues: N/A | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eDNServiceArea | This parameter defines the EDN service area (see clause 7.3.3.4 in TS 23.558 [81]). allowedValues: N/A | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| 5GCNfConnEcmInfoList | The attribute specifies a list of 5GCNfConnInfo which is defined as a datatype (see clause 5.3.120). It is used to provide 5GC NFs, such as PCF, NEF, SCEF, that are connected EDN NFs, such as EAS, EES, and ECS. allowedValues: N/A | type: 5GCNfConnEcmInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| 5GCNFType | It indicates the type of a NF instance. allowedValues:"PCF", "NEF", "SCEF". | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| 5GCNFIpAddress | This parameter defines address of a NF instance, It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN (See TS 23.003 [13]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| 5GCNFRef | This attribute holds the DN of a NF instance. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ednIdentifier | The identifier of the edge data network (See TS 23.558 [81]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASIpAddress | This parameter defines address of an EAS instance. It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESIpAddress | This parameter defines address of an EES instance. It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eCSIpAddress | This parameter defines address of an ECS instance. It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uPFConnectionInfo | The attribute is defined as a datatype UPFConnInfo (see clause 5.3.121). It is used to provide the UPF IP address and UPF DN. allowedValues: N/A | type: UPFConnInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uPFRef | This attribute holds the DN of an UPF instance. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uPFIpAddress | This parameter defines address of an UPF instance, It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN (See TS 23.003 [13]). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ecmConnectionType | It indicates the type of ECM connection (i.e., user plane connection via UPF, control plane connection via PCF or NEF. allowedValues: "USERPLANE", "CONTROLPLANE", "BOTH". | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nwdafEvents | This attribute represents the Analytic functionalities (identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF instance. MnS consumer can configure this attribute to specify which Analytic functionalities (identified by nwdafEvent) can be performed the NWDAF instance. If the value of this attribute is not present, the NWDAF instance can perform any NWDAFEvents allowedValues: the detailed ENUM value for NwdafEvent see the Table 5.1.6.3.4-1 in TS 29.520[85]. | type: NwdafEvent multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| administrativeState | This attribute determines whether the NWDAF is enabled or disabled. MnS consumer can configure this attribute to activate or de-activate the analytic functionalities (identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF instance. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PCFFunction.groupId | It indicates the identity of the PCF group that is served by the PCF instance. If not provided, the PCF instance does not pertain to any PCF group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dnnList | It represents the DNNs supported by the PCF. The DNN, as defined in clause 9A of TS 23.003 [13], shall contain the Network Identifier and it may additionally contain an Operator Identifier, as specified in TS 23.003 [13] clause 9.1.1 and 9.1.2. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. If not provided, the PCF can serve any DNN. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supiRanges | It represents list of ranges of SUPIs that can be served by the PCF instance. allowedValues: N/A | type: SupiRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| PcfInfo.gpsiRanges | It represents list of ranges of GPSIs that can be served by the PCF instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SupiRange.start | It indicates the first value identifying the start of a SUPI range, to be used when the range of SUPI's can be represented as a numeric range (e.g., IMSI ranges). This string shall consist only of digits. Pattern: "^[0-9]+$" allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SupiRange.end | It indicates the last value identifying the end of a SUPI range, to be used when the range of SUPI's can be represented as a numeric range (e.g. IMSI ranges). This string shall consist only of digits. Pattern: "^[0-9]+$" allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SupiRange.pattern | It indicates the pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of SUPI's belonging to this range. A SUPI value is considered part of the range if and only if the SUPI string fully matches the regular expression. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| IdentityRange.start | It indicates the first value identifying the start of an identity range, to be used when the range of identities can be represented as a numeric range (e.g., MSISDN ranges). This string shall consist only of digits. Pattern: "^[0-9]+$" allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| IdentityRange.end | It indicates the last value identifying the end of an identity range, to be used when the range of identities can be represented as a numeric range (e.g. MSISDN ranges). This string shall consist only of digits. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| IdentityRange.pattern | It indicates the pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of identities belonging to this range. An identity value is considered part of the range if and only if the identity string fully matches the regular expression. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rxDiamHost | It indicates the Diameter host of the Rx interface for the PCF. See TS 29.571 [61]. String contains a Diameter Identity (FQDN). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rxDiamRealm | It indicates the Diameter realm of the Rx interface for the PCF. See TS 29.571 [61]. String contains a Diameter Identity (FQDN). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| v2xSupportInd | It indicates whether V2X Policy/Parameter provisioning is supported by the PCF. TRUE: Supported FALSE: Not Supported allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseSupportInd | It indicates whether ProSe capability is supported by the PCF. TRUE: Supported FALSE: Not Supported allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseCapability | It indicates the supported ProSe Capability by the PCF. | type: ProSeCapability multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| v2xCapability | It indicates the supported V2X Capability by the PCF. | type: V2xCapability multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| proseDirectDiscovery | It indicates whether the PCF supports ProSe Direct Discovery: - TRUE: ProSe Direct Discovery is supported by the PCF - FALSE: ProSe Direct Discovery is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseDirectCommunication | It indicates whether the PCF supports ProSe Direct Communication: - TRUE: ProSe Direct Communication is supported by the PCF - FALSE: ProSe Direct Communication is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseL2UetoNetworkRelay | It indicates whether the PCF supports ProSe Layer-2 UE-to-Network Relay: - TRUE: ProSe Layer-2 UE-to-Network Relay is supported by the PCF - FALSE: ProSe Layer-2 UE-to-Network Relay is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseL3UetoNetworkRelay | It indicates whether the PCF supports ProSe Layer-3 UE-to-Network Relay: - TRUE: ProSe Layer-3 UE-to-Network Relay is supported by the PCF - FALSE: ProSe Layer-3 UE-to-Network Relay is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseL2RemoteUe | It indicates whether the PCF supports ProSe Layer-2 Remote UE: - TRUE: ProSe Layer-2 Remote UE is supported by the PCF - FALSE: ProSe Layer-2 Remote UE is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| proseL3RemoteUe | It indicates whether the PCF supports ProSe Layer-3 Remote UE: - TRUE: ProSe Layer-3 Remote UE is supported by the PCF - FALSE: ProSe Layer-3 Remote UE is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| V2xCapability.lteV2x | It indicates whether the PCF supports LTE V2X capability: - TRUE: LTE V2X capability is supported by the PCF - FALSE (default): LTE V2X capability is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| V2xCapability.nrV2x | It indicates whether the PCF supports NR V2X capability: - TRUE: NR V2X capability is supported by the PCF - FALSE (default): NR V2X capability is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| UDMFunction.groupId | It indicates the identity of the UDM group that is served by the UDM instance. If not provided, the UDM instance does not pertain to any UDM group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| supiRanges | It represents list of ranges of SUPIs whose profile data is available in the UDM instance. allowedValues: N/A | type: SupiRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UdmInfo.gpsiRanges | It represents list of ranges of GPSIs whose profile data is available in the UDM instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UdmInfo.externalGroupIdentifiersRanges | It represents list of ranges of external groups whose profile data is available in the UDM instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| routingIndicators | It represents list of Routing Indicator information that allows to route network signalling with SUCI (see TS 23.003 [13]) to the UDM instance. If not provided, the UDM can serve any Routing Indicator. Pattern: '^[0-9]{1,4}$' allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UdmInfo.internalGroupIdentifiersRanges | It represents list of ranges of Internal Group Identifiers whose profile data is available in the UDM instance. If not provided, it does not imply that the UDM supports all internal groups. allowedValues: N/A | type: InternalGroupIdRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| InternalGroupIdRange.start | It indicates first value identifying the start of an identity range, to be used when the range of identities can be represented as a consecutive numeric range. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| InternalGroupIdRange.end | It indicates last value identifying the end of an identity range, to be used when the range of identities can be represented as a consecutive numeric range. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| InternalGroupIdRange.pattern | It indicates pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of identities belonging to this range. An identity value is considered part of the range if and only if the identity string fully matches the regular expression. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| suciInfos | It represents list of SuciInfo. A SUCI that matches this information can be served by the UDM . A SUCI that matches all attributes of at least one entry in this array shall be considered as a match of this information. allowedValues: N/A | type: SuciInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| routingInds | It indicates served Routing Indicator (see TS 23.003 [13], clause 2.2B). If not provided, the AUSF/UDM can serve any Routing Indicator. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| hNwPubKeyIds | It indicating served Home Network Public Key (see TS 23.003 [13], clause 2.2B). If not provided, the AUSF/UDM can serve any public key. allowedValues: N/A | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UDRFunction.groupId | It indicates the identity of the UDR group that is served by the UDR instance. If not provided, the UDR instance does not pertain to any UDR group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| supiRanges | It represents list of ranges of SUPI's whose profile data is available in the UDR instance. allowedValues: N/A | type: SupiRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UdrInfo.gpsiRanges | It represents list of ranges of GPSIs whose profile data is available in the UDR instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| externalGroupIdentifiersRanges | It represents list of ranges of external groups whose profile data is available in the UDR instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sharedDataIdRanges | It represents list of ranges of Shared Data IDs that identify shared data available in the UDR instance. allowedValues: N/A | type: SharedDataIdRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SharedDataIdRange.pattern | It indicates the pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of SharedDataIds belonging to this range. A SharedDataId value is considered part of the range if and only if the SharedDataId string fully matches the regular expression. EXAMPLE: sharedDataId range. "123456-sharedAmData{localID}" where "123456" is the HPLMN id (i.e. MCC followed by MNC) and "{localID}" can be any string. JSON: { "pattern": "^123456-sharedAmData.+$" } allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| udsfInfo | This attribute represents information related to UDSF, as described in clause 6.1.6.2.63 of TS 29.510 [23]. allowedValues: N/A | type: UdsFInfo multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| UdsfInfo.groupId | This attribute represents the identity of the UDSF group that is served by the UDSF instance. If not provided, the UDSF instance does not pertain to any UDSF group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| UdsfInfo.supiRanges | This attribute represents a list of ranges of SUPIs whose profile data is available in the UDSF instance If not provided, then the UDSF can serve any SUPI range. allowedValues: N/A | type: SupiRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UdsfInfo.storageIdRanges | It represents a map (list of key-value pairs) where realmId serves as key and each value in the map is an array of IdentityRanges. Each IdentityRange is a range of storageIds. A UDSF complying with this version of the specification shall include this IE. Absence indicates that the UDSF's supported realms and storages are determined by the UDSF's consumer by other means such as local provisioning. allowedValues: N/A | type: IdentityRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| seppInfo | This attributes represents information of a SEPP Instance, as described in clause 6.1.6.2.72 of TS 29.510 [23]. allowedValues: N/A | type: SeppInfo multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| seppPrefix | This attributes represents optional deployment specific string used to construct the apiRoot of the next hop SEPP, as described in clause 6.10 of TS 29.500 [76]. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| seppPorts | This attributes represents SEPP port number(s) for HTTP and/or HTTPS. This attribute shall be present if the SEPP uses non-default HTTP and/or HTTPS ports. When present, it shall contain the HTTP and/or HTTPS ports. The key of the map shall be "http" or "https". The value shall indicate the port number for HTTP or HTTPS respectively. Minimum: 0 Maximum: 65535 allowedValues: N/A | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| remotePlmnList | It represents a list of remote PLMNs reachable through the SEPP. The absence of this attribute indicates that any PLMN is reachable through the SEPP. allowedValues: N/A | type: PlmnId multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| remoteSnpnList | This attributes represents list of remote SNPNs reachable through the SEPP. The absence of this attribute indicates that no SNPN is reachable through the SEPP. allowedValues: N/A | type: PlmnIdNid multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| scpDomainInfoList | This attributes represents SCP domain specific information of the SCP that differs from the common information in NFProfile data type. The key of the map shall be the string identifying an SCP domain. allowedValues: N/A | type: ScpDomainInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| scpPrefix | Optional deployment specific string used to construct the apiRoot of the next hop SCP, as described in clause 6.10 of TS 29.500 [76]. allowedValues: N/A | type: String multiplicity: 0..1 Ordered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scpPorts | This attributes represents SCP port number(s) for HTTP and/or HTTPS. This attribute shall be present if the SCP uses non-default HTTP and/or HTTPS ports and if the SCP does not provision port information within ScpDomainInfo for each SCP domain it belongs to. allowedValues: 0 - 65535 | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| addressDomains | Pattern (regular expression according to the ECMA-262 dialect [75]) representing the address domain names reachable through the SCP. Absence of this IE indicates the SCP can reach any address domain names in the SCP domain(s) it belongs to. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ScpInfo.ipv4Addresses | This attributes represents list of IPv4 addresses reachable through the SCP. This IE may be present if IPv4 addresses are reachable via the SCP. If IPv4 addresses are reachable via the SCP, absence of both this IE and ipv4AddrRanges IE indicates the SCP can reach any IPv4 addresses in the SCP domain(s) it belongs to. | type: Ipv4Addr multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ScpInfo.ipv6Prefixes | List of IPv6 prefixes reachable through the SCP. This IE may be present if IPv6 addresses are reachable via the SCP. If IPv6 addresses are reachable via the SCP, absence of both this IE and ipv6PrefixRanges IE indicates the SCP can reach any IPv6 prefixes in the SCP domain(s) it belongs to. | type: Ipv6Addr multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ScpInfo.ipv4AddrRanges | List of IPv4 addresses ranges reachable through the SCP. This IE may be present if IPv4 addresses are reachable via the SCP. If IPv4 addresses are reachable via the SCP, absence of both this IE and ipv4Addresses IE indicates the SCP can reach any IPv4 addresses in the SCP domain(s) it belongs to. | type: Ipv4AddressRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ScpInfo.ipv6PrefixRanges | List of IPv6 prefixes ranges reachable through the SCP. This IE may be present if IPv6 addresses are reachable via the SCP. If IPv6 addresses are reachable via the SCP, absence of both this IE and ipv6Prefixes IE indicates the SCP can reach any IPv6 prefixes in the SCP domain(s) it belongs to. | type: Ipv6PrefixRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedNfSetIdList | List of NF set ID of NFs served by the SCP. Absence of this IE indicates the SCP can reach any NF set in the SCP domain(s) it belongs to. NF Set Identifier (see clause 28.12 of TS 23.003 [13]), formatted as the following string: "set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>", or "set<SetID>.<NFType>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with <MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition) <MNC> encoding the Mobile Network Code part of the PLMN, comprising 3 digits. If there are only 2 significant digits in the MNC, one "0" digit shall be inserted at the left side to fill the 3 digits coding of MNC. Pattern: '^[0-9]{3}$' <NFType> encoded as a value defined in Table 6.1.6.3.3-1 of 3GPP TS 29.510 [23] but with lower case characters <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| remotePlmnList | List of remote PLMNs reachable through the SCP. Absence of this IE indicates that no remote PLMN is reachable through the SCP. allowedValues: N/A | type: PlmnId multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| remoteSnpnList | This attribute represents the List of remote PLMNs reachable through the SCP. Absence of this IE indicates that no remote PLMN is reachable through the SCP. allowedValues: N/A | type: PlmnIdNid multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ipReachability | This attribute indicates the type(s) of IP addresses reachable via the SCP in the SCP domain(s) it belongs to. Absence of this IE indicates that the SCP can be used to reach both IPv4 addresses and IPv6 addresses in the SCP domain(s) it belongs to. allowedValues: "IPV4": Only IPv4 addresses are reachable. "IPV6": Only IPv6 addresses are reachable. "IPV4V6": Both IPv4 addresses and IPv6 addresses are reachable. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scpCapabilities | List of SCP capabilities supported by the SCP. This IE shall be present if the SCP supports at least one SCP capability. It may be present otherwise, with an empty array, to indicate that the SCP does not support any capability of the ScpCapability data type. The absence of this attribute shall not be interpreted as an SCP that does not support any capability; this only means that the SCP (e.g. pre-Rel-17 SCP) did not register the capabilities it may support. allowedValues: "INDIRECT_COM_WITH_DELEG_DISC", which indicating Indirect communication with delegated discovery supported | type: ENUM multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| PlmnIdNid.nid | This attribute represents network Identity; Shall be present if PlmnIdNid identifies an SNPN. (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in TS 23.501 [2]). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nwdafInfo | It represents specific data for the NWDAF. allowedValues: N/A | type: NwdafInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eventIds | It represents the EventId(s) supported by the Nnwdaf_AnalyticsInfo service, if none are provided the NWDAF can serve any eventId. (see clause TS 29.520) allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nwdafCapability | This attribute indicates the capability of the NWDAF. If not present, the NWDAF shall be regarded with no capability. allowedValues: N/A | type: NwdafCapability multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| analyticsDelay | It represents the supported Analytics Delay related to the eventIds and nwdafEvents. It is an unsigned integer identifying a period of time in units of seconds.(see clause 5.2.2 TS 29.571 [61]). allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NwdafInfo.servingNfTypeList | It contains the list of NF type(s) from which the NWDAF NF can collect data. The absence of this attribute indicates that the NWDAF can collect data from any NF type. allowedValues: N/A | type: NFType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NwdafInfo.servingNfSetIdList | It contains the list of NF type(s) from which the NWDAF NF can collect data. The absence of this attribute indicates that the NWDAF can collect data from any NF type. (see clause 5.4.2 NfSetId in TS 29.571 [61]) allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NwdafInfo.taiList | This attribute represents a List of TAIs the NWDAF can serve. It may contain one or more non-3GPP access TAIs. The absence of both this attribute and the taiRangeList attribute indicates that the NWDAF can be selected for any TAI in the serving network. allowedValues: N/A | type: Tai multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NwdafInfo.taiRangeList | This attribute represents the range of TAIs the NWDAF can serve. It may contain one or more non-3GPP access TAI ranges. The absence of both this attribute and the taiList attribute indicates that the NWDAF can be selected for any TAI in the serving network. allowedValues: N/A | type: TaiRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlAnalyticsList | It represents ML Analytics Filter information supported by the Nnwdaf_MLModelProvision service. allowedValues: N/A | type: MlAnalyticsInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| analyticsAggregation | It indicates whether the NWDAF supports analytics aggregation: - TRUE: analytics aggregation capability is supported by the NWDAF - FALSE: analytics aggregation capability is not supported by the NWDAF. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| analyticsMetadataProvisioning | It indicate whether the NWDAF supports analytics metadata provisioning: - TRUE: analytics metadata provisioning capability is supported by the NWDAF - FALSE: analytics metadata provisioning capability is not supported by the NWDAF. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mlAnalyticsIds | This attribute represents the Analytic functionalities (identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF instance. MnS consumer can configure this attribute to specify which Analytic functionalities (identified by nwdafEvent) can be performed the NWDAF instance. If the value of this attribute is not present, the NWDAF instance can perform any NWDAFEvents Analytics Id(s) supported by the Nnwdaf_MLModelProvision service, if none are provided the NWDAF can serve any mlAnalyticsId. allowedValues: the detailed ENUM value for NwdafEvent see the Table 5.1.6.3.4-1 in TS 29.520 [85]. | type: NwdafEvent multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| trackingAreaList | This attribute represents area of Interest of the ML model, if none are provided the ML model for the analytics can apply to any TAIs. If present, it represents the list of TAIs, it may contain one or more non-3GPP access TAIs. allowedValues: N/A | type: Tai multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nsacfInfo | This attribute represents the information of an NSACF NF Instance. (see TS 29.510 [23]). allowedValues: N/A | type: NsacfInfo multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| nsacfCapability | It represents NSACF service capability. allowedValues: N/A | type: NsacfCapability multiplicity: 0..1 isOrdered: N/A isUnique: NA defaultValue: None isNullable: False |
| NSACFFunction.taiList | This attribute represents the list of TAIs the NSACF can serve. It may contain one or more non-3GPP access TAIs. The absence of this attribute and the taiRangeList attribute indicate that the NSACF can be selected for any TAI in the serving network. allowedValues: N/A | type: Tai multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NSACFFunction.taiRangeList | This attribute represents the range of TAIs the NSACF can serve. It may contain non-3GPP access TAIs. The absence of this attribute and the taiList attribute indicate that the NSACF can be selected for any TAI in the serving network. allowedValues: N/A | type: TaiRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportUeSAC | This attribute indicates the service capability of the NSACF to monitor and control the number of registered UEs per network slice for the network slice that is subject to NSAC. allowedValues: TRUE: Supported FALSE: Not Supported | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| supportPduSAC | This attribute indicates the service capability of the NSACF to monitor and control the number of established PDU sessions per network slice for the network slice that is subject to NSAC. allowedValues: TRUE: Supported FALSE: Not Supported | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| nefId | It represents the NEF ID. (see clause 6.1.6.3.2 of TS 29.510 [23]) allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| appIds | It represents list of internal application identifiers of the managed PFDs. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| afIds | It represents list of application function identifiers of the managed PFDs. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pfdData | It represents PFD data, containing the list of internal application identifiers and/or the list of application function identifiers for which the PFDs can be provided. Absence of this attribute indicates that the PFDs for any internal application identifier and for any application function identifier can be provided. allowedValues: N/A | type: PfdData multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AfEventExposureData.afEvents | It represents AF Event(s) exposed by the NEF after registration of the AF(s) at the NEF. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| afEeData | It represents the AF provided event exposure data. The NEF registers such information in the NRF on behalf of the AF. allowedValues: N/A | type: AfEventExposureData multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servedFqdnList | It represents pattern (regular expression according to the ECMA-262 dialect [75]) representing the Domain names served by the NEF. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dnaiList | It represents list of Data network access identifiers supported by the NEF. The absence of this attribute indicates that the NEF can be selected for any DNAI. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| unTrustAfInfoList | It represents list of information corresponding to the AFs. allowedValues: N/A | type: UnTrustAfInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UnTrustAfInfo.afId | It represents associated AF id. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| UnTrustAfInfo. sNssaiInfoList | It represents S-NSSAIs and DNNs supported by the untrust AF. allowedValues: N/A | type: SnssaiInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| UnTrustAfInfo. mappingInd | When present, this attribute indicates whether the AF supports mapping between UE IP address (IPv4 address or IPv6 prefix) and UE ID (i.e. GPSI). allowedValues: True, False True: the AF supports mapping between UE IP address and UE ID; False: the AF does not support mapping between UE IP address and UE ID. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| SnssaiInfoItem.sNssai | It represents supported S-NSSAI. allowedValues: N/A | type: ExtSnssai multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SnssaiInfoItem.dnnInfoList | It represents list of parameters supported by the NF per DNN. allowedValues: N/A | type: DnnInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| snssaiExtension | It represents extensions to the Snssai. allowedValues: N/A | type: SnssaiExtension multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SnssaiExtension.sdRanges | It shall contain the range(s) of Slice Differentiator values supported for the Slice/Service Type value indicated in the sst attribute of the Snssai data type (see clause 5.4.4.2 in TS 29.571[61). | type: SdRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SnssaiExtension.wildcardSd | It indicates that all SD values are supported for the Slice/Service Type value indicated in the sst attribute of the Snssai data type (see clause 5.4.4.2 in TS 29.571[61]). allowedValues: True, False | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| SdRange.start | First value identifying the start of an SD range. This string shall be formatted as specified for the sd attribute of the Snssai data type in clause 5.4.4.2 of TS 29.571 [61]. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SdRange.end | Last value identifying the end of an SD range. This string shall be formatted as specified for the sd attribute of the Snssai data type in clause 5.4.4.2 in TS 29.571 [61]. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DnnInfoItem.dnn | It represents supported DNN or Wildcard DNN if the NF supports all DNNs for the related S-NSSAI. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uasNfFunctionalityInd | When present, this attribute shall indicate whether the NEF supports UAS NF functionality: allowedValues: True, False - True: UAS NF functionality is supported by the NEF. - False: UAS NF functionality is not supported by the NEF. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| ausfInfo | It represents the information of an AUSF NF Instance (see TS 29.510 [23]). allowedValues: N/A | type: AusfInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AUSFFunction.supiRanges | This attribute represents a list of ranges of SUPIs that can be served by the AUSF instance. (NOTE 1) allowedValues: N/A | type: SupiRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| AUSFFunction.routingIndicators | This attribute represents a list of Routing Indicator information that allows to route network signalling with SUCI (see TS 23.003 [13]) to the AUSF instance. If not provided, the AUSF can serve any Routing Indicator. Pattern: '^[0-9]{1,4}$' allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| AUSFFunction.suciInfos | This attribute represents a list of SuciInfo. A SUCI that matches this information can be served by the AUSF. (NOTE 2, NOTE 3) A SUCI that matches all attributes of at least one entry in this array shall be considered as a match of this information. allowedValues: N/A | type: SuciInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| smsfInfo | This attribute represents specific data for a SMSF. allowedValues: N/A | type: SmsfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| roamingUeInd | This attribute indicates whether the SMSF can serve roaming UE: - TRUE: the SMSF can support roaming UEs. - FALSE: the SMSF can not support roaming UEs. Absence of this IE indicates whether the SMSF can serve roaming UEs is not specified. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remotePlmnRangeList | This attribute indicates the list of ranges of remote PLMNs served by the SMSF, i.e. the SMSF can serve the roaming UEs which belong to the indicated remote PLMNs. If the roamingUeInd attribute is present with the value "true", absence of remotePlmnRangeList indicates that the SMSF can serve roaming UEs from any remote PLMN. allowedValues: N/A | type: PlmnRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| PlmnRange.start | This attribute indicates the first value identifying the start of a PLMN range. The string shall be encoded as follows: <MCC><MNC> Pattern: '^[0-9]{3}[0-9]{2,3}$' allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PlmnRange.end | This attribute indicates the last value identifying the end of a PLMN range. The string shall be encoded as follows: <MCC><MNC> Pattern: '^[0-9]{3}[0-9]{2,3}$' allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PlmnRange.pattern | This attribute indicates pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of PLMNs belonging to this range. A PLMN value is considered part of the range if and only if the PLMN string (formatted as <MCC><MNC>) fully matches the regular expression. To be noted, either the start and end attributes, or the pattern attribute, shall be present. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| udrInfo | This attribute represents the information of an UDR NF Instance (see TS 29.510 [23]). allowedValues: N/A | type: UdrInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| udmInfo | This attribute represents the information of an UDM NF Instance (see TS 29.510 [23]). allowedValues: N/A | type: UdmInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lmfInfo | This attribute represents information of an LMF NF Instance allowedValues: N/A | type: LmfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servingClientTypes | This attribute represents a list of external client type(s), e.g. emergency client. The NRF should only include this LMF instance to NF discovery with "client-type" query parameter indicating one of the external client types in the list. Absence of this attribute means the LMF is not dedicated to serve specific client types. allowedValues: see clause 6.1.6.3.3 of TS 29.572 [86] "EMERGENCY_SERVICES": External client for emergency services "VALUE_ADDED_SERVICES": External client for value added services "PLMN_OPERATOR_SERVICES": External client for PLMN operator services "LAWFUL_INTERCEPT_SERVICES": External client for Lawful Intercept services "PLMN_OPERATOR_BROADCAST_SERVICES": External client for PLMN Operator Broadcast services "PLMN_OPERATOR_OM": External client for PLMN Operator O&M "PLMN_OPERATOR_ANONYMOUS_STATISTICS": External client for PLMN Operator anonymous statistics "PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT": External client for PLMN Operator target MS service support | type: ENUM multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lmfId | This attribute represents the LMF identification. See clause 6.1.6.3.6 TS 29.572 [86] allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servingAccessTypes | This attribute contains the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF. If not included, it shall be assumed the both access types are supported. allowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servingAnNodeTypes | This attribute contains the AN node type (i.e. gNB or NG-eNB) supported by the LMF. If not included, it shall be assumed that all AN node types are supported. allowedValues: "GNB","NG_ENB" | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servingRatTypes | This attribute contains the RAT type (e.g. 5G NR, eLTE or any of the RAT Types specified for NR satellite access) supported by the LMF. If not included, it shall be assumed that all RAT types are supported allowedValues: see clause 5.4.3.2 of TS 29.571 [61]. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| LmfInfo.taiList | This attribute contains TAI list that the LMF can serve. It may contain one or more non-3GPP access TAIs. The absence of both this attribute and the taiRangeList attribute indicates that the LMF can be selected for any TAI in the serving network. allowedValues: N/A | type: TAI multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| LmfInfo.taiRangeList | This attribute contains TAI range list that the LMF can serve. It may contain one or more non-3GPP access TAI ranges. The absence of both this attribute and the taiList attribute indicates that the LMF can be selected for any TAI in the serving network. allowedValues: N/A | type: TAIRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedGADShapes | This attribute contains the GAD shapes supported by the LMF. If not included, it doesn't indicate that the LMF doesn't support any GAD shapes. allowedValues: see clause 6.1.6.3.4 of TS 29.572 [86] "POINT" indicates Ellipsoid Point "POINT_UNCERTAINTY_CIRCLE" indicates Ellipsoid point with uncertainty circle "POINT_UNCERTAINTY_ELLIPSE" indicates Ellipsoid point with uncertainty ellipse "POLYGON" indicates Polygon "POINT_ALTITUDE" indicates Ellipsoid point with altitude "POINT_ALTITUDE_UNCERTAINTY" indicates Ellipsoid point with altitude and uncertainty ellipsoid "ELLIPSOID_ARC" indicates Ellipsoid Arc "LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE" indicates Local 2D point with uncertainty ellipse "LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID" indicates Local 3D point with uncertainty ellipsoid | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SnssaiInfoItem | This attribute represents a list of S-NSSAIs and DNNs supported by the trusted AF. allowedValues: N/A | type: SnssaiInfoItem multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TrustAfInfo.afEvents | This attribute represents list of AF Event(s) supported by the trusted AF. allowedValues: "SVC_EXPERIENCE","UE_MOBILITY", "UE_COMM", "EXCEPTIONS", "USER_DATA_CONGESTION", "PERF_DATA", "COLLECTIVE_BEHAVIOUR", "DISPERSION", "MS_QOE_METRICS", "MS_CONSUMPTION", "MS_NET_ASSIST_INVOCATION", "MS_DYN_POLICY_INVOCATION", "MS_ACCESS_ACTIVITY" See clause 5.6.3.3 TS 29.517 [87]. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TrustAfInfo.appIds | This attribute represents a list of Application ID(s) supported by the trusted AF. The absence of this attribute indicate that the AF can be selected for any Application. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| internalGroupId | This attribute represents a list of Internal Group Identifiers supported by the trusted AF. If not provided, it does not imply that the AF supports all internal groups. String pattern: '^[A-Fa-f0-9]{8}-[0-9]{3}-[0-9]{2,3}-([A-Fa-f0-9][A-Fa-f0-9]){1,10}$'. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mappingInd | This attribute indicates whether the trusted AF supports mapping between UE IP address (IPv4 address or IPv6 prefix) and UE ID (i.e. SUPI). TRUE: the trusted AF supports mapping between UE IP address and UE ID; FALSE: the trusted AF does not support mapping between UE IP address and UE ID. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| sNssaiEasdfInfoList | This attribute represents a list of parameters supported by the EASDF per S-NSSAI. allowedValues: N/A | type: SnssaiEasdfInfoItem multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| easdfN6IpAddressList | This attribute represents N6 IP addresses of the EASDF. allowedValues: N/A | type: IpAddr multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| upfN6IpAddressList | This attribute represents N6 IP addresses of PSA UPFs. allowedValues: N/A | type: IpAddr multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SnssaiEasdfInfoItem.sNssai | This attribute represents a S-NSSAI. allowedValues: N/A | type: SnssaiExtension multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SnssaiEasdfInfoItem.dnnEasdfInfoList | This attribute represents a list of parameters supported by the EASDF per DNN. allowedValues: N/A | type: DnnEasdfInfoItem multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnEasdfInfoItem.dnn | This attribute represents a supported DNN or Wildcard DNN if the EASDF supports all DNNs for the related S-NSSAI. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NssafInfo.supiRanges | This attribute represents a List of ranges of SUPIs that can be served by the NSSAAF instance. allowedValues: N/A | type: SupiRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NssafInfo.internalGroupIdentifiersRanges | This attribute represents a List of ranges of Internal Group Identifiers that can be served by the NSSAAF instance. If not provided, it does not imply that the NSSAAF supports all internal groups. allowedValues: N/A | type: InternalGroupIdRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUdrInfo | This attribute contains all the udrInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the udrInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUdmInfo | This attribute contains all the udmInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the udmInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedAusfInfo | This attribute contains all the ausfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the ausfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedNwdafInfo | This attribute contains all the nwdafInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the nwdafInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedLmfInfo | This attribute contains all the lmfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the lmfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUdsfInfo | This attribute contains all the udsfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedTrustAfInfo | This attribute contains the trustAfInfo attribute locally configured in the NRF or that the NRF received during AF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedNssaafInfo | This attribute contains all the nssaafInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the nssaafInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| chfInfo | It represents the information of an CHF NF Instance (see TS 29.510 [23]). allowedValues: N/A | type: ChfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ChfInfo.supiRangeList | This attribute represents the list of ranges of SUPIs that can be served by the CHF instance. allowedValues: N/A | type: SupiRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ChfInfo.gpsiRangeList | This attribute represents the list of ranges of GPSI that can be served by the CHF instance. allowedValues: N/A | type: IdentityRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ChfInfo.plmnRangeList | This attribute represents the list of ranges of PLMNs (including the PLMN IDs of the CHF instance) that can be served by the CHF instance. If not provided, the CHF can serve any PLMN. allowedValues: N/A | type: PlmnRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ChfInfo.groupId | This attribute represents the identity of the CHF group that is served by the CHF instance. If not provided, the CHF instance does not pertain to any CHF group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ChfInfo.primaryChfInstance | This attribute represents the NF Instance Id of the primary CHF instance. This attribute shall be absent if the secondaryChfInstance is present. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ChfInfo.secondaryChfInstance | This attribute represents the NF Instance Id of the secondary CHF instance. This attribute shall be absent if the primaryChfInstance is present. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mfafInfo | This attribute represents information of an MFAF NF Instance. allowedValues: N/A | type: MfafInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MfafInfo.servingNfTypeList | This attribute represents a List of NF type(s) served by MFAF NF. The absence of this attribute indicates that the MFAF can be selected for any NF type allowedValues: N/A | type: NFType multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MfafInfo.servingNfSetIdList | This attribute represents a List of NF Set Id(s) served by MFAF NF. The absence of this attribute indicates that the MFAF can be selected for any NF Set Id. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MfafInfo.taiList | This attribute represents a List of TAIs the MFAF can serve. It may contain one or more non-3GPP access TAIs. The absence of both this attribute and the taiRangeList attribute indicates that the MFAF can be selected for any TAI in the serving network. allowedValues: N/A | type: Tai multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MfafInfo.taiRangeList | This attribute represents the range of TAIs the MFAF can serve. It may contain one or more non-3GPP access TAI ranges. The absence of both this attribute and the taiList attribute indicates that the MFAF can be selected for any TAI in the serving network. allowedValues: N/A | type: TaiRange multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dccfInfo | This attribute represents information of an DCCF NF Instance allowedValues: N/A | type: DccfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DccfInfo.servingNfTypeList | This attribute represents the list of NF type(s) from which the DCCF NF can collect data. The absence of this attribute indicates that the DCCF can collect data from any NF type. allowedValues: N/A | type: NFType multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DccfInfo.servingNfSetIdList | This attribute represents the list of NF Set Id(s) from which the DCCF NF can collect data. The absence of this attribute indicates that the DCCF can collect data from any NF Set. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DccfInfo.taiList | This attribute represents the list of TAIs the DCCF can serve. It may contain one or more non-3GPP access TAIs. The absence of both this attribute and the taiRangeList attribute indicates that the DCCF can be selected for any TAI in the serving network. allowedValues: N/A | type: TAI multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DccfInfo.taiRangeList | This attribute represents the range of TAIs the DCCF can serve. It may contain one or more non-3GPP access TAI ranges. The absence of both this attribute and the taiList attribute indicates that the DCCF can be selected for any TAI in the serving network. allowedValues: N/A | type: TAIRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| amfInfo | This attribute represents information of an AMF NF Instance. allowedValues: N/A | type: AmfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| smfInfo | This attribute represents information of an SMF NF Instance. allowedValues: N/A | type: SmfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| upfInfo | This attribute represents information of an UPF NF Instance. allowedValues: N/A | type: UpfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pcfInfo | This attribute represents information of a PCF NF Instance. allowedValues: N/A | type: PcfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nefInfo | This attribute represents information of an NEF NF Instance. allowedValues: N/A | type: NefInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servedUdrInfoList | This attribute contains list of UdrInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUdmInfoList | This attribute contains list of UdmInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedAusfInfoList | This attribute contains list of AusfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedAmfInfo | This attribute contains all the amfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the amfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedAmfInfoList | This attribute contains list of AmfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedSmfInfo | This attribute contains all the smfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the smfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedSmfInfoList | This attribute contains list of SmfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUpfInfo | This attribute contains all the upfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the upfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUpfInfoList | This attribute contains list of UpfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedPcfInfo | This attribute contains all the pcfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the pcfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedPcfInfoList | This attribute contains list of PcfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedBsfInfo | This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the bsfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedBsfInfoList | This attribute contains list of BsfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedChfInfo | This attribute contains all the chfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the chfInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedChfInfoList | This attribute contains list of ChfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedNefInfo | This attribute contains all the nefInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the nefInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedNwdafInfoList | This attribute contains list of nwdafInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedGmlcInfo | This attribute contains all the gmlcInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of which the nefInfo belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedUdsfInfoList | This attribute contains list of UdsfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedScpInfoList | This attribute contains list of ScpInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedSeppInfoList | This attribute contains list of SeppInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| AanfInfo.routingIndicators | This attribute represents the List of Routing Indicators supported by the AAnf instance. If not provided, the AAnf can serve any Routing Indicator. Pattern: '^[0-9]{1,4}$' allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aanfInfo | This attribute represents information of an AANF NF Instance. allowedValues: N/A | type: AanfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tsctsfInfo | This attribute represents information of an TSCTSF NF Instance. allowedValues: N/A | type: TsctsfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TsctsfInfo.sNssaiInfoList | This attribute represents the S-NSSAIs and DNNs supported by the TSCTSF. The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [92], with a maximum of 32 characters. allowedValues: N/A | type: SnssaiTsctsfInfoItem multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TsctsfInfo.externalGroupIdentifiersRanges | This attribute represents the ranges of External Group Identifiers that can be served by the TSCTSF. The absence of this IE indicates that the TSCTSF can serve any external group managed by the PLMN (or SNPN) of the TSCTSF instance. allowedValues: N/A | type: IdentityRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TsctsfInfo.supiRanges | This attribute represents the ranges of SUPIs that can be served by the TSCTSF instance. allowedValues: N/A | type: SupiRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TsctsfInfo.gpsiRanges | This attribute represents the ranges of GPSIs that can be served by the TSCTSF instance. allowedValues: N/A | type: IdentityRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TsctsfInfo.internalGroupIdentifiersRanges | This attribute represents the ranges of Internal Group Identifiers that can be served by the TSCTSF instance. The absence of this IE indicates that the TSCTSF can serve any internal group managed by the PLMN (or SNPN) of the TSCTSF instance. allowedValues: N/A | type: InternalGroupIdRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servingClientTypes | This attribute shall be present if the GMLC is dedicated to serve the listed external client type(s), e.g. emergency client. Absence of this attribute means the GMLC is not dedicated to serve specific client types. See clause 6.1.6.3.3 TS 29.572 [86]. allowedValues: "EMERGENCY_SERVICES": External client for emergency services "VALUE_ADDED_SERVICES": External client for value added services "PLMN_OPERATOR_SERVICES": External client for PLMN operator services "LAWFUL_INTERCEPT_SERVICES": External client for Lawful Intercept services "PLMN_OPERATOR_BROADCAST_SERVICES": External client for PLMN Operator Broadcast services "PLMN_OPERATOR_OM": External client for PLMN Operator O&M "PLMN_OPERATOR_ANONYMOUS_STATISTICS": External client for PLMN Operator anonymous statistics "PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT": External client for PLMN Operator target MS service support | type: <<enumeration>> multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| gmlcNumbers | This attribute represents each item of the array shall carry an OctetString indicating the ISDN number of the GMLC in international number format as described in ITU-T Rec. E.164 [94] and shall be encoded as a TBCD-string. Pattern for string: "^[0-9]{5,15}$" allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| gmlcInfo | This attribute represents information of an GMLC NF Instance. allowedValues: N/A | type: GmlcfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nTNPLMNRestrictionsList | This attribute defines the location restrictions per PLMN that relates to non-terrestrial network access. | type: NTNPLMNRestrictionsInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| blockedLocationInfoList | This defines the information related with the location for which the access restrictions are to be applied in case of NTN. | type: BlockedLocationInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| blockedLocation | This provides the geographical location at which the PLMN are not allowed in case of NTN. | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| blockedDurWindow | This provides the time durations for which the PLMN are not allowed at a given location in case of NTN | type: TimeWindow multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| blockedSlice | This provides the slice for which the access is not allowed at a given location in case of NTN. | type: S-NSSAI multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nwdafLogicalFuncSupported | It represents the logical functions supported by the NWDAF. If not present, the NWDAF shall be regarded with no logical decomposition, in that case the NWDAF only supports the analytics services. allowedValues: “NWDAF_WITH_ANLF” indicates the NWDAF containing Analytics logical function (AnLF), “NWDAF_WITH_MTLF” indicates the NWDAF containing Model Training logical function (MTLF), “NWDAF_WITH_ANLF_MTLF” indicates the NWDAF containing both Analytics logical function (AnLF) and Model Training logical function (MTLF). | type: ENUM multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| satelliteCoverageInfoList | This attribute defines the information related to NR Satellite RAT type and corresponding information of satellite coverage | type: SatelliteCoverageInfo multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRSatelliteRATtype | This attribute defines the RAT Type for NR satellite access. allowedValues: “NRLEO” “NRMEO” “NRGEO” “NROTHERSAT” | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| locationInfo | This attribute defines the information about location and corresponding time windows for which the satellite coverage will be available or unavailable. | type: NtnLocationInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| location | This defines the Location (geographical area) under consideration to which the satellite coverage info belongs | type: GeoArea multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availabilityWindows | This attribute defines the list of time windows at which the satellite coverage will be available for this location. Either availabilityWindows or nonAvailabilityWindows shall be present. | type: TimeWindow multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nonAvailabilityWindows | This attribute defines the list of time windows at which the satellite coverage will not be available for this location. Either availabilityWindows or nonAvailabilityWindows shall be present. | type: TimeWindow multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| n2InterfaceAmfInfo | This attribute represents the N2 interface information of the AMF. allowedValues: N/A | type: n2InterfaceAmfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| N2InterfaceAmfInfo.ipv4EndpointAddress | This attribute represents available AMF endpoint IPv4 address(es) for N2. allowedValues: N/A | type: Ipv4Addr multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| N2InterfaceAmfInfo.ipv6EndpointAddress | This attribute represents available AMF endpoint IPv6 address(es) for N2. allowedValues: N/A | type: Ipv6Addr multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| N2InterfaceAmfInfo.amfName | This attribute represents AMF Name FQDN as defined in clause 28.3.2.5 of TS 23.003 [13] allowedValues: N/A | type: Fqdn multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| amfOnboardingCapability | This attribute indicates the AMF supports SNPN Onboarding capability. This is used for the case of Onboarding of UEs for SNPNs (see TS 23.501 [2], clause 5.30.2.10). - FALSE: AMF does not support SNPN Onboarding; - TRUE: AMF supports SNPN Onboarding. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| highLatencyCom | This attribute indicates whether the AMF supports High Latency communication (e.g. for NR RedCap UE). This is used for CP NF to discover AMF supporting High Latency communication (see TS 23.501 [2], clause 6.3.5). - FALSE: AMF does not support High Latency communication e.g. for NR RedCap UE; - TRUE: AMF supports High Latency communication e.g. for NR RedCap UE; allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ismfSupportInd | This attribute may be used by an SMF to explicitly indicate the support of I-SMF capability and its preference to be selected as I-SMF. When present, this attribute shall indicate whether the I-SMF capability are supported by the SMF: - TRUE: I-SMF capability supported by the SMF - FALSE: I-SMF capability not supported by the SMF. Absence of this attribute indicates the I-SMF capability support of the SMF is not specified. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| smfOnboardingCapability | This attribute indicates the SMF supports SNPN Onboarding capability and User Plane Remote Provisioning. This is used for the case of Onboarding of UEs for SNPNs (see TS 23.501 [2], clauses 5.30.2.10 and 6.2.6.2). - FALSE: SMF does not support SNPN Onboarding; - TRUE: SMF supports SNPN Onboarding. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| smfUPRPCapability | This attribute IE indicates the SMF supports User Plane Remote Provisioning (UPRP) capability. This is used for the case of Onboarding of UEs for SNPNs (see TS 23.501 [2], clauses 5.30.2.10 and 6.2.6.2). - FALSE: SMF does not support UPRP; - TRUE: SMF supports UPRP. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| sNssaiUpfInfoList | This attribute represents a list of parameters supported by the UPF per S-NSSAI. allowedValues: N/A | type: SnssaiUpfInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sxaInd | This attribute indicates whether the UPF is configured to support Sxa interface. TRUE: Supported FALSE: Not Supported allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| a2xSupportInd | This attribute indicates whether A2X Policy/Parameter provisioning is supported by the PCF. TRUE: Supported FALSE: Not Supported allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| a2xCapability | This attribute shall be present if the PCF supports A2X Capability. When present, this attribute shall indicate the supported A2X Capability by the PCF. allowedValues: N/A | type: A2xCapability multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rangingSlPosSupportInd | Indicates whether ranging and sidelink positioning capability is supported by the PCF. TRUE: Supported FALSE: Not Supported allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| A2xCapability.lteA2x | This attribute indicates whether the PCF supports LTE A2X capability: - TRUE: LTE A2X capability is supported by the PCF - FALSE: LTE A2X capability is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| A2xCapability.nrA2x | This attribute indicates whether the PCF supports NR A2X capability: - TRUE: NR A2X capability is supported by the PCF - FALSE: NR A2X capability is not supported by the PCF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| multiMemAfSessQosInd | This attribute indicates whether the NEF supports Multi-member AF session with required QoS functionality: - TRUE: Multi-member AF session with required QoS functionality is supported by the NEF - FALSE: Multi-member AF session with required QoS functionality is not supported by the NEF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| memberUESelAssistInd | This attribute indicates whether the NEF supports member UE selection assistance functionality: - TRUE: member UE selection assistance functionality is supported by the NEF - FALSE: member UE selection assistance functionality is not supported by the NEF. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mbUpfInfo | This attribute represents information of an MB-UPF NF Instance. allowedValues: N/A | type: MbUpfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mbUpfInfo.sNssaiMbUpfInfoList | This attribute represents the list of parameters supported by the MB-UPF per S-NSSAI. allowedValues: N/A | type: SnssaiUpfInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbUpfInfo.mbSmfServingArea | This attribute represents the MB-SMF service area(s) the MB-UPF can serve. If not provided, the MB-UPF can serve any MB-SMF service area. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbUpfInfo.interfaceMbUpfInfoList | This attribute represents the list of User Plane interfaces configured on the MB-UPF. When this IE is provided in the NF Discovery response, the NF Service Consumer (e.g. MB-SMF) may use this information for MB-UPF selection. allowedValues: N/A | type: InterfaceUpfInfoItem multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbUpfInfo.taiList | This attribute represents the list of TAIs the MB-UPF can serve. The absence of this attribute and the taiRangeList attribute indicates that the MB-UPF can serve the whole MB-SMF service area defined by the MbSmfServingArea attribute. allowedValues: N/A | type: Tai multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbUpfInfo.taiRangeList | This attribute represents the range of TAIs the MB-UPF can serve. The absence of this attribute and the taiList attribute indicates that the MB-UPF can serve the whole MB-SMF service area defined by the MbSmfServingArea attribute. allowedValues: N/A | type: Tairange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbUpfInfo.priority | This attribute represents priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection for a service request matching the attributes of the MbUpfInfo; lower values indicate a higher priority. See the precedence rules in the description of the priority attribute in NFProfile, if Priority is also present in NFProfile. The NRF may overwrite the received priority value when exposing an NFProfile with the Nnrf_NFDiscovery service. allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SnssaiUpfInfoItem.sNssai | It represents supported S-NSSAI. allowedValues: N/A | type: ExtSnssai multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SnssaiUpfInfoItem.dnnUpfInfoList | This attribute represents a list of parameters supported by the UPF per DNN. allowedValues: N/A | type: DnnUpfInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SnssaiUpfInfoItem.redundantTransport | This attribute indicates whether the UPF supports redundant transport path on the transport layer in the corresponding network slice. allowedValues: TRUE: supported FALSE: not supported | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| DnnUpfInfoItem.dnaiList | This attribute represents a list of Data network access identifiers supported by the UPF for this DNN. The absence of this attribute indicates that the UPF can be selected for this DNN for any DNAI. Each item in the list is the DNAI (Data network access identifier), see TS 23.501 [2]. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.pduSessionTypes | This attribute represents a list of PDU session type(s) supported by the UPF for a specific DNN. The absence of this attribute indicates that the UPF can be selected for this DNN for any PDU session type supported by the UPF (see clause 6.1.6.2.13). allowedValues: “IPv4” “IPv6” “IPv4v6” as per clause 5.8.2.2.1 TS 23.501 [2] “UNSTRUCTURED” “ETHERNET” | type: <<enumeration>> multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.ipv4AddressRanges | This attribute represents a list of ranges of IPv4 addresses handled by UPF. allowedValues: N/A | type: Ipv4AddressRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.ipv6PrefixRanges | This attribute represents a list of ranges of IPv6 prefixes handled by the UPF. allowedValues: N/A | type: Ipv6PrefixRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.natedIpv4AddressRanges | This attribute represents a list of ranges of NATed IPv4 addresses. allowedValues: N/A | type: Ipv4AddressRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.natedIpv6PrefixRanges | This attribute represents a list of ranges of NATed IPv6 prefixes. allowedValues: N/A | type: Ipv6PrefixRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.ipv4IndexList | This attribute represents a list of Ipv4 Index supported by the UPF. This <<choice>> represents the IP Index to be sent from UDM to the SMF. (See clause 6.1.6.2.77 TS 29.503 [97]) It is a list of non-exclusive alternatives (Integer or String). allowedValues: N/A | type: <<choice>> multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.ipv6IndexList | This attribute represents a list of Ipv6 Index supported by the UPF. This <<choice>> represents the IP Index to be sent from UDM to the SMF. (See clause 6.1.6.2.77 TS 29.503 [97]) It is a list of non-exclusive alternatives (Integer or String). allowedValues: N/A | type: <<choice>> multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnUpfInfoItem.networkInstance | This attribute represents the N6 Network Instance (See TS 29.244 [56]) associated with the S-NSSAI and DNN. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DnnUpfInfoItem.dnaiNwInstanceList | This attribute represents a map of a network instance per DNAI for the DNN, where the key of the map is the DNAI (Data network access identifier), see TS 23.501 [2]. When present, the value of each entry of the map shall contain a N6 network instance that is configured for the DNAI indicated by the key. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbSmfInfo | This attribute represents information of an MB-SMF NF Instance allowedValues: N/A | type: MbSmfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MbSmfInfo.sNssaiInfoList | This attribute represents the list of S-NSSAIs and DNNs supported by the MB-SMF. The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [92], with a maximum of 32 characters. allowedValues: N/A | type: NFType multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MbSmfInfo.tmgiRangeList | This attribute represents the list of TMGI range(s) supported by the MB-SMF The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [92], with a maximum of 32 characters. allowedValues: N/A | type: TmgiRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MbSmfInfo.taiList | This attribute represents the list of TAIs the MB-SMF can serve. The absence of this attribute and the taiRangeList attribute indicates that the MB-SMF can be selected for any TAI in the serving network. allowedValues: N/A | type: TAI multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MbSmfInfo.taiRangeList | This attribute represents the range of TAIs the MB-SMF can serve. The absence of this attribute and the taiList attribute indicates that the MB-SMF can be selected for any TAI in the serving network. allowedValues: N/A | type: TAIRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MbSmfInfo.mbsSessionList | This attribute represents the list of MBS sessions currently served by the MB-SMF The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [92], with a maximum of 32 characters. allowedValues: N/A | type: MbsSession multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbsServiceIdStart | This attribute represents the first MBS Service ID value identifying the start of a TMGI range. The value shall be coded as defined for the mbsServiceId attribute of the Tmgi data type defined in 3GPP TS 29.571 [61]. Pattern: '^[A-Fa-f0-9]{6}$'s. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mbsServiceIdEnd | This attribute represents the last MBS Service ID value identifying the end of a TMGI range. The value shall be coded as defined for the mbsServiceId attribute of the Tmgi data type defined in 3GPP TS 29.571 [61]. Pattern: '^[A-Fa-f0-9]{6}$ allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mbsServiceId | This attribute represents MBS Service ID consisting of a 6-digit fixed-length hexadecimal number between 000000 and FFFFFF. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the MBS Service ID shall appear first in the string, and the character representing the 4 least significant bit of the MBS Service ID shall appear last in the string. Pattern: '^[A-Fa-f0-9]{6}$' allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Ssm.sourceIpAddr | This attribute represents IP unicast address used as source address in IP packets for identifying the source of the multicast service (e.g. AF/AS). allowedValues: N/A | type: IpAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Ssm.destIpAddr | This attribute represents IP multicast address used as destination address in related IP packets for identifying the multicast service associated with the source. allowedValues: N/A | type: IpAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MbsSession.mbsSessionId | This attribute represents the MBS Session Identifier. allowedValues: N/A | type: MbsSessionId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MbsSession.mbsAreaSessions | This attribute represents map of Area Session Id and related MBS Service Area information used for MBS session with location dependent content. The Area Session ID together with the mbsSessionId (TMGI) uniquely identifies the MBS session in a specific MBS service area. For an MBS session with location dependent content, one map entry shall be registered for each MBS Service Area served by the MBS session. The key of the map shall be the areaSessionId. | type: MbsServiceAreaInfo multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MbsServiceAreaInfo.areaSessionId | This attribute represents Area Session Identifier used for MBS session with location dependent content. allowedValues: 0..65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MbsServiceAreaInfo.mbsServiceArea | This attribute represents MBS Service Area for MBS session with location dependent content. allowedValues: N/A | type: MbsServiceArea multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MbsServiceArea.ncgiList | This attribute represents a list of NR cell ids with their pertaining TAIs. allowedValues: N/A | type: Ncgi multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pLMNId | This attribute represents a PLMN Identity. allowedValues: N/A | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrCellId | This attribute represents NR Cell Identity. It's a 36-bit string identifying an NR Cell Id as specified in clause 9.3.1.7 of TS 38.413 [5], in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Cell Id shall appear first in the string, and the character representing the 4 least significant bit of the Cell Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{9}$' Example: An NR Cell Id 0x225BD6007 shall be encoded as "225BD6007". allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| HssInfo.groupId | This attribute defines the identity of the HSS group that is served by the HSS instance. If not provided, the HSS instance does not pertain to any HSS group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| HssInfo.imsiRanges | This attribute defines the list of ranges of IMSIs whose profile data is available in the HSS instance. allowedValues: N/A | type: ImsiRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| HssInfo.imsPrivateIdentityRanges | This attribute defines the list of ranges of IMS Private Identities whose profile data is available in the HSS instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| HssInfo.imsPublicIdentityRanges | This attribute defines the list of ranges of IMS Public Identities whose profile data is available in the HSS instance (NOTE 1) allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| HssInfo.msisdnRanges | This attribute defines the list of ranges of MSISDNs whose profile data is available in the HSS instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| HssInfo.externalGroupIdentifiersRanges | This attribute defines the list of ranges of external group IDs that can be served by this HSS instance. If not provided, the HSS instance does not serve any external groups. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| HssInfo.hssDiameterAddress | This attribute defines the Diameter Address of the HSS allowedValues: N/A | type: NetworkNodeDiameterAddress multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| HssInfo.additionalDiamAddresses | This attribute defines the Additional Diameter Addresses of the HSS; may be present if hssDiameterAddress is present allowedValues: N/A | type: NetworkNodeDiameterAddress multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NetworkNodeDiameterAddress.name | This attribute indicates the Diameter name of the network node diameter address. See TS 29.571 [61]. String contains a Diameter Identity (FQDN). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkNodeDiameterAddress.realm | This attribute indicates the Diameter realm of the network node diameter addres. See TS 29.571 [61]. String contains a Diameter Identity (FQDN). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ImsiRange.start | This attribute indicates the first value identifying the start of a IMSI range. Pattern: "^[0-9]+$" allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ImsiRange.end | This attribute indicates the last value identifying the end of a IMSI range. Pattern: "^[0-9]+$" allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ImsiRange.pattern | This attribute indicates pattern (regular expression according to the ECMA-262 dialect [75]) representing the set of IMSIs belonging to this range. An IMSI value is considered part of the range if and only if the IMSI string fully matches the regular expression. Either the start and end attributes, or the pattern attribute, shall be present. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnpfInfo | This attribute represents information of an MNPF NF Instance allowedValues: N/A | type: MnpfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MnpfInfo.msisdnRanges | This attribute represents the list of ranges of MSISDNs whose portability status is available in the MNPF. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelRefList | This attribute holds a DN list of MLModel (See TS 28.105 [105]) . | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceFunctionRefList | This attribute holds a DN list of AIMLInferenceFunction (See TS 28.105 [105]) . | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TrustAfInfo.sNssaiInfoList | It represents S-NSSAIs and DNNs supported by the trust AF. allowedValues: N/A | type: SnssaiInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SnssaiTsctsfInfoItem.dnnInfoList | It represents list of parameters supported by the TSCTSF per DNN. allowedValues: N/A | type: DnnTsctsfInfoItem multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnnTsctsfInfoItem.dnn | It represents supported DNN or Wildcard DNN if the TSCTSF supports all DNNs for the related S-NSSAI. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mlModelInterInfo | This attribute defines the list of NWDAF vendors that are allowed to retrieve ML models from the NWDAF containing MTLF. The absence of this attribute indicates that none of the NWDAF vendors can retrieve the ML models. allowedValues: 6 decimal digits; if the SMI code has less than 6 digits, it shall be padded with leading digits "0" to complete a 6-digit string value. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| flCapabilityType | This attribute defines the federated learning capability type supported by NWDAF containing MTLF. allowedValues: “FL_SERVER” indicates NWDAF containing MTLF as Federated Learning Server, “FL_CLIENT” indicates NWDAF containing MTLF as Federated Learning Client, “FL_SERVER_AND_CLIENT” indicates NWDAF containing MTLF as Federated Learning Server and Client. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flTimeInterval | This attribute defines the time window at which the indicated flCapabilityType supported by NWDAF MTLF is available. This attribute shall be present only if flCapabilityType attribute is present. allowedValues: N/A | type: TimeWindow multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| qFMonitoredSatelliteBackhaulCategories | It specifies the satellite backhaul categories for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: "DYNAMIC_GEO" "DYNAMIC_MEO" "DYNAMIC_LEO" "DYNAMIC_OTHER_SAT" | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| AMFFunction.sliceExpiryInfo | This provides information related to a network slice validity. | type: SliceExpiryInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expiryTime | This attribute provides information about the time at which the slice is scheduled to be expired as it is not required anymore. This attribute will be set based on the sliceAvailability coming as part of ServiceProfile. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servedPcscfInfoList | This attribute contains all the pcscfInfo attributes locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedNfInfo | This attribute contains information of other NFs without corresponding NF type specific Info extensions locally configured in the NRF or the NRF received during NF registration. The key of the map is the nfInstanceId of the NF. The map entry is the NfInfo as defined in clause 5.3.229 representing the information of a generic NF instance. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedAanfInfoList | This attribute contains the aanfInfoList attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| PcscfInfo.dnnList | This attribute represents DNNs supported by the P-CSCF. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. If not provided, the P-CSCF can serve any DNN. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| gmFqdn | This attribute represents FQDN of the P-CSCF for the Gm interface. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gmIpv4Addresses | This attribute represents list of IPv4 addresses of of the P-CSCF for the Gm interface. allowedValues: N/A | type: Ipv4Addr multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| gmIpv6Addresses | This attribute represents list of IPv6 addresses of of the P-CSCF for the Gm interface. allowedValues: N/A | type: Ipv6Addr multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mwFqdn | This attribute represents FQDN of the P-CSCF for the Mw interface. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mwIpv4Addresses | This attribute represents list of IPv4 addresses of of the P-CSCF for the Mw interface. allowedValues: N/A | type: Ipv4Addr multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mwIpv6Addresses | This attribute represents list of IPv6 addresses of of the P-CSCF for the Mw interface. allowedValues: N/A | type: Ipv6Addr multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedIpv4AddressRanges | This attribute represents list of ranges of UE IPv4 addresses used on the Gm interface, served by P-CSCF. The absence of this attribute does not mean the P-CSCF can serve any IPv4 address. allowedValues: N/A | type: Ipv4AddressRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedIpv6PrefixRanges | This attribute represents list of ranges of UE IPv6 prefixes used on the Gm interface, served by P-CSCF. The absence of this attribute does not mean the P-CSCF can serve any IPv6 prefix. allowedValues: N/A | type: Ipv6PrefixRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| AMFFunction.satelliteBackhaulInfoList | This attribute defines the list of satellite backhaul information, including satellite backhaul categoty and corresponding information of (R)AN. allowedValues: N/A | type: SatelliteBackhaulInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| SatelliteBackhaulInfo.nTNGlobalRanNodeID | It specifies the unique identifier of a (R)AN node for NTN scenario. It is used to identify which (R)AN node the satellite backhaul type is applicable to. allowedValues: N/A | type: NTNGlobalRanNodeID multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SatelliteBackhaulInfo.satelliteBackhaulCategory | Define the type of the satellite used in the backhaul. Only a single backhaul category can be indicated. allowedValues: "GEO" "MEO" "LEO" "OTHER_SAT" "DYNAMIC_GEO" "DYNAMIC_MEO" "DYNAMIC_LEO" "DYNAMIC_OTHER_SAT" "NON_SATELLITE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SatelliteBackhaulInfo.geoSatelliteId | Unique identifier of a GEO satellite. See e.g. clause 5.43 in 3GPP TS 23.501 [2]. It shall be formatted as a fixed 5-digit string, padding with leading digits “0” to complete a 5-digit length. Pattern: '^[0-9]{5}$' allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID. pLMNId | This attribute represents a PLMN Identity. allowedValues: N/A | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID.n3IwfId | This represents the identifier of the N3IWF ID. (Ref. clause 9.3.1.57 of 3GPP TS 38.413 [11]) allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID.gNbId | This represents the identifier of the gNB. (Ref. clause 8.2 of 3GPP TS 38.300 [3]) allowedValues: 0..4294967295 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID.ngeNbId | This represents the identifier of the ng-eNB ID. (Ref. clause 9.3.1.8 of 3GPP TS 38.413 [11]) allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID.wagfId | This represents the identifier of the W-AGF ID. (Ref. clause 9.3.1.162 of 3GPP TS 38.413 [11]) allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID.tngfId | This represents the identifier of the TNGF ID. (Ref. clause 9.3.1.161 of 3GPP TS 38.413 [11]) allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NTNGlobalRanNodeID.twifId | This represents the TWIF identification. (Ref. clause 9.3.1.153 of 3GPP TS 38.413 [11]) allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SMFFunction.dnaiSatelliteMappingList | It specifies the mapping relationship between satellite ID and at least one DNAI. allowedValues: N/A | type: DnaiSatelliteMapping multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnaiSatelliteMapping.dnaiList | List of Data network access identifiers supported for this DNN. allowedValues: DNAI (Data network access identifier), see clause 5.6.7 of 3GPP TS 23.501 [2]. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| DnaiSatelliteMapping.geoSatelliteId | Unique identifier of a GEO satellite. See e.g. clause 5.43 in 3GPP TS 23.501 [2]. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mdtUserConsentReqList | It represents a list of MDT measurement names that are subject to user consent at MDT activation, as defined in clause 4.4.1. | See mdtUserConsentReqList in clause 4.4.1. |
| mappedCellIdInfoList | It provides the list of mapping between GEO area and Mapped Cell ID. allowedValues: Not applicable | type: MappedCellIdInfo multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ephemerisInfos | This is the list of Ephemeris related information. See clause 4.3.79. allowedValues: N/A | type: Ephemeris multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trpInfoList | This is the list of TRP (Transmission-Reception Point) related information on LMF (see TS 38.305 [107] clause 5.4.4). allowedValues: N/A | type: TrpInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TrpInfo.gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of subclause 8.2 of TS 38.300 [3]. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrpInfo.trpMappingInfoList | This is the list of TRP mapping between satellite and TRPs. allowedValues: N/A | type: TrpMappingInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TrpMappingInfo.satelliteId | This attribute indicates satellite Id. It shall be formatted as a fixed 5-digit string, padding with leading digits “0” to complete a 5-digit length. allowedValues: Follow the pattern: '^[0-9]{5}$' | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrpMappingInfo.trpIds | This attribute indicates TRPs uniquely within an NG-RAN node (see TS 38.455 [108] clause 9.2.24). A gNB may serve several TRPs. For NTN, a TRP may be located on board the satellite. allowedValues: 1..65535 | type: Integer multiplicity: * isOrdered: false isUnique: True defaultValue: None isNullable: False |
| servedHssInfoList | This attribute contains list of HssInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| served5gDdnmfInfo | This attribute contains all the 5gDdnmfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedMfafInfoList | This attribute contains list of MfafInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedEasdfInfoList | This attribute contains list of EasdfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedDccfInfoList | This attribute contains list of DccfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedMbSmfInfoList | This attribute contains list of MbSmfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedTsctsfInfoList | This attribute contains list of TsctsfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedMbUpfInfoList | This attribute contains list of MbUpfInfo attribute locally configured in the NRF or that the NRF received during NF registration. The key of the map is the nfInstanceId to which the map entry belongs to. allowedValues: N/A | type: AttributeValuePair multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| BsfInfo | This attribute represents information of a BSF NF Instance. allowedValues: N/A | type: BsfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| BsfInfo.ipv4AddressRanges | This attribute represents the list of ranges of IPv4 addresses handled by BSF. If not provided, the BSF can serve any IPv4 address. allowedValues: N/A | type: Ipv4AddressRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| BsfInfo.dnnList | This attribute represents the list of DNNs handled by the BSF. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. If not provided, the BSF can serve any DNN. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| BsfInfo.ipDomainList | This attribute represents the list of IPv4 address domains, as described in clause 6.2 of 3GPP TS 29.513 [28], handled by the BSF. If not provided, the BSF can serve any IP domain. allowedValues: N/A | type: TAIRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| BsfInfo.ipv6PrefixRanges | This attribute represents the list of ranges of IPv6 prefixes handled by the BSF. If not provided, the BSF can serve any IPv6 prefix. allowedValues: N/A | type: Ipv6PrefixRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| BsfInfo.rxDiamHost | This attribute represents the Diameter host of the Rx interface for the BSF. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| BsfInfo.rxDiamRealm | This attribute represents the Diameter realm of the Rx interface for the BSF. See TS 29.571 [61]. String contains a Diameter Identity (FQDN). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| BsfInfo.groupId | This attribute represents the identity of the BSF group that is served by the BSF instance. If not provided, the BSF instance does not pertain to any BSF group. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| BsfInfo.supiRanges | This attribute represents list of ranges of SUPI's served by the BSF instance allowedValues: N/A | type: SupiRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| BsfInfo.gpsiRanges | This attribute represents list of ranges of GPSI's served by the BSF instance allowedValues: N/A | type: IdentityRange multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predefinedPccRuleSetRefs | This holds a list of DN of PredefinedPccRuleSet instance. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nfStatus | It represents status of the NF Instance. allowedValues: refer to TS 29.510[23] clause 6.1.6.3.7 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| plmnList | It represents list of PLMN(s) of the Network Function. It shall be present if this information is available for the NF. allowedValues: N/A | type: PlmnId multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sNssais | It indicates S-NSSAIs of the Network Function. allowedValues: N/A | type: S-NSSAI multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nfServices | It indicates a list of NF Service Instances. allowedValues: N/A | type: NFService multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceInstanceId | It indicates the unique ID of the service instance within a given NF Instance. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceName | It indicates name of the service instance. allowedValues:refer to TS 29.510[23] clause 6.1.6.3.11 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NFService.versions | This attribute identifies the API versions (supported by the NF Service and if available, the corresponding retirement date of the NF Service. allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| schema | It indicates URI scheme (e.g. "http", "https"). allowedValues: “http”, “https” | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ipEndPoints | It indicates IP address(es) and port information of the Network Function (including IPv4 and/or IPv6 address) where the service is listening for incoming service requests. allowedValues: N/A | type: IpEndPoint multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| apiPrefix | It indicates an optional path segment(s) used to construct the {apiRoot} variable of the different API URIs allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NefInfo.gpsiRanges | It represents list of ranges of GPSIs that can be served by the NEF instance. allowedValues: N/A | type: IdentityRange multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nrfInfo | This attribute represents information of an NRF NF Instance. allowedValues: N/A | type: NrfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trustAfInfo | This attribute represents information of a trusted AF Instance. allowedValues: N/A | type: TrustAfInfo multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: If none of these parameters are provided, the AUSF can serve any SUPI managed by the PLMN of the AUSF instance. If "supiRanges" attribute is absent, and "groupId" is present, the SUPIs served by this AUSF instance is determined by the NRF (see TS 23.501 [2], clause 6.2.6.2). NOTE 2: The combination of SUCI informations, e.g. Routing Indicator and Home Network Public Key Id, can be used as criteria for AUSF discovery. This may only be used by the HPLMN in roaming scenarios in this release of the specification, i.e. an AMF in a visited network does not use the Home Network Public Key ID for AUSF selection. NOTE 3: If the suciInfos attribute is present and contains the routingInds sub-attribute, then the routingIndicators attribute shall also be present. |  |  |

## 5.5	Common notifications

### 5.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.111[109], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | O | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

Note:	notifyChangedAlarm is deprecated in TS 28.111 [109].

### 5.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyMOIChanges | O | -- |
| notifyEvent | O | -- |

### 5.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

# 5A	Information model definitions for SBA support of IMS

## 5A.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.705 [72], IOC, CSCFFunction | CSCFFunction |
| TS 28.705 [72], IOC, HSSFunction | HSSFunction |
| TS 28.705 [72], IOC, PCSCFFunction | PCSCFFunction |

## 5A.2	Class diagram

### 5A.2.1	Class diagram for SBA support of IMS

#### 5A.2.1.1	Relationships

The set of classes (e.g. IOCs) that encapsulate the information relevant for IMS network resource information for telecommunication network management purposes are described in TS 28.705 [72].

The set of classes IOC EP_RP for SBA support of SCSCFFunction, HSSFunction, PCSCFFunction and PCFFunction are described in this clause.

The Figure 5A.2.1.1-1 shows the transport view of SCSCFFunction NRM for SBA interfaces.

Figure 5A.2.1.1-1: Transport view of SCSCFFunction for SBA interfaces

The Figure 5A.2.1.1-2 shows the transport view of HSSFunction NRM for SBA interfaces.

Figure 5A.2.1.1-2: Transport view of HSSFunction for SBA interfaces

The Figure 5A.2.1.1-3 shows the transport view of PCFFunction NRM for SBA interfaces.

Figure 5A.2.1.1-3: Transport view of PCFFunction for SBA interfaces

The Figure 5A.2.1.1-4 shows the transport view of PCSCFFunction NRM for SBA interfaces.

Figure 5A.2.1.1-4: Transport view of PCSCFFunction for SBA interfaces

#### 5A.2.1.2	Inheritance

Figure 5A.2.1.2-1 shows the inheritance hierarchy from IOC EP_RP related to SBA interfaces of IMS nodes.

Figure 5A.2.1.2-1: Inheritance hierarchy from IOC EP_RP related to SBA interfaces of IMS

## 5A.3	Class definitions

### 5A.3.1	EP_N5

#### 5A.3.1.1	Definition

This IOC represents the N5 interface between P-CSCF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5A.3.1.2	Attributes

The EP_N5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

### 5A.3.2	EP_N70

#### 5A.3.2.1	Definition

This IOC represents the N70 interface between S/I-CSCF and HSS, which is defined in 3GPP TS 23.501 [2].

#### 5A.3.2.2	Attributes

The EP_N70 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

### 5A.3.3	EP_N71

#### 5A.3.3.1	Definition

This IOC represents the N71 interface between AF and HSS, which is defined in 3GPP TS 23.501 [2].

#### 5A.3.3.2	Attributes

The EP_N71 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

# 6	Information model definitions for network slice NRM

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |
| TS 28.622 [30], dataType, ProcessMonitor | ProcessMonitor |
| TS 28.622 [30], dataType, GeoArea | GeoArea |
| TS 28.622 [30], dataType, Tai | Tai |
| TS 28.622 [30], choice, TimeWindow | TimeWindow |
| TS 28.622 [30], choice, SchedulingTime | SchedulingTime |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

NOTE 3:	The instance tree of this NRM fragment would not contain the instances of NetworkService and VNF. However, the NetworkSliceSubNet instances would have an attribute holding the identifiers of NetworkService instances and the ManagedFunction instance would have an attribute holding identifiers of VNF instances.

NOTE 4:	Any instance of the NetworkSliceSubnet IOC is associated to 0 to 1 instance of the NetworkSlice IOC:

- 1: applies to the top/root NetworkSliceSubnet IOC instance directly associated to a NetworkSlice IOC instance.

- 0: applies to all non-top/non-root NetworkSliceSubnet IOC instances, also known as constituent network slice subnets, not directly associated to a NetworkSlice IOC instance.

Any instance of the NetworkSlice IOC is associated to exactly one instance of NetworkSliceSubnet IOC (i.e. the top/root NetworkSliceSubnet IOC instance)

Figure 6.2.1-2: Transport EP NRM fragment relationship

Figure 6.2.1-3: Containment relationship for network slice fragment

Figure 6.2.1-4: Containment relationship for feasibility check and resource reservation NRM fragment

Figure 6.2.1-5: Containment relationship for isolation profile fragment

Figure 6.2.1-6: IsolationProfile NRM fragment relationship related to NetworkSlice

Figure 6.2.1-7: IsolationProfile NRM fragment relationship related to NetworkSliceSubnet

### 6.2.2	Inheritance

Figure 6.2.2-1: Network slice inheritance relationship

Figure 6.2.2-2: inheritance relationship for feasibility check NRM fragment

Figure 6.2.2-3: inheritance relationship for NetworkSliceController and NetworkSliceSubnetController NRM fragment

## 6.3	Class definitions

### 6.3.1	NetworkSlice

#### 6.3.1.1	Definition

This IOC represents the properties of a network slice in a 5G network. For more information about the network slice, see 3GPP TS 28.530 [70].

#### 6.3.1.2	Attributes

The NetworkSlice IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |  |
| administrativeState | M | T | T | F | T |
| serviceProfileList | M | T | T | F | T |  |
| Attribute related to role |  |  |  |  |  |  |
| networkSliceSubnetRef | M | T | F | F | T |  |
| networkSliceControllerRef | M | T | F | F | T |  |
| isolationProfileRef | CM | T | T | F | T |  |

#### 6.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| isolationProfileRef S | Condition: Network slicing isolation feature is supported. |

#### 6.3.1.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.2	NetworkSliceSubnet

#### 6.3.2.1	Definition

This IOC represents the properties of a network slice subnet in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.530 [70].

The NetworkSliceSubnet can be categorized by following types:

-	RANSliceSubne represent the RAN network slice subnet in a 5G network, which is associated to one or multiple “RANSliceSubnetProfile”.

-	CNSliceSubnet represent the CN network slice subnet in a 5G network, which is associated to one or multiple “CNSliceSubnetProfile”.

-	TopSliceSubnet represent the top network slice subnet in a 5G network, which is associated to one or multiple “TopSliceSubnetProfile”.

The attribute epTransportRef is used to specify a list of EP_Transport instance as transport resources to be aggregated to a NetworkSliceSubnet instance. The MnS consumer determines the EP_Transport instance(s) to support EP_Application instances as part of the NetworkSliceSubnet instance and request the MnS producer to configure the attribute epTransportRef of the NetworkSliceSubnet.

The EP_Transport is name contained by SubNetwork, and an EP_Transport instance can be a new instance created for the EP_Application instances as part of NetworkSliceSubnet instance or an existing instance reused for EP_Application instance.

An instance of NetworkSliceSubnet that represents top network slice subnet shall be associated with one NetworkSlice instance. An instance of NetworkSliceSubnet that represents CN network slice subnet or RAN network slice subnet shall be associated with one instance of NetworkSliceSubnet that represents the top network slice subnet. An instance of NetworkSliceSubnet that represents CN network slice subnet or RAN network slice subnet may be associated with one or more constituent NetworkSliceSubnet instance(s) represented by attribute networkSliceSubnetRef.

#### 6.3.2.2	Attributes

The NetworkSliceSubnet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| nsInfo | CM | T | F | F | T |
| sliceProfileList | M | T | T | F | T |
| priorityLabel | O | T | T | F | T |
| networkSliceSubnetType | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| managedFunctionRef | M | T | F | F | T |
| networkSliceSubnetRef | M | T | F | F | T |
| epTransportRef | O | T | T | F | T |
| networkSliceSubnetControllerRef | M | T | F | F | T |
| isolationProfileRef | CM | T | T | F | T |

#### 6.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nsInfo S | Condition: It shall be supported if the NetworkSliceSubnet instance is realized in the virtualized environment. |
| isolationProfileRef S | Condition: Network slicing isolation feature is supported. |

#### 6.3.2.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.3	ServiceProfile <<dataType>>

#### 6.3.3.1	Definition

This data type represents the properties of the network slice related requirements that should be supported by a NetworkSlice instance in a 5G network. The network slice related requirements apply to a one-to-one relationship between a Network Slice Customer (NSC) and a Network Slice Provider (NSP). A network slice can be tailored based on the specific requirements adhered to an SLA agreed between NSC and NSP, see clause 2 of [50]. An NSP may add additional requirements not directly derived from SLA’s, associated to the NSP internal [business] goals. The GST defined by GSMA (see [50]) and the service performance requirements defined in 3GPP TS 22.261 [28] and TS 22.104 [51] are all considered as input for the network slice related requirements.

#### 6.3.3.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| pLMNInfoList | M | T | F | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageArea | O | T | T | F | T |
| dLLatency | O | T | T | F | T |
| uLLatency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| networkSliceSharingIndicator | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| dLDeterministicComm | O | T | T | F | T |
| uLDeterministicComm | O | T | T | F | T |
| dLThptPerSlice | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSlice | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| dLMaxPktSize | O | T | T | F | T |
| uLMaxPktSize | O | T | T | F | T |
| maxNumberofPDUSessions | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| userMgmtOpen | O | T | T | F | T |
| v2XCommModels | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| radioSpectrum | O | T | T | F | T |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |
| maxDLDataVolume | O | T | T | F | T |
| maxULDataVolume | O | T | T | F | T |
| nBIoT | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| positioning | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| energyEfficiency | O | T | T | F | T |
| nssaaSupport | O | T | T | F | T |
| n6Protection | O | T | T | F | T |
| nonIPSupport | O | T | T | F | T |
| supportedDataNetworks | O | T | T | F | T |
| dataNetworkAccess | O | T | T | F | T |
| sliceAvailability | O | T | T | F | T |
| dLPktDelayVariation | O | T | T | F | T |
| uLPktDelayVariation | O | T | T | F | T |

NOTE:	The attributes in ServiceProfile represent mapped requirements from an NSC (e.g. an enterprise) to an NSP

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.4	SliceProfile <<dataType>>

#### 6.3.4.1	Definition

This data type represents the properties of network slice subnet related requirement that should be supported by the NetworkSliceSubnet instance in a 5G network.

#### 6.3.4.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| pLMNInfoList | M | T | T | F | T |
| CNSliceSubnetProfile | CM | T | T | F | T |
| RANSliceSubnetProfile | CM | T | T | F | T |
| TopSliceSubnetProfile | CM | T | T | F | T |

#### 6.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CNSliceSubnetProfile S | Condition: CN domain slice profile is supported. |
| RANSliceSubnetProfile S | Condition: RAN domain slice profile is supported. |
| TopSliceSubnetProfile S | Condition: Top network slice subnet is supported. |

#### 6.3.4.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.5	NsInfo <<dataType>>

#### 6.3.5.1	Definition

This data type represents the properties of network service information (See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]) corresponding to the network slice subnet instance.

#### 6.3.5.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nsInstanceId | M | T | F | F | T |
| nsName | O | T | F | F | T |
| description | O | T | F | F | T |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.6	ServAttrCom <<dataType>>

#### 6.3.6.1	Definition

This data type represents the common properties of service requirement related attributes (see GSMA NG.116 [50] corresponding to Attribute categories, tagging and exposure).

#### 6.3.6.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| category | M | T | F | T | T |
| tagging | CM | T | F | T | T |
| exposure | M | T | F | T | T |

#### 6.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| tagging S | Condition: This attribute shall be supported when this data type is used in the service requirement attribute of GST in the character category. |

#### 6.3.6.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.7	DelayTolerance<<dataType>>

#### 6.3.7.1	Definition

This data type represents the delay tolerance (See Clause 3.4.3 of GSMA NG.116 [50]).

#### 6.3.7.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| support | M | T | F | F | T |

#### 6.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on delay tolerance (GSMA attribute) in ServiceProfile. |

#### 6.3.7.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.8	DeterministicComm <<dataType>>

#### 6.3.8.1	Definition

This data type represents the properties of the deterministic communication for periodic user traffic in downlink or uplink. Periodic traffic refers to the type of traffic with periodic transmissions.

#### 6.3.8.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | F | T |
| availability | M | T | F | F | T |
| periodicityList | M | T | T | F | T |

#### 6.3.8.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on deterministic communication for periodic user traffic per network slice (GSMA attribute) in ServiceProfile. |

#### 6.3.8.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.9	XLThpt<<dataType>>

#### 6.3.9.1	Definition

This data type can be used to represent downlink or uplink throughput per network slice , per network slice subnet,or per UE in a network slice (see clause 3.4.5, 3.4.6, 3.4.31 and 3.4.32 of GSMA NG.116 [50]).

#### 6.3.9.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | F | T |
| guaThpt | O | T | T | F | T |
| maxThpt | O | T | T | F | T |

#### 6.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This [attribute] is mandatory only when requirements are being defined on throughput per: network slice (GSMA attribute), per UE in a network slice (GSMA attribute) in ServiceProfile. |

#### 6.3.9.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.10	Void

### 6.3.11	MaxPktSize <<dataType>>

#### 6.3.11.1	Definition

This data type represents the maximum packet size (See Clause 3.4.11 of GSMA NG.116 [50]) in downlink or uplink.

#### 6.3.11.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| maxSize | M | T | T | F | T |

#### 6.3.11.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on maximum packet size per network slice (GSMA attribute) in ServiceProfile. |

#### 6.3.11.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.12	MaxNumberofPDUSessions <<dataType>>

#### 6.3.12.1	Definition

This data type represents the maximum number of concurrent PDU sessions supported by the network slice (see clause 3.4.16 of GSMA NG.116 [50]).

#### 6.3.12.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| 3GPPNoOfPDUSessions | CM | T | T | F | T |
| non3GPPnOofPDUSessions | CM | T | T | F | T |

#### 6.3.12.3	Attribute constraints

| Name | Definition |
| --- | --- |
| 3GPPNoOfPDUSessions S | Condition: This attribute is mandatory only when requirements are being defined for 3GPP access type. |
| non3GPPNoOfPDUSessions S | Condition: This attribute is mandatory only when requirements are being defined for non 3GPP access type. |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on maximum number of PDU sessions per network slice (GSMA attribute) in ServiceProfile. |

#### 6.3.12.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.13	Void

### 6.3.14	KPIMonitoring <<dataType>>

#### 6.3.14.1	Definition

This data type represents performance monitoring (see clause 3.4.18 of GSMA NG.116 [50]).

#### 6.3.14.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| kPIList | M | T | F | F | T |

#### 6.3.14.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on performance monitoring (GSMA attribute) in ServiceProfile. |

#### 6.3.14.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.15	UserMgmtOpen<<dataType>>

#### 6.3.15.1	Definition

This data type represents User management openness (See Clause 3.4.33 of GSMA NG.116 [50]).

#### 6.3.15.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| support | M | T | F | F | T |

#### 6.3.15.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on user management openness (GSMA attribute) in ServiceProfile. |

#### 6.3.15.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.16	V2XCommMode<<dataType>>

#### 6.3.16.1	Definition

This data type represents V2X communication mode (See Clause 3.4.35 of GSMA NG.116 [50]).

#### 6.3.16.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| v2XMode | M | T | F | F | T |

#### 6.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on V2X communication mode (GSMA attribute) in ServiceProfile. |

#### 6.3.16.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.17	TermDensity<<dataType>>

#### 6.3.17.1	Definition

This data type represents Terminal density (See Clause 3.4.30 of GSMA NG.116 [50]).

#### 6.3.17.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| density | M | T | T | F | T |

#### 6.3.17.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on terminal density (GSMA attribute) in ServiceProfile. |

#### 6.3.17.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.18	EP_Transport

#### 6.3.18.1	Definition

This IOC represents the logical transport interface which is part of a RAN or CN SubNetwork, including reference to transport level information.

MnS consumer can request MnS producer to create an EP_transport instance for one or multiple EP_Application instance(s) of one or multiple NetworkSliceSubnet(s).

The attribute "epApplicationRef" is used to specify a list of EP_N3 instances, EP_NgU instances and EP_F1U instances aggregated to the EP_transport instance.

The attribute "externalEndPointRefList" contains information required to identify associated model instances which reside outside of the 3GPP MIB.

MnS consumer can obtain all the information of the EP_Transport associated to a NetworkSliceSubnet from MnS producer and send to corresponding TN Management System as transport network related requirements.

#### 6.3.18.2	Attributes

The EP_Transport IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipAddress | M | T | F | F | T |
| localLogicalInterfaceInfo | M | T | T | F | T |
| qosProfile | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epApplicationRef | M | T | T | F | T |
| externalEndPointRefList | M | T | T | F | T |

#### 6.3.18.3	Attribute constraints

None.

#### 6.3.18.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.19	EP_Application <<ProxyClass>>

#### 6.3.19.1	Definition

This represents  <<IOC>>EP_N3 or <<IOC>>EP_NgU or <<IOC>>EP_F1U.

#### 6.3.19.2	Attributes

See that defined in <<IOC>>EP_N3 or <<IOC>>EP_NgU or <<IOC>>EP_F1U.

#### 6.3.19.3	Attribute constraints

See respective IOCs.

#### 6.3.19.4	Notifications

See respective IOCs.

### 6.3.20	NBIoT <<dataType>>

#### 6.3.20.1	Definition

This data type represents NB-IoT Support (see clause 3.4.14 of GSMA NG.116 [50]).

#### 6.3.20.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| support | M | T | F | F | T |

#### 6.3.20.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on NB-IoT support (GSMA attribute) in ServiceProfile. |

#### 6.3.20.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.21	Void

### 6.3.22	Void

### 6.3.23	CNSliceSubnetProfile<<dataType>>

#### 6.3.23.1	Definition

This data type represents the requirements for CN slice profile.

#### 6.3.23.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxNumberofUEs | O | T | T | F | T |
| dLLatency | O | T | T | F | T |
| uLLatency | O | T | T | F | T |
| dLThptPerSliceSubnet | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSliceSubnet | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| maxNumberofPDUSessions | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| dLMaxPktSize | O | T | T | F | T |
| uLMaxPktSize | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| energyEfficiency | O | T | T | F | T |
| dLDeterministicComm | O | T | T | F | T |
| uLDeterministicComm | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| nssaaSupport | O | T | T | F | T |
| n6Protection | O | T | T | F | T |
| nonIPSupport | O | T | T | F | T |
| availability | O | T | T | F | T |
| supportedDataNetworks | O | T | T | F | T |
| dataNetworkAccess | O | T | T | F | T |
| sliceSubnetAvailability | O | T | T | F | T |
| dLPktDelayVariation | O | T | T | F | T |
| uLPktDelayVariation | O | T | T | F | T |

#### 6.3.23.3	Attribute constraints

None.

#### 6.3.23.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.24	RANSliceSubnetProfile<<dataType>>

#### 6.3.24.1	Definition

This data type represents the requirements for RAN slice profile.

#### 6.3.24.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageAreaTAList | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| dLThptPerSliceSubnet | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSliceSubnet | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |
| nROperatingBands | O | T | T | F | T |
| dLLatency | O | T | T | F | T |
| uLLatency | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| dLMaxPktSize | O | T | T | F | T |
| uLMaxPktSize | O | T | T | F | T |
| energyEfficiency | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| dLDeterministicComm | O | T | T | F | T |
| uLDeterministicComm | O | T | T | F | T |
| positioning | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| availability | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| maxDLDataVolume | O | T | T | F | T |
| maxULDataVolume | O | T | T | F | T |
| dLPktDelayVariation | O | T | T | F | T |
| uLPktDelayVariation | O | T | T | F | T |

#### 6.3.24.3	Attribute constraints

None.

#### 6.3.24.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.25	TopSliceSubnetProfile<<dataType>>

#### 6.3.25.1	Definition

This data type represents the requirements for a top network slice subnet, a network slice subnet directly associated with the network slice. It includes an aggregated list of the attributes from RANSliceSubnetProfile and CNSliceSubnetProfile.

#### 6.3.25.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLLatency | O | T | T | F | T |
| uLLatency | O | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| dLThptPerSliceSubnet | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSliceSubnet | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| dLMaxPktSize | O | T | T | F | T |
| uLMaxPktSize | O | T | T | F | T |
| maxNumberofPDUSessions | O | T | T | F | T |
| nROperatingBands | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| energyEfficiency | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |
| dLDeterministicComm | O | T | T | F | T |
| uLDeterministicComm | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| positioning | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| nssaaSupport | O | T | T | F | T |
| n6Protection | O | T | T | F | T |
| nonIPSupport | O | T | T | F | T |
| availability | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| maxDLDataVolume | O | T | T | F | T |
| maxULDataVolume | O | T | T | F | T |
| supportedDataNetworks | O | T | T | F | T |
| dataNetworkAccess | O | T | T | F | T |
| sliceSubnetAvailability | O | T | T | F | T |
| dLPktDelayVariation | O | T | T | F | T |
| uLPktDelayVariation | O | T | T | F | T |

#### 6.3.25.3	Attribute constraints

None.

#### 6.3.25.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.26	Positioning <<dataType>>

#### 6.3.26.1	Definition

This data type represents positioning support (see clause 3.4.20 of GSMA NG.116 [50]).

#### 6.3.26.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| availability | O | T | F | F | T |
| predictionFrequency | O | T | T | F | T |
| accuracy | O | T | T | F | T |

#### 6.3.26.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on positioning support (GSMA attribute) in ServiceProfile. |

#### 6.3.26.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.27	Synchronicity <<dataType>>

#### 6.3.27.1	Definition

This data type represents synchronicity support (see clause 3.4.29 of GSMA NG.116 [50]).

#### 6.3.27.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| availability | O | T | F | F | T |
| accuracy | O | T | T | F | T |

#### 6.3.27.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on synchronicity support (GSMA attribute) in ServiceProfile. |

#### 6.3.27.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.28	Void

### 6.3.29	Void

### 6.3.30	EnergyEfficiency <<dataType>>

#### 6.3.30.1	Definition

This data type represents energyEfficiency support (see clause 3.4.7 of GSMA NG.116 [50]).

#### 6.3.30.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| performance | O | T | T | F | T |

#### 6.3.30.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on energy effieciency support (GSMA attribute) in ServiceProfile. |

#### 6.3.30.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.31	RadioSpectrum <<dataType>>

#### 6.3.31.1	Definition

This data type represents the radio spectrum in which the network slice should be supported (see clause 3.4.21 of GSMA NG.116 [50]).

#### 6.3.31.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| nROperatingBands | M | T | T | F | T |

#### 6.3.31.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on radio spectrum (GSMA attribute) in ServiceProfile. |

#### 6.3.31.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.32	N6Protection <<dataType>>

#### 6.3.32.1	Definition

This data type defines required security functions and corresponding rules of each function. It represents the N6 interface protection information in ServiceProfile and CNSliceSubnetProfile for CN.

#### 6.3.32.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| secFuncList | M | T | T | F | T |

#### 6.3.32.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: Only valid when the attribute is in ServiceProfile. |

#### 6.3.32.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.33	SecFunc <<dataType>>

#### 6.3.33.1	Definition

This data type defines each security control functions/features required by the Network Slice or Network Slice Subnet consumer. E.g. Firewall, NAT, antimalware, parental control, DDoS protection function, etc.

#### 6.3.33.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| secFunId | M | T | F | T | T |
| secFunType | M | T | F | F | T |
| secRules | O | T | T | T | T |

#### 6.3.33.3	Attribute constraints

None.

#### 6.3.33.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.34	NSSAASupport <<dataType>>

#### 6.3.34.1	Definition

This data type represents the Network Slice Specific Authentication and Authorization (NSSAA) (See Clause 3.4.37 of GSMA NG.116 [50]).

#### 6.3.34.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| support | M | T | F | F | T |

#### 6.3.34.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on Network Slice Specific Authentication and Authorization (GSMA attribute) in ServiceProfile. |

#### 6.3.34.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.35	LogicalInterfaceInfo <<dataType>>

#### 6.3.35.1	Definition

This data type represents the logical interface information of the logical transport interface.

#### 6.3.35.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| logicalInterfaceType | M | T | T | F | T |
| logicalInterfaceId | M | T | T | F | T |
| systemName | O | T | T | F | T |
| portName | O | T | T | F | T |
| routingProtocol | O | T | T | F | T |

#### 6.3.35.3	Attribute constraints

None.

#### 6.3.35.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.36	NetworkSliceSubnetProviderCapabilities

#### 6.3.36.1	Definition

The NetworkSliceSubnetProviderCapabilities IOC store the capabilities/features of the network slice subnet provider in terms of network slice subnets it can manage/provide. The attributes of this IOC can be queried, using getMOIAttributes operation, to know the capabilities of the provider. Based on the capabilities decision can be made e.g derivation of subnet requirements by the consumer.

#### 6.3.36.2	Attributes

The NetworkSliceSubnetProviderCapabilities IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes. The attributes here are subjected to updates, e.g. after each successful subnet allocation or because of any other operator internals decision.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLLatency | M | T | F | F | T |
| uLLatency | M | T | F | F | T |
| dLThptPerSliceSubnet | M | T | F | F | T |
| uLThptPerSliceSubnet | M | T | F | F | T |
| coverageAreaTAList | M | T | F | F | T |

#### 6.3.36.3	Attribute constraints

None

#### 6.3.36.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.37	FeasibilityCheckAndReservationJob

#### 6.3.37.1	Definition

This IOC represents a feasibility check and reservation job for network slicing related requirements (i.e. ServiceProfile for network slice related requirements, SliceProfile for network slice subnet related requirements) to determine whether the network slicing related requirements can be satisfied. It can be name-contained by SubNetwork.

When the MnS Consumer derives the network slicing related requirements (i.e. ServiceProfile, SliceProfile), before request the MnS producer to allocate or modify an NSI or NSSI, MnS consumer may express a feasibility check and reservation job requirement for the specified network slicing related requirements to MnS producer.

To express a feasibility check and reservation job requirement for specific network slicing related requirements (i.e. ServiceProfile, SliceProfile), MnS consumer needs to request MnS producer to create a FeasibilityCheckAndReservationJob instance on the MnS producer side with the network slicing related requirements specified, and to execute the feasibility check and resource reservation process.

For deletion of feasibility check and reservation job, the MnS consumer needs to request the MnS producer to delete the FeasibilityCheckAndReservationJob instance on the MnS producer side.

Attribute "resourceReservation" is used to represent MnS consumer's requirements for resource reservation for corresponding network slicing related requirements (i.e. ServiceProfile, SliceProfile). In case the value is "True", which means MnS producer needs to reserve corresponding resources (i.e. radio access network resources and/or core network resources) when the feasibility check result is feasible. In this case, attribute "requestedReservationExpiration" is used to represent MnS's requirements for the validity period of the resource reservation, which is specified by MnS consumer. While "reservationExpiration" is used to represent the actual validity period of the resource reservation, which is specified by MnS producer based on requested reservation expiration from MnS consumer and its own reservation capabilities. After the period expires, no guarantees are given for the resources associated to the corresponding network slicing related requirements (i.e. ServiceProfile, SliceProfile). In case the value by is "False" which means MnS producer only check the feasibility for corresponding network slicing related requirements, no guarantee for the corresponding resources.

Attribute "RecommendationRequest" is used to represent MnS consumer's request for recommended requirements when the feasibility check result for corresponding network slicing related requirements (i.e. ServiceProfile and SliceProfile information) is infeasible. In case the value is "True", which means MnS producer needs to derive the value of "recommendedRequirements" as recommended network slicing related requirements (i.e. ServiceProfile and SliceProfile information) which can be supported by the MnS producer when the feasibility check result is infeasible and provide these recommendations to MnS consumer.  The value of "recommendedRequirements" is a list of [attributeName of network slicing related requirements (i.e. ServiceProfile, SliceProfile), recommendedValueRange].

Attribute "feasibilityTimeWindow" represents MnS consumer's request for checking whether the network slicing related requirements (i.e. ServiceProfile and SliceProfile information) can be satisfied at a specified time window. If the resourceReservation is “TRUE”, MnS producer also reserve the resources for the MnS consumer at the specified time window, which means the resources associated to the corresponding network slicing related requirements are only guaranteed for the MnS consumer at the specified time window.

To obtain the progress information of a feasibility check and reservation job, MnS consumer needs to request MnS producer to query the values of attribute "processMonitor".

To obtain the feasibility check result of a feasibility check and reservation job, MnS consumer needs to request MnS producer to query the values of attribute “feasibilityResult” and “inFeasibleReason” when the feasibility check and reservation job is finished. If the feasibility check result indicated as feasible, MnS consumer can request MnS producer to allocate a network slice or network slice subnet with the checked network slicing related requirements (i.e. ServiceProfile or SliceProfile).  In case the feasibility check result is infeasible, MnS consumer may update the network slicing related requirements, and may trigger the feasibility check and reservation job again.

To obtain the resource reservation status, MnS consumer need to request MnS producer to query the value of the attribute "resourceReservationStatus ".

MnS producer will use the reserved resources to satisfy the corresponding network slicing related requirements in the allocation request. In case to use the reserved resources, MnS consumer will use the same ServiceProfileId or SliceProfileId value (which is obtained/queried from the FeasibilityCheckAndReservationJob) as input parameters for allocation request.

#### 6.3.37.2	Attributes

The FeasibilityCheckAndReservationJob IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfile | CM | T | T | F | T |
| serviceProfile | CM | T | T | F | T |
| resourceReservation | O | T | T | F | T |
| recommendationRequest | O | T | T | F | T |
| requestedReservationExpiration | O | T | T | F | T |
| feasibilityTimeWindow | O | T | T | F | T |
| processMonitor | M | T | F | F | T |
| feasibilityResult | M | T | F | F | T |
| inFeasibleReason | O | T | F | F | T |
| resourceReservationStatus | O | T | F | F | T |
| reservationFailureReason | O | T | F | F | T |
| reservationExpiration | O | T | F | F | T |
| recommendedRequirements | O | T | F | F | T |

NOTE:	the feasibility check and resource reservation for TN part is not defined in the present document.

#### 6.3.37.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sliceProfile S | Condition: The FeasibilityCheckAndReservationJob is used to check the feasibility and reserve resources for network slice subnet related requirements. |
| serviceProfile S | Condition: The FeasibilityCheckAndReservationJob is used to check the feasibility and reserve resources for network slice related requirements. |

#### 6.3.37.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.38	MaxNumberofUEs <<dataType>>

#### 6.3.38.1	Definition

This data type represents the maximum number of concurrent UEs supported by the network slice (see clause 3.4.17 of GSMA NG.116 [50]).

#### 6.3.38.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| 3GPPNoOfUEs | CM | T | T | F | T |
| non3GPPNoOfUEs | CM | T | T | F | T |

#### 6.3.38.3	Attribute constraints

| Name | Definition |
| --- | --- |
| 3GPPNoOfUEs S | Condition: This attribute is mandatory only when requirements are being defined for 3GPP access type. |
| non3GPPNoOfUEs S | Condition: This attribute is mandatory only when requirements are being defined for non 3GPP access type. |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on maximum number of concurrent UEs supported by the network slice (GSMA attribute) in ServiceProfile. |

#### 6.3.38.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.39	NetworkSliceController

#### 6.3.39.1	Definition

This IOC represents the network slice controller, that controls the lifecycle of the NetworkSlice MOI, with the requirements of a network slice in a 5G network. For more information about the network slice, see 3GPP TS 28.530 [70].

The MnS consumer communicates the network slice related requirements with either:

a)	inputServiceProfile attribute, in case of using createMOI operation (defined in TS 28.532 [35]) for network slice allocation.

b)	input parameter attributeListIn, in case of using AllocateNsi operation (defined in TS 28.531 [26]) for network slice allocation.

The attribute serviceProfileId defines the service profile identifier provided by the MnS producer. This attribute allows in uniquely identifying the network slice related requirements received from the MnS consumer.

Attribute operationalState indicates the ability of the MnS producer to handle the network slice related requirements:

- "Enabled": The MnS producer is running properly to fulfil the network slice related requirements.

- "Disabled": The MnS producer is not running properly to fulfil the network slice related requirements.

Attribute availabilityStatus indicates the availability status of the fulfilment of network slice related requirements by the MnS producer:

- "dependency": The MnS producer is not able to implement the network slice related requirement because some dependencies outside the control of the MnS producer cannot be fulfilled.

- "degraded": The MnS producer is able to fulfil parts of the network slice related requirements only, hence the network slice is not able to fully comply with the request.

Attribute administrativeState allows the MnS consumer to suspend the implementation of the network slice related requirements:

- "Unlocked": The MnS producer shall implement the network slice related requirements

- "Locked": The MnS producer shall suspend the implementation of the network slice related requirements.

The processMonitor attribute represents the status of the fulfilment of the network slice related requirements by the NetworkSliceController and includes information the MnS consumer can use to monitor the progress and result of the implementation of the NetworkSlice object. The data type of this attribute is ProcessMonitor.

The attribute networkSliceRef shall be a DN of NetworkSlice IOC (defined in clause 6.3.1).

#### 6.3.39.2	Attributes

This IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inputServiceProfile | O | T | T | F | T |
| serviceProfileId | M | T | F | T | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| availabilityStatus | O | T | F | F | T |
| processMonitor | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| networkSliceRef | M | T | F | F | T |

#### 6.3.39.3	Attribute constraints

None.

#### 6.3.39.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.40	NetworkSliceSubnetController

#### 6.3.40.1	Definition

This IOC represents the network slice subnet controller, that controls the lifecycle of the NetworkSliceSubnet MOI, with the requirements of a network slice subnet in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.530 [70].

The MnS consumer communicates the network slice subnet related requirements with either:

a)	inputSliceProfile attribute, in case of using createMOI operation (defined in TS 28.532 [35]) for network slice allocation.

b)	input parameter attributeListIn, in case of using AllocateNssi operation (defined in TS 28.531 [26]) for network slice allocation.

The attribute sliceProfileId defines the service profile identifier provided by the MnS producer. This attribute allows in uniquely identifying the network slice related requirements received from the MnS consumer.

Attribute operationalState indicates the ability of the MnS producer to handle the network slice subnet related requirements:

- "Enabled": The MnS producer is running properly to fulfil the network slice subnet related requirements.

- "Disabled": The MnS producer is not running properly to fulfil the network slice subnet related requirements.

Attribute availabilityStatus indicates the availability status of the fulfilment of network slice subnet related requirements by the MnS producer:

- "dependency": The MnS producer is not able to implement the requirement because some dependencies outside the control of the MnS producer cannot be fulfilled.

- "degraded": The MnS producer is able to fulfil parts of the network slice subnet related requirements only.

Attribute administrativeState allows the MnS consumer to suspend the implementation of the network slice subnet related requirements:

- "Unlocked": The MnS producer shall implement the network slice subnet related requirements

- "Locked": The MnS producer shall suspend the implementation of the network slice subnet related requirements.

The processMonitor attribute represents the status of the fulfilment of the network slice subnet related requirements by the NetworkSliceSubnetController and includes information the MnS consumer can use to monitor the progress and result of the implementation of the NetworkSubnetSlice object.

The attribute networkSliceSubnetRef shall be a DN of NetworkSliceSubnet IOC (defined in clause 6.3.2).

#### 6.3.40.2	Attributes

This IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inputSliceProfile | O | T | T | F | T |
| sliceProfileId | M | T | F | T | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| availabilityStatus | O | T | F | F | T |
| processMonitor | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| networkSliceSubnetRef | M | T | F | F | T |

#### 6.3.40.3	Attribute constraints

None.

#### 6.3.40.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.41	ConnectionPointInfo <<datatype>>

#### 6.3.41.1	Definition

This datatype contains information required to identify a connection point outside of scope of 3GPP MIB (e.g. transport domain), for more details see RFC 8345 [89] and YANG Data Models for 'Attachment Circuits'-as-a-Service (ACaaS) [90].

#### 6.3.41.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| connectionPointId | M | T | T | F | T |
| connectionPointIdType | M | T | T | F | T |

#### 6.3.41.3	Attribute constraints

None.

#### 6.3.41.4	Notifications

None.

### 6.3.42	NonIPSupport<<dataType>>

#### 6.3.42.1	Definition

This data type represents the non IP support of the slice (See Clause 3.4.27 of GSMA NG.116 [50]).

#### 6.3.42.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| support | M | T | F | F | T |

#### 6.3.42.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on non-IP support of the slice (GSMA attribute) in ServiceProfile. |

#### 6.3.42.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.43	IsolationProfile

#### 6.3.43.1	Definition

This IOC represents Isolation Profile with a set of isolation requirements for the managed resources.

The resourceIsolationRuleList attribute is used to specify the list of rules for isolation of managed resources.

As an example, if it is required is to have a dedicated User Plane and shared Control Plane each part would need to have their own CN network slice subnet, the first CN network slice subnet containing only UPFs and the second CN network slice subnet containing all other NF's (so that together the two CN network slice subnets form the complete CN network slice subnet). The two CN network slice subnet would be associated with different IsolationProfiles.

#### 6.3.43.2	Attributes

The IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkSlicingApplicability | M | T | T | F | T |
| resourceIsolationRuleList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| networkSliceSubnetRefList | CM | T | F | F | T |
| networkSliceRefList | CM | T | F | F | T |

#### 6.3.43.3	Attribute constraints

| Name | Definition |
| --- | --- |
| networkSliceRefList S | Condition: IsolationProfile IOC supports isolation requirements for the network slices. |
| networkSliceSubnetRefList S | Condition: IsolationProfile IOC supports isolation requirements for the network slice subnets. |

#### 6.3.43.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.44	ResourceIsolationRule <<dataType>>

#### 6.3.44.1	Definition

This data type represents isolation requirements for the managed resource.

The attribute resourceType is used to specify the managed resource type for isolation. The scope of managed resource type can be “MANAGED_FUNCTION” for the managed function instances, “NETWORK_SERVICE” for the Network Service (NS) information. The attribute isolationRule is used to specify the isolation rules to be applied for the managed resource specified in attribute resourceType.

As an example, to isolate user plane data in the core network for the slices for a particular tenant, attribute resourceType could be specified with value "MANAGED_FUNCTION", and attribute isolationRule value could be specified with value "DEDICATED".

#### 6.3.44.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| resourceType | M | T | T | F | T |
| isolationRule | M | T | T | F | T |

#### 6.3.44.3	Attribute constraints

None.

#### 6.3.44.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.45	DataNetwork <<dataType>>

#### 6.3.45.1	Definition

This data type represents the supported data network (See Clause 3.4.39 of GSMA NG.116 [50]).

#### 6.3.45.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| dnnList | M | T | T | F | T |

#### 6.3.45.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on supported data network (GSMA attribute) in ServiceProfile. |

#### 6.3.45.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.46	DataNetworkAccess <<dataType>>

#### 6.3.46.1	Definition

This data type represents the data network access attributes (See Clause 3.4.34 of GSMA NG.116 [50]).

#### 6.3.46.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| dataAccessList | M | T | T | F | T |
| tunnellingMechanismList | M | T | T | F | T |
| localBreakoutAllowedList | M | T | T | F | T |

#### 6.3.46.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on data network access (GSMA attribute) in ServiceProfile. |

#### 6.3.46.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.47	DataAccess <<dataType>>

#### 6.3.47.1	Definition

This data type represents the data network access (See Clause 3.4.34 of GSMA NG.116 [50]).

#### 6.3.47.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dataNetworkName | M | T | T | F | T |
| dataAccessUsed | M | T | T | F | T |

#### 6.3.47.3	Attribute constraints

None.

#### 6.3.47.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.48	TunnellingMechanism <<dataType>>

#### 6.3.48.1	Definition

This data type represents the data network access (See Clause 3.4.34 of GSMA NG.116 [50]).

#### 6.3.48.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dataNetworkName | M | T | T | F | T |
| tunellingMechanismUsed | M | T | T | F | T |

#### 6.3.48.3	Attribute constraints

None.

#### 6.3.48.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.49	LboAllowed <<dataType>>

#### 6.3.49.1	Definition

This data type represents the data network access (See Clause 3.4.34 of GSMA NG.116 [50]).

#### 6.3.49.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dataNetworkName | M | T | T | F | T |
| localBreakoutAllowed | M | T | T | F | T |

#### 6.3.49.3	Attribute constraints

None.

#### 6.3.49.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ServiceProfile.availability | This parameter specifies the communication service availability requirement, expressed as a percentage. This parameter is applicable for an end-to-end communication service provided by a network slice. The communication service availability is defined in clause 3.1 of TS 22.261 [28]. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the network slice or the network slice subnet. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the network slice or the network slice subnet. It describes the permission to use or prohibition against using the managed object instance, imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: NsInfo multiplicity: 0..1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nsInstanceId | This attribute specifies the identifier of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nsName | This attribute specifies the name of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| description | This attribute specifies the description of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| category | This attribute specifies the category of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: CHARACTER, SCALABILITY | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tagging | This attribute specifies the tagging of a service requirement/attribute of GST in character category (see GSMA NG.116 [50]). allowedValues: PERFORMANCE, FUNCTION, OPERATION | type: ENUM multiplicity: 1…3 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| exposure | This attribute specifies exposure mode of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: API, KPI | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice or network slice subnet instance. | type: MaxNumberofUEs multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MaxNumberofUE.3GPPNoOfUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice or network slice subnet instance on 3GPP access. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MaxNumberofUE.non3GPPNoOfUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice or network slice subnet instance on non-3GPP access. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageAreaTAList | An attribute specifies a list of Tracking Areas for the network slice. TAI uniquely identifies a Tracking Area. TAI is defined in clause 9.3.3.11 of TS 38.413 [5]. allowedValues: N/A | type: Tai multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dLLatency | An attribute specifies the required DL packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLLatency | An attribute specifies the required UL packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TopSliceSubnetProfile.dLLatency | An attribute specifies the required DL packet transmission latency (millisecond) through all domains of the network slice and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TopSliceSubnetProfile.uLLatency | An attribute specifies the required UL packet transmission latency (millisecond) through all domains of the network slice and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile.dLLatency | An attribute specifies the required DL packet transmission latency (millisecond) through CN domain of the network slice and is used to evaluate the delay in CN domain, e.g. time between received DL packet on N6 interface of UPF and successfully sent out the packet on N3 interface. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile.uLLatency | An attribute specifies the required UL packet transmission latency (millisecond) through CN domain of the network slice and is used to evaluate the delay in CN domain, e.g. time between received UL packet on N3 interface of UPF and successfully sent out the packet on N6 interface. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.dLLatency | An attribute specifies the required DL packet transmission latency (millisecond) in RAN including the air interface of the network slice and is used to evaluate the delay between NR-RAN and UE, e.g. time between received DL packet from UPF and the packet successfully received by UE. See clause 5.1.1.1.6 in TS 28.552 [69]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.uLLatency | An attribute specifies the required UL packet transmission latency (millisecond) in RAN including the air interface of the network slice and is used to evaluate the delay between UE and NG-RAN, e.g. time between the UL packet transmitted by UE and the packet transmitted by gNB to UPF. See clause 5.1.1.1.7 in TS 28.552 [69]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice. See 6.2.1 of TS 22.261 [28]. allowedValues: STATIONARY, NOMADIC, RESTRICTED_MOBILITY, FULL_MOBILITY. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceSharingIndicator | The attribute specifies whether a service, defined by the ServiceProfile, can share a NetworkSlice instance with other services or not. If “non-shared” the service needs a dedicated NetworkSlice instance. If “shared” the service may share a NetworkSlice instance with other service(s). allowedValues: SHARED, NON_SHARED. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceProfile.pLMNInfoList | It defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sliceProfile.pLMNInfoList | It defines which PLMN and S-NSSAI combinations that are served by the SliceProfile in case of network slicing feature is supported. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sliceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice subnet may be shared with another network slice subnet(s). allowedValues: SHARED, NON_SHARED. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice | type: ServiceProfile multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet. All members of the list, instances of SliceProfile, shall contain the same datatype representing slice profile requirements: TopSliceSubnetProfile, RANSliceSubnetProfile or CNSliceSubnetProfile. E.g. the sliceProfileList may contain only instances of sliceProfile containing RANSliceSubnetProfile datatype; the sliceProfileList may not contain instances of sliceProfile containing RANSliceSubnetProfile and CNSliceSubnetProfile datatypes Members of the list may contain TopSliceSubnetProfile datatype only when this attribute (sliceProfileList) belongs to a NetworkSliceSubnet that is directly referenced by a NetworkSlice | type: SliceProfile multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sST | This attribute specifies the slice/service type in a ServiceProfile to be supported by a network slice. See standardised SST values in clause 5.15.2 of TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| delayTolerance | An attribute specifies the properties of service delivery flexibility, especially for the vertical services that are not chasing a high system performance. See clause 4.3 of TS 22.104 [51]. | type: DelayTolerance multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DelayTolerance.support | An attribute specifies whether or not the network slice supports service delivery flexibility, especially for the vertical services that are not chasing a high system performance. allowedValues: "NOT_SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dLDeterministicComm | An attribute specifies the properties of the deterministic communication in downlink for periodic user traffic, see clause 4.3 of TS 22.104 [51]. | type: DeterministicComm multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLDeterministicComm | An attribute specifies the properties of the deterministic communication in uplink for periodic user traffic, see clause 4.3 of TS 22.104 [51]. | type: DeterministicComm multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DeterministicComm.availability | An attribute specifies whether or not the network slice supports deterministic communication for period user traffic. allowedValues: "NOT_SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DeterministicComm.periodicityList | An attribute specifies a list of periodicities supported by the network slice for deterministic communication. Each instance of periodicity is expressed in seconds, refer to NG.116 [50]. | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dLThptPerSlice | This attribute defines achievable data rate of the network slice in downlink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dLThptPerSliceSubnet | This attribute defines required data rate of the network slice subnet in downlink that should be available ubiquitously across the coverage area of the slice. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| guaThpt | This attribute describes the guaranteed data rate. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxThpt | This attribute describes the maximum data rate. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLThptPerSlice | This attribute defines achievable data rate of the network slice in uplink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLThptPerSliceSubnet | This attribute defines required data rate of the network slice subnet in uplink that should be available ubiquitously across the coverage area of the slice. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dLMaxPktSize | This parameter specifies the maximum packet size supported by the network slice or the network slice subnet, in downlink refer NG.116 [50]. | type: MaxPktSize multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLMaxPktSize | This parameter specifies the maximum packet size supported by the network slice or the network slice subnet in uplink, refer NG.116 [50]. | type: MaxPktSize multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MaxPktSize.maxsize | This parameter specifies the maximum packet size supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxNumberofPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice on 3GPP access type, refer NG.116 [50]. | type: MaxNumberofPDUSessions multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MaxNumberofPDUSessions.3GPPNoOfPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MaxNumberofPDUSessions.non3GPPNoOfPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice on non 3GPP access type, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceProfile.kPIMonitoring | An attribute specifies the name list of KQIs and KPIs, related to the network slice available for performance monitoring. | type: KPIMonitoring multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| KPIMonitoring.kPIList | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nBIoT | An attribute specifies whether NB-IoT is supported in the RAN in the network slice, see NG.116 [50]. | type: NBIoT multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NBIoT.support | An attribute specifies whether NB-IoT is supported in the RAN in the network slice, see NG.116 [50]. allowedValues: "NOT_SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| synchronicity | An attribute specifies whether synchronicity of communication devices is supported, Two cases are most important in this context, see clause 3.4.29 of NG.116 [50]: - Synchronicity between a base station and a mobile device and - Synchronicity between mobile devices. | type: Synchronicity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Synchronicity.availability | An attribute specifies whether synchronicity of communication devices is supported, see NG.116 [50]. allowedValues: "NOT_SUPPORTED", "BETWEEN_BS_AND_UE", "BETWEEN_BS_AND_UE_AND_UE_AND_UE". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Synchronicity.accuracy | An attribute specifies the accuracy of the synchronicity, see NG.116 [50]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.synchronicity | An attribute specifies whether synchronicity of communication devices is supported in the RAN domain, Two cases are most important in this context, see clause 3.4.29 of NG.116 [50]: - Synchronicity between a base station and a mobile device and - Synchronicity between mobile devices. | type: Synchronicity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userMgmtOpen | An attribute specifies whether or not the network slice supports the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. | type: UserMgmtOpen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| UserMgmtOpen.support | An attribute specifies whether or not the network slice supports the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. allowedValues: "NOT_SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| v2XCommModels | An attribute specifies whether or not the V2X communication mode is supported by the network slice. | type: V2XCommMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| V2XCommMode.v2XMode | An attribute specifies whether or not the V2X communication mode is supported by the network slice. allowedValues: "NOT_SUPPORTED", "SUPPORTED_BY_NR". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageArea | An attribute specifies the coverage area of the network slice, i.e. the geographic region where a 3GPP communication service is accessible, see Table 7.1-1 of TS 22.261 [28]) and NG.116 [50]. | type: GeoArea multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| termDensity | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: TermDensity multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TermDensity.density | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| positioning | An attribute specifies whether the network slice provides geo-localization methods or supporting methods, see clause 3.4.20 of NG.116 [50]. | type: Positioning multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.positioning | An attribute specifies whether the RAN domain of the network slice provides geo-localization methods or supporting methods, see clause 3.4.20 of NG.116 [50]. | type: Positioning multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Positioning.availability | An attribute specifies if this attribute is provided by the RAN domain of the network slice and contains a list of positioning methods provided by the RAN domain. If the list is empty this attribute is not available in the RAN domain and the other parameters might be ignored, see NG.116 [50]. Comma separated multiple values are allowed: CIDE_CID, OTDOA, RF_FINGERPRINTING, AECID, HYBRID_POSITIONING, NET_RTK. | type: ENUM multiplicity: 1..6 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| Positioning.predictionFrequency | An attribute specifies how often location information is provided. This parameter simply defines how often the customer is allowed to request location information. This is not related to the time it takes to determine the location, which is a characteristic of the positioning method, see NG.116 [50]. allowedValues: "PERSEC", "PERMIN", "PERHOUR". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Positioning.accuracy | An attribute specifies the accuracy of the location information. Accuracy depends on the respective positioning solution applied in the RAN domain of the network slice, measurement unit is meter, see NG.116 [50]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| activityFactor | An attribute specifies the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. See Table 7.1-1 of TS 22.261 [28]). | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uESpeed | An attribute specifies the maximum speed (in km/hour) supported by the network slice or network slice subnet at which a defined QoS can be achieved. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceProfile.dLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in DL packet spacing at the receiver compared to the sender for a pair of packets through the RAN, CN and TN part of an end-to-end network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceProfile.uLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in UL packet spacing at the receiver compared to the sender for a pair of packets through the RAN, CN and TN part of an end-to-end network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TopSliceSubnetProfile.dLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in DL packet spacing at the receiver compared to the sender for a pair of packets through the RAN, CN and TN part of an end-to-end network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TopSliceSubnetProfile.uLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in UL packet spacing at the receiver compared to the sender for a pair of packets through the RAN, CN and TN part of an end-to-end network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile.dLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in DL packet spacing at the receiver compared to the sender for a pair of packets through CN domain of the network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile. uLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in UL packet spacing at the receiver compared to the sender for a pair of packets through CN domain of the network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.dLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in DL packet spacing at the receiver compared to the sender for a pair of packets through RAN domain of the network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.uLPktDelayVariation | An attribute specifies the maximum allowed deviation (millisecond) of the difference in UL packet spacing at the receiver compared to the sender for a pair of packets through RAN domain of the network slice. How to measure inter-packet delay variation is documented by IETF in RFC 3393 [102] as measuring the IP packet delay variation and its applicability to use it as metric is documented in Section 4.1 of RFC 5481 [103]. The deviation is also defined in Annex C.4.4 of TS 22.104 [51] | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| survivalTime | An attribute specifies the time (millisecond) that an application consuming a communication service may continue without an anticipated message. See clause 5 of TS 22.104 [51]). | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dLReliability | An attribute specifies in the context of network layer DL packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uLReliability | An attribute specifies in the context of network layer UL packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSlice.networkSliceSubnetRef | This holds a DN of NetworkSliceSubnet relating to the NetworkSlice instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnet.networkSliceSubnetRef | This holds a list of DN of constituent NetworkSliceSubnet supporting NetworkSliceSubnet instance | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| managedFunctionRef | This holds a list of DN of ManagedFunction instances supporting the NetworkSliceSubnet instance. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ipAddress | This parameter specifies the IP address assigned to a logical transport interface/endpoint which is part of a RAN or CN SubNetwork. It can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). See note 1 | type: IpAddress multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localLogicalInterfaceInfo | This parameter specifies the information of a local logical transport interface. | type: LogicalInterfaceInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicalInterfaceType | This parameter specifies the type of a logical transport interface. It could be VLAN, MPLS or SEGMENT. allowedValues: VLAN,MPLS, SEGMENT | type:ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicalInterfaceId | This parameter specifies the identify of a logical transport interface which is part of a RAN or CN SubNetwork. It could be VLAN ID (See IEEE 802.1Q [39]), MPLS Tag or Segment ID. In case logical transport interface is VLAN, it is VLAN Id (See IEEE 802.1Q [39]). In case logical transport interface is MPLS, it is MPLS Tag. In case logical transport interface is Segment, it is Segment ID. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| externalEndPointRefList | This parameter is used to identify a list of connection point info(s). | Type: ConnectionPointInfo multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |
| connectionPointId | This parameter specifies the identifier of a TN object. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| connectionPointIdType | This parameter specifies the type of the connection point identifier. allowedValues: VLAN, MPLS, SEGMENT, IPV4, IPV6, ATTACHMENT_CIRCUIT | type:ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LogicalInterfaceInfo.systemName | This parameter specifies the identifier for a system. | Type: String Multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LogicalInterfaceInfo.portName | This parameter specifies the identifier for a port. | Type: String Multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LogicalInterfaceInfo.routingProtocol | This parameter specifies the Routing protocol. allowedValues: RIP, IGMP, OSPF, EGP, EIGRP, BGP, IS-IS, STATIC | type:ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qosProfile | This parameter specifies the QoS Profile for a logical transport interface. A QoS profile includes a set of parameters which are locally provisioned on both sides of a logical transport interface. An example of the parameter value could be “DSCP” (See RFC 8436 [74]). | type: String multiplicity: 0..1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| maxDLDataVolume | An attribute specifies the maximum DL PDCP data volume supported by the network slice instance (performance measurement definition see in TS 28.552 [69]). The unit is MByte/day. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxULDataVolume | An attribute specifies the maximum UL PDCP data volume supported by the network slice instance (performance measurement definition see in TS 28.552 [69]). The unit is MByte/day. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| radioSpectrum | This attribute represents the radio spectrum in which the network slice should be supported (see clause 3.4.21 of GSMA NG.116 [50]). | type: RadioSpectrum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nROperatingBands | This attribute represents which 5G NR frequency bands can be used to access the network slice. 5G NR operating bands are defined in 3GPP TS 38.101-1 [42]. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| epApplicationRef | This parameter specifies a list of application level EPs (i.e. EP_N3 or EP_NgU or EP_F1U) associated with the logical transport interface. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| epTransportRef | This parameter specifies a list of transport level EPs associated with the application level EP (i.e. EP_N3 or EP_NgU or EP_F1U) or network slice subnet. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sliceSimultaneousUse | This attribute describes whether a network slice can be simultaneously used by a device together with other network slices and if so, with which other classes of network slices. allowedValues: “0”, “1”, “2”, “3”, “4”. “0”: Can be used with any network slice “1”: Can be used with network slices with same SST value “2”: Can be used with any network slice with same SD value “3”: Cannot be used with another network slice “4”: Cannot be used by a UE in a specific location | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energyEfficiency | An attribute which describes the energy efficiency of a network slice, i.e. the ratio between the performance of a network slice and its energy consumption (EC) when assessed during the same time frame, see clause 3.4.7 of NG.116 [50]. | type: EnergyEfficiency multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergyEfficiency.performance | Depending on the sST value, EnergyEfficiency.performance will be - eMBBEEPerfReq or - uRLLCEEPerfReq or - mIoTEEPerfReq allowedValues: - eMBBEEPerfReq identifies the requirement in terms of energy efficiency, i.e. the performance per consumed Joule in type Real, where performance can take one of the following forms (type: ENUM): - number of bits (Integer) (see TS 28.554 [27] clause 6.7.2.2). - number of bits (Integer) for RAN-based network slice (see TS 28.554 [27] clause 6.7.2.2a). - uRLLCEEPerfReq identifies the requirement in terms of energy efficiency, i.e. the performance per consumed Joule in type Real, where performance can take one of the following forms (type: ENUM): - inverse of the latency in 0.1ms (Real) (see TS 28.554 [27] clause 6.7.2.3.2). - number of bits multiplied by the inverse of the latency in 0.1ms (Real) (see TS 28.554 [27] clause 6.7.2.3.3). - mIoTEEPerfReq identifies the requirement in terms of energy efficiency, i.e. the performance per consumed Joule in type Real, where performance can take one of the following forms (type: ENUM): - maximum number of registered subscribers (Integer) (see TS 28.554 [27] clause 6.7.2.4.1), - mean number of active UEs (Integer) (see TS 28.554 [27] clause 6.7.2.4.2). See NOTE 3. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TopSliceSubnetProfile.energyEfficiency | An attribute which describes the energy efficiency through all domains of the network slice, i.e. the ratio between the performance and the energy consumption (EC) when assessed during the same time frame, see clause 3.4.7 of NG.116 [50]. | type: EnergyEfficiency multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile.energyEfficiency | An attribute which describes the energy efficiency through CN domain of the network slice, i.e. the ratio between the performance and the energy consumption (EC) when assessed during the same time frame, see clause 3.4.7 of NG.116 [50]. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.energyEfficiency | An attribute which describes the energy efficiency through RAN domain of the network slice, i.e. the ratio between the performance and the energy consumption (EC) when assessed during the same time frame, see clause 3.4.7 of NG.116 [50]. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nssaaSupport | An attribute specifies whether for the Network Slice, devices need to be also authenticated and authorized by a AAA server using additional credentials different than the ones used for the primary authentication, see clause 3.4.37 of NG.116 [50]. allowedValues: N/A | type: NSSAASupport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nssaaSupport.support | An attribute specifies whether or not the Network Slice, devices need to be also authenticated and authorized by a AAA server using additional credentials different than the ones used for the primary authentication. allowedValues: "NOT_SUPPORTED", "SUPPORTED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceProfile.n6Protection | An attribute which includes required security functions and corresponding rules of each function for network slice N6 interface protection. allowedValues: N/A | type: N6Protection multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile.n6Protection | An attribute which includes required security functions and corresponding rules of each function for network slice N6 interface protection. allowedValues: N/A | type: N6Protection multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secFuncList | An attribute which holds the list of security control functions/features required by the Network Slice or Network Slice Subnet consumer. allowedValues: N/A | type: SecFunc multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| secFunId | An attribute which identifies a security function. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secFunType | An attribute which describes the type of the security function. E.g. Firewall, NAT, antimalware, parental control, DDoS protection function, etc. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secRules | An attribute which could be configured on each function. If it's absent, the default rules could be applied. allowedValues: N/A | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkSliceSubnetType | An attribute indicating type of network slice subnet, including: - Top network slice subnet - RAN network slice subnet - CN network slice subnet allowedValued: TOP_SLICESUBNET,RAN_SLICESUBNET,CN_SLICESUBNET | type:ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priorityLabel | An attribute specifies a label that consumer would assign a value on an instance of network slice subnet. The management system takes the value of this attribute into account. The effect of this attribute value to the subject managed entity is not standardized allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetProviderCapabilities.dLLatency | This attribute specifies the achievable packet transmission latency in downlink (millisecond) through the network slice subnet. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetProviderCapabilities.uLLatency | This attribute specifies the achievable packet transmission latency in uplink (millisecond) through the network slice subnet. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetProviderCapabilities.dLThptPerSliceSubnet | This attribute defines achievable data rate of the network slice subnet in downlink that is available ubiquitously across the coverage area of the slice. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetProviderCapabilities.uLThptPerSliceSubnet | This attribute defines achievable data rate of the network slice subnet in uplink that is available ubiquitously across the coverage area of the slice. | type: XLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetProviderCapabilities.coverageAreaTAList | An attribute specifies a list of Tracking Areas that a network slice subnet can serve. TAI uniquely identifies a Tracking Area. TAI is defined in clause 9.3.3.11 of TS 38.413 [5]. allowedValues: N/A | type: Tai multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| processMonitor | An attribute describes the process monitoring information of the feasibility check and reservation job. See corresponding processMonitor definition in TS 28.622[30]. | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityResult | An attribute which specifies the feasibility check result for the feasibility check and reservation job. This attribute is configured by MnS producer and can be read by MnS consumer. The feasibilityResult is configured once the "status" is "FINISHED" allowedValues: FEASIBLE: which means the specified network slicing related requirements (i.e. ServiceProfile, SliceProfile) can be satisfied by the MnS producer. INFEASIBLE: which means the specified network slicing related requirements (i.e. ServiceProfile, SliceProfile) cannot be satisfied by the MnS producer. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inFeasibleReason | An attribute that specifies the additional reason information if the feasibility check result is infeasible. This attribute can be absent if the feasibility check result is feasibile. allowedValues: the detailed content (ENUM Value) for the inFeasibleReason is not defined in the present document. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| resourceReservation | An attribute represents MnS consumer's requirements for resource reservation. allowedValues: TRUE: MnS producer need to reserve corresponding resources FALSE: no guarantee for the corresponding resources. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| recommendationRequest | An attribute represents MnS consumer's request for recommended network slice related requirements allowedValues: TRUE: MnS producer need to derive and provide the recommended network slicing related requirements FALSE: no guarantee for derive and provide the recommended network slicing related requirements. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| requestedReservationExpiration | An attribute which specifies MnS consumer's requirememts for the validity period of the resource reservation. The value of requestedReservationExpiration is specified by MnS consumer. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationExpiration | An attribute which specifies the actual validity period of the resource reservation. After the period expires, no guarantees are given for the resources associated to the corresponding network slicing related requirements (i.e. ServiceProfile, SliceProfile). which is specified by MnS producer based on requested reservation expiration from MnS consumer and its own reservation capabilities. In case MnS produer have the enpugh capability to satisfy MnS consumer's reservation requirememts, the value of reservationExpiration is same as requestedReservationExpiration. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| resourceReservationStatus | An attribute which specifies the resource reservation result for the feasibility check and reservation job. This attribute is configured by MnS producer and can be read by MnS consumer. allowedValues: RESERVED: which means the resources for the specified network slicing related requirements (i.e. ServiceProfile, SliceProfile) is reserved. UNRESERVED: which means the resources for the specified network slicing related requirements (i.e. ServiceProfile, SliceProfile) is not reserved. USED: which means the reserved resource for the specified network slicing related requirements is used. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedRequirements | An attribute which specifies the recommended network slicing related requirements (i.e. ServiceProfile and SliceProfile information) which can be supported by the MnS producer. This information is provided when the feasibility check result is infeasible. This information can be used by MnS consumer to adjust the network slicing related requirements. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationFailureReason | An attribute that specifies the additional reason information if the reservation is failed. This attribute can be absent if the reservation is successful. allowedValues: the detailed content (ENUM Value) for the reservationFailureReason is not defined in the present document. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FeasibilityCheckAndReservationJob.serviceProfile | An attribute specifies the network slice related requirements for the feasibility check and resource reservation job | type: ServiceProfile multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FeasibilityCheckAndReservationJob.sliceProfile | An attribute specifies the network slice subnet related requirements for the feasibility check and resource reservation job | type: SliceProfile multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityTimeWindow | An attribute represents MnS consumer's request for checking whether the network slicing related requirements (i.e. ServiceProfile and SliceProfile information) can be satisfied at a specified time window. If this attribute specified by MnS consumer, MnS producer determine whether the network slicing related requirements can be satisfied at the specified time window and reserve corresponding resources at the specified time window if resourceReservation is “TRUE”. | type: TimeWindow multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSlice.networkSliceControllerRef | This holds a list of DN of NetworkSliceController supported by the NetworkSlice MOI. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NetworkSliceSubnet.networkSliceSubnetControllerRef | This holds a list of DN of NetworkSliceSubnetController supported by the NetworkSliceSubnet MOI. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NetworkSliceController.inputServiceProfile | This attribute specifies the input network slice related requirements provided by the MnS consumer. | type: ServiceProfile multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceController.serviceProfileId | This attribute specifies the service profile identifier provided by the MnS producer for the network slice related requirements specified in inputServiceProfile attribute or as specified as part of AllocateNsi operation (defined in TS 28.531 [26]). | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceController.processMonitor | This attribute describes the process monitoring information of the fulfilment of the network slice life cycle management. | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceController.networkSliceRef | This attribute specifies the DN of the NetworkSlice MOI, that the MnS producer has selected to fulfil the network slice related requirements specified in inputServiceProfile attribute or as specified as part of AllocateNsi operation (defined in TS 28.531 [26]). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetController. inputSliceProfile | This attribute specifies the network slice subnet related requirements. | type: SliceProfile multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetController.sliceProfileId | This attribute specifies the service profile identifier provided by the MnS producer for the network slice subnet related requirements specified in inputSliceProfile attribute or as specified as part of AllocateNssi operation (defined in TS 28.531 [26]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetController.processMonitor | This attribute describes the process monitoring information of the fulfilment of the network slice subnet life cycle management. | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnetController.networkSliceSubnetRef | This attribute specifies the DN of the NetworkSliceSubnet MOI, that the MnS producer has selected to fulfil the network slice subnet related requirements specified in inputSliceProfile attribute or as specified as part of AllocateNssi operation (defined in TS 28.531 [26]). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availabilityStatus | It indicates the availability status of the fulfilment of network slice or the network slice subnet related requirements by the MnS producer. allowedValues: "IN_TEST", "FAILED", "POWER_OFF", "OFF_LINE ", "OFF_DUTY", "DEPENDENCY", "DEGRADED", "NOT_INSTALLED", "LOG_FULL". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nonIPSupport | An attribute specifies the non-IP Session support (Ethernet session and forwarding support) of the slice or slice subnet. See Clause 3.4.27 of GSMA NG.116 [50]. | type: NonIPSupport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NonIPSupport.support | An attribute specifies the non-IP Session support (Ethernet session and forwarding support) of the slice or slice subnet. See Clause 3.4.27 of GSMA NG.116 [50]. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSlice. isolationProfileRef | This holds the DN of IsolationProfile MOI representing the isolation requirements applied for the NetworkSlice MOI. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: False isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnet. isolationProfileRef | This holds the DN of IsolationProfile MOI representing the isolation requirements applied for the NetworkSliceSubnet MOI. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: False isUnique: N/A defaultValue: None isNullable: False |
| IsolationProfileRef.networkSliceSubnetRefList | This holds a list of DN of NetworkSliceSubnet MOI to which the isolation is applicable. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| IsolationProfileRef.networkSliceRefList | This holds a list of DN of NetworkSlice MOI to which the isolation is applicable. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| IsolationProfileRef.resourceIsolationRuleList | An attribute which describes a set of isolation rules for the managed resources. | type: ResourceIsolationRule multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| resourceType | An attribute which describes the managed resource type for isolation. MANAGED_FUNCTION: Indicates the managed function instances are selected using the the isolation rule which is specified by isolationRule attribute. NETWORK_SERVICE: Indicates the Network Service (NS) information are selected using the isolation rule which is specified by isolationRule attribute. allowedValues: “MANAGED_FUNCTION”, “NETWORK_SERVICE” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isolationRule | An attribute which describes the isolation requirement. DEDICATED: The network slices are provided with exclusive network resources, preventing any interference between other network slices. SHARED: Allows the network slice resources to be shared with other network slices. allowedValues: DEDICATED, SHARED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSlicingApplicability | An attribute which describes if the isolation requirements are associated with NetworkSlice(s) or NetworkSliceSubnet(s). allowedValues: NETWORKSLICE, NETWORKSLICESUBNET | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TopSliceSubnetProfile.availability | An attribute specifies the required communication service availability (percentage) through the RAN, CN, and TN part of an end-to-end network slice. See clause 3.1 of TS 22.261 [28]. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CNSliceSubnetProfile.availability | An attribute specifies the required communication service availability (percentage) through the CN domain of a network slice, i.e., CN slice subnet. The percentage value of the amount of time the CN slice subnet is delivered according to all the slice subnet related requirements listing in the CNSliceSubnetProfile, divided by the amount of time the system is expected to deliver the CN slice subnet. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RANSliceSubnetProfile.availability | An attribute specifies the required communication service availability (percentage) through the RAN domain of a network slice, i.e., RAN slice subnet. The percentage value of the amount of time the RAN slice subnet is delivered according to all the slice subnet related requirements listing in the RANSliceSubnetProfile, divided by the amount of time the system is expected to deliver the RAN slice subnet. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| TopSliceSubnetProfile.kPIMonitoring | An attribute specifies the name list of KPIs, related to all domains of the network slice, available for performance monitoring. | type: KPIMonitoring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| RANSliceSubnetProfile.kPIMonitoring | An attribute specifies the name list of KPIs, related to the RAN domain network slice subnet, available for performance monitoring. | type: KPIMonitoring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| supportedDataNetworks | An attribute specifies the supported data network of the slice or slice subnet. See Clause 3.4.39 of GSMA NG.116 [50]. | type: DataNetwork multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DataNetwork.dnnList | An attribute specifies the list of supported data network (DNN, see ) of the slice or slice subnet. See Clause 3.4.39 of GSMA NG.116 [50]. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataNetworkAccess | An attribute specifies how the supported data networks handle the user data of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: DataNetworkAccess multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DataNetworkAccess.dataAccessList | An attribute specifies Data access per data network for the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: DataAccess multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DataNetworkAccess.tunnellingMechanismList | An attribute specifies Tunnelling mechanism per data network for the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: TunnellingMechanism multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DataNetworkAccess.localBreakoutAllowedList | An attribute specifies whether a data network is available in Local Breakout while roaming for the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: LboAllowed multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DataAccess.dataNetworkName | An attribute specifies data network name of the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DataAccess.dataAccessUsed | An attribute specifies Data access per data network for the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. "DIRECT_INTERNET_ACCESS": Direct access to the Internet "TERM_PVT_NETWORK": Termination in a private network (e.g., via tunnelling mechanism such as L2TP, VPN Virtual Private Network, tunnel, etc.) "LOCAL_TRAFFIC": All data traffic stays local to an operator network and the devices do not have access to the Internet or private network. allowedValues: "DIRECT_INTERNET_ACCESS", "TERM_PVT_NETWORK", "LOCAL_TRAFFIC". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TunnellingMechanism.dataNetworkName | An attribute specifies data network name of the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TunnellingMechanism.tunellingMechanismUsed | An attribute specifies Tunnelling mechanism for the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. allowedValues: "L2TP_TUNNEL", "GRE_TUNNEL", "VPN_TUNNEL", "LABEL_BASED_ROUTING", “802.1Q_VLAN”, “SRV6”, "OTHER". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LboAllowed.dataNetworkName | An attribute specifies data network name of the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| LboAllowed.localBreakoutAllowed | An attribute specifies whether a data network is available in Local Breakout while roaming for the supported data networks of the slice or slice subnet. See Clause 3.4.34 of GSMA NG.116 [50]. allowedValues: "YES", "NO". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceProfile.sliceAvailability | This attribute provides information about the time at which the slice or slice subnet instance is scheduled to be available. | type: SchedulingTime multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| CNSliceSubnetProfile.sliceSubnetAvailability | This attribute provides information about the time at which the slice or slice subnet instance is scheduled to be available. | type: SchedulingTime multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TopSliceSubnetProfile.sliceSubnetAvailability | This attribute provides information about the time at which the slice or slice subnet instance is scheduled to be available. | type: SchedulingTime multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: There is no direct relationship between localAddress/remoteAddress in EP_RP and ipAddress in EP_transport. While the localAddress/remoteAddress in EP_RP could be exchanged as part of signalling between GTP-u tunnel end points, ipAddress in EP_transport is used for transport routing. NOTE 2: void NOTE 3: energy efficiency requirement for V2X is not part of the current document. |  |  |

## 6.5	Common notifications

### 6.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.111 [109], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | O | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

Note:	notifyChangedAlarm is deprecated in TS 28.111 [109].

### 6.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyMOIChanges | O | -- |
| notifyEvent | O | -- |

### 6.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

# 7	Solution Set (SS)

## 7.1	OpenAPI Definitions of NR and NG-RAN NRM

The OpenAPI/YAML definitions are specified in 3GPP Forge, refer to clause 4.3 of TS 28.623 [106] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA104/".

Directory: OpenAPI

File:  TS28541_NrNrm.yaml

## 7.2	OpenAPI Definitions of 5GC NRM

The OpenAPI/YAML definitions are specified in 3GPP Forge, refer to clause 4.3 of TS 28.623 [106] for the Forge location.

Directory: OpenAPI

File: TS28541_5GcNrm.yaml

## 7.3	OpenAPI Definitions of slice and network slice subnet NRM

The OpenAPI/YAML definitions are specified in 3GPP Forge, refer to clause 4.3 of TS 28.623 [106] for the Forge location.

Directory: OpenAPI

File: TS28541_SliceNrm.yaml

## 7.4	YANG Definitions for NR and NG-RAN

YANG definitions are specified in 3GPP Forge, refer to clause 4.4 of TS 28.623 [106] for the Forge location.

Directory: yang-models

Files:

_3gpp-common-antennafunction.yang

_3gpp-common-sectorequipmentfunction.yang

_3gpp-nr-nrm-beam.yang

_3gpp-nr-nrm-bwp.yang

_3gpp-nr-nrm-bwpset.yang

_3gpp-nr-nrm-cco.yang

_3gpp-nr-nrm-cesmanagementfunction.yang

_3gpp-nr-nrm-commonbeamformingfunction.yang

_3gpp-nr-nrm-cpciconfigurationfunction.yang

_3gpp-nr-nrm-danrmanagementfunction.yang

_3gpp-nr-nrm-desmanagementfunction.yang

_3gpp-nr-nrm-dlbofunction.yang

_3gpp-nr-nrm-dmrofunction.yang

_3gpp-nr-nrm-dpciconfigurationfunction.yang

_3gpp-nr-nrm-drachoptimizationfunction.yang

_3gpp-nr-nrm-ecmappingrule.yang

_3gpp-nr-nrm-ep.yang

_3gpp-nr-nrm-eutrancellrelation.yang

_3gpp-nr-nrm-eutranetwork.yang

_3gpp-nr-nrm-eutranfreqrelation.yang

_3gpp-nr-nrm-eutranfrequency.yang

_3gpp-nr-nrm-externalamffunction.yang

_3gpp-nr-nrm-externalenbfunction.yang

_3gpp-nr-nrm-externaleutrancell.yang

_3gpp-nr-nrm-externalgnbcucpfunction.yang

_3gpp-nr-nrm-externalgnbcuupfunction.yang

_3gpp-nr-nrm-externalgnbdufunction.yang

_3gpp-nr-nrm-externalnrcellcu.yang

_3gpp-nr-nrm-externalservinggwfunction.yang

_3gpp-nr-nrm-externalupffunction.yang

_3gpp-nr-nrm-gnbcucpfunction.yang

_3gpp-nr-nrm-gnbcuupfunction.yang

_3gpp-nr-nrm-gnbdufunction.yang

_3gpp-nr-nrm-nrcellcu.yang

_3gpp-nr-nrm-nrcelldu.yang

_3gpp-nr-nrm-nrcellrelation.yang

_3gpp-nr-nrm-nrfreqrelation.yang

_3gpp-nr-nrm-nrfrequency.yang

_3gpp-nr-nrm-nrnetwork.yang

_3gpp-nr-nrm-nroperatorcelldu.yang

_3gpp-nr-nrm-nrsectorcarrier.yang

_3gpp-nr-nrm-ntnfunction.yang

_3gpp-nr-nrm-operatordu.yang

_3gpp-nr-nrm-rimrsset.yang

_3gpp-nr-nrm-rrmpolicy.yang

Mount information

If the class ManagedElement and the underlying hierarchy is contained under a SubNetwork, the YANG module for ManagedElement shall be mounted at the mountpoint "children-of-SubNetwork" in the YANG module _3gpp-common-subnetwork, together with the YANG modules containing IOCs that can be contained under the ManagedElement directly or under other IOCs contained by the ManagedElement.

If the class ManagedElement and the underlying hierarchy is contained under a MeContext, the YANG module for ManagedElement shall be mounted at the mountpoint "children-of-MeContext" in the YANG module _3gpp-common-mecontext, together with the YANG modules containing IOCs that can be contained under the ManagedElement directly or under other IOCs contained by the ManagedElement.

See IETF RFC 8528 [45] that describes the mechanism that adds the schema trees defined by a set of YANG modules onto a mount point defined in the schema tree in another YANG module.

## 7.5	YANG Definitions for 5GC

YANG definitions are specified in 3GPP Forge, refer to clause 4.4 of TS 28.623 [106] for the Forge location.

Directory: yang-models

Files:

_3gpp-5gc-nrm-affunction.yang

_3gpp-5gc-nrm-amffunction.yang

_3gpp-5gc-nrm-amfregion.yang

_3gpp-5gc-nrm-amfset.yang

_3gpp-5gc-nrm-anlffunction.yang_3gpp-5gc-nrm-ausffunction.yang

_3gpp-5gc-nrm-configurable5qiset.yang

_3gpp-5gc-nrm-dnfunction.yang

_3gpp-5gc-nrm-dynamic5qiset.yang

_3gpp-5gc-nrm-ecmconnectioninfo.yang

_3gpp-5gc-nrm-ep.yang

_3gpp-5gc-nrm-externalnrffunction.yang

_3gpp-5gc-nrm-externalnssffunction.yang

_3gpp-5gc-nrm-externalseppfunction.yang

_3gpp-5gc-nrm-FiveQiDscpMappingSet.yang

_3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang

_3gpp-5gc-nrm-lmffunction.yang

_3gpp-5gc-nrm-n3iwffunction.yang

_3gpp-5gc-nrm-neffunction.yang

_3gpp-5gc-nrm-nfprofile.yang

_3gpp-5gc-nrm-nfservice.yang

_3gpp-5gc-nrm-ngeirfunction.yang

_3gpp-5gc-nrm-nrffunction.yang

_3gpp-5gc-nrm-nssffunction.yang

_3gpp-5gc-nrm-nwdaffunction.yang

_3gpp-5gc-nrm-pcffunction.yang

_3gpp-5gc-nrm-predefinedpccruleset.yang

_3gpp-5gc-nrm-QFQoSMonitoringControl.yang

_3gpp-5gc-nrm-scpfunction.yang

_3gpp-5gc-nrm-seppfunction.yang

_3gpp-5gc-nrm-smffunction.yang

_3gpp-5gc-nrm-smsffunction.yang

_3gpp-5gc-nrm-udmfunction.yang

_3gpp-5gc-nrm-udrfunction.yang

_3gpp-5gc-nrm-udsffunction.yang

_3gpp-5gc-nrm-upffunction.yang

_3gpp-5g-common-yang-types.yang

Mount information

If the class ManagedElement and the underlying hierarchy is contained under a SubNetwork, the YANG module for ManagedElement shall be mounted at the mountpoint "children-of-SubNetwork" in the YANG module _3gpp-common-subnetwork, together with the YANG modules containing IOCs that can be contained under the ManagedElement directly or under other IOCs contained by the ManagedElement.

If the class ManagedElement and the underlying hierarchy is contained under a MeContext, the YANG module for ManagedElement shall be mounted at the mountpoint "children-of-MeContext" in the YANG module _3gpp-common-mecontext, together with the YANG modules containing IOCs that can be contained under the ManagedElement directly or under other IOCs contained by the ManagedElement.

See IETF RFC 8528 [45] that describes the mechanism that adds the schema trees defined by a set of YANG modules onto a mount point defined in the schema tree in another YANG module.

## 7.6	YANG Definitions for slice and network slice subnet

YANG definitions are specified in 3GPP Forge, refer to clause 4.4 of TS 28.623 [106] for the Forge location.

Directory: yang-models

Files:

_3gpp-ns-nrm-common.yang

_3gpp-ns-nrm-networkslice.yang

_3gpp-ns-nrm-networkslicesubnet.yang

_3gpp-ns-nrm-serviceprofile.yang

_3gpp-ns-nrm-sliceprofile.yang

Annex A (normative):
Cell state handling

# A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"

The administrative state indicates the permission to use or prohibition against using the cell, imposed through the OAM services. The administrative state has three values: "LOCKED", "SHUTTING DOWN" or "UNLOCKED"

The meanings of these values are defined in ITU T Recommendation X.731 [18].

The relation between the administrative state and the "Pre-operation state of the gNB-DU Cell" is defined in clause 8.5 of TS 38.401 [4]. See below an extract from clause 8.5 of TS 38.401 [4] on the F1 startup and cell activation.

If the operationalState is "ENABLED" (i.e. the resource is physically installed and working) and if the administrativeState is "UNLOCKED", the step "0: Pre-operational state" will exit and the step "1: F1 Setup Request" will be executed."

8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

# A.2	Combined state diagram for gNB cell

This is the Combined state diagram for gNB cell.

Figure A.2-1: Combined gNB cell state diagram

The gNB-DU maintains cell states. The following table is the gNB cell state transition table.

In 3-split and 2-split deployment scenarios, the interactions between gNB-CU and gNB-DU are standardized. The interactions specified under the column "The state transition events and actions" of "The gNB Cell state transition table" below shall be present for the state transition.

In the non-split deployment scenarios, the interactions between gNB-CU and gNB-DU are not standardized. The interactions between gNB-CU and gNB-DU specified under the column "The state transition events and actions" of "The gNB Cell state transition table" can be replaced by other means that is not standardized.

Table A.2-1: The gNB Cell state transition table

| Transition number | The state transition event and actions |
| --- | --- |
| 1 | Event: Receive request to unlock. Action: None. |
| 2 | Event: Receive request to lock. Action: None. |
| 2a | Event: Receive request to lock Action: Send to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. |
| 3 | Event: When the required cell resource is physically installed and working. Action: none. |
| 4 | Event: When the required cell resource is not physically installed or is not working. Action: Send to gNB-CU the "gNB-DU Configuration update message" with cell to delete. |
| 4a | Event: When the required cell resource is physically uninstalled or is not working. Action: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 5 | Event: Receive from gNB-CU the "F1 Setup Response message" (identifying the cell to be activated). The cell is activated successfully. Actions: Do nothing or send gNB-CU the "gNB-DU Configuration Update message" with Cell stated as active' ----- or ----- Event: Receive from gNB-CU the "gNB-CU Configuration Update message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message"). Actions: The cell is activated successfully. Send to gNB-CU the "gNB-CU Configuration Update Response" to confirm the cell is in active state. ----- or ----- Event: Receive from gNB-CU the "gNB-DU Configuration Update Acknowledge message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message") and the cell is activated successfully Actions: Do nothing. |
| 6 | Event: Receive from gNB-CU the "gNB-CU Configuration Update message" and responds with gNB-CU Configuration Update Acknowledge messages. Actions: Respond with gNB-CU Configuration Update Acknowledge messages. ----- or ----- Event: Event: DU experiences an internal failure and decided to place the cell into inactive state. Actions: Send to gNB-CU the "gNB-DU Cell status Update message" |
| 7 | Event: Send to gNB-CU the "F1 Setup request" (identifying the cell that is configured and ready to be activated). Actions: none. ----- or ----- Send to gNB-CU the "gNB-DU Configuration Update message" with the served cell to add. Actions: none. |
| 8 | Event: Sends to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. Receive response from gNB-CU the "gNB-DU Configuration Update Acknowledge message". Actions: None. |
| 9 | Event: Receive request to shut down. Actions: None. |
| 10 | Event: Last user quit. Actions: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 11 | Event: When a cell is created and is configured. Actions: None |
| 12 | Event: When a cell is deleted. Action: None. |

Annex B (normative):
NetworkSlice and NetworkSliceSubnet state handling

# B.1	NetworkSlice instance state handling

A NetworkSlice instance is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time, the management system needs to know the state of an NetworkSlice instance.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state and operational state of systems in general.

Figure B.1: Combined NetworkSlice instance state diagram

The interactions specified under the column "The state transition events and actions" of " NetworkSlice instance  state transition table" below shall be present for the state transition.

Table B.1: The NetworkSlice instance state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | Operation allocateNsi results in the creation of a NetworkSlice instance. The administrative state is set to LOCKED and operationalState is set to DISABLED -- or – CM operation creates a NetworkSlice instance. The administrative state is set to LOCKED and operationalState is set to DISABLED |
| 1 | CM operation sets administrative state to UNLOCKED. |
| 2 | CM operation sets administrative state to LOCKED |
| 2a | CM operation sets administrative state to SHUTTING DOWN |
| 2b | The last user of the NetworkSlice instance stops using the NetworkSlice instance |
| 3 | The related NetworkSliceSubnet instance (identified by NetworkSlice.networkSliceSubnetRef) changes state to UNLOCKED and ENABLED. |
| 4 | The related NetworkSliceSubnet instance (identified by NetworkSlice.networkSliceSubnetRef) changes state to LOCKED -- or – The related NetworkSliceSubnet instance (identified by NetworkSlice.networkSliceSubnetRef) changes state to DISABLED |
| 5 | Operation deallocateNsi results in the deletion of NetworkSlice instance -- or – CM operation deletes NetworkSlice instance |

# B.2	State handling of NetworkSliceSubnet instance

A NetworkSliceSubnet instance is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time the management system needs to know the state of an NetworkSliceSubnet instance.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state and operational state of systems in general.

Figure B.2.1: Combined NetworkSliceSubnet instance state diagram

The interactions specified under the column "The state transition events and actions" of "NNetworkSliceSubnet instanceSSI state transition table" below shall be present for the state transition.

Table B.2.1: The NetworkSliceSubnet instance state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | Operation allocateNssi results in the creation of NetworkSliceSubnet instance. The administrative state is set to LOCKED and operationalState is set to DISABLED -- or – CM operation creates NetworkSliceSubnet instance. The administrative state is set to LOCKED and operationalState is set to DISABLED |
| 1 | CM operation sets administrative state to UNLOCKED. |
| 2 | CM operation sets administrative state to LOCKED |
| 2a | CM operation sets administrative state to SHUTTING DOWN |
| 2b | The last user of the NetworkSliceSubnet instance stops using the NetworkSliceSubnet instance |
| 3 | All constituent NetworkSliceSubnet instances (identified by NetworkSliceSubnet.networkSliceSubnetRef) change state to UNLOCKED and ENABLED. |
| 4 | At least one constituent NetworkSliceSubnet instance (identified by NetworkSliceSubnet.networkSliceSubnetRef) changes state to LOCKED -- or – At least one constituent NetworkSliceSubnet instance (identified by NetworkSliceSubnet.networkSliceSubnetRef) changes state to DISABLED |
| 5 | Operation deallocateNssi results in the deletion of NetworkSliceSubnet instance -- or – CM operation deletes NetworkSliceSubnet instance. |

Annex C (normative):
Void

Annex D (normative):
Void

Annex E (normative):
Void

Annex F (normative):
Void

Annex G (normative):
Void

Annex H (normative):
Void

Annex I (normative):
Void

Annex J (normative):
Void

Annex K (normative):
Void

Annex L (normative):
Relation of GSMA GST, ServiceProfile and SliceProfile

# L.1	General

This annex describes the relation between GSMA GST [50] and the ServiceProfile and SliceProfile captured in the network slice NRM fragment (see clause 6).

# L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the NSC (e.g., vertical industry) and the NSP. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST parameters [50] are used as input to ServiceProfile.  The ServiceProfile which defines the service requirements related to a particular NSC, is translated into the SliceProfile. In particular, the attributes captured in the ServiceProfile are mapped to TopSliceSubnetProfile attributes. Based on the TopSliceSubnetProfile attributes, the corresponding requirements for the dedicated domain specific network slice subnets are defined. For example, the CNSliceSubnetProfile attributes are used to carry 5GC domain requirements, the RANSliceSubnetProfile attributes are used to carry NG-RAN domain requirements, and the TN requirements are derived and provide input to the TN domain.

As shown in Table L.2.1 some of the attributes in CNSliceSubnetProfile and RANSliceSubnetProfile parameters can be translated to configurable parameters related to network function behaviour to satisfy SLS of the service in the control plane. While other information (e.g., delay tolerance, deterministic communication support) in CNSliceSubnetProfile and RANSliceSubnetProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice.

The following table show the translation of GST attributes.

Table L.2.1: GST translation

| GST parameters | ServiceProfile attributes | SliceProfile Parameter |  |  | Configuration Parameters |
| --- | --- | --- | --- | --- | --- |
|  |  | TopSlice SubnetProfile attributes | RANSlice SubnetProfile attributes | CNSlice SubnetProfile attributes |  |
| Maximum number of UEs | maxNumberofUEs | maxNumberofUE | maxNumberofUEs | maxNumberofUEs | attributes in clause 5.3.96 IOC NSACFFunction: NsacfInfoSnssai.maxNumberofUEs |
| Maximum number of PDU sessions | maxNumberofConns | maxNumberofPDUSessions | N/A | maxNumberofPDUSessions | attributes in clause 5.3.96 IOC NSACFFunction: NsacfInfoSnssai.maxNumberofPDUSessions |

NOTE:	Void.

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile

Annex M (normative):
Managed NF Service state handling

# M.1	Combined state diagram for a Managed NF Service

Figure M.1-1: Combined Managed NF Service state diagram

Table M.1-1: The Managed NF Service state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 1 | Event: Received information of deployment of a Network Function (NF) service. Action: Create a ManagedNFService instance (MSI) whose(Administrative/Operational/Registration) are set to Locked/Disabled/Deregistered. |
| 2 | Event: Received information of positive state change of the NF service. Action: Set the Operational state of the MSI to Enabled. |
| 3 | Event: Received CM operation to unlock the NF Service or the NF. Action: Set the Administrative state of the MSI to Unlocked. Note: Changing Administrative state on NF service level is optional |
| 4 | Event: Received information that the NF Service is registered to an NRF either by the NF itself or by an OAM system on behalf of the NF. Action: Set the registration state of the MSI to Registered. |
| 5 | Event: Received information that the NF Service is deregistered from the NRF either by the NF itself or by an OAM system on behalf of the NF. Action: Set registration state of the MSI to Deregistered. |
| 6 | Event: Received information that the NF Service is unavailable because of, for example, limitation of resource or other exceptions. Action: Set the Operational state of the MSI to Disabled. |
| 7 | Event: Received information that the NF Service is unavailable. Action: Deregister the NF Service on behalf of the NF, and set the registration state of the MSI to Deregistered. |
| 8 | Event: Received CM operation to lock the NF Service or the NF. Action: Set the Administrative state of the MSI to Locked. Note: Changing Administrative state on NF service level is optional |
| 9 | Event: Received information that the NF Service is terminated or deleted, Action: Delete the MSI and set its state to NULL. |

Annex N (normative):
Void

Annex O (informative): 
QoS model usage for NG-RAN

# O.1	Overview

The Configurable 5QI set IOC (as defined in 5.3.75) provides flexibility to support multiple scenarios:

-	Configurable 5QI sets can be name contained by SubNetwork, ManagedElement, GNBDUFunction, GNBCUUPFunction and GNBCUCPFunction.

-	Sets are referenced by attributes (configurable5QISetRef) in applicable MOIs.

Specific containment and/or referencing may be appropriate for certain scenarios.

# O.2	General usage

For consistency, referenced 5QI sets can be defined within the same subtree (see Figure 4.2.1.1-8) as follows:

-	a Configurable5QISet instance contained by ManagedEntity does not need a reference from the ManagedEntity instance which contains it.

-	a Configurable5QISet instance contained by ManagedElement is only referenced by ManagedFunction instances contained within that ManagedElement.

-	a Configurable5QISet instance contained by Subnetwork is only referenced by ManagedFunction instances contained within the same Subnetwork.

-	a Configurable5QISet instance contained by ManagedFunction is only used by that ManagedFunction instance.

# O.3	NG-RAN MOCN network sharing usage recommendation

The operator specific IOCs, including OperatorDU (as defined in 4.3.67) which are configured per operator, are used to support the NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast scenario.

In addition to the general usage described in O.2, the following recommendations apply for NG-RAN MOCN network sharing with multiple Cell Identity broadcast scenario:

-	Configurable5QISet instances applicable to multiple operators sharing a gNB can be associated with the shared GNBDUFunction

-	Configurable5QI instances which are operator specific can be referenced by the OperatorDU defined for the applicable operator

Annex P (informative): 
Mapping between GSMA GST and ServiceProfile

Table P.1 shows the mapping from the GSMA GST, see reference [50] to the ServiceProfile which  can be found in  clause 6.3.3 of this  document.

Table P.1: Mapping between GSMA GST and ServiceProfile

| GSMA GST attribute name [50] | GST reference location [50] | ServiceProfile attribute | Clause number (see NOTE) |
| --- | --- | --- | --- |
| Availability | 3.4.1 | availability | 6.4.1 |
| Area of service | 3.4.2 | coverageArea | 6.4.1 |
| Delay tolerance | 3.4.3 | delayTolerance | 6.4.1 and 6.3.7 |
| Deterministic communication | 3.4.4 | dLDeterministicComm | 6.4.1 and 6.3.8 |
| Determinstic communication | 3.4.4 | uLDeterministicComm | 6.4.1 and 6.3.8 |
| Maximum supported packet size | 3.4.11 | dLMaxPktSize | 6.4.1 and 6.3.11 |
| Maximum supported packet size | 3.4.11 | uLMaxPktSize | 6.4.1 and 6.3.12 |
| Downlink throughput per network slice | 3.4.5 | dLThptPerSlice | 6.4.1 and 6.3.9 |
| Uplink throughput per network slice | 3.4.31 | uLThptPerSlice | 6.4.1 and 6.3.9 |
| Downlink maximum throughput per UE | 3.4.6 | dLThptPerUE | 6.4.1 and 6.3.9 |
| Uplink maximum throughput per UE | 3.4.32 | uLThptPerUE | 6.4.1 and 6.3.9 |
| Energy efficiency | 3.4.7 | energyEfficiency | 6.4.1 and 6.3.30 |
| Performance monitoring | 3.4.18 | kPIMonitoring | 6.4.1 and 6.3.14 |
| Maximum number of PDU sessions | 3.4.16 | maxNumberofPDUSessions | 6.4.1 and 6.3.12 |
| Maximum number of UEs | 3.4.17 | maxNumberofUEs | 6.4.1 and 6.3.38 |
| NB-IoT support | 3.4.14 | nBIoT | 6.4.1 and 6.3.20 |
| Positioning support | 3.4.20 | positioning | 6.4.1 and 6.3.26 |
| Radiospectrum | 3.4.21 | radioSpectrum | 6.4.1 and 6.3.31 |
| Simultaneous use of the network slice | 3.4.25 | sliceSimultaneousUse | 6.4.1 |
| Synchronicity | 3.4.29 | synchronicity | 6.4.1 and 6.3.27 |
| UE density | 3.4.30 | termDensity | 6.4.1 and 6.3.17 |
| Supported device velocity | 3.4.28 | uESpeed | 6.4.1 |
| User management openness | 3.4.33 | userMgmtOpen | 6.4.1 and 6.3.15 |
| V2X-PC5 Network Scheduled Mode | 3.4.35 | v2XCommModels | 6.4.1 and 6.3.16 |
| Network Slice Specific Authentication and Authorization (NSSAA) Required | 3.4.37 | nssaaSupport | 6.4.1 and 6.3.34 |
| Slice quality of service | 3.4.26 | NA | NA |
| Performance prediction | 3.4.19 | NA | NA |
| Data network access | 3.4.34 | dataNetworkAccess | 6.4.1 and 6.3.46 |
| Isolation level | 3.4.9 | NA | NA |
| Group communication support | 3.4.8 | NA | NA |
| Latency from (last) UPF to Application Server | 3.4.36 | NA | NA |
| Mission critical support | 3.4.12 | NA | NA |
| MMTel support | 3.4.13 | NA | NA |
| Network function owned by Network Slice Customer | 3.4.15 | NA | NA |
| Monitoring and analytics | 3.4.23 | NA | NA |
| Session and Service Continuity support | 3.4.24 | NA | NA |
| Support for non-IP traffic | 3.4.27 | nonIPSupport | 6.4.1 and 6.3.42 |
| Latency from (last) UPF to Application Server | 3.4.36 | NA | NA |
| Multimedia Priority Service | 3.4.38 | NA | NA |
| Maximum number of UEs with at least one PDU session/PDN connection | 3.4.40 | NA | NA |
| Supported data network | 3.4.39 | supportedDataNetworks | 6.4.1 and 6.3.45 |
| V2X-PC5 parameter provisioning | 3.4.41 | NA | NA |
| PDU Set Support | 3.4.42 | NA | NA |
| NOTE: The attribute definitions can be found in clause 6.4, there where the attribute is defined by a custom data type the clause for the data type definition is also mentioned. |  |  |  |

Annex Q (informative):
PlantUML source code

# Q.1	5GC NRM relationship diagram

## Q.1.1	The 5GC NF NRM containment/naming relationship

@startuml Figure 5.2.1.1-1: 5GC NF NRM containment/naming relationship

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

left to right direction

skinparam ranksep 100

skinparam nodesep 4

class ManagedElement <<InformationObjectClass>>

class LMFFunction <<InformationObjectClass>>

class NGEIRFunction <<InformationObjectClass>>

class NSSFFunction <<InformationObjectClass>>

class SEPPFFunction <<InformationObjectClass>>

class SCPFFunction <<InformationObjectClass>>

class SMSFFunction <<InformationObjectClass>>

class UDRFunction <<InformationObjectClass>>

class N3IWFFunction <<InformationObjectClass>>

class "5G DDNMFFunction" <<InformationObjectClass>>

class NSSAAFFunction  <<InformationObjectClass>>

class MFAFFunction <<InformationObjectClass>>

class NSACFFunction <<InformationObjectClass>>

class GMLCFunction <<InformationObjectClass>>

class TSCTSFFunction <<InformationObjectClass>>

class MBUPFFunction <<InformationObjectClass>>

class MNPFFunction <<InformationObjectClass>>

class AMFFunction <<InformationObjectClass>>

class SMFFunction <<InformationObjectClass>>

class UPFFunction <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class NEFFunction <<InformationObjectClass>>

class NRFFunction <<InformationObjectClass>>

class UDMFunction <<InformationObjectClass>>

class AUSFFunction <<InformationObjectClass>>

class AFFunction <<InformationObjectClass>>

class EASDFFunction <<InformationObjectClass>>

class CHFFFunction <<InformationObjectClass>>

class DCCFFunction <<InformationObjectClass>>

class NWDAFFunction <<InformationObjectClass>>

class AANFFunction <<InformationObjectClass>>

class BSFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

LMFFunction        "*" --* "1" ManagedElement: <<names>>

NGEIRFunction      "*" --* "1" ManagedElement: <<names>>

NSSFFunction       "*" --* "1" ManagedElement: <<names>>

SEPPFFunction      "*" --* "1" ManagedElement: <<names>>

SCPFFunction       "*" --* "1" ManagedElement: <<names>>

SMSFFunction       "*" --* "1" ManagedElement: <<names>>

UDRFunction        "*" --* "1" ManagedElement: <<names>>

N3IWFFunction      "*" --* "1" ManagedElement: <<names>>

"5G DDNMFFunction" "*" --* "1" ManagedElement: <<names>>

NSSAAFFunction     "*" --* "1" ManagedElement: <<names>>

MFAFFunction       "*" --* "1" ManagedElement: <<names>>

NSACFFunction      "*" --* "1" ManagedElement: <<names>>

GMLCFunction       "*" --* "1" ManagedElement: <<names>>

TSCTSFFunction     "*" --* "1" ManagedElement: <<names>>

MBUPFFunction      "*" --* "1" ManagedElement: <<names>>

MNPFFunction       "*" --* "1" ManagedElement: <<names>>

ManagedElement "1" *-- "*" AMFFunction  : <<names>>

ManagedElement "1" *-- "*" SMFFunction  : <<names>>

ManagedElement "1" *-- "*" UPFFunction  : <<names>>

ManagedElement "1" *-- "*" PCFFunction  : <<names>>

ManagedElement "1" *-- "*" NEFFunction  : <<names>>

ManagedElement "1" *-- "*" NRFFunction  : <<names>>

ManagedElement "1" *-- "*" UDMFunction  : <<names>>

ManagedElement "1" *-- "*" AUSFFunction : <<names>>

ManagedElement "1" *-- "*" AFFunction   : <<names>>

ManagedElement "1" *-- "*" EASDFFunction: <<names>>

ManagedElement "1" *-- "*" CHFFFunction : <<names>>

ManagedElement "1" *-- "*" DCCFFunction : <<names>>

ManagedElement "1" *-- "*" NWDAFFunction : <<names>>

ManagedElement "1" *-- "*" AANFFunction : <<names>>

ManagedElement "1" *-- "*" BSFFunction  : <<names>>

ManagedElement "1" *-- "*" MBSMFFunction: <<names>>

class SubNetwork <<InformationObjectClass>>

class ExternalNRFFunction <<InformationObjectClass>>

class ExternalNSSFFunction <<InformationObjectClass>>

class ExternalAMFFunction <<InformationObjectClass>>

ExternalNRFFunction      "*" -u-* "1" SubNetwork: <<names>>

ExternalNSSFFunction     "*" -u-* "1" SubNetwork: <<names>>

SubNetwork "1" *-d- "*" ExternalAMFFunction  : <<names>>

SubNetwork "1" *-- "*" ManagedElement  : <<names>>

@enduml

## Q.1.2	The Figure 5.2.1.1-2 for the transport view of AMF NRM

'Figure 5.2.1.1-2: Transport view of AMF NRM

@startuml Figure 5.2.1.1-2: Transport view of AMF NRM

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam linetype ortho

skinparam nodesep 2

skinparam ranksep 100

left to right direction

class AMFFunction <<InformationObjectClass>>

class EP_N2 <<InformationObjectClass>>

class EP_N8 <<InformationObjectClass>>

class EP_N11 <<InformationObjectClass>>

class EP_N12 <<InformationObjectClass>>

class EP_N14 <<InformationObjectClass>>

class EP_N15 <<InformationObjectClass>>

class EP_N17 <<InformationObjectClass>>

class EP_N22 <<InformationObjectClass>>

class EP_N26 <<InformationObjectClass>>

class EP_N20 <<InformationObjectClass>>

class EP_NL1 <<InformationObjectClass>>

class EP_NL2 <<InformationObjectClass>>

class EP_N60 <<InformationObjectClass>>

class EP_N41 <<InformationObjectClass>>

class EP_N42 <<InformationObjectClass>>

class EP_N89 <<InformationObjectClass>>

class EP_N11mb <<InformationObjectClass>>

class FarEnd_N2 <<ProxyClass>>

class UDMFunction <<InformationObjectClass>>

class SMFFunction <<InformationObjectClass>>

class AUSFFunction <<InformationObjectClass>>

class "AMFFunction " <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class NGEIRFunction <<InformationObjectClass>>

class NSSFFunction <<InformationObjectClass>>

class MMEFunction <<InformationObjectClass>>

class SMSFFunction <<InformationObjectClass>>

class LMFFunction <<InformationObjectClass>>

class GMLCFunction <<InformationObjectClass>>

class NSACFFunction <<InformationObjectClass>>

class "CHFFunction (HPLMN)" <<InformationObjectClass>>

class "CHFFunction (VPLMN)" <<InformationObjectClass>>

class TSCTSFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

AMFFunction "1" *-- "*" EP_N2: <<names>>

AMFFunction "1" *-- "*" EP_N8: <<names>>

AMFFunction "1" *-- "*" EP_N11: <<names>>

AMFFunction "1" *-- "*" EP_N12: <<names>>

AMFFunction "1" *-- "*" EP_N14: <<names>>

AMFFunction "1" *-- "*" EP_N15: <<names>>

AMFFunction "1" *-- "*" EP_N17: <<names>>

AMFFunction "1" *-- "*" EP_N22: <<names>>

AMFFunction "1" *-- "*" EP_N26: <<names>>

AMFFunction "1" *-- "*" EP_N20: <<names>>

AMFFunction "1" *-- "*" EP_NL1: <<names>>

AMFFunction "1" *-- "*" EP_NL2: <<names>>

AMFFunction "1" *-- "*" EP_N60: <<names>>

AMFFunction "1" *-- "*" EP_N41: <<names>>

AMFFunction "1" *-- "*" EP_N42: <<names>>

AMFFunction "1" *-- "*" EP_N89: <<names>>

AMFFunction "1" *-- "*" EP_N11mb: <<names>>

EP_N2  "1" --> "1" FarEnd_N2

EP_N8  "1" --> "1" UDMFunction

EP_N11 "1" --> "1" SMFFunction

EP_N12 "1" --> "1" AUSFFunction

EP_N14 "1" --> "1" "AMFFunction "

EP_N15 "1" --> "1" PCFFunction

EP_N17 "1" --> "1" NGEIRFunction

EP_N22 "1" --> "1" NSSFFunction

EP_N26 "1" --> "1" MMEFunction

EP_N20 "1" --> "1" SMSFFunction

EP_NL1 "1" --> "1" LMFFunction

EP_NL2 "1" --> "1" GMLCFunction

EP_N60 "1" --> "1" NSACFFunction

EP_N41 "1" --> "1" "CHFFunction (HPLMN)"

EP_N42 "1" --> "1" "CHFFunction (VPLMN)"

EP_N89 "1" --> "1" TSCTSFFunction

EP_N11mb "1" --> "1" MBSMFFunction

note top of FarEnd_N2: "This proxy class represents\n GNBFunction or \n GNBCUCPFunction (TS\n 28.541) or ENBFunction (TS\n 28.658) or N3IWFFunction"

@enduml

## Q.1.3	The Figure 5.2.1.1-3 for the transport view of SMF NRM

@startuml Figure 5.2.1.1-3: Transport view of SMF NRM

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam nodesep 3

skinparam ranksep 100

skinparam linetype ortho

left to right direction

class SMFFunction <<InformationObjectClass>>

class EP_NS5C <<InformationObjectClass>>

class EP_N4 <<InformationObjectClass>>

class EP_N7 <<InformationObjectClass>>

class EP_N10 <<InformationObjectClass>>

class EP_N11 <<InformationObjectClass>>

class EP_N16 <<InformationObjectClass>>

class EP_N40 <<InformationObjectClass>>

class EP_N88 <<InformationObjectClass>>

class EP_N16mb <<InformationObjectClass>>

class FarEnd_S5C <<ProxyClass>>

class UPFFunction <<InformationObjectClass>>

class UDMFunction <<InformationObjectClass>>

class AMFFunction <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class "SMFFunction " <<InformationObjectClass>>

class CHFFunction <<InformationObjectClass>>

class EASDFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

SMFFunction "1" *-- "*" EP_NS5C : <<names>>

SMFFunction "1" *-- "*" EP_N4   : <<names>>

SMFFunction "1" *-- "*" EP_N7   : <<names>>

SMFFunction "1" *-- "*" EP_N10  : <<names>>

SMFFunction "1" *-- "*" EP_N11  : <<names>>

SMFFunction "1" *-- "*" EP_N16  : <<names>>

SMFFunction "1" *-- "*" EP_N40  : <<names>>

SMFFunction "1" *-- "*" EP_N88  : <<names>>

SMFFunction "1" *-- "*" EP_N16mb  : <<names>>

EP_NS5C  "1" --> "1" FarEnd_S5C

EP_N4    "1" --> "1" UPFFunction

EP_N7    "1" --> "1" PCFFunction

EP_N10   "1" --> "1" UDMFunction

EP_N11   "1" --> "1" AMFFunction

EP_N16   "1" --> "1" "SMFFunction "

EP_N40   "1" --> "1" CHFFunction

EP_N88   "1" --> "1" EASDFFunction

EP_N16mb   "1" --> "1" MBSMFFunction

note top of FarEnd_S5C: "It represents ServingGWFunction\(from TS 28.708)"

@enduml

## Q.1.4	The Figure 4.2.1.1-23: NRM fragment to support energy saving based on Energy Cost Reporting

@startuml TS28.541 Figure NRECMappingRule

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

top to bottom direction

skinparam nodesep 40

class ManagedEntity <<ProxyClass>>

note left

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

class " ManagedEntity" <<ProxyClass>>

note left

Represents the following IOCs:

GNBCUCPFunction

end note

class NRECMappingRule <<InformationObjectClass>>

NRECMappingRule "*" -u-* "1" ManagedEntity: <<names>>

NRECMappingRule "0..1" <-u "*" " ManagedEntity": " +nRECMappingRuleRef"

@enduml

## Q.1.5	(Void)

## Q.1.6	(Void)

## Q.1.7	Figure 5.2.1.1-X Transport view of MME NRM

@startuml

'UML diagram from 3GPP TS 28.541 relationship between AMF and MME

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class AMFFunction <<InformationObjectClass>>

class MMEFunction  <<InformationObjectClass>>

class EP_N26  <<InformationObjectClass>>

left to right direction

MMEFunction "1" *-- "*"EP_N26

EP_N26 "1" --> "1" AMFFunction

@enduml

# Q.2	5GC NRM inheritance diagram

## Q.2.1	The inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

@startuml Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

left to right direction

skinparam linetype ortho

skinparam nodesep 2

class ManagedFunction <<InformationObjectClass>>

class ExternalNRFFunction <<InformationObjectClass>>

class ExternalNSSFFunction <<InformationObjectClass>>

class LMFFunction <<InformationObjectClass>>

class NGEIRFunction <<InformationObjectClass>>

class NSSFFunction <<InformationObjectClass>>

class SCPFFunction <<InformationObjectClass>>

class SEPPFFunction <<InformationObjectClass>>

class SMSFFunction <<InformationObjectClass>>

class UDRFunction <<InformationObjectClass>>

class N3IWFFunction <<InformationObjectClass>>

class NSACFFunction <<InformationObjectClass>>

class EASDFFunction <<InformationObjectClass>>

class ExternalAMFFunction <<InformationObjectClass>>

class AMFFunction <<InformationObjectClass>>

class SMFFunction <<InformationObjectClass>>

class UPFFunction <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class NEFFunction <<InformationObjectClass>>

class NRFFunction <<InformationObjectClass>>

class UDMFunction <<InformationObjectClass>>

class AUSFFunction <<InformationObjectClass>>

class "5G DDNMFFunction" <<InformationObjectClass>>

class AFFunction <<InformationObjectClass>>

class NWDAFFunction <<InformationObjectClass>>

class NSSAAFFunction  <<InformationObjectClass>>

class CHFFFunction <<InformationObjectClass>>

class DCCFFunction <<InformationObjectClass>>

class MFAFFunction <<InformationObjectClass>>

class AANFFunction <<InformationObjectClass>>

class BSFFunction <<InformationObjectClass>>

class GMLCFunction <<InformationObjectClass>>

class TSCTSFFunction <<InformationObjectClass>>

class MBUPFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

class MNPFFunction <<InformationObjectClass>>

class ExternalSeppFunction <<InformationObjectClass>>

class UDSFFunction <<InformationObjectClass>>

ExternalNSSFFunction --|> ManagedFunction

LMFFunction          --|> ManagedFunction

ExternalNRFFunction  --|> ManagedFunction

NGEIRFunction        --|> ManagedFunction

NSSFFunction         --|> ManagedFunction

SCPFFunction         --|> ManagedFunction

SEPPFFunction        --|> ManagedFunction

SMSFFunction         --|> ManagedFunction

UDRFunction          --|> ManagedFunction

N3IWFFunction        --|> ManagedFunction

NSACFFunction        --|> ManagedFunction

EASDFFunction        --|> ManagedFunction

NSSAAFFunction       --|> ManagedFunction

MFAFFunction         --|> ManagedFunction

AANFFunction         --|> ManagedFunction

BSFFunction          --|> ManagedFunction

MBUPFFunction        --|> ManagedFunction

MNPFFunction         --|> ManagedFunction

UDSFFunction         --|> ManagedFunction

ManagedFunction <|-- AMFFunction

ManagedFunction <|-- SMFFunction

ManagedFunction <|-- ExternalAMFFunction

ManagedFunction <|-- UPFFunction

ManagedFunction <|-- PCFFunction

ManagedFunction <|-- NEFFunction

ManagedFunction <|-- NRFFunction

ManagedFunction <|-- UDMFunction

ManagedFunction <|-- AUSFFunction

ManagedFunction <|-- "5G DDNMFFunction"

ManagedFunction <|-- AFFunction

ManagedFunction <|-- NWDAFFunction

ManagedFunction <|-- CHFFFunction

ManagedFunction <|-- DCCFFunction

ManagedFunction <|-- GMLCFunction

ManagedFunction <|-- TSCTSFFunction

ManagedFunction <|-- MBSMFFunction

ManagedFunction <|-- ExternalSeppFunction

@enduml

## Q.2.2	Figure 5.2.1.2-2 the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM

@startuml igure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

left to right direction

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class EP_RP <<InformationObjectClass>>

class EP_N2 <<InformationObjectClass>>

class EP_N3 <<InformationObjectClass>>

class EP_N4 <<InformationObjectClass>>

class EP_N5 <<InformationObjectClass>>

class EP_N6 <<InformationObjectClass>>

class EP_N7 <<InformationObjectClass>>

class EP_N8 <<InformationObjectClass>>

class EP_N9 <<InformationObjectClass>>

class EP_N10 <<InformationObjectClass>>

class EP_N11 <<InformationObjectClass>>

class EP_N12 <<InformationObjectClass>>

class EP_N13 <<InformationObjectClass>>

class EP_N14 <<InformationObjectClass>>

class EP_N15 <<InformationObjectClass>>

class EP_N16 <<InformationObjectClass>>

class EP_N17 <<InformationObjectClass>>

'class EP_N18 <<InformationObjectClass>>

class EP_N22 <<InformationObjectClass>>

class EP_N26 <<InformationObjectClass>>

class EP_N27 <<InformationObjectClass>>

class EP_N28 <<InformationObjectClass>>

class EP_N31 <<InformationObjectClass>>

class EP_N32 <<InformationObjectClass>>

class EP_N33 <<InformationObjectClass>>

class EP_N40 <<InformationObjectClass>>

class EP_N41 <<InformationObjectClass>>

class EP_N42 <<InformationObjectClass>>

class EP_N58 <<InformationObjectClass>>

class EP_N60 <<InformationObjectClass>>

class EP_Npc4 <<InformationObjectClass>>

class EP_Npc6 <<InformationObjectClass>>

class EP_Npc7 <<InformationObjectClass>>

class EP_Npc8 <<InformationObjectClass>>

class EP_N59 <<InformationObjectClass>>

class EP_N88 <<InformationObjectClass>>

class EP_NL1 <<InformationObjectClass>>

class EP_NL2 <<InformationObjectClass>>

class EP_N20 <<InformationObjectClass>>

class EP_N22 <<InformationObjectClass>>

' AANFFunction

class EP_N61 <<InformationObjectClass>>

class EP_N62 <<InformationObjectClass>>

class EP_N63 <<InformationObjectClass>>

' GMLC

class EP_NL3 <<InformationObjectClass>>

class EP_NL6 <<InformationObjectClass>>

class EP_NL5 <<InformationObjectClass>>

class EP_NL9 <<InformationObjectClass>>

' TSCTSFFunction

class EP_N86 <<InformationObjectClass>>

class EP_N84 <<InformationObjectClass>>

class EP_N85 <<InformationObjectClass>>

class EP_N87 <<InformationObjectClass>>

class EP_N89 <<InformationObjectClass>>

class EP_N96 <<InformationObjectClass>>

' MB-SMF & MB-UPF

class EP_N11mb <<InformationObjectClass>>

class EP_N16mb <<InformationObjectClass>>

class EP_N19mb <<InformationObjectClass>>

class EP_Nmb1  <<InformationObjectClass>>

class EP_Nmb9  <<InformationObjectClass>>

class EP_N3mb  <<InformationObjectClass>>

class EP_N4mb  <<InformationObjectClass>>

' MNPF

class EP_SM12 <<InformationObjectClass>>

class EP_SM13 <<InformationObjectClass>>

class EP_SM14 <<InformationObjectClass>>

EP_N2  --|> EP_RP

EP_N3  --|> EP_RP

EP_N4  --|> EP_RP

EP_N5  --|> EP_RP

EP_N6  --|> EP_RP

EP_N7  --|> EP_RP

EP_N8  --|> EP_RP

EP_N9  --|> EP_RP

EP_N10 --|> EP_RP

EP_N11 --|> EP_RP

EP_N12 --|> EP_RP

EP_N13 --|> EP_RP

EP_N14 --|> EP_RP

EP_N15 --|> EP_RP

EP_N16 --|> EP_RP

EP_N17 --|> EP_RP

EP_N22 --|> EP_RP

EP_N26 --|> EP_RP

EP_N27 --|> EP_RP

EP_N28 --|> EP_RP

EP_N31 --|> EP_RP

EP_N32 --|> EP_RP

EP_N33 --|> EP_RP

EP_N40 --|> EP_RP

EP_N41 --|> EP_RP

EP_N42 --|> EP_RP

'since SA5#151

EP_Nmb1 --|> EP_RP

EP_Nmb9 --|> EP_RP

EP_N3mb --|> EP_RP

EP_N4mb --|> EP_RP

EP_SM12 --|> EP_RP

EP_RP <|-- EP_N58

EP_RP <|-- EP_N60

EP_RP <|-- EP_Npc4

EP_RP <|-- EP_Npc6

EP_RP <|-- EP_Npc7

EP_RP <|-- EP_Npc8

EP_RP <|-- EP_N59

EP_RP <|-- EP_N88

EP_RP <|-- EP_NL1

EP_RP <|-- EP_NL2

EP_RP <|-- EP_N20

EP_RP <|-- EP_N22

EP_RP <|-- EP_N61

EP_RP <|-- EP_N62

EP_RP <|-- EP_N63

EP_RP <|-- EP_NL3

EP_RP <|-- EP_NL6

EP_RP <|-- EP_NL5

EP_RP <|-- EP_NL9

EP_RP <|-- EP_N86

EP_RP <|-- EP_N84

EP_RP <|-- EP_N85

EP_RP <|-- EP_N87

EP_RP <|-- EP_N89

EP_RP <|-- EP_N96

'since SA5#151

EP_RP <|-- EP_N11mb

EP_RP <|-- EP_N16mb

EP_RP <|-- EP_N19mb

EP_RP <|-- EP_SM13

EP_RP <|-- EP_SM14

@enduml

# Q.3 NetworkSlice NRM relationship diagram

## Q.3.1	The NetworkSlice NRM containment/naming relationship

@startuml TS 28.541 figure 6.2.1-1 (for Controller)

' UML diagram from 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class NetworkSlice <<InformationObjectClass>>

class ServiceProfile <<dataType>>

class NetworkSliceSubnet <<InformationObjectClass>>

class SliceProfile <<dataType>>

class ManagedFunction <<InformationObjectClass>>

class NetworkService <<OpenModelClass,Preliminary>>

class EP_Transport <<InformationObjectClass>>

class VNF <<OpenModelClass,Preliminary>>

class NetworkSliceController <<InformationObjectClass>>

class NetworkSliceSubnetController <<InformationObjectClass>>

NetworkSlice "1 " -d-> "*" ServiceProfile

NetworkSlice "1 " <-d-> "*"  NetworkSliceController

NetworkSlice "0..1" -d-> "1" NetworkSliceSubnet

NetworkSliceSubnet "*" o--> "*" NetworkSliceSubnet

NetworkSliceSubnet "1" -d-> "*" SliceProfile

NetworkSliceSubnet "1" <-d-> "*"  NetworkSliceSubnetController

NetworkSliceSubnet "*" o-d-> "*" ManagedFunction

NetworkSliceSubnet "*" -d-> "*" NetworkService

NetworkSliceSubnet "*" o-d-> "*" EP_Transport

ManagedFunction "0..1" -d-> "*" VNF

NetworkService "*" o-d- "*" VNF : <<NsIncludesNf>>

@enduml

## Q.3.2	The Transport EP NRM containment/naming relationship

@startuml Figure 6.2.1-2: Transport EP NRM fragment relationship

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor black

skinparam arrowColor black

top to bottom direction

class EP_Transport <<InformationObjectClass>>

class EP_Application <<ProxyClass>>

EP_Transport "*" -- "*" EP_Application

Note left of EP_Application

This represents

EP_N3 or EP_NgU or EP_F1U

end note

@enduml

## Q.3.3	The NetworkSlice NRM containment/naming relationship

@startuml Figure 6.2.1-3: containment relationship for network slice fragment

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

class SubNetwork <<InformationObjectClass>>

class NetworkSlice <<InformationObjectClass>>

class NetworkSliceController <<InformationObjectClass>>

class NetworkSliceSubnet <<InformationObjectClass>>

class NetworkSliceSubnetController <<InformationObjectClass>>

class NetworkSliceSubnetProviderCapabilities <<InformationObjectClass>>

class EP_Transport <<InformationObjectClass>>

SubNetwork "1" *-- "*" NetworkSlice: <<names>>

SubNetwork "1" *-- "*" NetworkSliceController: <<names>>

SubNetwork "1" *-- "*" NetworkSliceSubnet: <<names>>

SubNetwork "1" *-- "*" NetworkSliceSubnetController: <<names>>

SubNetwork "1" *-- "*" NetworkSliceSubnetProviderCapabilities: <<names>>

SubNetwork "1" *-- "*" EP_Transport: <<names>>

@enduml

## Q.3.4	The feasibility check and resource reservation containment/naming relationship

@startuml Figure 6.2.1-4: containment relationship for feasibility check and resource reservation NRM fragment

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor black

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

class ServiceProfile <<dataType>>

class SliceProfile <<dataType>>

class SubNetwork <<InformationObjectClass>>

class FeasibilityCheckAndReservationJob <<InformationObjectClass>>

SubNetwork "1" *-- "*" FeasibilityCheckAndReservationJob: <<names>>

FeasibilityCheckAndReservationJob "1" --> "1" ServiceProfile

FeasibilityCheckAndReservationJob "1" --> "1" SliceProfile

note "{XOR}" as Note1

Note1 .. ServiceProfile), (FeasibilityCheckAndReservationJob

Note1 .. SliceProfile), (FeasibilityCheckAndReservationJob, SliceProfile)

@enduml

## Q.3.5	The IsolationProfile NRM containment/naming relationship

@startuml Figure 6.2.1-4: containment relationship for isolation profile NRM fragment

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

class SubNetwork <<InformationObjectClass>>

class IsolationProfile <<InformationObjectClass>>

SubNetwork "1" *-- "*" IsolationProfile: <<names>>

@enduml

## Q.3.6	The IsolationPofile to NetworkSlice relationship

@startuml Figure 6.2.1-4: Figure 6.2.1-6: IsolationProfile NRM fragment relationship related to NetworkSlice

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

left to right direction

class NetworkSlice <<InformationObjectClass>>

class IsolationProfile <<InformationObjectClass>>

NetworkSlice "*" <--> "1" IsolationProfile

@enduml

## Q.3.7	The IsolationPofile to NetworkSliceSubnet relationship

@startuml Figure 6.2.1-7: IsolationProfile NRM fragment relationship related to NetworkSliceSubnet

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

left to right direction

class NetworkSliceSubnet <<InformationObjectClass>>

class IsolationProfile <<InformationObjectClass>>

NetworkSliceSubnet "*" <--> "1" IsolationProfile

@enduml

# Q.4	NetworkSlice inheritance diagram

## Q.4.1	Network slice inheritance diagram

@startuml

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

abstract class Top <<InformationObjectClass>>

class NetworkSlice <<InformationObjectClass>>

class NetworkSliceSubnet <<InformationObjectClass>>

class NetworkSliceSubnetProviderCapabilities <<InformationObjectClass>>

class EP_Transport <<InformationObjectClass>>

class IsolationProfile <<InformationObjectClass>>

Top  <|-- NetworkSlice

Top  <|-- NetworkSliceSubnet

Top  <|-- NetworkSliceSubnetProviderCapabilities

Top  <|-- EP_Transport

Top  <|-- IsolationProfile

@enduml

## Q.4.2	Feasibility check inheritance diagram

@startuml

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

abstract class Top <<InformationObjectClass>>

class FeasibilityCheckAndReservationJob <<InformationObjectClass>>

Top  <|-- FeasibilityCheckAndReservationJob

@enduml

## Q.4.3	NetworkSliceController inheritance diagram

@startuml

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

skinparam ClassStereotypeFontStyle normal

hide circle

hide method

hide members

abstract class Top <<InformationObjectClass>>

class NetworkSliceController <<InformationObjectClass>>

class NetworkSliceSubnetController <<InformationObjectClass>>

Top  <|-- NetworkSliceController

Top  <|-- NetworkSliceSubnetController

@enduml

Annex R (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-09 | SA#81 |  |  |  |  | EdiHelp review | 15.0.1 |
| 2018-12 | SA#82 | SP-181046 | 0001 | 1 | F | Fix issues raised by EditHelp | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0002 | 2 | F | Update NR Stage 2 definition to align with TS 37.340 for MR-DC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0003 | 1 | F | Update NRM Stage 2 defintion to align with TS 23.501 for 5G architecture | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0005 | 1 | F | Update Stage 3 XML definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0006 | 1 | F | Update Stage 3 JSON definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0007 | 1 | F | Update Stage 3 YANG definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0008 | 1 | F | Update Stage 3 XML definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0009 | 1 | F | Update Stage 3 JSON definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0011 | 1 | F | Update stage 3 XML definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0012 | 1 | F | Update Stage 3 JSON definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0013 | 1 | F | Update stage 3 YANG definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0014 | 1 | F | Correct the term sNSSAIList and nRTAClist | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0015 | 1 | F | Update the inheritance hierarchy figure for NR NRM to include BWP IOC and NRSectorCarrier IOC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0016 | 1 | F | Change the term nCGI to nCI | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0019 | 1 | F | Align properties of cell state | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0021 | 1 | F | Add missing attribute definition and condition | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0022 | 1 | F | Add missing detail definition for attribute | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0023 | 1 | F | Adding missing attribute, and correction of reference | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0025 | - | F | Remove NSSF from the abbrevations | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0027 | - | F | Replace symbol for network slice state management | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0031 | 1 | F | Remove the ExternalENBFunction definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0033 | 1 | F | Align the management of external function and cell with TS 28.658 | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0034 | 1 | F | Update NR NRM with Cell Relation | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0038 | 3 | F | RRM Policy enhancements | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0039 | 1 | F | Fix containment issue in YANG definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0040 | - | F | Implement minor corrections | 15.1.0 |
| 2018-12 | SA#82 | SP-181042 | 0041 | - | F | Update Stage 3 NRM for RRM Policy enhancements | 15.1.0 |
| 2019-03 | SA#83 | SP-190121 | 0043 | 1 | F | Align NR attributes definition related to SSB with corresponding NG-RAN IE definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0044 | 1 | F | Correct the use of nCI and PLMN | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0045 | - | F | Remove duplicate definition for ExternalNRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0046 | 2 | F | Correct class diagram for view on external entities | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0047 | 1 | F | Correct the definition for resourceSharingLevel | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0048 | 1 | F | Correction of references | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0052 | 1 | F | Align the term mFIdList and constituentNSSIIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0053 | 1 | F | Correct the definition of nSSIId | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0054 | 1 | F | Add missing attribute constraint for class definition of NSSFFunction | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0055 | 1 | F | Correct attribute constraints for RRMpolicy related attributes in NRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0057 | - | F | Correct cardinality of End Point (EP) to target | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0058 | 0 | F | Correct Import table | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0059 | - | F | Remove ExternalNRCellCU.pLMNIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0060 | - | F | Use 'bS' (not 'bs') to prefix all BS (base station) attributes | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0061 | 1 | F | Correction of State attributes descriptions | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0062 | - | F | Update 5G JSON Solution Set to align with generic NRM | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0063 | 1 | F | Update YANG Solution Set to align with Stage 2 definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0064 | 1 | F | Update Information Service to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0065 | 1 | F | Update Solution Set to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0066 | 1 | F | Add availability in service profile of network slice resource model | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0068 | 1 | F | Add sST attribute to ServiceProfile | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0069 | 1 | F | Update to sST attribute stage 3 | 15.2.0 |
| 2019-03 | SA#83 | SP-190149 | 0073 | 2 | F | Replace CoverageAreaTAList type definition | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0074 | 1 | F | Name datatypes SliceProfile and ServiceProfile | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0075 | 1 | F | Add datatype definition for S-NSSAI | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0076 | 1 | F | Remove incomplete description for TAC | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0079 | 1 | F | Name datatype RRMPolicyRatio2 | 16.0.0 |
| 2019-06 | SA#84 | SP-190374 | 0083 | - | A | Remove attribute availabilityStatus in NRCellDU IOC | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0085 | 1 | F | Correct the definition for nsInfo | 16.1.0 |
| 2019-06 | SA#84 | SP-190374 | 0088 | 1 | A | Update Information Service of NR to fix unclear Note issue | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0096 | 2 | A | Correct the use of plmnIdList | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0098 | 1 | F | Add missing clauses to RRMPolicyRatio2 data type | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0099 | 1 | F | Update RRMPolicyRatio2 data type name in stage 3 | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0102 | - | F | Fix the implementation errors | 16.1.0 |
| 2019-09 | SA#85 | SP-190745 | 0089 | 2 | B | Update 5GC Information Service to align with Managed Service Definition | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0107 | 1 | A | Correct description for NR deployment scenario | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0109 | 1 | A | Correct NR NRM model to be applicable for all NG-RAN architecture | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0114 | 1 | C | Support NF Profile management | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0121 | 1 | A | Clarification of sNSSAIList attribute | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0123 | - | A | Remove pLMNId from GNBDUFunction | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0126 | 2 | A | Update class definition with inheritance information | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0128 | 1 | A | Correct description of NRCellCU and NRCellDU to be applicable for all deployment scenarios | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0130 | - | A | Correct XML solution set for NR | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0132 | - | A | Correct XML solution set for Network slice | 16.2.0 |
| 2019-09 | SA#85 | SP-190750 | 0133 | 1 | F | Clarification on slice model | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0142 | 1 | A | Add YANG mount info | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0143 | - | A | Add YANG solution | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0149 | 1 | F | generate JSON definition for 5GC NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0150 | 1 | A | Fix NR NRM to add missed ID info | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0152 | - | F | XML Solution Set for 5GC | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0154 | - | A | Correct ETSI NFV reference | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0157 | 1 | A | generate JSON definition for Slice NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0158 | 1 | A | generate JSON definition for NR NRM based on new style guideline | 16.2.0 |
| 2019-12 | SA#86 | SP-191159 | 0146 | 3 | F | To syn up with v1540 stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0156 | 2 | A | Correct Import table | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0161 | 1 | C | Extensions to PCF and UPF IOCs for support of TSC (Time Sensitive Communication) | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0166 | 1 | F | Correct XML solution set for NR | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0167 | 1 | F | Correct Network slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0168 | 2 | A | Correct NR TAC attribute property | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0170 | - | A | Correction of the duplicated IOC NSSFFunction in daigram | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0172 | - | A | Correction of the wrong IOC names in transport view diagram---Not implemented, wrong baseline (MCC) | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0175 | 2 | F | XML Solution Set for 5GC | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0177 | 3 | C | Update on slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0178 | 2 | B | Add relation of GST and profiles | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0180 | 3 | F | Update SEPP Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0182 | 1 | C | Add NEF Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0184 | 1 | C | Add SCP Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0185 | - | C | Add Stage 3 definitions of 5GC NRM to align with stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0186 | 1 | C | Support communication model in 5GC NF - Stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0192 | 1 | F | Fix merging errors of the specification | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0195 | - | C | Add State Handling diagram for NF service | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0197 | - | B | Updates to YANG SS | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0198 | 1 | C | Update XML definitions of ServiceProfile NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0199 | 2 | C | Update JSON definitions of ServiceProfile NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0200 | 1 | C | Add managedNFProfile definition for ngc NRM - stage3 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0202 | 2 | B | Add the RIM monitoring parameters for remote interference management | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0212 | 2 | F | Correct Network slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0213 | - | F | Update SEPP Stage 3 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191180 | 0222 | 2 | B | Management of NR ANR, Stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191180 | 0223 | - | B | Management of NR ANR, Stage 3 | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0226 | 1 | A | Add Stages 2 NRM Info Model definitions for beam managed object classes | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0227 | - | A | Add Stages 2 NRM Info Model definitions for beam managed object classes | 16.3.0 |
| 2020-03 | SA#87E | SP-200169 | 0163 | 4 | F | Correct the parameter sNSSAIList | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0179 | 3 | C | Update of RRM Policy | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0235 | - | F | Correction of reference | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0239 | 1 | F | Update the NR NRM to align with NG-RAN overview architecture | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0241 | - | F | Some correction on the NR NRM | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0242 | - | F | Fix merging errors of the specification | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0243 | 1 | F | Update NRM attribute definitions | 16.4.0 |
| 2020-03 | SA#87E | SP-200233 | 0245 | 2 | B | Add the RIM parameters for remote interference management | 16.4.0 |
| 2020-03 | SA#87E | SP-200234 | 0248 | 1 | F | Update on slice NRM and solution sets | 16.4.0 |
| 2020-03 | SA#87E | SP-200234 | 0250 | 1 | F | Update of GNBCUUPFunction NRM | 16.4.0 |
| 2020-03 | SA#87E | SP-200232 | 0253 | 2 | B | Add Stage 3 NRM Info Model definitions for RRMPolicy and PLMNInfo related CRs | 16.4.0 |
| 2020-03 | SA#87E | SP-200178 | 0254 | 1 | F | Correct CR implementation errors | 16.4.0 |
| 2020-03 | SA#87E | SP-200235 | 0255 | 1 | F | Add OpenAPI definitions required by the ProvMnS | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0258 |  | F | Correct errors in yang solution set | 16.4.0 |
| 2020-03 | SA#87E |  |  |  |  | Correction of implementation errrors | 16.4.1 |
| 2020-06 | SA#88-e | SP-200489 | 0259 | 1 | F | Update on the RRMpolicyRatio | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0260 | - | F | Replace DN with better identifier for whitelists and blacklists management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200603 | 0261 | 1 | B | Add IOC for control of QoS monitoring per QoS flow per UE | 16.5.0 |
| 2020-06 | SA#88-e | SP-200604 | 0262 | 1 | B | Add IOC for control of GTP-U path QoS monitoring | 16.5.0 |
| 2020-06 | SA#88-e | SP-200489 | 0263 | 1 | F | Correction of reference | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0268 | - | B | ANR management for EN-DC architecture | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0269 | 1 | F | Clarification on network slice related identifiers | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0270 | - | F | Stage 3 update for clarification on network slice related identifiers | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0274 | 1 | F | Correct sNSSAI definition in XML solution set | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0275 | 1 | F | Clarify the NR NRM used for different deployment scenarios | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0278 | - | F | Add missing notification types to the definition of common notifications | 16.5.0 |
| 2020-06 | SA#88-e | SP-200491 | 0279 | 1 | A | Update on NRCellDU | 16.5.0 |
| 2020-06 | SA#88-e | SP-200491 | 0281 | 1 | A | Update Clause 4.2.1.2 Inheritance UML diagram | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0283 | 2 | B | new NRM fragment to support RIM stage 2 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0284 | 1 | B | new NRM fragment to support RIM stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200489 | 0285 | - | F | Update stage 3 on the RRMpolicyRatio | 16.5.0 |
| 2020-06 | SA#88-e | SP-200605 | 0286 | 2 | B | Add IOC for configurable 5QIs | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0287 | 1 | B | Add IOC for 5QI to DSCP mapping | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0289 | - | B | Stage3 add the NRM fragment for SON management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0290 | - | B | ANR management for EN-DC architecture | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0291 | 1 | B | Add the NRM fragment for SON management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0293 | - | F | Add CommModelList NRM definition | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0294 | 1 | F | Update NRM attribute definitions | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0295 | 1 | F | Correct NRM definition in XML solution | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0300 | 1 | F | Clarification on the relation of GST, ServiceProfile and SliceProfile | 16.5.0 |
| 2020-06 | SA#88-e | SP-200496 | 0301 | 1 | B | Add ES coverage relation in NRCellRelation | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0302 | - | F | Update the decription for RRMPolicy_ and resouceType | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0303 | - | F | Update definition for attribute localAddress in EP_RP IOC | 16.5.0 |
| 2020-06 | SA#88-e | SP-200486 | 0305 | 1 | A | Correction of references | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0306 | 1 | F | add transport information and slice mapping on backhaul endpoints | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0307 | - | F | add transport information and slice mapping on backhaul endpoints stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0312 | 1 | F | Update SliceProfile attributes solution 1 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0315 | 1 | B | Add configuredMaxTxEIRP on NRSectorCarrier | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0316 | - | B | Stage 3 Add configuredMaxTxEIRP on NRSectorCarrier | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0318 | - | F | Update NRM YANG for 28.541 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200496 | 0319 | - | B | Add ES coverage relation in NRCellRelation Stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200612 | 0320 | 1 | F | Update openAPI for NRCellRelation and NRFreqRelation | 16.5.0 |
| 2020-09 | SA#89-e | SP-200729 | 0321 | - | F | Correction of NRM YANG errors | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0322 | 1 | F | Correct on NR NRM | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0323 | - | F | Correct the openAPI definition for NR NRM | 16.6.0 |
| 2020-09 | SA#89-e | SP-200730 | 0325 | - | A | Correct on frequency related IOC | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0329 | 1 | B | Add IOC for predefined PCC rules | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0330 | 2 | B | Add IOC for predefined PCC rules | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0331 | - | B | Enable PCF to support configurable 5QIs | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0332 | - | B | Add IOC for dynamic 5QIs - stage 2 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0333 | - | B | Add IOC for dynamic 5QIs - stage 3 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0334 | - | B | Add TCE mapping info in GNBCUCPFunction | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0335 | - | B | Add TCE mapping info in openAPI solution | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0336 | - | F | Add missing definitions for perfReq | 16.6.0 |
| 2020-09 | SA#89-e | SP-200754 | 0338 | 1 | F | Delete supportedAccessTech to align with GST | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0339 | - | F | Correction on duplicated annex numbering | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0345 | - | F | Update NRM attribute definitions | 16.6.0 |
| 2020-09 | SA#89-e | SP-200749 | 0362 | - | F | Deleting SupportedAccessTech - Stage 3 - XML | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0368 | 1 | F | Add relation between transport and application level endpoints | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0369 | - | F | Add relation between transport and application level endpoints stage 3 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0370 | 1 | F | Cleanup stage 2 editorial issue and stage 3 yaml error | 16.6.0 |
| 2020-09 | SA#89-e | SP-200749 | 0371 | - | F | Add clarifying note to ServiceProfile | 16.6.0 |
| 2020-09 | SA#89-e | SP-200752 | 0337 | - | B | Add the MLB support indicator in NRcellrelation | 17.0.0 |
| 2020-09 | SA#89-e | SP-200749 | 0341 | 1 | F | Update maxNumberofConns | 17.0.0 |
| 2020-09 | SA#89-e | SP-200749 | 0342 | - | B | Add NB-IoT support in ServiceProfile | 17.0.0 |
| 2020-09 | SA#89-e | SP-200729 | 0366 | 1 | B | Addition of attribute for network slice supporting maximum of data volume | 17.0.0 |
| 2020-11 |  |  |  |  |  | No technical changes. Cleanup of diverse issues in order to improve performance of the file: hidden XML, watermarks,etc.. | 17.0.1 |
| 2020-12 | SA#90e | SP-201057 | 0380 | - | A | Correct the definition for configurable5QI and dynamic5QI | 17.1.0 |
| 2020-12 | SA#90e | SP-201066 | 0382 | 1 | F | Change RACH control attributes from beam to cell | 17.1.0 |
| 2020-12 | SA#90e | SP-201045 | 0384 | 1 | A | Move Distributed RACH control IOC from CU to DU | 17.1.0 |
| 2020-12 | SA#90e | SP-201045 | 0386 | 2 | A | Move Distributed PCI control IOC from DU to CU | 17.1.0 |
| 2020-12 | SA#90e | SP-201057 | 0388 | - | A | Correction of cell neighbour relations related attributes in openAPI solution | 17.1.0 |
| 2020-12 | SA#90e | SP-201057 | 0393 | - | A | Correction of NRM YANG errors | 17.1.0 |
| 2020-12 | SA#90e | SP-201057 | 0395 | 1 | A | Correct Network slice NRM | 17.1.0 |
| 2020-12 | SA#90e | SP-201053 | 0399 | 1 | A | Fix description related to service profile | 17.1.0 |
| 2020-12 | SA#90e | SP-201050 | 0405 | - | A | Add containment relationship for network slice IOCs | 17.1.0 |
| 2020-12 | SA#90e | SP-201050 | 0406 | - | F | Add containment relationship for network slice IOCs stage 3 | 17.1.0 |
| 2020-12 | SA#90e | SP-201045 | 0407 | - | F | Add subclause reference of MRO related attribute | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0410 | 1 | A | Correction of NRM YANG errors | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0412 | - | A | YANG improvements | 17.1.0 |
| 2020-12 | SA#90e | SP-201056 | 0414 | - | A | Add serviceProfileId and sliceProfileId to stage 3 yaml | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0419 | - | A | Update notifyThresholdCrossing to be a common notification. | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0421 | - | A | pLMNInfoList faulty attribute definition | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0423 | - | A | Fix containment relationship for EP_Transport IOC | 17.1.0 |
| 2021-03 | SA#91e | SP-210153 | 0432 | 1 | A | Correction on Dynamic5QISet IOC based on LS reply from SA2 | 17.2.0 |
| 2021-03 | SA#91e | SP-210154 | 0435 | 3 | A | Correct the NF name in definition of EP_NgU | 17.2.0 |
| 2021-03 | SA#91e | SP-210153 | 0440 | - | A | Add missing inheritance description information in the attribute definition for several IOCs | 17.2.0 |
| 2021-03 | SA#91e | SP-210153 | 0442 | 2 | A | Correct multiplicity issue for several attributes of NR NRM | 17.2.0 |
| 2021-03 | SA#91e | SP-210146 | 0445 | 2 | A | Fix containment relationship for EP_Transport IOC | 17.2.0 |
| 2021-03 | SA#91e | SP-210155 | 0457 | - | C | Remove the XML Solution set | 17.2.0 |
| 2021-03 | SA#91e | SP-210144 | 0459 | 1 | B | Update the information model definitions for network slice NRM | 17.2.0 |
| 2021-03 | SA#91e | SP-210143 | 0461 | 1 | A | Update of the PCI and DESManagementFunction | 17.2.0 |
| 2021-03 | SA#91e | SP-210154 | 0467 | 1 | A | Correction to NSI and NSSI state management | 17.2.0 |
| 2021-03 | SA#91e | SP-210155 | 0472 | - | A | YANG compilation error and missing stage 2 corrections | 17.2.0 |
| 2021-03 | SA#91e | SP-210146 | 0474 | - | A | Fix compilation and other errors | 17.2.0 |
| 2021-03 | SA#91e |  |  |  |  | Fixing CR implementation error in E.5.13 | 17.2.1 |
| 2021-06 | SA#92e | SP-210407 | 0430 | 4 | F | Correction of ServiceProfile attributes | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0479 | 1 | B | Add positioning support in RANSliceSubnetProfile | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0480 | 1 | B | OpenAPI of adding positioning support in RANSliceSubnetProfile | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0481 | 1 | B | Add synchronicity support in RANSliceSubnetProfile | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0482 | 1 | B | OpenAPI of adding synchronicity support in RANSliceSubnetProfile | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0485 | 1 | C | perfReq mapping to domain specific attributes | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0486 | 1 | B | Add reliability to CN SliceProfile | 17.3.0 |
| 2021-06 | SA#92e | SP-210401 | 0487 | 1 | B | Enhancement of NRM definition for the NWDAF - Stage 2 | 17.3.0 |
| 2021-06 | SA#92e | SP-210401 | 0488 | 1 | B | OpenAPI Enhancement of NRM definition for the NWDAF | 17.3.0 |
| 2021-06 | SA#92e | SP-210411 | 0490 | - | A | Correct the description for GNBDUFunction and EP_NgC | 17.3.0 |
| 2021-06 | SA#92e | SP-210401 | 0491 | 1 | F | Improve the readability of EP_Transport | 17.3.0 |
| 2021-06 | SA#92e | SP-210465 | 0493 | 2 | B | Add energyEfficiency attribute | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0495 | 1 | B | enhance 5GC NRM to support network slice admission control | 17.3.0 |
| 2021-06 | SA#92e | SP-210407 | 0498 | 1 | F | Add note for RRMPolicy | 17.3.0 |
| 2021-06 | SA#92e | SP-210467 | 0499 | 1 | F | Inclusive language review | 17.3.0 |
| 2021-06 | SA#92e | SP-210406 | 0501 | 1 | A | Fix editorial issue of network slice NRM | 17.3.0 |
| 2021-06 | SA#92e | SP-210406 | 0503 | 1 | A | fix inheritance relation of network slice NRM | 17.3.0 |
| 2021-06 | SA#92e | SP-210406 | 0506 | 1 | C | Correction of 5QI definitions in NRM | 17.3.0 |
| 2021-06 | SA#92e | SP-210410 | 0508 | - | F | Correction on mapping GST attributes | 17.3.0 |
| 2021-06 | SA#92e | SP-210411 | 0510 | - | A | Correct inconsistencies in definitions around network slice management | 17.3.0 |
| 2021-06 | SA#92e | SP-210406 | 0514 | 1 | A | Correction to definition for domain centralized SON | 17.3.0 |
| 2021-09 | SA#93e | SP-210871 | 0518 | - | A | YANG NR-NRM model structure repair and cleanup | 17.4.0 |
| 2021-09 | SA#93e | SP-210870 | 0520 | - | C | Use of TopSliceSubnetProfile | 17.4.0 |
| 2021-09 | SA#93e | SP-210885 | 0522 | - | A | Deprecate Top-Attr and use Top instead | 17.4.0 |
| 2021-09 | SA#93e | SP-210885 | 0524 | - | A | Fix incorrect attributes inheritance description | 17.4.0 |
| 2021-09 | SA#93e | SP-210870 | 0525 | - | B | Add survival time to CNSliceProfile | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0526 | 1 | B | Add NRM IOC definitions for N5, N70 and N71 reference points | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0527 | 1 | B | Adding NRM for N33 | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0528 | - | B | Enhance 5GC NRM to support 5G_DDNMF | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0530 | - | A | Remove the attribute definition which is not used | 17.4.0 |
| 2021-09 | SA#93e | SP-210882 | 0531 | 1 | B | Add NPN Identity on NR cell to support access control for NPN UEs | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0534 | 1 | A | Fix the issue caused by the updated NetworkSliceSubnet inheritence relationship | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0535 | 1 | F | Update logicInterfaceId of EP_transport | 17.4.0 |
| 2021-09 | SA#93e | SP-210870 | 0539 | 1 | B | Add radio spectrum support in slicing profiles | 17.4.0 |
| 2021-09 | SA#93e | SP-210870 | 0542 | - | C | Update maxPktSize and determinComm to support UL and DL requirements | 17.4.0 |
| 2021-09 | SA#93e | SP-210870 | 0543 | - | F | Remove obsolete coverageArea attribute in TopSliceSubnetProfile | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0545 |  | A | Correction for attribute description of servAttrCom | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0547 | - | A | Correcion of YAML references | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0548 | - | F | Revise description of NextHopInfo and qosProfileRefList attribute in EP_transport IOC | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0549 |  | C | Update resourceType PRB for UL (Uplink) and DL (Downlink) | 17.4.0 |
| 2021-09 | SA#93e | SP-210887 | 0551 | - | F | Correction of ServiceProfile | 17.4.0 |
| 2021-09 | SA#93e | SP-210887 | 0554 | - | B | Enhance 5G Core AMF NRM fragment | 17.4.0 |
| 2021-09 | SA#93e | SP-210885 | 0555 | - | A | Remove isINEF attribute from NEFFunction IOC | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0556 | - | A | YANG updates to correct YANG merging problems | 17.4.0 |
| 2021-09 | SA#93e | SP-210885 | 0557 | 1 | A | Fix inconsistent clauses and attributes used in TS 38.211 and TS 28.541 | 17.4.0 |
| 2021-09 | SA#93e | SP-210871 | 0558 | 1 | A | Moving RIM monitoring related attributes to NRCellDU | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0559 | 1 | C | Extend NRM fragment to support EP_transport for mid-haul | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0562 | - | B | Enhance 5G Core managed NF Profile NRM fragment | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0564 | - | F | Delete AMFInfo datatype in NRM fragment | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0566 | 1 | F | Correction of YANG Solution set | 17.4.0 |
| 2021-09 | SA#93e | SP-210867 | 0568 | - | F | Add missing openAPI definition update for S5-213508 | 17.4.0 |
| 2021-12 | SA#94e | SP-211471 | 0468 | 3 | B | Update NR NRM to support MOCN network sharing scenario | 17.5.0 |
| 2021-12 | SA#94e | SP-211472 | 0571 | - | A | Align different (abbreviated) names for support qualifier to S | 17.5.0 |
| 2021-12 | SA#94e | SP-211454 | 0575 | 1 | A | Clarify the usage of pLMNId in first entry in pLMNInfoList | 17.5.0 |
| 2021-12 | SA#94e | SP-211452 | 0576 | 1 | B | Add Stage 2 solutions to support D-LBO | 17.5.0 |
| 2021-12 | SA#94e | SP-211452 | 0577 | 1 | B | Add Stage 3 solutions to support D-LBO | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0578 | - | C | Update latency to support UL and DL requirements | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0579 | - | F | Align attribute names for CNSliceSubnetProfile | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0580 | - | F | YAML update for RRMPolicy | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0581 | 1 | B | Add attribute networkSliceSubnetType for NetworkSliceSubnet IOC | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0582 | 1 | C | Add maxnumber of PDU Sessions in NsacfInfoSnssai | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0583 | 1 | C | Add serving area information for NSACF discovery and selection | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0584 | 1 | C | Enhance NRM to support local NEF selection | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0585 | 1 | C | Update NRM to support EASDF | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0587 | 1 | F | Update relationship between GST and Network Slice NRM fragment | 17.5.0 |
| 2021-12 | SA#94e | SP-211454 | 0589 | 1 | A | cNSIId description clarificaiton | 17.5.0 |
| 2021-12 | SA#94e | SP-211454 | 0591 | 1 | A | Correct NRM for AMFRegion and AMFSet | 17.5.0 |
| 2021-12 | SA#94e | SP-211457 | 0592 |  | B | Adding transport view NRM from 5GC to EDN | 17.5.0 |
| 2021-12 | SA#94e | SP-211462 | 0595 | 1 | A | DMRO correction | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0599 | 1 | D | Introduce missing GST references | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0600 | 1 | B | Enhance 5G Core managed NF Profile NRM fragment (Stage 2) | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0601 | 1 | B | 5GC NRM enhancements for AMFFunction and ManagedNFProfile (Stage 3) | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0602 | 1 | B | NR NRM additions to support 5GC enhancements (Stage 3) | 17.5.0 |
| 2021-12 | SA#94e | SP-211454 | 0604 | - | A | Correct PLMNInfo support qualifier | 17.5.0 |
| 2021-12 | SA#94e | SP-211464 | 0606 | 1 | A | Clarify tenant relationship with ServiceProfileId | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0607 | - | F | Correction of YANG Solution set | 17.5.0 |
| 2021-12 | SA#94e | SP-211474 | 0608 | 1 | B | NRM for CHO | 17.5.0 |
| 2021-12 | SA#94e | SP-211474 | 0609 | 1 | B | NRM for CHO Stage 3 | 17.5.0 |
| 2021-12 | SA#94e | SP-211474 | 0610 | 1 | B | NRM for DAPS handover | 17.5.0 |
| 2021-12 | SA#94e | SP-211474 | 0611 | 1 | B | NRM for DAPS Stage 3 | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0612 | - | F | Remove editor notes | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0613 | - | F | Update 5GC NRM for 5G_DDNMF reference point | 17.5.0 |
| 2021-12 | SA#94e | SP-211463 | 0619 | 1 | D | Update inclusive language modification for TS 28.541 | 17.5.0 |
| 2021-12 | SA#94e | SP-211475 | 0621 | - | A | Correct the wrong reference for TS 32.160 | 17.5.0 |
| 2021-12 | SA#94e | SP-211471 | 0622 | - | B | Add YAML solution set for NG-RAN MOCN network sharing scenarios | 17.5.0 |
| 2021-12 | SA#94e | SP-211472 | 0624 | - | A | Fix stage3 definition for plmnId | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0626 | 1 | B | network slice protection on N6 interface | 17.5.0 |
| 2021-12 | SA#94e | SP-211466 | 0627 | 1 | B | network slice specific authentication | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0629 | 1 | B | Enhance NRM of UDM function | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0630 | 1 | F | Stage3 Update for UPF and PCF | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0631 | 1 | F | Introduce missing attribute nRFreqRelationRef in table of attribute properties (stage 2) | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0634 | 1 | F | Correct attribute in IOC NRCellRelation (stage 3) | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0636 | 1 | B | Enhance 5G Core managed NF Profile NRM fragment (Stage 2) | 17.5.0 |
| 2021-12 | SA#94e | SP-211473 | 0637 | 1 | B | 5GC NRM enhancements for ManagedNFProfile (Stage 3) | 17.5.0 |
| 2021-12 | SA#94e | SP-211475 | 0638 | - | A | Correct spelling of Attribute properties | 17.5.0 |
| 2022-03 | SA#95e | SP-220182 | 0633 | 2 | C | Update maximumDeviationHoTrigger | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0641 | - | F | Fix stage3 definition for 5G_DDNMF | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0643 | - | F | YANG corrections | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0644 | - | F | Fixing lists errors in AmfFunction-Single (stage 3) | 17.6.0 |
| 2022-03 | SA#95e | SP-200176 | 0645 | 1 | F | Update RANSliceSubnetProfile attributes | 17.6.0 |
| 2022-03 | SA#95e | SP-220182 | 0649 | - | F | Correct NRM fragment for DMRO Management | 17.6.0 |
| 2022-03 | SA#95e | SP-220173 | 0650 | 1 | B | Add Stage 2 solutions to support ECM | 17.6.0 |
| 2022-03 | SA#95e | SP-220176 | 0651 | 1 | F | Update energy efficiency attribute | 17.6.0 |
| 2022-03 | SA#95e | SP-220184 | 0652 | 1 | F | Alignment on NR NRM for MOCN network sharing | 17.6.0 |
| 2022-03 | SA#95e | SP-220184 | 0653 | 1 | B | Add administrativeState attribute in NROperatorCellDU | 17.6.0 |
| 2022-03 | SA#95e | SP-220179 | 0655 | - | A | Remove incorrect reference to TS 22.104 | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0658 | 1 | B | NRM enhacements for SMFFunction | 17.6.0 |
| 2022-03 | SA#95e | SP-220182 | 0659 | 1 | B | Add C-SON CCO NRM model stage3 | 17.6.0 |
| 2022-03 | SA#95e | SP-220182 | 0660 | 1 | B | Add C-SON CCO NRM model stage2 | 17.6.0 |
| 2022-03 | SA#95e | SP-220176 | 0666 | 1 | F | Clean up of eMA5SLA | 17.6.0 |
| 2022-03 | SA#95e | SP-200168 | 0667 | - | F | Update 5G NRM to solve CR clash in Figure 5.2.1.2-2 | 17.6.0 |
| 2022-03 | SA#95e | SP-200168 | 0670 | 1 | B | NRM enhancements for the SMFFunction (stage 3) | 17.6.0 |
| 2022-03 | SA#95e | SP-220178 | 0672 | - | F | Correct YANG Network Slice NRM solution set reference | 17.6.0 |
| 2022-03 | SA#95e | SP-200168 | 0674 | 1 | F | Update 5GC NRM for 5G_DDNMF | 17.6.0 |
| 2022-03 | SA#95e | SP-220179 | 0678 | - | A | Correct YANG mapping in TS document | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0679 | - | F | Correct NR YAML in TS document | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0680 | - | F | Correct 5GC YAML in TS document | 17.6.0 |
| 2022-03 | SA#95e | SP-220168 | 0681 | - | F | Correct Network Slicing YAML in TS document | 17.6.0 |
| 2022-03 | SA#95e | SP-220173 | 0682 | - | B | Add Stage 3 solutions to support ECM | 17.6.0 |
| 2022-03 | SA#96 | SP-220507 | 0642 | 2 | F | Update Figure L.2.1 and accompanying paragraph. | 17.7.0 |
| 2022-03 | SA#96 | SP-220508 | 0683 | - | F | Correct maximumDeviationHoTrigger for D-LBO | 17.7.0 |
| 2022-03 | SA#96 | SP-220497 | 0685 | - | A | Diagram fix for NRM fragment for RRM policies | 17.7.0 |
| 2022-03 | SA#96 | SP-220497 | 0689 | - | A | Fixing OpenAPI Discoverability issue in stage 3 5gcNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220497 | 0690 | - | A | Fixing OpenAPI Discoverability issue in stage 3- nrNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220497 | 0691 | - | A | Fixing OpenAPI Discoverability issue in stage 3 sliceNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220498 | 0693 | - | A | CT OpenAPI file relative-path URI references and dependence change for 5gcNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220564 | 0694 | - | F | Fixing a few issues with attribute related to nextHopInfoList in EP_transport | 17.7.0 |
| 2022-03 | SA#96 | SP-220498 | 0698 | 1 | A | OpenAPI file name and dependence change for 5gcNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220498 | 0699 | 1 | A | OpenAPI file name and dependence change for nrNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220498 | 0700 | 1 | A | OpenAPI file name and dependence change for sliceNrm.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220498 | 0702 | - | A | Correction to RRMPolicy_ IOC reference in RRMPolicyRatio IOC | 17.7.0 |
| 2022-03 | SA#96 | SP-220498 | 0704 | - | A | Add attribute properties for NetworkSliceSubnet attribute priorityLabel | 17.7.0 |
| 2022-03 | SA#96 | SP-220564 | 0705 | - | F | Fix to change Support Qualifier to S | 17.7.0 |
| 2022-03 | SA#96 | SP-220564 | 0706 | - | F | Define LogicInterfaceInfo datatype and fix attribute properties for logicInterfaceInfo | 17.7.0 |
| 2022-03 | SA#96 | SP-220564 | 0708 | 1 | F | Fixing attribute properties for ServiceProfile attribute networkSliceSharingIndicator | 17.7.0 |
| 2022-03 | SA#96 | SP-220510 | 0710 | 1 | A | Correct isOrdered-isUnique for multivalue attributes | 17.7.0 |
| 2022-03 | SA#96 | SP-220499 | 0712 | 1 | B | Network slice subnet provider capability IOC | 17.7.0 |
| 2022-03 | SA#96 | SP-220507 | 0715 | - | F | Correction on two SLA attributes | 17.7.0 |
| 2022-03 | SA#96 | SP-220507 | 0716 | - | F | Correction on attribute latency of SubnetProfiles | 17.7.0 |
| 2022-03 | SA#96 | SP-220510 | 0719 | - | A | Correction on minor errors in nrNRM.yaml | 17.7.0 |
| 2022-03 | SA#96 | SP-220510 | 0721 | - | A | Correction on the attribution definition in the wrong yaml file | 17.7.0 |
| 2022-03 | SA#96 | SP-220499 | 0722 | - | B | Add feasibility check NRM fragment | 17.7.0 |
| 2022-03 | SA#96 | SP-220509 | 0723 | - | F | Address the unnecessary reference for the yaml file | 17.7.0 |
| 2022-03 | SA#96 | SP-220510 | 0727 | - | A | Fix BWP association in NRCellDU | 17.7.0 |
| 2022-03 | SA#96 | SP-220510 | 0729 | - | A | Update 5QI set description - YANG module | 17.7.0 |
| 2022-03 | SA#96 | SP-220510 | 0731 | - | A | Update 5QI set reference attribute definition | 17.7.0 |
| 2022-03 | SA#96 | SP-220511 | 0711 | - | B | Access specific GST configuration | 18.0.0 |
| 2022-09 | SA#97e | SP-220847 | 0732 | - | B | Enhance 5G Core managed NF Profile NRM fragment | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0733 | - | B | NRM enhancements for UPFFunction | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0734 | - | C | NRM enhancements for NSSFFunction | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0735 | 1 | B | NRM enhancements for PCFFunction | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0736 | 1 | B | NRM enhancements for UDMFunction | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0737 | 1 | B | NRM enhancements for UDRFunction | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0741 | 1 | A | FiveQICharacteristics inheritance issue and reference issue in stage 3 | 18.1.0 |
| 2022-09 | SA#97e | SP-220859 | 0743 | 1 | A | Fix inconsistency in AMFFunction stage 2 and stage 3 | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0745 | - | A | Correction to DESManagementFunction and CESManagementFunction | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0748 | 1 | A | Correction to serviceType attribute | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0755 | 1 | B | Add BWP Set configuration support in NRM (stage 2) | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0756 | - | B | Add BWP Set configuration support in NRM (stage 3, YANG) | 18.1.0 |
| 2022-09 | SA#97e | SP-220859 | 0758 | 1 | A | Update stage2 and stage3 definition for FeasibilityCheckAndReservationJob | 18.1.0 |
| 2022-09 | SA#97e | SP-220859 | 0760 | - | A | Add missing notifyMOIChanges in configuration notification table | 18.1.0 |
| 2022-09 | SA#97e | SP-220861 | 0762 | - | A | Correction on two SLA attributes | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0764 | 1 | B | Add BWP set support to NRM (Stage3, YAML) | 18.1.0 |
| 2022-09 | SA#97e | SP-220847 | 0765 | 1 | B | Update NWDAFFunction IOC to support management and control purpose | 18.1.0 |
| 2022-09 | SA#97e | SP-220859 | 0767 | 1 | A | Update EASDF IOC | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0770 | - | F | YANG Corrections | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0772 | - | A | Add missing attributes n6Protection and nssaaSupport defined in CNSliceSubnetProfile to TopSliceSubnetProfile | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0774 | - | A | fix TaiList issues in stage 3 in TS28541_5gcNrm.yaml | 18.1.0 |
| 2022-09 | SA#97e | SP-220849 | 0784 | - | A | Correction to coverageAreaTAList | 18.1.0 |
| 2022-09 | SA#97e |  |  |  |  | Removing duplicated content due to CRs including duplicated changes (MCC). | 18.1.1 |
| 2022-09 | SA#97e |  |  |  |  | Aligning OpenAPI code from FORGE | 18.1.2 |
| 2023-01 | SA#98e | SP-221188 | 0753 | 3 | C | Add Enhanced QoS support in NRM (stage 2) | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0754 | 3 | C | Add Enhanced QoS support in NRM (stage 3, YANG) | 18.2.0 |
| 2023-01 | SA#98e | SP-221172 | 0785 | - | F | YANG Corrections in Word TS | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0787 | - | B | NRM enhancements for AUSFFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0788 | 1 | B | NRM enhancements for NEFFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0789 | - | B | NRM enhancements for NSACFFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0790 | 1 | B | NRM enhancements for NWDAFFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0791 | 1 | B | NRM enhancements for SCPFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0792 | 1 | B | NRM enhancements for SEPPFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0793 | - | B | NRM enhancements for UDSFFunction | 18.2.0 |
| 2023-01 | SA#98e | SP-221169 | 0796 | - | A | Correcting name of nSInstanceId | 18.2.0 |
| 2023-01 | SA#98e | SP-221167 | 0803 | 2 | A | Correction to multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC | 18.2.0 |
| 2023-01 | SA#98e | SP-221169 | 0806 | - | A | Correction to GSMA NG 116 reference for KPIMonitoring | 18.2.0 |
| 2023-01 | SA#98e | SP-221169 | 0809 | - | A | Correction to ServiceProfile attribute v2XCommModels name in YAML defintion | 18.2.0 |
| 2023-01 | SA#98e | SP-221169 | 0812 | - | A | Correction to inconsistencies in GNBCUCPFunction definition | 18.2.0 |
| 2023-01 | SA#98e | SP-221173 | 0816 | 1 | A | Adding YANG begin and End markers | 18.2.0 |
| 2023-01 | SA#98e | SP-221167 | 0820 | 1 | A | Address Editor's Note for the description of FeasibilityCheckAndReservationJob (6.3.9) | 18.2.0 |
| 2023-01 | SA#98e | SP-221180 | 0822 | - | A | Correct the definition for cellLocalId to support MOCN network sharing sceanrio (6.3.11) | 18.2.0 |
| 2023-01 | SA#98e | SP-221188 | 0823 | 1 | B | Update NWDAFFunction IOC to support management and control purpose | 18.2.0 |
| 2023-01 | SA#98e | SP-221181 | 0825 | - | A | Correct the misalignment information between stage2 and stage3 | 18.2.0 |
| 2023-01 | SA#98e | SP-221167 | 0828 | - | A | Replacing Support Qualifier with S | 18.2.0 |
| 2023-01 | SA#98e | SP-221182 | 0834 | - | A | Consistency in use of servAttrCom | 18.2.0 |
| 2023-01 | SA#98e | SP-221182 | 0837 | 1 | A | Correct kPIList | 18.2.0 |
| 2023-01 | SA#98e | SP-221182 | 0840 | 2 | A | Correct periodicityList | 18.2.0 |
| 2023-01 | SA#98e | SP-221167 | 0843 | 1 | A | Correct network slice state management table | 18.2.0 |
| 2023-01 | SA#98e |  |  |  |  | Fixing some implementation errors | 18.2.1 |
| 2023-02 |  |  |  |  |  | Removing revision marks | 18.2.2 |
| 2023-03 | SA#99 | SP-230196 | 0855 | - | A | Fix missing reference to mid-haul interface for EP Transport | 18.3.0 |
| 2023-03 | SA#99 | SP-230196 | 0857 | - | A | Fix duplicated SST attribute in RANSliceSubnetProfile | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0859 | 1 | B | NRM enhancements for SMSFFunction | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0860 | 1 | B | NRM enhancements for NRFFunction | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0861 | 1 | B | NRM enhancements for LMFFunction | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0862 | 1 | B | NRM enhancements for AFFunction | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0863 | - | B | NRM enhancements for EASDFFunction | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0864 | 1 | B | NRM enhancements for NSSAAFFunction | 18.3.0 |
| 2023-03 | SA#99 | SP-230199 | 0867 | 1 | A | Remove redundant stage 3 definition for Mnc and PlmnId | 18.3.0 |
| 2023-03 | SA#99 | SP-230196 | 0870 | 1 | A | Fix IpAddr definition and references | 18.3.0 |
| 2023-03 | SA#99 | SP-230207 | 0872 | - | A | ManagedNFProfile stage3 not consistent with Stage 2 | 18.3.0 |
| 2023-03 | SA#99 | SP-230207 | 0874 | 1 | A | Correct issues for feasibility check and resource reservation NRM fragment | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0875 | 1 | B | Add Enhanced QoS support in NRM (stage 3, YAML) | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0876 | 1 | F | Update NRM fragment for 5QI configuration for NG-RAN | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0877 | 1 | F | Correct the description and definition for NWDAFFunction IOC | 18.3.0 |
| 2023-03 | SA#99 | SP-230200 | 0881 | 1 | A | Clarify Monut information clauses | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0882 | - | F | Correct the RRMPolicyRatio Description | 18.3.0 |
| 2023-03 | SA#99 | SP-230206 | 0888 | 1 | C | Add Annex recommending QoS model usage | 18.3.0 |
| 2023-03 | SA#99 | SP-230196 | 0890 | 2 | A | fixing coverageArea | 18.3.0 |
| 2023-03 | SA#99 | SP-230208 | 0891 | - | F | YANG Corrections | 18.3.0 |
| 2023-04 | SA#99 |  |  |  |  | Editorial corrections in H.5.12 | 18.3.1 |
| 2023-06 | SA#100 | SP-230653 | 0800 | 5 | B | Add NetworkSliceController and NetworkSliceSubnetController IOCs to support asynchronous LCM operations | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0894 | 1 | B | NRM enhancements for NRFFunction | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0895 | 1 | B | NRM enhancements for CHFFunction | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0896 | 1 | B | NRM enhancements for MFAFFunction | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0897 | - | B | NRM enhancements for DCCFFunction | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0898 | - | B | NRM enhancements for class diagram | 18.4.0 |
| 2023-06 | SA#100 | SP-230649 | 0900 | - | A | Correction to dCHOControl by adding definition to attribute properties table | 18.4.0 |
| 2023-06 | SA#100 | SP-230681 | 0903 | 1 | A | Correction to multiplicity definition to nRPciList and stage 3 implementation of both NRPciList and CSonPciList | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0904 | - | F | Corrections to missing properties for a few attributes | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0905 | 1 | C | Improve EP Transport model to clarify connection point info | 18.4.0 |
| 2023-06 | SA#100 | SP-230648 | 0908 | 1 | A | clean up of incorrect use of multiplicity isOrdered isUnique and isNullable in attribute properties table | 18.4.0 |
| 2023-06 | SA#100 | SP-230658 | 0910 | 1 | A | Correct the feasibility check and resource reservation NRM fragment | 18.4.0 |
| 2023-06 | SA#100 | SP-230658 | 0912 | - | A | Correct issues for NR NRM | 18.4.0 |
| 2023-06 | SA#100 | SP-230658 | 0914 | - | A | Update SliceNRM YAML to align with stage2 | 18.4.0 |
| 2023-06 | SA#100 | SP-230651 | 0916 | - | F | YANG Corrections | 18.4.0 |
| 2023-06 | SA#100 | SP-230657 | 0921 | - | F | Update NRM enhancements for NWDAFFunction | 18.4.0 |
| 2023-06 | SA#100 | SP-230658 | 0927 | 1 | A | Fixing coverageArea | 18.4.0 |
| 2023-06 | SA#100 | SP-230651 | 0931 | 1 | F | Several editorial Corrections | 18.4.0 |
| 2023-06 | SA#100 |  |  |  |  | Various CR implementation corrections in YANG code | 18.4.1 |
| 2023-09 | SA#101 | SP-230953 | 0944 |  | A | Correct attribute name for frequency prediction | 18.5.0 |
| 2023-09 | SA#101 | SP-230964 | 0950 | 1 | A | Update Annex L with mapping table | 18.5.0 |
| 2023-09 | SA#101 | SP-230953 | 0952 |  | A | Update reference to GST | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0955 | 1 | A | Rel-18 CR TS 28.541 Correction to type defintion of coverageAreaTAList | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0961 |  | B | TS28.541 Rel18 NRM enhancements for NRFFunction | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0962 |  | B | TS28.541 Rel18 NRM enhancements for BSFFunction | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0963 |  | B | TS28.541 Rel18 NRM enhancements for AANFFunction | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0964 |  | B | TS28.541 Rel18 NRM enhancements for TSCTSFFunction | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0965 |  | B | TS28.541 Rel18 NRM enhancements for GMLCFunction | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0966 | 1 | B | TS28.541 Rel18 NRM enhancements for class Diagram | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0969 | 1 | A | TS28.541 Rel18 Removing redundant Tai definition in NR NRM | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0970 | 2 | C | TS28.541 Rel18 Improve LogicalInterfaceInfo model to add support of static route | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0973 | 1 | A | TS28.541 Rel18 fix small inconsitent issues in stage2, typo in stage 2 and 3 | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0976 | 2 | A | TS28.541 Rel18 fix a few inheritance diagram issues in NR NRM | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0979 | 1 | A | Update RRMPolicyRatio | 18.5.0 |
| 2023-09 | SA#101 | SP-230944 | 0981 |  | A | Rel-18 CR for TS28.541 Fix wrong annex reference | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0984 |  | A | Remove duplicated notifications | 18.5.0 |
| 2023-09 | SA#101 | SP-230965 | 0986 |  | A | Re-15 CR TS 28.541 Correct issues for NRM fragment for NG-RAN MOCN sharing | 18.5.0 |
| 2023-09 | SA#101 | SP-230958 | 0989 | 1 | A | Rel-18 CR TS 28.541 Correct the issues for the attribute with the ENUM type for network slicing NRM fragment | 18.5.0 |
| 2023-09 | SA#101 | SP-230966 | 0990 | 1 | C | Rel-18 CR 28.541 Update recommended QoS model usage for Shared RAN | 18.5.0 |
| 2023-09 | SA#101 | SP-230966 | 0991 | 1 | C | Rel-18 CR 28.541 Update NRM for operator specific QoS model for RAN Sharing | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0992 |  | F | Rel-18 CR TS28.541 Correct the stage2 and stage3 issues for BWP configuration | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 0993 | 1 | B | Rel-18 CR TS28.541 Update the feasiibility check NRM fragment to support the missing requirements | 18.5.0 |
| 2023-09 | SA#101 | SP-230941 | 0997 | 1 | A | Correction on DeterministicComm data type | 18.5.0 |
| 2023-09 | SA#101 | SP-230949 | 0999 |  | A | Correction of EcmConnectionInfo | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 1001 | 1 | B | Rel-18 CR 28.541 Non IP Support Stage-2 | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 1002 | 1 | B | Rel-18 CR 28.541 Non IP Support Stage-3 | 18.5.0 |
| 2023-09 | SA#101 | SP-230973 | 1005 | 1 | B | Rel-18 CR 28.541 NTN Location Restriction Stage-2 | 18.5.0 |
| 2023-09 | SA#101 | SP-230973 | 1006 | 1 | B | Rel-18 CR 28.541 NTN Location Restriction Stage 3 | 18.5.0 |
| 2023-09 | SA#101 | SP-230973 | 1007 | 1 | B | Rel-18 CR TS 28.541 Add NRM Info Model definitions for NTN management(stage2) | 18.5.0 |
| 2023-09 | SA#101 | SP-230973 | 1008 | 1 | B | Rel-18 CR TS 28.541 Add NRM Info Model definitions for NTN management(Stage3 Yaml) | 18.5.0 |
| 2023-09 | SA#101 | SP-230967 | 1012 | 1 | B | Update NRM enhancements for NWDAFFunction | 18.5.0 |
| 2023-09 | SA#101 | SP-230973 | 1013 | 1 | B | Rel-18 CR 28.541 NTN Coverage Availability Information Configuration Stage 2 | 18.5.0 |
| 2023-09 | SA#101 | SP-230947 | 1014 | 1 | F | Rel-18 CR TS 28.541 Add missing Attribute Constraints for NROperatorCellDU | 18.5.0 |
| 2023-09 | SA#101 | SP-230975 | 1015 | 1 | B | Rel-18 CR TS 28.541 UL and DL reliability configuration in slice and subnet profiles | 18.5.0 |
| 2023-09 | SA#101 | SP-230973 | 1016 | 1 | B | Rel-18 CR 28.541 NTN Coverage Availability Information Configuration Stage 3 | 18.5.0 |
| 2024-01 | SA#102 | SP-231481 | 0858 | 9 | B | Rel-18 CR TS 28.541 Add NRM for network slice isolation | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1018 | - | B | TS28.541 Rel18 NRM enhancements for NRFFunction and AmfInfo and SmfInfo | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1019 | - | B | TS28.541 Rel18 NRM enhancements for UpfInfo and PcfInfo and NetInfo | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1020 | - | B | TS28.541 Rel18 NRM enhancements for MBUPFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1021 | - | B | TS28.541 Rel18 NRM enhancements for MBSMFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1022 | 1 | B | TS28.541 Rel18 NRM enhancements for class diagram | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1024 | - | A | TS28.541 Rel18 correction to stage3 of NROperatorCellDU containment | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1026 | - | A | TS28.541 Rel18 correction to NPNIdentity stage2 stage 3 issue | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1027 | 1 | C | TS28.541 Rel18 Move normative OpenAPI and YANG code to Forge | 18.6.0 |
| 2024-01 | SA#102 | SP-231477 | 1029 | - | B | Rel-18 CR TS 28.541 Add YAML solution set for NRM for operator specific QoS model for RAN sharing | 18.6.0 |
| 2024-01 | SA#102 | SP-231477 | 1030 | - | F | Rel-18 CR TS28.541 Udpate clause O.3 RAN Sharing usage recommendation | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1033 | - | B | Rel-18 CR TS 28.541 Fix inconsitencies related to network slice SLA attribute availability | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1034 | 1 | B | Rel-18 CR TS 28.541 Fix inconsitencies related to network slice SLA attribute kPIMonitoring | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1036 | - | B | Rel-18 CR TS 28.541 Fix inconsitencies related to network slice SLA attribute maxDLDataVolume and maxULDataVolume | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1038 | 2 | B | Rel-18 CR TS 28.541 Add support for GSMA attributes related to data networks | 18.6.0 |
| 2024-01 | SA#102 | SP-231487 | 1041 | 1 | A | TS28.541 Rel18 correction to data type property for sD | 18.6.0 |
| 2024-01 | SA#102 | SP-231494 | 1043 | 1 | F | Rel-18 CR 28.541 YANG Corrections and inVariant | 18.6.0 |
| 2024-01 | SA#102 | SP-231457 | 1045 | 1 | A | Rel-18 CR TS 28.541 Correct issues for the attribute with the ENUM type for NR NRM fragment | 18.6.0 |
| 2024-01 | SA#102 | SP-231498 | 1046 | - | C | Rel-18 CR TS 28.541 Fix Slice NRM YAML definition for dLReliability and uLReliability | 18.6.0 |
| 2024-01 | SA#102 | SP-231477 | 1048 | 1 | C | Rel-18 CR 28.541 Update recommended QoS model usage for Shared RAN (yang update) | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1058 | - | B | TS28.541 Rel18 NRM enhancements for NRFFunction phase 4 | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1059 | - | B | TS28.541 Rel18 NRM enhancements for MNPFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231487 | 1061 | - | A | TS28.541 Rel18 correction to NSSF interface between NWDAF and NSSF | 18.6.0 |
| 2024-01 | SA#102 | SP-231487 | 1066 | - | A | TS28.541 Rel18 correction to DRACHOptimizationFunction stage 3 issue | 18.6.0 |
| 2024-01 | SA#102 | SP-231487 | 1069 | - | A | TS28.541 Rel18 correction to 5GC resourceType allowed values | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1086 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in DelayTolerance | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1087 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in DeterministicComm | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1088 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in XLThpt | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1089 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in MaxPktSize | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1090 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in MaxNumberofPDUSessions | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1091 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in KPIMonitoring | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1092 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in TermDensity | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1093 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in Positioning | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1094 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in Synchronicity | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1095 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in EnergyEfficiency | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1096 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in NSSAASupport | 18.6.0 |
| 2024-01 | SA#102 | SP-231494 | 1097 | - | F | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in MaxNumberofUEs | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1098 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in RadioSpectrum | 18.6.0 |
| 2024-01 | SA#102 | SP-231494 | 1099 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in UserMgmtOpen | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1100 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in V2XCommMode | 18.6.0 |
| 2024-01 | SA#102 | SP-231493 | 1101 | - | A | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in NBIoT | 18.6.0 |
| 2024-01 | SA#102 | SP-231494 | 1102 | - | F | Rel-18 CR TS 28.541 Corrections to servAttrCom attribute in NonIPSupport | 18.6.0 |
| 2024-01 | SA#102 | SP-231457 | 1103 | 1 | A | Rel-18 Correct dnai and dnaiList attribute definitions | 18.6.0 |
| 2024-01 | SA#102 | SP-231457 | 1105 | - | A | Rel-18 CR TS 28.541 Correct the issues for NrNrm YAML file | 18.6.0 |
| 2024-01 | SA#102 | SP-231459 | 1106 | 1 | B | Add NRM for AnLF | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1107 | - | F | Clarification on the usage of sNssaiInfoList in multiple places | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1108 | - | F | Update description of ExtSnssai | 18.6.0 |
| 2024-01 | SA#102 | SP-231465 | 1109 | 1 | B | Rel-18 CR 28.541 Add energy saving attributes to IOC UPFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231483 | 1110 | 1 | B | Update definitions of Ephemeris | 18.6.0 |
| 2024-01 | SA#102 | SP-231478 | 1111 | - | B | Stage 3 Add FL related attributes in NWDAFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231478 | 1112 | - | B | Add FL related attributes in NWDAFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231498 | 1114 | 1 | B | Rel-18 CR 28.541 Modify the attributes of UL and DL latency in RAN | 18.6.0 |
| 2024-01 | SA#102 | SP-231478 | 1120 | - | F | Correction of attributes defined in NWDAFFunction | 18.6.0 |
| 2024-01 | SA#102 | SP-231463 | 1121 | 2 | B | Add satellite backhaul information for QoS monitoring per QoS flow per UE | 18.6.0 |
| 2024-01 | SA#102 | SP-231458 | 1127 | 1 | F | Rel-18 CR 28.541 Correct redundant attribute definitions | 18.6.0 |
| 2024-01 | SA#102 | SP-231492 | 1129 | - | A | Rel-18 Ts 28.541 Correction of yang code for sliceprofile | 18.6.0 |
| 2024-01 | SA#102 | SP-231486 | 1135 | 1 | A | Rel-18 CR 28.541 Correction of attribute properties | 18.6.0 |
| 2024-01 | SA#102 | SP-231494 | 1136 | - | F | Rel-18 CR 28.541 YANG Corrections (SA5#152) | 18.6.0 |
| 2024-01 | SA#102 |  |  |  |  | Correct reference number for TS 38.331, update figure and clause numbering | 18.6.1 |
| 2024-03 | SA#103 | SP-240171 | 1031 | 5 | B | Rel-18 CR TS 28.541 Address EP_F1 issue for MOCN RAN sharing scenario | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1138 | 1 | B | Rel-18 CR TS28.541 Adding MCE ID | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1140 |  | F | TS28.541 Rel18 correction to Schema definition Issues for SubNetwork and ManagedElement of OpenAPI SS | 18.7.0 |
| 2024-03 | SA#103 | SP-240395 | 1141 |  | B | TS28.541 Rel18 NRM enhancements for NfInfo and PcscfInfo | 18.7.0 |
| 2024-03 | SA#103 | SP-240395 | 1142 | 1 | C | Rel-18 CR TS 28.541 Enhancement to clarify Network slice SLA requirement for inter-packet delay variation | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1143 | 1 | F | Rel-18 CR TS 28.541 Correction to attribute constriants of IsolationProfile | 18.7.0 |
| 2024-03 | SA#103 | SP-240185 | 1145 | 1 | A | Rel-18 CR TS 28.541 Clarify attribute constraints for SliceProfile | 18.7.0 |
| 2024-03 | SA#103 | SP-240155 | 1146 | 1 | B | Add AIML support for legacy features | 18.7.0 |
| 2024-03 | SA#103 | SP-240149 | 1147 | 1 | B | Rel-18 CR TS 28.541 Add new attribute to describe the mapping relationship between DNAI and satellite ID | 18.7.0 |
| 2024-03 | SA#103 | SP-240149 | 1148 | 1 | B | Rel-18 CR 28.541 Add satellite backhaul information to AMFFunction | 18.7.0 |
| 2024-03 | SA#103 | SP-240183 | 1151 |  | A | TS28.541 Rel18 correction to QFPacketDelayThresholdsType | 18.7.0 |
| 2024-03 | SA#103 | SP-240183 | 1154 |  | A | TS28.541 Rel18 corrections to ManagedFunction containment implementation in OpenAPI SS | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1155 | 1 | F | R18 CR TS 28.541 Correct the expression of parameter S in Attribute constraints | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1156 |  | F | R18 CR TS 28.541 Remove the duplications in attributes constraints of NRCellDU | 18.7.0 |
| 2024-03 | SA#103 | SP-240185 | 1158 |  | A | Rel-18 CR TS 28.541 Correct the referece in attributes of qOffsetRangeList and beamIndex | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1159 | 1 | F | R18 CR TS 28.541 Correct the clauses' number in Clause 5.3 | 18.7.0 |
| 2024-03 | SA#103 | SP-240395 | 1160 | 1 | C | Rel-18 CR TS 28.541 Slice Validity | 18.7.0 |
| 2024-03 | SA#103 | SP-240395 | 1161 | 1 | F | Rel-18 CR TS 28.541 Correct definition for FeasibilityCheckAndReservationJob | 18.7.0 |
| 2024-03 | SA#103 | SP-240395 | 1162 | 1 | B | Rel-18 CR TS 28.541 Add missing name contain information for ExternalGNBCUCPFunction and ExtenralGNBDUFunction (satge2 and stage3 yaml) | 18.7.0 |
| 2024-03 | SA#103 | SP-240186 | 1167 | 1 | F | Rel-18 CR 28.541 YANG Corrections | 18.7.0 |
| 2024-03 | SA#103 | SP-240179 | 1170 | 1 | F | Correction of inheritance relationships figure name for NTN management support | 18.7.0 |
| 2024-03 | SA#103 | SP-240164 | 1171 | 1 | F | Rel-18 CR TS 28.541 Align Annex P with Serviceprofile | 18.7.0 |
| 2024-03 | SA#103 | SP-240164 | 1172 | 1 | F | Rel-18 CR TS 28.541 Align slice imported information table with the class diagram | 18.7.0 |
| 2024-06 | SA#104 | SP-240804 | 1175 | 1 | A | Rel-18 CR TS 28.541 add missing attributes of ManagedNFProfile | 18.8.0 |
| 2024-06 | SA#104 | SP-240814 | 1178 | 1 | A | Rel-18 CR TS 28.541 Correct support qualifier condition in ServAttrCom data type | 18.8.0 |
| 2024-06 | SA#104 | SP-240822 | 1182 | 1 | A | Rel-18 CR TS 28.541 Remove presence defintion from support qualifier condition in NetworkSliceSubnet IOC | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1187 | - | F | Rel-18 CR TS 28.541 Update the description of uLPktDelayVariation and dLPktDelayVariation | 18.8.0 |
| 2024-06 | SA#104 | SP-240839 | 1197 | - | F | Rel-18 CR TS 28.541 Correct attribute names servingNfTypeList and servingNfSetIdList | 18.8.0 |
| 2024-06 | SA#104 | SP-240839 | 1204 | - | F | Rel-18 CR TS 28.541 Add and update attribute descriptions | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1206 | 1 | F | TS28.541 Rel18 correction to Forge reference - MCC: the change is Annex D could not be implemented due to Void clause | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1207 | - | F | TS28.541 Rel18 Rapporteur clean up for general and small issues | 18.8.0 |
| 2024-06 | SA#104 | SP-240814 | 1208 | 1 | A | TS28.541 Rel18 correction to using ENUM as dataType | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1211 | 1 | F | Rel-18 CR 28.541 Add missing datatype import, fix attribute properties | 18.8.0 |
| 2024-06 | SA#104 | SP-240804 | 1213 | 1 | A | Rel-18 CR TS 28.541 Correct some type errors of attribute properties in 5.4 | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1214 | 1 | B | Rel-18 CR TS28.541 Adding Mapped cell ID | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1215 | 1 | B | Rel-18 CR TS28.541 Adding NTN fragments for LMF on UE location verification | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1216 | 1 | F | Rel-18 CR for TS28.541 add missing Attribute properties for NrfInfo | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1218 | - | F | Rel-18 CR 28.541 YANG Corrections | 18.8.0 |
| 2024-06 | SA#104 | SP-240804 | 1225 | - | A | Rel-18 CR TS 28.541 correction for allowedSNPNs | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1227 | 1 | F | Rel-18 CR TS 28.541 corrections for PCFInfo | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1228 | 1 | F | Rel-18 CR TS 28.541 corrections for SMFInfo | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1229 | 1 | F | Rel-18 CR TS 28.541 corrections for UPFInfo | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1230 | 1 | F | Rel-18 CR TS 28.541 corrections of attributes of amfInfo | 18.8.0 |
| 2024-06 | SA#104 | SP-240804 | 1236 | - | A | Rel-18 CR TS 28.541 Remove presence defintion from support qualifier condition in data types of network slice NRM | 18.8.0 |
| 2024-06 | SA#104 | SP-240804 | 1243 | - | A | Rel-18 CR TS 28.541 Correct undefined dataType of attribute properties in clause 6.4 | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1244 | 1 | F | Rel-18 CR TS 28.541 Remove duplicate definition of NPNIdentity with TS 28.622 | 18.8.0 |
| 2024-06 | SA#104 | SP-240821 | 1245 | - | F | Rel-18 CR TS 28.541 add missing definition for attributes of BsfInfo | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1247 | - | F | TS28.541 Rel18 remove MnS root in stage 3-yaml | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1248 | - | F | TS28.541 Rel18 CR Adding MCE ID Yaml SS | 18.8.0 |
| 2024-06 | SA#104 | SP-240814 | 1251 | - | A | Rel18 correction to use 'data type' as a dataType in Attribute properties in NR NRM | 18.8.0 |
| 2024-06 | SA#104 | SP-240814 | 1254 | 1 | A | Rel-18 CR 28.541 correction on PredefinedPccRuleSet | 18.8.0 |
| 2024-06 | SA#104 | SP-240814 | 1257 | 1 | A | Rel-18 CR TS 28.541 Add missing definition of RIM related parameters | 18.8.0 |
| 2024-06 | SA#104 | SP-240814 | 1260 | 1 | A | Rel-18 CR TS 28.541 corrections on the inconsistent attribute names | 18.8.0 |
| 2024-06 | SA#104 | SP-240830 | 1261 | 1 | F | Rel-18 CR TS 28.541 updates to the terminology for ML entity | 18.8.0 |
| 2024-06 | SA#104 | SP-240824 | 1264 | - | F | Rel-18 CR TS 28.541 Stage 3 Add mdtUserConsentReqList attribute to YAML | 18.8.0 |
| 2024-06 | SA#104 | SP-240804 | 1269 | 1 | A | Rel-18 CR 28.541 Add missing NRNetwork and EUTraNetwork definitions | 18.8.0 |
| 2024-06 | SA#104 | SP-240809 | 1273 | - | F | Rel-18 CR 28.541 YANG Corrections | 18.8.0 |
| 2024-06 | SA#104 | SP-240830 | 1274 | 1 | F | Rel-18 CR TS 28.541 Add missing AIML support for AnLFFunction | 18.8.0 |
| 2024-09 | SA#105 | SP-241163 | 1276 | - | A | Rel-18 CR 28.541 Update the definition for NRNetwork and EUtraNetwork | 18.9.0 |
| 2024-09 | SA#105 | SP-241163 | 1289 | 1 | A | Rel-18 CR TS 28.541 Correct the capitalisation of stereotype in figure 5.2.1.1-25 | 18.9.0 |
| 2024-09 | SA#105 | SP-241173 | 1291 | 1 | F | Rel-18 CR TS 28.541 Correct some type errors of attribute names | 18.9.0 |
| 2024-09 | SA#105 | SP-241169 | 1295 | - | A | Rel-18 CR TS 28.541 Align data types for attributes rimRSMonitoringStartTime and rimRSMonitoringStopTime - Stage 2 and Stage 3 | 18.9.0 |
| 2024-09 | SA#105 | SP-241169 | 1299 | - | A | Rel18 correction to NFProfile used in NRFFunction | 18.9.0 |
| 2024-09 | SA#105 | SP-241173 | 1301 | 1 | F | Rel18 correction to attribute definition to InterfaceUpfInfoItem and IpInterface | 18.9.0 |
| 2024-09 | SA#105 | SP-241169 | 1311 | 1 | A | Rel-18 CR 28.541 correction on LMF related interface | 18.9.0 |
| 2024-09 | SA#105 | SP-241163 | 1316 | - | A | Rel-18 CR 28.541 YANG mount information for MeContext | 18.9.0 |
| 2024-09 | SA#105 | SP-241169 | 1321 | 1 | A | Rel-18 CR TS 28.541 Correct the NRM for RACH function | 18.9.0 |
| 2024-09 | SA#105 | SP-241196 | 1327 | 1 | F | Rel-18 CR TS 28.541 correct the description of NTN | 18.9.0 |
| 2024-09 | SA#105 | SP-241190 | 1330 | 1 | F | Rel-18 TS 28.541 Correct attribute constraints | 18.9.0 |
| 2024-09 | SA#105 | SP-241181 | 1332 | 1 | F | Rel-18 TS 28.541 Correct AIML associations | 18.9.0 |
| 2024-09 | SA#105 | SP-241163 | 1334 |  | A | Rel-18 CR TS 28.541 Move InterPlmnFQDN to ManagedNFProfile | 18.9.0 |
| 2024-09 | SA#105 | SP-241173 | 1336 | 2 | F | Rel-18 CR TS 28.541 Align with TS 38.423 to introduce NRM definitions for NG-RAN energy cost Reporting supporting | 18.9.0 |
| 2024-09 | SA#105 | SP-241199 | 1338 | 1 | F | Rel-18 CR TS 28.541 correction to misalignment of stage 3 for qFMonitoredSatelliteBackhaulCategories | 18.9.0 |
| 2024-09 | SA#105 | SP-241169 | 1342 | 1 | A | Rel-18 CR TS 28.541 Fix allowedValues property issues | 18.9.0 |
| 2024-09 | SA#105 | SP-241195 | 1347 | 1 | F | Rel-18 CR TS 28.541 Fix wrong attributes | 18.9.0 |
| 2024-12 | SA#106 | SP-241635 | 1305 | 3 | A | Correction to misalignment of stage 2 and stage 3 for CellIndividualOffset and QOffsetRangeList | 18.10.0 |
| 2024-12 | SA#106 | SP-241651 | 1355 | - | F | Rel-18 CR TS 28.541 Correct both stage2 and stage3 for NRNetwork and EUtraNetwork NRM fragement | 18.10.0 |
| 2024-12 | SA#106 | SP-241635 | 1365 | 1 | A | Rel-18 CR TS 28.541 Align allowed value for ueAccProbabilityDist | 18.10.0 |
| 2024-12 | SA#106 | SP-241651 | 1367 | 1 | F | Rel-18 CR TS 28.541 Correct the reference of alarm notifications | 18.10.0 |
| 2024-12 | SA#106 | SP-241632 | 1375 | - | A | Rel-18 CR TS 28.541 Correction on the property of attibutes | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1377 | - | F | Rel-18 CR TS 28.541 Correction to sliceAvailability and sliceSubnetAvailability | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1379 | 1 | B | Rel-18 CR TS 28.541 OpenAPI SS Stage 3 for energy cost mapping rule | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1381 | 1 | B | Rel-18 CR TS 28.541 Corrections to energy cost mapping rule description | 18.10.0 |
| 2024-12 | SA#106 | SP-241632 | 1387 | - | A | Rel-18 CR TS28.541 Fix Documentation column of NRCellDU.pLMNInfoList | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1389 | 1 | F | Rel-18 CR 28.541 YANG Corrections SA5-157 | 18.10.0 |
| 2024-12 | SA#106 | SP-241632 | 1395 | - | A | Rel-18 CR TS 28.541 Correct isWritable property for Slicing NRM | 18.10.0 |
| 2024-12 | SA#106 | SP-241651 | 1398 | - | F | Rel-18 CR TS 28.541 Correct the openAPI definition for maxNumberofUEs to align with satge 2 definition | 18.10.0 |
| 2024-12 | SA#106 | SP-241635 | 1402 | - | A | Rel-18 CR TS 28.541 Corrections on PLMNInfo DataType | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1409 | 1 | F | Rel18 CR TS28.541 correction to default value | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1411 | - | F | Rel18 CR TS28.541 correction to stage 2 and stage 3 attribute name inconsistence | 18.10.0 |
| 2024-12 | SA#106 | SP-241632 | 1414 | 1 | A | Rel-18 CR TS 28.541 Update configuration parameters for GST translation | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1416 | - | F | Rel18 CR TS28541 correction to stage 2 and stage 3 for blockListEntry and blockListEntryIdleMode | 18.10.0 |
| 2024-12 | SA#106 | SP-241641 | 1419 | - | F | Rel-18 CR TS 28.541 Fix some incorrect reference numbers | 18.10.0 |
| 2024-12 | SA#106 | SP-241632 | 1428 | - | A | Rel-18 CR 28.541 correction on DefaultNotificationSubscription | 18.10.0 |
| 2024-12 | SA#106 | SP-241651 | 1430 | 1 | F | Rel-18 CR 28.541 correction on dnnInfoList | 18.10.0 |
| 2024-12 | SA#106 | SP-241650 | 1436 | 1 | F | Rel-18 CR TS 28.541 Correction of attributes | 18.10.0 |
| 2024-12 | SA#106 | SP-241657 | 1438 | 1 | F | Rel-18 CR TS28.541 Delete unused attributes blockedDurStartTime and blockedDurEndTime | 18.10.0 |
| 2025-01 | SA#106 |  |  |  |  | Adding YANG files as attachments | 18.10.1 |
| 2025-03 | SA#107 | SP-250163 | 1443 | - | F | Rel-18 CR TS 28.541 Clarify the relation between servedNfInfo and NfInfo | 18.11.0 |
| 2025-03 | SA#107 | SP-250158 | 1454 | 1 | A | Rel-18 CR TS 28.541 Wrong support of notifications in 5GC NRM | 18.11.0 |
| 2025-03 | SA#107 | SP-250150 | 1456 | 1 | F | Rel18 CR TS28.541 correction to inconsistency between stage 2 and stage 3 | 18.11.0 |
| 2025-03 | SA#107 | SP-250165 | 1457 | - | F | Rel-18 CR TS 28.541 Correct stage3 definition for NTNFunction | 18.11.0 |
| 2025-03 | SA#107 | SP-250150 | 1461 | - | F | Rel-18 CR TS 28.541 Fix error in Transport view of AMF NRM diagram | 18.11.0 |
| 2025-03 | SA#107 | SP-250165 | 1465 | - | F | Rel-18 CR TS28.541 add missing satelliteCoverageInfoList for stage 3 OpenAPI | 18.11.0 |
| 2025-03 | SA#107 | SP-250163 | 1467 | 1 | F | Rel-18 CR 28.541 correction on NwdafInfo | 18.11.0 |
| 2025-03 | SA#107 | SP-250154 | 1475 | - | A | Rel-18 CR TS 28.541 Update stage3 for the allowed value for ueAccProbabilityDist | 18.11.0 |
| 2025-03 | SA#107 | SP-250163 | 1477 | 1 | F | Rel-18 CR TS 28.541 corrections for 5QISetRef | 18.11.0 |
| 2025-03 | SA#107 | SP-250150 | 1484 | 1 | F | Rel-18 CR TS 28.541 Add transport view of MMEFunction NRM | 18.11.0 |
| 2025-06 | SA#108 | SP-250557 | 1488 | - | F | Rel-18 CR 28.541 YANG stage-3 corrections (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250557 | 1493 | - | F | Rel18 CR TS28.541 correction to inconsistency between stage 2 and stage 3 concerning SubNetwork and ManagedElement containment (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250557 | 1494 | - | F | Rel18 CR TS28.541 correction to stage 3 of SatelliteBackhaulInfo (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250519 | 1497 | 1 | F | Rel-18 CR TS 28.541 Typo corrections and missing definitions | 18.12.0 |
| 2025-06 | SA#108 | SP-250555 | 1503 | - | A | Rel-18 CR 28.541 Update for allowed value ueAccProbabilityDist (stage3, yang) (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250549 | 1514 | - | F | Rel-18 CR 28541 Correct support qualifier condition for ephemerisInfoSetRef | 18.12.0 |
| 2025-06 | SA#108 | SP-250549 | 1522 | - | F | Rel-18 CR TS28.541 Fix nTNTAClist for stage 3 OpenAPI (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250557 | 1526 | 1 | F | Rel-18 CR 28.541 YANG stage-3 corrections (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250557 | 1544 | 1 | F | Rel-18 CR TS28.541 Fix stage 3 related to DnnUpfInfoItem (FORGE only) | 18.12.0 |
| 2025-06 | SA#108 | SP-250555 | 1553 | 1 | A | Rel-18 CR 28.541 Update sector equipment and antenna function definitions | 18.12.0 |