3GPP TSG-SA5 Meeting #160 	S5-251237

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0678 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Distribution of UE throughput |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current formula definition for the distribution of UL and DL UE throughput has an incorrect description of the UE scope which is not in line with the rest of the performance measurement definition text (in bullet c, that “the bin corresponding to the DL throughput experienced by the UE”). The intension is that the measurement shall provide samples for each UE in the cell; not just aggregated samples per granularity period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the UE values sum from the formulas |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading performance measurements definition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3.2, 5.1.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.1.3.2	Distribution of DL UE throughput in gNB

a)	This measurement provides the distribution of the UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC) and subcounters per supported S-NSSAI, and subcounters per PLMN ID.

b)	DER(N=1)

c)	Considering there are n samples during measurement time T and each sample has the same time period tn, the measurement of one sample for one UE is obtained by the following formula for a measurement period tn:

If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s]

UEsThpTimeDl>0UEsThpVolDlUEsThpTimeDl

If ThpTimeDl=0, 0 [kbit/s]

UEsThpTimeDl=0

For small data bursts, where all buffered data is included in one initial HARQ transmission, , otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time (ThpTimeDL)ThpTimeUL) may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

For each measurement sample of each UE, the bin corresponding to the DL throughput experienced by the UE is incremented by one. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC) and for each supported S-NSSAI.

d)	A set of integers, each representing the (integer) number of samples with a DL UE throughput in the range represented by that bin. If the optional QoS level subcounter and S-NSSAI subcounter and PLMN ID subcounter measurements are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs, and the number of PLMN IDs.

e)	The measurement name has the form 
DRB.UEThpDlDist.Bin where Bin represents the bin, or optionally DRB.UEThpDlDist.Bin.QOS, where QOS identifies the target quality of service class, and DRB.UEThpDlDist.Bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and DRB.UEThpDlDist.Bin.PLMN, where PLMN identifies the PLMN ID.

NOTE:	Number of bins and the range for each bin is left to implementation

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

Next change

##### 5.1.1.3.4	Distribution of UL UE throughput in gNB

a)	This measurement provides the distribution of the UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC) and subcounters per supported S-NSSAI, and subcounters per PLMN ID.

b)	DER(N=1)

c)	Considering there are n samples during measurement time T and each sample has the same time period tn, the measurement of one sample for one UE is obtained by the following formula for a measurement period tn:

If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s]

UEsThpTimeUl>0UEsThpVolUlUEsThpTimeUl

If ThpTimeUl=0, 0 [kbit/s]

UEsThpTimeUl=0

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise:

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
| T1 | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
| T2 | The point in time when transmission is started for the first data in data burst for a particular DRB. |
| ThpVolUL | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time (ThpTimeUL) may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

For each measurement sample of each UE, the bin corresponding to the UL throughput experienced by the UE is incremented by one. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC) and for each supported S-NSSAI, and for each PLMN ID.

d)	A set of integers, each representing the (integer) number of samples with a UL UE throughput in the range represented by that bin. If the optional QoS level subcounter and S-NSSAI subcounter and PLMN ID subcounter measurements are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs, and the number of PLMN IDs.

e)	The measurement name has the form 
DRB.UEThpUlDist.Bin where Bin represents the bin, or optionally DRB.UEThpUlDist.Bin.QOS, where QOS identifies the target quality of service class, and DRB.UEThpUlDist.Bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and DRB.UEThpUlDist.Bin.PLMN, where PLMN identifies the PLMN ID.

NOTE:	Number of bins and the range for each bin is left to implementation

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of changes