3GPP TSG-SA5 Meeting #160	S5-251434

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.849 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.849 EditHelp Editorial Changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CAPIF_Ph2_CH |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducing editorial changes as requested by EditHelp |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove empty clauses, correct terms used, gramatical corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study will not be clear, and its quality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.1, 5.3.2, 6.1.2.2, 6.1.2.3, 6.1.5.1.0, 6.1.5.2.3, 6.1.5.3.3, 6.1.5.4.3, 6.1.6.1, 6.1.6.2, 6.1.6.3, 6.2.6.1, 6.3.1, 6.3.5.3.3, 6.4.6.1, 7, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Second change

## 5.1	General Description

The Common API Framework (CAPIF) architecture is defined as service-based and interactions between the CAPIF functions are represented in two ways:

-	A service-based representation, where CAPIF functions enable other authorized CAPIF functions to access their services.

-	A reference point representation, where interactions between any two CAPIF functions (e.g. CCF, AEF) is shown by an appropriate point-to-point reference point (e.g. CAPIF-3).

The CAPIF functional architecture can be adopted by any 3GPP functionality providing 3GPP northbound service APIs.

Figure 5.1-1 shows the reference point based functional model for the CAPIF.

Figure 5.1-1: Functional model for the CAPIF

Third change

### 5.3.2	CAPIF business relationships for RNAA

As per the TS 23.222 [2] clause 5.2, the business roles related CAPIF for RNAA charging include:

Figure 5.3.2-1: CAPIF business relationships for RNAA

-	API invoker: see CAPIF business relationships in clause 5.2.1.

-	CAPIF see CAPIF business relationships in clause 5.2.1.

NOTE:	In the current release, both the CAPIF provider and the API provider belongs to the same enterprise (e.g. PLMN operator) and the service API arrangement is not required explicitly.

Fourth change

#### 6.1.2.2	Use Case #1.2: API invoker management via CAPIF

The service API invokers (e.g. the 3rd party application provider) and the CAPIF providers (i.e. Operators) has the service agreement about the API invoker management. The CAPIF providers the Service API about API invoker management.

As part of onboarding an API Provider is provisioned to get access to the service APIs. From a high-level perspective, the corresponding process has implications on the provisioning on the Operator's BSS/OSS Systems that will also be responsible for triggering the chargeable events. CAPIF can collect the charging information based on the following chargeable events, for example:

-	the service activation: Onboarding the API invoker to the CAPIF, as specified in TS 23.222 [2], clause 8.1;

-	the service in-activation: Offboarding the API invoker from the CAPIF, as specified in TS 23.222 [2], clause 8.2.

The charging party: the CAPIF providers.

The charged party: the API invoker.

The potential charging requirements for this UC are: REQ-3GPPCH-IVKM-01.

Fifth change

#### 6.1.2.3	Use Case #1.3: Service API discovery via CAPIF

The service API invokers (e.g. the 3rd party application provider) and the CAPIF providers (i.e. Operators) has the service agreement about the service API discovery. The Discovery Service API could be used to find a specific service by API Invoker.

The CAPIF core function enables the API invoker to discover, subscribe to, unsubscribe from and receive notifications from the CAPIF events, such as availability events of service APIs, change in service API information, monitoring service API invocations, API invoker onboarding events, etc., over CAPIF-1 or CAPIF-1e, as listed below:

-	discover service APIs, as specified in TS 23.222 [2] Clause 8.7.

-	subscription, unsubscription and notifications for the CAPIF events, as specified in TS 23.222 [2] Clause 8.8.

Note: 	The subscribing entity (i.e. the API invoker, the API exposing function, the API publishing function, the API management function) can subscribe to and unsubscribe the CAPIF events. The use case #1.3 only cover the relationship between API invoker and CAPIF.

The charging party: the CAPIF providers.

The charged party: the API invoker.

The potential charging requirements for this UC are: REQ-3GPPCH-IVKO-01.

Sixth change

##### 6.1.5.1.0 	General

A possible solution for key issue #1.1 covers the requirement REQ-3GPPCH-APIM-01.

The focus on this solution to provide the capability to charge the following service API in case required:

-	service API publish, as specified in the TS 23.222 [2] clause 8.3

-	service API unpublish, as specified in the TS 23.222 [2] clause 8.4

-	service API retrieve, as specified in the TS 23.222 [2] clause 8.5

-	service API update, as specified in the TS 23.222 [2] clause 8.6

NEF, currently, is able to perform converged charging by interacting with CHF using Nchf, as specified in TS 32.254 [3], clause 5.4.1.1. It already performs converged charging for the Northbound API access.

Additionally there is already a Service API Charging function which is responsible for reporting/generating the charging information based on API consumption by the external consumers through the CAPIF framework. To do this, this function subscribes to the CCF to receive the notifications related to the Service API lifecycle events (e.g. Service API publish, unpublish, update and invocation logs). The Service API invocation log has the following information that will be used by the CHF.

The following NEF charging scenarios (IEC; ECUR) can be extended to cover the CAPIF API Service. Therefore, when there are specific requests from or to NEF for those Service APIs, a charging event shall be triggered, so it can be charged. It is recommended to extend the Default Trigger conditions table 5.4.1.2.1.1 of TS 32.254 [3] as described below.

Table 6.1.5.1-1: CAPIF default trigger conditions in NEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Service API Publish | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Service API Publish Notification | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Service API Publish Acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Service API Unpublish | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Service API Unpublish Acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Service API update | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Service API update Acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |

The following scenarios have to be considered to cover the Service APIs. Therefore, these message flow focus on the different messages from/to NEF and its interaction with CHF as specified in TS 32.254 [3], clause 5.4.2.

Seventh change

##### 6.1.5.2.3	Procedures description

The Figure 6.1.5.2.3-1 describes the high-level charging procedure for CCF Converged charging for service API discovery over CAPIF-1 or CAPIF-1e, based on the procedure for API discover in Figure 8.7.3-1 and event subscription/unsubscription in Figures 8.8.3-1 and 8.8.5-1 in TS 23.222 [2]. The event-based charging, i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Pre-conditions:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The CAPIF core function is configured with a discovery policy information.

3.	The API invoker has the authorization to subscribe for the CAPIF events.

Figure 6.1.5.2.3-1: Event Charging Procedure for Service API discovery to the CAPIF 
(PEC as example)

1.	The CAPIF supports service API discovery with CAPIF_Discover_Service_API including operations, e.g. discover service API and subscribe/unsubscribe event, as defined in TS 23.222 [2] clause 10.2. The API invoker sends the service API discovery request, e.g. API discover request, event subscription request, to CAPIF core Function over CAPIF-1 or CAPIF-1e, including the API invoker identity.

2.	CAPIF core function performs service API discovery handling procedure which includes applies the discovery policy and performs filtering of service APIs information retrieved from the CAPIF core function, stores the subscription information, updates the subscription information, removes the subscription information.

3.	The CAPIF provides the successful service API discovery response.

3ch-a. CAPIF core function generates charging data related to the service API discovery and sends the request for the CHF to store related charging data for CDR generation purpose.

3ch-b.	CHF stores received information and creates a CDR related to the API invokers.

3ch-c. CHF informs the CAPIF core function on the result of the request.

The figure 6.1.5.2.3-2 describes the high-level charging procedure for CCF Converged charging for service API discovery over CAPIF-1 or CAPIF-1e, based on the procedure for CAPIF event notifications in Figure 8.8.3-1 in TS 23.222 [2]. The event-based charging, i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Pre-conditions:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The subscription procedure of API invoker has is performed.

Figure 6.1.5.2.3-2: Event Charging Procedure for CAPIF Event notifications (PEC as example)

1.	CAPIF core function performs service API discovery handling procedure for retrieving application subscriptions based on the generated events to be consumed by the subscribing entity (API Invokers).

2.	The CAPIF core function sends event notifications to API invokers.

3.	API invokers sends the event notification acknowledgement to the CAPIF core function.

3ch-a to 3ch-c is the same with the Figure 6.1.5.2.3-1.

Eighth change

##### 6.1.5.3.3	Procedures description

The Figure 6.1.5.3.3-1 describes the high-level charging procedure for CCF Converged charging for API invoker management over CAPIF-1 or CAPIF-1e, based on Figures 8.1.3-1 and 8.2.3-1 about the procedure for onboarding and offboarding API invoker in the TS 23.222 [2]. The event-based charging, i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Figure 6.1.5.3.3-1: Event Charging Procedure for API invoker management to the CAPIF (PEC as example)

1.	The CAPIF supports management of API invoker with CAPIF_API_invoker_management API, including onboard/offboard API invoker, as defined in TS 23.222 [2] clause 10.5. The API invoker triggers onboard/offboard API invoker request to the CAPIF core function over CAPIF-1 or CAPIF-1e.

2.	CAPIF core function performs API invoker management procedure which includes onboarding approval (enrolment of the API invoker to be a recognized as user of the CAPIF) and cancelling the API invoker enrolment from CAPIF.

3.	The CAPIF provides the successful onboarding/offboarding API invoker management response.

3ch-a. CAPIF core function generates charging data related to the API invoker management and sends the request for the CHF to store related charging data for CDR generation purpose.

3ch-b.	CHF stores received information and creates a CDR related to the API invokers.

3ch-c. CHF informs the CAPIF core function on the result of the request.

Nineth change

##### 6.1.5.4.3	Procedures description

The Figure 6.1.5.4.3-1 describes the high-level charging procedure for CCF Converged charging for API management via CAPIF-4, based on Figures 8.3.3-1, 8.4.3-1, 8.5.3-1 and 8.6.3-1 about the procedure for publish/unpublish/retrieve/update service APIs in the TS 23.222 [2]. The event-based charging, i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Figure 6.1.5.4.3-1: Event Charging Procedure for API management to the CAPIF (PEC as an example)

1.	The CAPIF supports management of the service APIs by the API provider with the CAPIF_Publish_Service_API, including publish/unpublish/get/update service API, as defined in clause 10.3 in TS 23.222 [2]. The API publishing function sent the service API management requests to CAPIF core function over CAPIF-4.

2.	CAPIF core function performs service API management procedure which includes storing /removing /retrieving/ updating the API information.

3.	The CAPIF sends the service API management response to the API publishing function.

3ch-a. CAPIF core function generates charging data related to the service API management and sends the request for the CHF to store related charging data for CDR generation purpose.

3ch-b.	CHF stores received information and creates a CDR related to the service APIs.

3ch-c. CHF informs the CAPIF core function on the result of the request.

Tenth change

#### 6.1.6.1	Solutions evaluation for Key issue #1.1

Solution #1.1 provides the capability to support Charging of the CAPIF Service APIs, by using the current NEF Charging architecture.

Solution #1.4 provides the CAPIF Core Function (CCF) capability to support CAPIF Service APIs publish, unpublish, retrieve and update charging. In the solution #1.4, the new charging specification is requested focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3].

Solution #1.5 provides the capability to support charging of API Invoker as well as APIs service Operation and Management, by using the current Exposure function northbound Application Program Interfaces (APIs) charging architecture and information. The new triggers and charging information are requested with the extension and adaptation specified in the TS 32.254 [3].

#### 6.1.6.2	Solutions evaluation for Key issue #1.2

Solution #1.3 provides the CAPIF Core Function (CCF) capability to support API invoker onboard/offboard charging via the CAPIF, which is supplement to solution #1.2. In the solution #1.3, the new charging specification is requested focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3].

Solution #1.5 provides the capability to support charging of API invoker management, by using the current NEF charging architecture and information. The new triggers and charging information are requested with the extension and adaptation specified in the TS 32.254 [3].

Solution #1.6 suggest having the support charging of API invoker onboard/offboard outside of 3GPP SA5.

#### 6.1.6.3	Solutions evaluation for Key issue #1.3

Solution #1.2 provides the CAPIF Core Function (CCF) capability to support CAPIF Service APIs discovery request and notification charging via CAPIF. In the solution #1.2, the new charging specification is requested focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3] for the CAPIF Service APIs discovery charging.

Eleventh change

#### 6.2.6.1	Solutions evaluation for Key issue #2.1

Solution #2.1 The solution evaluation is similar to the evaluation for solution #1.1, because the main difference between the two use cases is on the number of API Providers which can be used, and the possibility to have different charging scenarios which can be used for each of the API Providers identified by the tenant identifier.

Solution #2.2 This solution is using the CCF instead of the NEF, which will facilitate the charging by relying on the current CAPIF architecture. It will be required to enhance the current Charging architecture by having CTF in the CCF.

Twelfth change

### 6.3.1	General description and assumptions

Informative Annex B.3.2 of 3GPP TS 23.222 [2] describes on how the NEF (and SCEF) can implement the specific service aspect and how its aligned with the CAPIF Architecture. Additionally, it’s described in clause 8.8 of 3GPP TS 23.222 [2] on the flows for Subscription, unsubscription and notifications for the CAPIF events. Therefore, it would be necessary to evaluate the possibility to support Converged charging in those deployment models and on how the API Service Invoker could be supported by Converged Charging.

Thirteenth change

##### 6.3.5.3.3	Procedures description

The Figure 6.3.5.3.3-2 describes the high-level charging procedure for CCF Converged charging for API management via CAPIF-4, based on Figures 8.3.3-1, 8.4.3-1, 8.5.3-1 and 8.6.3-1 about the procedure for API Invocation in TS 23.222 [2]. The event-based charging, i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Figure 6.3.5.3.3-2: Event Charging Procedure for API object to the CAPIF (IEC)

1.	The API Provider sends a service request that relates to an API object lifecycle change (creating/deleting) or API object access (reading/updating).

2.	The AEF checks whether the API invoker is authorized.

3.	Corresponding to the nature of the request in step 1, the AEF sends the Service API status trigger to CTF, via the CAPIF-3 interface.

3ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

3ch-b. The CHF opens CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response.

4.	The CCF notifies the AEF about the outcome of the charging procedure.

5.	The AEF executes the API logic, fulfilling the API request.

6.	The AEF returns the API response.

Figure 6.3.5.3.3-3: Event Charging Procedure for API object to the CAPIF (PEC)

1.	The API Provider sends a service request that relates to an API object lifecycle change (creating/deleting) or API object access (reading/updating).

2.	The API invoker is authorized.

3.	Corresponding to the nature of the request in step 1, the AEF sends the Service API status trigger to CTF, via the CAPIF-3 interface.

4.	The AEF returns the API response.

5.	Corresponding to the nature of the request in step 1, the AEF sends the matching API object lifecycle trigger or API object access trigger to CaTF, via the CAPIF-3 interface.

5ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

5ch-b. The CHF opens CDR for this API Invocation.

5ch-c. The CHF acknowledges by sending Charging Data Response.

6. The CCF notifies the AEF about the outcome of the charging procedure.

Figure 6.3.5.3.3-4: Event Charging Procedure for API object to the CAPIF (ECUR)

1.	The API invoker sends a service request that relates to an API object lifecycle change (creating/deleting) or API object access (reading/updating).

2.	The AEF checks whether the API invoker is authorized to perform this operation.

3. Corresponding to the nature of the request in step 1, the AEF sends an Service API status trigger to CTF, via the CAPIF-3 interface, indicating initialization of the unit reservation.

3ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

3ch-b. The CHF opens CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response whether the request is within the spending limit.

4. The CCF notifies the AEF about the outcome of the charging procedure, including an indication whether or not the processing can go ahead.

5.	The AEF executes the API logic, fulfilling the API request, in case it has received permission in step 3ch-c.

6.	Corresponding to the nature of the request in step 1, the AEF sends a Service API status trigger to CTF, via the CAPIF-3 interface, indicating termination of the unit reservation.

6ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

6ch-b. The CHF opens CDR for this API Invocation.

6ch-c. The CHF acknowledges by sending Charging Data Response, closes the reservation and responds to the CCF.

7.	The CCF notifies the AEF about the outcome of the charging procedure.

8.	The AEF returns the API response.

Fourteenth change

#### 6.4.6.1	Solutions evaluation for Key issue #4.1

Solution #4.1 enables the option of charging a NEF API through CCF. It uses the current Northbound Application Program Interfaces (APIs) Charging, though it needs to be enhanced to trigger a charging event (i.e. CTF) for each one of the Charging scenarios (PEC; IEC; ECUR)

Solution #4.Y enables the option of charging a NEF API through CCF. It uses the CCF embedded CTF for charging when NEF implementing with the API provider domain. The CCF can reuse the NEF charging mechanism with the chargeable event (i.e. Triggers) for each one of the Charging scenarios (PEC; IEC; ECUR). A new charging specification is requested focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3].

Fifteenth change

# 7	Evaluation

There are three possible ways to add this in the specifications:

-	a new TS

-	a new annex in the TS 32.254 [3]

-	new architecture and information in the current TS 32.254 [3]

Adding a new TS could cause interoperability issues since in the case the NEF implements the CAPIF Core Function according to solution #4.1 it will be difficult to know which TS to use. The combination of new architecture and information together with informative annex on CAPIF architecture in TS 32.254 [3] provide the information needed without interoperability concerns.

# 8	Conclusion

The following solutions, for each one of the topics, are recommended to be included into normative work:

-	CAPIF Converged Charging support of Service APIs Operation and Management: solutions as per clause 6.1.7 conclusion.

-	CAPIF Converged Charging of multiple API Providers: solutions as per clause 6.2.7 conclusion.

-	API Service usage charging of CAPIF: solutions as per clause 6.3.7 conclusion.

-	CAPIF Converged Charging of NEF API: solutions as per clause 6.4.7 conclusion.

End of changes