3GPP TSG-SA5 Meeting #160	S5-251508

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Ericsson

# Title:	Handling of un-reported usage information after failure

# Document for:	Discussion

Agenda Item:	7.3

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TS 32.240: "Charging Architecture and Principles"

[2]	3GPP TS 32.255: "5G data connectivity domain charging; Stage 2"

[3]	3GPP TS 32.290: " 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

# 3	Rationale

## 3.1	Background

In the case of CTF detected failure the failure handling determines what to do if the sending of charging data request to the CHF, including retry handling, without response in a period of time (request times out), according to TS 32.290 [3].

If also the retries fails then there will be unreported charging information since the CTF could never report the information, this is independent of if it is event- or service-based charging, and if the CTF will refuse the continuation of the service in the case it cannot reach the CHF.

In the case, according to TS 32.290 [3], the CHF is determined not reachable the CTF may store Charging Data Request(s) or charging information. Once the connection to a CHF is established by the NF consumer (CTF), the CHF may receive the Charging Data Request(s) or charging information that were previously stored by NF consumer (CTF).

The handling of these messages in the CHF is today vendor specific, except that it may generate a CDR. This, means that the:

-	Charging Data Response(s) for an [Initial] may treated as a normal [Initial] and may be answered with a terminate even if it was allowed to continue according to the failure handling procedure.

-	Charging Data Response(s) for an [Update] may be rejected due to that the associated resource cannot be created in the CHF receiving the request, quota cannot be reserved, or answer with quota not applicable.

-	Charging Data Response(s) for an [Termination] could contain different error codes.

There are several issues with resending charging information at a later time for things that has already happened.

## 3.2	Possible solutions

### 3.2.1	Only allow Termination

Only allow sending termination in the case of resending charging information for the failure scenario avoiding the possibility to receive Initial and Update that require special treatment, this means that the CTF would have convert all messages to Charging Data Request [Termination]

### 3.2.2	Time based

The CHF could use the time of the request and the time of the service to determine if this is a request that previously failed to be reported and apply special treatment.

### 3.2.3	Introduce an indication

Introduce an indication in the Charging Data Request allowing the CHF to apply special treatment of request that are of resending charging information for the failure scenario.

### 3.2.3	Change to use offline

Move the requests to an offline type, i.e. any online request would be converted to offline including all online quota management event triggers by the CTF, of interaction instead of converged would limit any error scenario.

## 3.3	Evaluation

The solution that only allow Termination, introduces a big change to the current handling and post limitations on how the CTF may store the charging information. It may also cause backwards compatibility issues, but this can be avoided using supported features.

The solution that uses time based can be introduced without any impact on current interface specifications, it would however be difficult to define a time period that would be sufficiently long to not get false indications while capturing all resending charging information for the failure scenario.

The solution that introduces an indicator would allow the CTF to know the behaviour of the CHF as the same time the CHF can simplify the treatment since no checks of subscription is required. It may also cause backwards compatibility issues, but this can be avoided using supported features.

The solution only using offline for the would capture most issues with the current mechanism, however it would not always capture the cases where the CHF would reject the request due to things like unknown subscriber etc. It may also cause backwards compatibility issues, but this can be avoided using supported features.

## 3.4	Observations

Observation 1: Introducing a new indication would be the best way forward since it would allow for the current way of storing the charging information and only have limited impact on the interaction between the CHF and CTF.

# 4	Detailed proposal

Proposal 1: Specify a new feature supporting indication of resending charging information for the failure scenario