3GPP TSG-SA5 Meeting #160	S5-251546

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | InputToDraftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 InputToDraftCR TS 28.105 add procedure of management of AI/ML capabilities for NG-RAN and MDA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution proposes to add the procedure of managing AI/ML capabilities for NG-RAN and MDA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for management of AI/ML capabilities for NG-RAN and MDA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management of AI/ML capabilities for NG-RAN and MDA will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (New), X.1(New), X.2(New), X.3(New), A.X(New), A.X.1(New), A.X.2(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

X	Procedure for AI/ML management

X.1	Introduction

This clause describes the procedures for AI/ML management.

X.2	Procedure for Management of AI/ML capabilities for RAN

Figure X.2-1: Procedure for Management of AI/ML capabilities for RAN

1.	MnS Consumer sends a request to create an MLTrainingRequest instance (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with training information for specific NG-RAN inference type to be created. The detailed training information sees attributes of the concrete MLTrainingRequest IOC defined in clause 7.3a.

2.	Based on the received request, the MnS Producer creates the concrete MLTrainingRequest instance (i.e. instance of MLTrainingRequest IOC) and configure the new created MLTrainingRequest MOI with the received training information.

3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

4.	Based on the created MLTrainingRequest instance, MnS Producer performs the ML model training for specific NG-RAN inference Type according to the value of aIMLInferenceName attribute is ngRanInferenceType.

5.	MnS Producer notifies the ML training report to the MnS consumer, e.g. MnS consumer can trigger the procedure for querying ML training report instance.

6.	MnS Consumer sends a request to create an MLModelLoadingRequest instance (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with MLModelLoadingRequest information. The detailed loading information sees attributes of the concrete MLModelLoadingRequest IOC defined in clause 7.3a.

7.	Based on the received request, the MnS Producer creates the concrete MLModelLoadingRequest instance (i.e. instance of MLModelLoadingRequest IOC) and configure the new created MLModelLoadingRequest MOI with the received loading information.

8.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

- MnS Producer perform ML model loading to making the trained ML model available to the target NG-RAN inference function.

9.	NG-RAN AI/ML-based function perform inference to obtain the inference report.

Note: The interface between MnS producer and NG-RAN AI/ML-based function is not subject to standardization.

10. 	MnS Producer creates and configures the AIMLInferenceReport instance when creating an AIMLInferenceFunction instance.

11.	MnS Producer notifies MnS consumer the creation of the AIMLInferenceReport instance to report the inference results.

X.3	Procedure for Management of AI/ML capabilities for MDA

Figure X.3-1: Procedure for Management of AI/ML capabilities for RAN

1.	MnS Consumer sends a request to create an MLTrainingRequest instance (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with training information for specific MDA inference type to be created. The detailed training information sees attributes of the concrete MLTrainingRequest IOC defined in clause 7.3a.

2.	Based on the received request, the MnS Producer creates the concrete MLTrainingRequest instance (i.e. instance of MLTrainingRequest IOC) and configure the new created MLTrainingRequest MOI with the received training information.

3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

4.	Based on the created MLTrainingRequest instance, MnS Producer performs the ML model training for specific MDA inference Type according to the value of aIMLInferenceName attribute is MDAType.

5.	MnS Producer notifies the ML training report to the MnS consumer, e.g. MnS consumer can trigger the procedure for querying ML training report instance.

6.	MnS Consumer sends a request to create an MLModelLoadingRequest instance (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with MLModelLoadingRequest information. The detailed loading information sees attributes of the concrete MLModelLoadingRequest IOC defined in clause 7.3a.

7.	Based on the received request, the MnS Producer creates the concrete MLModelLoadingRequest instance (i.e. instance of MLModelLoadingRequest IOC) and configure the new created MLModelLoadingRequest MOI with the received loading information.

8.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

9.	MnS Producer perform ML model loading to making the trained ML model available to the target MDA inference function.

10	MnS Producer perform inference to obtain the inference result.

11. MnS Producer creates and configures the AIMLInferenceReport instance when creating an AIMLInferenceFunction instance.

12.	MnS Producer notifies MnS consumer the creation of the AIMLInferenceReport instance to report the inference results.

Next change

Annex A (informative):
PlantUML source code for NRM class diagrams

A.X	Procedures for AI/ML management

A.X.1	Procedure for Management of AI/ML capabilities for RAN

@startuml

'https://plantuml.com/sequence-diagram

hide footbox

autonumber

participant "MnS consumer" #white

participant "MnS producer of NG-RAN AI/ML-based function" #white

participant "NG-RAN AI/ML-based function" #white

group ML Model Traning

"MnS consumer" -> "MnS producer of NG-RAN AI/ML-based function": Request to create an MLTrainingRequest instance for specific NG-RAN AI/ML based function \n (ngRanInferenceType)

"MnS producer of NG-RAN AI/ML-based function" -> "MnS producer of NG-RAN AI/ML-based function": Create and configure MLTrainingRequest MOI

"MnS producer of NG-RAN AI/ML-based function" -> "MnS consumer": Response for create an MLTrainingRequest instance

"MnS producer of NG-RAN AI/ML-based function" -> "MnS producer of NG-RAN AI/ML-based function": Perform training for specific NG-RAN AI/ML based function

"MnS producer of NG-RAN AI/ML-based function" -> "MnS consumer": Notify the ML Training Report

end

group ML Model loading

"MnS consumer" -> "MnS producer of NG-RAN AI/ML-based function": Request to create an MLModelLoadingRequest instance \n(list of attributes of MLModelLoadingRequest IOC)

"MnS producer of NG-RAN AI/ML-based function" -> "MnS producer of NG-RAN AI/ML-based function": Create and configure MLModelLoadingRequest MOI

"MnS producer of NG-RAN AI/ML-based function" -> "MnS consumer": Response for create an MLModelLoadingRequest instance

Ref over "MnS producer of NG-RAN AI/ML-based function", "NG-RAN AI/ML-based function": ML model loading

end

group AI/ML Inference

"NG-RAN AI/ML-based function" -> "NG-RAN AI/ML-based function": Perform NG-RAN AI/ML based function inference

Ref over "MnS producer of NG-RAN AI/ML-based function", "NG-RAN AI/ML-based function": AIML Inference report

"MnS producer of NG-RAN AI/ML-based function" -> "MnS producer of NG-RAN AI/ML-based function": Create and configure AIMLInferenceReport MOI

"MnS producer of NG-RAN AI/ML-based function" -> "MnS consumer": Notify the AIML Inference report

end

@enduml

A.X.2	Procedure for Management of AI/ML capabilities for MDA

@startuml

'https://plantuml.com/sequence-diagram

hide footbox

autonumber

participant "MnS consumer" #white

participant "MnS producer of AI/ML-based MDA function" #white

group ML Model Traning

"MnS consumer" -> "MnS producer of AI/ML-based MDA function": Request to create an MLTrainingRequest instance for specific AI/ML based MDA function \n (MDA type)

"MnS producer of AI/ML-based MDA function" -> "MnS producer of AI/ML-based MDA function": Create and configure MLTrainingRequest MOI

"MnS producer of AI/ML-based MDA function" -> "MnS consumer": Response for create an MLTrainingRequest instance

"MnS producer of AI/ML-based MDA function" -> "MnS producer of AI/ML-based MDA function": Perform training for specific AI/ML based MDA function

"MnS producer of AI/ML-based MDA function" -> "MnS consumer": Notify the ML Training Report

end

group ML Model loading

"MnS consumer" -> "MnS producer of AI/ML-based MDA function": Request to create an MLModelLoadingRequest instance \n(list of attributes of MLModelLoadingRequest IOC)

"MnS producer of AI/ML-based MDA function" -> "MnS producer of AI/ML-based MDA function": Create and configure MLModelLoadingRequest MOI

"MnS producer of AI/ML-based MDA function" -> "MnS consumer": Response for create an MLModelLoadingRequest instance

"MnS producer of AI/ML-based MDA function" -> "MnS producer of AI/ML-based MDA function": Load ML model to specific MDA inference function

end

group AI/ML Inference

"MnS producer of AI/ML-based MDA function" -> "MnS producer of AI/ML-based MDA function": Perform AI/ML based MDA function inference

"MnS producer of AI/ML-based MDA function" -> "MnS producer of AI/ML-based MDA function": Create and configure AIMLInferenceReport MOI

"MnS producer of AI/ML-based MDA function" -> "MnS consumer": Notify the AIML Inference report

end

@enduml

End of change