3GPP TSG-SA5 Meeting #160	S5-251609

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | DraftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR Rel-19 TS 28.105 Use case on ML explainability for training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To enable a consumer to indicate the need to support local explainability during ML model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduction of an use case on ML explainability in training 2. Introduction of a requirement to have a capability allowing an authorized MnS consumer to indicate the support for local explanations in ML model training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for ML explainability during ML training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.2.Y, 6.2b.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	AI/ML management use cases and requirements

## 6.2b	ML model training

### 6.2b.1	Description

Before an ML model is deployed to conduct inference, the ML model algoritm associated with the ML model needs to be trained. The ML model training can be an initial training or the re-training of an already trained ML model.

The ML model is trained by the ML training MnS producer, and the training can be triggered by request(s) from one or more ML training MnS consumer(s), or initiated by the ML training MnS producer (e.g., as a result of model performance evaluation). The procedures in [x] for authentication and authorization are applicable for all ML model training use cases.

### 6.2b.2	Use cases

#### 6.2b.2.Y	ML explainability in training

Explainable ML refers to a process that enables the consumers to understand and trust the outputs provided by ML models. In essence, explainable ML is about making the decision-making of ML comprehensible to its consumers. On a broad level, explainable ML can be categorized into two types:

- Local explanation: The aim is to explain individual outputs provided by an ML model, i.e., it focuses on explaining why a specific output was generated by the ML model for a particular input data sample.

- Global explanation: The aim is to explain the whole ML model behaviour, i.e., it focuses on explaining how the ML model works in general across several or all possible input samples.

Local explanation is an additional information that the MnS consumer can use to analyze the ML model’s potential impact in terms of network performance gain and make informed decisions regarding deactivation of AI/ML inference or fallback to previous version of the ML model. The ML Training Request IOC initiated by the MnS consumer need to specify requirements for local explainability support in ML model training. It is essential for the MnS consumer to indicate this need so that the MnS producer can preprocess the training data or employ suitable techniques to generate local explanations.

Start of Next change

### 6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-MLKTL-1 | The 3GPP management system should have a capability to enable an authorized MnS consumer to discover or request all or part of the available shareable knowledge at a given MLKTL MnS producer. | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.1) |
| REQ-MLKTL-2 | The 3GPP management system should have a capability for an MLKTL MnS producer to provide to an authorized MnS consumer all or part of its available shareable knowledge | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.1) |
| REQ-MLKTL-3 | The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKTL MnS producer to initiate and execute a transfer learning instance to a specified ML model or ML-enabled function | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-MLKTL-4 | The 3GPP management system should have a capability to enable an authorized MnS consumer to manage or control the knowledge request or the knowledge process or transfer learning process, e.g. to suspend, re-activate or cancel the ML Knowledge Request; or to adjust the description of the desired knowledge NOTE: An example MnS consumers include an operator or the function that generated the request for available Knowledge | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-MLKTL-5 | The 3GPP management system should have a capability to enable an ML model or ML training function to register available knowledge to a shared knowledge repository, e.g. through a ML Knowledge Registration process | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-MLKTL-6 | The 3GPP management system should have a capability enabling an authorized MnS consumer to request the Knowledge Repository to provide some or all the knowledge available for sharing based on specific criteria | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-ML_TRAIN_CLUSTER-01 | The ML Training MnS producer should have a capability for an authorized MnS consumer to request training of a cluster of ML models as per clustering criteria associated to a set of multiple contexts from a previously trained ML model. | ML model training for multiple contexts (clause 6.2b.2.X) |
| REQ-ML_TRAIN-PRE-01 | The ML training MnS producer should have a capability allowing an authorized ML training MnS consumer to request pre-specialized training of a ML model. | ML Pre-specialized training (clause 6.2b.2.X) |
| REQ-ML_TRAIN_DST-01 | The 3GPP management system should enable an authorized consumer to provide information on the training dataset distribution. | Training data statistics (clause 6.2b.2.X) |
| REQ-ML_TRAIN_DST-02 | The 3GPP management system should enable an authorized consumer to provide information on the usage of outliers in the training dataset. | Training data statistics (clause 6.2b.2.X) |
| REQ-ML_DIST-TRNG-01 | The ML training MnS producer should have a capability allowing and authorized consumer to provide distributed training requirements to the MnS Producer. | Management of Distributed training (clause 6.2b.2.X) |
| REQ-ML_TRAIN_FL-1; The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-2; The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-3; The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting (including adding and removing) FL clients in Federated Learning to the MnS Producer.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-4; The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of the global ML model on each participating FL Client.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-5; The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-6; The ML training MnS producer should have a capability to report the candidate FL Clients for the FL process.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-RL_TRAIN_01; The ML training MnS producer should have a capability allowing an authorized MnS consumer to query if RL training is supported.; Enabling Reinforcement Learning (6.2b.2.x.2.1). REQ-RL_TRAIN_02; The ML Training MnS producer for RL agent training component shall have a capability to report RL types (i.e., online RL, offline RL) to an authorized consumer.; Enabling Reinforcement Learning (6.2b.2.x.2.1). REQ-RL_TRAIN_03; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to get the type and scope of the RL environment for which an RL model has been trained.; Exploration in Reinforcement Learning (6.2b.2.x.2.2). REQ-RL_TRAIN_04; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to select the type of the RL environment for which an RL model is to be trained.; Exploration in Reinforcement Learning (6.2b.2.x.2.2). REQ-RL_TRAIN_05; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to provide the scope of the RL environment for which an RL model is to be trained.; Exploration in Reinforcement Learning (6.2b.2.x2.2). REQ-RL_TRAIN_06; The ML training MnS producer should have a capability allowing an authorized MnS consumer to provide network performance requirements of performing RL training.; Exploration in Reinforcement Learning (6.2b.2.x2.2). REQ-RL_TRAIN_07; The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the configuration range that the RL agent is allowed to explore.; Exploration in Reinforcement Learning (6.2b.2.x2.2). REQ-ML-TRAIN-XAI-1; The ML training MnS producer should have a capability allowing an authorized MnS consumer to indicate the support for local explanations in ML model training.; ML explainability in training (6.2b.2.Y). NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |

End of changes