3GPP TSG-SA5 Meeting #160	S5-251911

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251247

Source:	Nokia

Title:	Pseudo-CR on CCL decision escalation

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for the use on CCL decision escalation as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.C	 CCL decision escalation – ESC

### 5.C.1	Description

This use case related to the capability to escalate decision making to another entity e.g. another CCL

### 5.C.2	Use Cases

## 5.C.2.1	Triggering CCL decision escalation – ESC_01

Not all decisions made by CCLs in different network contexts (states, status, conditions, etc.) are equally effective. The CCL may need to inform another entity about its lack of confidence in its decision with a request to escalate its decision making to that entity. For example, a CCL for optimizing energy saving may fail to decide the sequence in which cells may be deactivated when there is a failure for some cells. The CCL may escalate the scenario to a CCL on problem recovery.

The MnS consumer should be able to configure MnS producer regarding the escalation recipient to which the decision is escalated. The degree to which the CCL can independently execute decisions or escalates them, should be configurable by the MnS consumer through a confidence threshold. The confidence threshold is an index on a fixed scale say from 0 (indicating lowest confidence) to 10 (indicating highest confidence). It could be configured based on the sensitivity of the operations under the CCLs’ control, the trust level in the decisions of the CCL and the necessity to consider a bigger picture at times. Then, based on how much confidence the CCL has in its decisions, the CCL can escalate a decision or situation to an escalation recipient (e.g. another CCL or a CCL coordination entity) which has this bigger picture (say has wider scope), can execute a different(larger) set of actions or has better capabilities, e.g. a larger and more capable ML model.

NOTE 1:	The computation of confidence within the CCL is up to implementation as it depends on the CCL's purpose and the scenario that the CCL is addressing.

The escalation recipient CCL should enable the escalator CCL to request for escalation for a given network context or state with e.g. information about the escalator CCL preferences and observed constraints when driving decisions. Based on its evaluations, the escalation recipient CCL should provide to the escalator CCL a report that holds the outcomes for a given escalation request.

Figure 5.C.1-1: required interactions for CCL decision escalation

### 5.C.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ESC-TRG-01 | The 3GPP management system should have a capability to enabling an authorized MnS consumer to configure a CCL with the degree of autonomy of to define when the CCL can escalate and the entity to which to escalate decision making. | ESC- ESC_01 Clause 5.C.2.1 |
| REQ-ESC-TRG-02 | The 3GPP management system should have a capability to enabling an authorized MnS consumer (e.g. an escalator CCL) to request to escalate decision-making for a network context or state to an escalation recipient. | ESC- ESC_01 Clause 5.C.2.1 |
| REQ-ESC-TRG-03 | The 3GPP management system should have a capability enabling an escalation recipient CCL to report to an authorized MnS consumer (e.g. an escalator CCL) the outcomes for a given escalation request | ESC- ESC_01 Clause 5.C.2.1 |

* * * End of Changes * * * *