3GPP TSG-SA5 Meeting #160	S5-252035

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251251

Source:	Nokia

Title:	Pseudo-CR on CCL Coordination NRM

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add NRM for the CCL Coordination as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 6	Model

## 6.2		Class diagram

#### 6.2.1	Relationships

Figure 6.2.1-X: NRM fragment for Coordination entity

Figure 6.2.1-Y: NRM fragment for Coordination entity as a Closed Control Loop

#### 6.2.2	Inheritance

Figure 6.2.1-Y: Inheritance Hierarchy for Closed Control Loops

6.3		Class definitions

6.3.1		CoordinationEntity

6.3.A.1	Definition

This IOC represents the Coordination Entity that is responsible for coordinating closed control loops to avoid, detect or resolve CCL conflicts.

The CoordinationEntity is name-contained by SubNetwork or ManagedElement and is associated with one or more CCLs which it shall be responsible for coordinating.

A CoordinationEntity may be a CCL.

6.3.A.2	Attributes

The CoordinationEndity IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.A.2-x

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coordinationCapability | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| closedControlLoopRefList | M | T | T | F | T |

6.3.A.3	Attribute constraints

None

6.3.A.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

6.3.B		CoordinationCapability <<dataType>>

6.3.B.1	Definition

This data type represents the information and a capability of the CoordinationEntity for Coordinating CCL goals.

6.3.B.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLCoordinationCapabilityID | M | T | T | F | T |

6.3.B.3	Attribute constraints

None.

6.3.B.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

6.4	Attribute definitions

6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| coordinationCapability | It indicates a capability of a coordination entity to coordinate CCL conflicts | type: CoordinationCapability multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLCoordinationCapabilityID | It indicates an identifier for a specific CCL conflicts coordination capability | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| closedControlLoopRefList | It indicates a list of DN for ClosedControlLoop Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

Annex A (informative):
Appendix with UML code for model diagrams

# A.Y	UML code for CCL management model diagrams

This annex contains the PlantUML source code for the NRM diagrams defined in clause 6.3 of the present document.

# A.Y.1	NRM fragment for Coordination entity (Figure 6.2.1-X)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class CoordinationEntity <<InformationObjectClass>>

class CoordinationCapability <<dataType>>

class ClosedControlLoop <<InformationObjectClass>>

ManagedEntity "1" *-- "1" CoordinationEntity: <<names>>

CoordinationEntity "1" -r- "*" CoordinationCapability

ClosedControlLoop "*" -r- "*" CoordinationEntity

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CoordinationCapability

Represents the following capabilities:

GoalTargetCoordination

ScopeCoordinationCoordination

TriggerCoordination

ActionExecutionCoordination

DirectActionsCoordination

IndirectTargetsCoordination

end note

@enduml

# A.Y.2	NRM fragment for Coordination entity as a Closed Control Loop (Figure 6.2.1-Y)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class CoordinationCapability <<dataType>>

class ClosedControlLoop <<InformationObjectClass>>

ManagedEntity "1" *-- "1" ClosedControlLoop: <<names>>

ClosedControlLoop "1" -l- "*" CoordinationCapability

ClosedControlLoop "1" -- "*" ClosedControlLoop

note right of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CoordinationCapability

Represents the following capabilities:

GoalTargetCoordination

ScopeAssignmentsCoordination

TriggerCoordination

ActionExecutionCoordination

DirectActionsCoordination

IndirectTargetsCoordination

end note

@enduml

# A.Y.3	Inheritance relationships for Coordination entity (Figure 6.2.2-Y)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class CoordinationEntity <<InformationObjectClass>>

Top <|-- CoordinationEntity

@enduml

* * * End of Changes * * * *