3GPP TSG-SA5 Meeting #161 	S5-252230

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0198 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.423 Corrections on Trace Record Format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in DP (S5-250030), there are many misalignments between the current trace record contents for trace and MDT and the Trace Report File Format. All the misalignments make the current Trace file XML format difficult to use. Non-backward compatible changes cannot be avoided if we try to fix those issues in the current file format. This CR specifies how to output streaming trace record into file in TS32.423, and enhancement on the Trace Record Header. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining how to output streaming trace record into file. Minor corrections on existing streaming trace record text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Many misalignments exist between the current trace record contents for trace and MDT and the Trace Report File Format, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.6, 5.2.x (new), G.1, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 5.2.1	Introduction

GPB encoded trace records are formatted in GPB version 3 (proto3) [45]. Individual Trace records are preceded with a delimiter, which a GPB variable length 32 bit integer that indicates the size of the GPB encoded trace record.

Logical Layout

| Delimiter | Trace Record | Delimiter | Trace Record | Delimiter | Trace Record |
| --- | --- | --- | --- | --- | --- |

The Streaming Trace Record comprises a header, payload and an optional common trace payload that contains the trace administrative message as shown in Figure 5.2.1-1.

Trace Record

Payload

Header

Vendor specific
extension

Common
fields

Vendor Specified
Content

Size

Trace Record

Payload

Header

Vendor specific
extension

Common
fields

Vendor Specified
Content

Size

Figure 5.2.1.1: Streaming Trace Record

The format of the Trace Record Header in Trace Record specified in the clause 5.2.2. The format of the Payload and the Common Trace Payload carrying Trace Administrative Message in Streaming Trace Record specified in the clause 5.2.3 and 5.2.4 respectively.

*** START OF NEXT CHANGE ***

### 5.2.6	Streaming Trace Format

When streaming trace data individual trace records and their associated length delimeter are carried in the payload of the transport protocol messages Figure 5.2.6.1 illustrates the concept.

Figure 5.2.6.1: Transport of Trace Records

As depicted in the Figure 5.2.6.1, each protocol-specific message delivers one or more trace records from the MnS Producer to the MnS Consumer. The header of the transport protocol message is protocol-specific. It may contain protocol specific extensions and/or options related to the transport stream. The payload of the transport protocol carries one of more trace records. The format of the individual trace records is specified in clause 5.2.

The procedures related to the connection establishment and meta-data exchange between the Streaming Trace data reporting MnS Producer and MnS Consumer are out of scope of the present document and are specified in TS 28.532 [43]

*** START OF NEXT CHANGE ***

### 5.2.x	GPB encoded trace record file format

When writing GPB encoded trace record into file, the associated length delimeter specified in subclause 5.2.1 shall be included. The file may contain one or more trace record(s). File naming convention is specified in Annex B.1.

Figure 5.2.x.1: GPB encoded trace record file format

*** START OF NEXT CHANGE ***

# G.1	Transport Protocol Payload Format

The payload of one transport protocol message can carry one or more trace records as specified in clause 5.1. For GPB trace payload, the overall encoding format shall adhere to the following rules:

-	Each trace record is encoded as a single StreamingTraceRecord GPBv3 message following the schema in clause G.2.

-	Each StreamingTraceRecord message is preceded by a length field indicating the size in bytes of the following GPB message. This length field is encoded using the GPB ‘varint’ wire format.

-	If the transport message payload includes multiple trace records, the length field for the next StreamingTraceRecord message shall immediately follow the preceding message.

-	No extra padding (unused bytes) is allowed anywhere in the transport message payload.

NOTE:	The total length of the transport message payload is assumed to be available but encoding of this value is specific to the transport protocol in use.

*** START OF NEXT CHANGE ***

Annex X (informative):
Plant UML source code

# X.1	Transport of Trace Records

The following PlantUML source code is used to describe Transport of Trace Records. As depicted by Figure 5.2.6.1:

@startuml

skinparam Padding 3

rectangle "\n\n\n\n    ...    \n" as MoreBox

Frame "Transport protocol message" as OneBox {

Frame "Protocol Payload" {

note as R1

| Record \nLength | Streaming \nTrace Record |

endnote

note as R2

| Record \nLength | Streaming \nTrace Record |

endnote

note "..." as R3

}

Frame  "Protocol-specific header" {

rectangle "Protocol-specific extensions" #line.dashed

}

}

R1 .r[hidden] R2

R2 .r[hidden] R3

@enduml

# X.2	GPB encoded trace record file format

The following PlantUML source code is used to describe GPB encoded trace record file format. As depicted by Figure 5.2.x.1:

@startuml

skinparam Padding 3

object TraceFile {

<#white>| Record \nLength | Streaming \nTrace Record | Record \nLength | Streaming \nTrace Record | ... |

}

@enduml

*** END OF CHANGE ***