3GPP TSG-SA5 Meeting #161 	S5-252247

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0205 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 32.423 Corrections on Trace Reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Trace Reference is specificed in TS32.422 as MCC+MNC+Trace ID. MCC and MCN are provided by 3-digit and 2-or 3-digit numbers, where MCC+MNC is encoded as a 3-byte Octet String. Trace ID is a 3-byte Octet String. Therefore, in total the TR shall be 6-byte Octet String. However, it is specified as 3-byte Octet String in the trace recorder header which is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reverse TR length and add TR encoding Small corrections on reference error |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 5.2.2	Streaming Trace Record Header

The streaming trace record header contains the common fields as specified in the Table 5.2.2-1, in addition it may also contain vendor specific extensions.

Table 5.2.2.1 : Common fields in the streaming trace record header

| Trace Record Header field name | Description |
| --- | --- |
| timeStamp (M) | Time stamp (in milliseconds since Epoch) of when the streaming trace record is produced internally in the Producer encoded as (64 bit integer) |
| nfInstanceId (M) | Unique id of the Producer NF instance that produced this streaming trace record represented by a (String) |
| nfType (M) | Type of the Producer NF that produced this streaming trace record represented by a (String) |
| traceReference (M) | Trace Reference (see clause 5.6 of 3GPP TS 32.422 [3]) (represented by a 6 bytes octet string) See Note 6. |
| traceRecordingSessionReference (M) | Trace Recording Session Reference (see clause 5.7 of 3GPP TS 32.422 [3]) represented by a (2 byte octet string. See Note 1.) |
| traceRecordTypeId (M) | Identifier of the trace record type (see clause 5.2.4 for details) represented by an ENUM with the following values: NORMAL TRACE_SESSION_START, TRACE_SESSION_STOP, TRACE_RECORDING_SESSION_START, TRACE_RECORDING_SESSION_STOP, TRACE_STREAM_HEARTBEAT. TRACE_RECORDING_SESSION_NOT_STARTED, TRACE_RECORDING_SESSION_DROPPED_EVENTS, (See Note 2). |
| ranUeId (O) | RAN defined id to represent as of thea UE (8 byte octet string. See Note 3.) |
| payloadSchemaURI (O) | URI identifying the schema to be used in order to decode the payload represented by a (String. See Note 4.) |
| vendorExtension (O) | Vendor-specific extension(s) represented by a (See Note 5.) |
| NOTE 1: The traceRecordingSessionReference must be present for the Streaming Trace Records with non-zero size payload where the payload carries data captured for a Trace Recording Session and in administrative messages related to a Trace Recording Session (e.g. "Trace Recording Session Start" or "Trace Recording Session Stop"). NOTE 2: The traceRecordTypeId with value "NORMAL" is used for Streaming Trace Records that do not carry an administrative message. NOTE 3: The ranUeId field is present in the trace record header when the identifier is supported by RAN. If RAN UE Id (see 3GPP TS 38.473 [26]) has been captured in the traced signaling messages that value is used. NOTE 4: The payloadSchemaURI is not required for Streaming Trace Records with payload of zero-size, or payload using common payload format (e.g. used to convey Streaming Trace administrative messages). NOTE 5: The vendorExtension is typically a generic list of key-value pairs. NOTE 6: The encoding of the Trace Reference is a 6-byte Octet String in BCD format. The first 3-byte Octet String is the PLMN ID which consists of MCC and MNC. The next 3-byte Octet String is the Trace ID. The PLMN ID is encoded as specified in clause 9.3.3.5 of 3GPP TS 38.413 [23]. |  |

*** END OF CHANGE ***