3GPP TSG-SA5 Meeting #161	S5-252286

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Ericsson Hungary Ltd

# Title:	DP Deprecate isNullable property

# Document for:	Information, Discussion, Endorsement

Agenda Item:	6.4.26

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]		3GPP TS 32.156: "Model repertoire"

[2]		3GPP TS 28.622 "Integration Reference Point (IRP); Information Service (IS)"

[3]		3GPP TS 28.541 "5G Network Resource Model (NRM); Stage 2 and stage 3"

[4]		3GPP TS 28.620 " Umbrella Information Model (UIM)"

# 3	Rationale

Observation 1: Today we have two attribute properties each specifying if an attribute is allowed to carry no information: multiplicity and isNullable.

The current TS 32.156[1] specifies:

| multiplicity | Defines the number of values the attribute can simultaneously have. See subclause 7.3.44 of OMG "Unified Modelling Language (OMG UML), Superstructure" [2]; inherited from StructuralFeature. | See 5.2.8, default is 1 |
| --- | --- | --- |
| isNullable | Identifies if an attribute can carry no information. The implied meaning of carrying “no information” is context sensitive and is not defined in this Model Repertoire. Note, the property "isNullable: True" is semantically identical to adding the value "0" to the "multiplicity" specified. Usage of the "multiplicity" property is preferred to express an attribute can have no value or carry no information. | True, False (default) |

It is a problem to use multiple properties to specify the same thing: in this case whether no information / 0 (zero) values is allowed.

Having two properties makes a specification harder to understand. It is also an evident risk, that the two properties may be in conflict. Today many attributes have multiplicity including "0" and isNullable=False. This is confusing.

Today we have all combinations of multiplicity and isNullable. The following examples are from TS 28.622[2]. They include all four possible combinations: Multiplicity includes "0" or not, combined with isNullable equals true/false.

There is no clear description for e.g.  what a multiplicity=0 and isNullable=False or multiplicity>0 and isNullable=true means.

| numberOfFiles | Number of files in a file collection. allowedValues: NA | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| --- | --- | --- |
| cancelJob | Setting this attribute to "TRUE" cancels the file download job. As specified in the definition of ProcessMonitor, cancellation is possible in the "NOT_STARTED" and "RUNNING" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| collectionPeriodM7NR | It specifies the collection period for the Packet Loss Rate measurement (M7) for NR MDT taken by the gNB. The attribute is applicable only for Immediate MDT. See the clause 5.10.35 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| geoPolygon | It specifies the geographical area with a polygon. The polygon is specified by its corners. allowedValues: N/A | type: GeoCoordinate multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: True |

Having two properties instead of one makes documents longer, thus more difficult to read without providing any additional information.

TS 32.156 already states that "the property "isNullable: True" is semantically identical to adding the value "0" to the "multiplicity" specified ", thus isNullable carries no semantic meaning beyond what is included in multiplicity. There is no stage-2 use-case that would differentiate between an attribute having an explicit NULL value and an attribute having zero values. Not using isNullable would not remove any real information from the NRM specifications.

Note: Some solutions sets include a specific NULL value. The proposed changes have no effect on this, as TS 32.156 only specifies stage-2 usage and caters for stage-2 use-cases.

There are multiple possible solutions to address the situation:

1) Let multiplicity have priority over isNullable. If multiplicity includes 0 (zero), the attribute may contain no information/values independent of the isNullable property. If multiplicity does not include 0 (zero), the attribute must contain at least one value independent of the isNullable property. (The current practice in 96%)

2) Deprecate property isNullable and remove its default value (preferred). Specify that multiplicity has priority over isNullable; the last part is needed because we cannot remove isNullable immediately from all specifications.

3) Always set isNullable to follow the multiplicity range. If multiplicity includes 0 (zero), isNullable shall be set to True. If multiplicity does not include 0 (zero), isNullable may be set to false or use the default false value.

Alternative 1, which is the current common practice, does not work, as it is confusing. A multiplicity including 0 (zero) semantically conflicts with isNullable = False. Also, multiplicity not including a 0 (zero) semantically conflicts with isNullable = True. Alternative 1 in practice already deprecates isNullable without formally deprecating it.

Alternative 2 is the recommended solution. isNullable is a redundant property, and removing it removes the risk of conflicting specifications and minimizes the text that needs to be maintained. isNullable should be removed from all NRM specifications. As removing isNullable from all specifications cannot be done in one go, instead of removing isNullable from TS 32.156, it shall only be deprecated for now.

Alternative 3 may work but is a bad solution. Although conflicts are avoided, there would still be 2 separate properties specifying the allowed number of values: unneeded text is bad. All NRM specifications today erroneously specifying multiplicity including 0 (zero) cardinality and isNullable = False need to be updated. Future NRMs needs to unnecessarily specify isNullable, which is a recipe for future inconsistency if the 2 properties are set erroneously.

Recommendation 1 (Rel-19): Implement alternative 2. In TS 32.156[1]

- Deprecate the isNullable property

- Remove the default value of isNullable

- Specify that multiplicity has priority over isNullable

Recommendation 2 (Rel-19): After implementing recommendation 1, in NRM specifications do not use the isNullable property from any new attributes and remove it from any updated attributes.

Recommendation 3(Rel-19): After implementing recommendation 1, start removing the isNullable property from NRM specifications e.g. [2] and [3] . The more specifications isNullable is removed from the bigger the gain in quality.

Recommendation 4 (Rel-20): Remove the isNullable property from all NRM specifications in Release-20.

Recommendation 5 (Rel-20): Remove the isNullable property from TS 32.156 in Release-20.

# 4	Detailed proposal

Endorse recommendations 1,2, and 3 above for release release 19.

Endorse recommendations 4 and 5 for release 20.