3GPP TSG-SA5 Meeting #161	S5-252297

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0704 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add PLMN granularity performance measurement of PDU session for SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NetShare_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN. Therefore, performance measurement of Indirect Network Sharing includes RAN and CN parts and both parts require the performance measurement in the PLMN granularity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN granularity performance measurement of PDU session for SMF in the Indirect Network Sharing Scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance management of related NFs for indirect network sharing cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.1, 5.3.1.2, 5.3.1.3, 5.3.1.4, 5.3.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.130 CR 0052 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.3.1	Session Management

5.3.1.1	Number of PDU sessions (Mean)

a)	This measurement provides the mean number of PDU sessions.

b)	SI.

c)	The measurement is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF, and then taking the arithmetic mean. The measurement is optionally performed per S-NSSAI and per PLMN ID.

This measurement is optionally split into subcounters per 5G VN internal Group ID.

d)	A single integer value.

e)	SM.SessionNbrMean 
SM.SessionNbrMean_Filter,

where filter is a combination of PLMN and SNSSAI,

where PLMN represents the PLMN ID, and SNSSAI represents S-NSSAI.

SM.SessionNbrMean. InternalGroupID

Where internal Group ID identifies a 5G VN group communication, as specified in TS 23.501[4].

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.3.1.2	Number of PDU sessions (Maximum)

a)	This measurement provides the max number of PDU sessions.

b)	SI

c)	The measurement is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF, and then selecting the maximum value. The measurement is optionally performed per S-NSSAI and per PLMN ID.

This measurement is optionally split into subcounters per 5G VN internal Group ID.

d)	A single integer value

e)	SM.SessionNbrMax 
SM.SessionNbrMax _Filter,

where filter is a combination of PLMN and SNSSAI,

where PLMN represents the PLMN ID, and SNSSAI represents S-NSSAI.

SM.SessionNbrMean.InternalGroupID

Where internal Group ID identifies a 5G VN group communication, as specified in TS 23.501[4].

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

5.3.1.3	Number of PDU session creation requests

a)	This measurement provides the number of PDU sessions requested to be created by the SMF.

b)	CC

c)	On receipt by the SMF from AMF of Nsmf_PDUSession_CreateSMContext Request (see TS 23.502 [7]). Each PDU session requested to be created is added to the relevant filter per S-NSSAI and per PLMN ID, and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationReq

SM.PduSessionCreationReq _Filter,

where filter is a combination of PLMN and SNSSAI,

where PLMN represents the PLMN ID, and SNSSAI represents S-NSSAI.

SM.PduSessionCreationReq.ReqType.

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

5.3.1.4	Number of successful PDU session creations

a)	This measurement provides the number of PDU sessions successfully created by the SMF.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a successful PDU session creation (see TS 23.502 [7]). Each PDU session successfully created is added to the relevant filter per S-NSSAI and per PLMN ID, and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationSucc

SM.PduSessionCreationSucc _Filter,

where filter is a combination of PLMN and SNSSAI,

where PLMN represents the PLMN ID, and SNSSAI represents S-NSSAI.

SM.PduSessionCreationSucc.ReqType.

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

5.3.1.5	Number of failed PDU session creations

a)	This measurement provides the number of PDU sessions failed to be created by the SMF.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a rejected PDU session creation (see TS 23.502 [7]). The measurement is optionally performed per PLMN ID. Each PDU session rejected to be created is added to the relevant filter per PLMN ID, and the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationFail

SM.PduSessionCreationFail _Filter,

where filter is PLMN, where PLMN represents the PLMN ID

SM.PduSessionCreationFail.cause

Where cause indicates the rejection cause for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

End of change