3GPP TSG-SA5 Meeting #161	S5-252305

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0035 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.537 Correct procedures for heartbeat |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The emission of heartbeat notifications is fully controlled by HeartbeatControl instances, which is different with other notificationTypes. It means subscription for heartbeat notifications is not supported by NtfSubscriptionControl. Following are the observed issues for procedures for heartbeat: - The procedure for heartbeat notification subscription is missing. - The Figure 4.3.2-1: Procedure for requesting immediate heartbeat notification implies the MnS producer can send the notifyHearbeat notification to multiple notification receiver. However, notifyHearbeat notification can only send to the notification receiver represented by the notificationRecipientAddress in the parent NtfSubscriptionControl - The notification receiver can be the one different with MnS consumer who create and configure the HeartbeatControl instances. - The Figure 4.3.3-1: Procedure for notifying periodic heartbeat notifications is overlapping with Figure 4.3.1-1: Procedure for configuring heartbeat notification periodicity, needs to merge. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedure for heatbeat subscription - Correct the Figure 4.3.1-1 and Figure 4.3.2-1 to describe the MnS producer only send the notifyHearbeat notification to the notification receiver. - Merge Figure 4.3.3-1 to 4.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect procedures for heartbeat notifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.0 (new)，4.3.1, 4.3.2, A.0 (new), A.1, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

4.3	Procedures for heartbeat

4.3.0	Procedure for subscribing heartbeat notification

Figure 4.3.0-1 illustrates the procedure for subscribing heartbeat notifications using operations and notifications of the provisioning MnS (see clause 11.1.1 of [2]). The HeartBeatControl instance is name contained by NtfSubscriptionControl instance, which includes attribute “notificationRecipientAddress” represent the address of Notification Receiver. The Notification Receiver can be the same entity as MnS consumer or different entity.

Figure 4.3.0-1: Procedure for subscribing heartbeat notification

4.3.1	Procedure for configuring heartbeat notification periodicity

Figure 4.3.1-1 illustrates the procedure for configuring the periodicity of heartbeat notifications using operations and notifications of the provisioning MnS (see clause 11.1.1 of [2]).

Figure 4.3.1-1: Procedure for configuring heartbeat notification periodicity

4.3.2	Procedure for requesting immediate heartbeat notification

Figure 4.3.2-1 illustrates the procedure for requesting immediate heartbeat notification using operations and notifications of the provisioning MnS (see clause 11.1.1 of [2]).

Figure 4.3.2-1: Procedure for requesting immediate heartbeat notification

4.3.3	Void

Next change

Annex A (informative): 
Heartbeat PlantUML source code

A.0	Procedure for subscribing heartbeat notifications

The following PlantUML source code is used to describe the procedure for subscribing heartbeat notifications, as depicted by Figure 4.3.0-1:

@startuml

title "Subscribing heartbeat notification"

actor "MnS Consumer" as CONS

participant "MnS Producer" as PROV

CONS -> PROV: request to create HeartbeatControl instance

rnote over PROV

Create and configure the HeartbeatControl instance

Internal Countdown Timer := newHeartbeatPeriodValue

endrnote

PROV -> CONS: response for creating HeartbeatControl instance

hide footbox

@enduml

A.1	Procedure for configuring heartbeat notification periodicity

The following PlantUML source code is used to describe the procedure for configuring heartbeat notification periodicity, as depicted by Figure 4.3.1-1:

title "Configuring heartbeat notification periodicity and Notifying periodic heratbeat notification"

actor "MnS Consumer" as CONS

participant "MnS Producer" as PROV

participant "Notification Receiver" as Receiver

CONS -> PROV: modifyMOIAttributes ('heartbeatNtfPeriod', \n newHeartbeatPeriodValue)

rnote over PROV

Internal Countdown Timer := newHeartbeatPeriodValue

endrnote

PROV -> Receiver: notifyHeartbeat notification

rnote over PROV

reinitialize its internal countdown timer

to the value of heartbeatNtfPeriod

endrnote

hide footbox

rnote over PROV

Start countdown immediately

endrnote

hide footbox

@enduml

A.2	Procedure for requesting immediate heartbeat notification

The following PlantUML source code is used to describe the procedure for requesting immediate heartbeat notification, as depicted by Figure 4.3.2-1:

@startuml

title "Requesting immediate heartbeat notification"

actor "MnS Consumer" as CONS

participant "MnS Producer" as PROV

participant "Notification Receiver" as Receiver

CONS -> PROV: modifyMOIAttributes ('triggerHeartbeatNtf', TRUE)

rnote over PROV

Send immediate heartbeat notification

to Notification Receiver who previously

subscribed to heartbeat notifications

endrnote

PROV -> Receiver: notifyHeartbeat notification

rnote over PROV

triggerHeartbeatNtf:= FALSE

endrnote

hide footbox

@enduml

A.3	Void

End of Changes