3GPP TSG-SA5 Meeting #161	S5-252369

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on CCL DraftTS Rapporteur corrections

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL scope & action-execution conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 1	Scope

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[3]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Concepts and overview

## 4.1	Closed Control Loops

A Closed Control Loop (CCL) is a type of control mechanism that monitors and regulates a set of managed entities with the objective of achieving a specific goal. A CCL can be logically decomposed into several stages or steps, each providing a specific functionality and where the steps work together to achieve the stated goal. Any two CCLs with the same functionality may have the functionality supported in different count of steps implementing the functionality and similarly, any two CCLs with the same functionality and same count of steps, the respective steps may not have the same functionality.

A control loop is a building block for management of networks and services. The basic principle of any control loop is to adjust the value of an observed variable (expressed as for example an attribute) to control/influence the value of a desired goal (expressed as for example an attribute) for a controlled entity, such as a managed entity or managed function. The producers of the measurements or observations services, analysis services and  control service, are all required to fully realize and use a control loop.

A Closed Control Loop (CCL) is a control loop which operates without any intervention from a human operator or any other management entity other than possibly the initial configuration of the measurement producer and configuration of the control loop. In a closed control loop the input to the control loop provided by human operator or other management entity may include the goal or policies. Besides the provisioning needed to realize the goal, the output of the closed control loop may also include closed control loop status to a human operator or other management entity.

Examples of well-known Closed Loop types are OODA loop, composed of 4 stages/steps (Observe, Orient, Decide, Act) and MAPE, also composed of 4 stages/steps (Monitor, Analyse, Plan, Execute).

- b)

Figure 4.1-1: Open control loop entities versus Closed control loop entities (see TS28.535[2])

## 4.2	Functional steps of a closed control loop

A closed control loop may manage any managed entity, e.g. a network resource or a communication service as described in TS 28.535 [2] and TS 28.536 [3]. Generally, the control loop consists of the steps Monitoring/data collection, Analysis, Decision and Execution. The adjustment of the resources of the managed entity used is completed by the continuous iteration of the steps in a management control loop.

Figure 4.2-1: Steps of a Control Loop
a) the four functional steps; and
b) 2 steps combined into a single management function

The "Monitoring/data collection" step is responsible for collecting and pre-processing data from managed entities or from external sources.

The "Analysis" step derives insights from the available data obtained in the monitoring/data collection step. The insights provide answers to the questions, for example What is likely to happen, what has happened and why.

The "Decision" step is responsible for deriving workflows from insights provided by the analysis step. It decides which reactive, proactive or predictive actions should be taken in consideration of insights obtained in the analysis step.

The execution step manages the activation of commands on the controlled resources or entities. The decision step should decide which actions are required, but not necessarily how they should be taken in the managed entities. So, the translation from actions to commands is a responsibility of the execution step.

## 4.3	Characteristic information of a CCL

### 4.3.1		Overview

A CCL is associated with a set of Characteristic information that describes its properties, behaviours and impact. This information includes CCL Goals, CCL Triggers, CCL actions and action plans as well as CCL scopes.

### 4.3.2	CCL Goals and targets

The CCL is the set of outcomes that need to be achieved or realized by the CCL. Each expected outcome is called a CCL target. A CCL may have one more CCL goals each containing a set of CCL targets.

### 4.3.3	CCL actions and actions plans

A CCL action is a configuration change that a CCL can perform over a managed entity such as configuring an attribute of  managed object. A CCL may decide to perform several actions which can be combined in a single CCL action plan.

### 4.3.4	CCL scopes

The scope is the set of managed objects, their properties and network outcomes that are associated with the CCL for measurement, configuration and impact. The scopes for the different CCLs can be managed by the MnS consumer, i.e. they can be defined on to the CCL or revised by the MnS consumer. A CCL may have four scopes - the measurement scope, target (impact) scope, control scope and impact scope, defined as follows:

-	measurement scope: the measurement scope is where related measurements are collected

-	control scope: control scope is the scope to which the CCL's actions are desired to be applied, e.g.,  the set of network functions and attributes that are the planned candidates to be modified by the CCL. The control scope is also called the action-space as it describes the set of candidate actions that the CCL can (is configured to be able to) execute.

-	target scope: which relates to purpose of the CCL

-	desired impact scope: the scope to which the CCL's actions are desired to have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are planned to be influenced by the configuration’s actions of the CCL.

-	impact scope: impact scope is the scope to which the CCL's actions have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are influenced by the configuration actions of the CCL. This is different from the measurement scope, i.e. the scope where the CCLs measure and control scope, i.e. the scope where they act.

The impact scope may be known and bounded or unbounded and thus unknown - see figure 4.3.4-1. The bounded scope indicates that the area known by the CCL is the scope where its actions will impact. The unbounded impact-scope is the full network scope where the CCL’s action will have impact, but the CCL does have information that its action will have that impact to that scope.

Figure 4.3.4-1: Exemplification of known/bounded vs. unknown/unbounded impact scope: CCL A takes action in cell A expecting impact in cells A, B, C and D. if the impact is strictly in cells A, B, C and D, then the impact scope is known and bounded. However, if the impact scope includes cells E and F, then for the CCL, the true impact scope is unknown and thus unbounded.

## 4.4	Closed control loops purposes and uses

CCLs automate the management of network resources thereby taking control away from operators. TS 28.535 [2], TS 28.536 [3] describe the use of CCLs for service assurance. Additionally, CCLs may be used for several other use cases including among others use of CCLs for problem analysis and for fault management.

In each case, the CCL can be viewed as an entity to be managed, management capabilities related to the CCL will be exposed by the MnS producer that is associated with the CCL to enable the MnS consumer to manage the CCL. E.g., the characteristics and behaviours of the CCLs need to be directed by operators as consumers of CCL-related management services.  Moreover, the MnS also exposes capabilities for coordinating the CCL's activities. Example capabilities include capability for providing information on conflict resolution and feedback on monitoring the impact of the CCL's actions on other closed control loops or management functions.

The 3GPP management system should provide capabilities that enable a consumer to:

-	Manage the execution of CCLs. E.g. to request for and be notified about the instantiation of CCLs. For instance, if the consumer wants to request for instantiation of an Energy saving CCL for 10,000 cells.

-	Compose or request for and be notified about the composition of a CCL from a set of specific components (such as analytics services or SON functions).

-	Manage a closed loop composed from multiple components.

# 5	Management capabilities

## 5.1	Dynamic composition of CCLs - DynCCL

### 5.1.1	Description

CCLs may be dynamically realized. There are two aspects to dynamically realization of CCLs - dynamic instantiation of a CCL from an existing template and dynamically composing the CCL from discrete components based e.g. on the provided goals.

### 5.1.2		Use cases

#### 5.1.2.1	Composing a CCL from discrete components – DynCCL_0x

A CCL may be composed from steps provided by different management functions or management services. i.e. the CCLs is assembled on demand by MnS consumers, using capabilities offered by the Management system, e.g. from independent management functions. The CCLs components, as well as the communication and interoperation between components, are based the different 3GPP management services. Accordingly, the MnS consumer should be able to identify and indicate the MnFs or MnS producers that should be used to compose a CCL. Moreover, the MnS consumer may indicate towards the MnS producer the request to compose the CL of a particular type (e.g. for optimizing energy efficiency) without requiring to state the specific components that should be used.

Two approaches are possible:

-	Composition from management Functions: Different management functions may be used to realize the different steps of a closed loop, for example, an MDA function may realize the analytics step of the CCL while another management function may realize the decision step of the CCL.

Figure 5.1.2.1-1: Management functions as steps of a closed control loop

-	Composition from management services: Different management services may be used to realize the different steps of a closed loop, i.e. the management service provides the output expected from a specific step.

EXAMPLE:	A capability of the MDA MnS realizes an analytics step of the CCL while another capability may realize a specific data collection step of the CCL.

b)

Figure 5.1.2.1-2: Management services used as implementations of CCL steps:
a) MDA MnS and PM job the respective implementations of the analysis and data collection steps; and b) MDA MnS as the implementation of the decision step

The MnS consumer should be enabled to control the composition of such a CCL. The MnS consumer could request for and be notified about the composition of a CCL from a set of specific components (i.e. specific management functions or management services). The request could indicate components with specific given capabilities (such as analytics services with specific analytics types) which should be combined to achieve the closed loop. Moreover, the request could be for composition of a CCL required to achieve a specific set of desired outcomes or goals.

### 5.1.3	Requirements

Table 5.1.3-1

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-DynCCL_0x-01 | The CCL MnS Producer should support a capability enabling the MnS consumer to request for a CCL (instance) to be composed from a set of management function types or instances or management services | UC-DynCCL_0x Clause 5.1.2.1 |
| REQ-DynCCL_0x-01 | The MnS producer for CCL management should support a capability enabling the MnS consumer to request that a CCL of a specific type or fulfilling a stated goal should be composed from a set of management function types or instances or services | UC-DynCCL_0x Clause 5.1.2.1 |
| REQ-DynCCL_0x-01 | The MnS producer for CCL management should support a capability enabling the MnS consumer to provide conditions under which a CCL can be dynamically composed or instantiated triggered to execute | UC-DynCCL_0x Clause 5.1.2.1 |
| REQ-DynCCL_0x-01 | The MnS producer for CCL management should support a capability enabling the MnS consumer to be notified when a CCL is dynamically composed or instantiated or triggered to execute | UC-DynCCL_0x Clause 5.1.2.1 |

## 5.2		Historical CCL Capability - HISCCL

### 5.2.1		Description

TBD

### 5.2.2	Use Cases

#### 5.2.2.1	CCL creation based on Historical CCL data capability – HISCCL-01

This use case describes the need of maintaining information about the CCLs that existed in the past. Those CCLs are called Historical CCLs.

In an automation environment, before a consumer request to create a CCL it would like to know the data related with Historical CCLs that were available with the producer. This information will enable consumer to request for an optimal CCL. The information about historical CCL may include, scope of the CCL, configured goals/targets, controlled entity, etc.

Further, Historical CCL information serves as a valuable data source for predictive analytics within the CCL system executed as Analytics step. It enables the system to move from a reactive mode, where it responds to current issues, to a proactive mode, where it anticipates and prevents problems based on historical trends and patterns. This proactive approach enhances network reliability, minimizes downtime, and improves the overall efficiency of network operations.

The Historical CCL information may be used by the management system to setup or initialize a CCL. The Historical CCL information provides the profiles of a CCL for CCL at different hierarchies. For example, CCLs that do not do coordination which are at a lower hierarchy L and CCLs responsible for coordination (as coordination entities) which are at a higher hierarchy H. For a new CCL at a lower hierarchy, the management system obtains the profiles of the several CCLs at different hierarchies and correlates the information of the new CCL (e.g. its goal information) against the profiles of the CCLs at the different hierarchies. Based on this, the management system computes the complete profile of the new CCL (including e.g. its measurement and control scope) which is then configured onto the new CCL.

### 5.2.3	Requirements

Table 5.2.3-1

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-HISCCL_01-01 | The 3GPP management system should enable authorized MnS consumer to request for information (e.g. CCL identification, configured goals/targets and the related status, scope of the CCL, conflict information) related with Historical CCL. | UC-HISCCL_01 Clause 5.2.2.1 |
| REQ-HISCCL_01-02 | The 3GPP management system shall have the capability to configure the profile of a CCL based on the historical CCL information that describes the profile of other CCLs at different hierarchies. | UC-HISCCL_01 Clause 5.2.2.1 |

## 5.3		CCL Performance Monitoring - CCLPERF

### 5.3.1		Description

TBD

### 5.3.2	Use Cases

#### 5.3.2.1	Performance Evaluation of a Closed Control Loop – CCLPERF_01

The advanced monitoring functionalities of a CCL can provide real-time insights into the performance and outcomes of a CCL. The monitoring activity for a Closed Control Loop may result in further actions that happen in the operation phase, e.g., evaluate and update, in order to change the closed control loop settings and improve its performance. So, there is a need to evaluate the performance of a Closed Control Loop itself. Such metrics are important to understand and change a CCL's behaviour and to improve its performance to pursue the assigned goal(s).

For example, certain performance aspects of a CCL can be very crucial to know in order to evaluate and decide upon a CCL's performance, such as the number of breached goals, time taken to meet a breached goal, number of conflicts occurred by a CCL etc. With the knowledge of such performance aspects of an existing CCL, a MnS consumer can more effectively update or create a new CCL.

An operator can also compare different CCLs based on these performances and choose the best one for its network deployment.

### 5.3.3		Requirements

Table 5.3.3-1

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CCLPERF_01 -01 | The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of occurrences of a goal breach. | UC-CCLPERF_01 Clause 5.3.2.1 |
| REQ-CCLPERF_01-02 | The 3GPP management system should be able to obtain a CCL's performance with respect to the time taken by CCL to meet a breached goal. | UC-CCLPERF_01 Clause 5.3.2.1 |
| REQ-CCLPERF_01-03 | The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of conflicts occurred by a CCL | UC-CCLPERF_01 Clause 5.3.2.1 |

## 5.4	Closed Control Loops usage scenarios - CCLUSE

### 5.4.1		Description

. Close control loops can be used for different purposes or scenarios Two example scenarios are fault management and network performance problem recovery.

### 5.4.2	Use Cases

#### 5.4.2.1	Closed Control Loops for fault management – CCLUSE_01

This use case describes a scenario in which an MnS consumer may request a CCL for fault management. The consumer may request to identify the root cause of the fault and take actions to mitigate and/or resolve the root cause for a given list of alarms. Furthermore, the request may include policies and actions specified by an MnS consumer in order to mitigate and/or resolve the root causes for the given alarms.

Based on the request, a CCL may take action to further enhance the correlation of alarms, for example, correlation of alarms with change in PM/KPIs and/or fault supervision events to find solutions to mitigate and/or resolve the identified root causes.  In addition, a fault management CCL may clear the alarms that otherwise have to be manually cleared by the MnS consumer, which are defined as ADMC Alarms  in TS 28.111.

The MnS producer reports the result of fault management. The report may include information regarding the status of each alarm, including any identified root cause and correlation information, which may indicate the successful mitigation and/or resolving of root causes for any given alarm.

#### 5.5	Closed Control Loops for network performance problem recovery CCLUSE_02

Based on the concept in 3GPP TS 28.104 [3], MDA reports may contain root cause analysis of ongoing issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands. For example:

-	MDA for Coverage problem analysis can provide the following information in the MDA report:

-	coverageProblemId;

-	coverageProblemType;

-	coverageProblemAreas; and

-	recommendedActions.

MnS consumer may decide to resolve the observed performance problems based on the analytics reports (e.g. provided by MDA) and other management data (e.g. historical decisions made previously) if necessary. It can be possible that one MnF  is responsible for network performance problem observation and recovery, while another MnF is responsible for decision on whether the network performance problem needs to be resolved. In this scenario, The MnF for decision can decide whether needs the another MnF to recovery the observed network performance problems (e.g. coverage problem) based on MDA report (e.g. root cause information, recommended solutions) and other information (e.g. user experience information, information from other domains). If decides to recover the observed network performance problems, The MnF for decision needs to request another MnF to recover the specified network performance problems observed from the MDA report. MnS consumer may specifies the network scope and time window for network performance problem recovery, which means the MnS producer needs to recover the problem at the specified time window for the network scope. During problem recovery phase, as process for network performance problem recovery is complex and time-consuming, the MnS consumer needs to obtain the progress of the recovery process. When the last step of the network performance problem process is completed, MnS producer needs to send the result of this network performance problem recovery process to the MnS consumers.

If a closed control loop instance can be used to resolve network performance problem, the MnS consumer may need to know the result of resolving the network performance problem by the closed control loop instance, including the network performance problems which are resolved by the closed control loop as well as network performance problem resolution statistics (e.g. the number of network problem resolved by the closed control loop in the specified period).

### 5.4.3	Requirements

Table 5.5.2-1

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CCLUSE-CCLFM-01 | The 3GPP management system shall have the capability to allow MnS consumer to request a closed control loop for fault management | UC-CCLUSE_01 |
| REQ-CCLUSE-CCLFM-02 | The 3GPP management system shall have the capability to allow MnS consumer to get a report from the closed control loop regarding the status of fault management | UC-CCLUSE_01 |
| REQ-CCLUSE_02-01 | The 3GPP management system should have the capability to allow the MnS consumer to request a CCL for resolving the network performance problems. | UC-CCLUSE_02 Closed Control Loops for network performance problem recovery |
| REQ-CCLUSE_02-02 | The 3GPP management system should have the capability to allow the MnS consumer to obtain the result of network performance problem resolved by the closed control loop. | UC-CCLUSE_02 Closed Control Loops for network performance problem recovery |
| REQ-CCLUSE_02-03 | The 3GPP management system should have the capability to allow the MnS consumer to obtain the progress information of network performance problem recovery. | UC-CCLUSE_02 Closed Control Loops for network performance problem recovery |
| REQ-CCLUSE_02-04 | The 3GPP management system should have the capability to allow the MnS consumer to configure a CCL with the network scope and time window to be monitored for i resolving the network performance problems. | UC-CCLUSE_02 Closed Control Loops for network performance problem recovery |

## 5.6	Triggered CCL Capability - CCLTRG

### 5.6.1		Description

### 5.6.2	Use Cases

#### 5.6.2.1	CCL instantiation based on conditions – CCLTRG_01

The MnS consumer may want to request for a CCL to be instantiated not immediately but when certain conditions are met. For example, the MnS consumer may want that for a CCL of a stated type or that matches a set of stated characteristics (e.g. goal) to be instantiated under certain conditions and another with variations in goals to be instantiated under other conditions. The MnS consumer should be enabled to define those conditions so that the CCL is instantiated when the stated conditions are met. The MnS Producer monitors the conditions to check if they are met.

The conditions can be related to events based on management data (e.g., performance, fault, configuration).

Performance events are defined related with performance measurements and KPIs that need to be monitored by the producer to see if an CCL is to be initiated. For example, if the value of a particular performance measurement goes beyond a particular value, a CCL should be instantiated to keep the value of the same performance measurement below a defined value.

Fault events are defined by related information (e.g alarm type, alarm severity) that need to be monitored by the producer to see if a CCL is to be initiated. For example, if the total number of alarm with type QUALITY_OF_SERVICE_ALARM and perceivedSeverity MAJOR goes beyond a particular value, a CCL should be instantiated.

Provisioning events (e.g CreateMOI) are defined that need to be subscribed by the producer to see if a CCL is to be instantiated. For example, the creation on an Intent MOI can be a trigger to instantiate a CCL. For another example, when a pre-defined system time specified by operators can be a trigger to instantiated a CCL.

NOTE:	The use case requires to set the conditions and then the conditions need to be continuously monitored and tracked.

#### 5.6.2.2	CCL action execution based on conditions – CCLTRG_02

For the CCLs that have been instantiated, the MnS consumer may want to define conditions under which a CCL may execute actions on the network, e.g. when the performance on a certain threshold is crossed, or when the confidence on the decision is above a stated threshold. The consumer does not need to be aware of all decisions, but by providing conditions under which decisions may be activated or not, it is able to have supervision over the CCL without having to continuously track the decisions. The MnS consumer should be enabled to define those conditions for executing the CCL actions. Otherwise, the consumer should be enabled to define alternative actions, e.g. to notify the consumer of the decision that is not executed.

By supporting this, the execution can be affected by producer based on consumer's conditions or requirements. To ensure oversight or accountability by the MnS consumer, the MnS consumer may be notified by the CCL about the executed action for any conditionally execution. The MnS consumer can intervene as needed.

NOTE:	The use case requires to set the conditions and then the conditions need to be continuously monitored and tracked.

### 5.6.3	Requirements

Table 5.6.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CCLTRG_01-01 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger CCL instantiation. | UC–CCLTRG_01 Conditional trigger of a CCL |
| REQ-CCLTRG_01-02 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger CCL update. | UC–CCLTRG_01 Conditional trigger of a CCL |
| REQ-CCLTRG_01-03 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger CCL deletion. | UC–CCLTRG_01 Conditional trigger of a CCL |
| REQ-CCLTRG_02-01 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data under which a CCL may execute its actions. | UC–CCLTRG_02 Conditional execution of CCLs network changes |

## 5.7	CCL Conflict management Capability - CONF

### 5.7.1	Description

A CCL may experience direct conflicts on its goals, targets, scopes, trigger time and execution time. The management system needs to support capabilities to avoid, detect and resolve the conflicts.

### 5.7.2	Use Cases

#### 5.7.2.1	CCL Trigger-time conflicts handling - CONF_01

Typically, a CCL whose start is triggered based on conditions, needs to be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

The management system should support the capability for avoidance of action-execution-time conflicts. Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  For actions in a given network scope, the CCLs can be explicitly scheduled by the management system. Where the scopes overlap, the CCLs need to align the action plans, for example, which action plan to execute and when. There is a need to assess each plan and choose the most appropriate combination of action plan(s) based on the selection policy and then notify the selected action plan(s) to the related CCLs. The MnS consumer may also be notified when it is safe to ignore the conflict. The MnS consumer may configure the criteria for evaluating the severity of conflicts.

#### 5.7.2.2	CCL action conflicts handling –CONF_02

When two (or more) CCLs attempt to adjust the same network parameter but with different and contradicting values, the desired actions of the 2 CCL will be in conflict. For example, a CCL assuring throughput of a slice may be scaling-out the virtual resources of the slice. Whereas a CCL minimizing the energy consumption may be scaling-in the virtual resource of the same slice. It can be when the CCLs execute actions at the same time. However, it also happens when the CCLs execute at different times, and the scenario for actions to be separated in time is the more likely than actions occurring simultaneously. casein these conflict scenarios, the network parameter continuously ping-pongs between the two values. Such a conflict may be called an action conflict.

NOTE:	A potential conflict can for example be detected if a CCL observed that PMs on a certain object keep flipping between two values. The constant flipping can be an indication that 2 CCL instances are attempting to change the same scope.

The CCL may detect or observe events that identify the conflicts. The conflict can be avoided using some information or the policies (e.g. priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. It is needed to maximize the avoidance of conflict, including “requesting” information from MnS consumer and to inform MnS consumer about the potential conflict. CCL MnS Producer may also provide recommendations, for updating/deleting the conflicting CCLs, that would result in the resolution of detected conflict. The recommendation for update may include suggestions for modified targets.

Editor’s Note:	The exact information that can be exchanged is FFS

#### 5.7.2.3	CCL Indirect-target conflicts handling – CONF_03

Two (or more) CCLs configuring different control parameter may all influence the same metric. In other cases, the two CCLs influence two metrics Y1 and Y2 that are couple, i.e., which have a logical relationship between them. E.g. handover (HO) failure and SINR are coupled since a bad SINR can lead to more HO failures. If the two CL desire different values for the metric, or different values for two target metrics Y1 and Y2 but the targets are coupled, the CCLs are in conflict for the metric resulting into a target-metric-value conflict or indirect target conflicts.

Two target metrics Y1 and Y2 may be coupled such that actions to optimize any of them lead to correlated oscillations/degradations in Y1 or Y2, e.g. Y1 ensuring "HO failure is < 2 %" and Y2 wanting "SINR > 10dB". The correlated oscillations indicate a potential conflict, but the CCLs may not see the oscillations in the metric that is not of their interest. The management system should support the capability for detecting potential indirect targets conflicts. An MnS consumer may analyse the correlations to detect the potential conflict between CCL1 and CCL2. The MnS consumer should be able to inform CCL1 and CCL2 about the detected potential conflict represented by the correlated oscillations.

This severity of degradation in the performance metrics of the related CCLs could be the confirmation that a detected potential conflict is an actual harmful conflict. The management system should support the capability for detecting or confirming actual indirect targets conflicts. The threshold to determine the severity may be defined by the MnS consumer (e.g. the operator) so that if the degree of degradation is higher than the threshold then it is a confirmed conflict that requires resolution.

The management system should support the capability for avoiding potential indirect target conflicts. CCLs need to avoid large and frequent changes to network parameters which may affect network stability since they increase the probability of occurrence of conflicts. CCLs should take small smooth changes in the cases where the impact is not so clear and only make the large changes when the CCL is sure that the impact is positive. It is desirable for the CCL to notify to the MnS consumer the planned change, its claimed/predicted performance improvement and reliability/confidence in that action/decision. The MnS consumer may evaluate the claimed performance improvement and reliability/confidence to determine if the action should be allowed or not. The MnS consumer should be enabled notify the decision and possibly the failed criteria to the CCL - to either be executed or to be used to compute better decisions. Based on the inputs, the CCL may update its decision-making and repeat the decision evaluation process. If the CCL has consistently made good large-action-decisions, the MnS consumer should be enabled to inform the CCL that the CCL has consistently made good decisions and achieved its ultimate trust and that no more coordination of its decisions is needed.

The management system should support the capability for resolving detected indirect targets conflict. The MnS consumer should be enabled to trigger one or more CCLs to respond to the detected potential conflict. And if the triggered CCLs is unable to resolve that conflict, the CCL should inform the MnS consumer about the failure to resolve the problem. The MnS consumer can set the thresholds for performance degradation that triggers conflict detection and resolution.

Editor’s Note 1: The criteria for accurately setting the thresholds for performance degradation is FFS.

Editor’s Note 2: The name and description of this type of conflict will be revisited.

### 5.7.3	Requirements

Table 5.7.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CONF_01-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL trigger-time conflicts. | UC-CONF_01 |
| REQ-CONF_01-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL trigger-time conflict after it is confirmed. | UC-CONF_01 |
| REQ-CONF_01-03 | The 3GPP Management System should enable authorized MnS Consumer to provide information that can be used to support a capability to avoid or resolve a CCL trigger-time conflict. | UC-CONF_01 |
| REQ-CONF- _02-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential action conflict. | UC-CONF- _02 |
| REQ-CONF- _02-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about an actual action conflict. | UC-CONF- _02 |
| REQ-CONF- _02-03 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL action conflict. | UC-CONF- _02 |
| REQ-CONF- _02-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid the action conflict. | UC-CONF- _02 |
| REQ-CONF- _02-05 | The 3GPP Management System should support a capability to coordinate the resolution of CCL action conflicts among multiple CCLs | UC-CONF- _02 |
| REQ-CONF_03-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL Indirect targets conflicts. | UC-CONF- _03 |
| REQ-CONF_03-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL Indirect targets conflict after it is confirmed. | UC-CONF- _03 |
| REQ-CONF_03-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL Indirect targets conflict that has been detected | UC-CONF- _03 |

## 5.8	CCL decision escalation – ESC

### 5.8.1	Description

This use case related to the capability to escalate decision making to another entity e.g. another CCL.

### 5.8.2	Use Cases

#### 5.8.2.1	Triggering CCL decision escalation – ESC_01

Not all decisions made by CCLs in different network contexts (states, status, conditions, etc.) are equally effective. The CCL may need to inform another entity about its lack of confidence in its decision with a request to escalate its decision making to that entity. For example, a CCL for optimizing energy saving may fail to decide the sequence in which cells may be deactivated when there is a failure for some cells. The CCL may escalate the scenario to a CCL on problem recovery.

The MnS consumer should be able to configure MnS producer regarding the escalation recipient to which the decision is escalated. The degree to which the CCL can independently execute decisions or escalates them, should be configurable by the MnS consumer through a confidence threshold. The confidence threshold is an index on a fixed scale say from 0 (indicating lowest confidence) to 10 (indicating highest confidence). It could be configured based on the sensitivity of the operations under the CCLs’ control, the trust level in the decisions of the CCL and the necessity to consider a bigger picture at times. Then, based on how much confidence the CCL has in its decisions, the CCL can escalate a decision or situation to an escalation recipient (e.g. another CCL or a CCL coordination entity) which has this bigger picture (say has wider scope), can execute a different(larger) set of actions or has better capabilities, e.g. a larger and more capable ML model.

NOTE:	The computation of confidence within the CCL is up to implementation as it depends on the CCL's purpose and the scenario that the CCL is addressing. The escalation recipient CCL should enable the escalator CCL to request for escalation for a given network context or state with e.g. information about the escalator CCL preferences and observed constraints when driving decisions. Based on its evaluations, the escalation recipient CCL should provide to the escalator CCL a report that holds the outcomes for a given escalation request.

Figure 5.8.2.1-1: required interactions for CCL decision escalation

### 5.8.3	Requirements

Table 5.8.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ESC_01-01 | The 3GPP management system should have a capability to enabling an authorized MnS consumer to configure a CCL with the degree of autonomy of to define when the CCL can escalate and the entity to which to escalate decision making. | UC-ESC-_01 Clause 5.C.2.1 |
| REQ-ESC_01-02 | The 3GPP management system should have a capability to enabling an authorized MnS consumer (e.g. an escalator CCL) to request to escalate decision-making for a network context or state to an escalation recipient. | UC-ESC-_01 Clause 5.C.2.1 |
| REQ-ESC_01-03 | The 3GPP management system should have a capability enabling an escalation recipient CCL to report to an authorized MnS consumer (e.g. an escalator CCL) the outcomes for a given escalation request | UC-ESC-_01 Clause 5.C.2.1 |

## 5.9	CCL Coordination – COORD

### 5.9.1	Description

This use case related to the coordination of CCL, e.g. with other management capabilities

### 5.9.2	Use Cases

#### 5.9.2.1	Coordinating CCLs with other management functions – COORD_01

A CCL can make and execute decisions in different network contexts and for different network functions and parameters. Yet within the network, there may be other management functions or features including MDA functions, SON functions, and AIML Functions, which also make decisions that affect the same network functions and parameters as the CCL. The operation of CCLs needs to be coordinated with the other management functions.

NOTE 1:	This use-case only focuses on coordinating CCLs with other management functions for executing decisions.

For a given context, the CCL should indicate the set of network functions and corresponding parameters which it is interested in changing. Accordingly, the MnS consumer, say responsible for coordinating the CCLs with management functions may subscribe to be notified of changes on network functions and parameters. The MnS consumer should be able to inform the CCL of the latest changes to a network function or its parameter and a management entity/function (e.g. CCL, MDA, SON, AI/ML inference Function) responsible for the change to the parameter.

The CCL may want to obtain the history of previous values of the parameter. The history includes, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. The CCL may define a favourable range of values of the parameter based on the received information on the latest change and the history of previous changes to the parameter. The CCL can calculate a new value of the parameter considering the favourable range as a constraint for the new value. The CCL needs then to update the value of the parameter of the network function to the new value.

NOTE 2:	The MnS consumer may for example be the functionality that is responsible for coordinating CCL and other management functions.

### 5.9.3	Requirements

Table 5.9.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-COORD_01-01 | The CCL MnS producer should have a capability to indicate to an MnS consumer the set of network functions including their parameters which it is interested in changing | UC-COORD_01 |
| REQ-COORD_01-02 | The management system should have a capability enabling an authorized CCL instance acting as MnS consumer to receive information on the latest changes to a network function parameter and an identifier of a management entity/function including MDA Function, a SON Function or an AI/ML inference Function that responsible for the change to the parameter. | UC-COORD_01 |
| REQ-COORD_01-03 | The management system should have a capability enabling an authorized MnS consumer to receive the history of previous values of the parameter, including, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. | UC-COORD_01 |

# 6	Model

## 6.1	Imported and associated information entities

TBD

### 6.1.1	Imported information entities and local labels

TBD

### 6.1.2	Associated information entities and local labels

TBD

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Relations for common information models for AI/ML management

Editor’s Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

### 6.2.2	Inheritance

Figure 6.2.2-1: Inheritance Hierarchy for Closed Control Loops

### 6.3	Class definitions

### 6.3.1	ClosedControlLoop

#### 6.3.1.1	Definition

This IOC represents the closed control loop. It represents the information for controlling and monitoring a CCL associated with a stated scope.

The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contains reported  information about the CCL. Accordingly, the report about a CCL can exist even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLPurposes that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLPurpose(s) that indicate(s) a list of characteristics that describe what a CCL can/is expected to be able to do. The purpose describes the type of functionality that can be executed including problem recovery and fault management .

The operational information about the CCL is contained in the CCLScope(s), so the ClosedControlLoop is associated with one or more CCLScope(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc.

#### 6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

#### 6.3.1.3	Attribute constraints

None

#### 6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.2	CCLScope

#### 6.3.2.1	Definition

It indicates a scope of a CCL. It may be the measurement scope, control scope or impact scope.

The CCLScope includes the attribute scopeType that indicates the type of scope that represented by the particular scope instance.

#### 6.3.2.2	Attributes

The CCLScope IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.2.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | O | T | F | F | T |

#### 6.3.2.3	Attribute constraints

None.

#### 6.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.3	CCLReport

#### 6.3.3.1	Definition

This class represents the reported outcomes on a CCL instance, e.g., the information about the outcomes on one or the executing of the CCL. An CCLReport is contained by the entity containing the CCL, since the CCLReport can exist beyond the life of the CCL on which it is reporting.

There is one CCLReport per CCL for an observation time. The content of the CCLReport may be different for different observation time.

#### 6.3.3.2	Attributes

The CCLReport IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.4	CCLPurpose <<dataType>>

#### 6.3.4.1	Definition

This data type represents a single purpose that describes what a CCL can do. The purpose is alist of characteristics that describe the capabilities of the CCL.

#### 6.3.4.2	Attributes

Table 6.3.4.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions..

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

# 7	Procedures

## 7.1	CCL Performance Monitoring

When the PA (Performance Assurance)MnS consumer notices that a slice or network performance is degrading, it may require to know information about available CCLs that have the goals related to this performance degradation.This may imply that the performance of the related CCL is not as expected. This requires performance management to be done on the available CCL including further actions such as evaluating and updating closed control loops. The metrics for assessing performance  of CCLs, for example, total number of occurrences of a goal breach, time taken by CCL to meet a breached goal, total number of conflicts occurred by a CCL are defined in clause 8. A procedure for performance management of CCLs involving these performance metrics is described below

Figure 7.1-1: Performance monitoring procedure for a closed control loop

Step 1. PA/CCL MnS consumer notices that a certain performance metric of a SLS or a network starts degrading.

Step 2. PA/CCL MnS consumer sends getMOIAttributeRequest message to PA/CCL MnS producer for getting information about all CCLs attributes.

Step 3. PA/CCL MnS producer provides this information of all CCLs to the consumer in getMOIAttributeResponse message.

Step 4. PA/CCL MnS consumer identifies the CCL (n) which is responsible for maintaining the performance of slice or network.

Step 5. PA/CCL MnS consumer sends createMOI(PerfMetricJob) request to PA/CCL MnS producer for obtaining status of following performance metrics for that particular CCL(n) as defined in clause 8 - TotalAssuranceGoalBreach, TimeCorrectiveGoalMeet, TotalCclConflicts_Filter.

Step 6. PA/CCL MnS producer provides requested performance metric values via createMOI() Response message to PA/CCL MnS consumer.

Step 7. PA/CCL MnS consumer has two choices – either to update the existing CCL n (of step 4) to achieve the desired goal or to create a new CCL for the same. If PA/CCL MnS consumer chooses to modify an existing CCL, it sends a modifyMOIAttributes request message for that CCL or it can also update by sending changeMOIs request message to PA/CCL MnS producer.

Step8. Accordingly, PA/CCL MnS producer sends modifyMOIAttributes Response or changeMOIs response message to PA/CCL MnS consumer for the updated attributes of CCL n.

Step9. If PA/CCL MnS consumer chooses to create a new CCL for the desired goal, it does so by sending createMOI Request message to PA/CCL MnS producer.

Step10. PA/CCL MnS producer provides createMOI() Response message for the newly created CCL MOI to PA/CCL MnS consumer.

Annex A (informative):
UML code for model diagrams

# A.1		UML code for CCL management model diagrams

This annex contains the PlantUML source code for the NRM diagrams defined in clause 6.2 of the present document.

## A.1.1	CCL NRM fragment (Figure 6.2.1.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ClosedControlLoop <<InformationObjectClass>>

class CCLPurpose << ProxyClass >>

class CCLScope << InformationObjectClass >>

class CCLReport <<InformationObjectClass>>

ManagedEntity "1" *-- "*" ClosedControlLoop: <<names>>

ClosedControlLoop "1" *--> "*" CCLPurpose

ClosedControlLoop "1" *-- "*" CCLScope: <<names>>

ClosedControlLoop "1" *-- "*" CCLReport: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CCLPurpose

Can be any of these CCL purposes:

NetworkProblemRecovery

FaultManagement

...

end note

@enduml

Source code for Figure 6.2.1.1-1 CCL NRM fragment

## A.1.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLScope

Top <|-- CCLReport

@enduml

Source code for Figure 6.2.1.1-2 CCL inheritance relationships

Annex B (informative):
Selection of conflict coordination approach

# B.1	Alternative CCL coordination Approaches for conflicts handling

To address the conflicts, coordination interactions are required either among CCLs or between the CCLs and one or more higher hierarchy coordination functions to avoid or detect and resolve the conflicts. This is required when CCL are actuating in the same set of resources.

The coordination of CCLs could be accomplished via one of three approaches illustrated by Figure B.1-1:

-	Distributed coordination with distributed execution (Figure B.1-1 a), where the CCLs directly coordinate with one another, and each manages execution of its decisions. The CCL exchange information with each other avoid, detect or resolve conflicts. The information may for example include notifications of executed actions or observed impacts.

-	Hierarchical coordination with distributed execution (Figure B.1-1 b), where the CCLs coordinate through a separate coordination layer, say via a CCL coordination entity, but each manages execution of its coordinated decisions. The CCL exchange information with the CCL coordination entity to avoid, detect or resolve conflicts. A CCL may send notifications of its executed actions or observed impacts which the CCL coordination entity may relay to other CCLs. The CCL coordination entity may configure the CCLs but each CCL executes its action based the CCL coordination entity’s configuration.

-	Hierarchical coordination and execution (Figure B.1-1 c), where the CCLs coordinate through a separate coordination layer, say via a coordination entity that besides coordination also manages execution of the coordinated decisions. The CCL exchange information with the CCL coordination entity to avoid, detect or resolve conflicts including notifications of their executed actions or observed impacts which the CCL coordination entity may relay to other CCLs. The CCL coordination entity may configure the CCLs and the CCL execute their actions through the CCL coordination entity.

Figure B.1-1: Closed Control Loop Coordination approaches

Distributed coordination can lead to too many exchanges between the CCLs which may unnecessarily clog the system. On the other hand, "Hierarchical coordination and execution" implies that too much responsibility is concentrated in a single CCL. A desired behavior is that the individual CCLs are responsible for their own decision execution, so it is recommended that to follow the "hierarchical coordination with distributed execution" approach. In this approach, the CCLs are responsible for making their decisions and executing actions, but they coordinate with the CCL coordinator before, during or after execution.

# B.2	Hierarchical CCL-coordination-interactions for conflicts handling

To address the conflicts, coordination interactions are required between the CCLs and one or more higher hierarchy coordination functions to avoid or detect and resolve the conflicts among goals and their targets, control scopes or actions of the CCLs. The 3GPP management system includes at least one entity called the Coordination entity that undertakes the role of CCL coordination. The Coordination entity can be implemented as a CCL, an AIML inference engine or any other functionality that is found appropriate. The coordination entity may support coordination for conflict management for different conflicts described in clause 5.7-5.9 including Goal- and Goal-targets- Conflicts; scope conflicts, CCL-Trigger-time and CCL-action-execution-time conflicts, Direct actions conflicts as well as indirect targets conflicts.

The coordination of CCLs could be required at different execution points of the CCL translating into different CCL coordination use cases with corresponding CCL coordination services required at those points as illustrated by example Figure B.2-1. The coordination of CCLs could be achieved via direct interaction among the CCLs or via a third-party entity, say called the CCLs coordination Function (or simply CCL Coordinator).

Figure B.2-1: Exemplary Closed Control Loop Coordination interaction points

NOTE:	The terms at the top indicate general naming of the groupings of coordination interactions at the different execution points during the execution of the CCL. Action-space coordination implies coordinating the sets of actions that the different CCL can apply. Concurrency control implies coordinating the times at which different CCLs can execute actions. Action-impact assessment indicates interactions and processes on the evaluation of the impacts of the different CCLs.

The coordination purpose attributes contain the information and data needed or used by the coordination entity for interacting with the CCL when handling conflicts.

* * * End of Changes * * * *