3GPP TSG-SA5 Meeting #161	S5-252394

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0082 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.535 use case for triggered CCL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution proposes the new use case and requirement for the ACCL instantiation based on conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New UC for ACCL instantiation based on conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to instantiate ACCL on conditions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Proposed Changes

* * * First Change * * * *

### 6.1.x	ACCL instantiation based on conditions

The MnS consumer may want to request for a ACCL to be instantiated not immediately but when certain conditions are met. For example, the MnS consumer may want that for a ACCL of a stated type or that matches a set of stated characteristics (e.g. goal) to be instantiated under certain conditions and another with variations in goals to be instantiated under other conditions. The MnS consumer should be enabled to define those conditions so that the ACCL is instantiated when the stated conditions are met. The MnS Producer monitors the conditions to check if they are met.

The conditions can be related to events based on management data (e.g., performance, fault, configuration).

Performance events are defined related with performance measurements and KPIs that need to be monitored by the producer to see if an ACCL is to be initiated. For example, if the value of a particular performance measurement goes beyond a particular value, a ACCL should be instantiated to keep the value of the same performance measurement below a defined value.

Fault events are defined by related information (e.g alarm type, alarm severity) that need to be monitored by the producer to see if a ACCL is to be initiated. For example, if the total number of alarm with type QUALITY_OF_SERVICE_ALARM and perceivedSeverity MAJOR goes beyond a particular value, a ACCL should be instantiated.

Provisioning events (e.g CreateMOI) are defined that need to be subscribed by the producer to see if a ACCL is to be instantiated. For example, the creation on an Intent MOI can be a trigger to instantiate a ACCL. For another example, when a pre-defined system time specified by operators can be a trigger to instantiated a ACCL.

NOTE:	The use case requires to set the conditions and then the conditions need to be continuously monitored and tracked.

6.1.y		ACCL action execution based on conditions

For the ACCLs that have been instantiated, the MnS consumer may want to define conditions under which a ACCL may execute actions on the network, e.g. when the performance on a certain threshold is crossed, or when the confidence on the decision is above a stated threshold. The consumer does not need to be aware of all decisions, but by providing conditions under which decisions may be activated or not, it is able to have supervision over the ACCL without having to continuously track the decisions. The MnS consumer should be enabled to define those conditions for executing the ACCL actions. Otherwise, the consumer should be enabled to define alternative actions, e.g. to notify the consumer of the decision that is not executed.

By supporting this, the execution can be affected by producer based on consumer's conditions or requirements. To ensure oversight or accountability by the MnS consumer, the MnS consumer may be notified by the ACCL about the executed action for any conditionally execution. The MnS consumer can intervene as needed.

NOTE:	The use case requires to set the conditions and then the conditions need to be continuously monitored and tracked.

* * * Next Change * * * *

## 6.2	Requirements

REQ-CSA-CON-01 The 3GPP management system shall have the capability to take actions for a set of communication services serving certain group of UEs based on the target SLS.

REQ-CSA-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSA-CON-03 The 3GPP management system shall have the capability to analyse the performance information related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-05	The 3GPP management system shall have the capability to collect NSI related data from one or more 5GC NF(s).

NOTE 1:	An example for NSI related data may be QoE data.

REQ-CSA-CON-06	The 3GPP management system shall have the capability to derive which communication service is associated to the QoE data from the collected NSI related QoE data.

NOTE 1a: A communication service in the 3GPP management system is identified by an S-NSSAI (the Slice/ServiceType, SST in the S-NSSAI identifies a communication service which can be detailed using the SliceDifferentiattor, SD), see TS 23.003 [10].

REQ-CSA-CON-07	The 3GPP management system shall have the capability to ascertain SLS breach.

REQ-CSA-CON-08	The 3GPP management system shall have the capability to perform the root cause analysis (e.g., identifying the underlying reason) for an SLS breach.

REQ-CSA-CON-09	The 3GPP management system shall have the capability to take corrective actions to ensure the target goal.

REQ-CSA-CON-10 The 3GPP management system shall have the capability to translate network slice requirements to cross domain network slice subnet SLS goal and single domain network slice subnet SLS goal.

REQ-CSA-CON-11 The 3GPP management system shall have the capability to collect single domain SLS analysis as input to cross domain SLS analysis.

REQ-CSA-CON-12	The 3GPP management system shall have the capability to allow its authorized consumer to control the SLS assurance (e.g. specify the SLS to be assured, enable/disable, specify the assurance time and update the SLS assurance requirements).

REQ-CSA-CON-13	The 3GPP management system shall have the capability to allow its authorized consumer to obtain the SLS assurance fulfilment status information.

NOTE 2:	The management system refers to the producer of management service for SLS assurance.

REQ-CSA-CON-14	The 3GPP management system shall have the capability to do network prediction (e.g. network resource usage and network performance) by analysing the network operation information in special scenarios.

REQ-CSA-CON-15	The 3GPP management system shall have the capability to take actions such as network configuration and perform network resource reallocation according to the network prediction results.

REQ-CSA-CON-16	The 3GPP management system shall have the capability to allow its authorized consumer to limit the set of action capabilities executable by an assurance closed loop.

NOTE: This use case is not supported.

REQ-CSA-CON-17	The 3GPP management system shall allow an authorized consumer to set a condition to enable/disable an ACCL.

NOTE: This use case is not supported.

REQ-LCM-CON-01 The 3GPP management system shall have the capability of lifecycle management of a closed control loop.

REQ-CSA-CON-18 The 3GPP management system shall have the capability to configure SLS assurance goals for the 5GC management domain and the NG-RAN management domain.

REQ-CSA-CON-19 The 3GPP management system shall have the capability to allow closed control loops in cross management domain to collect SLS assurance goal status of closed control loops in 5GC management domain and NG-RAN management domain.

REQ-CSA-CON-20	The 3GPP management system shall have the capability to allow an authorized consumer to enable or disable pause point(s) connected to actions on managed entities during the ACCL’s execution phase and prior to the actions’ execution.”

NOTE: This requirement is not supported.

REQ-CSA-CON-21 The 3GPP Management System shall have the ability to provide SLS assurance within a particular location.

REQ-FUN-TRI-22 The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger ACCL instantiation.

REQ-FUN-TRI-23 The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger ACCL update.

REQ-FUN-TRI-24 The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger ACCL deletion.

REQ-FUN-TRI-25 The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data under which a ACCL may execute its actions.

* * * End of Changes * * * *