3GPP TSG-SA5 Meeting #161	S5-252453

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0082 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Add information model definitions for energy information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Both 3GPP SA1 and SA2 have introduced corresponding requirements for energy information, and SA5 should also provide relevant solution from a management perspective. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce information model definitions for energy information into TS 28.310. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator would not be able to configure the energy information, and hence would not be able to satisfy these requirements from both SA1 and SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (new), Y (New), Annex Z, (Normative stage 3 in forge) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1718 at commit 2bc3ed2f851f6f915002379c4e977b12c0001802 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[3]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface".

[4]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[5]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[6]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[7]	Void.

[8]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[9]	3GPP TS 32.435: "Telecommunication management; Performance measurement; eXtensible Markup Language (XML) file format definition".

[10]	3GPP TS 32.436: "Telecommunication management; Performance measurement: Abstract Syntax Notation 1 (ASN.1) file format definition".

[11]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[12]	3GPP TS 38.401: "NG-RAN; Architecture description".

[13]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[14]	Void.

[15]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[16]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[17]	3GPP TS 32.551: "Energy Saving Management (ESM); Concepts and requirements".

[18]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[19]	ETSI GR NFV-IFA 015 V3.4.1 (2020-06): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Report on NFV Information Model".

[20]	ETSI GR NFV 003 V1.6.1 (2021-03): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[21]	3GPP TS 28.530: " Management and orchestration; Concepts, use cases and requirements".

[22]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[23]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[24]	Void.

[25]	ETSI GS NFV-IFA 027 (V4.3.1) (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[26]	ETSI ES 202 336-12 (V1.2.1) (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[27]	ETSI GS NFV-IFA 008 V3.5.1 (2021-11): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[28]	ETSI GS NFV-TST 008 V3.3.1 (2020-12): "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification".

[29]	ETSI GS NFV-IFA 006 (V3.7.1) (2022-11): " Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Vi-Vnfm reference point – Interface and Information Model Specification".

[30]	3GPP TS 28.111: "Fault management".

[31]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[32]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[33]	ISO/IEC 30134-3:2016: "Information technology -- Data centres -- Key performance indicators -- Part 3: Renewable energy factor (REF)".

[Y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[Z]	ITU-T L.1333: "Carbon data intensity for network energy performance monitoring".

Next change

# X	Information model definitions for Energy Information NRM

## X.1	Imported and associated information

### X.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [Y], IOC, SubNetwork | SubNetwork |
| TS 28.622 [Y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [Y], IOC, Top | Top |
| TS 28.622 [Y], ProxyClass, ManagedEntity | ManagedEntity |

### X.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [Y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [Y], IOC, SubNetwork | SubNetwork |

## X.2	Class diagram

### X.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for energy consumption and efficiency. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure X.2.1-1: Energy information NRM fragment

### X.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure X.2.2-1: Energy supply information NRM fragment

Figure X.2.2-2: Energy information group NRM fragment

## X.3	Class definitions

### X.3.1	EnergySupplyInfo

#### X.3.1.1	Definition

This IOC defines the information related to energy supply and the sources used to produce the energy. This information is configured by the operator with the information obtained from sources external to 3GPP e.g an energy supplier.

#### X.3.1.2	Attributes

The EnergySupplyInfo IOC includes the attributes inherited from Top IOC (defined in in TS 28.622 [B]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySupplyMode | M | T | T | F | T |
| energySourceList | M | T | T | F | T |

#### X.3.1.3	Attribute constraints

None.

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	EnergySourceInfo <<dataType>>

#### X.3.2.1	Definition

This data type defines the information related to a type of energy source. This information is configured by the operator with the information obtained from sources external to 3GPP.

#### X.3.2.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySourceType | M | T | T | F | T |
| energySourceCef | M | T | T | F | T |
| renewableEnergy | M | T | T | F | T |
| energyCompositionPercentage | M | T | T | F | T |

#### X.3.2.3	Attribute constraints

None.

#### X.3.2.4	Notifications

The clause X.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### X.3.3	EnergyInfoGroup

#### X.3.3.1	Definition

This IOC represents information related to the energy supply modes that are used to power a group of Network Elements and/or SubNetworks . This information is configured by the operator.

#### X.3.3.2	Attributes

The EnergyInfoGroup IOC includes the attributes inherited from Top IOC (defined in TS 28.622 [Y]) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |  |
| energySupplyModeRefList | M | T | T | F | T |  |
| memberDNList | M | T | T | F | T |  |

#### X.3.3.3	Attribute constraints

None

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

## X.4	Attribute definitions

### X.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| energyCompositionPercentage | It indicates the percentage of energy source type used by the energy supply mode. This information is configured by the operator with the information obtained from external sources. allowedValues: 0, …,100 | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| energySourceCef | The Carbon Emission Factor of a type of energy source. Its unit is kg CO2eq / kWh. This information is configured by the operator with the information obtained from external sources. allowedValues: non-negative | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySourceList | It represents the list of sources used to produce the energy by the supplier. This includes energy related information i.e., carbon emission and renewable energy related information, and the percentage of the energy produced using the source. For example, an energy supply from a grid uses solar as 50% and hydropower as 50% as energy sources, the attribute energySourceList can be configured with two elements in the list. The first element of the energySourceList being configured with attribute energySourceType with value of "solar", attribute energySourceCef with value of 0, attribute renewableEnergy with value true and attribute energyCompositionPercentage with value of 50. The second element of the energySourceList being configured with attribute energySourceType with value of "coal", attribute energySourceCef with value of 0, attribute renewableEnergy with value false and attribute energyCompositionPercentage with value of 50. This information is configured by the operator with the information obtained from external sources. allowedValues: N/A | type: EnergySourceInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: N/A isNullable: False |
| energySourceType | The type of energy used by the energy supply mode. For example, the values can be "solar", "wind", “coal”, “nuclear”, "biogas", "biofuel", "aerothermal", "geothermal", "hydropower", "biomass", "landfillgas", etc. This information is configured by the operator with the information obtained from external sources. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyMode | The mode by which energy is supplied to a Network Element. This information is configured by the operator with the information obtained from external sources. The mode can be one of the following: GRID_ELECTRICITY: Energy from local grid electricity (See Grid electricity in clause 6 of ITU-T Recommendation L.1333 [D]) BACKUP_ENERGY: Energy from backup diesel or other types (from battery banks/UPS), or from generators such as solar panels. (See Backup energy in clause 6 of ITU-T Recommendation L.1333[D]) LOCALLY_GENERATED_ENERGY: Energy provided by local generators not used for backup functionality, e.g., solar panels, wind generator etc. (See Locally generated energy in clause 6 of ITU-T Recommendation L.1333[D]) allowedValues: - “GRID_ELECTRICITY” - “BACKUP_ENERGY” - “LOCALLY_GENERATED_ENERGY” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyModeRefList | The list of energy supply modes for the group. This information is configured by the operator with the information obtained from external sources. allowedValues: DN of EnergySupplyInfo MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| memberDNList | This attribute contains the DNs of managed entities that are members of the energy info group. This attribute provides the list of Network Elements in the group, represented using ManagedElement MOIs or represented using SubNetwork MOIs. In the case, memberDNList is configured by DN of SubNetwork, then all the ManagedElement MOIs that are name-contained by the SubNetwork MOI are part of the group. allowedValues: DN of ManagedElement or SubNetwork MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| renewableEnergy | This attribute represents if the energy source is renewable or non-renewable. This information is configured by the operator with the information obtained from external sources. true: the energy source is renewable. false: the energy source is non-renewable. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

## X.5	Common notifications

### X.5.1	Alarm notifications

None.

### X.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [16], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyMOIChanges | O | -- |
| notifyEvent | O | -- |

### X.5.3	Threshold Crossing notifications

None.

Next change

# Y	Solution Set (SS)

## Y.1	YANG Definitions for for Energy related Information NRM

YANG definitions are specified in 3GPP Forge, refer to clause 4.4 of TS 28.623 [106] for the Forge location.

Directory: yang-models

Files: _3gpp-engy-nrm-energyinformation

Next change

# Annex Z Energy relationship diagram

## Z.1	The Figure X.2.1-1: Energy information NRM fragment

@startuml TS28.310 Figure Energy information NRM

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

top to bottom direction

skinparam nodesep 40

class ManagedEntity <<ProxyClass>>

note left

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

class SubNetwork <<InformationObjectClass>>

class EnergyInfoGroup <<InformationObjectClass>>

class EnergySupplyInfo <<InformationObjectClass>>

EnergyInfoGroup "*" -u-> "1..*" EnergySupplyInfo

ManagedEntity "1..*" <-U--o "0..1" EnergyInfoGroup

EnergyInfoGroup "*" -u-* "1" SubNetwork: <<names>>

EnergySupplyInfo "*" -u-* "1" SubNetwork: <<names>>

@enduml

Next change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1718 at commit 2bc3ed2f851f6f915002379c4e977b12c0001802

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-engy-nrm-energyinformation.yang ***

<CODE BEGINS>

module _3gpp-engy-nrm-energyinformation {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-engy-nrm-energyinformation;

prefix engyinfo3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the Energy Information NRM.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI,

TSDSI, TTA, TTC). All rights reserved.";

reference "3GPP TS 28.310";

revision 2025-05-07 { reference CR-0082 ; }

feature energyinformationUnderSubNetwork {

description "The energyinformation shall be contained under SubNetwork";

}

feature energyinformationUnderManagedElement {

description "The energyrelatedinformation shall be contained under

ManagedElement";

}

grouping EnergySourceInfoGrp {

description "This data type defines the information related to a type

of energy source. This information is configured by the operator with

the information obtained from sources external to 3GPP. The attribute

energySourceType is used to indicate the type of energy source used by

an energy supply mode to produce the energy. The attribute energySourceCef

is used to indicate the carbon emission factor of the energy source.

The attribute renewableEnergy is used to indicate if the energy source

is renewable or non-renewable. The attribute energyCompositionPercentage

is used to indicate the percentage of energy source used to produce the

energy.";

leaf energySourceType {

type string;

mandatory true;

description "The attribute is used to indicate the type of energy source

used by an energy supply mode to produce the energy.";

}

leaf energySourceCef {

type decimal64 {

fraction-digits 2;

}

mandatory true;

units "kg CO2eq/kWh";

description "The attribute is used to indicate the Carbon Emission Factor

of a type of energy source. Its unit is kg CO2eq / kWh.";

}

leaf renewableEnergy {

description "This attribute represents if the energy source is renewable

or non-renewable. This information is configured by the operator with the

information obtained from external sources.";

type boolean;

mandatory true;

}

leaf energyCompositionPercentage {

type uint8 {

range 0..100;

}

default 100;

description "The attribute is used to indicate the percentage of energy

source type used by the energy supply mode.";

}

}

grouping EnergyInfoGroupGrp {

leaf energySupplyModeRefList {

description "The list of energy supply modes for the Network Elements

of the group. This information is configured by the operator with the

information obtained from external sources.";

type types3gpp:DistinguishedName;

}

leaf memberDNList {

description "This attribute contains the DNs of managed entities

that are members of the energy info group.";

type types3gpp:DistinguishedName;

}

}

grouping EnergySupplyInfoGrp {

leaf energySupplyMode {

description "The mode by which energy is supplied to a Network Element.

This information is configured by the operator with the information

obtained from external sources.The mode can be one of the following:

GRID_ELECTRICITY: Energy from local grid electricity

(See Grid electricity in clause 6 of ITU-T Recommendation L.1333 [D])

BACKUP_ENERGY: Energy from backup diesel or other types (from battery

banks/UPS), or from generators such as solar panels. (See Backup energy

in clause 6 of ITU-T Recommendation L.1333[D]) LOCALLY_GENERATED_ENERGY:

Energy provided by local generators not used for backup functionality,

e.g., solar panels, wind generator etc. (See Locally generated energy

in clause 6 of ITU-T Recommendation L.1333[D])";

mandatory true;

type enumeration {

enum GRID_ELECTRICITY;

enum BACKUP_ENERGY;

enum LOCALLY_GENERATED_ENERGY;

}

}

list energySourceList {

description "This attribute represents the list of sources used to

produce the energy by the supplier. This includes energy related

information i.e., carbon emission and renewable energy related

information, and the percentage of the energy produced using the

source.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses EnergySourceInfoGrp;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature energyinformationUnderSubNetwork;

list EnergyInfoGroup {

description "This IOC represents a group of Network Elements, represented

by ManagedElement IOC and information related to the energy supply modes

that are used to power them. This information is configured by the

operator. Attribute memberDNList can be configured with the DNs of

ManagedElement or DNs of SubNetwork. This attribute provides the list

of Network Elements in the group, represented using ManagedElement MOIs

or represented using SubNetwork MOIs. In the case, memberDNList is

configured by DN of SubNetwork, then all the ManagedElement MOIs that

are name-contained by the SubNetwork MOI are part of the group. Attribute

energySupplyModeRefList is used to indicate the mode of energy supply

for powering the Network Elements of the group.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EnergyInfoGroupGrp;

}

}

list EnergySupplyInfo {

description "This IOC defines the information related to energy supply

and the sources used to produce the energy. This information is configured

by the operator with the information obtained from sources external to

3GPP e.g an energy supplier. ";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EnergyInfoGroupGrp;

}

}

}

augment "/me3gpp:ManagedElement" {

if-feature energyinformationUnderManagedElement;

list EnergyInfoGroup {

description "This IOC represents a group of Network Elements, represented

by ManagedElement IOC and information related to the energy supply modes

that are used to power them. This information is configured by the

operator. Attribute memberDNList can be configured with the DNs of

ManagedElement or DNs of SubNetwork. This attribute provides the list

of Network Elements in the group, represented using ManagedElement MOIs

or represented using SubNetwork MOIs. In the case, memberDNList is

configured by DN of SubNetwork, then all the ManagedElement MOIs that

are name-contained by the SubNetwork MOI are part of the group. Attribute

energySupplyModeRefList is used to indicate the mode of energy supply

for powering the Network Elements of the group.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EnergyInfoGroupGrp;

}

}

list EnergySupplyInfo {

description "This IOC defines the information related to energy supply

and the sources used to produce the energy. This information is configured

by the operator with the information obtained from sources external to

3GPP e.g an energy supplier. ";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EnergyInfoGroupGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

End of change