3GPP TSG-SA5 Meeting #161	S5-252495

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0070 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 Addition of the CAPIF Charging Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support API charging for the industry, the Common API Framework and service operations are specifed by SA6 and CT3 to support any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves. The corresponding charging support should be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clause to specify the message flows for CAPIF charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAPIF service is not supported from the charging aspect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after the new clause 5.X in CR 0069. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

5.X.2	Message flows

5.X.2.1	Introduction

The interaction between CCF and CHF for the API invoker/API provider domain service operation is the same with the interaction between NEF and CHF for the API invoker service operation. The message flows about CAPIF Core Function converged charging for IEC/PEC/ECUR charging scenarios is the same with message flows about NEF converged charging in clause 5.4.2.

Based on the message flows described in clase 5.4.2, the detailed PEC charging scenarios below focus on the different messages between API provider domain/API invoker and the CCF and corresponding interaction with the CHF for example, based on scenarios specified in TS 23.222 [244].

Figure 5.x.2.1-1: API Provider Invocation Request to CCF using PEC

1-2.	CCF receives a CAPIF API Request (e.g. CAPIF_Events_API, CAPIF_API_Provider_Management_API specified in the TS 29.222 [245]) from an API provider via the API publishing function, API exposing function or API management.

3.	CCF performs the actions needed to fulfil the CAPIF API invoked.

4.	If authorized, the CCF continues the CAPIF API invocation processing and sends the CAPIF API Response to API provider domain.

4ch-a. The CCF sends Charging Data Request [Event] to CHF for the received CAPIF API Invocation.

4ch-b. The CHF creates a CDR for this CAPIF API Invocation.

4ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CCF.

5. The API publishing function, API exposing function or API management forward the CAPIF API response to Serive provider.

Note: in the different deployment modes described in the TS 23.222 [244] clause Annex B (informative) CAPIF relationship with network exposure aspects of 3GPP systems, NEF may be implemented as the CAPIF core functioun and API provder domain, or API provider domain.

Figure 5.x.2.1-2: API Invocation Request to CCF using PEC

1.	CCF receives a CAPIF API Request (e.g. CAPIF_Events_API, CAPIF_API_Invoker_Management_API specified in the TS 29.222 [245]) from an API invoker.

2.	CCF performs the actions needed to fulfil the CAPIF API invoked.

3.	If authorized, the CCF continues the CAPIF API invocation processing and sends the CAPIF API Response to API invoker.

3ch-a. The CCF sends Charging Data Request [Event] to CHF for the received CAPIF API Invocation.

3ch-b. The CHF creates a CDR for this CAPIF API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CCF.

5.X.3	CDR generation

5.X.3.1	Introduction

The northbound API CHF CDRs for NEF Charging generated by the CHF are applicable for the CCF Charging, to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

5.X.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [6].

5.X.5	Bea CDR file transfer

Details of the Bea protocol application are specified in TS 32.297 [5].

End of Change