3GPP TSG-SA5 Meeting #161	S5-252515

Fukuoka, Japan, 19 - 23 May 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TS 28.561 NDT Use case Update

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

- According to clause 4.2, NDT MnS Service is defined, so the description of the use cases should be updated under the context of SBMA.

- No requirements for Control of NDT instances – NDTCTR.

- Synchronization with network is not mandatory, since NDT MnS Producer may create an NDT instance according to the physical network provide by the NDT MnS Consumer. This should be captured in the description.

Proposed Changes

Start of First Changes

5.1.2.1	Life cycle management of NDT instances - DTLCM

NDTs can be used for many application use cases. In each case, the NDT needs to implement a defined network scenario, run a simulation/emulation of that scenario, and subsequently provide an output representing the statuses of different network for the simulated network scenario. The lifecycle management of NDT refers to the management processes that control and transition an NDT instance through the processes of creation, activation, operation, deactivation, deletion and any updates during these processes. At each step, it is critical to ensure that the NDT instance accurately reflects the status of physical entities and support efficient simulation/emulation.

-	For creation, the NDT MnS Consumer requests the creation of an NDT instance based on the capabilities of the NDT MnS producer. The NDT MnS consumer may request the NDT MnS producer to create a new NDT instance, which may include creating from scratch, updating existing NDT models or to combining existing NDT models to generate a new NDT.

-	At any point during the life of an NDT instance, the MnS consumer may provide physical network data to refine and adjust the NDT instance to accurately reflect the status of the physical entity and adapt it to specific application scenarios. For example, the consumer may trigger data collection from the network and synchronization of the state with the NDT.

5.1.2.2	Control of NDT instances - NDTCTR

The 3GPP management system should support capabilities enabling consumers to define the network scenario, to control (start, suspend, resume or stop) the simulation/emulation of that network scenario and to provide outcomes representing measurements and counters from metrics in the simulated/emulated network scenario.

An NDT may have dependencies on other NDTs or DTs, each of which implements a specific network scope. The MnS consumer should also be enabled to provide orchestration information to NDT MnS Producer to create an NestedNDT, i.e., configure or modify the dependencies (e.g., to select which environment or radio propagation DT to be applied).

5.1.2.3	Synchronization with network - DTSYNC

The NDT needs to obtain information from physical network including network functions and entities. The collected information includes the following:

-	Performance data: Performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3].

-	MDT/Trace data: MDT measurements as defined in 3GPP TS 32.422 [4].

-	Fault management data: Fault management data as defined in 3GPP TS 28.111[5]

-	Configuration data: Configuration data defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [7], any of the available MOIs;

Mobile networks are generally composed of numerous network elements. The data about these numerous network elements can potentially be vast. When collecting this vast amount of data to NDT, the collection might cause congestion in the network bandwidth used for data collection. NDT MnS Producer may create an NDT instance according to the physical network provide by the NDT MnS Consumer or collect the data from the physical network for synchronization. The MnS producer should enable the MnS consumer to control the frequency of data to be collected by the NDT to minimize unnecessary collection of data and minimal congestion.

5.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-DTLCM-DTLCM-01: | The 3GPP management system should support a capability enabling an MnS consumer to define the network scenario that should be modelled and simulated/emulated. | DTLCM-DTLCM |
| REQ-DTLCM-DTLCM-02: | The 3GPP management system should support a capability enabling an MnS consumer to control the simulation/emulation of a network scenario, e.g. to start, or stop the simulation/emulation. | DTLCM-DTLCM |
| REQ-DTLCM-DTLCM-03: | The 3GPP management system should support a capability to provide to the MnS consumer an output of a modelling or simulation process, the output representing the status of the network for the simulated scenario. | DTLCM-DTLCM |
| REQ-DTLCM-NDTCTR-01: | The 3GPP management system should support a capability to allow the MnS consumer to provide orchestration information to NDT MnS Producer to create an NestedNDT. | DTLCM-NDTCTR |
| REQ-DTLCM- DTSYNC-01: | The 3GPP management system should have the capability enabling the MnS consumer to control the frequency of network data collection. | DTLCM-DTSYNC |

End of First Changes

5.2.2.2	Support for evaluation of high-risk network operations

Each operation for network optimization and maintenance on mobile network may cause potential network failures, including network congestion and network breakdown. High-risk operations are the operations with increased risk of failure, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching. Operators should not carry out the high-risk network operations or perform direct optimization in the physical network. The NDT can be used for evaluation of high-risk network operations.

Using NDT, high-risk operations causing potential network failures can be evaluated. NDT MnS Producer can also be requested to create an instance for optimization and verification of network policies and solutions for the further risk avoidance. After simulating/emulating a scenario, NDT MnS Producer can generate the results and report them back to the MnS consumer. Based on results, the MnS consumer can generate possible network policies and solutions to minimize the impact of high-risk operations.

As an example, the NDT can predict SLA degradation and single node failures in mobile networks. When the prediction indicates insufficient network resources to maintain SLA compliance or potential hardware resource failures, the NDT results help operators to take appropriate preventive actions to avoid network outages.