3GPP TSG-SA5 Meeting #161	S5-252517

Fukuoka, Japan, 19 - 23 May 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TS 28.561 Simplify the procedure for consuming NDT management service

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

The current procedure is premature and related description is confusing.

- Firstly, Terms used in different steps are not aligned. For example, the consumer is “MnS Consumer” but the producer is “NDT MnS Producer”. Besides, the term “NDTJob”, “NDTJob instance”, “NDT Job” and the entities they are referring to are unclear.

- Secondly, there are two steps in the procedure diagram, it seems that the step 1 is to create an NDTJob instance and the step 4 is to trigger simulation/emulation. However, the step 4 may be not needed, since the consumer can trigger the simulation/emulation by configuring the relevant attribute (to be defined) in step 1. Step 4 needs to be justified.

Therefore, this contribution proposes to simplify the procedure and add a note to emphasize that it should be aligned with the agreed NRM.

Proposed Changes

Start of First Changes

6.4 Procedure for consuming NDT management service

Figure 6.4-1: Procedure of consuming NDT management service

1. NDT MnS Producer (the entity who provides the NDT for network simulation/emulation) receives a request from MnS consumer to create an NDTJob instance (see createMOI operation defined in TS 28.532 [8]) which represents theconsumer’s requirements for the NDT job. The NDT job can be a signalling storm replay, network configuration verification, etc.

2. NDT MnS Producer creates the NDTJob instance (i.e., the instance of NDTJob IOC) and configures the new created NDTJob MOI according to the received request from MnS consumer.

3. MnS Producer sends a response (see createMOI operation defined in TS 28.532[8]) to the MnS Consumer.

4. MnS Producer receives a request from MnS consumer to start the execution of the NDT job.

5. MnS Producer executes the NDT job, creates an NDTReport instance (i.e., the instance of NDTReport IOC) and configures NDTJob MOI according to the simulation/emulation results.

6. MnS Producer sends the NDT report to MnS consumer.

Note: the Procedure should be further updated to align with the agreed NRM.

Note: It’s FFS that if step 4 is needed.

End of First Changes