3GPP TSG-SA5 Meeting #161	S5-252554

Fukuoka, Japan, 19 - 23 May 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-19 TS 28.567 Add Stage-2 Solutions for Fault Management CCL

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	V0.2.0

Work Item:	Closed Control Loop Management

Comments

Introduction of the Stage-2 solutions for Fault Management CCL, as described in TR 28.867

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[3]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

[A]	3GPP TS 28.111: "Technical Specification Group Services and System Aspects; Management and orchestration; Fault Management (FM)".

[B]	3GPP TS 28.622: "Technical Specification Group Services and System Aspects; Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

* * * Second Change * * * *

### 6.3.3	CCLReport

#### 6.3.3.1	Definition

This class represents the reported outcomes on a CCL instance, e.g., the information about the outcomes on one or the executing of the CCL. An CCLReport is contained by the entity containing the CCL, since the CCLReport can exist beyond the life of the CCL on which it is reporting.

There is one CCLReport per CCL for an observation time. The content of the CCLReport may be different for different observation time.

#### 6.3.3.2	Attributes

The CCLReport IOC includes attributes inherited from Top IOC (defined TS 28.622[B]) and the following attributes:

Table 6.3.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| FaultManagementCCLReport | CM | T | F | F | T |
| Attributes related to role |  |  |  |  |  |

#### 6.3.3.3	Attribute constraints

Table 6.3.3.3-1

| Name | Definition |
| --- | --- |
| FaultManagementCCLReport | Condition: fault management is supported by CCL |

#### 6.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

* * * Third Change * * * *

### 6.3.4	CCLPurpose <<dataType>>

#### 6.3.4.1	Definition

This data type represents a single purpose that describes what a CCL can do. The purpose is a list of characteristics that describe the capabilities of the CCL.

#### 6.3.4.2	Attributes

Table 6.3.4.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| FaultManagementCCLPurpose | O | T | T | F | F |
| Attributes related to role |  |  |  |  |  |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

* * * Fourth Change * * * *

### 6.3.X	FaultManagementCCLPurpose <<dataType>>

#### 6.3.X.1	Definition

This data type represents the Fault Management CCL purpose, which is a list of attributes that describe the capabilities of the Fault Management CCL.

#### 6.3.X.2	Attributes

Table 6.3.X.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| FaultManagementAlarmIdList | M | T | T | F | F |
| FaultManagementTimeWindow | M | T | T | F | F |
| FaultManagementBackUpObjectRequirement | O | T | T | F | F |
| FaultManagementIsolateObjectRequirement | O | T | T | F | F |
| clearUserId | CM | T | T | F | F |

#### 6.3.X.3	Attribute constraints

Table 6.3.X.3-1

| Name |  | Definition |
| --- | --- | --- |
| clearUserId | These attributes shall be supported for Fault Management CCL that clears ADMC alarms, as specified in TS 28.111 [A]. |  |

#### 6.3.X.4	Notifications

None.

* * * Fifth Change * * * *

### 6.3.Y	FaultManagementCCLReport <<dataType>>

#### 6.3.Y.1	Definition

This data type represents the Fault Management CCL report, which is a list of attributes that describe the result of the Fault Management.

#### 6.3.Y.2	Attributes

Table 6.3.Y.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| GeneratedAlarmResultList | M | T | F | T | T |
| FaultManagementCCLReportTime | M | T | F | T | T |

#### 6.3.Y.3	Attribute constraints

None.

#### 6.3.Y.4	Notifications

None.

* * * Sixth Change * * * *

### 6.3.Y	GeneratedAlarmResult <<dataType>>

#### 6.3.Y.1	Definition

This data type represents the alarm result information generated by the CCL, which is a list of attributes that describe the result of the Fault Management for each alarm.

#### 6.3.Y.2	Attributes

Table 6.3.Y.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| alarmId | M | T | F | T | F |
| alarmClearedStatus | M | T | F | T | F |
| identifiedRootCauseInformation | M | T | F | T | F |
| enhancedCorrelationInformation | M | T | F | T | F |

#### 6.3.Y.3	Attribute constraints

None.

#### 6.3.Y.4	Notifications

None.

* * * Seventh Change * * * *

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| FaultManagementCCLPurpose | It describes the Fault Management CCL purpose. allowedValues: Not Applicable | type: FaultManagementCCLPurpose multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| FaultManagementAlarmIdList | It describes the list of IDs of alarms to be managed by Fault Management CCL. allowedValues: A list of alarmIds as specified in TS 28.111 [A], clause 7.4.1 | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| FaultManagementTimeWindow | It describes the information of a time window (including start and end time) for fault management allowedValues: timeWindow as defined in 3GPP TS 28.622 [B], clause 4.4.1 | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| FaultManagementBackUpObjectRequirement | It describes whether to back-up the alarmed object is required by the consumer before fault management. allowedValues: True, False | type: Booelan multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultManagementIsolateObjectRequirement | It describes whether to isolate the alarmed object is required by the consumer before fault management. allowedValues: True, False | type: Booelan multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearUserId | It carries the identity of the Fault Management CCL who is the consumer that invokes the clearAlarms operation. allowedValues: clearUserId as defined in 3GPP TS 28.111 [A], clause 7.4.1 | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultManagementCCLReport | It describes the Fault Management CCL report. allowedValues: Not Applicable | type: FaultManagementCCLReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GeneratedAlarmResultList | It describes the list of generated alarm results allowedValues: A list of GeneratedAlarmResult | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GeneratedAlarmResult | It describes the result for each alarmId listed in FaultManagemetAlarmIdList allowedValues: Not Applicable | type: GeneratedAlarmResult multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultManagementCCLReportTime | It describes the time when the FaultManagementCCLReport is created. allowedValues: DateTime as specified in TS 28.622 [B]. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmId | It identifies an AlarmRecord as specified in TS 28.111 [A] allowedValues: A string as specified in TS 28.111 [A] | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmClearedStatus | It describes whether an alarm is cleared by the Fault Management CCL allowedValues: True, False | type: Booelan multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| identifiedRootCauseInformation | It describes root cause information identified by the Fault Management CCL The content and format are vendor specific allowedValues: Not Applicable | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enhancedCorrelationInformation | It describes the list of correlated alarm Ids identified by the Fault Management CCL allowedValues: A list of alarmId | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * End of Changes * * * *