3GPP TSG-SA5 Meeting #161	S5-252561

Fukuoka, Japan, 19 - 23 May 2025

Source:	ZTE Corporation

Title:	DP on the NRM Design for AIMLInferenceFunction

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	AIML_MGT_Ph2

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

3.1	How to trigger AI/ML Inference via AIMLInferenceFunction

The AIMLInferenceFunction IOC is designed and agreed at end of Rel-18 in a rush time, which leaves a lot of issues needed to be solved. In TS 28.105 v19.2.0, the AIMLInferenceFunction IOC includes following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| activationStatus | M | T | T | F | T |
| managedActivationScope | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| usedByFunctionRefList | M | T | F | F | T |
| mLModelRefList | M | T | F | T | T |

Observation 1 - Observation on producer initiated AI/ML Inference.

As specified in the clause 7.3a.4.2.5.1, this AIMLInferenceFunction instance can be created by the system or pre-installed and it can be only created by the MnS producer but not consumer. If the consumer cannot request to create an AIMLInferenceFunction MOI on the Producer, then all attributes in the table above should be configured by the Producer when the instance is created or pre-installed, i.e., the isWritable property of “activationStatus” and “managedActivationScope” should be “F”.

In other words, producer initiated AIML inference is supported in current NRM but the “isWritable” property should be “F” to align with the documentation.

Furthermore, the requirement (see clause 6.5.1.3	Requirements for AI/ML inference performance management) below intends to allow the consumer to indicate the performance policy to the producer, i.e., the consumer can provide the performance policy to the producer and modify it later. However, according to the analysis above, the consumer can only request to modify the policy (i.e., managedActivationScope), i.e., the requirement is not supported.

| REQ-AI/ML_PERF-POL-1 | The AI/ML MnS producer shall have a capability allowing the authorized MnS consumer to indicate a performance policy related to AI/ML inference. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |
| --- | --- | --- |

Observation 2 - Observation on consumer initiated AI/ML Inference.

Consumer can request for AI/ML Inference in two ways: per aIMLInferenceName or per ML Model. However, the “aIMLInferenceName” is missing in the AIMLInferenceFunction IOC and the isWritable property of mLModelRefList is “F”, which is used to indicate the associated ML Model in the AIMLInferenceFunction.

In other words, consumer initiated AI/ML Inference is not supported.

3.2	Relationships of AIMLInferenceFunction

The relationships between AIMLInferenceFunction and AIMLSupportedFunction including MDAF is depicted below. The IOCs representing the functions/functionalities (Note) that use the AI/ML inference function include MDAFunction, AnLFFunction, DMROFunction, DLBOFunction, and DESManagementFunction.

Observation 3 - relationship between AIMLSupportedFunction and AIMLInferenceFunction

The relationship between AIMLSupportedFunction and AIMLInferenceFunction remains ambiguous in the current documentation. It indicates that AIMLSupportedFunction represents the IOCs employing AI/ML inference functions. However, MDAFunction can be instantiated for AI/ML inference tasks using MDA models without AIMLInfrenceFunction.

For MDA, AIMLInferenceFunction should be viewed as a complement to MDAFunction, controlling its activation and deactivation. Once the AI/ML function is activated, the consumer can request MDA output by creating an MDARequest MOI on the MDA producer. The MDARequest MOI is reusable, eliminating the need for a specific AIMLInferenceRequest for MDA.

However, for other AIMLSupportedFunctions, based on the current NRM, AIMLInferenceName is not an attribute of AIMLSupportedFunction. This means the consumer cannot request inference output, nor can the producer trigger an inference task for a specific aIMLInferenceName.

3.3	Identified Issues

1. The documentation needs to be improved by clarifying the relation between MDAFunction and AIMLInferenceFunction.

2. The consumer cannot request inference output for a non-MDA AIMLInferenceName based on current NRM solution.

3. The producer cannot trigger an inference task for a non-MDA AIMLInferenceName based current NRM solution.

4	Recommendation

1. Add the following description to clarify the relation between MDAF and AIMLInferenceFunction:

“For MDA, AIMLInferenceFunction should be viewed as a complement to MDAFunction, controlling its activation and deactivation. Once the AI/ML function is activated, the consumer can request MDA output by creating an MDARequest MOI on the MDA producer (see TS 28.104).”

2. Define the AIMLInferenceRequest IOC, which is name-contained within the AIMLInferenceFunction. The AIMLInferenceRequest MOI represents the inference requirements from the consumer, allowing the consumer to request creation of the AIMLInferenceRequest MOI on the producer. Add the attribute “aIMLInferenceName” and move “activationStatus” and “managedActivationScope” to the AIMLInferenceRequest IOC. Note that “aIMLInferenceName” in the AIMLInferenceRequest MOI does not accept MDA types as valid values to avoid the duplication or conflict with MDARequest.

5	Detailed proposal

The group is asked to endorse the issues reported above.

The group is asked to agree to define the AIMLInferenceRequest IOC to solve the issues identified above.