3GPP TSG-SA5 Meeting #161	S5-252567

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.659 | CR | 0046 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.659 Add attributes to support Store and Forward Satellite operation for eNB stage3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NTN_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As defined in TS 36.331, trackingAreaList has been introduced in PLMN- IdentityInfoList information element for an NTN cell. Add solution for NTN Tracking area management which is identified in RAN specs and studied in SA5 TR 28.874. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add solution set for NTN TACList configuration in eNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Broadcasting multiple Tracking Area Codes in an NTN cell is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.2,A.3.2,B.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### A.2.2.2	IOC EUtranGenericCell

Mapping from NRM IOC EUtranGenericCell attributes and associations to SS equivalent MOC EUtranGenericCell attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| cellLocalId | cellLocalId | unsignedShort |
| cellLocalIdList | cellLocalIdList | genericEUTRANNRMAttributeTypes:: cellLocalIdListType |
| cellSize | cellSize | genericEUTRANNRMAttributeTypes:: cellSizeEnumType |
| plmnIdList | plmnIdList Note: the first plmnId in the SS attribute plmnIdList is the primary PLMN id | genericEUTRANNRMAttributeTypes:: plmnIdListType |
| cellAccessInfoList | cellAccessInfoList | genericEUTRANNRMAttributeTypes:: cellAccessInfoListType |
| tac | tac | long |
| nTNTAClist | nTNTAClist | genericEUTRANNRMAttributeTypes:: nTNTAClistType |
| pci | pci | short |
| pciList | pciList | genericEUTRANNRMAttributeTypes:: pciListType |
| maximumTransmissionPower | maximumTransmissionPower | short |
| referenceSignalPower | referenceSignalPower | short |
| pb | pb | short |
| partOfSectorPower | partOfSectorPower | short |
| relatedTmaList | relatedTmaList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| relatedAntennaList | relatedAntennaList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| relatedSector | relatedSector | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |
| operationalState | operationalState | StateManagementIRPOptConstDefs:: OperationalStateTypeOpt |
| administrativeState | administrativeState | StateManagementIRPOptConstDefs:: AdministrativeStateTypeOpt |
| availabilityStatus | availabilityStatus | StateManagementIRPOptConstDefs:: AvailabilityStatusTypeOpt |
| cellResvInfo | cellResvInfo | genericEUTRANNRMAttributeTypes:: cellResvInfoType |
| nbIoTcellFlag | nbIoTcellFlag | genericEUTRANNRMAttributeTypes:: yesNoType |
| allowedAccessClasses | allowedAccessClasses | genericEUTRANNRMAttributeTypes:: allowedAccessEnumClassesType |
| isChangeForEnergySavingAllowed | isChangeForEnergySavingAllowed | GenericNetworkResourcesIRPSystem:: AttributeTypes::yesNoType |
| Note: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

Next Change

## A.3.2	IDL specification "EUtranNetworkResourcesNRMDefs.idl"

//File:EUtranNetworkResourcesNRMDefs.idl

#ifndef _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

#define _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#include "EPCResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module EUtranNetworkResourcesNRMDefs

{

/*

* Definitions for MO class ENBFunction

*/

interface ENBFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ENBFunction";

// Attribute Names

//

const string intraANRSwitch= "intraANRSwitch";

const string iRATANRSwitch= "iRATANRSwitch";

const string eNBId = "eNBId";

const string x2BlackList= "x2BlackList";

const string x2WhiteList= "x2WhiteList";

const string x2HOBlackList= "x2HOBlackList";

const string x2IpAddressList= "x2IpAddressList";

const string tceIDMappingInfoList= "tceIDMappingInfoList";

const string sharNetTceMappingInfoList= "sharNetTceMappingInfoList";

const string netListeningRSForRIBS= "netListeningRSForRIBS";

const string lWIPSeGWList = "lWIPSeGWList";

const string ephemerisInfoSetRef = " ephemerisInfoSetRef";

};

/*

* Definitions for MO class RNFunction

*/

interface RNFunction: ENBFunction

{

const string CLASS = "RNFunction";

// Attribute Names

//

const string servingCell = "servingCell";

const string candidateDeNBCells = "candidateDeNBCells";

};

/*

* Definitions for MO class DeNBCapability

*/

interface DeNBCapability: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "DeNBCapability";

// Attribute Names

//

const string servedRN= "servedRN";

const string maxNbrRNAllowed= "maxNbrRNAllowed";

};

/*

* Definitions for MO class ExternalRNFunction

*/

interface ExternalRNFunction: ExternalENBFunction

{

const string CLASS = "ExternalRNFunction";

// Attribute Names

//

};

/*

/*

* Definitions for MO class EUtranGenericCell

*/

interface EUtranGenericCell: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranGenericCell";

// Attribute Names

//

const string cellLocalId = "cellLocalId";

const string cellLocalIdList = "cellLocalIdList";

const string cellSize = "cellSize";

const string plmnIdList = "plmnIdList";

const string cellAccessInfoList = "cellAccessInfoList";

const string tac = "tac";

const string nTNTAClist = "nTNTAClist";

const string pci = "pci";

const string pciList = "pciList";

const string operationalState = "operationalState";

const string administrativeState = "administrativeState";

const string availabilityStatus = "availabilityStatus";

const string maximumTransmissionPower = "maximumTransmissionPower";

const string referenceSignalPower = "referenceSignalPower";

const string pb = "pb";

const string partOfSectorPower = "partOfSectorPower";

const string relatedTmaList = "relatedTmaList";

const string relatedAntennaList = "relatedAntennaList";

const string relatedSector = "relatedSector";

const string allowedAccessClasses = "allowedAccessClasses";

const string isChangeForEnergySavingAllowed = "isChangeForEnergySavingAllowed";

const string cellResvInfo = "cellResvInfo";

const string nbIoTcellFlag = "nbIoTcellFlag";

};

/*

* Definitions for MO class ExternalEUtranGenericCell

*/

interface ExternalEUtranGenericCell: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ExternalEUtranGenericCell";

// Attribute Names

//

const string pci= "pci";

const string plmnIdList = "plmnIdList";

const string cellLocalId = "cellLocalId";

const string eNBId = "eNBId";

};

/*

* Definitions for MO class EUtranCellFDD

*/

interface EUtranCellFDD: EUtranGenericCell

{

const string CLASS = "EUtranCellFDD";

// Attribute Names

//

const string earfcnDl = "earfcnDl";

const string earfcnUl = "earfcnUl";

};

/*

* Definitions for MO class ExternalEUtranCellFDD

*/

interface ExternalEUtranCellFDD: ExternalEUtranGenericCell

{

const string CLASS = "ExternalEUtranCellFDD";

// Attribute Names

//

const string earfcnDl = "earfcnDl";

const string earfcnUl = "earfcnUl";

};

/*

* Definitions for MO class EUtranCellTDD

*/

interface EUtranCellTDD: EUtranGenericCell

{

const string CLASS = "EUtranCellTDD";

// Attribute Names

//

const string earfcn = "earfcn";

const string sfAssignment = "sfAssignment";

const string specialSfPatterns = "specialSfPatterns";

};

/*

* Definitions for MO class ExternalEUtranCellTDD

*/

interface ExternalEUtranCellTDD: ExternalEUtranGenericCell

{

const string CLASS = "ExternalEUtranCellTDD";

// Attribute Names

//

const string earfcn = "earfcn";

};

/*

* Definitions for MO class EUtranRelation

*/

interface EUtranRelation: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EUtranRelation";

// Attribute Names

//

const string id = "id";

const string tCI = "tCI";

const string isRemoveAllowed = "isRemoveAllowed";

const string isHOAllowed = "isHOAllowed";

const string adjacentCell = "adjacentCell";

const string isICICInformationSendAllowed = "isICICInformationSendAllowed";

const string isLBAllowed = "isLBAllowed";

const string cellIndividualOffset = "cellIndividualOffset";

const string qOffset = "qOffset";

};

/*

* Definitions for MO class Link_ENB_ENB

*/

interface Link_ENB_ENB: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_ENB_ENB";

// Attribute Names

//

};

/*

* Definitions for MO class Cdma2000Relation

*/

interface Cdma2000Relation:GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "Cdma2000Relation";

// Attribute Names

//

const string id = "id";

const string adjacentSector = "adjacentSector";

};

/*

* Definitions for MO class ExternalENBFunction

*/

interface ExternalENBFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ExternalENBFunction";

// Attribute Names

//

const string eNBId = "eNBId";

};

/*

* Definitions for MO class MCEFunction

*/

interface MCEFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MCEFunction";

// Attribute Names

//

};

/*

* Definitions for MO class Link_MCE_ENB

*/

interface Link_MCE_ENB: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_MCE_ENB";

// Attribute Names

//

};

/*

* Definitions for MO class Link_MCE_MME

*/

interface Link_MCE_MME: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_MCE_MME";

// Attribute Names

//

};

/*

* Definitions for MO class MBSFNArea

*/

interface MBSFNArea: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "MBSFNArea";

// Attribute Names

//

const string id = "id";

const string mbsfnAreaId= "mbsfnAreaId";

const string cellIdList= "cellIdList";

};

/*

* Definitions for MO class EnergySavingProperties

*/

interface EnergySavingProperties: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EnergySavingProperties";

// Attribute Names

//

const string id = "id";

const string energySavingState= "energySavingState";

const string energySavingControl= "energySavingControl";

};

/*

* Definitions for MO class CellOutageCompensationInformation

*/

interface CellOutageCompensationInformation: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "CellOutageCompensationInformation";

// Attribute Names

//

const string id = "id";

const string cellOutageCompensationStatus = "cellOutageCompensationStatus";

const string isCOCAllowed =

"isCOCAllowed";

};

/*

* Definitions for MO class EUtranCellNMCentralizedSON

*/

interface EUtranCellNMCentralizedSON: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EUtranCellNMCentralizedSON";

// Attribute Names

//

const string id = "id";

const string a1ThresholdRsrp = "a1ThresholdRsrp";

const string a1ThresholdRsrq = "a1ThresholdRsrq";

const string a2ThresholdRsrp = "a2ThresholdRsrp";

const string a2ThresholdRsrq = "a2ThresholdRsrq";

const string a3Offset = "a3Offset";

const string a4ThresholdRsrp = "a4ThresholdRsrp";

const string a4ThresholdRsrq = "a4ThresholdRsrq";

const string a5Threshold1Rsrp = "a5Threshold1Rsrp";

const string a5Threshold1Rsrq = "a5Threshold1Rsrq";

const string b1ThresholdUtraRscp = "b1ThresholdUtraRscp";

const string b1ThresholdUtraEcN0 = "b1ThresholdUtraEcN0";

const string b1ThresholdGeran = "b1ThresholdGeran";

const string b1ThresholdCdma2000 = "b1ThresholdCdma2000";

const string b2Threshold1Rsrp = "b2Threshold1Rsrp";

const string b2Threshold1Rsrq = "b2Threshold1Rsrq";

const string b2Threshold2UtraRscp = "b2Threshold2UtraRscp";

const string b2Threshold2UtraEcN0 = "b2Threshold2UtraEcN0";

const string b2Threshold2Geran = "b2Threshold2Geran";

const string b2Threshold2Cdma2000 = "b2Threshold2Cdma2000";

const string commonChannelPowerOffset = "commonChannelPowerOffset";

const string configurationIndex = "configurationIndex";

const string contentionResolutionTimer = "contentionResolutionTimer";

const string hysteresisEutraA1 = "hysteresisEutraA1";

const string hysteresisEutraA2 = "hysteresisEutraA2";

const string hysteresisEutraA3 = "hysteresisEutraA3";

const string hysteresisEutraA4 = "hysteresisEutraA4";

const string hysteresisEutraA5 = "hysteresisEutraA5";

const string hysteresisIratB1 = "hysteresisIratB1";

const string hysteresisIratB2 = "hysteresisIratB2";

const string numberOfRaPreambles = "numberOfRaPreambles";

const string preambleInitialReceivedTargetPower = "preambleInitialReceivedTargetPower";

const string preambleTransMax = "preambleTransMax";

const string pMax = "pMax";

const string powerRampingStep = "powerRampingStep";

const string qHyst = "qHyst";

const string qOffsetUtra = "qOffsetUtra";

const string qOffsetGeran = " qOffsetGeran";

const string qOffsetCdma2000 = "qOffsetCdma2000";

const string qQualMinUtra = "qQualMinUtra";

const string qRxLevMinEUtraSib1 = "qRxLevMinEUtraSib1";

const string qRxLevMinEUtraSib3 = "qRxLevMinEUtraSib3";

const string qRxLevMinGeran = "qRxLevMinGeran";

const string qRxLevMinUtra = "qRxLevMinUtra";

const string responseWindowSize = "responseWindowSize";

const string rootSequenceIndex = "rootSequenceIndex";

const string sIntraSearch = "sIntraSearch";

const string sizeOfRAPreamblesGroupA = "sizeOfRAPreamblesGroupA";

const string timeToTriggerEutraA1 = "timeToTriggerEutraA1";

const string timeToTriggerEutraA2 = "timeToTriggerEutraA2";

const string timeToTriggerEutraA3 = "timeToTriggerEutraA3";

const string timeToTriggerEutraA4 = "timeToTriggerEutraA4";

const string timeToTriggerEutraA5 = "timeToTriggerEutraA5";

const string timeToTriggerIratB1 = "timeToTriggerIratB1";

const string timeToTriggerIratB2 = "timeToTriggerIratB2";

const string tReselectionCdma2000 = "tReselectionCdma2000";

const string tReselectionEUtra = "tReselectionEUtra";

const string tReselectionGeran = "tReselectionGeran";

const string tReselectionUtra = "tReselectionUtra";

const string tStoreUeContext = "tStoreUeContext";

};

/*

* Definitions for MO class EUtranFreqRelation

*/

interface EUtranFreqRelation: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranFreqRelation";

// Attribute Names

//

const string cellIndividualOffset = "cellIndividualOffset";

const string blackListEntry = "blackListEntry";

const string blackListEntryIdleMode = "blackListEntryIdleMode";

const string cellReselectionPriority = "cellReselectionPriority";

const string cellReselectionSubPriority = "cellReselectionSubPriority";

const string pMax = "pMax";

const string qOffsetFreq = "qOffsetFreq";

const string qQualMin = "qQualMin";

const string qRxLevMin = "qRxLevMin";

const string threshXHighP = "threshXHighP";

const string threshXHighQ = "threshXHighQ";

const string threshXLowP = "threshXLowP";

const string threshXLowQ = "threshXLowQ";

const string tReselectionEutra = "tReselectionEutra";

const string tReselectionEutraSfHigh = "tReselectionEutraSfHigh";

const string tReselectionEutraSfMedium = "tReselectionEutraSfMedium";

const string eUtraFrequencyRef = "eUtraFrequencyRef";

};

/*

* Definitions for MO class EUtranFrequency

*/

interface EUtranFrequency: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranFrequency";

// Attribute Names

//

const string earfcnDL = "earfcnDL";

const string multiBandInfoListEutra = "multiBandInfoListEutra";

};

/*

* Definitions for MO class WTFunction

*/

interface WTFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "WTFunction";

// Attribute Names

//

const string id = "id";

const string wLANInfoList = "wLANInfoList";

};

/*

* Definitions for MO class EP_Xw

*/

interface EP_Xw: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EP_Xw";

};

/*

* Definitions for MO class WLANMobilitySet

*/

interface WLANMobilitySet: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "WLANMobilitySet";

// Attribute Names

//

};

/*

* Definitions for MO class MemberWLAN

*/

interface MemberWLAN

{

const string CLASS = "MemberWLAN";

// Attribute Names

//

const string wLANId = "memberWlanList";

const string wLANGeoLocation = "wLANGeoLocation";

const string isLWASupported = "isLWASupported";

const string isLWIPSupported = "isLWIPSupported";

};

module genericEUTRANNRMAttributeTypes

{

typedef sequence<string> cellLocalIdListType;

enum CellSizeEnumType

{

verysmall,

small,

medium,

large

};

enum AllowedAccessClassesValues

{

EmergencyCall,

ForPLMNUse,

SecurityServices,

PublicUtilities,

EmergencyServices,

PLMNStaff

};

typedef sequence < AllowedAccessClassesValues,6> AllowedAccessClasses

struct PlmnIdType

{

// A bounded string of 3 characters representing 3 digits.

string mcc;

// A bounded string of 2 or 3 characters representing 2 or 3 digits.

string mnc;

};

const short PLMNID_LIST_LENGTH = 6;

typedef sequence<PlmnIdType > plmnIdListType;

struct CellAccessInfoType

{

PlmnIdType plmnId;

long tac;

unsigned short cellId;

string managementServiceExposed;

};

typedef sequence <CellAcessInfoType, 5> cellAcessInfoListType;

const short NO_OF_PCIS = 504;

typedef sequence<short,NO_OF_PCIS> pciListType;

const short MAX_TAC = 12;

typedef sequence<long, MAX_TAC> nTNTAClistType;

typedef sequence<string> ipAddressListType;

enum CellResvInfoType

{

reservedCell,

nonReservedCell

};

struct QciDscpMappingType

{

short qci;

short dscp;

};

typedef sequence<QciDscpMappingType> QciDscpMappingListType;

struct EcgiType

{

// A bounded string of 3 characters representing 3 digits.

string mcc;

// A bounded string of 2 or 3 characters representing 2 or 3 digits.

string mnc;

unsignedlong eci

};

typedef sequence <EcgiType> EcgiListType;

enum isEsCoveredByEnumType

{

no,

partial,

yes

};

enum yesNoType

{

no,

yes

};

struct TceIDMappingInfo

{

short tceID;

string tceIPAddr;

};

typedef sequence<TceIDMappingInfo> TceIDMappingInfoListType;

struct SharNetTceMappingInfo

{

PlmnIdType plmnId;

short tceID;

string tceIPAddr;

};

typedef sequence<SharNetTceMappingInfo> SharNetTceMappingInfoListType;

enum CellOutageCompensationState

{

cOCActivating,

cOCActive,

cOCDeactivating,

cOCDeactive

};

enum qOffsetEnumType

{

dB-24,

dB-22,

dB-20,

dB-18,

dB-16,

dB-14,

dB-12,

dB-10,

dB-8,

dB-6,

dB-5,

dB-4,

dB-3,

dB-2,

dB-1,

dB0,

dB1,

dB2,

dB3,

dB4,

dB5,

dB6,

dB8,

dB10,

dB12,

dB14,

dB16,

dB18,

dB20,

dB22,

dB24

};

enum contentionResolutionTimerEnumType

{

sf8,

sf16,

sf24,

sf32,

sf40,

sf48,

sf56,

sf64

};

enum numberOfRaPreamblesEnumType

{

n4,

n8,

n12,

n16,

n20,

n24,

n28,

n32,

n36,

n40,

n44,

n48,

n52,

n56,

n60,

n64

};

enum preambleInitialReceivedTargetPowerEnumType

{

dBm-120,

dBm-118,

dBm-116,

dBm-114,

dBm-112,

dBm-110,

dBm-108,

dBm-106,

dBm-104,

dBm-102,

dBm-100,

dBm-98,

dBm-96,

dBm-94,

dBm-92,

dBm-90

};

enum preambleTransMaxEnumType

{

n3,

n4,

n5,

n6,

n7,

n8,

n10,

n20,

n50,

n100,

n200

};

enum powerRampingStepEnumType

{

dB0,

dB2,

dB4,

dB6

};

enum qHystEnumType

{

dB0,

dB1,

dB2,

dB3,

dB4,

dB5,

dB6,

dB8,

dB10,

dB12,

dB14,

dB16,

dB18,

dB20,

dB22,

dB24

};

enum responseWindowSizeEnumType

{

sf2,

sf3,

sf4,

sf5,

sf6,

sf7,

sf8,

sf10

};

enum sizeOfRAPreambleGroupAEnumType

{

n4,

n8,

n12,

n16,

n20,

n24,

n28,

n32,

n36,

n40,

n44,

n48,

n52,

n56,

n60,

};

enum timeToTriggerEUtraEnumType

{

ms0,

ms40,

ms64,

ms80,

ms100,

ms128,

ms160,

ms256,

ms320,

ms480,

ms512,

ms640,

ms1024,

ms1280,

ms2560,

ms5120

};

typedef sequence<string> DnList;

struct CellOutageCompensationStatus

{

CellOutageCompensationState cellOutageCompensationState;

DnList errorList;

};

struct ReferenceSignalPattern

{

yesNoType prs_supported;

};

enum ReferenceSignalPeriodicity

{

ms1280,

ms2560,

ms5120,

ms10240

};

const short MAX_CONFIGURATION_NO = 4;

struct NetListeningRefSignalType

{

ReferenceSignalPattern referenceSignalPattern;

short num_of_crs_ports;

ReferenceSignalPeriodicity referenceSignalPeriodicity;

unsigned short referenceSignalOffset;

};

typedef sequence<NetListeningRefSignalType, MAX_CONFIGURATION_NO> NetListeningRSForRIBS;

struct WLANInfoType

{

string WLANId;

enum WLANOperationalState

{

Disabled, Enabled

};

string EnbWithLWARelation;

};

typedef sequence<WLANInfoType> WLANInfoListType;

struct GeoLocationType

{

short latitude;

short longitude

};

struct lWIPSeGWInfoType

{

string LWIPSeGWId;

ipAddressListType LWIPSeGWIpAddressList;

};

typedef sequence<lWIPSeGWInfoType> lWIPSeGWListType;

};

#endif // _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

Next Change

B.3.3	XML schema "eutranNrm.xsd"

<?xml version="1.1" encoding="UTF-8"?>

<!--

3GPP TS 28.659 E-UTRAN Network Resource Model IRP

XML schema definition

eutranNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema" xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nr="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm" xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm" xmlns:epc="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm" xmlns:un="http://www.3gpp.org/ftp/specs/archive/28_series/28.653#utranNrm" xmlns:gn="http://www.3gpp.org/ftp/specs/archive/28_series/28.656#geranNrm" xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP" xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.653#utranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.656#geranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<complexType name="IpAddressList">

<sequence>

<element name="ipAddress" type="string" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<simpleType name="EnbId">

<restriction base="unsignedLong">

<maxInclusive value="268435455"/>

</restriction>

</simpleType>

<simpleType name="Eci">

<restriction base="unsignedLong">

<maxInclusive value="268435455"/>

</restriction>

</simpleType>

<simpleType name="CellLocalId">

<restriction base="unsignedShort">

<maxInclusive value="255"/>

</restriction>

</simpleType>

<complexType name="CellLocalIdList">

<sequence>

<element name="cellLocalId" type="en:CellLocalId" minOccurs="0"/>

</sequence>

</complexType>

<simpleType name="cellSize">

<restriction base="string">

<enumeration value="verysmall"/>

<enumeration value="small"/>

<enumeration value="medium"/>

<enumeration value="large"/>

</restriction>

</simpleType>

<simpleType name="allowedAccessClassesElementType">

<restriction base="string">

<enumeration value="EmergencyCall"/>

<enumeration value="ForPLMNUse"/>

<enumeration value="SecurityServices"/>

<enumeration value="PublicUtilities"/>

<enumeration value="EmergencyServices"/>

<enumeration value="PLMNStaff"/>

</restriction>

</simpleType>

<complexType name="allowedAccessClassesType">

<sequence minOccurs="0" maxOccurs="6">

<element name="allowedAccessClassesElement" type="en:allowedAccessClassesElementType"/>

</sequence>

</complexType>

<complexType name="PLMNId">

<sequence>

<element name="mcc" type="MCC"/>

<element name="mnc" type="MNC"/>

</sequence>

</complexType>

<simpleType name="MCC">

<restriction base="string">

<pattern value="[0-9]{3}"/>

</restriction>

</simpleType>

<simpleType name="MNC">

<restriction base="string">

<pattern value="[0-9]{3}|[0-9]{2}"/>

</restriction>

</simpleType>

<complexType name="PLMNIdList">

<sequence>

<element name="pLMNId" type="en:PLMNId" maxOccurs="6"/>

<!-- The first pLMNId of the pLMNIdList is primary PLMN id -->

</sequence>

</complexType>

<complexType name="CellAccessInfo">

<sequence>

<element name="plmnId" type="en:PLMNId"/>

<element name="tac" type="long"/>

<element name="cellId" type="long"/>

</sequence>

</complexType>

<complexType name="CellAccessInfoList">

<sequence>

<element name="cellAccessInfo" type="en:CellAccessInfo" maxOccurs="5"/>

</sequence>

</complexType>

<complexType name="EcgiList">

<sequence>

<element name="plmnId" type="en:PLMNId" minOccurs="0"/>

<element name="eci" type="en:Eci" minOccurs="0"/>

</sequence>

</complexType>

<simpleType name="Pci">

<restriction base="unsignedShort">

<maxInclusive value="503"/>

<!-- Minimum value is 0, maximum value is 3x167+2=503 -->

</restriction>

</simpleType>

<complexType name="NTNTAClist">

<sequence>

<element name="tac" type="long" minOccurs="1" maxOccurs="12"/>

</sequence>

</complexType>

<complexType name="PciList">

<sequence>

<element name="pci" type="en:Pci" maxOccurs="504"/>

</sequence>

</complexType>

<simpleType name="cellResvInfoType">

<restriction base="string">

<enumeration value="reservedCell"/>

<enumeration value="nonReservedCell"/>

</restriction>

</simpleType>

<simpleType name="mbsfnAreaIdType">

<restriction base="unsignedLong">

<maxInclusive value="255"/>

</restriction>

</simpleType>

<complexType name="QciDscpMappingType">

<sequence>

<element name="qci" type="short"/>

<element name="dscp" type="short"/>

</sequence>

</complexType>

<complexType name="QciDscpMappingListType">

<sequence>

<element name="QciDscpMappingPair" type="en:QciDscpMappingType"/>

</sequence>

</complexType>

<simpleType name="isEsCoveredByEnumType">

<restriction base="string">

<enumeration value="no"/>

<enumeration value="partial"/>

<enumeration value="yes"/>

</restriction>

</simpleType>

<simpleType name="yesNoType">

<restriction base="string">

<enumeration value="yes"/>

<enumeration value="no"/>

</restriction>

</simpleType>

<simpleType name="QOffsetEnumType">

<restriction base="string">

<enumeration value="dB-24"/>

<enumeration value="dB-22"/>

<enumeration value="dB-20"/>

<enumeration value="dB-18"/>

<enumeration value="dB-16"/>

<enumeration value="dB-14"/>

<enumeration value="dB-12"/>

<enumeration value="dB-10"/>

<enumeration value="dB-8"/>

<enumeration value="dB-6"/>

<enumeration value="dB-5"/>

<enumeration value="dB-4"/>

<enumeration value="dB-3"/>

<enumeration value="dB-2"/>

<enumeration value="dB-1"/>

<enumeration value="dB0"/>

<enumeration value="dB1"/>

<enumeration value="dB2"/>

<enumeration value="dB3"/>

<enumeration value="dB4"/>

<enumeration value="dB5"/>

<enumeration value="dB6"/>

<enumeration value="dB8"/>

<enumeration value="dB10"/>

<enumeration value="dB12"/>

<enumeration value="dB14"/>

<enumeration value="dB16"/>

<enumeration value="dB18"/>

<enumeration value="dB20"/>

<enumeration value="dB22"/>

<enumeration value="dB24"/>

</restriction>

</simpleType>

<simpleType name="ContentionResolutionTimerEnumType">

<restriction base="string">

<enumeration value="sf8"/>

<enumeration value="sf16"/>

<enumeration value="sf24"/>

<enumeration value="sf32"/>

<enumeration value="sf40"/>

<enumeration value="sf48"/>

<enumeration value="sf56"/>

<enumeration value="sf64"/>

</restriction>

</simpleType>

<simpleType name="NumberOfRaPreamblesEnumType">

<restriction base="string">

<enumeration value="n4"/>

<enumeration value="n8"/>

<enumeration value="n12"/>

<enumeration value="n16"/>

<enumeration value="n20"/>

<enumeration value="n24"/>

<enumeration value="n28"/>

<enumeration value="n32"/>

<enumeration value="n36"/>

<enumeration value="n40"/>

<enumeration value="n44"/>

<enumeration value="n48"/>

<enumeration value="n52"/>

<enumeration value="n56"/>

<enumeration value="n60"/>

<enumeration value="n64"/>

</restriction>

</simpleType>

<simpleType name="PreambleInitialReceivedTargetPowerEnumType">

<restriction base="string">

<enumeration value="dBm-120"/>

<enumeration value="dBm-118"/>

<enumeration value="dBm-116"/>

<enumeration value="dBm-114"/>

<enumeration value="dBm-112"/>

<enumeration value="dBm-110"/>

<enumeration value="dBm-108"/>

<enumeration value="dBm-106"/>

<enumeration value="dBm-104"/>

<enumeration value="dBm-102"/>

<enumeration value="dBm-100"/>

<enumeration value="dBm-98"/>

<enumeration value="dBm-96"/>

<enumeration value="dBm-94"/>

<enumeration value="dBm-92"/>

<enumeration value="dBm-90"/>

</restriction>

</simpleType>

<simpleType name="PreambleTransMaxEnumType">

<restriction base="string">

<enumeration value="n3"/>

<enumeration value="n4"/>

<enumeration value="n5"/>

<enumeration value="n6"/>

<enumeration value="n7"/>

<enumeration value="n8"/>

<enumeration value="n10"/>

<enumeration value="n20"/>

<enumeration value="n50"/>

<enumeration value="n100"/>

<enumeration value="n200"/>

</restriction>

</simpleType>

<simpleType name="PowerRampingStepEnumType">

<restriction base="string">

<enumeration value="dB0"/>

<enumeration value="dB2"/>

<enumeration value="dB4"/>

<enumeration value="dB6"/>

</restriction>

</simpleType>

<simpleType name="QHystEnumType">

<restriction base="string">

<enumeration value="dB0"/>

<enumeration value="dB1"/>

<enumeration value="dB2"/>

<enumeration value="dB3"/>

<enumeration value="dB4"/>

<enumeration value="dB5"/>

<enumeration value="dB6"/>

<enumeration value="dB8"/>

<enumeration value="dB10"/>

<enumeration value="dB12"/>

<enumeration value="dB14"/>

<enumeration value="dB16"/>

<enumeration value="dB18"/>

<enumeration value="dB20"/>

<enumeration value="dB22"/>

<enumeration value="dB24"/>

</restriction>

</simpleType>

<simpleType name="ResponseWindowSizeEnumType">

<restriction base="string">

<enumeration value="sf2"/>

<enumeration value="sf3"/>

<enumeration value="sf4"/>

<enumeration value="sf5"/>

<enumeration value="sf6"/>

<enumeration value="sf7"/>

<enumeration value="sf8"/>

<enumeration value="sf10"/>

</restriction>

</simpleType>

<simpleType name="SizeOfRAPreambleGroupAEnumType">

<restriction base="string">

<enumeration value="n4"/>

<enumeration value="n8"/>

<enumeration value="n12"/>

<enumeration value="n16"/>

<enumeration value="n20"/>

<enumeration value="n24"/>

<enumeration value="n28"/>

<enumeration value="n32"/>

<enumeration value="n36"/>

<enumeration value="n40"/>

<enumeration value="n44"/>

<enumeration value="n48"/>

<enumeration value="n52"/>

<enumeration value="n56"/>

<enumeration value="n60"/>

</restriction>

</simpleType>

<simpleType name="TimeToTriggerEUtraEnumType">

<restriction base="string">

<enumeration value="ms0"/>

<enumeration value="ms40"/>

<enumeration value="ms64"/>

<enumeration value="ms80"/>

<enumeration value="ms100"/>

<enumeration value="ms128"/>

<enumeration value="ms160"/>

<enumeration value="ms256"/>

<enumeration value="ms320"/>

<enumeration value="ms480"/>

<enumeration value="ms512"/>

<enumeration value="ms640"/>

<enumeration value="ms1024"/>

<enumeration value="ms1280"/>

<enumeration value="ms2560"/>

<enumeration value="ms5120"/>

</restriction>

</simpleType>

<simpleType name="ThresholdRsrpRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="97"/>

</restriction>

</simpleType>

<simpleType name="ThresholdRsrqRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="34"/>

</restriction>

</simpleType>

<simpleType name="OffsetRangeType">

<restriction base="short">

<minInclusive value="-30"/>

<maxInclusive value="30"/>

</restriction>

</simpleType>

<simpleType name="ThresholdUtraRscpRangeType">

<restriction base="short">

<minInclusive value="-5"/>

<maxInclusive value="91"/>

</restriction>

</simpleType>

<simpleType name="ThresholdUtraEcN0RangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="49"/>

</restriction>

</simpleType>

<simpleType name="ThresholdGeranRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="63"/>

</restriction>

</simpleType>

<simpleType name="ThresholdCDMA2000RangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="63"/>

</restriction>

</simpleType>

<simpleType name="CommonChannelPowerOffsetRangeType">

<restriction base="short">

<minInclusive value="-350"/>

<maxInclusive value="150"/>

</restriction>

</simpleType>

<simpleType name="ConfigurationIndexRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="63"/>

</restriction>

</simpleType>

<simpleType name="HysteresisRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="30"/>

</restriction>

</simpleType>

<simpleType name="PMaxRangeType">

<restriction base="short">

<minInclusive value="-30"/>

<maxInclusive value="33"/>

</restriction>

</simpleType>

<simpleType name="QOffsetRangeType">

<restriction base="short">

<minInclusive value="-15"/>

<maxInclusive value="15"/>

</restriction>

</simpleType>

<simpleType name="QQualMinUtraRangeType">

<restriction base="short">

<minInclusive value="-24"/>

<maxInclusive value="0"/>

</restriction>

</simpleType>

<simpleType name="QRxLevMinEUtraRangeType">

<restriction base="short">

<minInclusive value="-77"/>

<maxInclusive value="-22"/>

</restriction>

</simpleType>

<simpleType name="QRxLevMinGeranRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="63"/>

</restriction>

</simpleType>

<simpleType name="QRxLevMinUtraRangeType">

<restriction base="short">

<minInclusive value="-60"/>

<maxInclusive value="-13"/>

</restriction>

</simpleType>

<simpleType name="RootSequenceIndexRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="837"/>

</restriction>

</simpleType>

<simpleType name="SIntraSearchRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="31"/>

</restriction>

</simpleType>

<simpleType name="TReselectionRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="7"/>

</restriction>

</simpleType>

<simpleType name="TStoreUeContextRangeType">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="1023"/>

</restriction>

</simpleType>

<complexType name="TceIDMappingInfo">

<sequence>

<element name="tceID" type="short"/>

<element name="tceIPAddr" type="string"/>

</sequence>

</complexType>

<complexType name="TceIDMappingInfoList">

<sequence>

<element name="tceIDMappingInfo" type="en:TceIDMappingInfo" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="SharNetTceMappingInfo">

<sequence>

<element name="pLMNId" type="en:PLMNId"/>

<element name="tceID" type="short"/>

<element name="tceIPAddr" type="string"/>

</sequence>

</complexType>

<complexType name="SharNetTceMappingInfoList">

<sequence>

<element name="sharNetTceMappingInfo" type="en:SharNetTceMappingInfo" minOccurs="0"/>

</sequence>

</complexType>

<simpleType name="cellOutageCompensationState">

<restriction base="string">

<enumeration value="cOCActivating"/>

<enumeration value="cOCActive"/>

<enumeration value="cOCDeactivating"/>

<enumeration value="cOCDeactive"/>

</restriction>

</simpleType>

<complexType name="cellOutageCompensationStatus">

<sequence>

<element name="cellOutageCompensationState" type="en:cellOutageCompensationState"/>

<element name="errorList" type="xn:dnList"/>

</sequence>

</complexType>

<element name="CellOutageCompensationInformation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="cellOutageCompensationStatus"

type="en:cellOutageCompensationStatus"/>

<element name="isCOCAllowed" type="boolean"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<simpleType name="NumOfCRSPorts">

<restriction base="unsignedShort">

<minInclusive value="1"/>

<maxInclusive value="2"/>

</restriction>

</simpleType>

<complexType name="ReferenceSignalPattern">

<sequence>

<element name="prsSupported" type="en:yesNoType"/>

</sequence>

</complexType>

<simpleType name="ReferenceSignalPeriodicity">

<restriction base="string">

<enumeration value="ms1280"/>

<enumeration value="ms2560"/>

<enumeration value="ms5120"/>

<enumeration value="ms10240"/>

</restriction>

</simpleType>

<simpleType name="ReferenceSignalOffset">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="10239"/>

</restriction>

</simpleType>

<complexType name="NetListeningRefSignalType">

<sequence>

<element name="referenceSignalPattern" type="en:ReferenceSignalPattern"/>

<element name="numOfCRSPorts" type="en:NumOfCRSPorts"/>

<element name="referenceSignalPeriodicity" type="en:ReferenceSignalPeriodicity"/>

<element name="referenceSignalOffset" type="en:ReferenceSignalOffset"/>

</sequence>

</complexType>

<complexType name="NetListeningRSForRIBS">

<sequence>

<element name="netListeningRefSignal" type="en:NetListeningRefSignalType" maxOccurs="4"/>

</sequence>

</complexType>

<simpleType name="WLANOperationalStateType">

<restriction base="string">

<enumeration value="enabled"/>

<enumeration value="disabled"/>

</restriction>

</simpleType>

<complexType name="WLANInfoType">

<sequence>

<element name="wLANId" type="string"/>

<element name="wLANOperationalState" type="en:WLANOperationalStateType"/>

<element name="enbWithLWARelation" type="xn:dn"/>

</sequence>

</complexType>

<complexType name="WLANInfoTypeListType">

<sequence>

<element name="wLANInfo" type="en:WLANInfoType"/>

</sequence>

</complexType>

<complexType name="GeoLocationType">

<sequence>

<element name="latitude" type="short"/>

<element name="longitude" type="short"/>

</sequence>

</complexType>

<complexType name="lWIPSeGWListType">

<sequence>

<element name="LWIPSeGWId" type="string"/>

<element name="LWIPSeGWIpAddressList" type="en:IpAddressList"/>

</sequence>

</complexType>

<element name="EUtranCellNMCentralizedSON">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="a1ThresholdRsrp" type="en:ThresholdRsrpRangeType" minOccurs="0"/>

<element name="a1ThresholdRsrq" type="en:ThresholdRsrqRangeType" minOccurs="0"/>

<element name="a2ThresholdRsrp" type="en:ThresholdRsrpRangeType" minOccurs="0"/>

<element name="a2ThresholdRsrq" type="en:ThresholdRsrqRangeType" minOccurs="0"/>

<element name="a3Offset" type="en:OffsetRangeType" minOccurs="0"/>

<element name="a4ThresholdRsrp" type="en:ThresholdRsrpRangeType" minOccurs="0"/>

<element name="a4ThresholdRsrq" type="en:ThresholdRsrqRangeType" minOccurs="0"/>

<element name="a5Threshold1Rsrp" type="en:ThresholdRsrpRangeType" minOccurs="0"/>

<element name="a5Threshold1Rsrq" type="en:ThresholdRsrqRangeType" minOccurs="0"/>

<element name="b1ThresholdUtraRscp" type="en:ThresholdUtraRscpRangeType" minOccurs="0"/>

<element name="b1ThresholdUtraEcN0" type="en:ThresholdUtraEcN0RangeType" minOccurs="0"/>

<element name="b1ThresholdGeran" type="en:ThresholdGeranRangeType" minOccurs="0"/>

<element name="b1ThresholdCdma2000" type="en:ThresholdCDMA2000RangeType" minOccurs="0"/>

<element name="b2Threshold1Rsrp" type="en:ThresholdRsrpRangeType" minOccurs="0"/>

<element name="b2Threshold1Rsrq" type="en:ThresholdRsrqRangeType" minOccurs="0"/>

<element name="b2Threshold2UtraRscp" type="en:ThresholdUtraRscpRangeType" minOccurs="0"/>

<element name="b2Threshold2UtraEcN0" type="en:ThresholdUtraEcN0RangeType" minOccurs="0"/>

<element name="b2Threshold2Geran" type="en:ThresholdGeranRangeType" minOccurs="0"/>

<element name="b2Threshold2Cdma2000" type="en:ThresholdCDMA2000RangeType" minOccurs="0"/>

<element name="commonChannelPowerOffset" type="en:CommonChannelPowerOffsetRangeType" minOccurs="0"/>

<element name="configurationIndex" type="en:ConfigurationIndexRangeType"  minOccurs="0"/>

<element name="contentionResolutionTimer" type="en:ContentionResolutionTimerEnumType" minOccurs="0"/>

<element name="hysteresisEutraA1" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="hysteresisEutraA2" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="hysteresisEutraA3" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="hysteresisEutraA4" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="hysteresisEutraA5" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="hysteresisIratB1" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="hysteresisIratB2" type="en:HysteresisRangeType" minOccurs="0"/>

<element name="numberOfRaPreambles" type="en:NumberOfRaPreamblesEnumType" minOccurs="0"/>

<element name="preambleInitialReceivedTargetPower" type="en:PreambleInitialReceivedTargetPowerEnumType" minOccurs="0"/>

<element name="preambleTransMax" type="en:PreambleTransMaxEnumType" minOccurs="0"/>

<element name="pMax" type="en:PMaxRangeType" minOccurs="0"/>

<element name="powerRampingStep" type="en:PowerRampingStepEnumType" minOccurs="0"/>

<element name="qHyst" type="en:PreambleInitialReceivedTargetPowerEnumType" minOccurs="0"/>

<element name="qOffsetUtra" type="en:QOffsetRangeType" minOccurs="0"/>

<element name="qOffsetGeran" type="en:QOffsetRangeType" minOccurs="0"/>

<element name="qOffsetCdma2000" type="en:QOffsetRangeType" minOccurs="0"/>

<element name="qQualMinUtra" type="en:QQualMinUtraRangeType" minOccurs="0"/>

<element name="qRxLevMinEUtraSib1" type="en:QRxLevMinEUtraRangeType" minOccurs="0"/>

<element name="qRxLevMinEUtraSib3" type="en:QRxLevMinEUtraRangeType" minOccurs="0"/>

<element name="qRxLevMinGeran" type="en:QRxLevMinGeranRangeType" minOccurs="0"/>

<element name="qRxLevMinUtra" type="en:QRxLevMinUtraRangeType" minOccurs="0"/>

<element name="responseWindowSize" type="en:ResponseWindowSizeEnumType" minOccurs="0"/>

<element name="rootSequenceIndex" type="en:RootSequenceIndexRangeType"  minOccurs="0"/>

<element name="sIntraSearch" type="en:SIntraSearchRangeType" minOccurs="0"/>

<element name="sizeOfRAPreamblesGroupA" type="en:SizeOfRAPreambleGroupAEnumType" minOccurs="0"/>

<element name="timeToTriggerEutraA1" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="timeToTriggerEutraA2" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="timeToTriggerEutraA3" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="timeToTriggerEutraA4" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="timeToTriggerEutraA5" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="timeToTriggerIratB1" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="timeToTriggerIratB2" type="en:TimeToTriggerEUtraEnumType" minOccurs="0"/>

<element name="tReselectionCdma2000" type="en:TReselectionRangeType" minOccurs="0"/>

<element name="tReselectionEUtra" type="en:TReselectionRangeType" minOccurs="0"/>

<element name="tReselectionGeran" type="en:TReselectionRangeType" minOccurs="0"/>

<element name="tReselectionUtra" type="en:TReselectionRangeType" minOccurs="0"/>

<element name="tStoreUeContext" type="en:TStoreUeContextRangeType" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ENBFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="intraANRSwitch" type="boolean" minOccurs="0"/>

<element name="iRATANRSwitch" type="boolean" minOccurs="0"/>

<element name="enbId" type="en:EnbId" minOccurs="0"/>

<element name="x2BlackList" type="xn:dnList" minOccurs="0"/>

<element name="x2WhiteList" type="xn:dnList" minOccurs="0"/>

<element name="x2HOBlackList" type="xn:dnList" minOccurs="0"/>

<element name="x2IpAddressList" type="string" minOccurs="0"/>

<element name="tceIDMappingInfoList" type="en:TceIDMappingInfoList" minOccurs="0"/>

<element name="sharNetTceMappingInfoList" type="en:SharNetTceMappingInfoList" minOccurs="0"/>

<element name="netListeningRSForRIBS" type="en:NetListeningRSForRIBS" minOccurs="0"/>

<element name="lWIPSeGWList" type="en:lWIPSeGWListType" minOccurs="0"/>

<element name=" ephemerisInfoSetRef " type="xn:dn" minOccurs="0"/>

<!-- linkList attribute is to be added when defined in the IS -->

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:EUtranCellFDD"/>

<element ref="en:EUtranCellTDD"/>

<element ref="epc:EP_RP_EPS"/>

<element ref="nn:EP_X2C"/>

<element ref="nn:EP_X2U"/>

<element ref="nn:EP_NgC"/>

<element ref="nn:EP_NgU"/>

<element ref="nn:EP_XnC"/>

<element ref="nn:EP_XnU"/>

<element ref="en:ENBFunctionOptionallyContainedNrmClass"/>

<element ref="en:DeNBCapability"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="RNFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="servingCell" type="xn:dn"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="epc:EP_RP_EPS"/>

<element ref="en:RNFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="DeNBCapability">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="servedRN" type="xn:dnList" minOccurs="0"/>

<element name="maxNbrRNAllowed" type="unsignedShort"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalRNFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="candidateDeNBCells" type="en:EcgiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="epc:EP_RP_EPS"/>

<element ref="en:ExternalRNFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalENBFunction" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="enbId" type="en:EnbId" minOccurs="0"/>

<!-- Attributes are to be added when defined in the IS -->

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:ExternalEUtranCellFDD"/>

<element ref="en:ExternalEUtranCellTDD"/>

<element ref="en:ExternalENBFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:InterRatEsPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EUtranCellFDD">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EUtranGenericCell-->

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="cellLocalId" type="en:CellLocalId"/>

<element name="cellLocalIdList" type="en:CellLocalIdList" minOccurs="0"/>

<element name="cellSize" type="en:cellSize"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="cellAccessInfoList" type="en:CellAccessInfoList"/>

<element name="tac" type="long"/>

<element name="nTNTAClist" type="en:nTNTAClistType" minOccurs="0"/>

<element name="pci" type="en:Pci"/>

<element name="pciList" type="en:PciList" minOccurs="0"/>

<element name="maximumTransmissionPower" type="short"/>

<element name="partOfSectorPower" type="short" minOccurs="0"/>

<element name="referenceSignalPower" type="short"/>

<element name="pb" type="short"/>

<element name="relatedTmaList" type="xn:dnList" minOccurs="0"/>

<element name="relatedAntennaList" type="xn:dnList" minOccurs="0"/>

<element name="relatedSector" type="xn:dn" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType"

minOccurs="0"/>

<element name="availabilityStatus" type="sm:availabilityStatusType"

minOccurs="0"/>

<element name="allowedAccessClasses" type="en:allowedAccessClassesType"/>

<element name="cellResvInfo" type="en:cellResvInfoType" minOccurs="0"/>

<element name="nbIoTcellFlag" type="en:yesNoType" minOccurs="0"/>

<element name="isChangeForEnergySavingAllowed"

type="en:yesNoType" minOccurs="0"/>

<element name="ngranCellFlag" type="en:yesNoType" minOccurs="0"/>

<element name="nSSAI" type="nr:nSSAI" minOccurs="0"/>

<!-- End of inherited attributes from EUtranGenericCell -->

<element name="earfcnDl" type="short"/>

<element name="earfcnUl" type="short"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:EUtranRelation"/>

<element ref="en:Cdma2000Relation"/>

<element ref="gn:GsmRelation"/>

<element ref="un:UtranRelation"/>

<element ref="en:EUtranCellFDDOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalEUtranCellFDD" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from ExternalEUtranGenericCell-->

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pci" type="en:Pci"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="cellAccessInfoList" type="en:CellAccessInfoList"/>

<element name="cellLocalId" type="en:CellLocalId"/>

<element name="enbId" type="en:EnbId" minOccurs="0"/>

<!-- End of inherited attributes from ExternalEUtranGenericCell -->

<element name="earfcnDl" type="short"/>

<element name="earfcnUl" type="short"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:ExternalEUtranCellFDDOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0">

<element ref="en:CellOutageCompensationInformation"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:InterRatEsPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EUtranCellTDD">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EUtranGenericCell-->

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="cellLocalId" type="en:CellLocalId"/>

<element name="cellLocalIdList" type="en:CellLocalIdList" minOccurs="0"/>

<element name="cellSize" type="en:cellSize"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="cellAccessInfoList" type="en:CellAccessInfoList"/>

<element name="tac" type="long"/>

<element name="nTNTAClist" type="en:nTNTAClistType" minOccurs="0"/>

<element name="pci" type="en:Pci" />

<element name="pciList" type="en:PciList" minOccurs="0"/>

<element name="maximumTransmissionPower" type="short"/>

<element name="partOfSectorPower" type="short" minOccurs="0"/>

<element name="referenceSignalPower" type="short"/>

<element name="pb" type="short"/>

<element name="relatedTmaList" type="xn:dnList" minOccurs="0"/>

<element name="relatedAntennaList" type="xn:dnList" minOccurs="0"/>

<element name="relatedSector" type="xn:dn" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType"

minOccurs="0"/>

<element name="availabilityStatus" type="sm:availabilityStatusType"

minOccurs="0"/>

<element name="allowedAccessClasses" type="en:allowedAccessClassesType"/>

<element name="cellResvInfo" type="en:cellResvInfoType" minOccurs="0"/>

<element name="nbIoTcellFlag" type="en:yesNoType" minOccurs="0"/>

<element name="isChangeForEnergySavingAllowed"

type="en:yesNoType" minOccurs="0"/>

<element name="ngranCellFlag" type="en:yesNoType" minOccurs="0"/>

<element name="nSSAI" type="nr:nSSAI" minOccurs="0"/>

<!-- End of inherited attributes from EUtranGenericCell -->

<element name="earfcn" type="short"/>

<element name="sfAssignment" type="short"/>

<element name="specialSfPatterns" type="short"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:EUtranRelation"/>

<element ref="en:Cdma2000Relation"/>

<element ref="gn:GsmRelation"/>

<element ref="un:UtranRelation"/>

<element ref="en:EUtranCellTDDOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0">

<element ref="en:CellOutageCompensationInformation"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalEUtranCellTDD" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from ExternalEUtranGenericCell-->

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>                  <element name="pci" type="en:Pci"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="cellLocalId" type="en:CellLocalId"/>

<element name="enbId" type="en:EnbId" minOccurs="0"/>

<!-- End of inherited attributes from ExternalEUtranGenericCell -->

<element name="earfcn" type="short"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:ExternalEUtranCellTDDOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:InterRatEsPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EUtranRelation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="tCI" type="long" minOccurs="0"/>

<element name="isRemoveAllowed" type="boolean" minOccurs="0"/>

<element name="isHOAllowed" type="boolean" minOccurs="0"/>

<element name="isICICInformationSendAllowed" type="boolean" minOccurs="0"/>

<element name="isLBAllowed" type="boolean" minOccurs="0"/>

<element name="adjacentCell" type="xn:dn"/>

<element name="isEsCoveredBy" type="en:isEsCoveredByEnumType" minOccurs="0"/>

<element name="cellIndividualOffset" type="en:QOffsetEnumType" minOccurs="0"/>

<element name="qOffset" type="en:QOffsetEnumType" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:EUtranRelationOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Cdma2000Relation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="adjacentSector" type="xn:dn"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:Cdma2000RelationOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Link_ENB_ENB" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from Link -->

<element name="aEnd" type="xn:dn"/>

<element name="linkType" type="xn:linkType" minOccurs="0"/>

<element name="protocolName" type="string" minOccurs="0"/>

<element name="protocolVersion" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="zEnd" type="xn:dn"/>

<!-- End of inherited attributes from Link -->

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:Link_ENB_ENBOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MCEFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<!-- Attributes are to be added when defined in the IS -->

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:MCEFunctionOptionallyContainedNrmClass"/>

<element ref="en:MBSFNArea"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MBSFNArea" >

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="mbsfnAreaId" type="en:mbsfnAreaIdType" minOccurs="0"/>

<element name="cellIdList" type="xn:dnList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:MBSFNAreaOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Link_MCE_ENB" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from Link -->

<element name="aEnd" type="xn:dn" minOccurs="0"/>

<element name="linkType" type="xn:linkType" minOccurs="0"/>

<element name="protocolName" type="string" minOccurs="0"/>

<element name="protocolVersion" type="string" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<element name="zEnd" type="xn:dn" minOccurs="0"/>

<!-- End of inherited attributes from Link -->

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:Link_MCE_ENBOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Link_MCE_MME" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from Link -->

<element name="aEnd" type="xn:dn" minOccurs="0"/>

<element name="linkType" type="xn:linkType" minOccurs="0"/>

<element name="protocolName" type="string" minOccurs="0"/>

<element name="protocolVersion" type="string" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<element name="zEnd" type="xn:dn" minOccurs="0"/>

<!-- End of inherited attributes from Link -->

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:Link_MCE_MMEOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="QciDscpMapping"

substitutionGroup="en:RNFunctionOptionallyContainedNrmClass"

>

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="QciDscpMappingList" type="en:QciDscpMappingListType"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EUtranFreqRelation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="cellIndividualoffset" type="nr:qOffsetRangeList"/>

<element name="blackListEntry" type="nr:blackListEntry" minOccurs="0"/>

<element name="blackListEntryIdleMode" type="nr:blackListEntryIdleMode" minOccurs="0"/>

<element name="cellReselectionPriority" type="nr:cellReselectionPriority"/>

<element name="cellReselectionSubPriority" type="nr:cellReselectionSubPriority"/>

<element name="pMax" type="nr:PMaxRangeType" minOccurs="0"/>

<element name="qOffserFreq" type="nr:qOffserFreq" minOccurs="0"/>

<element name="qQualMin" type="nr:qQualMin" minOccurs="0"/>

<element name="qRxLevMin" type="nr:qRxLevMin" minOccurs="0"/>

<element name="threshXHighP" type="nr:threshxhighp" minOccurs="0"/>

<element name="threshXHighQ" type="nr:threshxhighq" minOccurs="0"/>

<element name="threshXLowP" type="nr:threshxlowp" minOccurs="0"/>

<element name="threshXLowQ" type="nr:threshxlowp" minOccurs="0"/>

<element name="tReselectionEutran" type="nn:Treselectionnr" minOccurs="0"/>

<element name="tReselectionEutranSfHigh" type="nn:Treselectionnrsfhigh" minOccurs="0"/>

<element name="tReselectionEutranSfMedium" type="nn:Treselectionnrsfmedium" minOccurs="0"/>

<element name="eUtranFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EUtranFrequency" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="earfcnDl" type="short"/>

<element name="multiFrequencyBandListEutra" type="nn:MultifrequencyBandlistnr" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="WTFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="wLANInfoList" type="en:WLANInfoListType" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:WTFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_Xw">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="farEndEntity" type="xn:dn"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:EP_xwOptionallyContainedNrmClass"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="WLANMobilitySet" substitutionGroup="en:ENBFunctionOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="en:WLANMobilitySetOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MemberWLAN" substitutionGroup="en:WLANMobilitySetOptionallyContainedNrmClass">

<complexType>

<complexContent>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="wLANId" type="string"/>

<element name="wLANGeoLocation" type="en:GeoLocationType" minOccurs="0"/>

<element name="isLWASupported" type="boolean"/>

<element name="isLWIPSupported" type="boolean"/>

</all>

</complexType>

</element>

</complexContent>

</complexType>

</element>

<!--  The element definition for EP_RP_EPS is available through

the epcNrm.xsd (3GPP TS 28.709), by using epc:EP_RP_EPS -->

<element name="ENBFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="ExternalENBFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="EUtranCellFDDOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="ExternalEUtranCellFDDOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="EUtranCellTDDOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="ExternalEUtranCellTDDOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="EUtranRelationOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="Cdma2000RelationOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="Link_ENB_ENBOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="MCEFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="Link_MCE_ENBOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="Link_MCE_MMEOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="MBSFNAreaOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="RNFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="ExternalRNFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="WTFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="EP_xwOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="WLANMobilitySetOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

</schema>