3GPP TSG-SA5 Meeting #161	S5-252603

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0087 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Update EE KPIs overview |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion for multi-dimensional network energy efficiency metrics in TR 28.880, it is proposed to update EE KPIs overview accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding overview content for the multi-dimensional energy efficiency metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 4.1	EE KPIs Overview

Telecommunication networks energy efficiency KPIs are defined by various SDOs / organizations and are of various natures. They can be applied to either:

-	whole networks (i.e. end-to-end), or to

-	sub-networks (e.g. the radio access network), or to

-	single network elements, or to

-	telecommunication sites, which contain network elements and site equipment.

NOTE 1:	Data centers used by network operators are considered in the present document as telecommunication sites.

Moreover, EE KPIs can also be categorized according to the operator's network life cycle phase they may apply to, e.g.:

-	during the Buy phase, mobile network operators may be willing to compare network elements from various vendors from an EE standpoint. Some EE KPIs and measurement methods have been specified for this purpose.

-	during the Design / Build phase, mobile network operators are always faced to several design options, and may be willing to compare them from an EE standpoint. This may happen for the whole network, sub-networks and for telecom sites. For telecom sites, EE KPIs have been specified.

-	during the Run phase, mobile network operators need to assess the energy efficiency of the live network, as a whole (i.e. end-to-end), or for sub-networks, or for single network elements or telecom sites. Some EE KPIs and measurement methods have also been specified for this purpose.

NOTE 2: 	EE KPIs in the present document are only applicable for the Run phase.

Generally, EE KPI is defined as the ratio between performance and energy consumption. Depending on different EE evaluation scenarios, the performance may be measured based on different performance indicators, e.g. data volume, latency, number of active users, etc.

The EE performance indicators can be classified into following performance dimensions:

-	Network data traffic, which measures the number or amount of network traffic provided, such as the amount of data transferred as Data Volume (DV) or the number of connections.

-	Network quality, which measures the quality of network provided, such as user throughput, latency.

-	Network availability, which measures the availability of network, such as cell availability and radio access network availability.

The calculation of the energy efficiency of 5G networks relies on the following principles:

- 	it is based on the two high-level EE KPIs defined in ETSI ES 203 228 [2]:

- , and

-

- 	EEMN,DV may apply to the whole 5G network whereas EEMN,CoA may apply only to NG-RAN;

- 	EEMN,DV requires the collection of both Data Volumes (DV) and Energy Consumption (EC) of 5G Network Functions (NF);

- 	In NG-RAN, DV is measured per cell;

- 	In 5GC, DV is measured per NF;

- 	EC definition and measurement method for 5G PNFs rely on ETSI ES 202 336-1 [3] and ETSI ES 202 336-12 [4];

- 	EC is measured by PEE parameters (cf. ETSI ES 202 336-12 [4] – Annexes A and B);

- 	PEE measurements requirements for all deployment scenario in NG-RAN: The 3GPP management system responsible for the management of the gNB (single or multiple vendor gNB) shall be able to collect PEE measurements data from all PNFs in the gNB, in the same way as the other PM measurements;

- 	When gNBCU/gNBCU-CP/gNBCU-UP energy consumption is assumed to be very small compared to gNBDU and given that, in some cases, the gNBCU/gNBCU-CP/gNBCU-UP may be virtualized, the present document only considers the energy consumed in gNBDU(s) (in case of split scenarios) and in non-split gNBs (see clause 4.2.1 of 3GPP TS 28.541 [11] and clause 6.1.1 of 3GPP TS 38.401 [12]). There might be a need for some correction in KPI between the different deployment scenarios.

NOTE 3: 	The vendor(s) of 2-split (gNBDU/gNBCU) or 3-split gNB/en-gNB components (gNBDU/gNBCU-CP/gNBCU-UP) may be same or different depending on the implementations.

- 	In the present document, it is assumed that NG-RAN is only composed of base stations with built-in sensors (see ETSI ES 202 336-12 [4] – clause 4.4.1).

Besides the parameters required to calculate the energy efficiency, e.g. DV and EC, other parameters may be used to interpret variations in energy efficiency KPI values from different networks. These parameters can be classified into demography, topography and climate classes (see ETSI ES 203 228 [2] – section 4.3), which describe the network characteristics with regard to population density, geographical conditions and climate zones. For each class of parameters, there can be subclasses, e.g. demography can be further classified into dense urban, urban, sub-urban, rural or unpopulated scenarios. For each class / subclass, the energy efficiency KPI values may be interpreted differently.

End of change